3GPP TSG-RAN5 Meeting #107	R5-252687

# Malta, Malta, May.19th – 23rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0976 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of minimum test time for V2X Demod test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add minimum test time for V2X Demod test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Minimum test time for V2X Demod test cases is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.5.4, G.5.5, G.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

G.5.4	Test time for PSSCH performance requirements with throughput

The throughput requirements are tested in terms of PSSCH BLER. The maximum BLER requirement for all tests is 10%. The tests are performed in a variety of AWGN and fading propagation models. For each test in Table G.5.4-1 the test time (in subframes) is to be applied to the test method described in section G.5.2

Table G.5.4-1: Minimum test time for PSSCH performance requirements with throughput

| Test num. | Performance Scenario | Minimum Number of Active Subframes (MNAS) | Minimum Number of Subframes (MNS)(Note 1) | Test Time in Subframes (Note 2) |
| --- | --- | --- | --- | --- |
| 11.1.2-1 | R.PSSCH.2-1.1 1x2 Low TDLA30-2700 | 15 000 (MNAS not simulated, but conservatively estimated based on existing TDL A fading scenarios with close but smaller Doppler spread) | 15 000 | 30 000 |
| 11.1.2-2 | R.PSSCH.2-1.2 1x2 Low TDLA30-1400 | 15 000 (MNAS not simulated, but conservatively estimated based on existing TDL A fading scenarios with close but smaller Doppler spread) | 15 000 | 30 000 |
| 11.1.2-3 | R.PSSCH.2-1.3 1x2 Low TDLA30-180 | 20 000 (MNAS not simulated, but conservatively estimated based on existing TDLA fading scenarios with close but smaller Doppler spread) | 20 000 | 20 000 |
| 11.1.6-1 | R.PSSCH.2-1.4 1x2 Low AWGN 2 PSSCH transmissions | 30 000 (MNAS not simulated, but conservatively estimated based on existing scenarios) | 30 000 | 30 000 |
| 11.1.7-1 | R.PSSCH.2-1.5 1x2 Low AWGN Number of PSSCH transmissions depends on UE capability | 30 000 (MNAS not simulated, but conservatively estimated based on existing scenarios) | 30 000 | 30 000 |
| Note 1: The Minimum Number of Subframes is the total minimum number of subframes (active and inactive) required for this demodulation scenario and is derived from the MNAS, according to the resource pool configuration defined for each test case. Note 2: The Test Time is based on the Minimum Number of Subframes (MNS) according to the formula: Test Time in Subframes = 1000*CEIL(MNS/1000). |  |  |  |  |

G.5.5	Test time for PSCCH performance requirements with miss-detection probability

The probability of miss-detection requirements are tested in terms of Probability of PSCCH miss-detection. The maximum Probability of PSCCH miss-detection for all tests is 1%. The tests are performed in a variety of Static, AWGN and fading propagation models. For each test in Table G.5.5-1 the Test Time in Subframes is to be applied to the test method described in section G.5.2.

Table G.5.5-1: Minimum Test Time for PSCCH Performance Requirements with Probability of Miss-Detection

| Test num. | Performance Scenario | Minimum Number of Active Subframes (MNAS) | Minimum Number of Subframes (MNS)(Note 1) | Test Time in Subframes (Note 2) |
| --- | --- | --- | --- | --- |
| 11.1.3-1 | R.PSCCH.2-1.1 20 / 30 1x2 Low TDLA30-1400 | 15 000 (MNAS not simulated, but conservatively estimated based on existing fading scenarios with close but smaller Doppler spread) | 15 000 | 15 000 |
| 11.1.8-1 | R.PSCCH.2-1.1 40 / 30 1x2 Low Static | 30 000 (MNAS not simulated, but conservatively estimated based on existing fading scenarios) | 30 000 | 30 000 |
| Note 1: The Minimum Number of Subframes is the total minimum number of subframes (active and inactive) required for this demodulation scenario and is derived from the MNAS, according to the resource pool configuration defined for each test case. Note 2: The Test Time is based on the Minimum Number of Subframes (MNS) according to the formula: Test Time in Subframes = 1000*CEIL(MNS/1000). |  |  |  |  |

G.5.6	Test time for PSFCH performance requirements with miss-detection probability

The probability of miss-detection requirements are tested in terms of Probability of PSFCH miss-detection. The maximum Probability of PSFCH miss-detection for all tests is 1%. The tests are performed in a variety of Static, AWGN and fading propagation models. For each test in Table G.5.6-1 the Test Time in Subframes is to be applied to the test method described in section G.5.3.

Table G.5.6-1: Minimum Test Time for PSFCH Performance Requirements with Probability of Miss-Detection

| Test num. | Performance Scenario | Minimum Number of Active Subframes (MNAS) | Minimum Number of Subframes (MNS)(Note 1) | Test Time in Subframes (Note 2) |
| --- | --- | --- | --- | --- |
| 11.1.5-1 | 20 / 30 1x2 Low TDLA30-180 | 15 000 (MNAS not simulated, but conservatively estimated based on existing fading scenarios with close but smaller Doppler spread) | 15 000 | 15 000 |
| 11.1.9-1 | 40 / 30 1x2 Low Static | 30 000 (MNAS not simulated, but conservatively estimated based on existing fading scenarios) | 30 000 | 30 000 |
| Note 1: The Minimum Number of Subframes is the total minimum number of subframes (active and inactive) required for this demodulation scenario and is derived from the MNAS, according to the resource pool configuration defined for each test case. Note 2: The Test Time is based on the Minimum Number of Subframes (MNS) according to the formula: Test Time in Subframes = 1000*CEIL(MNS/1000). |  |  |  |  |

<End of Change 1>