3GPP TSG-RAN5 Meeting #107	R5-252150

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4956 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SL SDAP TC 12.1.11.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1.The groupcast DRB should be pre-configured. 2.There are some incomplete IE in specific message content. 3.TP2 need to be updated, since the reception side cannot distinguish QoS flows with different PQFI for groupcast SL-DRB without SDAP header. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 1.The pre-configuration for groupcast SL-DRB has been updated. - 2.The missing IE contents have been added. - 3.TP2 has been updated. - 4.Editorial changes have been done. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be incorrect and IE will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.11.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

12.1.11.1.2	PC5-only operation / SDAP / data transfer / reception side

12.1.11.1.2.1	Test Purpose (TP)

(1)

with { UE is configured to perform sidelink reception. UE has established PC5-RRC connection and a unicast SL DRB with peer UE with more than one PC5 QoS flows mapped }

ensure that {

when { UE receives an SDAP PDU with SDAP header on the unicast SL DRB }

then { UE retrieves the SDAP SDU and delivers it to upper layer }

}

(2)

with { UE is configured to perform sidelink reception. UE has established a groupcast SL DRB }

ensure that {

when { UE receives an SDAP PDU without SDAP header on the groupcast SL DRB }

then { UE retrieves the SDAP SDU and delivers it to upper layer }

}

12.1.11.1.2.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 37.324, clauses 5.2.4. Unless otherwise stated these are Rel-16 requirements.

[TS 37.324 clause 5.2.4]

At the reception of an SDAP data PDU from lower layers for a PC5 QoS flow, the receiving SDAP entity shall:

-	if the SL-DRB from which this SDAP data PDU is received is configured by RRC (TS 38.331 [3]) with the presence of SDAP header:

-	retrieve the SDAP SDU from the SL SDAP data PDU as specified in the clause 6.2.2.4;

-	else:

-	retrieve the SDAP SDU from the SL SDAP data PDU as specified in the clause 6.2.2.1;

-	deliver the retrieved SDAP SDU to the upper layer.

12.1.11.1.2.3	Test description

12.1.11.1.2.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 is as defined in TS 38.508-1 [4], configured for and operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3 except for those listed in Table 12.1.11.1.2.3.1-1.

-	The UE uses GNSS as the synchronization reference source.

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

Table 12.1.11.1.2.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°119 (V2X) supported |  |
| EFVST |  | As per TS 38.508-1 [4] clause 4.8.3.3.3 |  |
| EFV2XP_PC5 |  | As per TS 38.508-1[4] clause 4.8.3.3.3 SL-PreconfigurationNR included in V2X data policy over PC5 is defined in Table 12.1.11.1.2.3.3-1 |  |

12.1.11.1.2.3.2	Test procedure sequence

Table 12.1.11.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | NR-SS-UE1 transmits a DIRECT LINK MODIFICATION REQUEST to add QoS flows | <-- | PC5-S: DIRECT LINK MODIFICATION REQUEST | - | - |
| 2 | UE transmits a DIRECT LINK MODIFICATION ACCEPT message. | --> | PC5-S: DIRECT LINK MODIFICATION ACCEPT | - | - |
| 3 | NR-SS-UE1 sends an RRCReconfigurationSidelink message. | <-- | PC5 RRC: RRCReconfigurationSidelink | - | - |
| 4 | UE sends an RRCReconfigurationCompleteSidelink message. | --> | PC5 RRC: RRCReconfigurationCompleteSidelink | - | - |
| 5 | Trigger UE to close UE test loop mode E. (RECEIVE). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 6 | NR-SS-UE1 sends a SL SDAP Data PDU with SDAP header on SL-DRB 1 for PQFI =5 | <-- | SL SDAP PDU | - | - |
| 7 | NR-SS-UE1 requests the UE to report the counter of successful reception of packets. NOTE: Requesting the UE to report the counter of successful reception of packets may be performed by MMI or AT command (+CUSPCREQ). | - | - | - | - |
| 8 | Check: Does the reported counters of successful reception of SL SDAP SDU packets equal to 1? NOTE: The UE reporting the counter of successful reception of SL SDAP SDU packets may be performed by MMI or AT command (+CUSPCREQ). | - | - | 1 | P |
| 9 | Trigger UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 10 | Trigger UE to close UE test loop mode E. (RECEIVE). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 11 | NR-SS-UE1 sends a SL SDAP Data PDU without SDAP header on SL-DRB 2 for PQFI =2 | <-- | SL SDAP PDU | - | - |
| 12 | NR-SS-UE1 requests the UE to report the counter of successful reception of packets. NOTE: Requesting the UE to report the counter of successful reception of packets may be performed by MMI or AT command (+CUSPCREQ). | - | - | - | - |
| 13 | Check: Does the reported counters of successful reception of SL SDAP SDU packets equal to 1? NOTE: The UE reporting the counter of successful reception of SL SDAP SDU packets may be performed by MMI or AT command (+CUSPCREQ). | - | - | 2 | P |
| 14 | Trigger UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |

12.1.11.1.2.3.3	Specific message contents

Table 12.1.11.1.2.3.3-1: SL-PreconfigurationNR (Preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.10.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-PreconfigurationNR-r16 ::= SEQUENCE { |  |  |  |
| sidelinkPreconfigNR-r16 SEQUENCE { |  |  |  |
| sl-RadioBearerPreConfigList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16 { | 2 entries |  |  |
| SL-RadioBearerConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| slrb-Uu-ConfigIndex-r16 | 1 |  |  |
| sl-SDAP-Config-r16 SEQUENCE { |  |  |  |
| sl-SDAP-Header-r16 | present |  |  |
| sl-DefaultRB-r16 | true |  |  |
| sl-CastType-r16 | unicast |  |  |
| } |  |  |  |
| } |  |  |  |
| SL-RadioBearerConfig-r16[1] SEQUENCE { |  | entry 2 |  |
| slrb-Uu-ConfigIndex-r16 | 2 |  |  |
| sl-SDAP-Config-r16 SEQUENCE { |  |  |  |
| sl-SDAP-Header-r16 | absent |  |  |
| sl-DefaultRB-r16 | false |  |  |
| sl-MappedQoS-Flows-r16 CHOICE { |  |  |  |
| sl-MappedQoS-FlowsList-r16 SEQUENCE (SIZE (1..maxNrofSL-QFIs-r16)) OF SL-QoS-Profile-r16 { | 1 entry |  |  |
| SL-QoS-Profile-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-PQI-r16 CHOICE { |  |  |  |
| sl-StandardizedPQI-r16 | 23 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-CastType-r16 | groupcast |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.11.1.2.3.3-2: DIRECT LINK MODIFICATION REQUEST (Step 1, Table 12.1.11.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7D.1-9 with condition Rx |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| PC5 QoS flow description 1 |  |  |  |
| PQFI | '00 0010'B | PQFI 2 |  |
| Parameters list |  |  |  |
| Parameter 1 |  |  |  |
| Parameter identifier | '01'H | PQI |  |
| Length of parameter contents | 1 |  |  |
| Parameter contents | 23 |  |  |
| PC5 QoS flow description 2 |  |  |  |
| PQFI | '00 0101'B | PQFI 5 |  |
| Parameters list |  |  |  |
| Parameter 1 |  |  |  |
| Parameter identifier | '01'H | PQI |  |
| Length of parameter contents | 1 |  |  |
| Parameter contents | 21 |  |  |

Table 12.1.11.1.2.3.3-3: DIRECT LINK MODIFICATION ACCEPT (Step 2, Table 12.1.11.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7D.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| PC5 QoS flow description 1 |  |  |  |
| PQFI | '00 0010'B | PQFI 2 |  |
| Parameters list |  |  |  |
| Parameter 1 |  |  |  |
| Parameter identifier | '01'H | PQI |  |
| Length of parameter contents | 1 |  |  |
| Parameter contents | 23 |  |  |
| PC5 QoS flow description 2 |  |  |  |
| PQFI | '00 0101'B | PQFI 5 |  |
| Parameters list |  |  |  |
| Parameter 1 |  |  |  |
| Parameter identifier | '01'H | PQI |  |
| Length of parameter contents | 1 |  |  |
| Parameter contents | 21 |  |  |

Table 12.1.11.1.2.3.3-4: RRCReconfigurationSidelink (Step 3, Table 12.1.11.1.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1A-3 with condition SL_DRB and RX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |
| slrb-ConfigToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Config-r16 { | 1 entry |  |  |
| SLRB-Config-r16[1] SEQUENCE { |  | entry 1 |  |
| slrb-PC5-ConfigIndex-r16 | 1 |  |  |
| sl-SDAP-ConfigPC5-r16 SEQUENCE { |  |  |  |
| sl-MappedQoS-FlowsToAddList-r16 SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16 { |  |  |  |
| SL-PQFI-r16 [1] | 5 |  |  |
| sl-SDAP-Header-r16 | present |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-MAC-LogicalChannelConfigPC5-r16 SEQUENCE { |  |  |  |
| sl-LogicalChannelIdentity-r16 | LogicalChannelIdentity with condition DRB1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.11.1.2.3.3-5: RRCReconfigurationCompleteSidelink (Step 4, Table 12.1.11.1.2.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1A-4 with condition TX