3GPP TSG-RAN5 Meeting #107	R5-252151

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4957 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction to SL PDCP TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Tejet, SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The heading level of 12.1.10.1.1,12.1.10.1.2 and 12.1.10.1.3 is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The heading style of 12.1.10.1.1,12.1.10.1.2 and 12.1.10.1.3 has been updated to heading 5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The errors will exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

12.1.10.1	PC5-only operation / PDCP / data transfer

12.1.10.1.1	PC5-only operation / PDCP / data transfer / PDCP SN numbering / 12 bits SN

12.1.10.1.1.1	Test Purpose (TP)

(1)

with { UE has established unicast link with peer UE and has established a unicast SL DRB with sl-PDCP-SN-Size = len12bits }

ensure that {

when { UE is configured by upper layer to perform NR sidelink transmission }

then { UE increments SN with 1 for each transmitted PDU for PDCP_SN from first SN transmitted to maximum PDCP_SN (4095) and to first SN transmitted again }

}

(2)

with { UE has established unicast link with peer UE and has established a unicast SL DRB with sl-PDCP-SN-Size = len12bits }

ensure that {

when { UE is configured by upper layer to perform NR sidelink transmission, and after incrementation TX_NEXT is larger than the maximum PDCP_SN (4095) }

then { UE sets PDCP_SN to 0 in the next transmitted PDCP PDU }

}

12.1.10.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, subclause 5.2.1, 5.2.3, and 5.7.4. Unless otherwise stated these are Rel-16 requirements.

[TS 38.323, subclause 5.2.3]

For sidelink transmission of the SLRB, the UE shall follow the procedures in clause 5.2.1 with following modification:

-	perform the header compression using ROHC as specified in clause 5.7.4, if SDU Type is IP;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[sl-PDCP-SN-Size]..

[TS 38.323, subclause 5.2.1]

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	start the discardTimer associated with this PDCP SDU (if configured).

For a PDCP SDU received from upper layers, the transmitting PDCP entity shall:

-	associate the COUNT value corresponding to TX_NEXT to this PDCP SDU;

NOTE 1:	Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	perform integrity protection, and ciphering using the TX_NEXT as specified in the clause 5.9 and 5.8, respectively;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[pdcp-SN-SizeUL];

-	increment TX_NEXT by one;

-	submit the resulting PDCP Data PDU to lower layer as specified below.

[TS 38.323, subclause 5.7.4]

If ROHC is configured, the ROHC protocol generates two types of output packets:

-	ROHC compressed packets, each associated with one PDCP SDU;

-	standalone packets not associated with a PDCP SDU, i.e. interspersed ROHC feedback.

A ROHC compressed packet is associated with the same PDCP SN and COUNT value as the related PDCP SDU. The header compression is not applicable to the SDAP header and the SDAP Control PDU if included in the PDCP SDU.

12.1.10.1.1.3	Test description

12.1.10.1.1.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 is as defined in TS 38.508-1 [4], configured for and operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 is configured following the test model in 38.523-3 [3] cl. 5.3.2.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3 except for those listed in Table 12.1.10.1.1.3.1-1.

- The UE uses GNSS as the synchronization reference source.

Table 12.1.10.1.1.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°119 (V2X) supported |  |
| EFVST |  | As per TS 38.508-1 [4] clause 4.8.3.3.3 |  |
| EFV2XP_PC5 |  | As per TS 38.508-1[4] clause 4.8.3.3.3 SL-PreconfigurationNR included in V2X data policy over PC5 is defined in Table 12.1.10.1.1.3.3-1 |  |

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

12.1.10.1.1.3.2	Test procedure sequence

Table 12.1.10.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger UE to close UE test loop mode E. (TRANSMIT). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). NOTE: The UE sends multiple packets continuously. | - | - | - | - |
| 2 | Void | - | - | - | - |
| 3 | Check: Does UE transmit 2[sl-PDCP-SN-Size]+1 PDCP PDUs starting with first SN once the test loop E is closed until PDCP_SN= the first SN again ? (Note 6) | --> | PDCP PDUs | 1 | P |
| 4 | Void | - | - | - | - |
| 5 | Check: Does UE transmit a PDCP PDU with SN=0 after PDCP PDU with PDCP_SN= 2[sl-PDCP-SN-Size]-1 ? | --> | PDCP PDU | 2 | P |
| 6 | Trigger UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| Note 1: void Note 2: void Note 3: void Note 4: void Note 5: void Note 6: The verdict shall be provided every 256 PDUs received for 12-bit SN and 4096 PDUs received for 18-bit SN respectively |  |  |  |  |  |

12.1.10.1.1.3.3	Specific message contents

Table 12.1.10.1.1.3.3-1: SL-PreconfigurationNR (Pre-configuration, UE under test)

| Derivation Path: TS 38.508-1 [4], Table 4.10.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-PreconfigurationNR-r16 ::= SEQUENCE { |  |  |  |
| sidelinkPreconfigNR-r16 SEQUENCE { |  |  |  |
| sl-RadioBearerPreConfigList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16 { |  |  |  |
| SL-RadioBearerConfig-r16 ::= SEQUENCE { |  |  |  |
| SL-PDCP-Config-r16 ::= SEQUENCE { |  |  |  |
| sl-PDCP-SN-Size-r16 | len12bits |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

12.1.10.1.2	PC5-only operation / PDCP / data transfer / PDCP SN numbering / 18 bits SN

12.1.10.1.2.1	Test Purpose (TP)

(1)

with { UE has established unicast link with peer UE and has established a unicast SL DRB with sl-PDCP-SN-Size = len18bits }

ensure that {

when { UE is configured by upper layer to perform NR sidelink transmission }

then { UE increments SN with 1 for each transmitted PDU for PDCP_SN from first SN transmitted to maximum PDCP_SN (262143) and to first SN transmitted again }

}

(2)

with { UE has established unicast link with peer UE and has established a unicast SL DRB with sl-PDCP-SN-Size = len18bits }

ensure that {

when { UE is configured by upper layer to perform NR sidelink transmission, and after incrementation TX_NEXT is larger than the maximum PDCP_SN (262143) }

then { UE sets PDCP_SN to 0 in the next transmitted PDCP PDU }

}

12.1.10.1.2.2	Conformance requirements

Same as conformance requirements in clause 12.1.10.1.1.2.

12.1.10.1.2.3	Test description

12.1.10.1.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 12.1.10.1.1.3.1 exception of PDCP parameters according to Table 12.1.10.1.2.3.1-1.

Table 12.1.10.1.2.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°119 (V2X) supported |  |
| EFVST |  | As per TS 38.508-1 [4] clause 4.8.3.3.3 |  |
| EFV2XP_PC5 |  | As per TS 38.508-1[4] clause 4.8.3.3.3 SL-PreconfigurationNR included in V2X data policy over PC5 is defined in Table 12.1.10.1.2.3.3-1 |  |

12.1.10.1.2.3.2	Test procedure sequence

Same as test procedure in clause 12.1.10.1.1.3.2.

12.1.10.1.2.3.3	Specific message contents

Table 12.1.10.1.2.3.3-1: SL-PreconfigurationNR (Pre-configuration, UE under test)

| Derivation Path: TS 38.508-1 [4], Table 4.10.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-PreconfigurationNR-r16 ::= SEQUENCE { |  |  |  |
| sidelinkPreconfigNR-r16 SEQUENCE { |  |  |  |
| sl-RadioBearerPreConfigList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16 { |  |  |  |
| SL-RadioBearerConfig-r16 ::= SEQUENCE { |  |  |  |
| SL-PDCP-Config-r16 ::= SEQUENCE { |  |  |  |
| sl-PDCP-SN-Size-r16 | len18bits |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

12.1.10.1.3	PC5-only operation / PDCP / data transfer / reordering

12.1.10.1.3.1	Test Purpose (TP)

(1)

with { UE has established unicast link with peer UE and has established an unicast SL DRB }

ensure that {

when { A PDCP PDU is received from the lower layer and the COUNT value of its corresponding PDCP SDU is smaller than RX_DELIV }

then { UE discards the PDCP PDU and doesn't submit its corresponding PDCP SDU to upper layer }

}

(2)

with { UE has established unicast link with peer UE and has established an unicast SL DRB }

ensure that {

when { A PDCP PDU is received from the lower layer and the COUNT value of its corresponding PDCP SDU is no less than RX_DELIV and not equal to the COUNT of a PDCP PDU which has been received before }

then { UE stores the corresponding PDCP SDU in reception buffer }

}

(3)

with { UE has established unicast link with peer UE and has established an unicast SL DRB }

ensure that {

when { A PDCP PDU is received from the lower layer and the COUNT value of its corresponding PDCP SDU equals to the COUNT of a PDCP PDU which has been received before}

then { UE discards the PDCP PDU and doesn't submit more than one PDCP SDUs with same COUNT to upper layer }

}

(4)

with { UE has established unicast link with peer UE and has established an unicast SL DRB }

ensure that {

when { A PDCP PDU is received from the lower layer and the COUNT value of its corresponding PDCP SDU equals to RX_DELIV }

then { UE submits PDCP SDU with COUNT = RX_DELIV and all PDCP SDUs with consecutive COUNT values from COUNT = RX_DELIV }

}

(5)

with { UE has established unicast link with peer UE and has established an unicast SL DRB. t-Reordering timer is running }

ensure that {

when { t-Reordering timer expires }

then { UE stops t-Reordering timer, delivers stored PDCP SDUs with COUNT less than RX_REORD and all PDCP SDUs with consecutive COUNT values from the PDCP SDU COUNT = RX_REORD }

}

12.1.10.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, subclause 5.2.2 and 5.2.4. Unless otherwise stated these are Rel-16 requirements.

[TS 38.323, subclause 5.2.4]

For sidelink reception of the SLRB, the UE shall follow the procedures in clause 5.2.2 with following modification:

-	perform the header decompression using ROHC as specified in clause 5.7.5, if SDU Type is IP.

[TS 38.323, subclause 5.2.2]

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU and consider it as not received;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers after performing header decompression using EHC.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

...

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value >= RX_REORD;

-	if RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

12.1.10.1.3.3	Test description

12.1.10.1.3.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 is as defined in TS 38.508-1 [4], configured for and operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3.

-	The UE uses GNSS as the synchronization reference source.

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22. The  PDCP parameter t-Reordering of UE under test is acquired according to px_NR_SidelinkPDCP_ReorderTimer.

12.1.10.1.3.3.2	Test procedure sequence

Table 12.1.10.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger UE to close UE test loop mode E. (RECEIVE). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 2 | The NR-SS-UE1 sends PDCP Data PDU #0 on SL DRB with PDCP SN = 0. | <-- | PDCP Data PDU #0 | - | - |
| 3 | The NR-SS-UE1 sends PDCP Data PDU #6 on SL DRB with PDCP SN = 0. | <-- | PDCP Data PDU #6 | - | - |
| 4 | NR-SS-UE1 requests the UE to report the counter of successful reception of packets. (Note 1) | - | - | - | - |
| 5 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to 1? (Note 2) | - | - | 3 | P |
| 6 | The NR-SS-UE1 sends PDCP Data PDU #2 on SL DRB with PDCP SN = 2. The UE starts t-Reordering timer. | <-- | PDCP Data PDU #2 | - | - |
| 7 | NR-SS-UE1 requests the UE to report the counter of successful reception of packets.(t-Reordering is running) (Note 1) | - | - | - | - |
| 8 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to 1? (Note 2) | - | -- | 2 | P |
| 9 | Wait until t-Reordering expires. | - | - | - | - |
| 10 | NR-SS-UE1 requests the UE to report the counter of successful reception of packets. (Note 1) | - | - | - | - |
| 11 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to 2? (Note 2) | - | - | 2,5 | P |
| 12 | The NR-SS-UE1 sends PDCP Data PDU #1 on SL DRB with PDCP SN = 1. | <-- | PDCP Data PDU #1 | - | - |
| 13 | NR-SS-UE1 requests the UE to report the counter of successful reception of packets. (Note 1) | - | - | - | - |
| 14 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to 2? (Note 2) | - | - | 1 | P |
| 15 | The NR-SS-UE1 sends PDCP Data PDU #4 on SL DRB with PDCP SN = 4. The UE starts t-Reordering timer. | <-- | PDCP Data PDU #4 | - | - |
| 16 | The NR-SS-UE1 sends PDCP Data PDU #3 on SL DRB with PDCP SN = 3.(t-Reordering is running) | <-- | PDCP Data PDU #3 | - | - |
| 17 | NR-SS-UE1 requests the UE to report the counter of successful reception of packets. (Note 1) | - | - | - | - |
| 18 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to 4? (Note 2) | - | - | 4 | P |
| 19 | Trigger UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| Note 1: Requesting the UE to report the counter of successful reception of packets may be performed by MMI or AT command (+CUSPCREQ). Note 2: The UE reporting the counter of successful reception of SDAP SDU packets may be performed by MMI or AT command (+CUSPCREQ). |  |  |  |  |  |

12.1.10.1.3.3.3	Specific message contents

None.