3GPP TSG-RAN5 Meeting #107	R5-252577

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4908 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LTE Band 106: Initial Test Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_900MHz_US-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To implement band 106 with CBW = 3MHz, the test model needs to be enhanced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added CCE Start indices. Added default UL grant values. Added downlink resources specifications for DCI combination 1 and DCI combination 2. Updated and added XLS calculation sheet to cover 3MHz bandwidth. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to implement band 106. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1, 7.2, 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.1.1.1	FDD candidates selection

Table 7.1.1.1-1 gives the CCE resources utilized for m' and m'' for default values of common search space aggregation level =4, UE-specific search space aggregation L=2 resulting in 6 PDCCH candidates m=0..5 and channel Bandwidth of 5 MHz. This give Max_CCE =20 for FDD. The table also gives the corresponding CCE start indices of PDCCH candidates for m' and m''.

Table 7.1.1.1-1: CCE Start indices(m' & m'') to be used for various C-RNTIs (5 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 0 | 1 | 0 | 0 | 0 | 3 | 4 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 12 | 8 | 14 | 8 | 12 | 8 | 8 | 8 | 14 | 10 |
|  |  | m'' | 1 | 2 | 1 | 1 | 1 | 4 | 5 | 1 | 1 | 1 |
|  |  | CCE_St_Ind'' | 14 | 10 | 16 | 10 | 14 | 10 | 10 | 10 | 16 | 12 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | 0 | 2 | 0 | 0 | 4 | 4 | 1 | 0 | 0 |
|  |  | CCE_St_Ind' | 12 | 16 | 8 | 14 | 10 | 8 | 8 | 8 | 18 | 16 |
|  |  | m'' | 1 | 1 | 3 | 1 | 1 | 5 | 5 | 2 | 5 | 1 |
|  |  | CCE_St_Ind'' | 14 | 18 | 10 | 16 | 12 | 10 | 10 | 10 | 8 | 18 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 0 | 0 | 0 | 2 | 3 | 0 | 0 | 0 | 0 | 4 |
|  |  | CCE_St_Ind' | 16 | 10 | 14 | 8 | 8 | 10 | 14 | 8 | 18 | 8 |
|  |  | m'' | 1 | 1 | 1 | 3 | 4 | 1 | 1 | 1 | 5 | 5 |
|  |  | CCE_St_Ind'' | 18 | 12 | 16 | 10 | 10 | 12 | 16 | 10 | 8 | 10 |
| tsc_C_RNTI_Def4 | '1FF1'H 8177 | m' | 0 | 0 | 0 | 0 | 3 | 0 | 0 | 0 | 2 | 4 |
|  |  | CCE_St_Ind' | 12 | 12 | 18 | 16 | 8 | 18 | 18 | 18 | 8 | 8 |
|  |  | m'' | 1 | 1 | 5 | 1 | 4 | 5 | 5 | 5 | 3 | 5 |
|  |  | CCE_St_Ind'' | 14 | 14 | 8 | 18 | 10 | 8 | 8 | 8 | 10 | 10 |
| tsc_C_RNTI_Def5 | '04D2'H 1234 | m' | 0 | 2 | 0 | 4 | 0 | 2 | 3 | 0 | 1 | 0 |
|  |  | CCE_St_Ind' | 10 | 8 | 10 | 8 | 14 | 8 | 8 | 14 | 8 | 10 |
|  |  | m'' | 1 | 3 | 1 | 5 | 1 | 3 | 4 | 1 | 2 | 1 |
|  |  | CCE_St_Ind'' | 12 | 10 | 12 | 10 | 16 | 10 | 10 | 16 | 10 | 12 |
| tsc_C_RNTI_Def6 | '0929'H 2345 | m' | 4 | 0 | 4 | 0 | 0 | 1 | 3 | 3 | 4 | 2 |
|  |  | CCE_St_Ind' | 8 | 10 | 8 | 12 | 14 | 8 | 8 | 8 | 8 | 8 |
|  |  | m'' | 5 | 1 | 5 | 1 | 1 | 2 | 4 | 4 | 5 | 3 |
|  |  | CCE_St_Ind'' | 10 | 12 | 10 | 14 | 16 | 10 | 10 | 10 | 10 | 10 |
| tsc_C_RNTI_Def7 | '0D80'H 3456 | m' | 2 | 0 | 2 | 0 | 0 | 0 | 3 | 0 | 0 | 2 |
|  |  | CCE_St_Ind' | 8 | 16 | 8 | 18 | 14 | 14 | 8 | 16 | 14 | 8 |
|  |  | m'' | 3 | 1 | 3 | 5 | 1 | 1 | 4 | 1 | 1 | 3 |
|  |  | CCE_St_Ind'' | 10 | 18 | 10 | 8 | 16 | 16 | 10 | 18 | 16 | 10 |
| tsc_C_RNTI_Def8 | '11D7'H 4567 | m' | 0 | 0 | 0 | 2 | 0 | 0 | 3 | 2 | 0 | 2 |
|  |  | CCE_St_Ind' | 8 | 16 | 8 | 8 | 14 | 16 | 8 | 8 | 8 | 8 |
|  |  | m'' | 1 | 1 | 1 | 3 | 1 | 1 | 4 | 3 | 1 | 3 |
|  |  | CCE_St_Ind'' | 10 | 18 | 10 | 10 | 16 | 18 | 10 | 10 | 10 | 10 |
| tsc_C_RNTI_Def9 | '162E'H 5678 | m' | 0 | 3 | 0 | 0 | 0 | 2 | 0 | 0 | 3 | 2 |
|  |  | CCE_St_Ind' | 12 | 8 | 12 | 16 | 8 | 8 | 16 | 18 | 8 | 8 |
|  |  | m'' | 1 | 4 | 1 | 1 | 1 | 3 | 1 | 5 | 4 | 3 |
|  |  | CCE_St_Ind'' | 14 | 10 | 14 | 18 | 10 | 10 | 18 | 8 | 10 | 10 |
| tsc_C_RNTI_Def10 | '1A85'H 6789 | m' | 0 | 0 | 0 | 3 | 0 | 1 | 0 | 1 | 3 | 2 |
|  |  | CCE_St_Ind' | 16 | 8 | 16 | 8 | 8 | 8 | 16 | 8 | 8 | 8 |
|  |  | m'' | 1 | 1 | 1 | 4 | 1 | 2 | 1 | 2 | 4 | 3 |
|  |  | CCE_St_Ind'' | 18 | 10 | 18 | 10 | 10 | 10 | 18 | 10 | 10 | 10 |

Table 7.1.1.1-1a gives the CCE resources utilized for m' and m'' for default values of common search space aggregation level =4, UE-specific search space aggregation L=2 resulting in 6 PDCCH candidates m=0..5 and channel Bandwidth of 5 MHz and CFI=2, suitable for eMBMS test cases. This give Max_CCE =12 for FDD. The table also gives the corresponding CCE start indices of PDCCH candidates for m' and m''.

Table 7.1.1.1-1a: CCE Start indices(m' & m'') to be used for various C-RNTIs with CFI=2 (5 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 4 | 3 | 3 | 0 | 2 | 1 | 2 | 0 | 3 | 1 |
|  |  | CCE_St_Ind' | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  | m'' | 5 | 4 | 4 | 1 | 3 | 2 | 3 | 1 | 4 | 2 |
|  |  | CCE_St_Ind'' | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | 4 | 0 | 0 | 3 | 4 | 0 | 0 | 1 | 0 |
|  |  | CCE_St_Ind' | 8 | 8 | 8 | 10 | 8 | 8 | 8 | 10 | 8 | 8 |
|  |  | m'' | 1 | 5 | 1 | 5 | 4 | 5 | 1 | 5 | 2 | 1 |
|  |  | CCE_St_Ind'' | 10 | 10 | 10 | 8 | 10 | 10 | 10 | 8 | 10 | 10 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 2 | 0 | 0 | 4 | 0 | 0 | 0 | 2 | 0 | 2 |
|  |  | CCE_St_Ind' | 8 | 10 | 10 | 8 | 10 | 10 | 10 | 8 | 10 | 8 |
|  |  | m'' | 3 | 5 | 5 | 5 | 5 | 5 | 5 | 3 | 5 | 3 |
|  |  | CCE_St_Ind'' | 10 | 8 | 8 | 10 | 8 | 8 | 8 | 10 | 8 | 10 |

Tables 7.1.1.1-2, 7.1.1.1-3 and 7.1.1.1-4 give the CCE resources utilized for m' and m'' for default values of common search space aggregation level =4, UE-specific search space aggregation L=2 resulting in 6 PDCCH candidates m=0..5 and bandwidths of 10/15/20 MHz respectively. This gives Max_CCE =25(10 MHz)/37(15 MHz)/50(20 MHz) for FDD. The tables also give the corresponding CCE start indices of PDCCH candidates for m' and m''. These are in general to be applied in MAC Transport block size test cases defined in clause 7.1.7 of TS 36.523-1 [1].

Table 7.1.1.1-2: CCE Start indices (m' & m'') to be used for default C-RNTI (10 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 0 | 3 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 12 | 8 | 8 | 20 | 16 | 18 | 16 | 8 | 14 | 18 |
|  |  | m'' | 1 | 4 | 4 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | CCE_St_Ind'' | 14 | 10 | 10 | 22 | 18 | 20 | 18 | 10 | 16 | 20 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | 4 | 0 | 0 | 0 | 4 | 0 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 8 | 8 | 20 | 10 | 14 | 8 | 20 | 22 | 18 | 8 |
|  |  | m'' | 1 | 5 | 1 | 1 | 1 | 5 | 1 | 5 | 1 | 1 |
|  |  | CCE_St_Ind'' | 10 | 10 | 22 | 12 | 16 | 10 | 22 | 8 | 20 | 10 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 0 | 0 | 0 | 4 | 0 | 0 | 0 | 2 | 0 | 0 |
|  |  | CCE_St_Ind' | 16 | 10 | 10 | 8 | 22 | 22 | 22 | 8 | 10 | 16 |
|  |  | m'' | 1 | 1 | 1 | 5 | 5 | 5 | 5 | 3 | 1 | 1 |
|  |  | CCE_St_Ind'' | 18 | 12 | 12 | 10 | 8 | 8 | 8 | 10 | 12 | 18 |
| tsc_C_RNTI_Def4 | '1FF1'H 8177 | m' | 2 | 0 | 0 | 4 | 0 | 0 | 3 | 0 | 2 | 0 |
|  |  | CCE_St_Ind' | 8 | 20 | 14 | 8 | 10 | 18 | 8 | 22 | 8 | 12 |
|  |  | m'' | 3 | 1 | 1 | 5 | 1 | 1 | 4 | 5 | 3 | 1 |
|  |  | CCE_St_Ind'' | 10 | 22 | 16 | 10 | 12 | 20 | 10 | 8 | 10 | 14 |
| tsc_C_RNTI_Def5 | '04D2'H 1234 | m' | 3 | 0 | 0 | 0 | 0 | 2 | 3 | 3 | 1 | 0 |
|  |  | CCE_St_Ind' | 8 | 16 | 22 | 12 | 22 | 8 | 8 | 8 | 8 | 22 |
|  |  | m'' | 4 | 1 | 5 | 1 | 5 | 3 | 4 | 4 | 2 | 5 |
|  |  | CCE_St_Ind'' | 10 | 18 | 8 | 14 | 8 | 10 | 10 | 10 | 10 | 8 |
| tsc_C_RNTI_Def6 | '0929'H 2345 | m' | 0 | 0 | 2 | 2 | 0 | 1 | 0 | 0 | 0 | 2 |
|  |  | CCE_St_Ind' | 20 | 18 | 8 | 8 | 18 | 8 | 18 | 22 | 12 | 8 |
|  |  | m'' | 1 | 1 | 3 | 3 | 1 | 2 | 1 | 5 | 1 | 3 |
|  |  | CCE_St_Ind'' | 22 | 20 | 10 | 10 | 20 | 10 | 20 | 8 | 14 | 10 |
| tsc_C_RNTI_Def7 | '0D80'H 3456 | m' | 4 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 4 |
|  |  | CCE_St_Ind' | 8 | 20 | 20 | 8 | 14 | 22 | 10 | 8 | 18 | 8 |
|  |  | m'' | 5 | 1 | 1 | 2 | 1 | 5 | 1 | 1 | 1 | 5 |
|  |  | CCE_St_Ind'' | 10 | 22 | 22 | 10 | 16 | 8 | 12 | 10 | 20 | 10 |
| tsc_C_RNTI_Def8 | '11D7'H 4567 | m' | 2 | 0 | 0 | 0 | 0 | 4 | 3 | 2 | 4 | 0 |
|  |  | CCE_St_Ind' | 8 | 8 | 12 | 8 | 10 | 8 | 8 | 8 | 8 | 20 |
|  |  | m'' | 3 | 1 | 1 | 1 | 1 | 5 | 4 | 3 | 5 | 1 |
|  |  | CCE_St_Ind'' | 10 | 10 | 14 | 10 | 12 | 10 | 10 | 10 | 10 | 22 |
| tsc_C_RNTI_Def9 | '162E'H 5678 | m' | 0 | 0 | 2 | 4 | 0 | 0 | 2 | 0 | 1 | 0 |
|  |  | CCE_St_Ind' | 8 | 10 | 8 | 8 | 16 | 16 | 8 | 14 | 8 | 16 |
|  |  | m'' | 1 | 1 | 3 | 5 | 1 | 1 | 3 | 1 | 2 | 1 |
|  |  | CCE_St_Ind'' | 10 | 12 | 10 | 10 | 18 | 18 | 10 | 16 | 10 | 18 |
| tsc_C_RNTI_Def10 | '1A85'H 6789 | m' | 0 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 3 | 0 |
|  |  | CCE_St_Ind' | 12 | 12 | 20 | 8 | 12 | 18 | 20 | 10 | 8 | 12 |
|  |  | m'' | 1 | 1 | 1 | 4 | 1 | 1 | 1 | 1 | 4 | 1 |
|  |  | CCE_St_Ind'' | 14 | 14 | 22 | 10 | 14 | 20 | 22 | 12 | 10 | 14 |

Table 7.1.1.1-3: CCE Start indices (m' & m'') to be used for default C-RNTI (15 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 8 | 14 | 14 | 20 | 16 | 18 | 28 | 20 | 26 | 30 |
|  |  | m'' | 5 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | CCE_St_Ind'' | 10 | 16 | 16 | 22 | 18 | 20 | 30 | 22 | 28 | 32 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 32 | 12 | 20 | 34 | 14 | 8 | 8 | 10 | 30 | 32 |
|  |  | m'' | 1 | 1 | 1 | 5 | 1 | 5 | 1 | 1 | 1 | 1 |
|  |  | CCE_St_Ind'' | 34 | 14 | 22 | 8 | 16 | 10 | 10 | 12 | 32 | 34 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 0 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 16 | 22 | 22 | 8 | 10 | 22 | 34 | 28 | 34 | 28 |
|  |  | m'' | 1 | 1 | 1 | 5 | 1 | 1 | 5 | 1 | 5 | 1 |
|  |  | CCE_St_Ind'' | 18 | 24 | 24 | 10 | 12 | 24 | 8 | 30 | 8 | 30 |

Table 7.1.1.1-4: CCE Start indices (m' & m'') to be used for default C-RNTI (20 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 0 |
|  |  | CCE_St_Ind' | 8 | 36 | 34 | 38 | 42 | 22 | 10 | 8 | 8 | 20 |
|  |  | m'' | 4 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 3 | 1 |
|  |  | CCE_St_Ind'' | 10 | 38 | 36 | 40 | 44 | 24 | 12 | 10 | 10 | 22 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
|  |  | CCE_St_Ind' | 12 | 46 | 8 | 34 | 20 | 10 | 10 | 26 | 28 | 8 |
|  |  | m'' | 1 | 1 | 3 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
|  |  | CCE_St_Ind'' | 14 | 48 | 10 | 36 | 22 | 12 | 12 | 28 | 30 | 10 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 1 | 0 | 0 | 0 | 3 | 0 | 2 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 8 | 20 | 24 | 34 | 8 | 10 | 8 | 38 | 48 | 20 |
|  |  | m'' | 2 | 1 | 1 | 1 | 4 | 1 | 3 | 1 | 5 | 1 |
|  |  | CCE_St_Ind'' | 10 | 22 | 26 | 36 | 10 | 12 | 10 | 40 | 8 | 22 |

Table 7.1.1.1-5 gives the CCE resources utilized for m' and m'' for default values of common search space aggregation level =4, UE-specific search space aggregation L=2 resulting in 6 PDCCH candidates m=0..5 and channel Bandwidth of 3 MHz. This give Max_CCE =12 for FDD. The table also gives the corresponding CCE start indices of PDCCH candidates for m' and m''.

Table 7.1.1.1-5: CCE Start indices (m' & m'') to be used for various C-RNTIs (3 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 4 | 3 | 3 | 0 | 2 | 1 | 2 | 0 | 3 | 1 |
|  |  | CCE_St_Ind' | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  | m'' | 5 | 4 | 4 | 1 | 3 | 2 | 3 | 1 | 4 | 2 |
|  |  | CCE_St_Ind'' | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | 4 | 0 | 0 | 3 | 4 | 0 | 0 | 1 | 0 |
|  |  | CCE_St_Ind' | 8 | 8 | 8 | 10 | 8 | 8 | 8 | 10 | 8 | 8 |
|  |  | m'' | 1 | 5 | 1 | 5 | 4 | 5 | 1 | 5 | 2 | 1 |
|  |  | CCE_St_Ind'' | 10 | 10 | 10 | 8 | 10 | 10 | 10 | 8 | 10 | 10 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 2 | 0 | 0 | 4 | 0 | 0 | 0 | 2 | 0 | 2 |
|  |  | CCE_St_Ind' | 8 | 10 | 10 | 8 | 10 | 10 | 10 | 8 | 10 | 8 |
|  |  | m'' | 3 | 5 | 5 | 5 | 5 | 5 | 5 | 3 | 5 | 3 |
|  |  | CCE_St_Ind'' | 10 | 8 | 8 | 10 | 8 | 8 | 8 | 10 | 8 | 10 |
| tsc_C_RNTI_Def4 | '1FF1'H 8177 | m' | 2 | 0 | 3 | 4 | 0 | 1 | 3 | 0 | 2 | 4 |
|  |  | CCE_St_Ind' | 8 | 8 | 8 | 8 | 10 | 8 | 8 | 10 | 8 | 8 |
|  |  | m'' | 3 | 1 | 4 | 5 | 5 | 2 | 4 | 5 | 3 | 5 |
|  |  | CCE_St_Ind'' | 10 | 10 | 10 | 10 | 8 | 10 | 10 | 8 | 10 | 10 |
| tsc_C_RNTI_Def5 | '04D2'H 1234 | m' | 3 | 2 | 0 | 4 | 0 | 2 | 3 | 3 | 1 | 0 |
|  |  | CCE_St_Ind' | 8 | 8 | 10 | 8 | 10 | 8 | 8 | 8 | 8 | 10 |
|  |  | m'' | 4 | 3 | 5 | 5 | 5 | 3 | 4 | 4 | 2 | 5 |
|  |  | CCE_St_Ind'' | 10 | 10 | 8 | 10 | 8 | 10 | 10 | 10 | 10 | 8 |
| tsc_C_RNTI_Def6 | '0929'H 2345 | m' | 0 | 1 | 2 | 2 | 1 | 1 | 1 | 0 | 4 | 2 |
|  |  | CCE_St_Ind' | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 10 | 8 | 8 |
|  |  | m'' | 1 | 2 | 3 | 3 | 2 | 2 | 2 | 5 | 5 | 3 |
|  |  | CCE_St_Ind'' | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 8 | 10 | 10 |
| tsc_C_RNTI_Def7 | '0D80'H 3456 | m' | 4 | 0 | 0 | 1 | 3 | 0 | 0 | 0 | 1 | 4 |
|  |  | CCE_St_Ind' | 8 | 8 | 8 | 8 | 8 | 10 | 10 | 8 | 8 | 8 |
|  |  | m'' | 5 | 1 | 1 | 2 | 4 | 5 | 5 | 1 | 2 | 5 |
|  |  | CCE_St_Ind'' | 10 | 10 | 10 | 10 | 10 | 8 | 8 | 10 | 10 | 10 |
| tsc_C_RNTI_Def8 | '11D7'H 4567 | m' | 2 | 0 | 4 | 0 | 0 | 4 | 3 | 2 | 4 | 0 |
|  |  | CCE_St_Ind' | 8 | 8 | 8 | 8 | 10 | 8 | 8 | 8 | 8 | 8 |
|  |  | m'' | 3 | 1 | 5 | 1 | 5 | 5 | 4 | 3 | 5 | 1 |
|  |  | CCE_St_Ind'' | 10 | 10 | 10 | 10 | 8 | 10 | 10 | 10 | 10 | 10 |
| tsc_C_RNTI_Def9 | '162E'H 5678 | m' | 0 | 0 | 2 | 4 | 2 | 2 | 2 | 3 | 1 | 2 |
|  |  | CCE_St_Ind' | 8 | 10 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  | m'' | 1 | 5 | 3 | 5 | 3 | 3 | 3 | 4 | 2 | 3 |
|  |  | CCE_St_Ind'' | 10 | 8 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| tsc_C_RNTI_Def10 | '1A85'H 6789 | m' | 4 | 4 | 0 | 3 | 4 | 1 | 0 | 0 | 3 | 4 |
|  |  | CCE_St_Ind' | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 10 | 8 | 8 |
|  |  | m'' | 5 | 5 | 1 | 4 | 5 | 2 | 1 | 5 | 4 | 5 |
|  |  | CCE_St_Ind'' | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 8 | 10 | 10 |

<TEXT SKIPPED>

## 7.2	Uplink Grant

### 7.2.0	UL Sync and Grant Allocation Types

The Network/SS informs the UE if it is allowed to make Uplink Data transmission by transmitting 'DCI format 0' on PDCCH. The UE shall transmit (4 TTI later for FDD or variable for TDD) a Transport Block (TB) of exactly the same size as specified in DCI format 0. The UE has no control of its own on TB size, and has to merely follow the network, even if that means lots of MAC padding or resource starving.

The UE has the following means to communicate if it has UL data ready for transmission and subsequently the estimate of quantity of data to be transmitted.

-	RACH procedure: UE in idle mode, handed over to a new cell or connected mode but PUCCH is unsynchronized (sometimes referred to as PUCCH is not configured) will trigger RACH procedure on data ready for transmission in UL.

-	Scheduling Request: UE in connected mode, no grant configured, PUCCH is synchronized and has data ready for transmission in UL, will transmit a scheduling request on PUCCH.

-	Buffer Status Reports: UE in connected mode, PUCCH synchronized, has a configured grant for current TTI, but grant is not sufficient to transmit all the data will include MAC control element BSR in the UL MAC PDU.

RACH and SR indicate on data availability and BSR provides an estimate of data available for transmission.

CQI/PMI/RI feedback from the UE which indicates the channel conditions and recommended number of layers.

Hence to determine the exact need of the grant requirement of the UE a network/SS needs to act on all four of the above. This eventually complicates the SS implementation and hence the grant allocation procedure is simplified such that SS needs only to react on reception of SR and grant allocation configured from the TTCN.

The SS disables aperiodic CQI/PMI/RI feedback from the UE by setting the ‘CQI request field’ to 0 in DCI format 0/RAR grant.

When request for periodic CQI/PMI/RI feedback is requested due to TTCN configuration, the SS does not react on periodic CQI/PMI/RI feedback received and still allocates grants as configured from TTCN.

The SS, if configured for maintaining PUCCH synchronization at UE, shall periodically transmit automatically MAC PDUs containing the MAC control element 'Timing Advance'. The period as configured by the TTCN is set to 80 % of the 'Time Alignment Timer' default value (750 ms) configured at UE.

Additionally the SS can be configured to automatically transmit a 'configured' UL grant at every reception of a SR. This grant should be selected under the following restrictions:

-	All UE categories can handle this, i.e. (for UE Cat 1 and above TBS ≤ 5160 bits, for UE Cat 0/M1 TBS ≤ 1000 bits).

-	It is sufficiently large that most of UL signalling messages can be transmitted. In case the grant is not sufficient to fit the whole UL data, the UE will have to wait for the expiry of retxBSR-Timer and retransmit a SR. And hence the procedure is repeated.

The following 5 types of grant allocation configurations are possible. Grant allocation types 1 to 3 are applicable, when the UE is in connected state. Grant allocation type 4 is applicable when UE is establishing / re-establishing the RRC Connection, or during handover or in connected state but PUCCH is not synchronised. Grant allocation type 5 is applicable when the UE is in connected state and is used for lower category UEs when the UL grants greater than Max TBS supported by the UE is needed.

Grant Allocation Type 1:

-	SS is configured to maintain PUCCH Synch.

-	SS is configured to send an automatically 'configured Grant' (in terms of IMCS and NPRB) to the UE on every reception of a Scheduling Request, within 10 subframes.

-	defined in Table 7.2.0-1The default configured grant is, unless explicitly specified in test cases.

-	By default this type of grant allocation is applied. The majority of Idle mode, RRC and NAS test cases, the preambles of all tests and the postambles of those tests for which UE is still PUCCH synchronised at the end of test body. A few Layer 2 tests also use this type of grant.

Grant Allocation Type 2:

-	SS is configured to maintain PUCCH Synch.

-	SS is configured to periodically transmit a grant (IMCS and NPRB). Number of grants (1 or more) and period (in ms) is configured by TTCN.

-	The first grant transmitted is as specified in the explicit timing information. If timing information is "now" the SS selects the first suitable subframe for UL transmission.

-	The grant allocation period for TDD shall be assigned without conflict with the allowed UL subframes in the TDD subframe configurations. As example of allocation period, the TDD UL Grant allocation can be assigned as in multiples of 5 ms.

-	This type of grant allocation is applicable to the majority of RLC, PDCP and a few MAC test cases.

-	No additional grant is allocated on reception of any SRs.

Grant Allocation Type 3:

-	SS may or may not be configured to maintain PUCCH Synch.

-	SS is configured to transmit a one time grant (IMCS and NPRB) in the time requested by TTCN. The one time transmission is achieved by setting Number of grants=1 and period=Only once.

-	This type of grant allocation is suitable for MAC and DRB tests when UE is in UL Synchronised state.

Grant Allocation Type 4 (RACH configuration):

-	In addition to the 3 types of UL grant allocations, a fourth type of grant allocation during the RACH procedure is also possible, where the SS behaves as per the RACH procedure configured and allocates the configured grant during the RACH procedure.

-	This UL Grant type is used in the configuration for the preamble in many situations, basically in MAC test cases. This type of grant is further used when UE is establishing/re-establishing the RRC connection or during handover, or when the UE is not PUCCH synchronised.

Grant Allocation Type 5:

-	SS is configured to maintain PUCCH Synch.

-	SS is configured to periodically transmit a grant (IMCS and NPRB) on reception of SR.  Number of grants (1 or more) and period (in ms) are configured by TTCN.  The first grant is sent within 10 subframes of SR.  Reception of SR starts/restarts periodic grant transmission.

-	The default configured grant is defined in Table 7.2.0-1, unless explicitly specified in test cases.

-	This type of grant allocation is suitable when the UE category is lower than 2.  The majority of Idle mode, RRC and NAS test cases, the preambles of all tests and the postambles of those tests which UE is still PUCCH synchronised at the end of the test body.  A few Layer 2 tests also use this type of grant.

All the UL grant allocation types define grant allocation in terms of IMCS and NPRB to be used. The SS shall allocate RBs corresponding to PRB indices 0..(NPRB-1).

Table 7.2.0-1 specifies the default UL grant settings for grant allocation types 1 and 5 depending on the UE Category, whether the test case is applicable to CE mode A UEs, and whether the grant size is default or big.

Table 7.2.0-1: Default UL grant settings for grant allocation types 1 and 5

| UE Category | CE mode A test case | Grant size type | TBS (bits) {IMCS ; NPRB} | Grant allocation type | Total grant size (bits) |
| --- | --- | --- | --- | --- | --- |
| CatM1 | n/a | Default | 1000 {14 ; 4} | Type 1 | 1000 |
|  | n/a | Big | 1000 {14 ; 4} | Type 5 {20ms ; 10x} | 10000 |
| Cat0 | n/a | Default | 1000 {14 ; 4} | Type 1 | 1000 |
|  | n/a | Big | 1000 {14 ; 4} | Type 5 {20ms ; 10x} | 10000 |
| Cat1/1bis | yes | Default | 1000 {14 ; 4} | Type 1 | 1000 |
|  |  | Big | 1000 {14 ; 4} | Type 5 {20ms ; 10x} | 10000 |
|  | no | Default | 4008 {9 ; 25} | Type 1 | 4008 |
|  |  | Default 3MHz | 4008 {19 ; 11} | Type 1 | 4008 |
|  |  | Big | 4968 {12 ; 25} | Type 5 {20ms ; 2x} | 9936 |
|  |  | Big 3MHz | 4968 {18 ; 15} | Type 5 {20ms ; 2x} | 9936 |
| CatX with X≥2 | yes | Default | 1000 {14 ; 4} | Type 1 | 1000 |
|  |  | Big | 1000 {14 ; 4} | Type 5 {20ms ; 10x} | 10000 |
|  | no | Default | 4008 {9 ; 25} | Type 1 | 4008 |
|  |  | Default 3MHz | 4008 {19 ; 11} | Type 1 | 4008 |
|  |  | Big | 10680 {20 ; 25} | Type 1 | 10680 |
|  |  | Big 3MHz | 5352 {19 ; 15} | Type 1 | 10704 |
| NOTE: For CE mode B only test cases (e.g. 7.1.4.24c), other UL Grant settings are used. |  |  |  |  |  |

The type of test cases configured with big grant size is defined in Table 7.2.0-2:

Table 7.2.0-2: Test cases configured with big grant size

| UE Category | CE mode A test case | Condition to be fulfilled to use big grant in a test case |
| --- | --- | --- |
| CatM1/0 | n/a | if the UE supports IMS |
| CatX with X≥1 | yes | if the UE supports IMS |
|  | no | if the test case includes IMS signalling within the test body |

### 7.2.1	Exception test case list

This clause contains the exception test case list where the explicit UL grant allocation types other than UL grant types 1 and 5 are specified.

Table 7.2.1-1: Exception test case list with explicit UL grant types other than UL grant type 1 and 5

| Group | Test Case | UL Grant Type 2 | UL Grant Type 3 |
| --- | --- | --- | --- |
| RLC | 7.2.2.6 | X |  |
|  | 7.2.2.7 | X |  |
|  | 7.2.3.1 |  | X |
|  | 7.2.3.2 | X |  |
|  | 7.2.3.4 |  | X |
|  | 7.2.3.5 |  | X |
|  | 7.2.3.6 | X |  |
|  | 7.2.3.7 | X |  |
|  | 7.2.3.9 | X |  |
|  | 7.2.3.10 | X | X |
|  | 7.2.3.13 | X | X |
|  | 7.2.3.15 | X |  |
|  | 7.2.3.17 | X |  |
|  | 7.2.3.18 |  | X |
|  | 7.2.3.21 |  | X |
| MAC | 7.1.4.1 | X |  |
|  | 7.1.4.2 |  | X |
|  | 7.1.4.3 | X |  |
|  | 7.1.4.4 |  | X |
|  | 7.1.4.7 |  | X |
|  | 7.1.4.8 | X | X |
|  | 7.1.4.10 |  | X |
|  | 7.1.4.11 |  | X |
|  | 7.1.4.14 |  | X |
|  | 7.1.4.15 | X |  |
|  | 7.1.4.16 | X |  |
|  | 7.1.5.1 | X |  |
|  | 7.1.5.2 | X |  |
|  | 7.1.5.3 | X |  |
|  | 7.1.5.4 | X |  |
|  | 7.1.5.5 | X |  |
|  | 7.1.6.1 |  | X |
| PDCP | 7.3.5.4 |  | X |
| RRC | 8.2.1.5 | X |  |
| NAS | 9.2.1.1.24 |  | X |
| DRB | 12.1.1 |  | X |
|  | 12.1.2 |  | X |

## 7.3	Downlink Resource Allocation

The DL resource allocation is an SS emulation function. In order to ensure similar DL behaviours (within defined tolerances) on the different SS platforms in the timing stringent requirements, all downlink resource allocation schemes specified in the present clause shall be supported by the SS.

When the DL data is to be sent with a specific scheduling requirement, for instance, in a TTI in advance rather than “now”, the TTCN shall ensure that the data is scheduled at least 100 ms in advance. The 100 ms time in general covers all time delays, from the time DL data is sent by the TTCN at the EUTRA PTC to the completion of the transmission at the SS (TTCN delays, codec delays, adaptor delays and SS processing delays at various protocol Layers). In the case of more than one NAS PDU is piggy-backed in a scheduled RRC PDU, 20ms shall be added per additional NAS PDU: 100ms + ((NoOfNASPDUs – 1) * 20ms; this calculation is based on the assumption that there are not more than 7 piggy-backed NAS messages; this is valid for LTE.

NOTE:	The DL data means DL signalling and/or data in the present clause.

### 7.3.1	PDCCH DCI default formats

Two types of DCI combinations are identified as default formats for the signalling and protocol test.

DCI combination 1 uses:

-	DCI format 1A, resource allocation type 2 localised, for all DL scheduling types.

DCI combination 2 uses:

-	DCI format 1C, resource allocation type 2 distributed, for scheduling of PCCH/BCCH/RAR; and

-	DCI format 1 resource allocation type 0, for UE dedicated scheduling.

#### 7.3.1.1	Default DCI Format to be used in test cases configuring MIMO

Transmission mode 3 will be used in MIMO test cases configuring 2 Transmit antenna SS environment. As per 36.213 Table 7.1-5, in Transmission mode 3, UE is expected to decode only DCI formats 2A and 1A. Similarly for Transmission mode 4, UE is expected to decode only DCI formats 2 and 1A. Hence for all test cases configuring 2TX (2 antenna ports) at SS, DCI combination 1 is the default DCI combination to be applied. This allows DCI format 1A to be used as default DL scheduling scheme for test sequences not explicitly specified to use DCI formats 2A or 2(i.e. preamble, postamble etc.)

### 7.3.2	Radio parameters configured

The SS shall support DL QPSK, 16QAM and 64QAM modulation schemes. The configured radio parameters, including DCI format, resource allocation types, maximum allowed modulation scheme, first virtual / physical resource block to be used, maximum available resource blocks and redundancy version, are provided to the SS.

In the normal signalling test condition, DL RLC and HARQ retransmissions are rare. The redundancy version is provided to allow the occasional HARQ retransmissions. For those MAC, RLC tests contained in table 7.3.2-1 where timing requirements are involved the DL or UL HARQ retransmissions are not tolerable. Table 7.3.2-2 lists the RLC tests where timing requirements are involved, only one DL or UL HARQ retransmission per transport block is tolerable. Unless otherwise specified, if HARQ retransmissions occur in the test cases contained in table 7.3.2-1 or more than one HARQ retransmission occurs in the test cases of table 7.3.2-2, the test cases will be terminated with verdict inconclusive.

NOTE:	If the test is expecting the reporting of UL ACK/NACK for the DL MAC PDUs, or is configuring the PHICH in a certain mode, HARQ retransmissions other than those that are already specified in the prose will have an impact on the test sequence. If test cases perform scheduling of data transmissions and/or receptions, or the testing timers in the test cases are less than 900 ms (i.e. the tolerance for 90 ms), HARQ retransmissions will make it difficult to continue testing.

Table 7.3.2-1: TC list intolerable of HARQ retransmissions

| Test case | Comment |  |
| --- | --- | --- |
| MAC |  |  |
| 7.1.3.1, 7.1.3.2, 7.1.3.4, 7.1.3.5, 7.1.3.6, 7.1.3.9, 7.1.6.1, 7.1.6.2 |  | HARQ feedback reporting enabled or DL CRC errors introduced; DL HARQ unspecified (re)transmissions will result in ‘Fail' in test body, UL HARQ retransmissions are allowed; |
| 7.1.4.8 |  | Strict relationship between grant and UL data |
| 7.1.4.3 |  | Up to 104 PDUs to be sent in DL every TTI; |
| 7.1.4.2, 7.1.4.11, 7.1.4.12, 7.1.4.14, 7.1.5.4 |  | HARQ feedback transmission specified or PHICH errors introduced |
| 7.1.4.15, 7.1.4.16 |  | Periodic UL grants |
| RLC |  |  |
| 7.2.2.6, 7.2.2.7, 7.2.2.8, 7.2.2.10, 7.2.3.1, 7.2.3.2, 7.2.3.4, 7.2.3.5, 7.2.3.10, 7.2.3.13, 7.2.3.14, 7.2.3.15, 7.2.3.18 |  | Tolerating HARQ retransmissions is not feasible due to rigid timing and scheduling conditions. Testing timer < 900 ms |

Table 7.3.2-2: TC list intolerable of more than one HARQ retransmission per transport block

| Test case | Comment |  |
| --- | --- | --- |
| RLC |  |  |
| 7.2.3.6, 7.2.3.7, 7.2.3.8, 7.2.3.9, 7.2.3.17 |  | Tolerating more than one HARQ retransmission is not feasible due to rigid timing and scheduling conditions. Testing timer < 900 ms |

### 7.3.2.1	HARQ Retransmission when MIMO is configured

For test cases configuring MIMO, if in a TTI more than one transport blocks are scheduled (DCI format 2/ 2A/2B), the HARQ retransmission is handled independently for each transport block by SS. In case UE ACKs one Transport block and NACKs the other and there is no fresh data scheduled for transmission, SS only schedules the NACKed transport block for retransmission, using same Imcs as used in initial transmission, mapped to codeword 0. Acked Transport block (and hence codeword 1) is disabled by setting corresponding and rvidx = 1. Resource allocation (Nprb) used in retransmission is same as in initial transmission.

It is assumed that retransmission and fresh data scheduled in one TTI will not happen.

### 7.3.3	General DL scheduling scheme

The rules in the present clause, unless particularly specified, are applied to both default DCI combinations.

The bandwidth of 3/5/10/15/20 MHz makes 15/25/50/75/100 available physical resource blocks respectively. The 15/25/50/75/100 resource blocks are divided into three distinct sets. Exact set sizes and the elements contained in the individual sets depend upon the DCI combination to be applied.

-	The first set is reserved for BCCH mapped to DL-SCH (SI-RNTI).

-	The second set is reserved for PCCH mapped to DL-SCH (P-RNTI).

-	The third set is used for one of mutually exclusive transmissions of:

-	'Random Access Response' mapped to DL-SCH (RA-RNTI); or

-	UE-dedicated scheduling mapped to DL-SCH (C-RNTI/ SPS C-RNTI/ Temp C-RNTI).

For bandwidth of 3 MHz, resource blocks reserved for UE-dedicated scheduling mapped to DL-SCH (C-RNTI/ SPS C-RNTI/ Temp C-RNTI) and PBCH overlap. SS shall ensure that UE-dedicated scheduling is not transmitted in the same subframes as PBCH.

For each subframe for which data of one or more types is scheduled, the SS shall select a Transport Block Size (TBS), independently for each type of data scheduled, such that:

-	All the scheduled data is transmitted respecting the timing information. More details on the timing information can be found in clause 7.8.

-	Not more than MaxRbCnt resource blocks are used, for DCI format 1C, NPRB = MaxRbCnt.

-	Minimum MAC Padding is performed.

-	If all scheduled Data cannot be transmitted in the indicated subframe, for example due to TDD and half duplex configuration, it shall be transmitted in the next available subframe.

#### 7.3.3.1	Additional rules for BCCH scheduling scheme

This scheme is applicable for Data transmission on logical channel BCCH mapped to DL-SCH, PDCCH scrambled by SI-RNTI. For both DCI combinations, 4 physical resource blocks are reserved for BCCH transmission. The maximum modulation scheme is restricted to QPSK.

Following additional rules are applied for TBS selection:

-	The Max TBS, the maximum TBS allowed for the scheduling scheme, is restricted to 600. (nearest value achievable for ITBS = 9 and NPRB = 4, as per table 7.1.7.2.1-1 of TS 36.213 [30]).

-	If the scheduled Data cannot fit into a TBS smaller or equal to Max TBS, SS generates an error (it's a TTCN error). TTCN should gracefully exit the test case as a fatal error, assigning inconclusive verdict.

-	Rules in clause 7.3.3.1.1 for DCI combination 1 and in clause 7.3.3.1.2 for DCI combination 2 shall be applied.

##### 7.3.3.1.1	BCCH with DCI combination 1

TS 36.213 [30], table 7.1.7.2.1-1, rows with ITBS =0..26 and columns with NPRB =2 (corresponding to TPC LSB =0) and NPRB =3 (corresponding to TPC LSB =1), TBS <=Max TBS are applicable.

Distinct TBSs and all (TPC LSB, ITBS) combinations for each distinct TBS are listed in the sheet.

If a TBS can have two (TPC LSB, ITBS) combinations, the combination with TPC LSB =0 is selected.

RIV indicates 4 PRBs with index 0..3 allocated.

##### 7.3.3.1.2	BCCH with DCI combination 2

TS 36.213 [30], table 7.1.7.2.3-1, ITBS =0..17 with TBS ≤Max TBS are applicable.

RIV indicates 4 virtual RBs with index 0..3 allocated. These virtual RBs correspond to the physical RBs

-	with index 0, 4, 8, 12 in even slots and 8, 12, 0, 4 in odd slots for 3 MHz bandwidth;

-	with index 0, 6, 12, 18 in even slots and 12, 18, 0, 6 in odd slots for 5 MHz bandwidth;

-	with index 0, 12, 27, 39 in even slots and 27, 39, 0, 12 in odd slots for 10 MHz bandwidth;

-	with index 0, 16, 48, 64 in even slots and 48, 64, 0, 16 in odd slots for 15 MHz bandwidth;

-	with index 0, 24, 48, 72 in even slots and 48, 72, 0, 24 in odd slots for 20 MHz bandwidth.

#### 7.3.3.2	Additional rules for PCCH specific scheduling scheme

This scheme is applicable for Data transmission on logical channel PCCH mapped to DL-SCH, PDCCH scrambled by P-RNTI. For DCI combination 1, one physical resource block is reserved. For DCI combination 2, two physical resource blocks are reserved for 3 MHz and 5 MHz bandwidth, and four physical resource blocks are reserved for 10 MHz, 15 MHz or 20 MHz bandwidth. The maximum modulation scheme is restricted to QPSK.

Following additional rules are applied for TBS selection:

-	If the scheduled Data cannot fit into Max TBS, SS generates an error (it's a TTCN error). TTCN should gracefully exit the test case as a fatal error, assigning inconclusive verdict.

-	Rules in clause 7.3.3.2.1 for DCI combination 1 and clause 7.3.3.2.2 for DCI combination 2 shall be applied.

##### 7.3.3.2.1	PCCH with DCI combination 1

TS 36.213 [30], table 7.1.7.2.1-1, rows with ITBS =0..26 and columns with NPRB =2 (corresponding to TPC LSB =0) and NPRB =3 (corresponding to TPC LSB =1) TBS < =Max TBS are applicable.

The Max TBS is restricted to 120 (nearest value achievable for ITBS = 9 and NPRB =1, as per table 7.1.7.2.1-1 of TS 36.213 [30]).

Distinct TBSs and all (TPC LSB, ITBS) combinations for each distinct TBS are listed in the sheet.

If a TBS can have two (TPC LSB, ITBS) combinations, the combination with TPC LSB =0 is selected.

RIV indicates 1 PRBs with index 4 allocated for 5/10/15/20 MHz, index 2 allocated for 3 MHz.

##### 7.3.3.2.2	PCCH with DCI combination 2

TS 36.213 [30], table 7.1.7.2.3-1, ITBS =0..11 for 3 MHz and 5 MHz/ ITBS =0..17 for 10, 15 or 20 MHz with TBS ≤ Max TBS are applicable.

The Max TBS is restricted to

296 bits (nearest value achievable for ITBS = 9 and NPRB =2) for 3 MHz and 5 MHz bandwidth,

600 bits (nearest value achievable for ITBS = 9 and NPRB = 4) for 10, 15 or 20 MHz bandwidth.

RIV indicates either two virtual RBs with index 4 and 5 allocated, or four virtual RBs with index 4 to 7 allocated. These virtual RBs correspond to physical RBs:

with index 1 and 5 in even slots and 9 and 13 in odd slots for 3 MHz bandwidth;

with index 1 and 7 in even slots and 13 and 19 in odd slots for 5 MHz bandwidth;

with index 1, 13, 28, 40 in even slots and 28, 40, 1, 13in odd slots for 10 MHz bandwidth;

with index 1, 17, 49, 65 in even slots and 49, 65, 1, 17 in odd slots for 15 MHz bandwidth;

with index 1, 25, 49, 73 in even slots and 49, 73, 1, 25 in odd slots for 20 MHz bandwidth.

#### 7.3.3.3	Additional rules for RAR specific scheduling scheme

This scheme is applicable for transmission of Random Access Response mapped to DL-SCH, PDCCH scrambled by RA-RNTI. For both DCI combinations four physical resource blocks are reserved. The maximum modulation scheme is restricted to QPSK.

Following additional rules are applied for TBS selection:

-	The Max TBS is restricted to 600 bits (nearest value achievable for ITBS = 9 and NPRB =4, as per table 7.1.7.2.1-1 of TS 36.213 [30]).

-	If the scheduled Data cannot fit into Max TBS, SS generates an error (it's a TTCN error). TTCN should gracefully exit the test case as a fatal error, assigning inconclusive verdict.

-	Rules in clause 7.3.3.3.1 for DCI combination 1 and clause 7.3.3.3.2 for DCI combination 2 shall be applied.

##### 7.3.3.3.1	RAR with DCI combination 1

TS 36.213 [30], table 7.1.7.2.1-1, rows with ITBS = 0..26 and columns with NPRB = 2 (corresponding to TPC LSB = 0) and 3 (corresponding to TPC LSB = 1) TBS ≤ Max TBS are applicable

Distinct TBSs and all (TPC LSB, ITBS ) combinations for each distinct TBS are listed in the sheet.

If a TBS can have two (TPC LSB, ITBS ) combinations, the combination with TPC LSB = 0 is selected.

RIV indicates 4 PRBs with index 5..8 allocated for 5/10/15/20 MHz, index 11..14 allocated for 3 MHz.

##### 7.3.3.3.2	RAR with DCI combination 2

TS 36.213 [30], table 7.1.7.2.3-1, ITBS = 0..17 with TBS <= Max TBS are applicable.

RIV indicates 4 virtual RBs are allocated. These corresponds to physical RB

with index 9, 13, 2, 6 in even slots and 1, 5, 10, 14 in odd slots for 3 MHz bandwidth;

with index 13, 19, 2, 8 in even slots and 1, 7, 14, 20 in odd slots for 5 MHz bandwidth;

with index 2, 14, 29, 41 in even slots and 29, 41, 2, 14 in odd slots for 10 MHz bandwidth;

with index 2, 18, 50, 66 in even slots and 50, 66, 2, 18 in odd slots for 15 MHz bandwidth;

with index 2, 26, 50, 74 in even slots and 50, 74, 2, 26 in odd slots for 20 MHz bandwidth.

#### 7.3.3.4	Additional rules for UE-dedicated scheduling scheme in normal mode

The UE-dedicated DL scheduling can work in the normal mode or in the explicit mode. The two resource allocation schemes shall be reconfigurable from each other when the UE and SS are not sending and receiving data, for instance, at end of the test preamble and before the beginning of the test body.

The present clause is specified for the use of the normal mode. The explicit mode is referred to clause 7.3.3.6.

The scheme specified in the present clause is applicable for transmission of data dedicated to a UE in a DL subframe, mapped to DL-SCH, PDCCH scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI when spatial multiplexing MIMO mode is not configured. The maximum modulation scheme is restricted to 64QAM. For the DCI combination 1, 20 physical resource blocks (5 to 24), 11 physical resource blocks (4 to 14) for 3MHz and for the DCI combination 2, 17 physical resource blocks, 7 physical resource blocks for 3MHz are reserved. In the case when three intra frequency cells are applied to the test in the DCI combination 1, for the purpose of interference reduction, only 9 PRBs (16 to 24) are reserved.

Editor’s Note: DCI combination 1 resource allocation for 3MHz and intra frequency cells is FFS

In TDD normal TBS selection mode, no data is transmitted in DwPTS of the special subframe. For FDD, data can be transmitted in any subframe, except for half-duplex Type B UEs for which scheduling restrictions are specified in clause 7.26. In addition for BL/CE UEs, only valid subframes, according to fdd-DownlinkOrTddSubframeBitmapBR (when configured in the SS), shall be considered as suitable.

The following additional rules are applied for TBS selection:

-	Multiple ASPs can also carry same explicit timing information; indicating different ASP payloads, eventually needs to be transmitted in 1 TTI.

-	For UE Category ≠ 0 the Max TBS is restricted to 10296 bits (Max supported by UE category type 1).

-	For UE Category = 0 and M1, the Max TBS is restricted to 1000 bits. Hence in all TBS sheets only TB combinations with TBSize ≤ 1000 are used.

For 3 MHz bandwidth and the DCI combination 1 with 11 PRBs or DCI combination 2, the TBS 8248 is blocked as they result in coding rates higher than 0.93.

For 5 MHz bandwidth and the DCI combination 1 with 20 PRBs or DCI combination 2, the TBS 8248, 8760, and 9528 are blocked as they result in coding rates higher than 0.93.

For 5 MHz bandwidth and special DCI combination 1 with 9 PRBs, the TBS 2216, 5992 and 6712 are blocked as they result in coding rates higher than 0.93.

For 10 MHz, 15 MHz and 20 MHz bandwidths none of TBSs are blocked due to coding rates higher than 0.93.

TBS 280 is blocked for all bandwidths as it can be achieved by only Nprb =1 and Itbs=15, requiring 64 QAM modulation scheme. This TBS would normally be used for “RACH procedure MSG4+RRCconnectionSetup”.

The blocked TBS are considered to be not available for selection.

-	Data pending for transmission in a given sub-frame consists of (listed in transmission priority order):

-	MAC Control Elements that the SS needs to send.

-	AMD STATUS PDU(s) that the SS needs to send.

-	Data not sent in previous subframe(s).

-	Fresh Data scheduled for transmission in this subframe for all logical channels.

-	Distinct TBSs and all (NPRB, ITBS) combinations for each distinct TBS are listed in the sheet.

-	If a TBS size can be achieved with more than one combination of IMCS (ITBS) and NPRB:

-	Select combination with lowest delta between NPRB and IMCS.

-	If still more than one combination remain, select combination with highest NPRB.

-	Not more than one RLC Data PDU shall be placed in a MAC PDU per logical channel (i.e. minimize RLC segmentation).

-	In a subframe, in case there is data pending for transmission from more than one logical channel, for each type of data pending for transmission as defined above, priority shall be given to the logical channel with the lowest logical channel priority value. In case of more than one logical channel with the same logical channel priority value, these logical channels should be served equally. Data pending for transmission from more than one logical channel will rarely happen for the signalling and protocol test.

-	Data not transmitted within a subframe is scheduled as pending for transmission in the next available subframe according to the priorities given above. Pending data for transmission will rarely happen for the signalling and protocol test.

-	TBS selected in a context by various platforms shall be within an allowed deterministic tolerance of:

-	2 bytes for potential Timing Advance Command MAC Control Element (1 byte data + 1 byte MAC sub header).

-	4 bytes each for AMD STATUS PDU (2 bytes data + 2 bytes MAC subheader).

-	Therefore in the worst case the SS may add up to (2 + 4 x NAMRB ) bytes to the data scheduled for transmission in a certain subframe, where NAMRB is the number of AM radio bearers (SRB or DRB) actively sending DL data in the test, in any subframe.

-	For DCI combination 1 RIV is calculated based on physical resource blocks corresponding to NPRB of the selected TBS and (NPRB, ITBS) combination. The physical resource blocks that can be allocated are the first NPRB resources of index range

-	4..14 for 3 MHz bandwidth,

-	5..24 for 5 MHz bandwidth,

-	28..49 for 10 MHz bandwidth,

-	9..30 for 15 & 20 MHz bandwidth.

-	For DCI combination 2, RBG assignment is calculated based on physical resource blocks corresponding to NPRB of the selected TBS and (NPRB, ITBS) combination. The size of RBG is 2 for 3 MHz and 5 MHz, 3 for 10 MHz and 4 for 15 & 20 MHz. The available physical resource blocks for allocation are:

-	For 3 MHz bandwidth, RBG1(2,3), RBG3(6,7), RBG5(10,11) and RBG7(14). If NPRB is even, the first NPRB /2 available RBGs are allocated. If NPRB is odd, then first (NPRB -1)/2 RBGs and RBG7 are allocated.

-	For 5 MHz bandwidth, RBG1(2,3), RBG2(4,5), RBG4(8,9), RBG5(10,11), RBG7(14,15), RBG8(16,17), RBG10(20,21), RBG11(22,23) and RBG12(24). If NPRB is even, the first NPRB /2 available RBGs are allocated. If NPRB is odd, then first (NPRB -1)/2 RBGs and RBG 12 are allocated.

-	For 10 MHz bandwidth, RBG1(3,4,5), RBG2(6,7,8), RBG3(9,10,11), RBG5(15,16,17), RBG6(18,19,20), RBG10(30,31,32), RBG11(33,34,35), RBG12(36,37,38) and RBG16(48,49). If NPRB mod 3 is 0, the first NPRB /3 RBGs are allocated. If NPRB mod 3 is 2, then first (NPRB -2)/3 available RBGs and RBG 16 are allocated.

-	For 15 MHz bandwidth, RBG1(4,5,6,7), RBG2(8,9,10,11), RBG3(12,13,14,15), RBG5(20,21,22,23), RBG6(24,25,26,27), RBG7(28,29,30,31), RBG11(44,45,46,47), RBG13(52,53,54,55), RBG14(56,57,58,59), RBG15(60,61,62,63), RBG17(68,69,70,71), RBG18(72.73.74). If NPRB mod 4 is 0, the first NPRB /4 RBGs are allocated. If NPRB mod 4 is 3, then first (NPRB -3)/4 available RBGs and RBG 18 are allocated.

-	For 20 MHz bandwidth, RBG1(4,5,6,7), RBG2(8,9,10,11), RBG3(12,13,14,15), RBG4(16,17,18,19), RBG5(20,21,22,23), RBG7(28,29,30,31), RBG8(32,33,34,35), RBG9(36,37,38,39), RBG10(40,41,42,43), RBG14(56,57,58,59), RBG15(60,61,62,63), RBG16(64,65,66,67), RBG17(68,69,70,71), RBG19(76.77.78.79) and RBG20(80,81,82,83). The first NPRB /4 RBGs are allocated.

#### 7.3.3.5	DL Resource allocation bitmaps

##### 7.3.3.5.1	DCI combination 1

Table 7.3.3.5.1-1: Physical resource allocation bitmap
for DCI combination 1 (5 MHz) with 20 PRBs

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-2: Physical resource allocation bitmap
for DCI combination 1 (5 MHz) with 9 PRBs

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-3 (columns 0–34): Physical resource allocation bitmap
for DCI combination 1 (10 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9..22 | 23..27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  | Not Used | Used for PBCH and other common signals |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-3 (columns 35–49): Physical resource allocation bitmap
for DCI combination 1 (10 MHz)

| NPRB | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-3A (columns 0–20): Physical resource allocation bitmap
for DCI combination 1 (15 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-3B (columns 21-74): Physical resource allocation bitmap
for DCI combination 1 (15 MHz)

| NPRB | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31..33 | 34..41 | 42..74 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  | Not Used | Used for PBCH and other common signals | Not Used |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-4 (columns 0–20): Physical resource allocation bitmap
for DCI combination 1 (20 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-5 (columns 21-99): Physical resource allocation bitmap
for DCI combination 1 (20 MHz)

| NPRB | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31..46 | 47..52 | 53..99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  | Not Used | Used for PBCH and other common signals | Not Used |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-6: Physical resource allocation bitmap
for DCI combination 1 (3 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 7.3.3.5.2	DCI combination 2

Table 7.3.3.5.2-1: Physical resource allocation bitmap
for DCI combination 2 (5 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  |  |  | 1 |  |  |  |  |  | 2 |  |  |  |  |  | 3 |  |  |  |  |  |  |
| BCCH-Odd | 2 |  |  |  |  |  | 3 |  |  |  |  |  | 0 |  |  |  |  |  | 1 |  |  |  |  |  |  |
| PCCH-Even |  | 4 |  |  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  | 5 |  |  |  |  |  |
| RAR-Even |  |  | 8 |  |  |  |  |  | 9 |  |  |  |  | 6 |  |  |  |  |  | 7 |  |  |  |  |  |
| RAR-Odd |  | 6 |  |  |  |  |  | 7 |  |  |  |  |  |  | 8 |  |  |  |  |  | 9 |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.2-2 (columns 0-20): Physical resource allocation bitmap
for DCI combination 2 (10 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| BCCH-Odd | 2 |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |
| PCCH-Even |  | 4 |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |  |
| PCCH-Odd |  | 6 |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |
| RAR-Even |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |  |
| RAR-Odd |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  | 11 |  |  |  |  |  |  |
| UE-Specific | x | x |  |  |  |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |
| RBGs | 0 |  |  | 1 |  |  | 2 |  |  | 3 |  |  | 4 |  |  | 5 |  |  | 6 |  |  |

Table 7.3.3.5.2-2 (columns 21-41): Physical resource allocation bitmap
for DCI combination 2 (10 MHz)

| NPRB | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |
| BCCH-Odd |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| PCCH-Even |  |  |  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |  |  | 7 |  |
| PCCH-Odd |  |  |  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  | 5 |  |
| RAR-Even |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  | 11 |
| RAR-Odd |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  | 9 |
| UE-Specific |  | x | x | x | x | x | x | x |  |  |  |  |  |  |  |  |  |  | x | x |  |
| RBGs | 7 |  |  | 8 |  |  | 9 |  |  | 10 |  |  | 11 |  |  | 12 |  |  | 13 |  |  |

Table 7.3.3.5.2-2 (columns 42-49): Physical resource allocation bitmap
for DCI combination 2 (10 MHz)

| NPRB | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  | Not Used |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |
| RBG's | 14 |  |  | 15 |  |  | 16 |  |

Table 7.3.3.5.2-2a (columns 0-19): Physical resource allocation bitmap
for DCI combination 2 (15 MHz)

| nprb | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |
| BCCH-Odd | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |
| PCCH-Even |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |
| PCCH-Odd |  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |
| RAR-Even |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| RAR-Odd |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11 |  |
| UE-Specific | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x | x |  |  |
| RBG's | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |

Table 7.3.3.5.2-2a (columns 20-39): Physical resource allocation bitmap
for DCI combination 2 (15 MHz)

| nprb | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RBG's | 5 |  |  |  | 6 |  |  |  | 7 |  |  |  | 8 |  |  |  | 9 |  |  |  |

Table 7.3.3.5.2-2a (columns 40-59): Physical resource allocation bitmap
for DCI combination 2 (15 MHz)

| nprb | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |  |  |
| RBG's | 10 |  |  |  | 11 |  |  |  | 12 |  |  |  | 13 |  |  |  | 14 |  |  |  |

Table 7.3.3.5.2-2a (columns 60-74): Physical resource allocation bitmap
for DCI combination 2 (15 MHz)

| nprb | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  | 11 |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  | x | x |  |  |  |  |  |  |  |  |  |
| RBG's | 15 |  |  |  | 16 |  |  |  | 17 |  |  |  | 18 |  |  |

Table 7.3.3.5.2-3 (columns 0-19): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RBGs | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |

Table 7.3.3.5.2-3 (columns 20-39): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  | 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RBGs | 5 |  |  |  | 6 |  |  |  | 7 |  |  |  | 8 |  |  |  | 9 |  |  |  |

Table 7.3.3.5.2-3 (columns 40-59): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  | x | x | x |  |  |  |  |  |  |  |  |  |
| RBG's | 10 |  |  |  | 11 |  |  |  | 12 |  |  |  | 13 |  |  |  | 14 |  |  |  |

Table 7.3.3.5.2-3 (columns 60-79): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11 |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |
| RBGs | 15 |  |  |  | 16 |  |  |  | 17 |  |  |  | 18 |  |  |  | 19 |  |  |  |

Table 7.3.3.5.2-3 (columns 80-99): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Not Used |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RBGs | 20 |  |  |  | 21 |  |  |  | 22 |  |  |  | 23 |  |  |  | 24 |  |  |  |

Table 7.3.3.5.2-4: Physical resource allocation bitmap
for DCI combination 2 (3 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |
| BCCH-Odd | 2 |  |  |  | 3 |  |  |  | 0 |  |  |  | 1 |  |  |
| PCCH-Even |  | 4 |  |  |  | 5 |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  | 4 |  |  |  | 5 |  |
| RAR-Even |  |  | 8 |  |  |  | 9 |  |  | 6 |  |  |  | 7 |  |
| RAR-Odd |  | 6 |  |  |  | 7 |  |  |  |  | 8 |  |  |  | 9 |
| UE-Dedicated | x | x |  |  | x | x |  |  | x | x |  |  | x | x |  |
| RBGs | 0 |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |

NOTE:	Odd and even refer to slots.

#### 7.3.3.6	UE-dedicated scheduling scheme in explicit mode

This scheme applies to:

1.	spatial multiplexing MIMO configurations or

2.	transmit diversity MIMO configurations and non-MIMO configuration where the normal mode scheduling scheme is inappropriate.

SS is configured with an exact TBS (modulation and coding scheme, Imcs, and number of resource blocks, Nprb) to use. Other parameters, such as the HARQ process number and redundancy version to use for each transmission, are also configured by the TTCN. SS shall use TBS sheets with matching DCI format and Resource allocation Type. If the parameter 'FirstRbIndex' is configured different than specified in respective TBS sheet, the resource block bit maps in TBS sheet s are shifted by ' FirstRbIndex' and applied, with an exception for Resource allocation type 0 where only the full size 'Resource block groups' are shifted by ' FirstRbIndex'; if the last Resource block group is not full size, and is part of resource block bitmap, it is applied without any shift.

All data scheduled for a certain subframe shall be transmitted in the single indicated subframe, using configured parameters. The TTCN shall ensure that the configured parameters are consistent, in particular that the scheduled data size and the configured TBS match each other. Data scheduled by the prose, and hence also by the TTCN, provides possible space for the Timing Advance MAC control element and the RLC Status PDU. The SS shall include one of these if so triggered, else the bits reserved for these are filled by MAC padding.

Additionally, in the case of MIMO data scheduled for transmission in a given sub-frame, this consists of (listed in transmission priority order):

-	MAC Control Elements that the SS needs to send (if triggered).

-	AMD STATUS PDU(s) that the SS needs to send (if triggered).

-	Fresh data scheduled for transmission in this subframe for one or more logical channels, as per logical channel priority [lower value = higher priority]; if data is available for more than one logical channel with the same priority, then the logical channel corresponding to the DRB-ID with the lower value has the higher priority.

-	MAC padding.

The following additional rules need to be applied on data scheduled for transmission to be mapped on two transport blocks corresponding to two code words:

-	Higher priority data (as stated above) maps on to Transport Block 1 and lower priority data maps on Transport Block 2 (if Transport Block 1 gets full); and

-	Minimum MAC padding is performed in Transport Block 1; and

-	If data from one logical channel needs to be mapped on to two transport blocks, the PDCP PDUs with lower PDCP sequence numbers get mapped on to Transport Block 1.

By default no data is scheduled in TDD special subframes (i.e. subframes 1 and 6 for default TDD subframe configuration). For testing DL data reception in TDD special subframe, explicit mode TBS selection shall be used. The TTCN shall ensure that the configured parameters are consistent, in particular that the scheduled data size and the configured TBS match each other. As per TS36.213 [30] clause 7.1.7, for special subframe configuration 9 with normal cyclic prefix or special subframe configuration 7 with extended cyclic prefix, the actual Nprb used for TB size calculation will be a max( floor{ Nprb in DCI command *0.375},1). Tables 7.3.3.6-1/2 give the mapping for Nprb in DCI command and Nprb used for TB size determination.

Table 7.3.3.6-1: Nprb DCI to Nprb TBS mapping for 20 Mhz

| Nprb in DCI command (configured by TTCN) | 4 | 8 | 12 | 16 | 20 | 24 | 28 | 32 | 36 | 40 | 44 | 48 | 54 | 60 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nprb used for TBS determination | 1 | 3 | 4 | 6 | 7 | 9 | 10 | 12 | 13 | 15 | 16 | 18 | 20 | 22 |

Table 7.3.3.6-2: Nprb DCI to Nprb TBS mapping for 5 Mhz

| Nprb in DCI command (configured by TTCN) | 4 | 6 | 8 | 12 | 14 | 16 |
| --- | --- | --- | --- | --- | --- | --- |
| Nprb used for TBS determination | 1 | 2 | 3 | 4 | 5 | 6 |

##### 7.3.3.6.1	DL Scheduling in Transport Block Size Selection Test Cases

The MAC transport block size selection test cases defined in clause 7.1.7 of TS 36.523-1 [1], use bandwidths of 3/5/10/15/20MHz. For the preamble and post amble in these tests, the default scheduling rules defined in clauses 7.3.3.1 to 7.3.3.4 for 3/5/10/15/20 MHz and DCI combination 1A are applied respectively. During the test body, when the actual TB sizes with appropriate DCI and resource allocation formats needed are to be tested, the SS is configured in explicit mode for UE-dedicated scheduling.

#### 7.3.3.7	Resource allocation sheets

Attached with this Technical Specification, the DL resource allocation tables can be found, providing physical resource allocations for various transport block sizes, developed as per rules specified in clause 7.3.3, in Microsoft Excel format. Each individual sheet in the workbook represents various scheduling schemes as per table 7.3.3.7-1.

Table 7.3.3.7-1: DL resource allocation sheets

| S. No | Sheet Name | Description |
| --- | --- | --- |
| 1 | DCI-1A-PCCH | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by P-RNTI (3 MHz, 5 MHz, 10 MHz, 15 MHz and 20 MHz) |
| 2 | DCI-1A-BCCH | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by SI-RNTI (3 MHz, 5 MHz, 10 MHz, 15 MHz and 20 MHz) |
| 3 | DCI-1A-RAR | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by RA-RNTI (3 MHz, 5 MHz, 10 MHz, 15 MHz and 20 MHz) |
| 4 | DCI-1A-UE-Specific | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (5 MHz) |
| 5 | DCI-1A-3-IntraFreq-UE-Specific | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI and three Intra Freq cells are configured (5 MHz) |
| 6 | DCI-1A-UE-Specific-10MHz | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (10 MHz) |
| 6A | DCI-1A-UE-Specific-15MHz | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (15 MHz) |
| 7 | DCI-1A-UE-Specific-20MHz | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (20 MHz) Also in preamble/postamble phase of MAC TBS test cases with 15 MHz bandwidth configured |
| 7A | DCI-1A-UE-Dedicated-3MHz | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (3 MHz) |
| 8 | DCI-1C-PCCH | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (5 MHz) |
| 8A | DCI-1C-PCCH-3MHz | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (3 MHz) |
| 9 | DCI-1C-BCCH | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (5 MHz) |
| 9A | DCI-1C-BCCH-3MHz | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (3 MHz) |
| 10 | DCI-1C-RAR | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (5 MHz) |
| 10A | DCI-1C-RAR-3MHz | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (3 MHz) |
| 11 | DCI-1-UE-Specific | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (5 MHz) |
| 11A | DCI-1-UE-Dedicated-3MHz | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (5 MHz) |
| 12 | DCI-1C-PCCH-10MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (10 MHz) |
| 13 | DCI-1C-BCCH-10MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (10 MHz) |
| 14 | DCI-1C-RAR-10MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (10 MHz) |
| 15 | DCI-1-UE-Specific-10MHz-Gap1 | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (10 MHz) |
| 15A | DCI-1C-PCCH-15MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (15 MHz) |
| 15B | DCI-1C-BCCH-15MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (15 MHz) |
| 15C | DCI-1C-RAR-15MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (15 MHz) |
| 15D | DCI-1-UE-Specific-15MHz-Gap1 | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (15 MHz) |
| 16 | DCI-1C-PCCH-20MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (20 MHz) |
| 17 | DCI-1C-BCCH-20MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (20 MHz) |
| 18 | DCI-1C-RAR-20MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (20 MHz) |
| 19 | DCI-1-UE-Specific-20MHz-Gap1 | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (20 MHz) |
| 20 | DCI-1-RA0-ExplicitConfig | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI |
| 21 | DCI-1-RA1-ExplicitConfig | DL Resource scheduling for DCI format 1, Resource allocation 1 and PDCCH is scrambled by C-RNTI Editor’s note: 3 MHz is FFS |
| 22 | DCI1A-ExplicitConfig | DL Resource scheduling for DCI format 1A, Resource allocation 2(localised & distributed) and PDCCH is scrambled by C-RNTI |
| 23 | DCI-2A-RA0-ExplicitConfig | DL Resource scheduling for DCI format 2A, Resource allocation 0 and PDCCH is scrambled by C-RNTI |
| 24 | DCI-2A-RA1-ExplicitConfig | DL Resource scheduling for DCI format 2A, Resource allocation 1 and PDCCH is scrambled by C-RNTI |