3GPP TSG-RAN5 Meeting #107	R5-253160

# St Julian’s, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4981 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New test cases for Multi-carrier enhancements for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add new Further Multi-carrier enhancements for NR test case for CFRA msg1 repitition |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test cases 7.1.1.3.27.1 Multi-cell PUSCH scheduling with a single DCI / Scheduled cells indicatorfield based / Intra-band Contiguous CA 7.1.1.3.27.2 Multi-cell PUSCH scheduling with a single DCI / Scheduled cells indicator field based / Inter-band CA 7.1.1.3.27.3 Multi-cell PUSCH scheduling with a single DCI / Scheduled cells indicator field based / Intra-band non-Contiguous CA Are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for Further Multi-carrier enhancements for NR remains incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 7.1.1.3.27, 7.1.1.3.27.1, 7.1.1.3.27.2, 7.1.1.3.27.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.1.1.3.27	Multi-cell PUSCH scheduling with a single DCI / Scheduled cells indicator field based

###### 7.1.1.3.27.1	Multi-cell PUSCH scheduling with a single DCI / Scheduled cells indicator field based / Intra-band Contiguous CA

7.1.1.3.27.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state, SCell is configured, DCI format 0-3 is configured and ScheduledCellComboListDCI-0-3 is configured }

ensure that {

when { UE receives DCI format 0-3 on scheduling Cell, and Scheduled cell indicator field of DCI format 0-3 indicated that PCell is scheduled Cell }

then { UE transmits MAC PDU on PCell }

}

(2)

with { UE in RRC_CONNECTED state, SCell is configured, DCI format 0-3 is configured and ScheduledCellComboListDCI-0-3 is configured }

ensure that {

when { UE receives DCI format 0-3 on scheduling Cell, and Scheduled cell indicator field of DCI format 0-3 indicated that SCell is scheduled Cell }

then { UE transmits MAC PDU on SCell }

}

(3)

with { UE in  RRC_CONNECTED state, SCell is configured, DCI format 0-3 is configured and ScheduledCellComboListDCI-0-3 is configured }

ensure that {

when { UE receives DCI format 0-3 on scheduling Cell, and Scheduled cell indicator field of DCI format 0-3 indicated that PCell and SCell are scheduled Cell }

then { UE transmits MAC PDU on PCell and SCell }

}

7.1.1.3.27.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.212 clauses 7.3.1, 7.3.1.0 and 7.3.1.1.4. Unless otherwise stated these are Rel-18 requirements.

[TS 38.212, clause 7.3.1]

…

The size of each DCI format except for DCI format 0_3/1_3 is determined by the configuration of the corresponding active bandwidth part of the scheduled cell and shall be adjusted as described in clause 7.3.1.0 if necessary.

For a cell set configured by higher layer parameter mc-DCI-SetofCellsToAddModList, the size of DCI format 0_3/1_3 is determined as follows and shall be adjusted as described in Clause 7.3.1.0 if necessary:

-	If scheduledCellComboListDCI-0-3 for the cell set is configured, the size of DCI format 0_3 is determined by the configuration of the corresponding active bandwidth part(s) of the scheduled cells in the entry which results in the largest size among the entries in the higher layer parameter scheduledCellComboListDCI-0-3; Otherwise, the size of DCI format 0_3 is determined by the configuration of the corresponding active bandwidth part(s) of the cells configured by higher layer parameter scheduledCellListDCI-0-3 for the cell set.

-	If scheduledCellComboListDCI-1-3 for the cell set is configured, the size of DCI format 1_3 is determined by the configuration of the corresponding active bandwidth part(s) of the scheduled cells in the entry which results in the largest size among the entries in the higher layer parameter scheduledCellComboListDCI-1-3; Otherwise, the size of DCI format 1_3 is determined by the configuration of the corresponding active bandwidth part(s) of the cells configured by higher layer parameter scheduledCellListDCI-1-3 for the cell set.

[TS 38.212, clause 7.3.1.0]

Step 3:

-	If both of the following conditions are fulfilled the size alignment procedure is complete

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

Step 4:

-	Otherwise

Step 4A:

…

Step 4B:

…

Step 4C:

-	If the total number of different DCI sizes configured to monitor is more than 4 for the cell after applying the above steps, or if the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell after applying the above steps

-	If the number of information bits in the DCI format 0_1 prior to padding is less than the payload size of the DCI format 1_1 for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_1 until the payload size equals that of the DCI format 1_1.

-	If the number of information bits in the DCI format 1_1 prior to padding is less than the payload size of the DCI format 0_1 for scheduling the same serving cell, zeros shall be appended to the DCI format 1_1 until the payload size equals that of the DCI format 0_1.

[TS 38.212, clause 7.3.1.1.4]

DCI format 0_3 is used for the scheduling of one PUSCH in one cell, or multiple PUSCHs in multiple cells with one PUSCH per cell.

The following information is transmitted by means of the DCI format 0_3 with CRC scrambled by C-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Scheduled cell set indicator - log2Nset bits, where Nset is the number of cell sets which are configured by higher layer parameter MC-DCI-SetofCellsToAddModList to be respectively scheduled by DCI format 0_3/1_3 from the cell on which this format is carried by PDCCH. If present, this field is used to indicate the scheduled cell set according to Table 7.3.1.1.4-1; otherwise, the scheduled cell set is the cell set configured to be scheduled by DCI format 0_3/1_3 from the cell by higher layer parameter MC-DCI-SetofCellsToAddModList.

-	Scheduled cells indicator - number of bits determined by the following:

-	0 bit if the higher layer parameter scheduledCellComboListDCI-0-3 for the scheduled cell set is not configured;

-	otherwise log2IUL bits indicating the scheduled cells in the scheduled cell set according to Table 7.3.1.1.4-2, where IUL is the number of entries in the higher layer parameter scheduledCellComboListDCI-0-3. If only one entry is configured in the higher layer parameter scheduledCellComboListDCI-0-3, the scheduled cells are the cells configured by higher layer parameter scheduledCellComboListDCI-0-3.

-	Bandwidth part indicator - 0, 1 or 2 bits determined as log2nBWP, max, where

-	nBWP, max=nBWP,RRCmax+1 if nBWP,RRCmax≤3, nBWP,RRCmax is the maximum number of UL BWPs configured by higher layers, excluding the initial UL bandwidth part, across all the cells configured by higher layer parameter scheduledCellListDCI-0-3 in the scheduled cell set, in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise nBWP, max=nBWP,RRCmax, in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

The field is only applicable to a scheduled cell with the number of configured UL BWPs larger than 1, including the initial UL bandwidth part, and is applied to the applicable scheduled cells in the scheduled cell set independently. If a UE does not support active BWP change via DCI, the UE ignores this bit field. If this field indicates a code point that does not correspond to a configured BWP of a scheduled cell, the UE ignores this bit field for the scheduled cell, and operates on the active BWP of the scheduled cell.

-	Frequency domain resource assignment - number of bits determined by the following, where NRBUL, BWP is the size of the active UL bandwidth part:

-	block number 1, block number 2,…, block number NcellUL

If scheduledCellComboListDCI-0-3 for the scheduled cell set is configured with more than one entry, NcellUL is the number of scheduled cells indicated by Scheduled cells indicator field; if scheduledCellComboListDCI-0-3 for the scheduled cell set is configured with only one entry, NcellUL is the number of cells configured by higher layer parameter scheduledCellComboListDCI-0-3; otherwise, NcellUL is the number of cells configured by higher layer parameter scheduledCellListDCI-0-3 in the scheduled cell set. Each block corresponds to the frequency domain resource assignment for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the frequency domain resource assignment for the cell with the smallest serving cell index. Each block is defined by the following fields:

-	If higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is not configured

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 6.1.2.2.1 of [6, TS 38.214]

-	log2NRBG, K1NRBG, K1+1/2 bits if only resource allocation type 1 is configured, or max log2NRBG, K1NRBG, K1+1/2, NRBG+1 bits if resourceAllocationDCI-0-3 is configured as 'dynamicSwitch', where NRBG, K1=NRBUL, BWP+NUL, BWPstartmodK1/K1, NRBUL, BWP is the size of the active UL bandwidth part, NUL, BWPstart is defined as in clause 4.4.4.4 of [4, TS 38.211] and K1 is given by higher layer parameter resourceAllocationType1GranularityDCI-0-3. If the higher layer parameter resourceAllocationType1GranularityDCI-0-3 is not configured, K1 is equal to 1.

-	If resourceAllocationDCI-0-3 is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 6.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the log2NRBG, K1NRBG, K1+1/2 LSBs provide the resource allocation as follows:

-	For PUSCH hopping with resource allocation type 1:

-	NUL_hop MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where NUL_hop=1 if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and NUL_hop=2 if the higher layer parameter frequencyHoppingOffsetLists contains four offset values

-	log2NRBG, K1NRBG, K1+1/2-NUL_hop bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	log2NRBG, K1NRBG, K1+1/2 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocationDCI-0-3 is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bit width of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bit width of the "Frequency domain resource assignment" field of the indicated bandwidth part.

-	If the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured

-	5 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz. The 5 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation.

-	6 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz. The 6 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation.

The value of Y is determined by log2NRB-set,ULBWPNRB-set,ULBWP+12  where NRB-set,ULBWP  is the number of RB sets contained in the active UL BWP as defined in clause 7 of [6, TS38.214].

If the higher layer parameter scheduledCellComboListDCI-0-3 for the scheduled cell set is not configured, each block is also used to indicate whether the corresponding cell is scheduled or not as follows:

-	if all bits of a block are set to 0 for resource allocation type 0, or set to 1 for resource allocation type 1, or set to 0 or 1 for dynamic switch resource allocation type, or set to 0 for resource allocation type 2 with μ=1, or set to 1 for resource allocation type 2 with μ=0, the cell corresponding to the block is not scheduled;

-	otherwise, the cell corresponding to the block is scheduled.

…

Table 7.3.1.1.4-2: Scheduled cells indicator in DCI format 0_3

| Bit field mapped to index | Scheduled cells |
| --- | --- |
| 0 | The cells configured by the 1st entry in scheduledCellComboListDCI-0-3 |
| 1 | The cells configured by the 2nd entry in scheduledCellComboListDCI-0-3 |
| 2 | The cells configured by the 3rd entry in scheduledCellComboListDCI-0-3, if any |
| 3 | The cells configured by the 4th entry in scheduledCellComboListDCI-0-3, if any |
| 4 | The cells configured by the 5th entry in scheduledCellComboListDCI-0-3, if any |
| 5 | The cells configured by the 6th entry in scheduledCellComboListDCI-0-3, if any |
| 6 | The cells configured by the 7th entry in scheduledCellComboListDCI-0-3, if any |
| 7 | The cells configured by the 8th entry in scheduledCellComboListDCI-0-3, if any |
| 8 | The cells configured by the 9th entry in scheduledCellComboListDCI-0-3, if any |
| 9 | The cells configured by the 10th entry in scheduledCellComboListDCI-0-3, if any |
| 10 | The cells configured by the 11th entry in scheduledCellComboListDCI-0-3, if any |
| 11 | The cells configured by the 12th entry in scheduledCellComboListDCI-0-3, if any |
| 12 | The cells configured by the 13th entry in scheduledCellComboListDCI-0-3, if any |
| 13 | The cells configured by the 14th entry in scheduledCellComboListDCI-0-3, if any |
| 14 | The cells configured by the 15th entry in scheduledCellComboListDCI-0-3, if any |
| 15 | The cells configured by the 16th entry in scheduledCellComboListDCI-0-3, if any |

7.1.1.3.27.1.3	Test description

7.1.1.3.27.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that Test loop function (On) System information combination NR-4 and in addition NR Cell 3 is configured as NR Active Scell.

7.1.1.3.27.1.3.2	Test procedure sequence

Table 7.1.1.3.27.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to configure SCell (NR Cell 3) and DCI format 0-3 on NR SpCell (NR Cell 1). (Note 1) | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 2A | The SS indicates a new transmission on PDCCH of NR SpCell and transmits Activation MAC control element to activate NR SCell. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 3 | The SS indicates a new transmission on PDCCH of NR SpCell and transmits a MAC PDU (containing an RLC PDU). | <-- | MAC PDU | - | - |
| 4 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_3 (Scheduled cells indicator indicates that only SpCell is scheduled) | - | - |
| 5 | Check: Does the UE transmit a MAC PDU on SpCell containing the loop back PDU? | --> | MAC PDU | 1 | P |
| 6 | Void |  |  |  |  |
| 7 | The SS indicates a new transmission on PDCCH of NR SpCell and transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 5. | <-- | MAC PDU | - | - |
| 8 | The SS indicates a new transmission on PDCCH of NR SpCell and transmits a MAC PDU (containing an RLC PDU). | <-- | MAC PDU | - | - |
| 9 | The SS sends on NR SpCell an UL grant suitable for transmitting loop back PDU. | <-- | (UL Grant) DCI format 0_3 (Scheduled cells indicator indicates that only SCell is scheduled) | - | - |
| 10 | Check: Does the UE transmit a MAC PDU on SCell containing the loop back PDU? | --> | MAC PDU | 2 | P |
| 11 | Void |  |  |  |  |
| 12 | The SS indicates a new transmission on PDCCH of NR SpCell and transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 10. | <-- | MAC PDU | - | - |
| 13 | The SS indicates new transmissions on PDCCH of SpCell, and transmits MAC PDU on SpCell (containing two RLC PDUs). | <-- | MAC PDU | - | - |
| 14 | The SS sends an UL grant suitable for transmitting loop back PDU on SpCell.(Note 3) | <-- | (UL Grant) DCI format 0_3 (Scheduled cells indicator indicates that both SpCell and SCell are scheduled) | - | - |
| 15 | Check: Does the UE transmit MAC PDUs on both SpCell and SCell containing the loop back PDU?(Note 4) | --> | MAC PDU | 3 | P |
| 16 | Void |  |  |  |  |
| 17 | The SS indicates a new transmission on PDCCH of NR SpCell and transmits MAC PDU containing RLC status PDU acknowledging reception of RLC PDUs in step 15. | <-- | MAC PDU | - | - |
| Note 1: Scheduling cell is NR SpCell (NR Cell 1), and reference cell is also NR SpCell (NR Cell 1). Therefore, the single DCI (DCI format 0_3) is counted on the NR Cell 1, and DCI size alignment procedure specified in 38.212 clause 7.3.1.0 for NR Cell 1 is complete in step4C. Note 2: The NR RRCReconfigurationComplete message is scheduled by DCI format 0_0 to avoid DCI size misalignment for DCI format 0_1 (according to TS 38.212 [14] clause 7.3.1.0 step 4C) between UE and SS during the reconfiguration. Note 3: The UL grant for SpCell is suitable to transmit one RLC PDU in step 13, and the UL grant for SCell is suitable to transmit one RLC PDU in step 13. Note 4: UE transmits two MAC PDUs, one MAC PDU on SpCell and one MAC PDU on SCell. |  |  |  |  |  |

7.1.1.3.27.1.3.3	Specific message contents

Table 7.1.1.3.27.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.3.27.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.27.1.3.3-2: CellGroupConfig (Table 7.1.1.3.27.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 7.1.1.3.27.1.3.3-3 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex as per TS 38.508-1 [4] table 4.6.3-154 |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add |  |  |
| sCellConfigDedicated | ServingCellConfig | Table 7.1.1.3.27.1.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.27.1.3.3-3: ServingCellConfig (Table 7.1.1.3.27.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 with condition DCI 0_3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.3.27.1.3.3-4 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated | Table 7.1.1.3.27.1.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.27.1.3.3-4: BWP-DownlinkDedicated (Table 7.1.1.3.27.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config with condition DCI_0_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.27.1.3.3-5: BWP-UplinkDedicated (Table 7.1.1.3.27.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config with condition DCI_0_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.27.1.3.3-6: ServingCellConfig (Table 7.1.1.3.27.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 with condition No_UL and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.3.27.1.3.3-7A |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated | Table 7.1.1.3.27.1.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.27.1.3.3-7: BWP-UplinkDedicated (Table 7.1.1.3.27.1.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config with condition DCI_0_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.27.1.3.3-7A: BWP-DownlinkDedicated (Table 7.1.1.3.27.1.3.3-6)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config | Not present |  |  |
| } |  |  |  |

Table 7.1.1.3.27.1.3.3-8: Physical layer parameters for DCI format 0_3 (steps 4, 9 and 14, Table 7.1.1.3.27.1.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.1.4-1 with condition CellInd and UL_CELL2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Scheduled cells indicator | 1(SpCell) | ‘01’B | Step 4 |
|  | 2(SCell) | ‘10’B | Step 9 |
|  | 3(SCell, SpCell) | ‘11’B | Step 14 |

###### 7.1.1.3.27.2	Multi-cell PUSCH scheduling with a single DCI / Scheduled cells indicator field based / Inter-band CA

The scope and description of the present TC is the same as test case 7.1.1.3.27.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: NR Cell 10 replaces NR Cell 3

-	NR Cell 1 and NR Cell 10 have same SCS/carrier type.

###### 7.1.1.3.27.3	Multi-cell PUSCH scheduling with a single DCI / Scheduled cells indicator field based / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 7.1.1.3.27.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA.