3GPP TSG-RAN5 Meeting #107	R5-251935

# St Julian’s, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3673 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR_Mob_enh2: Initial Test Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Sequence of intra-NR inter-cell LTM (RACH less) is needed for test cases like 7.1.1.1.29, 7.1.1.3.28. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added ‘7.3.5.1a Sequence of intra-NR inter-cell LTM (RACH less)’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for Further NR Mobility enhancements remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.3.5.3.1A (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

ASP	Abstract Service Primitive

ATS	Abstract Test Suite

AWGN	Additive White Gaussian Noise

BWP	Bandwidth Part

CA	Carrier Aggregation

CBRA	Contention Based Random Access

CC	Component Carriers

CCE	Control Channel Element

CFRA	Contention Free Random Access

CG-SDT	Configured Grant-based SDT

CORESET	Control Resource Set

CSS	Common Search Space

DAI	Downlink Assignment Index

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink Shared Channel

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

EHC	Ethernet Header Compression

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

FDD	Frequency Division Duplex

FD-FDD	Full-Duplex FDD

FR1	Frequency Range 1

FR2	Frequency Range 2

GSO	Geosynchronous Orbit

HD-FDD	Half-Duplex FDD

HO	Handover

H-SFN	Hyper SFN

ICS	Implementation Conformance Statement

IUT	Implementation Under Test

IXIT	Implementation eXtra Information for Testing

LBT	Listen Before Talk

LSB	Least Significant Bit

LTM	L1/L2 Triggered Mobility

MBS	Multicast/Broadcast Service

MCCH	MBS Control Channel

MCG	Master Cell Group

MN	Master Node

MRB	MBS Radio Bearer

MSB	Most Significant Bit

MTCH	MBS Traffic Channel

MUSIM	Multi-Universal Subscriber Identity Module

NCD-SSB	Non Cell Defining SSB

NE-DC	NR-E-UTRA Dual Connectivity

NGSO	Non-Geosynchronous Orbit

NR	NR Radio Access

NR-DC	NR-NR Dual Connectivity

NTN	Non-Terrestrial Network

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PEI	Paging Early Indication

PEI-O	Paging Early Indication-Occasion

PF	Paging Frame

PH	Paging Hyperframe

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PSCell	Primary SCG Cell

PSS	Primary Synchronisation Signal

PTC	Parallel Test Component

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

RA	Random Access

RA-SDT	Random Access-based SDT

RACH	Random Access Channel

RAR	Random Access Response

RAT	Radio Access Technology

RMSI	Remaining Minimum SI

SCell	Secondary Cell

SCG	Secondary Cell Group

SDT	Small Data Transmission

SFN	System Frame Number

SN	Secondary Node

SRS	Sounding Reference Signal

SS	System Simulator

SSB	Synchronization Signal and PBCH block

SSS	Secondary Synchronisation Signal

TC	Test Case

TDD	Time Division Duplex

UDC	Uplink Data Compression

UL	Uplink

UL-SCH	Uplink Shared Channel

USS	UE-specific Search Space

UT	Upper Tester

VNG	Virtual Noise Generator

<SECTIONS SKIPPED>

##### 7.3.5.3.1	Sequence of intra-NR inter-cell handover

In general, the intra-NR inter-cell handover is done without activation time, i.e. the timing information for configuration of the SS and sending of the RRCReconfiguration is ‘Now'.

The sequence may be interrupted if other events need to be handled. E.g. when a Mobility procedure is performed in the target cell and there are procedures left to be executed on the source cell.

1.	Target Cell:		Configuration of DRBs

2.	Transfer of the PDCP Count for AM DRBs and SRBs (if applied) from source to target cell:

a)	Source Cell: Get PDCP COUNT.

b)	Target Cell: Set PDCP COUNT.

NOTE 1:	No further sending/receiving of DRB data before the HO is done.

NOTE 2:	For AM DRBs the PDCP count is maintained, for UM DRBs the PDCP count is reset. For SRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content.

3.	Target Cell:	Inform the SS about the HO and about the source cell id.

4.	Target Cell:	Configure RACH procedure either dedicated or C-RNTI based.

NOTE 3:	The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI.

5.	Target Cell:	Activate security.

6.	Void

7.	Source Cell:	Stop periodic TA.

NOTE 4:	Unless explicitly specified UL grant configuration keeps configured as per default at the source cell.

8.	Target Cell:	Configure UL grant configuration ("OnSR", periodic TA is not started).

9.	Source Cell:	Send RRCReconfiguration.

10.	Target Cell:	Receive RRCReconfigurationComplete.

11.	Target Cell:	Start periodic TA.

12.	Target Cell:	Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS REPORT PDU).

13.	Target Cell:	Re-configure RACH procedure as for initial access.

14.	Target Cell:	Configure measurement gap (if configured in the source cell or as provided in the RRCReconfiguration message).

15.	Source Cell:	Reset SRBs and release DRBs.

16.	Source Cell:	Release MeasGapConfig configuration.

##### 7.3.5.3.1A	Sequence of intra-NR inter-cell LTM (RACH-less)

In general, the intra-NR inter-cell LTM is done without activation time, i.e. the timing information for configuration of the SS and sending of the LTM Cell Switch Command MAC CE is ‘Now'.

The sequence may be interrupted if other events need to be handled. E.g. when a Mobility procedure is performed in the target cell and there are procedures left to be executed on the source cell.

NOTE 1:	Target cell DL synchronization (configured RRC measurements reporting of target cell) and UL synchronization (PDCCH order in source cell and PRACH preamble in target cell) happens before this procedure and is not included.

1.	Target Cell:		Configuration of DRBs

2.	Transfer of the PDCP Count for AM DRBs and SRBs (if applied) from source to target cell:

a)	Source Cell: Get PDCP COUNT.

b)	Target Cell: Set PDCP COUNT.

NOTE 2:	No further sending/receiving of DRB data before the HO is done.

NOTE 3:	For AM DRBs the PDCP count is maintained, for UM DRBs the PDCP count is reset. For SRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content sent to UE before LTM procedure.

3.	Target Cell:	Inform the SS about the HO and about the source cell id.

4.	Target Cell:	Activate security.

5.	Source Cell:	Stop periodic TA.

6. Target Cell:	Configure UL grant configuration ("OnSR", periodic TA is not started).

NOTE 4:	Unless explicitly specified UL grant configuration keeps configured as per default at the source cell.

7.	Source Cell:	Send LTM Cell Switch Command MAC CE.

8.  Target Cell:	SS autonomously sends UL Grant (C-RNTI) on SR reception

9.	Target Cell:	Receive RRCReconfigurationComplete.

10.	Target Cell:	Start periodic TA.

11.	Target Cell:	Inform the SS about completion of the Mobility (e.g. to trigger PDCP STATUS REPORT PDU).

12.	Target Cell:	Configure measurement gap (if configured in the source cell).

13.	Source Cell:	Reset SRBs and release DRBs.

14.	Source Cell:	Release MeasGapConfig configuration.

##### 7.3.5.3.2	Sequence of intra-NR intra-cell handover

For intra-NR intra-cell handover dedicated timing information is used: the sequence starts at time T with sending of the RRCReconfiguration. T is set to 300 ms in advance of the handover.

1.	Before T:	Get PDCP count for AM DRBs and SRBs (if applied).

2.	At T:	Send RRCReconfiguration.

3.	At T + 5ms:	Release SRBs and DRBs.

4.	At T + 5ms:	Configure RACH procedure either dedicated or C-RNTI based.

NOTE 1:	Since the RACH procedure may require a new C-RNTI to be used it cannot be configured before sending out the RRCReconfiguration.

NOTE 2:	The FollowOnFlag is set to true in the ASPs reconfiguring the RadioBearerList (step 3) and C-RNTI (step 4).

5.	At T + 5ms:	(Re-)configure measurement gap if provided in the RRCReconfiguration message.

6.	At T + 10ms:	(Re-) configure SRBs and DRBs.

7.	At T + 10ms:	Set PDCP COUNT for AM DRBs and SRBs (if applied).

NOTE 3:	For AM DRBs the PDCP count is maintained while for UM DRBs the PDCP count is reset. For SRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content.

8.	At T + 10ms:	Re-establish security, disable TA transmission.

9.	(after step 7)	Receive RRCReconfigurationComplete.

10.	(after step 8)	Re-configure RACH procedure as for initial access, enable TA transmissions.

NOTE 4:	The FollowOnFlag is set to true in the ASP reconfiguring RACH.