3GPP TSG-RAN5 Meeting #107	R5-252293

# Malta, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-1 | CR | 0011 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of details for Functional Alias for MCVideo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, FirstNet, Element, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Configuration details for usage of Functional Alias is missing for MCVideo. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configuration details for usage of Functional Alias was added for MCVideo. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functional Alias can not be tested for MCVideo. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.2, 5,5,3,5, 5.5.8.7 and 5.5.8.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.3.2	MCS Info Lists

##### 5.5.3.2.1	MCS Info Lists from the UE

##### -	MCPTT

Table 5.5.3.2.1-1: MCPTT-Info from the UE

| Derivation Path: TS 24.379 [9] clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-access-token | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcptt-access-token> with mcpttString set to access token as assigned to the UE in the Token Response | The access token is opaque to the MCPTT client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG, GROUPCONFIG |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | GROUP-CALL AND INVITE_REFER |
|  | "private" |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | “chat” |  |  | CHAT-GROUP-CALL AND INVITE_REFER |
|  | "first-to-answer" |  |  | FIRST-TO-ANSWER AND INVITE_REFER |
| mcptt-request-uri | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_A_ID | The URI of the group |  | (GROUP-CALL OR CHAT-GROUP-CALL) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_ID_User_B | The URI of the invited MCPTT Client |  | PRIVATE-CALL AND INVITE_REFER |
|  | encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_ID_User_A |  |  | POC-SETTINGS-EVENT |
| mcptt-calling-user-id | not present or encrypted (NOTE 2) <mcptt-calling-user-id> with mcpttURI set to px_MCPTT_ID_User_A |  |  |  |
|  | not present |  |  | CONFIG, GROUPCONFIG, POC-SETTINGS-EVENT |
| mcptt-called-party-id | not present |  |  |  |
|  | not present or encrypted (NOTE 2) <mcptt-called-party-id> with mcpttURI set to px_MCPTT_ID_User_A |  |  | INVITE-RSP |
| mcptt-calling-group-id | not present |  |  |  |
| required | not present |  |  |  |
| emergency-ind | not present or encrypted (NOTE 2) <emergency-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) <emergency-ind> with mcpttBoolean set to "true" |  |  | EMERGENCY-CALL AND INVITE_REFER |
| alert-ind | not present or encrypted (NOTE 2) <alert-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) <alert-ind> with mcpttBoolean set to pc_MCX_EmergencyIndWithAlertInd |  |  | EMERGENCY-CALL AND INVITE_REFER |
| imminentperil-ind | not present or encrypted (NOTE 2) <imminentperil-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) < imminentperil -ind> with mcpttBoolean set to "true" |  |  | IMMPERIL-CALL AND INVITE_REFER |
| broadcast-ind | not present or “false” |  |  |  |
|  | "true" |  |  | BROADCAST-CALL |
| mc-org | not present |  |  |  |
| floor-state | not present |  |  |  |
| associated-group-id | not present |  |  |  |
|  | px_MCPTT_Group_A_ID if mcptt-request-uri contains a temporary group identity; otherwise, not present | if the <mcptt-request-uri> element contains a group identity then this element can include an MCPTT group ID associated with the group identity in the <mcptt-request-uri> element. E.g. if the <mcptt-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCPTT group ID | TS 24.379 [9] clause F.1.3 | GROUP-CALL |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcptt-client-id | not present |  |  |  |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | The UUID URN of the MCPTT Client | RFC 4122 [106] TS 24.379 [9] clause 4.10 | (GROUP-CALL OR CHAT-GROUP-CALL OR EMERGENCY-CALL OR IMMPERIL-CALL) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) |  |  | (PRIVATE-CALL OR FIRST-TO-ANSWER) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | in general mcptt-client-id is not mandatory (e.g. for SIP SUBSCRIBE) | RFC 4122 [106] TS 24.379 [9] clause 4.10 | CONFIG, GROUPCONFIG |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | mcptt-client-id is mandatory in the SIP REGISTER or SIP PUBLISH for service authorisation according to TS 24.379 [9] clauses 7.2.1 and 7.2.2 | RFC 4122 [106] TS 24.379 [9] clause 4.10 | CONFIG AND REGISTER_PUBLISH |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | mcptt-client-id is mandatory in SIP PUBLISH for MCPTT service settings only, according to TS 24.379 [9] clause 7.2.3 | RFC 4122 [106] TS 24.379 [9] clause 4.10 | POC-SETTINGS-EVENT |
| alert-ind-rcvd | not present |  |  |  |
| gw-mcptt-usage | not present | Rel-18 |  |  |
| anyExt | not present |  | TS 24.379 [9], clause F.1.3 |  |
| anyExt |  | anyExt shall not contain any further elements than listed below unless specified otherwise in the specific message content of a test case | TS 24.379 [9], clause F.1.3 | FUNCTIONAL_ALIAS |
| functional-alias-URI | encrypted (NOTE 2) <functional-alias-URI> with mcpttURI set to px_MCPTT_ID_FA_A | set to the value of the functional alias that is used together with the "mcptt-calling-user-id" |  |  |
| NOTE 1: The SS shall check the mcptt-client-id - at the first time being sent by the UE to be a valid UUID URN with a format like "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" according to RFC 4122 [106] - to be all the same UUID URN in subsequent messages. NOTE 2: Encrypted element as described in Table 5.5.3.2.1-1A |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| REGISTER_PUBLISH | MCPTT-Info in SIP REGISTER or SIP PUBLISH request for service authorisation |
| INVITE_REFER | MCPTT-Info in SIP INVITE or SIP REFER request for call establishment |
| INVITE-RSP | MCPTT-Info in SIP response to a SIP INVITE NOTE: INVITE-RSP is inherited from the SIP response, i.e. it shall be considered as true whenever set for the SIP response |
| FUNCTIONAL_ALIAS | An active Functional Alias is used |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.3.2.1-1A: Encrypted MCPTT info parameter sent by the UE

| Derivation Path: TS 24.379 [9] clauses F.1.2, F.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the mcptt parameter |  |  |  |

##### -	MCVideo

Table 5.5.3.2.1-2: MCVideo-Info from the UE

| Derivation Path: TS 24.281 [86] Clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-access-token | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcvideo-access-token> with mcvideoString set to access token as assigned to the UE in the Token Response | The access token is opaque to the MCVideo client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG GROUPCONFIG |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | GROUP-CALL AND INVITE_REFER |
|  | "private" |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | “chat” |  |  | CHAT-GROUP-CALL AND INVITE_REFER |
| mcvideo-request-uri | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_Group_A_ID | The URI of the group |  | (GROUP-CALL OR CHAT-GROUP-CALL) AND INVITE_REFER |
|  | not present or Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_User_B_ID | The URI of the invited MCVideo Client |  | PRIVATE-CALL AND INVITE_REFER |
|  | Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_User_A_ID |  |  | POC-SETTINGS-EVENT |
| mcvideo-calling-user-id | not present or Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_ID_User_A |  |  |  |
|  | not present |  |  | CONFIG, GROUPCONFIG, POC-SETTINGS-EVENT |
| mcvideo-called-party-id | not present |  |  |  |
|  | not present or Encrypted (NOTE 2) <mcvideo-request-uri> with mcvideoURI set to px_MCVideo_ID_User_A |  |  | INVITE-RSP |
| mcvideo-calling-group-id | not present |  |  |  |
| required | not present |  |  |  |
| emergency-ind | not present or encrypted (NOTE 2) <emergency-ind> with mcvideoBoolean set to "false" |  |  |  |
|  | encrypted (NOTE 2) <emergency-ind> with mcvideoBoolean set to true |  |  | EMERGENCY-CALL AND INVITE-REFER |
| alert-ind | not present or encrypted (NOTE 2) <alert-ind> with mcvideoBoolean set to "false" |  |  |  |
|  | encrypted (NOTE 2) <alert-ind> with mcvideoBoolean set to pc_MCX_EmergencyIndWithAlertInd |  |  | EMERGENCY-CALL AND INVITE_REFER |
| imminentperil-ind | not present or encrypted (NOTE 2) <imminentperil-ind> with mcvideoBoolean set to "false" |  |  |  |
|  | encrypted (NOTE 2) <imminentperil-ind> with mcvideoBoolean set to true |  |  | IMMPERIL-CALL AND INVITE-REFER |
| broadcast-ind | not present or “false” |  |  |  |
|  | "true" |  |  | BROADCAST-CALL |
| mc-org | not present |  |  |  |
| associated-group-id | not present |  |  |  |
|  | px_MCVideo_Group_A_ID if mcvideo-request-uri contains a temporary group identity; otherwise, not present | if the <mcvideo-request-uri> element contains a group identity then this element can include an MCVideo group ID associated with the group identity in the <mcvideo-request-uri> element. E.g. if the <mcvideo-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCVideo group ID | TS 24.281 [86] clause F.1.3 | GROUP-CALL |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcvideo-client-id | not present |  |  |  |
|  | encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | The UUID URN of the MCVIDEO Client | RFC 4122 [106] TS 24.281 [86] clause 4.9 | (GROUP-CALL OR CHAT-GROUP-CALL OR EMERGENCY-CALL OR IMMPERIL-CALL) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | in general mcvideo-client-id is not mandatory (e.g. for SIP SUBSCRIBE) | RFC 4122 [106] TS 24.281 [86] clause 4.9 | CONFIG, GROUPCONFIG |
|  | encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | mcvideo-client-id is mandatory in the SIP REGISTER or SIP PUBLISH for service authorisation according to TS 24.281 [86] clauses 7.2.1 and 7.2.2 | RFC 4122 [106] TS 24.281 [86] clause 4.9 | CONFIG AND REGISTER_PUBLISH |
|  | encrypted (NOTE 2) < mcvideo-client-id> with mcvideoString set to valid UUID URN (NOTE 1) | mcvideo-client-id is mandatory in SIP PUBLISH for MCVideo service settings only, according to TS 24.281 [86] clause 7.2.3 | RFC 4122 [106] TS 24.281 [86] clause 4.9 | POC-SETTINGS-EVENT |
| alert-ind-rcvd | not present |  |  |  |
| multiple-devices-ind | not present |  |  |  |
| video-pull-url | not present |  |  |  |
| gw-mcvideo-usage | not present | Rel-18 |  |  |
| anyExt | not present |  | TS 24.281 [86] clause F.1.3 |  |
| anyExt |  |  |  | FUNCTIONAL_ALIAS |
| functional-alias-URI | encrypted (NOTE 2) <functional-alias-URI> with mcvideoURI set to px_MCVideo_ID_FA_A | set to the value of the functional alias that is used together with the "mcvideo-calling-user-id" |  | FUNCTIONAL_ALIAS |
| NOTE 1: The SS shall check the mcvideo-client-id - at the first time being sent by the UE to be a valid UUID URN with a format like "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" according to RFC 4122 [106] - to be all the same UUID URN in subsequent messages. NOTE 2: Encrypted element as described in Table 5.5.3.2.1-2A |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| REGISTER_PUBLISH | MCVideo-Info in SIP REGISTER or SIP PUBLISH request for service authorisation |
| INVITE_REFER | MCVideo-Info in SIP INVITE or SIP REFER request for call establishment |
| INVITE-RSP | MCVideo-Info in SIP response to a SIP INVITE |
| FUNCTIONAL_ALIAS | An active Functional Alias is used |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.3.2.1-2A: Encrypted MCVideo info parameter sent by the UE

| Derivation Path: TS 24.281 [86] clauses F.1.2, F.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the mcvideo parameter |  |  |  |

##### -	MCData

Table 5.5.3.2.1-3: MCData-Info from the UE

| Derivation Path: TS 24.282 [87], Clause D.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-access-token | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcdata-access-token> with mcdataString set to access token as assigned to the UE in the Token Response | The access token is opaque to the MCData client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG GROUPCONFIG |
| request-type | not present |  |  |  |
|  | "one-to-one-sds" |  |  | MCD_1to1 |
|  | “group-sds” |  |  | MCD_grp |
| mcdata-request-uri | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID |  |  | MCD_grp |
|  | Encrypted (NOTE 2) <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID |  |  | POC-SETTINGS-EVENT |
| mcdata-calling-user-id | not present |  |  |  |
| mcdata-called-party-id | not present |  |  |  |
| mcdata-calling-group-id | not present |  |  |  |
| alert-ind | not present |  |  |  |
| originated-by | not present |  |  |  |
| mcdata-client-id | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) |  |  | MCD_grp |
|  | Encrypted (NOTE 2) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) |  |  | CONFIG AND PUBLISH |
|  | not present or encrypted (NOTE 2) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) | in general mcdata-client-id is not mandatory (e.g. for SIP SUBSCRIBE) |  | (CONFIG OR GROUPCONFIG) AND NOT REGISTER (NOTE 3) |
|  | Encrypted (NOTE 2) <mcdata-client-id> with mcdataString set to valid UUID URN (NOTE 1) | mcdata-client-id is mandatory in SIP PUBLISH for MCData service settings only, according to TS 24.282 [87] clause 7.2.3 | RFC 4122 [106] | POC-SETTINGS-EVENT |
| mcdata-controller-psi | not present |  |  |  |
| gw-mcdata-usage | not present | Rel-18 |  |  |
| anyExt | not present |  |  |  |
| anyExt |  | anyExt shall not contain any further elements than listed below unless specified otherwise in the specific message content of a test case |  | PRE_ESTABLISHED_SESSION OR FUNCTIONAL_ALIAS |
| pre-established-session-ind | “true” | TS 24.282 [87], Clause 18.3.2.1 |  | PRE_ESTABLISHED_SESSION |
| functional-alias-URI | encrypted (NOTE 2) <functional-alias-URI> with mcdataURI set to px_MCData_ID_FA_A | set to the value of the functional alias that is used together with the "mcdata-calling-user-id" |  | FUNCTIONAL_ALIAS |
| NOTE 1: The SS shall check the mcdata-client-id - at the first time being sent by the UE to be a valid UUID URN with a format like "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" according to RFC 4122 [106] - to be all the same UUID URN in subsequent messages. NOTE 2: Encrypted element as described in Table 5.5.3.2.1-3A NOTE 3: In contrast to MCPTT and MCVideo for MCData TS 24.282 [87] clause 7.2.1 does not specify the client-id to be included in the REGISTER request. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCD_1to1 | A one-to-one MCData call |
| MCD_grp | A goup MCData call |
| REGISTER | MCData-Info in SIP REGISTER request for service authorisation |
| PUBLISH | MCData-Info in SIP PUBLISH request for service authorisation |
| PRE_ESTABLISHED_SESSION | A pre-established sessions is being established |
| FUNCTIONAL_ALIAS | An active Functional Alias is used |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.3.2.1-3A: Encrypted MCData info parameter sent by the UE

| Derivation Path: TS 24.282 [87] clauses D.1.2, D.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the mcdata parameter |  |  |  |

<<< TEXT SKIPPED >>>

#### 5.5.3.5	PIDF

##### 5.5.3.5.1	PIDF from the UE

##### -	MCPTT

Table 5.5.3.5.1-1: PIDF for MCPTT from the UE

| Derivation Path: RFC 3863 [114] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | RFC 3863 [114] |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| affiliation |  | MCPTT extension | TS 24.379 [9] clause 9.3.1 | AFFILIATION |
| group | Encrypted URI (NOTE 1) with value set to px_MCPTT_Group_A_ID |  |  |  |
| client | not present |  |  |  |
| status | not present |  |  |  |
| expires | not present |  |  |  |
| functionalAlias |  | MCPTT extension | TS 24.379 [9] Table 9A.3.1.2-1 | FUNCTIONAL_ALIAS_STATUS_CHANGE |
| functionalAliasID attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_FA_A |  |  |  |
| user attribute | not present |  |  |  |
| status attribute | not present |  |  |  |
| expires attribute | not present |  |  |  |
| contact | not present |  |  |  |
| note | not present |  |  |  |
| timestamp | not present |  |  |  |
| note | not present |  |  |  |
| p-id | any allowed value if present |  | TS 24.379 [9] clause 9.3.1 | AFFILIATION |
| p-id-fa | Any allowed value | a globally unique value set to an identifier of a SIP PUBLISH request | TS 24.379 [9] clause 9A.2.1.2 | FUNCTIONAL_ALIAS_STATUS_CHANGE |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FUNCTIONAL_ALIAS_STATUS_CHANGE | PIDF sent by the UE in request for functional alias status change |
| For further conditions see table 5.5.1-1 |  |

##### -	MCVideo

Table 5.5.3.5.1-2: PIDF for MCVideo from the UE

| Derivation Path: RFC 3863 [114] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | RFC 3863 [114] |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCVideo_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| affiliation |  |  | TS 24.281 [86] clause 8.3.1 | AFFILIATION |
| group | Encrypted URI (NOTE 1) with value set to px_MCVideo_Group_A_ID |  |  |  |
| client | not present |  |  |  |
| status | not present |  |  |  |
| expires | not present |  |  |  |
| functionalAlias |  | MCVideo extension | TS 24.281 [86] Table 20.3.1.2-1 | FUNCTIONAL_ALIAS_STATUS_CHANGE |
| functionalAliasID attribute | Encrypted URI (NOTE 1) with value set to px_MCVideo_ID_FA_A |  |  |  |
| user attribute | not present |  |  |  |
| status attribute | not present |  |  |  |
| expires attribute | not present |  |  |  |
| p-id | any allowed value if present |  |  | AFFILIATION |
| p-id-fa | Any allowed value | a globally unique value set to an identifier of a SIP PUBLISH request | TS 24.281 [86] clause 20.2.1.2 | FUNCTIONAL_ALIAS_STATUS_CHANGE |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FUNCTIONAL_ALIAS_STATUS_CHANGE | PIDF sent by the UE in request for functional alias status change |
| For further conditions see table 5.5.1-1 |  |

##### -	MCData

Table 5.5.3.5.1-3: PIDF for MCData from the UE

| Derivation Path: RFC 3863 [114] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | RFC 3863 [114] |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCData_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| affiliation |  |  | TS 24.282 [87] clause 8.4.1 | AFFILIATION |
| group | Encrypted URI (NOTE 1) with value set to px_MCDATA_Group_A_ID |  |  |  |
| client | not present |  |  |  |
| status | not present |  |  |  |
| expires | not present |  |  |  |
| functionalAlias |  | MCData extension | TS 24.282 [87] Table 22.3.1.2-1 | FUNCTIONAL_ALIAS_STATUS_CHANGE |
| functionalAliasID attribute | Encrypted URI (NOTE 1) with value set to px_MCData_ID_FA_A |  |  |  |
| user attribute | not present |  |  |  |
| status attribute | not present |  |  |  |
| expires attribute | not present |  |  |  |
| p-id | any allowed value or same value as sent in SIP PUBLISH | set to an identifier of a SIP PUBLISH request |  | AFFILIATION |
| p-id-fa | Any allowed value | a globally unique value set to an identifier of a SIP PUBLISH request | TS 24.282 [87] clause 22.2.1.2 | FUNCTIONAL_ALIAS_STATUS_CHANGE |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FUNCTIONAL_ALIAS_STATUS_CHANGE | PIDF sent by the UE in request for functional alias status change |
| For further conditions see table 5.5.1-1 |  |

##### 5.5.3.5.2	PIDF from the SS

##### -	MCPTT

Table 5.5.3.5.2-1: PIDF for MCPTT from the SS

| Derivation Path: RFC 3863 [114] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | RFC 3863 [114] |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| affiliation |  | MCPTT extension | TS 24.379 [9] clause 9.3.1 | AFFILIATION |
| group | Encrypted URI (NOTE 1) with value set to px_MCPTT_Group_A_ID |  |  |  |
| client | not present |  |  |  |
| status | "affiliating" |  |  |  |
| expires | not present |  |  |  |
| functionalAlias |  | MCPTT extension | TS 24.379 [9] Table 9A.3.1.2-1 | FUNCTIONAL_ALIAS_ACTIVATED |
| functionalAliasID attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_FA_A |  |  |  |
| user attribute | not present |  |  |  |
| status attribute | "activated" |  |  |  |
| expires attribute | not present |  |  |  |
| contact | not present |  |  |  |
| note | not present |  |  |  |
| timestamp | not present |  |  |  |
| note | not present |  |  |  |
| p-id | not present |  |  | AFFILIATION |
| p-id-fa | same value as received in the SIP PUBLISH message |  | TS 24.379 [9] clause 9A.2.2.2.5 | NOTIFY_FOR_PUBLISH |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FUNCTIONAL_ALIAS_ACTIVATED | PIDF sent by the SS in notification for functional alias getting activated |
| NOTIFY_FOR_PUBLISH | PIDF sent by the SS in notification associated with a previous SIP PUBLISH message sent by the UE |
| For further conditions see table 5.5.1-1 |  |

##### -	MCVideo

Table 5.5.3.5.2-2: PIDF for MCVideo from the SS

| Derivation Path: RFC 3863 [114] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | RFC 3863 [114] |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCVideo_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| affiliation |  |  | TS 24.281 [86] clause 8.3.1 | AFFILIATION |
| group | Encrypted URI (NOTE 1) with value set to px_MCVideo_Group_A_ID |  |  |  |
| client | not present |  |  |  |
| status | “affiliating” |  |  |  |
| expires | not present |  |  |  |
| functionalAlias |  | MCVideo extension | TS 24.281 [86] Table 20.3.1.2-1 | FUNCTIONAL_ALIAS_ACTIVATED |
| functionalAliasID attribute | Encrypted URI (NOTE 1) with value set to px_MCVideo_ID_FA_A |  |  |  |
| user attribute | not present |  |  |  |
| status attribute | "activated" |  |  |  |
| expires attribute | not present |  |  |  |
| p-id | not present |  |  | AFFILIATION |
| p-id-fa | same value as received in the SIP PUBLISH message |  | TS 24.281 [86] clause 20.2.2.2.5 | NOTIFY_FOR_PUBLISH |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FUNCTIONAL_ALIAS_ACTIVATED | PIDF sent by the SS in notification for functional alias getting activated |
| NOTIFY_FOR_PUBLISH | PIDF sent by the SS in notification associated with a previous SIP PUBLISH message sent by the UE |
| For further conditions see table 5.5.1-1 |  |

##### -	MCData

Table 5.5.3.5.2-3: PIDF for MCData from the SS

| Derivation Path: RFC 3863 [114] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  | RFC 3863 [114] |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCDATA_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| id attribute | Encrypted URI (NOTE 1) with value set to the mcptt-client-id as provided by the UE at registration |  |  |  |
| status |  |  |  |  |
| affiliation |  |  | TS 24.282 [87] clause 8.4.1 | AFFILIATION |
| group | px_MCDATA_Group_A_ID |  |  |  |
| client | not present |  |  |  |
| status | “affiliating” |  |  |  |
| expires | not present |  |  |  |
| functionalAlias |  | MCData extension | TS 24.282 [87] Table 22.3.1.2-1 | FUNCTIONAL_ALIAS_ACTIVATED |
| functionalAliasID attribute | Encrypted URI (NOTE 1) with value set to px_MCData_ID_FA_A |  |  |  |
| user attribute | not present |  |  |  |
| status attribute | "activated" |  |  |  |
| expires attribute | not present |  |  |  |
| p-id | not present |  |  | AFFILIATION |
| p-id-fa | same value as received in the SIP PUBLISH message |  | TS 24.282 [87] clause 22.2.2.2.5 | NOTIFY_FOR_PUBLISH |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FUNCTIONAL_ALIAS_ACTIVATED | PIDF sent by the SS in notification for functional alias getting activated |
| NOTIFY_FOR_PUBLISH | PIDF sent by the SS in notification associated with a previous SIP PUBLISH message sent by the UE |
| For further conditions see table 5.5.1-1 |  |

<<< TEXT SKIPPED >>>

#### 5.5.8.7	MCVideo User Profile

The structure of a user profile document is specified in TS 24.484 [14] clause 9.3. Single MCVideo group configuration parameters are defined in TS 24.483 [13] clause 13.2.

Table 5.5.8.7-1: MCVideo User Profile Defaults

| Derivation Path: TS 24.484 [14], clause 9.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| mcvideo-user-profile |  |  |  |  |  |  |  |  |  |
| XUI-URI attribute |  | "sip:" & px_MCVideo_ID_User_A |  | same as the XUI value of the Document URI |  |  |  |  |  |
| user-profile-index attribute |  | "42" |  | value arbitrarily selected |  |  |  |  |  |
| Status |  | “true” |  | MCVideo user profile is enabled |  |  |  |  |  |
| ProfileName |  | "mcvideo-user-profile-" & user-profile-index & ".xml" |  | name of the user profile document; user-profile-index is the value of the user-profile-index attribute |  | TS 24.483 [13] clause 13.2.3; |  |  |  |
| Common |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  | Index for the particular MCVideo user profile |  |  |  |  |  |
| MCVideoUserID |  |  |  | Indicates an MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user |  | TS 24.483 [13] clause 13.2.7 |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCVideo_ID_User_A |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user |  |  |  |  |  |
| UserAlias |  |  |  |  |  |  |  |  |  |
| alias-entry |  | px_MCVideo_User_A_Alias |  | Alphanumeric aliases of MCVideo user |  | TS 24.483 [13] clause 13.2.11 |  |  |  |
| ParticipantType |  | px_MCX_User_A_ParticipantType |  | The functional category of the participant (e.g., first responder, second responder, dispatch, dispatch supervisor), typically defined by the MCVideo administrators. |  | TS 24.483 [13] clause 13.2.15 |  |  |  |
| MissionCriticalOrganization |  | px_MCX_DomainName_Organization_A |  | Indicates the organization an MCVideo user belongs to |  | TS 24.483 [13] clause 13.2.16 |  |  |  |
| PrivateCall |  |  |  |  |  |  |  |  |  |
| PrivateCallList |  |  |  |  |  |  |  |  |  |
| PrivateCallOnNetwork[1] |  |  |  |  |  |  |  |  |  |
| PrivateCallURI |  |  |  |  |  |  |  |  |  |
| index attribute |  | 0 |  |  |  |  |  |  |  |
| uri-entry |  | px_MCVideo_ID_User_B |  |  |  |  |  |  |  |
| display-name |  | "User B Name" |  |  |  |  |  |  |  |
| PrivateCallKMSURI |  |  |  |  |  |  |  |  |  |
| uri-entry |  | "" |  | According to TS 24.484 [14] if the entry element is empty, the KMS URI present in the MCS initial configuration document is used |  |  |  |  |  |
| PrivateCallOnNetwork[2] |  |  |  |  |  |  |  |  |  |
| PrivateCallURI |  |  |  |  |  |  |  |  |  |
| index attribute |  | 1 |  |  |  |  |  |  |  |
| uri-entry |  | px_MCVideo_ID_User_C |  |  |  |  |  |  |  |
| display-name |  | "User C Name" |  |  |  |  |  |  |  |
| PrivateCallKMSURI |  |  |  |  |  |  |  |  |  |
| uri-entry |  | "" |  | According to TS 24.484 [14] if the entry element is empty, the KMS URI present in the MCS initial configuration document is used |  |  |  |  |  |
| PrivateCallOffNetwork |  | not present |  |  |  |  |  |  |  |
| EmergencyCall |  |  |  |  |  |  |  |  |  |
| MCVideoPrivateRecipient |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| entry-info attribute |  | "UsePreConfigured" |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCVideo_ID_User_B |  |  |  |  |  |  |  |
| display-name |  | "User B Name" |  |  |  |  |  |  |  |
| ProSeUserID-entry |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| DiscoveryGroupID |  | '123456'O |  |  |  |  |  |  |  |
| User-Info-ID |  | '555555555555'O |  |  |  |  |  |  |  |
| MCVideo-group-call |  |  |  |  |  |  |  |  |  |
| MaxSimultaneousCallsN6 |  | 3 |  |  |  |  |  |  |  |
| EmergencyCall |  |  |  |  |  |  |  |  |  |
| MCVideoGroupInitiation |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| entry-info attribute |  | "UseCurrentlySelectedGroup" |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCVideo_Group_A_ID |  |  |  |  |  |  |  |
| display-name |  | px_MCVideo_Group_A_Name |  |  |  |  |  |  |  |
| ImminentPerilCall |  |  |  |  |  |  |  |  |  |
| MCVideoGroupInitiation |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| entry-info attribute |  | "UseCurrentlySelectedGroup" |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCVideo_Group_A_ID |  |  |  |  |  |  |  |
| display-name |  | px_MCVideo_Group_A_Name |  |  |  |  |  |  |  |
| EmergencyAlert |  |  |  |  |  |  |  |  |  |
| MCVideoGroupInitiation |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| entry-info attribute |  | "UseCurrentlySelectedGroup" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCVideo_Group_A_ID |  |  |  |  |  |  |  |
| display-name |  | px_MCVideo_Group_A_Name |  |  |  |  |  |  |  |
| Priority |  | 10 |  |  |  |  |  |  |  |
| OnNetwork |  |  |  |  |  |  |  |  |  |
| index |  | "1" |  |  |  |  |  |  |  |
| MCVideoGroupInfo |  |  |  |  |  |  |  |  |  |
| MCVideo-Group-ID |  | px_MCVideo_Group_A_ID |  |  |  |  |  |  |  |
| GMS-Serv-Id |  | tsc_MCX_GMS_Hostname |  |  |  |  |  |  |  |
| IdMS-Token-Endpoint |  | "https://" & px_MCX_IdMS_token_IPAddress & ":" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath |  | Identity management server token endpoint identity information |  | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A |  | IPv4 |  |
|  |  | "https://[" & px_MCX_IdMS_token_IPAddress & "]:" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath |  | Identity management server token endpoint identity information |  | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A |  | IPv6 |  |
| RelativePresentationPriority |  | "7" |  |  |  | TS 24.483 [13] clause 13.2.51 |  |  |  |
| GroupKMSURI |  | tsc_MCX_KMS_Hostname |  |  |  |  |  |  |  |
| MaxAffiliationsN2 |  | "10" |  |  |  | TS 24.483 [13] clause 13.2.67 |  |  |  |
| PrivateEmergencyAlert |  |  |  |  |  | TS 24.483 [13] clause 13.2.87 |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| entry-info attribute |  | "UsePreConfigured" |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCVideo_ID_User_B |  |  |  |  |  |  |  |
| display-name |  | "User B Name" |  |  |  |  |  |  |  |
| RemoteGroupSelectionURIList |  |  |  |  |  | TS 24.483 [13] clause 13.2.87 |  |  |  |
| entry[1] |  | px_MCVideo_ID_User_A |  |  |  |  |  |  |  |
| entry[2] |  | px_MCVideo_ID_User_B |  |  |  |  |  |  |  |
| entry[3] |  | px_MCVideo_ID_User_C |  |  |  |  |  |  |  |
| anyExt |  |  |  |  |  |  |  |  |  |
| FunctionalAliasList |  |  |  |  |  | TS 24.483 clause 13.2.87A3 |  |  |  |
| entry[1] |  |  |  |  |  |  |  |  |  |
| uri-entry[1] |  | px_MCVideo_ID_FA_A |  |  |  |  |  |  |  |
| anyExt |  |  |  |  |  |  |  |  |  |
| LocationCriteriaForActivation |  |  |  |  |  |  |  |  |  |
| EnterSpecificArea |  |  |  |  |  |  |  |  |  |
| EllipsoidArcArea |  |  |  |  |  |  |  |  |  |
| Center |  |  |  |  |  |  |  |  |  |
| latitude |  | "3331608" |  | Latitude of 35.74428 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| longitude |  | "6510401" |  | Longitude of 139.69806 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| Radius |  | "10" |  | Radius of 50 meters encoded according to TS 23.032 [65] clause 6.6 |  |  |  |  |  |
| OffsetAngle |  | "0" |  | 0 degrees |  |  |  |  |  |
| IncludedAngle |  | "179" |  | Full circle: 360 degrees encoded according to TS 23.032 [65] clause 6.7 |  |  |  |  |  |
| ExitSpecificArea |  |  |  |  |  |  |  |  |  |
| EllipsoidArcArea |  |  |  |  |  |  |  |  |  |
| Center |  |  |  |  |  |  |  |  |  |
| latitude |  | "3331608" |  | Latitude of 35.74428 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| longitude |  | "6510349" |  | Longitude of 139.69695 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| Radius |  | "10" |  | Radius of 50 meters encoded according to TS 23.032 [65] clause 6.6 |  |  |  |  |  |
| OffsetAngle |  | "0" |  | 0 degrees |  |  |  |  |  |
| IncludedAngle |  | "179" |  | Full circle: 360 degrees encoded according to TS 23.032 [65] clause 6.7 |  |  |  |  |  |
| LocationCriteriaForDeactivation |  |  |  |  |  |  |  |  |  |
| EnterSpecificArea |  |  |  |  |  |  |  |  |  |
| EllipsoidArcArea |  |  |  |  |  |  |  |  |  |
| Center |  |  |  |  |  |  |  |  |  |
| latitude |  | "3331608" |  | Latitude of 35.74428 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| longitude |  | "6510349" |  | Longitude of 139.69695 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| Radius |  | "10" |  | Radius of 50 meters encoded according to TS 23.032 [65] clause 6.6 |  |  |  |  |  |
| OffsetAngle |  | "0" |  | 0 degrees |  |  |  |  |  |
| IncludedAngle |  | "179" |  | Full circle: 360 degrees encoded according to TS 23.032 [65] clause 6.7 |  |  |  |  |  |
| ExitSpecificArea |  |  |  |  |  |  |  |  |  |
| EllipsoidArcArea |  |  |  |  |  |  |  |  |  |
| Center |  |  |  |  |  |  |  |  |  |
| latitude |  | "3331608" |  | Latitude of 35.74428 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| longitude |  | "6510401" |  | Longitude of 139.69806 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| Radius |  | "10" |  | Radius of 50 meters encoded according to TS 23.032 [65] clause 6.6 |  |  |  |  |  |
| OffsetAngle |  | "0" |  | 0 degrees |  |  |  |  |  |
| IncludedAngle |  | "179" |  | Full circle: 360 degrees encoded according to TS 23.032 [65] clause 6.7 |  |  |  |  |  |
| manual-deactivation-not-allowed-if-location-criteria-met |  | "false" |  |  |  | TS 24.483 [13] clause 10.2.97B3D |  |  |  |
| RulesForAffiliation |  |  |  |  |  | TS 24.483 [13] clause 5.2.48B4A |  |  |  |
| ListOfLocationCriteria |  |  |  |  |  |  |  |  |  |
| EnterSpecificArea |  |  |  |  |  |  |  |  |  |
| EllipsoidArcArea |  |  |  |  |  |  |  |  |  |
| Center |  |  |  |  |  |  |  |  |  |
| Latitude |  | "3331608" |  | Latitude of 35.74428 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| Longitude |  | "6510349" |  | Longitude of 139.69695 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| Radius |  | "10" |  | Radius of 50 meters encoded according to TS 23.032 [65] clause 6.6 |  |  |  |  |  |
| OffsetAngle |  | "0" |  | 0 degrees |  |  |  |  |  |
| IncludedAngle |  | "179" |  | Full circle: 360 degrees encoded according to TS 23.032 [65] clause 6.7 |  |  |  |  |  |
| ExitSpecificArea |  |  |  |  |  |  |  |  |  |
| EllipsoidArcArea |  |  |  |  |  |  |  |  |  |
| Center |  |  |  |  |  |  |  |  |  |
| Latitude |  | "3331608" |  | Latitude of 35.74428 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| Longitude |  | "6510401" |  | Longitude of 139.69806 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| Radius |  | "10" |  | Radius of 50 meters encoded according to TS 23.032 [65] clause 6.6 |  |  |  |  |  |
| OffsetAngle |  | "0" |  | 0 degrees |  |  |  |  |  |
| IncludedAngle |  | "179" |  | Full circle: 360 degrees encoded according to TS 23.032 [65] clause 6.7 |  |  |  |  |  |
| RulesForDeaffiliation |  |  |  |  |  | TS 24.483 [13] clause 5.2.48B4B |  |  |  |
| ListOfLocationCriteria |  |  |  |  |  |  |  |  |  |
| EnterSpecificArea |  |  |  |  |  |  |  |  |  |
| EllipsoidArcArea |  |  |  |  |  |  |  |  |  |
| Center |  |  |  |  |  |  |  |  |  |
| latitude |  | "3331608" |  | Latitude of 35.74428 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| longitude |  | "6510401" |  | Longitude of 139.69806 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| Radius |  | "10" |  | Radius of 50 meters encoded according to TS 23.032 [65] clause 6.6 |  |  |  |  |  |
| OffsetAngle |  | "0" |  | 0 degrees |  |  |  |  |  |
| IncludedAngle |  | "179" |  | Full circle: 360 degrees encoded according to TS 23.032 [65] clause 6.7 |  |  |  |  |  |
| ExitSpecificArea |  |  |  |  |  |  |  |  |  |
| EllipsoidArcArea |  |  |  |  |  |  |  |  |  |
| Center |  |  |  |  |  |  |  |  |  |
| latitude |  | "3331608" |  | Latitude of 35.74428 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| longitude |  | "6510349" |  | Longitude of 139.69695 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| Radius |  | "10" |  | Radius of 50 meters encoded according to TS 23.032 [65] clause 6.6 |  |  |  |  |  |
| OffsetAngle |  | "0" |  | 0 degrees |  |  |  |  |  |
| IncludedAngle |  | "179" |  | Full circle: 360 degrees encoded according to TS 23.032 [65] clause 6.7 |  |  |  |  |  |
| manual-deaffiliation-not-allowed-if-affiliation-rules-are-met |  | "false" |  |  |  | TS 24.483 [13] clause 5.2.48B6 |  |  |  |
| OffNetwork |  |  |  |  |  |  |  |  |  |
| index |  | "1" |  |  |  |  |  |  |  |
| MCVideoGroupInfo |  |  |  |  |  |  |  |  |  |
| MCVideo-Group-ID |  | px_MCVideo_Group_A_ID |  |  |  |  |  |  |  |
| GMS-App-Serv-Id |  | tsc_MCX_GMS_Hostname |  |  |  |  |  |  |  |
| IdMS-Token-Endpoint |  | "https://" & px_MCX_IdMS_token_IPAddress & ":" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath |  | Identity management server token endpoint identity information |  | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A |  | IPv4 |  |
|  |  | "https://[" & px_MCX_IdMS_token_IPAddress & "]:" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath |  | Identity management server token endpoint identity information |  | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A |  | IPv6 |  |
| RelativePresentationPriority |  | "7" |  |  |  | TS 24.483 [13] clause 13.2.51 |  |  |  |
| User-Info-Id |  | '555555555555'O |  |  |  | TS 24.483 [13] clause 13.2.102 |  |  |  |
| cp:ruleset |  |  |  |  |  |  |  |  |  |
| cp:rule |  |  |  |  |  |  |  |  |  |
| cp:id attribute |  | "rule1" |  |  |  |  |  |  |  |
| cp:actions |  |  |  |  |  |  |  |  |  |
| allow-presence-status |  | "true" |  |  |  |  |  |  |  |
| allow-request-presence |  | "true" |  |  |  |  |  |  |  |
| allow-query-availability-for-private-calls |  | "true" |  |  |  |  |  |  |  |
| allow-enable-disable-user |  | "true" |  |  |  |  |  |  |  |
| allow-enable-disable-UE |  | "true" |  |  |  |  |  |  |  |
| allow-private-call |  | "true" |  |  |  |  |  |  |  |
| allow-manual-commencement |  | "true" |  |  |  |  |  |  |  |
| allow-automatic-commencement |  | "true" |  |  |  |  |  |  |  |
| allow-force-auto-answer |  | "true" |  |  |  |  |  |  |  |
| allow-failure-restriction |  | "true" |  |  |  |  |  |  |  |
| allow-emergency-group-call |  | "true" |  |  |  |  |  |  |  |
| allow-emergency-private-call |  | "true" |  |  |  |  |  |  |  |
| allow-cancel-group-emergency |  | "true" |  |  |  |  |  |  |  |
| allow-cancel-private-emergency-call |  | "true" |  |  |  |  |  |  |  |
| allow-imminent-peril-call |  | "true" |  |  |  |  |  |  |  |
| allow-cancel-imminent-peril |  | "true" |  |  |  |  |  |  |  |
| allow-activate-emergency-alert |  | "true" |  |  |  |  |  |  |  |
| allow-cancel-emergency-alert |  | "true" |  |  |  |  |  |  |  |
| allow-offnetwork |  | "true" |  |  |  |  |  |  |  |
| allow-imminent-peril-change |  | "true" |  |  |  |  |  |  |  |
| allow-private-call-media-protection |  | "true" |  |  |  |  |  |  |  |
| allow-request-affiliated-groups |  | "true" |  |  |  |  |  |  |  |
| allow-request-to-affiliate-other-users |  | "true" |  |  |  |  |  |  |  |
| allow-recommend-to-affiliate-other-users |  | "true" |  |  |  |  |  |  |  |
| allow-private-call-to-any-user |  | "true" |  |  |  |  |  |  |  |
| allow-regroup |  | "true" |  |  |  |  |  |  |  |
| allow-private-call-participation |  | "true" |  |  |  |  |  |  |  |
| allow-manual-off-network-switch |  | "true" |  |  |  |  |  |  |  |
| allow-off-network-group-call-change-to-emergency |  | "true" |  |  |  |  |  |  |  |
| allow-revoke-transmit |  | "true" |  |  |  |  |  |  |  |
| allow-create-group-broadcast-group |  | "true" |  |  |  |  |  |  |  |
| allow-create-user-broadcast-group |  | "true" |  |  |  |  |  |  |  |
| anyExt |  |  |  |  |  |  |  |  |  |
| allow-request-remote-initiated-ambient-viewing |  | "true" |  |  |  |  |  |  |  |
| allow-request-locally-initiated-ambient-viewing |  | "true" |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| IPv4 | IP address is IPv4 address |
| IPv6 | IP address is IPv6 address |

#### 5.5.8.8	MCVideo Service Configuration

The structure of a service configuration document is specified in TS 24.484 [14] clause 9.4. Single MCVideo group configuration parameters are defined in TS 24.483 [13] clause 14.2.

Table 5.5.8.8-1: MCVideo Service Configuration Defaults

| Derivation Path: TS 24.484 [14], clause 9.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| service configuration |  |  |  |  |
| domain attribute | px_MCX_DomainName_Organization_A | Mandatory attribute: domain name of the mission critical organization |  |  |
| Common |  |  |  |  |
| min-length-alias | "2" | Indicates minimum length of an alphanumeric identifier (i.e., alias) |  |  |
| broadcast-group |  |  |  |  |
| num-levels-group-hierarchy | "1" | Indicates the number of levels of group hierarchy for group-broadcast groups |  |  |
| num-levels-user-hierarchy | "1" | Indicates the number of levels of user hierarchy for user-broadcast groups |  |  |
| on-network |  |  |  |  |
| signalling-protection |  |  |  |  |
| confidentiality-protection | "true" |  |  |  |
| integrity-protection | "true" |  |  |  |
| protection-between-mcvideo-servers |  |  |  |  |
| allow-signalling-protection | "true" |  |  |  |
| allow-transmission-control-protection | "true" |  |  |  |
| emergency-resource-priority |  |  |  |  |
| resource-priority-namespace | "mcpttp" | MCVideo uses the MCPTT namespace values of RFC 8101 [45] | RFC 8101 [45] |  |
| resource-priority-priority | "7" |  | RFC 8101 [45] |  |
| imminent-peril-resource-priority |  |  |  |  |
| resource-priority-namespace | "mcpttp" | MCVideo uses the MCPTT namespace values of RFC 8101 [45] | RFC 8101 [45] |  |
| resource-priority-priority | "4" |  | RFC 8101 [45] |  |
| normal-resource-priority |  |  |  |  |
| resource-priority-namespace | "mcpttp" | MCVideo uses the MCPTT namespace values of RFC 8101 [45] | RFC 8101 [45] |  |
| resource-priority-priority | "0" |  | RFC 8101 [45] |  |
| anyExt |  |  |  |  |
| functional-alias-list |  |  |  |  |
| functional-alias-entry[1] |  |  |  |  |
| functional-alias | px_MCVideo_ID_FA_A |  |  |  |
| max-simultaneous-activations | "1" |  |  |  |
| allow-takeover | "true" |  |  |  |
| mcdata-user-list |  |  |  |  |
| entry[1] |  |  |  |  |
| uri-entry | px_MCVideo_ID_User_A |  |  |  |
| functional-alias-priority | "1" |  |  |  |
| off-network |  |  |  |  |
| default-prose-per-packet-priority |  |  |  |  |
| mcvideo-private-call-signalling | "1" | Indicates the default ProSe Per-Packet Priority (PPPP) value |  |  |
| mcvideo-private-call-media | "1" | Indicates the default ProSe Per-Packet Priority (PPPP) value |  |  |
| mcvideo-emergency-private-call-signalling | "8" | Indicates the default ProSe Per-Packet Priority (PPPP) value |  |  |
| mcvideo-emergency-private-call-media | "8" | Indicates the default ProSe Per-Packet Priority (PPPP) value |  |  |
| private-call |  |  |  |  |
| mcvideo-max-duration | "600" | Value in seconds | TS 24.483 [13] clause 14.2.17 |  |
| num-levels-priority-hierarchy | "4" |  | TS 24.483 [13] clause 14.2.18 |  |
| default-pqi | not present | Rel-18 |  |  |