3GPP TSG-RAN5 Meeting #107	R5-252294

# Malta, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-1 | CR | 0012 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Group regroup IEs for MCVideo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, FirstNet, Element, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Configuration details for usage of group regroup is missing for MCVideo. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configuration details for usage of group regroup was added for MCVideo. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Group regroup cannot be tested for MCVideo. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.3.16	MCS-Regroup

##### 5.5.3.16.1	Common conditions for MCS-Regroup

The following conditions apply throughout clause 5.5.3.16:

Table 5.5.3.16.1-1: Conditions

| Condition | Explanation |
| --- | --- |
| GROUP_REGROUP | Creating a group regroup using preconfigured group |
| USER_REGROUP | Creating a user regroup using preconfigured group |
| REMOVE | Remove a user or group regroup using a preconfigured group |

##### 5.5.3.16.2	MCS-Regroup from the UE

##### -	MCPTT

Table 5.5.3.16.2-1: MCPTT-Regroup from the UE

| Derivation Path: TS 24.379 [9] clause F.7.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttregroup |  |  |  |  |
| mcpttregroup-Params |  |  |  |  |
| preconfig-group-id | Encrypted (NOTE 1) |  |  |  |
| preconfigured-group | not present |  |  |  |
|  | px_MCPTT_Group_A_ID | The URI of a group to be used as the configuration of the group regroup |  | GROUP_REGROUP, USER_REGROUP |
| mcptt-regroup-uri | Encrypted (NOTE 2) |  |  |  |
| mcptt-regroup-uri | px_MCPTT_Group_T_ID | The URI of the group regroup |  |  |
| groups-for-regroup | not present |  |  |  |
| groups-for-regroup | Encrypted (NOTE 3) |  |  | GROUP_REGROUP |
| group [1] | px_MCPTT_Group_A_ID | The URI of a group to regroup |  |  |
| group [2] | px_MCPTT_Group_B_ID | The URI of a group to regroup |  |  |
| users-for-regroup | not present |  |  |  |
| users-for-regroup | Encrypted (NOTE 4) |  |  | USER_REGROUP |
| user [1] | px_MCPTT_ID_User_A |  |  |  |
| user [2] | px_MCPTT_ID_User_B |  |  |  |
| user [3] | px_MCPTT_ID_User_D |  |  |  |
| regroup-action | "create" |  |  |  |
|  | “remove” |  |  | REMOVE |
| NOTE 1: Element content encryption either of element <preconfig-group-id> or of its sub-element <preconfigured-group> (if present) as described in Table 5.5.13.2-1 NOTE 2: Element content encryption either of element <mcptt-regroup-uri> or of its sub-element <mcptt-regroup-uri> as described in Table 5.5.13.2-1 NOTE 3: Element content encryption either of element <groups-for-regroup> or of each of its sub-elements <group> as described in Table 5.5.13.2-1 NOTE 4: Element content encryption either of element <users-for-regroup> or of each of its sub-elements <user> as described in Table 5.5.13.2-1 |  |  |  |  |

##### -	MCVideo

Table 5.5.3.16.2-2: MCVideo-Regroup from the UE

| Derivation Path: TS 24.281 [86] clause F.6.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoregroup |  |  |  |  |
| mcvideoregroup-Params |  |  |  |  |
| preconfig-group-id | Encrypted (NOTE 1) |  |  |  |
| preconfigured-group | not present |  |  |  |
|  | px_MCVideo_Group_A_ID | The URI of a group to be used as the configuration of the group regroup |  | GROUP_REGROUP, USER_REGROUP |
| mcvideo-regroup-uri | Encrypted (NOTE 2) |  |  |  |
| mcvideo-regroup-uri | px_MCVideo_Group_T_ID | The URI of the group regroup |  |  |
| groups-for-regroup | not present |  |  |  |
| groups-for-regroup | Encrypted (NOTE 3) |  |  | GROUP_REGROUP |
| group [1] | px_MCVideo_Group_A_ID | The URI of a group to regroup |  |  |
| group [2] | px_MCVideo_Group_B_ID | The URI of a group to regroup |  |  |
| users-for-regroup | not present |  |  |  |
| users-for-regroup | Encrypted (NOTE 4) |  |  | USER_REGROUP |
| user [1] | px_MCVideo_ID_User_A |  |  |  |
| user [2] | px_MCVideo_ID_User_B |  |  |  |
| user [3] | px_MCVideo_ID_User_D |  |  |  |
| regroup-action | "create" |  |  |  |
|  | “remove” |  |  | REMOVE |
| NOTE 1: Element content encryption either of element <preconfig-group-id> or of its sub-element <preconfigured-group> (if present) as described in Table 5.5.13.2-1 NOTE 2: Element content encryption either of element <mcvideo-regroup-uri> or of its sub-element <mcvideo-regroup-uri> as described in Table 5.5.13.2-1 NOTE 3: Element content encryption either of element <groups-for-regroup> or of each of its sub-elements <group> as described in Table 5.5.13.2-1 NOTE 4: Element content encryption either of element <users-for-regroup> or of each of its sub-elements <user> as described in Table 5.5.13.2-1 |  |  |  |  |

##### 5.5.3.16.3	MCS-Regroup from the SS

##### -	MCPTT

Table 5.5.3.16.3-1: MCPTT-Regroup from the SS

| Derivation Path: TS 24.379 [9] clause F.7.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttregroup |  |  |  |  |
| mcpttregroup-Params |  |  |  |  |
| preconfig-group-id |  |  |  |  |
| preconfig-group-id | Encrypted (NOTE 1) |  |  |  |
| preconfigured-group | not present |  |  |  |
|  | px_MCPTT_Group_A_ID | The URI of a group to be used as the configuration of the group regroup |  | GROUP_REGROUP, USER_REGROUP |
| mcptt-regroup-uri | Encrypted (NOTE 2) |  |  |  |
| mcptt-regroup-uri | px_MCPTT_Group_T_ID | The URI of the group regroup |  |  |
| groups-for-regroup | not present |  |  |  |
| groups-for-regroup | Encrypted (NOTE 3) |  |  | GROUP_REGROUP |
| group [1] | px_MCPTT_Group_A_ID | The URI of a group to regroup |  |  |
| group [2] | px_MCPTT_Group_B_ID | The URI of a group to regroup |  |  |
| users-for-regroup | not present |  |  |  |
| users-for-regroup | Encrypted (NOTE 4) |  |  | USER_REGROUP |
| user [1] | px_MCPTT_ID_User_A |  |  |  |
| user [2] | px_MCPTT_ID_User_B |  |  |  |
| user [3] | px_MCPTT_ID_User_D |  |  |  |
| regroup-action | "create" |  |  |  |
|  | “remove” |  |  | REMOVE |
| NOTE 1: Element content encryption of element <preconfig-group-id>’s sub-element <preconfigured-group> (if present) as described in Table 5.5.13.2-2 NOTE 2: Element content encryption of element <mcptt-regroup-uri>’s sub-element <mcptt-regroup-uri> as described in Table 5.5.13.2-2 NOTE 3: Element content encryption of each of element <groups-for-regroup>’s sub-elements <group> as described in Table 5.5.13.2-2 NOTE 4: Element content encryption of each of element <users-for-regroup>’s sub-elements <user> as described in Table 5.5.13.2-2 |  |  |  |  |

##### -	MCVideo

Table 5.5.3.16.3-2: MCVideo-Regroup from the SS

| Derivation Path: TS 24.281 [86] clause F.6.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoregroup |  |  |  |  |
| mcvideoregroup-Params |  |  |  |  |
| preconfig-group-id |  |  |  |  |
| preconfig-group-id | Encrypted (NOTE 1) |  |  |  |
| preconfigured-group | not present |  |  |  |
|  | px_MCVideo_Group_A_ID | The URI of a group to be used as the configuration of the group regroup |  | GROUP_REGROUP, USER_REGROUP |
| mcvideo-regroup-uri | Encrypted (NOTE 2) |  |  |  |
| mcvideo-regroup-uri | px_MCVideo_Group_T_ID | The URI of the group regroup |  |  |
| groups-for-regroup | not present |  |  |  |
| groups-for-regroup | Encrypted (NOTE 3) |  |  | GROUP_REGROUP |
| group [1] | px_MCVideo_Group_A_ID | The URI of a group to regroup |  |  |
| group [2] | px_MCVideo_Group_B_ID | The URI of a group to regroup |  |  |
| users-for-regroup | not present |  |  |  |
| users-for-regroup | Encrypted (NOTE 4) |  |  | USER_REGROUP |
| user [1] | px_MCVideo_ID_User_A |  |  |  |
| user [2] | px_MCVideo_ID_User_B |  |  |  |
| user [3] | px_MCVideo_ID_User_D |  |  |  |
| regroup-action | "create" |  |  |  |
|  | “remove” |  |  | REMOVE |
| NOTE 1: Element content encryption of element <preconfig-group-id>’s sub-element <preconfigured-group> (if present) as described in Table 5.5.13.2-2 NOTE 2: Element content encryption of element <mcptt-regroup-uri>’s sub-element <mcptt-regroup-uri> as described in Table 5.5.13.2-2 NOTE 3: Element content encryption of each of element <groups-for-regroup>’s sub-elements <group> as described in Table 5.5.13.2-2 NOTE 4: Element content encryption of each of element <users-for-regroup>’s sub-elements <user> as described in Table 5.5.13.2-2 |  |  |  |  |