3GPP TSG-RAN5 Meeting #107	R5-252753

# Malta, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-1 | CR | 0014 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of group rule based configurations for affiliation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, FirstNet, Element, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Configuration details for group rule based affiliation is missing for MCPTT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configuration details for group rule based affiliation was added to the definition od MCPTT group A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Group rule based affiliation cannot be tested for MCPPT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.7.1	MCPTT Group Configuration

The structure of a group configuration document is specified in TS 24.481 [11] clause 7, single MCPTT group configuration parameters are defined in TS 24.483 [13] clause 6.3.

The structure of the configuration document is based on several XML schemas. To distinguish the schemas the prefixes of their corresponding name spaces are used in the 'Information Element' column as according to table 7.2.2-2 of TS 24.481 [11].

Table 5.5.7.1-1: MCPTT Group Configuration Defaults

| Derivation Path: TS 24.481 [11] clause 7.2.2 |  |
| --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| list-service[1] |  |  |  | Group 1 |  |  |  |  |  |
| uri attribute |  | px_MCPTT_Group_A_ID |  | Value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 |  | TS 24.483 [13] clause 6.2.7 |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | Value is a <display-name> element specified in OMA OMA-TS-XDM_Group-V1_1 |  | TS 24.483 [13] clause 6.2.8 |  |  |  |
| list |  |  |  |  |  |  |  |  |  |
| entry[1] |  |  |  | group member 1 |  |  |  |  |  |
| uri attribute |  | px_MCPTT_ID_User_A |  | Indicates an MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user |  | TS 24.483 [13] clause 6.2.11 |  |  |  |
| display-name |  | Not present |  |  |  |  |  |  |  |
| mcpttgi:user-priority |  | "3" |  | Indicates the user priority of the MCPTT group member |  | TS 24.483 [13] clause 6.2.12 |  |  |  |
| mcpttgi:participant-type |  | px_MCX_User_A_ParticipantType |  | Participant type of the MCPTT group |  | TS 24.483 [13] clause 6.2.13 |  |  |  |
| mcpttgi:multi-talker-allowed |  | Present |  | Presence of the <multi-talker-allowed> element indicates that the MCPTT group member is authorized for multi-talker floor control in a MCPTT group call of the MCPTT group in on-network MCPTT procedures when the MCPTT group supports multi-talker-control. Absence of the <multi-talker-allowed> element indicates that the MCPTT group member identified by the <entry> element is not authorized for multi-talker floor control |  |  |  |  |  |
| entry[2] |  |  |  | group member 2 |  |  |  |  |  |
| uri attribute |  | px_MCPTT_ID_User_B |  | Indicates an MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user |  | TS 24.483 [13] clause 6.2.11 |  |  |  |
| display-name |  | Not present |  |  |  |  |  |  |  |
| mcpttgi:user-priority |  | "2" |  | Indicates the user priority of the MCPTT group member |  | TS 24.483 [13] clause 6.2.12 |  |  |  |
| mcpttgi:participant-type |  | px_MCX_User_B_ParticipantType |  | Participant type of the MCPTT group |  | TS 24.483 [13] clause 6.2.13 |  |  |  |
| mcpttgi:multi-talker-allowed |  | Present |  | Presence of the <multi-talker-allowed> element indicates that the MCPTT group member is authorized for multi-talker floor control in a MCPTT group call of the MCPTT group in on-network MCPTT procedures when the MCPTT group supports multi-talker-control. Absence of the <multi-talker-allowed> element indicates that the MCPTT group member identified by the <entry> element is not authorized for multi-talker floor control |  |  |  |  |  |
| entry[3] |  |  |  | group member 3 |  |  |  |  |  |
| uri attribute |  | px_MCPTT_ID_User_C |  | Indicates an MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user |  | TS 24.483 [13] clause 6.2.11 |  |  |  |
| display-name |  | Not present |  |  |  |  |  |  |  |
| mcpttgi:user-priority |  | "1" |  | Indicates the user priority of the MCPTT group member |  | TS 24.483 [13] clause 6.2.12 |  |  |  |
| mcpttgi:participant-type |  | px_MCX_User_C_ParticipantType |  | Participant type of the MCPTT group |  | TS 24.483 [13] clause 6.2.13 |  |  |  |
| mcpttgi:multi-talker-allowed |  | Present |  | Presence of the <multi-talker-allowed> element indicates that the MCPTT group member is authorized for multi-talker floor control in a MCPTT group call of the MCPTT group in on-network MCPTT procedures when the MCPTT group supports multi-talker-control. Absence of the <multi-talker-allowed> element indicates that the MCPTT group member identified by the <entry> element is not authorized for multi-talker floor control |  |  |  |  |  |
| cp:ruleset |  |  |  |  |  |  |  |  |  |
| cp:rule |  |  |  |  |  |  |  |  |  |
| cp:id attribute |  | "rule1" |  |  |  |  |  |  |  |
| cp:actions |  |  |  |  |  |  |  |  |  |
| cp:on-network-allow-getting-member-list |  | "true" |  | Indicates that the identity is allowed to get the MCS group member list of the MCS group in on-network procedures |  |  |  |  |  |
| cp:allow-initiate-conference |  | "true" |  |  |  |  |  |  |  |
| cp:join-handling |  | "true" |  |  |  |  |  |  |  |
| cp:allow-MCPTT-emergency-call |  | "true" |  | Indicates whether an MCPTT emergency group call is permitted on the MCPTT group |  | TS 24.483 [13] clause 6.2.19 |  |  |  |
| cp:allow-imminent-peril-call |  | "true" |  | Indicates whether an MCPTT imminent peril group call is permitted on the MCPTT group |  | TS 24.483 [13] clause 6.2.20 |  |  |  |
| cp:allow-MCPTT-emergency-alert |  | "true" |  | Indicates whether an MCPTT emergency alert is possible on the MCPTT group |  | TS 24.483 [13] clause 6.2.21 |  |  |  |
| cp:on-network-allow-getting-affiliation-list |  | "true" |  | Indicates that the identity is allowed to get the list of MCPTT users affiliated to the MCPTT group in on-network MCPTT procedures |  |  |  |  |  |
| cp:on-network-allow-conference-state |  | "true" |  | indicates that the identity is allowed to subscribe to the conference event package of an MCPTT group session of the MCPTT group in on-network MCPTT procedures |  |  |  |  |  |
| oxe:supported-services |  |  |  |  |  |  |  |  |  |
| oxe:service |  |  |  |  |  | TS 24.481 [11] |  |  |  |
| oxe:enabler |  | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |  |  |  |  |
| oxe:group-media |  |  |  |  |  |  |  |  |  |
| mcpttgi:mcptt-speech |  | Present |  |  |  |  |  |  |  |
| mcpttgi:owner |  | px_MCX_Group_A_Owner_Organization |  | Group’s owner (Mission Critical Organisation). |  | TS 24.483 [13] clause 6.2.15 |  |  |  |
| mcpttgi:preferred-voice-encodings |  |  |  |  |  |  |  |  |  |
| mcpttgi:encoding- |  |  |  |  |  |  |  |  |  |
| mcpttgi:name[1] |  | px_MCPTT_Group_A_preferred_VCodec |  | Preferred voice codec is a RTP payload. MCPTT clients shall support the AMR-WB codec. |  | RFC 4566 [27] TS 26.171 [66] TS 24.483 [13] clause 6.2.16 |  |  |  |
| mcpttgi:level-within-group-hierarchy |  | "0" |  | Indicates the level within a group hierarchy (only applicable for group-broadcast group). |  | TS 24.483 [13] clause 6.2.17 |  |  |  |
| mcpttgi:level-within-user-hierarchy |  | "0" |  | Indicates the level within user hierarchy (only applicable for user-broadcast group). |  | TS 24.483 [13] clause 6.2.18 |  |  |  |
| mcpttgi:protect-media |  | "true" |  | Indicates whether confidentiality and integrity of media is required on the MCPTT group |  | TS 24.483 [13] clause 6.2.22 |  |  |  |
| mcpttgi:protect-floor-control-signalling |  | "true" |  | Indicates whether confidentiality and integrity of floor control signalling is required on the MCPTT group |  | TS 24.483 [13] clause 6.2.23 |  |  |  |
| mcpttgi:off-network-ProSe-layer-2-group-id |  | tsc_MCX_Group_A_ProSeLayer2GroupID |  | Indicates the Prose layer-2 group ID |  | TS 23.303 [68] TS 24.483 [13] clause 6.2.27 |  |  |  |
| mcpttgi:off-network-IP-multicast-address |  | "0.0.0.0" |  | Indicates the ProSe group IP multicast address;the IP version is implicitly given by the notation of the IP address |  | TS 23.303 [68] TS 24.483 [13] clause 6.2.28 |  |  |  |
| mcpttgi:off-network-ProSe-relay-service-code |  | "123456" |  | Indicates the connectivity service that the ProSe UE-to-network relay provides to public safety applications |  | TS 23.303 [68] TS 24.483 [13] clause 6.2.29 |  |  |  |
| mcpttgi:off-network-in-progress-emergency-state-cancellation-timeout |  | "PT18H12M15S" |  | Indicates the timeout value for the cancellation of an in progress emergency for an MCPTT group call. "PT18H12M15S" corresponds to 65535 seconds what is maximum allowed value according to TS 24.483 [13] |  | TS 24.483 [13] clause 6.2.31 |  |  |  |
| mcpttgi:off-network-in-progress-imminent-peril-state-cancellation-timeout |  | "PT18H12M15S" |  | Indicates the timeout value for the cancellation of an in progress imminent peril for an MCPTT group call. "PT18H12M15S" corresponds to 65535 seconds what is maximum allowed value according to TS 24.483 [13] |  | TS 24.483 [13] clause 6.2.32 |  |  |  |
| mcpttgi:off-network-hang-timer |  | "PT5S" |  | Indicates the group call hang timer. "PT5S" corresponds to 5 seconds |  | TS 24.483 [13] clause 6.2.33 |  |  |  |
| mcpttgi:off-network-maximum-duration |  | "PT1M" |  | Indicates the max duration of group calls. "PT1M" corresponds to 1 minute |  | TS 24.483 [13] clause 6.2.34 |  |  |  |
| mcpttgi:off-network-queue-usage |  | "true" |  | Indicates if queuing is enabled or not |  | TS 24.483 [13] clause 6.2.34A |  |  |  |
| mcpttgi:off-network-ProSe-signalling-PPPP |  | "1" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.36 |  |  |  |
| mcpttgi:off-network-ProSe-media-PPPP |  | "1" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.37 |  |  |  |
| mcpttgi:off-network-ProSe-emergency-call-signalling-PPPP |  | "8" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.38 |  |  |  |
| mcpttgi:off-network-ProSe-emergency-call-media-PPPP |  | "8" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.39 |  |  |  |
| mcpttgi:off-network-ProSe-imminent-peril-call-signalling-PPPP |  | "7" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.40 |  |  |  |
| mcpttgi:off-network-ProSe-imminent-peril-call-media-PPPP |  | "7" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 24.483 [13] clause 6.2.41 |  |  |  |
| mcpttgi:multi-talker-control |  | "false" |  | "true" indicates that multi-talker control is enabled for the group "false" indicates that multi-talker control is disabled for the group |  |  |  |  |  |
| mcpttgi:max-number-simultaneous-talkers |  | "1" |  | Indicates the maximum number of parallel talkers in a MCPTT group session in on-network MCPTT procedures |  |  |  |  |  |
| mcpttgi:audio-mixing-entity |  | Not present |  | Absence of the <audio-mixing-entity> element indicates that audio mixing is performed in the network |  |  |  |  |  |
| mcpttgi:permitted-geographic-area |  |  |  |  |  | TS 24.481 [11] |  |  |  |
| mcpttgi:EllipsoidArcArea |  |  |  |  |  |  |  |  |  |
| mcpttgi:Center |  |  |  |  |  |  |  |  |  |
| mcpttgi:Latitude |  | "3331608" |  | Latitude of 35.74428 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| mcpttgi:Longitude |  | "6510349" |  | Longitude of 139.69695 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| mcpttgi:Radius |  | "10" |  | Radius of 50 meters encoded according to TS 23.032 [65] clause 6.6 |  |  |  |  |  |
| mcpttgi:OffsetAngle |  | "0" |  | 0 degrees |  |  |  |  |  |
| mcpttgi:IncludedAngle |  | "179" |  | Full circle: 360 degrees encoded according to TS 23.032 [65] clause 6.7 |  |  |  |  |  |
| mcpttgi:mandatory-geographic-area |  |  |  |  |  | TS 24.481 [11] |  |  |  |
| mcpttgi:EllipsoidArcArea |  |  |  |  |  |  |  |  |  |
| mcpttgi:Center |  |  |  |  |  |  |  |  |  |
| mcpttgi:Latitude |  | "3331608" |  | Latitude of 35.74428 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| mcpttgi:Longitude |  | "6510349" |  | Longitude of 139.69695 degrees encoded according to TS 23.032 [65] clause 6.1 |  |  |  |  |  |
| mcpttgi:Radius |  | "10" |  | Radius of 50 meters encoded according to TS 23.032 [65] clause 6.6 |  |  |  |  |  |
| mcpttgi:OffsetAngle |  | "0" |  | 0 degrees |  |  |  |  |  |
| mcpttgi:IncludedAngle |  | "179" |  | Full circle: 360 degrees encoded according to TS 23.032 [65] clause 6.7 |  |  |  |  |  |