3GPP TSG-RAN5 Meeting #107	R5-251937

# St Julian’s, Malta, 19th - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-5 | CR | 0011 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR/5GC enhancement of MCX IPCAN Test Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - At RAN5#106 the "MCX test model with TTCN based NR/5GC implementation (MCX NR5GC test model) has been added to 37.579-5 but there is no support of an "MCX test model with SS based NR/5GC implementation" yet. - The IPCAN procedures for the SS based E-UTRA/EPC or NR/5GC implementation of the MCX IPCAN test model have not been documented. - As IP addresses are assigned via NAS signalling, for the MCX IPCAN model even the IP addresses for MCX PDN shall be provided to the SS. - Editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enhancement of the MCX IPCAN test model for support of NR5GC. - Annex F added with documentation for the procedures of the MCX IPCAN test model. - Clarification added to Table 5.2.1-1. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of NR5GC with MCX IPCAN test model, no documentation for IPCAN procedures, remaining issues in EUTRA IPCAN test model. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2, annex F (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 37.579-1: "Mission Critical (MC) services; Part 1: Common test environment".

[3]	3GPP TS 37.579-2: "Mission Critical (MC) services; Part 2: Mission Critical Push To Talk (MCPTT) User Equipment (UE) Protocol conformance specification".

[4]	Void.

[5]	3GPP TS 37.579-4: "Mission Critical (MC) services; Part 4: Test Applicability and Implementation Conformance Statement (ICS) proforma specification".

[6]	3GPP TS 36.523-1: "User Equipment (UE) conformance specification; Part 1: Protocol conformance specification"

[7]	3GPP TS 22.179: "Mission Critical Push To Talk (MCPTT) over LTE; Stage 1".

[8]	Void.

[9]	3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control; Protocol specification".

[10]	3GPP TS 24.380: "Mission Critical Push To Talk (MCPTT) floor control; Protocol specification".

[11]	3GPP TS 24.481: "Mission Critical Services (MCS) group management; Protocol specification".

[12]	3GPP TS 24.482: "Mission Critical Services (MCS) identity management; Protocol specification".

[13]	3GPP TS 24.483: "Mission Critical Services (MCS) Management Object (MO)".

[14]	3GPP TS 24.484: "Mission Critical Services (MCS) configuration management; Protocol specification".

[15]	3GPP TS 33.179: "Security of Mission Critical Push-To-Talk (MCPTT)".

[16]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[17]	3GPP TS 24.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 3".

[18]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[19]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[20]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[21]	3GPP TS 23.003: "Numbering, addressing and identification".

[22]	ISO/IEC 9646-1: "Information technology - Open Systems Interconnection - Conformance testing methodology and framework - Part 1: General concepts".

[23]	ISO/IEC 9646-7: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[24]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[25]	IETF RFC 4566 (July 2006): "SDP: Session Description Protocol".

[26]	3GPP TS 26.171: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; General description".

[27]	3GPP TS 36.523-3: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 3: Test suites".

[28]	3GPP TS 34.229-3: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[29]	ISO/IEC 9646-1: "Information technology - Open Systems Interconnection - Conformance testing methodology and framework - Part 1: General concepts".

[30]	ISO/IEC 9646-7: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[31]	ETSI ES 201 873-1: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language".

[32]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)".

[33]	3GPP TS 27.007: "AT command set for User Equipment (UE)".

[34]	IETF RFC 4661: "An Extensible Markup Language (XML)-Based Format for Event Notification Filtering".

[34]	IETF RFC 4826: "Extensible Markup Language (XML) Formats for Representing Resource Lists".

[35]	W3C: "XML Encryption Syntax and Processing Version 1.1", https://www.w3.org/TR/xmlenc-core1/.

[36]	W3C: "XML Signature Syntax and Processing (Second Edition)", http://www.w3.org/TR/xmldsig-core/.

[37]	OMA - poc_listService-v1_0: "List Service".

[40]	OMA - xdm_commonPolicy-V1_0: "XDM - Common Policy".

[39]	OMA - xdm_extensions-v1_0: "XDM - XDM2 - Extensions".

[40]	OMA - xdm_rsrclst_uriusage-v1_0: "Resource List - URI usage".

[41]	W3C: "XML Encryption Syntax and Processing Version 1.1", https://www.w3.org/TR/xmlenc-core1/.

[42]	W3C: "XML Signature Syntax and Processing (Second Edition)", http://www.w3.org/TR/xmldsig-core/.

[43]	3GPP TS 33.180: "Security of the mission critical service".

[44]	IETF RFC 6507: "Elliptic Curve-Based Certificateless Signatures for Identity-Based Encryption (ECCSI)".

[45]	IETF RFC 6508: "Sakai-Kasahara Key Encryption (SAKKE)".

[46]	IETF RFC 6509 (February 2012): ''MIKEY-SAKKE: Sakai-Kasahara Key Encryption in Multimedia Internet KEYing (MIKEY)''.

[47]	IETF RFC 3394: "Advanced Encryption Standard (AES) Key Wrap Algorithm".

[48]	W3C: "XML Signature Syntax and Processing (Second Edition)", http://www.w3.org/TR/xmldsig-core/.

[49]	IETF RFC 7515: "JSON Web Signature (JWS)".

[50]	IETF RFC 5261: "An Extensible Markup Language (XML) Patch Operations Framework Utilizing XML Path Language (XPath) Selectors".

[51]	IETF RFC 5874: "An Extensible Markup Language (XML) Document Format for Indicating a Change in XML Configuration Access Protocol (XCAP) Resources".

[52]	IETF RFC 4354: "A Session Initiation Protocol (SIP) Event Package and Data Format for Various Settings in Support for the Push-to-Talk over Cellular (PoC) Service".

[53]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[54]	3GPP TS 23.280: "Common functional architecture to support mission critical services".

[55]	3GPP TS 24.281: "Mission Critical Video (MCVideo) signalling control; Protocol specification"

[56]	3GPP TS 24.581: "Mission Critical Video (MCVideo) media plane control; Protocol specification"

[57]	3GPP TS 24.282: "Mission Critical Data (MCData) signalling control; Protocol specification"

[58]	3GPP TS 24.582: "Mission Critical Data (MCData) media plane control; Protocol specification"

[59]	3GPP TS 37.579-6: "Mission Critical (MC) services; Part 6: Mission Critical Video (MCVideo) User Equipment (UE) Protocol conformance specification"

[60]	3GPP TS 37.579-7: "Mission Critical (MC) services; Part 7: Mission Critical Data (MCData) User Equipment (UE) Protocol conformance specification"

[61]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common Test Environments for User Equipment (UE) Conformance Testing".

[62]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace"

[63]	IETF RFC 5245: "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols"

[64]	IETF RFC 3830: "MIKEY: Multimedia Internet KEYing".

[65]	IETF RFC 6509 (February 2012): ''MIKEY-SAKKE: Sakai-Kasahara Key Encryption in Multimedia Internet KEYing (MIKEY)''.

[66]	IETF RFC 6043: "MIKEY-TICKET: Ticket-Based Modes of Key Distribution in Multimedia Internet KEYing (MIKEY)".

[67]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[68]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)".

[69]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[70]	ETSI ES 201 873-11: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 11: Using JSON with TTCN-3".

[71]	OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C: "RESTful Network API for Network Message Storage".

[72]	3GPP TS 38.523-3: "5GS; User Equipment (UE) conformance specification; Part 3: Protocol Test Suites".

[73]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[74]	3GPP TS 38.508-2: "5GS; User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma".

<End of modification>

<Start of modification>

## 5.2	MCX test model with SS based E-UTRA/EPC or NR/5GC implementation (MCX IPCAN test model)

### 5.2.1	MCX Client on-network test model

In contrast to the MCX EUTRA test model (specified in clause 5.1.1) or NR5GC test model (specified in clause 5.3.1) the MCX IPCAN test model shown in figure 5.2.1-1 uses an external IPCAN (black-box) implementation which is integrated in the system simulator. In general, this IPCAN implementation shall be conformant in terms of TS 36.508 [61] and TS 38.508-1 [73] but this is out of the scope for the MCX IPCAN test model as long as the IPCAN implementation obeys the commands at the system interface as specified in annex E.

Figure 5.2.1-1: MCX Client on-network test model with SS based E-UTRA/EPC or NR/5GC implementation

NOTE 1:	Whether or how the MCX IPCAN test model may use other IP-connectivity access networks is out of scope of this specification.

NOTE 2:	The SYS_IND port shown in figure 5.2.1-1 is used only in case of NR5GC to indicate the PDU session id in a system indication and to assign the QFI in the corresponding acknowledgement.

NOTE 3:	In the rest of this clause the term "PDN" is used for a PDN connectivity in EUTRA or a PDU session in NR5GC.

NOTE 4:	The major procedures for configuration and control of the system simulator are illustrated in annex F.

As described in clauses 5.4.1A and 5.4.1B of TS 37.579-1 [2] after switch on the UE may register to an internet PDN, an IMS PDN and the MCX PDN. Nevertheless, in TTCN the MCX IPCAN test model supports signalling for the MCX PDN only: The IPCAN test model does not handle any signalling for the other PDNs and especially not the SIP signalling for an IMS registration to the IMS PDN. 
 It is up to SS implementation to handle the potential registrations to an internet PDN and/or an IMS PDN.

Tables 5.2.1-1 and 5.2.1-2 show the information provided to the SS for configuration of the PDNs for EUTRA and NR5GC.

Table 5.2.1-1: PDN info for EUTRA

| PDN | Parameter | Value/Comment |
| --- | --- | --- |
| MCX | EPS Bearer ID (NOTE 2) | px_MCX_IPCAN_EpsBearerId_MCX (NOTE 1) |
|  | NW IP address | px_IPv4_Address1_NW, px_IPv6_Address1_NW (NOTE 3) |
|  | UE IP address | px_IPv4_Address1_UE, px_IPv6_Address1_UE (NOTE 3) |
| IMS | EPS Bearer ID (NOTE 2) | px_MCX_IPCAN_EpsBearerId_IMS (NOTE 1) |
|  | NW IP address | px_IPv4_Address2_NW, px_IPv6_Address2_NW |
|  | UE IP address | px_IPv4_Address2_UE, px_IPv6_Address2_UE |
| Internet | EPS Bearer ID (NOTE 2) | px_MCX_IPCAN_EpsBearerId_INTERNET (NOTE 1) |
|  | NW IP address | px_IPv4_Address3_NW, px_IPv6_Address3_NW |
|  | UE IP address | px_IPv4_Address3_UE, px_IPv6_Address3_UE |
| NOTE 1: Values depending on SS implementation NOTE 2: The DRB associated with the EPS bearer shall have the DRB ID := EPS Bearer ID – 4. NOTE 3: Even though for MCX the user plane signalling is handled in TTCN still the SS needs the IP addresses to be provided to the UE in NAS signalling. |  |  |

Table 5.2.1-2: PDU session info for NR5GC

| PDN | Parameter | Value/Comment |
| --- | --- | --- |
| MCX | APN | pc_APN_ID_MCX (NOTE 2) |
|  | NW IP address | px_IPv4_Address1_NW, px_IPv6_Address1_NW (NOTE 3) |
|  | UE IP address | px_IPv4_Address1_UE, px_IPv6_Address1_UE (NOTE 3) |
|  | QFI (NOTE 1) | 11 (NOTE 4) |
| IMS | APN | pc_APN_ID_IMS (NOTE 2) |
|  | NW IP address | px_IPv4_Address2_NW, px_IPv6_Address2_NW |
|  | UE IP address | px_IPv4_Address2_UE, px_IPv6_Address2_UE |
| Internet | APN | pc_APN_ID_Internet (NOTE 2) |
|  | NW IP address | px_IPv4_Address3_NW, px_IPv6_Address3_NW |
|  | UE IP address | px_IPv4_Address3_UE, px_IPv6_Address3_UE |
| NOTE 1: Provided in acknowledgement of system indication indicating the PDU session id. NOTE 2: Defined in TS 38.508-2 [74] Table A.4.4-2A. NOTE 3: Even though for MCX the user plane signalling is handled in TTCN still the SS needs the IP addresses to be provided to the UE in NAS signalling. NOTE 4: According to TS 38.508-1 [73] Table 4.8.2.1-10 and Table 4.8.2.3-9. |  |  |

Dedicated EPS bearers and QoS flows use packet filters according to TS 36.508 [61] clause 6.6.2 and TS 38.508-1 [73] clause 4.8.2.2 as shown in table 5.2.1-3.

Table 5.2.1-3: Packet filters for dedicated EPS bearers and QoS flows

| MC Service | Value/Comment |  |  |
| --- | --- | --- | --- |
| MCPTT | Audio | Remote port range | tsc_MCX_MediaPort_Audio .. tsc_MCX_MediaPort_Audio + 1 |
|  |  | Protocol | UDP |
|  | Media Control | Single remote port | tsc_MCX_MediaControlPortNW |
|  |  | Protocol | UDP |
| MCVideo | Audio | Remote port range | tsc_MCX_MediaPort_Audio .. tsc_MCX_MediaPort_Audio + 1 |
|  |  | Protocol | UDP |
|  | Video | Remote port range | tsc_MCX_MediaPort_Video .. tsc_MCX_MediaPort_Video + 1 |
|  |  | Protocol | UDP |
|  | Media Control | Single remote port | tsc_MCX_MediaControlPortNW |
|  |  | Protocol | UDP |
| MCData |  | Single remote port | tsc_MCData_MediaPort_MSRP |
|  |  | Protocol | TCP |

The respective packet filter is provided to the SS togther with the EPS bearer id (EUTRA) or the QoS flow (NR5GC) at call establishment. EPS bearer ids and QoS flows are as shown in table 5.2.1-4.

Table 5.2.1-4: Dedicated EPS bearer ids and QoS flows

| MC Service | EPS bearer id (EUTRA; NOTE 1) | QoS flow (NR5GC) |  |
| --- | --- | --- | --- |
|  |  | PDU session id | QFI |
| MCPTT | px_MCX_IPCAN_DedicatedEpsBearerId_MCPTT (NOTE 2) | (NOTE 3) | 12 (NOTE 4) |
| MCVideo | px_MCX_IPCAN_DedicatedEpsBearerId_MCVideo (NOTE 2) | (NOTE 3) | 13 (NOTE 4) |
| MCData | px_MCX_IPCAN_DedicatedEpsBearerId_MCData (NOTE 2) | (NOTE 3) | 14 (NOTE 4) |
| NOTE 1: The DRB associated with the EPS bearer shall have the DRB ID := EPS Bearer ID – 4. NOTE 2: Values depending on SS implementation. NOTE 3: As chosen by the UE for the MCX PDU session at registration. NOTE 4: According to TS 38.508-1 [73] Table 4.8.2.1-11 and Table 4.8.2.3-10. |  |  |  |

<End of modification>

<Start of modification>

Annex F (informative):
Procedures of the MCX IPCAN test model

# F.1	Introduction

This annex describes the major procedures for the SS based E-UTRA/EPC or NR/5GC implementation of the MCX IPCAN test model. The procedures are described in tables showing the primitives exchanged between TTCN and the system simulator at the IPCAN model specific ports SYS_REQ and SYS_IND as shown in Figure 5.2.1-1 and the related actions.

In general, the procedures correspond to the procedures specified in TS 37.579-1 [2] clause 5.4.

# F.2	Procedures

## F.2.1	Initialisation

This procedure is used to initialise the system simulator and to retrieve the RAN type (EUTRA_FDD, EUTRA_TDD, NR) to be used.

Table F.2.1-1: Initialisation

| Step | Action/Step | System Primitives at SYS_REQ port | Direction TTCN-SS | Relevant parameters / Comments |
| --- | --- | --- | --- | --- |
| 1 | Common initialisation | Init (NOTE 1) | --> | - |
| 2 | Confirmation that initialisation is completed | Confirmation (Init) (NOTE 2) | <-- | RAN type |
| 3 | Cell configuration | CellConfig (NOTE 1) | --> | cell id |
| 4 | Confirmation that cell configuration is completed | Confirmation (Other) (NOTE 2) | <-- | - |
| NOTE 1: MCX_IPCAN_SYSTEM_CTRL_REQ with Request as specified in annex E (MCX_IPCAN_SystemRequest_Type). NOTE 2: MCX_IPCAN_SYSTEM_CTRL_CNF) with Confirmation as specified in annex E (MCX_IPCAN_SystemConfirmation_Type). |  |  |  |  |

## F.2.2	Initial Registration

This procedure is used for initial registration and corresponds to the procedures of TS 37.579-1 [2] clause 5.4.2.

Table F.2.2-1: Initial Registration

| Step | Action/Step | System Primitives at SYS_REQ/IND port | Direction TTCN-SS | Relevant parameters / Comments |
| --- | --- | --- | --- | --- |
| 1 | Configuration of the cell power | CellPower (NOTE 1) | --> | cell id, absolute cell power |
| - | EXCEPTION: IF the RAN type is EUTRA THEN step 2a1 is perfomed | - | - | - |
| 2a1 | Start of IP handling (Address, IMS, HTTP servers) for MCX | - | - | cell id, MCX default EPS bearer id |
| 3 | Switch on the UE | - | - | - |
| 4 | Start of initial registration | EUTRA: InitialRegistration4G (NOTE 1) | --> | list of PDN infos: PDN type, EPS bearer id, IP addresses |
|  |  | NR5GC: InitialRegistration5G (NOTE 1) | --> | list of PDU session infos: PDN type, APN, IP addresses |
| - | EXCEPTION: Depending on the UE configuration, IF the UE does not register to MCX automatically at initial registration THEN steps 5a1-5a5 are performed | - | - | - |
| 5a1 | Confirmation that the initial registration is completed | Confirmation (Other) (NOTE 2) | <-- | (NOTE 5) |
| 5a2 | Request of RRC connection release | RRCConnectionRelease (NOTE 1) | --> | - |
| 5a3 | Confirmation for RRC connection release | Confirmation (Other) (NOTE 2) | <-- |  |
| 5a4 | Make the UE (MCX client) request service authorisation/configuration | - | - | - |
| 5a5 | Start of explicit MCX registration | EUTRA: ExplicitMCXRegistration4G (NOTE 1) | --> | MCX PDN info: PDN type, EPS bearer id, IP addresses |
|  |  | NR5GC: ExplicitMCXRegistration5G (NOTE 1) | --> | MCX PDU session info: PDN type, APN, IP addresses |
| - | EXCEPTION: IF the RAN type is NR/5GC THEN Steps 6a1-6a3 are performed (NOTE 6) | - | - | - |
| 6a1 | Indication of PDU SESSION ESTABLISHMENT REQUEST | PduSessionEstReqInd (NOTE 3) | <-- | PDU session id |
| 6a2 | Start of IP handling (Address, IMS, HTTP servers) for MCX | - | - | cell id, MCX default QoS flow (PDU session id, QFI) |
| 6a3 | Acknowledgement of PDU session id and assignment of the QFI to be used for MCX default QoS flow | PduSessionEstReqAck (NOTE 4) | --> | QFI |
| 7 | HTTP and SIP signalling according to TS 37.579-1 [2] Table 5.3.2.2.1-1 | - | - | - |
| 8 | Request of RRC connection release | RRCConnectionRelease (NOTE 1) | --> | - |
| 9 | Confirmation for RRC connection release | Confirmation (Other) (NOTE 2) | <-- | - |
| NOTE 1: MCX_IPCAN_SYSTEM_CTRL_REQ with Request as specified in annex E (MCX_IPCAN_SystemRequest_Type). NOTE 2: MCX_IPCAN_SYSTEM_CTRL_CNF) with Confirmation as specified in annex E (MCX_IPCAN_SystemConfirmation_Type). NOTE 3: MCX_IPCAN_SYSTEM_IND with Indication as specified in annex E (MCX_IPCAN_SystemIndication_Type). NOTE 4: MCX_IPCAN_SYSTEM_ACK with Acknowledgement as specified in annex E (MCX_IPCAN_SystemAcknowledgement_Type) NOTE 5: A confirmation is requested by the InitialRegistration at step 4 if the UE does not register to MCX automatically. NOTE 6: Basically, the SS should indicate the PDU session id provided by the UE in the PDU SESSION ESTABLISHMENT REQUEST (step 6a1) and wait for the acknowledgement before sending the PDU SESSION ESTABLISHMENT ACCEPT. Nevertheless, it is up to SS implementation how to know the PDU session id, but the SS is expected to indicate the PDU session id early enough so that the HTTP server is in place when the initial HTTP request from the UE needs to be handled in TTCN. |  |  |  |  |

## F.2.3	MO RRC connection setup

This procedure is used for MO RRC connection setup and corresponds to the procedures of TS 37.579-1 [2] clause 5.4.3: Steps 2-8 of Table 5.4.3.2.3-1 for EUTRA and steps 1-7 of Table 5.4.3.3.3-1 for NR5GC, with m=1 at step 7 of Table 5.4.3.2.3-1 and at step 6 of Table 5.4.3.3.3-1 in case of a pre-established session and m=0 otherwise.

Table F.2.3-1: MO RRC connection setup

| Step | Action/Step | System Primitives at SYS_REQ port | Direction TTCN-SS | Relevant parameters / Comments |
| --- | --- | --- | --- | --- |
| 1 | Request of MO RRC connection setup | EUTRA: RRCConnectionSetup4G_MO (NOTE 1) | --> | list of dedicated EPS bearer ids (NOTE 2) |
|  |  | NR5GC: RRCConnectionSetup5G_MO (NOTE 1) | --> | list of dedicated QoS flows (PDU session id, QFI) (NOTE 2) |
| NOTE 1: MCX_IPCAN_SYSTEM_CTRL_REQ with Request as specified in annex E (MCX_IPCAN_SystemRequest_Type). NOTE 2: In case of pre-established session, the RRC connection setup request provides a list of dedicated EPS bearer ids (EUTRA/EPC) or dedicated QoS flows (NR/5GC) with a single entry. When there is no pre-established session, the list is empty. |  |  |  |  |

## F.2.4	MT RRC connection setup

This procedure is used for MT RRC connection setup and corresponds to the procedures of TS 37.579-1 [2] clause 5.4.4: Steps 1-8 of Table 5.4.4.2.3-1 for EUTRA and steps 1-8 of Table 5.4.4.3.3-1 for NR5GC, with m=1 at step 7 of Table 5.4.4.2.3-1 and at step 7 of Table 5.4.4.3.3-1 in case of a pre-established session and m=0 otherwise.

Table F.2.4-1: MT RRC connection setup

| Step | Action/Step | System Primitives at SYS_REQ port | Direction TTCN-SS | Relevant parameters / Comments |
| --- | --- | --- | --- | --- |
| 1 | Request of MT RRC connection setup | EUTRA: RRCConnectionSetup4G_MT (NOTE 1) | --> | list of dedicated EPS bearer ids (NOTE 2) |
|  |  | NR5GC: RRCConnectionSetup5G_MT (NOTE 1) | --> | list of dedicated QoS flows (PDU session id, QFI) (NOTE 2) |
| 2 | Confirmation for RRC connection establishment | Confirmation (Other) (NOTE 3) | <-- | - |
| NOTE 1: MCX_IPCAN_SYSTEM_CTRL_REQ with Request as specified in annex E (MCX_IPCAN_SystemRequest_Type). NOTE 2: In case of pre-established session, the RRC connection setup request provides a list of dedicated EPS bearer ids (EUTRA/EPC) or dedicated QoS flows (NR/5GC) with a single entry. When there is no pre-established session, the list is empty. NOTE 3: MCX_IPCAN_SYSTEM_CTRL_CNF) with Confirmation as specified in annex E (MCX_IPCAN_SystemConfirmation_Type). |  |  |  |  |

## F.2.5	MO communication establishment

This procedure is used for MO communication establishment and corresponds to the procedures of TS 37.579-1 [2] clause 5.4.3: Steps 2-16a3 of Table 5.4.3.2.3-1 for EUTRA and steps 1-8a3 of Table 5.4.3.3.3-1 for NR5GC, with m=0 at step 7 of Table 5.4.3.2.3-1 and at step 6 of Table 5.4.3.3.3-1.

Table F.2.5-1: MO communication establishment

| Step | Action/Step | System Primitives at SYS_REQ port | Direction TTCN-SS | Relevant parameters / Comments |
| --- | --- | --- | --- | --- |
| 1 | MO RRC connection setup as specified in Table F.2.3-1 | - | - | - |
| 2 | SIP signalling: MO INVITE | - | - | - |
| - | EXCEPTION: In parallel to steps 3a1-5 the SIP signalling for completion of the MO call establishment happens (NOTE 3) | - | - | - |
| - | EXCEPTION: Steps 3a1-3b1 depend on the kind of MCX service. | - | - | - |
| 3a1 | IF the MCX service is MCPTT or MCVideo THEN RTP/RTCP loopback gets configured |  |  | cell id and DRB id (EUTRA) or QoS flow (PDU session id, QFI; NR5GC) |
| 3b1 | ELSE IF the MCX service is MCData THEN MSRP server gets started in TTCN | - | - | cell id and DRB id (EUTRA) or QoS flow (PDU session id, QFI; NR5GC) |
| 4 | Activate/Add dedicated resources | EUTRA: ActivateDedicatedEpsBearer (NOTE 1) | --> | dedicated EPS bearer id, packet filters |
|  |  | NR5GC: AddQosFlow (NOTE 1) | --> | dedicated QoS flow (PDU session id, QFI), packet filters |
| 5 | Confirmation for establishment of dedicated resources | Confirmation (Other) (NOTE 2) | <-- | - |
| NOTE 1: MCX_IPCAN_SYSTEM_CTRL_REQ with Request as specified in annex E (MCX_IPCAN_SystemRequest_Type). NOTE 2: MCX_IPCAN_SYSTEM_CTRL_CNF) with Confirmation as specified in annex E (MCX_IPCAN_SystemConfirmation_Type). NOTE 3: The UE is expected to send the SIP ACK not before dedicated resources are established. |  |  |  |  |

## F.2.6	MT communication establishment

This procedure is used for MT communication establishment and corresponds to the procedures of TS 37.579-1 [2] clause 5.4.4: Steps 1-17a3 of Table 5.4.4.2.3-1 for EUTRA and steps 1-9a3 of Table 5.4.4.3.3-1 for NR5GC, with m=0 at step 7 of Table 5.4.4.2.3-1 and at step 7 of Table 5.4.4.3.3-1.

Table F.2.6-1: MT communication establishment

| Step | Action/Step | System Primitives at SYS_REQ port | Direction TTCN-SS | Relevant parameters / Comments |
| --- | --- | --- | --- | --- |
| 1 | MT RRC connection setup as specified in Table F.2.4-1 | - | - | - |
| - | EXCEPTION: In parallel to steps 3a1-4 the SIP signalling for MT call establishment happens | - | - | - |
| - | EXCEPTION: Steps 3a1-3b1 depend on the kind of MCX service. | - | - | - |
| 3a1 | IF the MCX service is MCPTT or MCVideo THEN RTP/RTCP loopback gets configured |  |  | cell id and DRB id (EUTRA) or QoS flow (PDU session id, QFI; NR5GC) |
| 3b1 | ELSE IF the MCX service is MCData THEN MSRP server gets started in TTCN | - | - | cell id and DRB id (EUTRA) or QoS flow (PDU session id, QFI; NR5GC) |
| 4 | Activate/Add dedicated resources | EUTRA: ActivateDedicatedEpsBearer (NOTE 1) | --> | dedicated EPS bearer id, packet filters |
|  |  | NR5GC: AddQosFlow (NOTE 1) | --> | dedicated QoS flow (PDU session id, QFI), packet filters |
| 5 | Confirmation for establishment of dedicated resources | Confirmation (Other) (NOTE 2) | <-- | - |
| NOTE 1: MCX_IPCAN_SYSTEM_CTRL_REQ with Request as specified in annex E (MCX_IPCAN_SystemRequest_Type). NOTE 2: MCX_IPCAN_SYSTEM_CTRL_CNF) with Confirmation as specified in annex E (MCX_IPCAN_SystemConfirmation_Type). |  |  |  |  |

## F.2.7	Connection release

This procedure is used for communication release and corresponds to the procedures of TS 37.579-1 [2] clause 5.4.14.

Table F.2.7-1: Connection release

| Step | Action/Step | System Primitives at SYS_REQ port | Direction TTCN-SS | Relevant parameters / Comments |
| --- | --- | --- | --- | --- |
| - | EXCEPTION: IF there is a dedicated bearer or QoS flow AND no pre-established session is established OR a pre-established session shall be released THEN steps 1a1-1a2 are performed. | - | - | - |
| 1a1 | Deactivate/Remove dedicated resources | EUTRA: DeactivateDedicatedEpsBearer (NOTE 1) | --> | dedicated EPS bearer id |
|  |  | NR5GC: RemoveQosFlow (NOTE 1) | --> | dedicated QoS flow (PDU session id, QFI |
| 1a2 | Confirmation for removal of dedicated resources | Confirmation (Other) (NOTE 2) | <-- | - |
| 2 | Request of RRC connection release | RRCConnectionRelease (NOTE 1) | --> | - |
| 3 | Confirmation for RRC connection release | Confirmation (Other) (NOTE 2) | <-- | - |
| NOTE 1: MCX_IPCAN_SYSTEM_CTRL_REQ with Request as specified in annex E (MCX_IPCAN_SystemRequest_Type). NOTE 2: MCX_IPCAN_SYSTEM_CTRL_CNF) with Confirmation as specified in annex E (MCX_IPCAN_SystemConfirmation_Type). |  |  |  |  |

<End of modification>