3GPP TSG-RAN5 Meeting #107	R5-252757

# Malta, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-7 | CR | 0005 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to test case 6.7.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, FirstNet, Element, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When this test case was added, in Rel-16, IP connectivity in was besed on GRE (Generic Routing Encapsulation) for the IP tunnel between two MCData clients. For Rel-17 the core requirements were updated to use GRE-in-UDP Encapsulation. As this test case was never implemented in TTCN for Rel-16, it can be updated to Rel-17. See discussion paper R5-246421. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test case updated to Rel-17 requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.579-4 CR 002 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.7.2	On-network / IP Connectivity / Client Terminated (CT)

6.7.2.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User receives a SIP INVITE to initiate a IP Connectivity session }

then { UE (MCDATA Client) responds by sending a SIP 200 (OK) message and responds to any received ICMP ECHO request sent over the IP Connectivity session }

}

(2)

with { UE (MCDATA Client) being in a IP Connectivity session initiated by the SS (MCDATA server) }

ensure that {

when { UE (MCDATA Client) receives a SIP BYE message }

then { UE (MCDATA Client) responds by sending a SIP 200 (OK) message }

}

6.7.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clause 20.2.2 and 20.2.0a, TS 24.582, clause 13.1.3. Unless otherwise stated, these are Rel-17 requirements.

[TS 24.282, clause 20.2.2]

Upon receipt of a SIP INVITE request for IP Connectivity session for terminating MCData client"request, the MCData client shall follow the procedures for termination of multimedia sessions in the IM CN subsystem as specified in 3GPP TS 24.229 [5] with the clarifications below.

The MCData client:

1)	may reject the SIP INVITE request if either of the following conditions are met:

a)	MCData client does not have enough resources to handle the IP Connectivity session; or

b)	any other reason outside the scope of this specification;

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCData function either with appropriate reject code as specified in 3GPP TS 24.229 [5] and warning texts as specified in clause 4.9 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorised to restrict the reason for failure and skip the rest of the steps of this clause;

3)	may provide to the MCData user or user application the MCData ID of the inviting MCData user;

3A)	may display to the MCData user the functional alias of the inviting MCData user, if provided;

3B)	may display to the MCData user the functional alias used in the initial communication request, if provided;

4)	shall accept the SIP INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

5)	shall include the option tag "timer" in a Require header field of the SIP 200 (OK) response;

6)	shall include the Session-Expires header field in the SIP 200 (OK) response and start the SIP session timer according to IETF RFC 4028 [38]. The "refresher" parameter in the Session-Expires header field shall be set to "uas";

7)	shall include the g.3gpp.mcdata.ipconn media feature tag in the Contact header field of the SIP 200 (OK) response;

8)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn" in the Contact header field of the SIP 200 (OK) response;

9)	shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP INVITE request according to 3GPP TS 24.229 [5] with the clarifications given in clause 20.2.0b; and

10)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of an SIP ACK message to the sent SIP 200 (OK) message, the MCData client:

1)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] clause 13.1.3.

[TS 24.282, clause 20.2.0b]

When the MCData client receives an initial SDP offer for a MCData including an attribute for IP Connectivity, the MCData client shall process the SDP offer and shall compose an SDP answer.

When composing an SDP answer, the MCData client:

1)	shall accept the MCData media stream in the SDP offer;

2)	shall set the IP address of the MCData client for the accepted MCData media stream; and

NOTE:	The MC service operator policy determines if the MCData client can use an already assigned IP address or can request a new IP address following the procedures defined in 3GPP TS 24.301 [43].

3)	shall include an "m=application" media-level section for the accepted MCData media stream consisting of:

a)	the port number selected for the media plane as specified in 3GPP TS 24.582 [15] clause 13.5; and

b)	the 'fmtp' attribute as specified in 3GPP TS 24.582 [15] clause 13.6.

[TS 24.582, clause 13.1.3]

The successful outcome of the procedure 20.2.2 in 3GPP TS 24.282 [8] shall be the trigger to start the establishment of the IP tunnel. The IP tunnel is based on GRE-in-UDP Encapsulation as specified in IETF RFC 8086 [23], and as specified in clause 13.4.

The MCData client shall perform encapsulation and decapsulation in accordance with clause 13.4.

The MCData client acts as an IP relay for IP traffic between the IP tunnel and the IP application. Once the IP tunnel is established, the IP applications can exchange IP data. The client that receives IP packets from the IP tunnel shall perform de-capsulation from the tunnelling protocol, removing the UDP header and the GRE header from the received packet, before passing the IP data to the IP application. When the terminating MCData client receives an IP packet from the IP application, it shall perform encapsulation to the tunnelling protocol adding a GRE header and a UDP header to the IP packet, and send the outgoing UDP traffic to the IP address and port present in the SDP offer.

6.7.2.3	Test description

6.7.2.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.7.2.3.2	Test procedure sequence

Table 6.7.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-5 | Check: Does the UE (MCData client) correctly perform steps 1 through 5 of procedure 'CT MCData Call Establishment' as described in TS 37.579-1 [2] Table 5.3C.3.3-1? | - | - | 1,2 | P |
| - | EXCEPTION: The communication described in in steps 6 and 7 is transmitted over an IP connection that has been established based on GRE-in-UDP Encapsulation. | - | - | - | - |
| 6 | The SS (MCX Server) sends a ICMP ECHO request via the GRE-in-UDP tunnel. | - | - | - | - |
| 7 | Check: Does the UE (MCData client) provide an ICMP ECHO reply via the GRE-in-UDP tunnel? | - | - | 1 | P |
| 8 | Check: Does the UE (MCData client) correctly perform procedure 'CT MCData call release' as described in TS 37.579-1 [2] Table 5.3C.7.3-1? (NOTE 1) | - | - | 2 | P |
| 9 | The procedure 'MCX communication release' as described in TS 37.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| NOTE 1: The procedure does not release the RRC connection. |  |  |  |  |  |

6.7.2.3.3	Specific message contents

Table 6.7.2.3.3-1: SIP INVITE from the SS (step 2, Table 6.7.2.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3C.3.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.5.2-1, condition MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact |  |  |  |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN |  |  |  |
| port | protected server port of UE | as assigned during registration |  |  |
| feature-param | "+g.3gpp.mcdata.ipconn" |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn" |  |  |  |
| Accept-Contact |  |  |  |  |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcdata.ipconn" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP message as described in Table 6.7.2.3.3-2 |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.7.2.3.3-3 |  |  |  |

Table 6.7.2.3.3-2: SDP for SIP INVITE (Table 6.7.2.3.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_SDS, SDP_OFFER, MCD_1to1, SDS_SESSION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Media description[1] |  | Media description for data |  |  |
| media description |  | m= line media = application |  |  |
| media | “application” |  | TS 24.582 [32] clause 13.5 |  |
| port | port number assigned by the SS |  |  |  |
| proto | “udp” |  | TS 24.582 [32] clause 13.5 |  |
| fmt | “MCDATA” |  | TS 24.582 [32] clause 13.5 |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  | TS 24.582 [32] clause 13.5 |  |
| fmtp-attr-ipconn | “MCDATA” |  | TS 24.582 [32] clause 13.5 |  |
| attr-param | "mcdata-ipconn-s-port=<port number assigned by the SS>” | UDP source port of the GRE-in-UDP tunnel | TS 24.582 [32] clause 13.5 |  |

Table 6.7.2.3.3-3: MCData-Info (Table 6.7.2.3.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-3, condition MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-ipconn" |  |  |  |

Table 6.7.2.3.3-4: SIP 200 (OK) from the UE (step 4, Table 6.7.2.3.2-1;
step 4, TS 37.579-1 [2] Table 5.3C.3.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.17.1.1-1, condition INVITE-RSP, MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact |  |  |  |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCData_SessionId |  |  |  |
| port | not present |  |  |  |
| feature-param | "+g.3gpp.mcdata.ipconn" |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.ipconn" |  |  |  |
| feature-param | "isfocus" |  |  |  |
| Content-Type |  |  |  |  |
| value | "application/sdp" |  |  |  |
| Message-body |  |  |  |  |
| SDP message | As described in Table 6.7.2.3.3-5 |  |  |  |

Table 6.7.2.3.3-5: SDP for SIP 200 (OK) (Table 6.7.2.3.3-4)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_SDS, SDP_ANSWER, SDS_SESSION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Media description[1] |  | Media description for data |  |  |
| media description |  | m= line media = application |  |  |
| port | same number as indicated by the <attr-param> parameter in Table 6.7.2.3.3-2 |  |  |  |
| proto | “udp” |  | TS 24.582 [32] clause 13.5 |  |
| fmt | “MCDATA” |  | TS 24.582 [32] clause 13.5 |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp | "fmtp" |  | TS 24.582 [32] clause 13.5 |  |
| fmtp-attr-ipconn | “MCDATA” |  | TS 24.582 [32] clause 13.5 |  |
| attr-param | "mcdata-ipconn-s-port=<any allowed value>” | UDP source port of the GRE-in-UDP tunnel | TS 24.582 [32] clause 13.5 |  |

Table 6.7.2.3.3-6: GRE encapsulated user data packet (step 6, Table 6.7.2.3.2-1)

| Derivation Path: 24.582 [32], Table 13.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GRE header |  |  |  |
| C | 0 |  |  |
| Reserved | 0 |  |  |
| K | 0 |  |  |
| S | 0 |  |  |
| Reserved | 0000 |  |  |
| Reserved | 00000 |  |  |
| Ver | 000 |  |  |
| Protocol Type | ‘0800’H |  | IPv4 |
| Protocol Type | ‘86DD’H |  | IPv6 |
| Payload packet |  |  |  |
| Payload packet | ICMP ECHO | ICMP ECHO reply |  |