3GPP TSG RAN meeting #108										RP-251175

Prague, Czech Republic, June 9-13, 2025

## Status Report to TSG

Agenda item:			10.4.4

| WI / SI Name | Introduction of IoT-NTN TDD mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | IoT_NTN_TDD |  |  |  |  |
| Unique ID | 1050123 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-243293 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: N/A | Core part: Overall: 76% RAN1: 100% RAN2: 75% RAN4: 55% | Performance Part: Overall: 0% RAN4: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Andjela Ilic-Savoia |
|  | Company | Iridium Satellite LLC |
|  | Email | andjela.savoia@iridium.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

2.1.1 	Agreements during RAN1#120-bis

#### RAN1 #120-bis List of Observations and Agreements

Agreement

Confirm the following working assumption:

- Uplink transmission gaps do not apply in NB-IoT NTN TDD.

Agreement

NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s). The unit of postponement is one PRU.

- NOTE: “U NB-IoT subframes” are subframes contained within the set of U=8 usable consecutive uplink subframes in the TDD structure.

NPRACH periodicities of 40ms and 80ms are not supported in NB-IoT NTN TDD mode

- Instead, periodicities of 90ms and 180ms are supported

Agreement

The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be 50ms at the ULSRP.

- NOTE: This roughly corresponds to the largest separation between DL and UL Iridium slot pairs.

- Note: Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA.

Agreement

For the issue of handling DL gaps in NB-IoT NTN TDD mode, down-select among the following options:

- Option 1: DL gaps do not apply for NB-IoT NTN TDD mode.

- Option 2: DL gaps apply based on current specifications.

- Option 3: DL gaps apply, where new periodicity(ies) are introduced (either implicity e.g. the periodicity is scaled by a fixed factor F, or explicitly)

Agreement

For the issue of handling NPDCCH postponing, down-select among the following options:

- Option 1: No new periodicities are introduced for NPDCCH search space monitoring

  - FFS: Changes in postponing / overlapping rules, if any

  - FFS: Introduction of constraints on the configuration of search space parameters (e.g. Rmax, G…), if any

- Option 2: New periodicities are introduced for NPDCCH search space monitoring

  - FFS: Details of the new periodicity, e.g. the existing periodicity is scaled by a fixed factor F, explicit signalling of the new periodicity, reusing the NB-IoT TN TDD periodicities, etc.

Working assumption

DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) is not present in a NB-IoT NTN TDD cell.

- NOTE1: All D NB-IoT subframes (except those carrying NPSS/NSSS/SIB1-NB/NPBCH) are NB-IoT DL subframes.

- NOTE2: “D NB-IoT subframes” are subframes contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

Agreement

SIB1-NB is dropped in non-D NB-IoT subframes

- NOTE 1: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

- NOTE 2: RAN1 observes that some combinations of {TBS, number of repetitions, additional SIB1 repetitions}, and depending on aspects such as minimum elevation angle or antenna gain, may not meet the link budget. It is up to the network to properly configure such values to ensure the link budget is met. No specification impact.

Agreement

For GNSS measurement gap, RAN1 to further discuss at least the following options:

- Option 1: No enhancement on the Rel-18 timeline for the start of GNSS gap

- Option 2: The start of the GNSS measurement gap starts at a non-D/non-U subframe

- Option 3: GNSS measurement gap does not apply for NB-IoT NTN TDD

Working assumption

For precompensation, from RAN1 perspective:

- The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.

  - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed.

- FFS: whether spec impact is in RAN1, RAN4 or both

Inform RAN4 of the WA above and ask if there are any issues with the above WA.
The LS to RAN4 in R1-2503142 is agreed.

RAN1 #120-bis UE Features for IOT NTN TDD
Agreement:

Introduce the following FG for IoT-NTN TDD mode

| Support of IoT-NTN TDD mode | 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. [FFS Whether/how to capture components in Rel-17 2-1b, in case the FG is not defined to be the prerequisite one.] [FFS other component(s)/FG(s)] |
| --- | --- |

#### 2.1.2  Agreements during RAN1#121

RAN1 #121 RRC parameters for IOT NTN TDD (new/changed)

#### RAN1 #121 List of Observations and Agreements

Agreement

For GNSS measurement gap:

- GNSS measurement gap can be applied in NB-IoT NTN TDD

- RAN1 does not specify any enhancement on the Rel-18 timeline for the start of GNSS gap.

Conclusion

Additional SIB1-NB transmissions are supported based on current specifications without the need of indicating DL bitmap.

- NOTE: RAN1 assumes that all NTN-IoT TDD UEs can interpret additionalTransmissionSIB1-r15 in MIB

Agreement

Confirm the working assumption of RAN1#120b with the following modifications:

DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) does not apply to a NB-IoT NTN TDD cell.

- NOTE1: All D NB-IoT subframes (except those carrying NPSS/NSSS/SIB1-NB/NPBCH) are NB-IoT DL subframes.

- NOTE2: “D NB-IoT subframes” are subframes contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

- Up to RAN2 to decide whether DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) is constrained to not being present in a NB-IoT NTN TDD cell, or it can be configured with values all set to ‘1’.

Agreement

Confirm the following working assumption with modifications:

For precompensation, from RAN1 perspective:

- The UE may adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.

- The UE may adjust its time/frequency pre-compensation at the beginning of an NPUSCH/NPRACH transmission (same behavior as Rel-18)

  - Segmented precompensation is not supported.

  - It is not supported to perform precompensation within the set of 8 consecutive uplink subframes other than at the beginning of an NPUSCH/NPRACH transmission

- FFS: whether spec impact is in RAN1, RAN4 or both.

NOTE: RAN1 may revisit this agreement if RAN4 reply LS shows concerns, including concerns on meeting the requirements without segmented precompensation

Agreement

Confirm the following WA with the following modification:

For NPDCCH monitoring, new periodicities are introduced for NPDCCH monitoring by scaling the periodicity by a scaling factor F = 11.25 supporting values of G={11.25*4, 11.25*8} instead of G={4,8}

Agreement

The TP below is endorsed for TS 36.300 (for LS Reply to RAN2)

<Subclause 3.1>

Anchor carrier: in NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD and IoT-NTN TDD or NPSS/NSSS/NPBCH for TDD are transmitted.

<Subclause 23.21>

In this release of the specification, NTN is only applicable to FDD and IoT-NTN TDD system.

<Subclause 5.0>

Downlink and uplink transmissions are organized into radio frames with 10 ms duration. Three radio frame structures are supported:

- Type 1, applicable to FDD

- Type 2, applicable to TDD;

…

For IoT-NTN TDD mode, Frame Structure Type-1 is used where uplink and downlink transmissions are separated in the time domain and constitute of set of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (GP). This pattern is repeated every N radio frames. IoT-NTN TDD mode is applicable for the IoT-NTN TDD band (1616-1626.5 MHz) specified in [36.102].

R1-2504882	TP for 36.300 for IOT NTN TDD mode	Qualcomm

R1-2504883	TP for 36.300 for IOT NTN TDD mode	RAN1, Qualcomm

Agreement

The draft LS in R1-2504882 is endorsed.

Final LS in R1-2504883.

Agreement

For the issue of handling DL gaps in NB-IoT NTN TDD mode:

- DL gaps apply based on current specifications, with the following modification:

- dl-GapPeriodicity with a value of “sf1024” is supported instead of the value of “sf64”

RAN1 #121 UE Features for IOT NTN TDD

Agreement:

Adopt the following two updates for FG 2-1:

- For “Mandatory/optional” column, adopt “Optional without capability signalling”

- For “Note” column, add “Note: UE supporting band 249 must support this FG”

Agreement:

- Confirm “Rel-17 2-1b” as a prerequisite FG for 2-1.

- Remove “[FFS Whether/how to capture components in Rel-17 2-1b, in case the FG is not defined to be the prerequisite one.]” from FG2-1 component

- Add a note: “Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG must support IoT NTN FDD band”

Agreement:

On whether/how to define “Type” for FG 2-1, support Alt-3 from the following:

- Alt-1: Per UE

- Alt-2: Per band

- Alt-3: n/a

Agreement:

Update Components for FG 2-1 as follows.

- Add “SIB1-NB” in component 5

- Add a new component “Support of NPRACH periodicities of 90ms and 180ms, instead of 40ms and 80ms”

- Remove “[FFS other component(s)/FG(s)]”

[Rapporteour’s note: UE FG’s above, as per:  R1-2207924]

#### 2.1.3 Remaining open issues

#### [Rapporteour’s note:

#### None, RAN1 is 100% completed.

#### CRs for 36.213 and 36.211 in the progress]

## 2.2	RAN2

#### 2.2.1 Agreements during RAN2#129

Agreements:

1.	Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe.

2.	In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell.

3.	Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes

4.	When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe

5.	When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

6.	For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

#### 2.2.2 Agreements during RAN2#130

CR Rapporteurs:

- 36.300: Iridium

- 36.304: Xiaomi

- 36.306: Samsung

- 36.321: Toyota

- 36.331: Huawei

#### Agreements:

1. The SI-message transmission can be postponed to the next valid D frame within the SI-Window

2. It is up to NW implementation to avoid SI-window overlap

3. SI repetitions will not overlap (in case of collision the subsequent SI repetition is postponed)

4. In IoT-NTN TDD mode, the RA-RNTI should be calculated based on the SFN of the first radio frame in which the Random-Access Preamble is transmitted (i.e. no spec change)

5. For the timer of ra-ResponseWindowSize and mac-ContentionResolutionTimer, the absolute value limitation for FDD (i.e., 10.24s) is used for IoT NTN TDD.

6. In IoT-NTN TDD mode the same formula as for RA-RNTI calculation for FDD is reused

7. No extension is needed on the value range of timer in unit of ms or s for IoT NTN TDD

8. The remaining paging repetitions falling on the invalid DL SFNs are postponed to the next valid DL SFNs.

9. It is up to network to configure the gap between two POs (i.e., parameter NB) to be sufficiently long such that it includes enough number of valid DL subframes for NumRepetitionPaging-r13 (no spec impact)

10. Introduce the following definition for IoT-NTN TDD mode in the impacted RAN2 specifications: IoT-NTN TDD mode: allows use of NB-IoT channels with TDD mode for NTN with fixed values of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (can revisit this based on the TP being prepared by RAN1)

11. In Rel19, RAN2 will not work on any specific enhancements to ensure that the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode

#### 2.2.3 Remaining open issues

- On neighbour cells

  - Weather to introduce a new list of neighbor cells operating in TDD mode for measurements and cell reselection.

- On SC-PTM reception

  - Wheather the UE drops the non-DL subframes for SC-PTM reception

- On drx-Cycle value

  - Wheather RAN2 to introduce a drx-Cycle value which is aligned with the 90 ms TDD frame periodicity

- On SIB1 scheduling mechanism

  - When operating in IoT-NTN TDD mode, wheather there is a need to update the SIB1 scheduling mechanism in the existing specifications

## 2.3	RAN3

## 2.4	RAN4

2.4.1 	Agreements during RAN4#114-bis UE/SAN RF

Topic #1: General part and band definition for [IoT_NTN_TDD]

Sub-topic 1-2: Other general topics

Issue 1-2-1: Remaining issues from other bands

Agreement: Remaining issues from 254 band introduction will remain out of scope of the IoT_NTN_TDD WI.

Issue 1-2-2: Operating bands for category NB1 and NB2

Agreement: Update the following phrase:

## 5.2B Operating bands for category NB1 and NB2 Category NB1 and NB2 UE are designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1. Category NB1 and NB2 UE operate in HD-FDD duplex mode. Category NB1 and NB2 UE operate in TDD duplex mode for 249 band.

Topic #2: UE Requirements for [IoT_NTN_TDD]

Sub-topic 2-1: UE general core requirements

Issue 2-1-1: Discussion on UE Power Class (PC) for NB-IoT TDD NTN in TS 36.102

Agreement: Use PC3 and PC5 as work assumptions. The NB-IoT TDD NTN WI shall not consider the definition of a new power class.

Note: the intention of NB-IoT TDD NTN WI was to re-use current legacy NB-IoT devices.

Issue 2-1-2: Discussion on UE AMPR for NB-IoT TDD NTN in TS 36.102

Agreement: RAN4 to further discuss what are the regulatory documents applicable for the emission requirements of 249.

Agreement: RAN4 to consider at least the following references: ETSI EN 301 441, ITU-R M.1343-1, other references not precluded.

Agreement: RAN4 should discuss if regulatory requirements are defined per band or per channel, and the simulation/measurement assumption for NB-IoT NTN channel positioning with respect to the band.

Agreement: As a starting point, RAN4 can consider for discussions:

  - Option 1: Since similar frequency range between 249 and 254, A-MPR requirement for 249 shall follow simulation results in TS 36.764 and A-MPR requirements (to be defined) for 254 in TS 36.102.

1a/ As starting point, for 1-tone allocation (priority 1) – see TR 36.764:

Table 6.2.3.2-3: 1-Tone A-MPR for NS_04N power class 3

| Modulation | π/4 QPSK |  |  |
| --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/3/4/5/6/7/8/9 |
| A-MPR | ≤ 0.5 dB | 0 dB | 0 dB |

Table 6.2.3.2-4: 1-Tone A-MPR for NS_05N power class 3

| Modulation | π/4 QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/9 | 3/4/5/6/7/8 |
| A-MPR | ≤ 1.5 dB | ≤ 1.0 dB | 0 dB | 0 dB |

1b/ As starting point, for multiple-tone allocation (priority 2) – see TR 36.764:

Table 6.2.3.2-1: A-MPR for NS_04N power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 0.7 dB | 0.2 dB |  | ≤ 0.7 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | 0 dB |  | 0 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

Table 6.2.3.2-2: A-MPR for NS_05N power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 1.5 dB | 0.5 dB |  | ≤ 1.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | ≤ 0.7 dB |  | ≤ 0.7 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

1c/ Other options not precluded.

  - Option 2: Re-evaluate A-MPR for 249 based on PC3/PC5 (priority for PC3) assumption using at least the following references: ETSI EN 301 441, ITU-R M.1343-1

Issue 2-1-3: Discussion on frequency error requirement for NB-IoT TDD NTN in TS 36.102

Agreement:

  - Do not change the current frequency error requirement from TS 36.102 for NTN TDD NB-IoT:

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

  - Add a note to the following text [Clause 6.4B.1of TS36.102] as follows:

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts”.

Sub-topic 2-2: Work Split for UE specification TS 36.102

Issue 2-2-1: Work Split for NB-IoT TDD NTN in TS 36.102

| Clause/Requirement | Company |
| --- | --- |
| Band definition (Clause 5.2) | Iridium |
| REFSENS update (Clause 7.3B) | THALES |
| A-MPR |  |
| Frequency error requirement | THALES |
| Blocking update (Clause 7.6B) | ZTE |
| Other topics (if any) e.g. [EVM Annex] |  |

Topic #3: SAN Requirements for [IoT_NTN_TDD]

Sub-topic 3-1: TDD specific requirements for NTN NB-IoT in TS 36.108

Issue 3-1-1: Add ON/OFF/transient definitions in TS 36.108

Agreement:

- Add the following definitions (currently not available in TS 36.108):

  - Transmitter ON period: time period during which the SAN transmitter is allowed to transmit data and/or reference symbols.

  - Transmitter OFF period: time period during which the SAN transmitter is not allowed to transmit.

  - Transmitter transient period: time period during which the transmitter is changing from the OFF period to the ON period or vice versa.

- Update relevant clause “transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement for OTA Transmit ON/OFF power.

Issue 3-1-2: OTA Transmit ON/OFF power in TS 36.108

Agreement:

- Remove Clause 9.5 (OTA transmit ON/OFF power) previously agreed at RAN#114 (while keeping Clause 6.4):

## OTA transmit ON/OFF power

The requirement is not applicable in this version of the specification.

- Update relevant clause “transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement for OTA Transmit ON/OFF power.

Issue 3-1-3: Requirement set applicability TS 36.108

Agreement: RAN4 to update the following Requirement set applicability from TS 36.108:

Table 4.6-1: Requirement set applicability

| Requirement | Requirement set |  |
| --- | --- | --- |
|  | SAN type 1-H | SAN type 1-O |
| SAN output power | 6.2 |  |
| … | … |  |
| Transmit ON/OFF power | NA 6.4 |  |
| … | … |  |
| Transmitter intermodulation | NA | NA |
| … | … |  |
| Performance requirements | 8 |  |
| Radiated transmit power | 9.2 | 9.2 |
| … |  | … |
| OTA transmit ON/OFF power |  | NA 9.5 |
| … |  | … |
| OTA ACLR | NA | 9.7.3 |
| … |  | … |

Sub-topic 3-2: In-channel selectivity in TS 36.108

Issue 3-2-1: In-channel selectivity/OTA in-channel selectivity from specification TS 36.108

Agreement: Do not consider in-channel selectivity/OTA in-channel selectivity in the Rel-19 specification TS 36.108 (clauses 7.8 for conducted receiver characteristics and 10.9 for radiated receiver characteristics) as SAN requirement for TDD NB-IoT NTN band 249, since it was not considered in previous Rel-18 TS 36.108 specification for previous defined NB-IoT NTN bands.

Issue 3-2-2: Tables A.14-1 for TS 36.108

Agreement: Remove “in-channel selectivity” from e.g. tables A.14-1.

Sub-topic 3-3: REFSENS in TS 36.108

Issue 3-3-1: 15 kHz tone vs 3.75 kHz tone

Agreement:

- Option 1: Keep single tone 15 kHz (with different variants) and remove 3.75 kHz single-tone configuration for SAN reference sensitivity for NTN TDD NB-IoT in TS 36.108.

- Option 2: 3.75 kHz can be kept if companies are bringing contributions to support this configuration with a Tx time below 8 ms.

Issue 3-3-2: New A14-1 variants (FRC REFSENS)

- Option 1: Consider at least one of the new A14-1 configurations below for NTN TDD NB-IoT:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | QPSK | QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | ⅓ | ⅓ | ⅓ |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

- Option 2: 3.75 kHz can be kept for NTN TDD NB-IoT if companies are bringing contributions to support this configuration with a Tx time below 8 ms.

Issue 3-3-3: SNR targets for new A14-1 variants (FRC REFSENS)

- Option 1: Consider the SNR target values corresponding to the new NTN TDD NB-IoT A14-1 configurations:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | QPSK | QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

- Option 2: 3.75 kHz can be kept for NTN TDD NB-IoT if companies are bringing SNR values to support this configuration with a Tx time below 8 ms.

Issue 3-3-4: Equations and parameter values to compute REFSENS

Agreement: As starting point, the following equations and parameter values to compute REFSENS for TDD NTN NB-IoT SAN can be considered:

REFSENS [dBm] = -174 dBm/Hz noise density +10*LOG10(15*1000 Hz) bandwidth + SAN NF + IM + SNR_target,

where:

- SAN NF (Noise Figure):

  - NF LEO = 4.3 dB

  - NF GEO = 7.4 dB

- IM (Implementation Margin) = 2 dB

- (new TDD only) SNR_target values:

  - A14-1 new Option1 for TDD: -1.17dB;

  - A14-1 new Option2 for TDD: 0.54 dB;

  - A14-1 new Option3 for TDD: -0.61 dB,

Issue 3-3-5: Resulted REFSENS values (different options)

Agreement: In accordance with new A14-1 configurations, RAN4 to select between following (one or more) REFSENS values for TDD NB-IoT NTN band 249 SAN specification:

- Option 1:

  - GEO: -124.0 dBm;

  - LEO: -127.1 dBm;

- Option 2:

  - GEO: -122.3 dBm;

  - LEO: -125.4 dBm;

- Option 3:

  - GEO: -123.4 dBm;

  - LEO: -126.5 dBm;

- Other options not precluded.

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | QPSK | QPSK |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | .. as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| REFSENS (dBm) | GEO: -124.9 LEO: -128.0 (current) |  | GEO: -124.0 LEO: -127.1 (new proposed) | GEO: -122.3 LEO: -125.4 (new proposed) | GEO: -123.4 LEO: -126.5 (new proposed) |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

Issue 3-3-6: REFSENS (with updates from previous meeting RAN4#114)

Agreement:

- Option 1: For SAN TS 36.108, adapt FRC configuration with new TDD frame structure (maximum 8 ms transmission time), for example:

  - For conducted specifications:

For SAN supporting NTN NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.1 with parameters specified in Table 7.2.2-3 and 7.2.2-4 for SAN type 1-H in all FDD operating bands and specified in Table 7.2.2-5 and 7.2.2-6 for SAN type 1-H in all TDD operating bands.

Table 7.2.2-3: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 |

Table 7.2.2-4: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 |

Table 7.2.2-5: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

Table 7.2.2-6: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

  - For OTA specifications:

For SAN supporting standalone NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameter specified in Table 10.3.2-3 and Table 10.3.2-4 for FDD bands and Table 10.3.2-5 and Table 10.3.2-6 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 - ΔOTAREFSENS |

Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 - ΔOTAREFSENS |

Table 10.3.2-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD- ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

Table10.3.2-6: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD - ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

- Option 2: 3.75 kHz can be kept for NTN TDD NB-IoT REFSENS if companies are bringing contributions to support this configuration with a Tx time below 8 ms.

Sub-topic 3-4: Dynamic range in TS 36.108

- Issue 3-4-1: A15-1 & A15-2 configurations for dynamic range

Agreement:

- Option 1:

  - Reference channel A15-1 (with a 15kHz configuration) can be reused for NTN TDD IoT NTN dynamic range computation.

  - Reference channel A15-2 (with a 3.75kHz configuration) can be removed for NTN TDD IoT NTN dynamic range computation.

Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 for both FDD & TDD | A15-2 for FDD only |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

- Option 2:

  - Reference channel A15-1 (with a 15kHz configuration) can be reused for NTN TDD IoT NTN dynamic range computation.

  - If companies are providing a 3.75kHz configuration for Tx time below 8 ms, reference channel A15-2 can be kept for NTN TDD IoT NTN dynamic range computation.

- Issue 3-4-2: Equations and parameter values to compute dynamic range

Agreement: RAN4 to consider the following equations and parameter values to compute dynamic range for TDD NTN NB-IoT SAN:

- P_int = -174 dBm/Hz + 10 x log10 (BW) + NF + IoT;

- P_wan = -174 dBm/Hz + 10 x log10 (BW) + NF +IoT + SNR +IM;

with the following parameters:

  - SNR dynamic range NB-IoT = 4.8 dB (as per TR 36.802 and not -2,1 dB as for REFSENS);

  - IM=2.7 dB;

  - BW_Channel = 200 kHz for interferer;

  - BW_Channel =15 kHz for wanted signal;

  - IoT LEO=15 dB (IoT GEO =N/A);

  - NF LEO=4.3 dB (NF GEO=7.4 dB).

- Issue 3-4-3: Dynamic range double-check

Agreement:

- Re-evaluate dynamic range for NB-IoT LEO case. RAN4 to double-check the dynamic range values for TDD NTN NB-IoT SAN since computed values are P_int=-101,7 dBm; P_want=-105,4 dBm for LEO (it seems there is an error also for FDD NTN NB-IoT SAN values for LEO, and GEO should not have been considered):

Table 7.3.2-3: Dynamic range of SAN supporting standalone NB-IoT operation (LEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -89.4 | -85.7 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -95.3 | -85.7 | AWGN |

- Further discuss if dynamic range for NB-IoT GEO case should be de-scoped or not, in Rel-17 (and should have been the case also for NB-IoT in Rel-18) it was decided not to define any dynamic range requirement for SAN GEO class.

Table 7.3.2-2: Dynamic range of SAN supporting standalone NB-IoT operation  (GEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -97.3 | -93.6 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -103.2 | -93.6 | AWGN |

Sub-topic 3-5: General SAN requirements for NTN TDD NB-IoT

- Issue 3-5-1: Number of tones

Agreement: RAN4 may consider number of tones =1 for deriving the requirements, as long as transmission time less than 8ms.

- Issue 3-5-2: Repetition number

Agreement: RAN4 can consider Repetition number=1 to be used for TDD NB-IoT SAN Rx requirements compatible with D/U=8 (8 ms transmission/reception time) and transmission periodicity N=9 (90 ms) for the new TDD frame structure.

Sub-topic 3-6: Work Split for SAN specification TS 36.108

- Issue 3-6-1: Work Split for NB-IoT TDD NTN in TS 36.108

Agreement:

| Clause/Requirement | Company |
| --- | --- |
| Definitions/Abbreviations | Iridium |
| Band definition (Clause 5.2) | THALES |
| Conducted REFSENS update | THALES |
| Radiated REFSENS update |  |
| Dynamic range update | Iridium |
| Conducted ON/OFF requirements | Iridium |
| EVM Annex | ZTE |
| Other, miscellaneous |  |

2.4.2 	Agreements during RAN4#114-bis RRM

## Issue 0: General aspects

Agreement (online):

For TDD NB-IoT in NTN, discuss whether to define the RRM requirements for:

- Earth moving and fixed cell, normal coverage with SNR ≥ -1dB

Agreement:

Further discuss the following questions in the next meeting:

- Q1: Is the measurement/evaluation solely based on NPSS, NSSS or NRS?

- Q2: How many samples of the reference signal(s) were needed in a normal coverage scenario with SNR ≥ [-6, -1] dB?

- Q3: Is the UE required to monitor the reference signals during DL NB-IoT subframes that are in the middle of ongoing UL transmissions?

|  | Question Q1 | Question Q2 | Question Q3 |
| --- | --- | --- | --- |
| Issue 1: Cell reselection (4.6A.2) |  |  |  |
| Issue 2: Cell measurement in connected mode (8.14A) |  |  |  |
| Issue 3: RRC re-establishment (6.5A) |  |  |  |
| Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A) |  |  |  |
| Issue 5: RLM (7.23A) |  |  |  |

## Issue 1: Cell reselection (4.6A.2)

Agreement (online):

For the cell reselection for TDD NB-IoT in NTN,

- The requirements are applicable only if UE is provided with the active periods of the neighbour cells configured for the measurements.

- For earth fixed cell and earth moving cell:

  - The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable if it is confirmed that the current requirement is achievable without requiring more than one NPSS/NSSS sample for half the DRX cycle length [due to the TDD pattern]

    - Further discuss the values if -1dB will be used.

Ksatellite = 1 for the cells measured in DL opportunities of a neighbour satellite that are in non-overlapping with DL opportunities to be measured in the serving satellite

2.4.3 	Agreements during RAN4#115 UE/SAN RF

SAN Requirements

Issue 1-1-1: 15 kHz tone vs 3.75 kHz tone

Agreement (update from previous meeting):

RAN4 to keep single tone 15 kHz (with different variants) and remove 3.75 kHz single-tone configuration for SAN reference sensitivity for NTN TDD NB-IoT in TS 36.108 (Option 1/REFSENS in TS 36.108).

Issue 1-1-2: New A14-1 variants (FRC REFSENS)

Agreement (update from previous meeting):

RAN4 to consider new FRC configuration A14-1 Option 1 below for NTN TDD NB-IoT:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | [π/4] QPSK | [π/4] QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | ⅓ | ⅓ | ⅓ |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

Issue 1-1-3: SNR targets for new A14-1 variants (FRC REFSENS)

Agreement (update from previous meeting):

RAN4 to consider new SNR target value corresponding to the new configuration NTN TDD NB-IoT A14-1 Option 1 below for NTN TDD NB-IoT:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | [π/4]QPSK | [π/4]QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

Issue 1-1-4: Equations and parameter values to compute REFSENS

Agreement (update from previous meeting):

The following equations and parameter values to compute REFSENS for TDD NTN NB-IoT SAN are used:

REFSENS [dBm] = -174 dBm/Hz noise density +10*LOG10(15*1000 Hz) bandwidth + SAN NF + IM + SNR_target,

where:

- SAN NF (Noise Figure):

  - NF LEO = 4.3 dB

  - NF GEO = 7.4 dB

- IM (Implementation Margin) = 2 dB

- (new TDD only) SNR_target values:

  - A14-1 new Option1 for TDD: -1.17dB;

  - A14-1 new Option2 for TDD: 0.54 dB;

  - A14-1 new Option3 for TDD: -0.61 dB,

Issue 1-1-5: Resulted REFSENS values

Agreement (update from previous meeting):

  - Proposal 1 (P5/R4-2507100): In accordance with new A14-1 configurations, RAN4 to select between following REFSENS values for TDD NB-IoT NTN band 249 SAN specification:

- Option 1:

  - GEO: -124.0 dBm;

  - LEO: -127.1 dBm;

- Option 2:

  - GEO: -122.3 dBm;

  - LEO: -125.4 dBm;

- Option 3:

  - GEO: -123.4 dBm;

  - LEO: -126.5 dBm;

- Other options not precluded.

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | QPSK | QPSK |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | .. as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| REFSENS (dBm) | GEO: -124.9 LEO: -128.0 (current) |  | GEO: -124.0 LEO: -127.1 (new proposed) | GEO: -122.3 LEO: -125.4 (new proposed) | GEO: -123.4 LEO: -126.5 (new proposed) |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

Issue 1-1-6: REFSENS updates from previous meeting

Agreement (update from previous meeting):

For SAN TS 36.108, adapt FRC configuration with new TDD frame structure (maximum 8 ms transmission time). Moreover, no GEO class requirements for NB-IoT operation in TDD are considered.

    - For conducted specifications:

For SAN supporting NTN NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.1 with parameters specified in Table 7.2.2-3 and 7.2.2-4 for SAN type 1-H in all FDD operating bands and specified in Table 7.2.2-5 for SAN type 1-H in all TDD operating bands.

Table 7.2.2-3: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 |

Table 7.2.2-4: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 |

Table 7.2.2-5: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 new Option 1 in Annex A.14 | -127.1 |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

    - For OTA specifications:

For SAN supporting standalone NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameter specified in Table 10.3.2-3 and Table 10.3.2-4 for FDD bands and Table 10.3.2-5 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 - ΔOTAREFSENS |

Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 - ΔOTAREFSENS |

Table10.3.2-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 new Option 1 in Annex A.14 | -127.1 - ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

Issue 1-2-1: A15-1 & A15-2 configurations for dynamic range

Agreement (update from previous meeting):

With respect to NTN TDD IoT NTN dynamic range:

- Reference channel A15-1 (with a 15kHz configuration) is reused for NTN TDD IoT NTN dynamic range computation.

- Reference channel A15-2 (with a 3.75kHz configuration) is removed for NTN TDD IoT NTN dynamic range computation.

Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 for both FDD & TDD | A15-2 for FDD only |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

Issue 1-2-2: LEO dynamic range of SAN supporting standalone NB-IoT operation in TDD

Agreement (update from previous meeting):

RAN4 to use a new table for NTN TDD IoT NTN dynamic range for LEO:

Table 7.3.2-x: Dynamic range of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -105.4 | -101.7 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -95.3 | -85.7 | AWGN |

Issue 1-2-3: LEO dynamic range of SAN supporting standalone NB-IoT operation in FDD

In RAN4#114-bis it was agreed (among other proposals): - Issue 3-4-2: Equations and parameter values to compute dynamic range Agreement: RAN4 to consider the following equations and parameter values to compute dynamic range for TDD NTN NB-IoT SAN: - P_int = -174 dBm/Hz + 10 x log10 (BW) + NF + IoT; - P_wan = -174 dBm/Hz + 10 x log10 (BW) + NF +IoT + SNR +IM; with the following parameters: - SNR dynamic range NB-IoT = 4.8 dB (as per TR 36.802 and not -2,1 dB as for REFSENS); - IM=2.7 dB; - BW_Channel = 200 kHz for interferer; - BW_Channel =15 kHz for wanted signal; - IoT LEO=15 dB (IoT GEO =N/A); - NF LEO=4.3 dB (NF GEO=7.4 dB).

Agreement for RAN4#115:
RAN4 to consider correcting previous NTN FDD IoT NTN dynamic range for LEO during maintenance phase:

Table 7.3.2-3: Dynamic range of SAN supporting standalone NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | [-89.4TBD] | [-85.7TBD] | AWGN |
| 200 | FRC A15-2 in Annex A.15 | [-95.3TBD] | [-85.7TBD] | AWGN |

Issue 1-2-4: GEO dynamic range of SAN supporting standalone NB-IoT operation in TDD

Agreement:

RAN4 shall not define any NTN TDD IoT NTN dynamic range for GEO.

Issue 1-2-5: GEO dynamic range of SAN supporting standalone NB-IoT operation in FDD

Agreement:

RAN4 to consider removing previous NTN FDD IoT NTN dynamic range for GEO during maintenance phase.

Table 7.3.2-2: Dynamic range of SAN supporting standalone NB-IoT operation  (GEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -97.3 | -93.6 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -103.2 | -93.6 | AWGN |

Issue 1-2-7: GEO OTA dynamic range of SAN supporting standalone NB-IoT operation in TDD

Agreement:

RAN4 shall not define any OTA NTN TDD IoT NTN dynamic range for GEO.

Issue 1-3-2: Regional Requirements

Agreement:

Update the regional requirement (clause 4.5) in TS 36.108 as follows

## 4.5	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or as mandatory requirements set by local and regional regulation. It is normally not stated in the 3GPP specifications under what exact circumstances the regional requirements apply, since this is defined by local or regional regulation.

Table 4.5-1 lists all requirements in the present specification that may be applied differently in different regions.

Table 4.5-1: List of regional requirements

| Clause number | Requirement | Comments |
| --- | --- | --- |
| 5.2 | Operating bands | Satellite operating bands may be applied regionally. |
| 6.6.4, 9.7.4 | Out-of-band emissions OTA out-of-band emissions | For band 249 and band 255 operation in US, Limits in FCC Title 47 apply. |
| 6.6.5 9.7.5 | Tx spurious emissions, OTA Tx spurious emissions | For band 249 and band 255 operation in US, Limits in FCC Title 47 apply. |

Issue 1-4-1: (Updated) Work Split for NB-IoT TDD NTN in TS 36.108

Agreement:

RAN4 shall consider updating Work Split for TS 36.108.

| Clause/Requirement | Company |
| --- | --- |
| Definitions/Abbreviations | Iridium |
| Band definition (Clause 5.2) | THALES |
| Conducted REFSENS update | THALES |
| Radiated REFSENS update | [THALES] TBD |
| Dynamic range update | Iridium |
| Conducted ON/OFF requirements | Iridium |
| EVM Annex | ZTE |
| Other, miscellaneous | TBD |
| ACS requirement (double-check) | Iridium |

Note: Other identified Clauses are not precluded

Agreement: (EVM requirement)

“For TDD the averaging in the time domain can be calculated from subframes of different frames and should have a minimum of 108 subframes averaging length for the transmission time. TDD special fields (DwPTS and GP) are not included in the averaging.”

Issue 1-5-1: Update TS 36.181

Agreement:

Since TS 36.108 decisions look stable now, RAN4 shall start to discuss/update TS 36.181 starting from next meeting RAN4#116.

- Note: There are many corrections to be made, for instance ETC6 is defined in Clause 4.2.7 and then never used throughout all Technical Specification.

Issue 1-5-2: Work Split for NB-IoT TDD NTN in TS 36.181

Agreement:

| Clause/Requirement | Company (other companies are encouraged to contribute) |
| --- | --- |
| Definitions/Abbreviations | Iridium |
| Conducted/Radiated REFSENS update | THALES |
| Conducted/Radiated Dynamic range update | Iridium |
| Conducted ON/OFF requirements | Iridium |
| ACS requirement (double-check) | Iridium |
| EVM Annex?, Other.. |  |

Note: Other identified Clauses are not precluded

UE Requirements

Issue 2-1-1: Discussion on 254 & 249 frequency bands relationship

(Continue to reuse previous agreements from RAN4#114-bis):

Agreement: RAN4 to consider at least the following references: ETSI EN 301 441, ITU-R M.1343-1, other references not precluded.

Agreement: Remaining issues from 254 band introduction will remain out of scope of the IoT_NTN_TDD WI.

Note: IoT_NTN_TDD WI currently considers the definition of 249 band in Rel-19. 254 band has been previously introduced during Rel-18 work.

Issue 2-1-7: UE maximum output power for category NB1 and NB2

At RAN4#114-bis it was also decided: Agreement: Use PC3 and PC5 as work assumptions. The NB-IoT TDD NTN WI shall not consider the definition of a new power class. Note: the intention of NB-IoT TDD NTN WI was to re-use current legacy NB-IoT devices.

Tentative Agreement for RAN4#115:

RAN4 shall update section 6.2B.1 as follows

### 6.2B.1	UE maximum output power for category NB1 and NB2

Category NB1 and NB2 UE Power Classes are specified in Table 6.2B.1-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75 kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2B.1-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2 | 20 | +/-2 |
| 255 | 23 | +/-2 | 20 | +/-2 |
| 254 | 23 | +/-2 | 20 | +/-2 |
| 253 | 23 | +/-2 | 20 | +/-2 |
| 249 | 23 | +/-2 | 20 | +/-2 |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

The UE shall meet the following additional requirements for maximum transmission power density specified in Table 6.2B.1-2 when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2B.1-2: Additional requirements for transmit EIRP density

| EUTRABand | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum EIRP density limit |
| --- | --- | --- | --- | --- |
| 254 | NS_04N | 0.2 | 1610 - 1618.25 | 27dBm/4kHz (mean EIRP limit) 15dBm/4kHz (peak EIRP limit) |
|  | NS_05N | 0.2 | 1618.25 - 1626.5 |  |
| 249 | NS_0xN | 0.2 | 1616 - 1626.5 | 27dBm/4kHz (mean EIRP density) or 15dBm/4kHz (peak EIRP density) (NOTE 1) |
| NOTE 1: As per [ETSI EN 301 441]. |  |  |  |  |

Issue 2-2-1: (Updated) Work Split for NB-IoT TDD NTN in TS 36.102

Agreement:

- Update the Work Split for NB-IoT TDD NTN in TS 36.102 if needed

| Clause/Requirement | Company |
| --- | --- |
| Band definition (Clause 5.2) | Iridium |
| UE maximum output power for category NB1 and NB2 (Clause 6.2B.1) | [THALES]TBD |
| REFSENS update (Clause 7.3B) | THALES |
| A-MPR | [Nordic]TBD |
| Frequency error requirement | THALES |
| Blocking update (Clause 7.6B) | ZTE |
| Other topics (if any) e.g. [EVM Annex] | TBD |

Note: Other identified Clauses are not precluded

2.4.4 	Agreements during RAN4#115 RRM

## Issue 0: General aspects

Agreement: online

- RRM requirements in IoT-NTN TDD mode in this WI are defined for earth-moving cells, and are not applicable for earth-fixed cells.

Session Chair: (for information-only)

- As starting point, RAN4 to define requirement considering the Cell dwell time: Maximum is 83.02 seconds; Minimum is 25.33 seconds.

Agreement: online

- Target Coverage: Normal coverage

- Target SNR in AWGN: -1 dB

- Requirement applicability: Capture in specification clearly that the new requirements are applicable only for the new punctured TDD Frame structure where N=9 and D=U=8, as described in the WID

Agreement: online

- Is the measurement/evaluation solely based on NPSS, NSSS or NRS?

  - Issue 1: Cell reselection (4.6A.2)

    - Tdetect: NPSS, NSSS, [NPBCH]

    - Tmeasure: NRS

    - Treselection: NRS

  - Issue 2: Cell measurement in connected mode (8.14A)

    - Detection for neighbour cell (8.14A.6): NPSS, NSSS, [NPBCH]

    - Measurement: NRS

      - Further discussion on NSSS

  - Issue 3: RRC re-establishment (6.5A)

    - Detection for unknown cell: NPSS, NSSS, [NPBCH]

    - Measurement: NRS

      - Further discussion on NSSS

  - Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A)

    - NA

  - Issue 5: RLM (7.23A)

    - NRS

Agreement: online

- Q2: How many samples of the reference signal(s) were needed in a normal coverage scenario with SNR ≥ -1 dB?

  - Issue 1 & 2 & 3: Cell reselection (4.6A.2), Cell measurement in connected mode (8.14A) and RRC re-establishment

    - Detection: 12 NPSS subframes + 2 NSSS subframes + 1 NSSS subframe

    - T_SI: 8 NPBCH subframes is included

    - Measurement: 8 NRS subframes

  - Issue 5: RLM (7.23A): further discussion

2.4.5 	Remaining open issues

Remaining open issues – UE/SAN RF

Issue 1-2-6: LEO OTA dynamic range of SAN supporting standalone NB-IoT operation in TDD

Agreement:

RAN4 shall consider adding a new table for OTA NTN TDD IoT NTN dynamic range for LEO:

Table 10.4.2-x: Dynamic range of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -105.4 - ΔOTAREFSENS | -101.7 - ΔOTAREFSENS | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -95.3 - ΔOTAREFSENS | -85.7 - ΔOTAREFSENS | AWGN |

Issue 1-3-1: ACS SAN for NTN TDD NB-IoT

Agreement:

Further discuss in next RAN4#116 meeting the NTN NB-IoT TDD test configuration for SAN ACS requirement (interfering signal mean power, wanted signal power) and if any update needs to be done.

Issue 2-1-2: Measurement BW for 249

RAN4 to further discuss the measurement bandwidth larger than channel bandwidth for 249 in the context of satisfying requirements in ETSI EN 301 441 and ITU-R M.1343-1 that are currently under NS_04N in TS 36.102.

Issue 2-1-5: Discussion on UE AMPR for NB-IoT TDD NTN in TS 36.102

Agreement:

As starting point, the following tables could be used for band 249 NS_0xN A-MPR:

- For 1-tone allocation (priority 1)

1-Tone A-MPR for NS_0xN power class 3

| Modulation | π/4 QPSK |  |  |
| --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/3/4/5/6/7/8/9 |
| A-MPR | ≤ 0.2 dB | 0 dB | 0 dB |
| NOTE: The A-MPR is defined for a measurement bandwidth Bn (TBD) ≥ CBW (200kHz) as per [ETSI EN 301 441]. |  |  |  |

- For multiple-tone allocation (priority 2)

A-MPR for NS_0xN power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 0.2 dB | 0 dB |  | ≤ 0.2 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | 0 dB |  | 0 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |
| NOTE: The A-MPR is defined for a measurement bandwidth Bn (TBD) ≥ CBW (200kHz) as per [ETSI EN 301 441]. |  |  |  |  |

Agreement:

At RAN4#116, companies to share more details on the UE SEM measurement and/or simulation results including measurement/simulation assumptions.

Issue 2-1-6: NS flags for band 249

Agreement:

Further discuss if new NS_0xN for band 249, NS_0xN different from previous NS_04N and NS_05N.

Issue 2-1-8: Discussion on frequency error requirement for NB-IoT TDD NTN in TS 36.102

Moderator Note: RAN4 should by default not exclude any kind of UE-specific implementation for the frequency pre-compensation mechanism. For the 249 band, a UE adjusts its frequency pre-compensation before/at the beginning of/during each set of consecutive uplink burst of 8 consecutive sub-frames. When segmentation is used (when segments are configured by the eNB), a UE adjusts its frequency pre-compensation at the beginning of each segment.

When segments are configured by eNB, and UE supports optional capability ntn-SegmentedPrecompensationGaps, then UE adjust also its timing pre-compensation before/at the beginning of/during each segment.

However, for the definition of the requirement:

Agreement:

The pre-compensation during the segments of an uplink burst of 8 consecutive sub-frames is considered as optional.

Agreement:

RAN4 is considering for the definition of the frequency error requirement:

- Do not change the current frequency error requirement from TS 36.102 for NTN TDD NB-IoT:

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

- Add a note a new phrase/paragraph to the following text [Clause 6.4B.1of TS36.102] as follows:

  - Option 1:

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted sub-frames. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. For band 249, UE may also adjust its frequency pre-compensation during or at the beginning the 8 consecutive uplink transmitted sub-frames.

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts.”.

  - Option 2:

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted sub-frames. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. For band 249, UE may adjust its frequency pre-compensation before, at the beginning of, and/or during each set of consecutive uplink sub-frames.

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts.”.

  - Option 3:

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted subframes. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. [The frequency pre-compensation during the uplink burst is up to UE implementation.]

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts.”.

Issue 3-1-1: LS reply to RAN1 on pre-compensation

Agreement: The LS reply shall be based on both RF (frequency error) and RRM (timing error) requirement.

Agreement: The reply LS shall take into account both transparent and regenerative payload design and shall consider satellite and UE position error.

Note: Satellite and UE position error (17*Ts) were not included in the initial analysis for timing error, and other timing errors could be considered.

Agreement:

As starting point, RAN4 shall use as baseline for the discussion at next RAN4#116 meeting the following draft LS reply (to RAN1):

<<Start LS Reply>>

Working assumption

For precompensation, from RAN1 perspective:

- The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.

  - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed.

- FFS: whether spec impact is in RAN1, RAN4 or both

RAN4 received from RAN1 the following LS with the following RAN1 working assumptions:

RAN4 would like to thank RAN1 for informing RAN4 about the agreement regarding UE behaviour in IoT NTN TDD system regarding the pre-compensation. RAN4 does not see any issues with the agreement and has agreed to incorporate the agreement in TS 36.102 clause 6.4B.1 as follows:

At the RAN4#115 meeting, RAN4 reached the following agreement:

- For frequency error requirement (TS 36.102):

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted subframes. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. [The frequency pre-compensation during the uplink burst is up to UE implementation.]

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts.”.

Note: For TDD mode, the same TS 36.102 specification for NB-IoT NTN UE frequency error applies:

“Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

“

- For timing error requirement (TS 36.133):

[For NB-IoT NTN TDD, UE may adjust its time at the beginning of the uplink burst of 8 consecutive transmitted sub-frames. Moreover, the time pre-compensation during the uplink burst of 8 consecutive transmitted sub-frames is considered up to UE implementation.]

Note: For TDD mode, the same TS 36.133 specification for NB-IoT NTN UE timing error applies:

“Table 7.20A.2-1: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97*TS |
| Note 1: TS is the basic timing unit defined in TS 36.211 |  |

“

As a general assumption for both frequency error and timing error, no pre-compensation gap is needed before the beginning of each uplink burst of 8 consecutive transmitted subframes. [However, when segmentation is applied (i.e. network configures the segmentation), UE pre-compensation gap may or may not be used for timing adjustment between 2 consecutive segments. Therefore, RAN4 estimates that the use of pre-compensation gaps on the UE side may be allowed but is optional depending on the UE implementation. RAN4 also considers that segmentation can be applied during the transmission of an uplink burst of 8 consecutive transmitted subframes.]

<<End LS Reply>>

Moderator Note:

- one can find below a computation of frequency error per different configurations:

| Time considered for frequency variation computation | Maximum frequency shift | Observation |
| --- | --- | --- |
| 1 Tx burst (8ms of consecutive 8 UL sub-frames) | 3.45 Hz at 1.6 GHz | Negligible with respect to 160Hz (0.1 ppm at 1.6 GHz) |
| 9 radio frames (90ms transmission periodicity) | 38.88 Hz at 1.6 GHz | Under 160Hz (0.1 ppm at 1.6 GHz) |
| 2 x 9 radio frames (180ms - multiple of 90ms transmission periodicity) | 77.76 Hz at 1.6 GHz | Under 160Hz (0.1 ppm at 1.6 GHz) |

- one can find below a computation of timing error per different configurations:

| Time considered for timing variation computation | Maximum timing shift | Observations |
| --- | --- | --- |
| 1 Tx burst for regenerative payload (8ms of consecutive 8 UL sub-frames) with an assumption of +/-48 µs/sec. | 0.381 µsec @ 10deg elevation @600km LEO And other examples: [0.317 µsec @ 30deg elevation @600km LEO 0.348 µsec at 10deg elevation @800km LEO 0.306 µsec at 30deg elevation @800km LEO] | < 3.1575 µs (97*TS) Maximum timing shift = approx. 12% of Te timing error limit (without UE & Sat positioning error included and other errors) |
| 1 Tx burst for transparent payload (8ms of consecutive 8 UL sub-frames) with an assumption of +/-96 µs/sec. | 0.744 µsec @ 10deg @600km LEO As other examples: [0.634 µsec @ 30deg elevation @600km LEO 0.696 µsec @ 10deg @800km LEO 0.612 µsec @ 30deg elevation @800km LEO] | < 3.1575 µs (97*TS) Maximum timing shift = approx. 23.5% of Te timing error limit (without UE & Sat positioning error included and other errors). The total shift > 0.744 (caused by timing drift for 600km LEO @ 10deg elevation) + 0.544 µs (under assumption of Te_sat + Te_UE = 17*Ts µs which in practice may be higher) and representing 41% of Te timing error limit excluding other errors (from radio channel, propagator prediction errors etc.). Other example: The total shift > 0.612 (caused by timing drift for 800km LEO @ 10deg elevation) + 0.544 µs (under assumption of Te_sat + Te_UE = 17*Ts µs which in practice may be higher) and representing 37% of Te timing error limit excluding other errors (from radio channel, propagator prediction errors etc.). RAN4 may therefore further consider this timing drift issue for NB-IoT NTN TDD. |

Issue 3-2-1: Revisit the demodulation performance requirements for UE side

Agreement:

Demodulation and performance schedule is supposed to start in September 2025.

Companies are encouraged to discuss offline. In any case, the demodulation performance may be based on max 8ms transmission time, which may not require the modification of the current DL demodulation test configuration, just the SNR target value which needs to be re-computed.

| RAN#109 (Sep. 2025) | RAN1#123 |  |  |
| --- | --- | --- | --- |
|  | RAN2#132 |  |  |
|  | RAN4#117 | 0.25 | - Start discussion on demodulation performance and specific configuration related to the new TDD pattern |

Remaining open issues – RRM

Further discussion:

- Q3: Is the UE required to monitor the reference signals during DL NB-IoT subframes that are in the middle of ongoing UL transmissions?

  - Issue 1: Cell reselection (4.6A.2)

    - NA

  - Issue 2: Cell measurement in connected mode (8.14A)

    - Option 1: No impacts on defining requirements. Could be clarified in RAN1 if needed. (HW)

    - Option 2: From RAN4 point of view, the UEs are expected to perform measurements in the DL subframes that occur in between sets of UL subframes where UE is performing active transmission

    - Option 3: The UE may be capable of processing reference signals during DL NB-IoT subframes while an ongoing UL transmission has not yet completed. However, the requirement and corresponding test case should be defined such that this capability is not mandatory.

  - Issue 3: RRC re-establishment (6.5A)

    - NA

  - Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A)

    - NA

  - Issue 5: RLM (7.23A)

    - Option 1: No impacts on defining requirements. Could be clarified in RAN1 if needed.

    - Option 2: From RAN4 point of view, the UEs are expected to perform measurements in the DL subframes that occur in between sets of UL subframes where UE is performing active transmission

    - Option 3: The UE may be capable of processing reference signals during DL NB-IoT subframes while an ongoing UL transmission has not yet completed. However, the requirement and corresponding test case should be defined such that this capability is not mandatory.

## Issue 1: Cell reselection (4.6A.2)

Further discuss in the next meeting:

- The exact requirement can be discussed after Issue #0 is concluded for a more systematic approach.

## Issue 2: Cell measurement in connected mode (8.14A)

Further discuss in the next meeting:

- The exact requirement can be discussed after Issue #0 is concluded for a more systematic approach.

## Issue 3: RRC re-establishment (6.5A)

Further discuss in the next meeting:

- The exact requirement can be discussed after Issue #0 is concluded for a more systematic approach.

## Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A)

Further discuss in the next meeting:

- The exact requirement can be discussed after Issue #0 is concluded for a more systematic approach.

## Issue 5: RLM (7.23A)

Further discuss in the next meeting:

- The exact requirement can be discussed after Issue #0 is concluded for a more systematic approach.

## Issue 7: LS from RAN1 on precompensation

This is currently under discussion in the thread of [115][316] IoT_NTN_TDD_UE_SAN_RF, in conjunction with the RF aspects, as part of the reply LS preparation. If no conclusion is reached in this meeting, RAN4 will revisit the topic in the August meeting.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

## 4.1	RAN1

## RAN1#120-bis meeting, Wuhan, China, April 7th – 11th, 2025:

IoT-NTN TDD mode – papers submitted

- R1-2501724	Discussion on IoT-NTN TDD mode	THALES

- R1-2501826	Discussion on IoT-NTN TDD mode	vivo

- R1-2501884	Discussion on IoT-NTN TDD mode	Spreadtrum, UNISOC

- R1-2501903	Discussion on the IoT-NTN TDD mode	ZTE Corporation, Sanechips

- R1-2501976	Discussion on Physical layer design of IoT-NTN TDD	CATT

- R1-2502057	Remaining aspects of loT-NTN TDD mode	Iridium Satellite LLC

- R1-2502059	Stage 2 proposal (for RAN2 TS 36.300) for Introduction of IoT-NTN TDD mode	Iridium Satellite LLC

- R1-2502095	Discussion on IoT-NTN TDD mode	Nokia, Nokia Shanghai Bell

- R1-2502177	Discussion on the new NB-IoT NTN TDD mode	CMCC

- R1-2502223	Discussion on IoT-NTN TDD mode	Huawei, HiSilicon

- R1-2502269	Discussion on IoT-NTN TDD mode	OPPO

- R1-2502345	IoT NTN TDD guard period and uplink segmentation	Sharp

- R1-2502387	Discussion on IoT-NTN TDD mode	Samsung

- R1-2502458	Discussion on IoT-NTN TDD mode	Xiaomi

- R1-2502633	Discussion on IoT-NTN TDD mode	Apple

- R1-2502782	Discussion on IoT-NTN TDD mode	NTT DOCOMO, INC.

- R1-2502819	Discussion on IoT-NTN TDD mode	LG Electronics

- R1-2502824	Discussion on IoT-NTN TDD mode	Lenovo

- R1-2502858	IOT-NTN TDD mode	Qualcomm Incorporated

- R1-2502881	On R19 TDD IoT-NTN	Nordic Semiconductor ASA

- R1-2502999	Feature lead summary #1 on IoT-NTN TDD mode	Moderator (Qualcomm Incorporated)

- R1-2503081	Feature lead summary #2 on IoT-NTN TDD mode	Moderator (Qualcomm Incorporated)

- R1-2503101	Feature lead summary #3 on IoT-NTN TDD mode	Moderator (Qualcomm Incorporated)

- R1-2503142	LS on precompensation for NB-IoT NTN TDD mode	RAN1, Qualcomm Incorporated

IoT-NTN TDD mode – papers submitted into UE features for IoT-NTN TDD mode

- R1-2501837	Discussion on UE features for IoT-NTN TDD mode	 vivo

- R1-2501944	Discussion on the UE feature for IoT-NTN TDD  ZTE Corporation, Sanechips

- R1-2502058	UE features for IoT-NTN TDD mode	Iridium Satellite LLC

- R1-2502141	Views on Rel-19 NTN for Internet of Things (IoT) TDD mode UE features	Nokia

- R1-2502250	UE features for IoT-NTN TDD mode	Huawei, HiSilicon

- R1-2502272	Discussion on UE features for IoT-NTN TDD mode	OPPO

- R1-2502399	UE features for IoT-NTN TDD mode	Samsung

- R1-2502647	On Rel-19 IoT-NTN TDD mode UE Features	Apple

- R1-2502872	UE features for IOT-NTN TDD mode	Qualcomm Incorporated

- R1-2502969	Session Notes of AI 9.15.10	Ad-Hoc Chair (NTT DOCOMO, INC.)

## RAN1#121 meeting, St. Julian’s, Malta, May19th – 23rd, 2025

IoT-NTN TDD mode – papers submitted

- R1-2503284	Discussion on IoT-NTN TDD mode	Huawei, HiSilicon

- R1-2503382	Remaining issues on IoT-NTN TDD mode	vivo

- R1-2503531	Discussion on IoT-NTN TDD mode	Spreadtrum, UNISOC

- R1-2503586	Discussion on IoT-NTN TDD mode	Samsung

- R1-2503639	Discussion on the IoT-NTN TDD mode	ZTE Corporation, Sanechips

- R1-2503688	Discussion on IoT-NTN TDD mode	THALES

- R1-2503778	Discussion on Physical layer design of IoT-NTN TDD	CATT

- R1-2503849	Discussion on the new NB-IoT NTN TDD mode	CMCC

- R1-2503899	Discussion on IoT-NTN TDD mode	Xiaomi

- R1-2503958	Remaining aspects of loT-NTN TDD mode	Iridium Satellite LLC

- R1-2503959	Stage 2 proposal (for RAN2 TS 36.300) for Introduction of IoT-NTN TDD mode	Iridium Satellite LLC

- R1-2503961	Remaining issues on IoT NTN TDD	Sharp

- R1-2504035	Discussion on IoT-NTN TDD mode	Nokia, Nokia Shanghai Bell

- R1-2504203	Discussion on IoT-NTN TDD mode	OPPO

- R1-2504346	Discussion on IoT-NTN TDD mode	Apple

- R1-2504414	IOT-NTN TDD mode	Qualcomm Incorporated

- R1-2504461	Discussion on IoT-NTN TDD mode	Lenovo

- R1-2504518	Discussion on IoT-NTN TDD mode	NTT DOCOMO, INC.

- R1-2504565	Discussion on IoT-NTN TDD mode	LG Electronics

- R1-2504576	On R19 TDD IoT-NTN	Nordic Semiconductor ASA

- R1-2504718	Feature lead summary #1 on IoT-NTN TDD mode	Moderator (Qualcomm Inc)

- R1-2504719	Feature lead summary #2 on IoT-NTN TDD mode	Moderator (Qualcomm Inc)

- R1-2504875	Feature lead summary #3 on IoT-NTN TDD mode	Moderator (Qualcomm Inc)

- R1-2504882 	Draft LS on TP for 36.300 for IOT NTN TDD mode	Qualcomm

- R1-2504883	LS on TP for 36.300 for IOT NTN TDD mode	RAN1, Qualcomm

IoT-NTN TDD mode – papers submitted into UE features for IoT-NTN TDD mode

- R1-2503287	UE features for IoT-NTN TDD mode	Huawei, HiSilicon

- R1-2503394	Discussion on UE features for IoT-NTN TDD mode	vivo

- R1-2503407	NTN for Internet of Things (IoT) TDD mode UE features	Nokia

- R1-2503600	UE features for IoT-NTN TDD mode	Samsung

- `

- R1-2503908	Discussion on UE features for IoT-NTN TDD mode	Xiaomi

- R1-2504198	Discussion on UE features for IoT-NTN TDD mode	OPPO

- R1-2504359	On Rel-19 IoT-NTN TDD mode UE Features	Apple

- R1-2504428	UE features for IOT-NTN TDD Mode	Qualcomm Incorporated

- R1-2504531	Discussion on UE features for for IoT-NTN TDD mode	NTT DOCOMO, INC.

- R1-2504577	On R19 TDD IoT-NTN features	Nordic Semiconductor ASA

- R1-2504667	Summary of discussion on Rel-19 LTE UE features for IoT-NTN TDD mode	Moderator (NTT DOCOMO, INC.)

- R1-2504668	Session Notes of AI 9.15.10	Ad-Hoc Chair (NTT DOCOMO, INC.)

## 4.2	RAN2

RAN2#129-bis meeting, Wuhan, China, April 7th – 11th, 2025:

IoT-NTN TDD mode – papers submitted

- R2-2502071	Discussion on IoT NTN TDD mode	OPPO

- R2-2502358	Discussion on TDD support in IoT NTN	Lenovo

- R2-2502459	On RAN2 aspect of IoT NTN TDD	Samsung

- R2-2502517	Discussion on supporting IoT NTN TDD mode	Apple	discussion

- R2-2502527	Consideration on IoT-NTN TDD mode	ZTE Corporation, Sanechips

- R2-2502538	Discussion on the IoT NTN TDD mode	Xiaomi	discussion

- R2-2502621	Indication of IoT-NTN TDD Mode Support	Toyota ITC

- R2-2502658	Discussion on new NB-IoT NTN TDD mode	Qualcomm Incorporated

- R2-2502703	Support of IoT-NTN TDD mode	CMCC

- R2-2501970	Discussion on RAN2 impacts of IoT-NTN TDD	Huawei, HiSilicon

- R2-2502059	Discussion on support of NB-IoT NTN TDD	CATT

- R2-2501780	Discussion on RAN2 Imapcts of IoT-NTN TDD mode	vivo

- R2-2501989	Discussion on IoT-NTN TDD mode	Iridium Satellite LLC

- R2-2502560	On SI scheduling, H-SFN change and postponing impacts in IoT-NTN TDD mode	Nordic Semiconductor ASA

- R2-2502193	Discussion on support of IoT-NTN TDD mode	THALES

- R2-2502100	Discussion on support of TDD mode for IoT-NTN	Nokia, Nokia Shanghai Bell

## RAN2#130 meeting, St. Julian’s, Malta, May19th – 23rd, 2025

IoT-NTN TDD mode – papers submitted

- R2-2503350	Discussion on support of IoT-NTN TDD mode	Xiaomi

- R2-2503357	Further Discussion on IoT-NTN TDD mode	vivo

- R2-2503389	SI transmission of IoT-NTN TDD mode	NEC

- R2-2503462	Discussion on support of NB-IoT NTN TDD	CATT

- R2-2503531	Discussion on IoT NTN TDD mode	OPPO

- R2-2503689	IoT-NTN TDD mode SI scheduling and UE procedures	Iridium Satellite LLC

- R2-2503911	DL and UL impacts of TDD pattern in IoT NTN	Lenovo

- R2-2503987	Discussion on NTN IoT-NTN TDD mode	TOYOTA ITC

- R2-2504070	Discussion on RAN2 impacts of IoT-NTN TDD	Huawei, HiSilicon

- R2-2504082	Consideration on IoT-NTN TDD mode	ZTE Corporation, Sanechips

- R2-2504093	On RAN2 aspects for IoT NTN TDD	Samsung

- R2-2504320	Discussion on new NB-IoT NTN TDD mode	Qualcomm Incorporated

- R2-2504334	On SI scheduling, postponing impacts, and early implementation of the IoT-NTN TDD mode	Nordic Semiconductor ASA

- R2-2504395	Support of IoT-NTN TDD mode	CMCC

- R2-2504510	Further discussion on support of TDD mode for IoT-NTN	Nokia, Nokia Shanghai Bell

- R2-2504633	Discussion on support of IoT-NTN TDD mode	THALES

- R2-2504673	Report from Break-out session on NR-NTN and IoT-NTN Session Chair (ZTE Corp.)

## 4.3	RAN3

## 4.4	RAN4

RAN4#114-bis meeting, Wuhan, China, April 7th – 11th, 2025:

- R4-2503134	Discussion on RRM requirements for IoT-NTN TDD mode	MediaTek inc.

- R4-2503200	Work plan for WID: introduction of IoT-NTN TDD mode	Iridium Satellite LLC

- R4-2503744	draftCR to TS36.102 Introduction of IoT-NTN TDD band	ZTE Corporation, Sanechips

- R4-2503745	Discussion on SAN RF requirements for IoT-NTN TDD band	ZTE Corporation, Sanechips

- R4-2503746	draftCR to TS36.108 Introduction of IoT-NTN TDD band	ZTE Corporation, Sanechips

- R4-2503799	Discussion on IoT NTN TDD mode	ZTECorporation,Sanechips

- R4-2504529	RRM requirements for IoT NTN in TDD mode	Qualcomm Incorporated

- R4-2504915 	WF on RRM requirements for IoT_NTN_TDD	Moderator (Qualcomm Inc)

- R4-2504719	WF for IoT_NTN_TDD_UE_SAN_RF_v04_clean	Moderator (Thales)

## RAN4#115 meeting, St. Julian’s, Malta, May19th – 23rd, 2025

- R4-2505579	Topic summary for [115][230] IoT_NTN_TDD	Moderator (Qualcomm)

- R4-2507848	Topic summary for [115][316] IoT_NTN_TDD_UE_SAN_RF	Moderator (Thales)

- R4-2505592	Discussion on UE RF requirements for NTN NB-IoT TDD mode	Nordic Semiconductor ASA

- R4-2506123	On NS flags and A-MPR values for the IOT TDD band	Apple

- R4-2506164	Nominated BW use for 249 and freq error	Qualcomm Incorporated

- R4-2506266	draftCR to TS36.108 Add EVM requirement for IoT-NTN TDD band	ZTE Corporation, Sanechips

- R4-2506267	draftCR to TS36.102 Add blocking requirement for IoT-NTN TDD band	ZTE Corporation, Sanechips

- R4-2506447	Discussion on time/frequency pre-compensation for uplink transmission in NB-IoT NTN TDD mode	LG Electronics France

- R4-2506488	Discussion on RRM requirements for IoT-NTN TDD mode	MediaTek inc.

- R4-2506715	Discussion on RRM impact due to periodic pattern for IoT NTN	Huawei, HiSilicon

- R4-2507100	Updates on SAN RF requirements for NTN NB-IoT TDD	THALES, Iridium Satellite LLC

- R4-2507117	Updates on UE RF requirements for NTN NB-IoT TDD	THALES, Iridium Satellite LLC

- R4-2507119	Updates on RRM requirements for NTN NB-IoT TDD	THALES, Iridium Satellite LLC

- R4-2507189	Discussion on IoT NTN TDD mode	ZTECorporation,Sanechips

- R4-2507364	On the measurement framework for the new IOT TDD Punctured frame	Nokia

- R4-2507538	Draft CR on TS 36.102 Clause 5 - IoT_NTN_TDD	THALES, Iridium Satellite LLC

- R4-2507539	Draft CR on TS 36.108 Clauses 3, 4.5, 4.6 and 5 - IoT_NTN_TDD	THALES, Iridium Satellite LLC

- R4-2507732	Proposal for LS reply to RAN1 on uplink pre-compensation for NB-IoT NTN TDD mode	Iridium Satellite LLC, THALES

- R4-2507733	LS Reply on precompensation for NB-IoT NTN TDD mode	Qualcomm Incorporated

- R4-2507806	RRM requirements for IoT NTN in TDD mode	Qualcomm Incorporated

- R4-2508290	WF on RRM requirements for IoT_NTN_TDD	Moderator (Qualcomm Inc)

- R4-2508781	WF for IoT_NTN_TDD_UE_SAN_RF_v10_clean	Moderator (Thales)