3GPP TSG RAN meeting 108										      RP-251187

Prague, Czech Republic, June 9-13, 2025

## Status Report to TSG

Agenda item:			9.3.2.1

| WI / SI Name | NR mobility enhancements Phase 4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_Mob_Ph4 |  |  |  |  |
| Unique ID | 1020091 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-250339 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 91% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Naveen Palle |
|  | Company | Apple Inc. |
|  | Email | naveen.palle@apple.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#120bis

Agreement

- Repetition=off is supported for candidate cell CSI-RS in Rel-19.

- Repetition=on is not supported for candidate cell CSI-RS in Rel-19.

Conclusion

RAN1 assumes the SSB QCLed with CSI-RS is used for timing reference of candidate cell CSI-RS measurement in LTM.

Agreement

In the resource set configured for candidate cell L1-RSRP measurement, a sequence of candidate cell indices is included together with a sequence of CSI-RS resource indices, where each CSI-RS resource index is associated with one candidate cell index.

Agreement

Regarding the LS from RAN2 (R1-2501686) the following contents are included in the reply LS.

- RAN1 assumes at maximum 16 beams can be reported by a single event triggered reporting

  - Regardless whether or not the report includes the current beam

  - Note: The maximum number of beams that can be reported by a UE subject to UE capability is under discussion in RAN1

Agreement

For candidate cell CSI acquisition

- In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported

  - Support of semi-persistent CSI-RS resource is subject to separate UE capability.

- MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2

Send an LS to RAN2 and RAN3 to inform this agreement

Agreement

Reply LS on number of beam measurements in the measurement report MAC CE is agreed.

RAN1 would like to thank RAN2 for their LS on number of beam measurements in the measurement report MAC CE.

RAN1 assumes at maximum 16 beams can be reported by a single event triggered reporting regardless whether or not the report includes the current beam. It is 	noted that the maximum number of beams that can be reported by a UE is subject to UE capability which is under discussion in RAN1.

Final LS in R1-2503077.

Agreement

LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells is agreed. Final LS in R1-2503079.

Agreement

For the RS type determination for event triggered reporting with event LTM2,

- Alt 1-2: At least one candidate RS shall be configured

Agreement

For the container of LTM CSI report to a target cell, UCI is used with a solution to avoid blind detection at gNB if valid CSI is not available

- FFS: the definition of valid CSI

Agreement

For the solution to avoid blind detection for LTM CSI report if valid CSI is not available,

- Option 1: CSI is reported when valid CSI is not available

  - Option 1-1: one CQI codepoint is used to indicate that valid CSI is not available

    - Option 1-1-1: Codepoint with the lowest CQI index is reported

Agreement

Regarding CSI acquisition, for a candidate cell,

- A single CSI report configuration is configured

- Multiple CSI-RS resources for CMR can be associated with the CSI report configuration

  - The number of CSI-RS resources for CMR is subject to UE capability

Agreement

Following restrictions are introduced

- For the codebook configurations in report configuration, typeI-SinglePanel is supported for LTM CSI acquisition

- For report frequency configuration in report configuration, wideband CQI and wideband PMI are supported for LTM CSI acquisition

- For the report quantity in report configuration, cri-RI-PMI-CQI is supported for LTM CSI acquisition

  - The supported max rank is up to separate UE capability

- For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition

  - Up to 128 ports is supported

  - The supported max number of CSI-RS ports is up to separate UE capability

Way forward

- Mobility Chair to join the mobility dinner on Wednesday in Malta

- No mobility session on Friday

RAN1#121

Conclusion

- From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for CSI acquisition for candidate cells

Send an LS to RAN4 (CC RAN2) and ask them to take the above conclusion in their further work. LS in R1-2504828.

Agreement

A list of interference measurement resources for candidate cells is supported for LTM CSI acquisition

- If this list is not configured, CMR is used for interference measurement

Agreement

When SpCellInclusion is configured for gNB configured reporting for L1-RSRP based on CSI-RS, the following rule is used to determine the CSI-RS for current SpCell:

- NZP-CSI-RS resources in [ltm-CSI-NZP-CSI-RS-ResourceList] associated with the current SpCell are the entries where PCI (given by ltm-CandidatePCI) and frequency information (given by ssb-Frequency for the SSBs QCLed with NZP-CSI-RSs) of the candidate cell associated with the LTM-CandidateId (given by the corresponding entry in ltm-CandidateIdList) is equal to the PCI and center frequency of cell-defining SSB of the current SpCell.

Conclusion

RRC configuration for time restriction is NOT introduced for CSI-RS based L1-RSRP measurement for either event triggered reporting or gNB scheduled reporting, i.e. it is up to RAN4 how many samples are used to define their requirement.

Agreement

CSI report configuration for CSI acquisition is determined from the field of Target Configuration ID in CSC MAC CE

Agreement

For PUSCH to convey the early CSI report,

- For RACH-less LTM, the first CG or DG PUSCH after CSC is used

- For RACH-based LTM with CFRA, PUSCH scheduled by RAR or Msg.A

- For RACH-based LTM with CBRA, the first CG or DG PUSCH after HARQ-ACK transmission for Msg.4 or Msg.B

For the reporting LTM early CSI reporting, Table 6.3.1.1.2-7 in TS 38.212 is used as a UCI report format

Conclusion

There is no RAN1 consensus in Rel-19 to provide the UE with an indication of a subset of CSI-RS resources for CSI measurement before CSC

Conclusion

There is no RAN1 consensus to introduce delay requirements based on Z and Z’

Conclusion

No CPU is defined for LTM event triggered reporting.

Conclusion

For the LTM CSI acquisition after CSC until the completion of LTM procedure, CPU is not defined

Conclusion

Definition of active CSI-RS resources after CSC until the completion of LTM procedure:

- Active CSI-RS ports/resources for the candidate cells including target cell are not defined after CSC until the completion of LTM procedure

Conclusion

For the LTM CSI measurement before CSC, CPU is not defined

Conclusion

For the counting of active CSI-RS ports/resources before CSC, no new UE capability is introduced. The total number of active CSI-RS ports/resources is limited by legacy capability report. The total number of active CSI-RS ports/resources is shared by serving cell and candidate cell(s).

#### 2.1.2	Remaining Open issues

- None

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2-129bis:

Agreements

No L1/L2 filtering issue will be discussed in RAN2.

A list of sk-counters is linked to a Rel-19 ID configured by the SN.

At RLF and reconfiguration with sync failure:

- If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

- if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

How to indicate whether an LTM candidate configuration is a complete configuration is the same as Rel-18 (no different indication for MCG and SCG part of the candidate configuration).

The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message. We include this agreement in the LS to RAN3.

Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

Introducing a new R19 LTM cell switch command MAC CE with indicating the NCC value for R19 inter-CU LTM cell switch. Details will be further discussed during CR phase.

For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE.

When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

When current beam is configured to be included in the L1 MR MAC CE, 7 bits of RSRP is included and no RSRI is needed.

The current beam information of SpCell (if configured to be included) is always placed at the end of the normal (not truncated) MAC CE.

Same as Rel-18: When the C-LTM execution is triggered, if the TA of the target cell acquired by early RACH is valid, UE applies the TA acquired by early RACH, otherwise if UE has successfully measured the TA of the target cell, UE applies the measured TA.

FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

No need to define a separate capability for the reference configuration for inter-CU LTM.

Define per-UE capabilities for security key change of MCG LTM (e.g. ltm-KeyUpdate-MCG-r19) and security key change of SCG LTM (e.g. ltm-KeyUpdate-SCG-r19). UE supports these capabilities should also support ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18 respectively. No new UE capability on inter-CU MCG LTM with SN unchanged and inter-CU MCG LTM SN with SCG addition.

Define a per-band capability for L1 execution condition, e.g. cltm-ExecutionConditionL1-r19 is defined to indicate whether the UE supports L1 execution condition for CLTM and subsequent CLTM.

Define a per-band capability for L3 execution condition, e.g. cltm-ExecutionConditionL3-r19 is defined to indicate whether the UE supports L3 execution condition for CLTM and subsequent CLTM and whether the UE supports 2 trigger events for same execution condition.

A UE that supports conditional LTM should indicate the support for at least one of cltm-ExecutionConditionL3-r19 or cltm-ExecutionConditionL1-r19.

When a UE indicates support for both conditional LTM and ltm-RACH-LessCG-r18, it implies that the UE supports RACH-less conditional LTM with a configured grant. Whether/how to update the field description of ltm-RACH-LessCG-r18 can be addressed in the running CR review.

Rely on the R18 capability (i.e., ue-TA-Measurement-r18) to indicate whether UE supports UE-based TA measurement for C-LTM. There is no need to define a separate capability for this purpose.

Define a new per UE capability for UE support of early TA MAC CE reception for CLTM and also the max number of maintaining TA values. The value range is (1~8).

Rely on the R18 capability (i.e., ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18) to indicate whether the UE supports MAC-CE activated joint/separate LTM TCI states for CLTM.

Agreements on inter-CU LTM

- For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

- NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

- NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

- From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

- MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

- An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

Agreements on L1 event-triggered MR

- Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

- The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

- In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to).

- In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event). Introduce two bits per beam.

- RAN2 will consider only the following options:

- Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting.

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

- Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

- For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

Agreements on C-LTM

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

- For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

RAN2-130:

Agreements on stage-3 details

- 16 beams in total including the current beam in MR MAC CE.

- The beam offset for current beam of serving cell, i.e. Obs in the event evaluation inequality, is needed for the LTM event evaluation. Obs offset is configured per-cell, while applied at the level of indidual beam. Obs offset is applied to LTM3.

- The Semi-persistent CSI-RS resource set for candidate cell MAC CE is initially deactiated upon (re-)configuration by upper layer.

- RAN2 assumes there is no L1-SINR quantity for L1 event triggered MR.

- It is up to the NW configuration whether the UE can include measurements for LTM candidates in the MR MAC CE triggered by LTM2, i.e. based on the current configuration in RRC on the number of beams for LTM2 included in the MAC CE.

- For MR triggered by LTM2, if measurements for LTM candidates is configured to be included in the MR MAC CE, which LTM candidate beam should be included: Only RS(s) associated with the report ID for LTM2.

- The triggered MR is not cancelled when transmitting truncated MAC CE. The triggered MR is cancelled if regular MAC CE is reported, as in current MAC running CR.

- When a beam (or multiple beam) meets the entry condition for TTT, and before the UL grant occasion, this beam (or all these beams) meets the leaving condition, then, the corresponding MR should be cancelled.

- When a beam (or multiple beam) meets the leaving condition for TTT, and before the UL grant occasion, this beam (or all these beams) meets the entry condition, then, the corresponding MR should be cancelled.

- When MR is triggered, if the dedicated SR configuration for L1 measurement report MAC CE transmission is not configured, UE will trigger RACH procedure, as in legacy.

- Support the co-existence of CLTM and (e)RedCap.

- Support the co-existence of CLTM and CovEnh.

- (RRC-2) The appliedLTM-CandidateId is included in both MN and SN RRCReconfigurationComplete message (Rel18 specification should be okay).

- For RRC-3, keep current text in the running RRC CR.

- (RRC-6) In inter-MN LTM failure (LTM cell switch failure) the UE fall back to the previous source PCell configuration before selecting a cell (same as legacy procedure).

- (RRC-7) The legacy sk-counter is used for inter-MN LTM (this means sk-counter value can be re-used).

- (RRC-9) Use the term “LTM” for parts which are common to LTM and CLTM. Use the term “CLTM” only for parts which apply only to CLTM (but not LTM). This will be used in all RAN2 specifications.

- (RRC-10) Follow RAN1 agreement which indicate that at least one candidate RS (within the LTM CSI resource configuration) shall be configured for event LTM2.

Agreements on inter-CU LTM

- RAN2 agree to not support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell.

- If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

- RAN2 not support CSI-RS based CFRA resources indicated via LTM cell switch command MAC CE.

- The Enhanced LTM CSC MAC CE always contains an NCC value field. The Enhanced LTM CSC MAC CE is used when the R19 set IDs of the target cell and the source cell are different. Otherwise, the legacy LTM CSC MAC CE is used.

Agreements on L1 measurement triggered MR

- UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell.

- Instead of candidate cell id and SP CSI-RS resource set id, LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE.

- Confirm working assumption, i.e. four codepoints to represent one of {type1, type2, type3, type4}, as an agreement. For detailed description of type 1, it can be handled as part of running CR preparation.

- (MAC-6) For the truncated MR MAC CE, the UE determines the beam to be included in the MAC CE, based on the priority: Type 1 beam > Type 2 beam > Type 3 beam > Type 4 beam. If the UE cannot include all of the beams with the same priority, the beams are selected in the (truncated) MR MAC CE based on measured RS quality. Among all selected beams, the UE puts the best quality beam as the reference beam as the first one included in the (truncated) MR MAC CE.

- Working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

- Same priority between regular MR MAC CE and truncated MR MAC CE. It is up to UE implementation how to handle/include if more than one triggering events are pending and UL grant is not sufficient for all regular L1 MR MAC CEs.

- No prohibit timer is applied to L1 event-triggered MR.

Agreements on C-LTM

- (MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one. If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed.

- For L3 based CLTM, the UE performs RACH-based CLTM if there is no configured grant.

- Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed.

- During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM.

- In case CG resource is only associated with SSB, if the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS.

- RAN2 will not do anything for the separate TCI mode optimization.

#### 2.2.2	Remaining Open issues

- Inter-CU LTM procedure

  - Conclusion on handling of SP CSI-RS resources after cell switch.

  - Further signalling details and issues (if any) of inter-CU LTM

    - Without DC and with DC.

- Event triggered L1.

  - Remaining stage-3 details on signaling structures of event reporting using MAC CE (for eg., inclusion of current serving beam in MR).

- Support of intra-CU conditional LTM

  - Further details (if any) on the design of CLTM and on subsequent LTM in conditional LTM.

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3#127bis:

- BL CR to 37.340 (R3-251554), BL CR to 37.483 (R3-251555), BL CR to 38.401 (R3-251557), BL CR to 38.420 (R3-251553), BL CR to 38.423 (R3-251558), BL CR to 38.473 (R3-251559) and BL CR to 38.300 (R3-251556) are endorsed.

For inter-CU LTM:

- Reuse the LTM configuration update procedure to transfer UE’s 5G security capabilities and/or UE’s UP security policy from the new serving gNB to the other candidate gNBs in subsequent LTM.

- The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure.

- Introduce two new Class-1 UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s).

- Introduce a new class-1 UE-associated procedures over XnAP for the SP CSI-RS activation/deactivation.

- For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy.

- The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure.

- Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

- Include the following information in the TA information transfer message:

  - Candidate Cell ID

  - TA value

  - Preamble index

  - RA-RNTI

  - FFS on the XnAP IDs.

- Turn the WA into agreement: Use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB.

- For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB.

- Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s).

- TP for BLCR of TS38.300 (R3-252482), TP for BLCR of TS38.401 (R3-252438), TP for BLCR of TS38.473 (R3-252463), TP for BLCR of TS38.420 (R3-252444) and TP for BLCR of TS38.423(R3-252431) are agreed.

For inter-CU LTM in DC:

- Include the following information in SN Change Required (Source SN to MN) message:

  - LTM request indication (explicit way)

  - Suggested PScell list (implicit way in INM, FFS on explicit way)

  - Measurement results (implicit way in INM)

  - Upper limit number of PScell(s) for each candidate SN (FFS)

  - CSI Resource configuration request (explicit way)

  - Early sync information request (explicit way)

- Include the following information in SN Addition Request (MN to Candidate SN) message:

  - LTM request indication (explicit way)

  - Suggested PScell list (implicit way in INM, FFS on explicit way)

  - Measurement results (implicit way in INM)

  - Upper limit number of PScell(s) for the candidate SN (explicit way)

  - CSI Resource configuration request (explicit way)

  - Early sync information request (explicit way)

- It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM via SN modification request acknowledge message. Then the MN forwards the LTM configuration ID mapping list to the candidate SN via SN Modification Request message.

- For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s).

- The TA information transfer message can be reused and enhanced to transfer the TA information from the candidate SN to the source SN via the MN.

- Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message.

- TP for BLCR of TS37.340 (R3-252462), and TP for BLCR of TS38.423 for DC (R3-252461) are agreed.

For intra-CU Conditional LTM:

- For CLTM focus firstly on scenarios without CU-UP change.

- The gNB-DU signals the generated L1 execution conditions over F1AP for CLTM via a new IE in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages.

- Reuse the ACCESS SUCCESS message from Candidate DU to CU to inform the cell change.

- For Inter-DU CLTM, reuse the UE CONTEXT MODIFICATION REQUEST message from CU to Source DU to inform the cell change and stop data transmission.

- For CLTM, the existing LTM network signaling for LTM Cell Switch Completion phase is reused.

- TP for BLCR of TS38.473(R3-252435) and TPs for BLCR of TS38.401 (R3-252423) are agreed.

RAN3#128:

- BL CR to 37.340 (R3-253084), BL CR to 37.483 (R3-253085), BL CR to 38.401 (R3-253087), BL CR to 38.420 (R3-253088), BL CR to 38.423 (R3-253089), BL CR to 38.423 for DC (R3-253090), BL CR to 38.470 (R3-253091), BL CR to 38.473 (R3-253149) and BL CR to 38.300 (R3-253184) are endorsed.

For inter-CU LTM:

- WA: RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

- For Inter-CU LTM, LTM CONFIGURATION UPDATE procedure is per node level basis with a list of cells, and security key is per cell.

- Remove Note in TS 38.300 BL CR “Editor’s Note: step 6 and 7 are optional.”

- Update the TA INFORMATION TRANSFER message to provide a list of TA values of multiple candidate cells.

- CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE message.

- Reuse the CSI-RS coordination procedure over F1AP and XnAP for source gNB/gNB-DU to activate or deactivate the SP CSI-RS resource for CSI acquisition in candidate cell.

- In Xn interface, candidate gNB provides the LTM CFRA Resource Configuration of each candidate cell to source gNB for LTM cell switch command generation.

- Source gNB generate the UE Based TA Measurement Configuration, and transfer it to all candidate gNB(s) via LTM Configuration Update message.

- Include the Rel-19 set IDs of source cell and each candidate cell(s) in UE Context Modification Request message. Introduce Rel-19 set IDs into LTM Security Information IE.

- TP for BLCR of TS38.300 (R3-253926), TP for BLCR of TS38.401 (R3-253898), TP for BLCR of TS38.473 (R3-253895) and TP for BLCR of TS38.423(R3-253959) are agreed.

For inter-CU LTM in DC:

- Update the text description of CELL SWITCH NOTIFICATION message to capture the RAN3#127bis agreement: Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message, and reusing the existing IEs.

- The SCG reference configuration is provided by an implicit way in the CG-Config RRC container in the SN Change Required message.

- The SCG reference configuration is provided by an implicit way in the CG-ConfigInfo RRC container in the SN Addition Request message from the MN to the candidate SN.

- A per cell indicator indicating complete candidate configuration is included in the SN Addition Request Acknowledge message.

- The SN Change Required message and the SN Modification Request message from MN to source SN should design a mechanism to support multiple candidate SNs, i.e., SN can include multiple candidate SNs information in a single SN Change Required message, and MN includes multiple candidate SNs information in a single SN Modification Request message.

- The SN Addition Request message should design a mechanism to support single candidate SN, i.e., MN sends SN addition request towards only one candidate SN.

- The suggested PSCell list should be explicitly included in the SN Change Required message.

- The maximum number of PSCells that can be prepared by each candidate SN is included in the SN Change Required message.

- The LTM Configuration ID Mapping List IE may be included in the SN Change Required message and the SN Addition Request message.

- The LTM Security Configuration List IE including a list of pair of {security key, sk-counter} is included in the SN Addition Request message and the SN Modification Request message to support subsequent inter-CU SCG LTM.

- The SN Security Key IE included in the SN Addition Request message should be ignored if the procedure is triggered for the LTM.

- WA: Different candidate PSCells in the same SN can have different Rel-19 set IDs.

- If the source SN has the SCG reference configuration, the source SN will provide the SCG reference configuration in the SN Change Required message and thus the MN will not request source SN to provide the SCG reference configuration.

- Early data forwarding is supported in inter-CU LTM in DC.

- Normal data forwarding can be initiated after the source SN decides to trigger the LTM cell switch for the UE. The exact timing of its initiation is left to implementation.

- Enhance XN-U ADDRESS INDICATION message and define IE to cover Inter-CU MCG LTM case.

- MN uses SN modification request message to notify the Source SN that UE has successfully accessed to the target SN.

- The source SN generates the common LTM CSI resource configuration for inter-CU SCG LTM and then provides the common LTM CSI resource configuration to the MN via the SN Modification Request ACK message.

- For the SSB based L1 measurement, the common CSI resource configuration may be included in the SN Change Required message and the SN Addition Request message.

- The CSI Resource Configuration IE is the common CSI resource configuration, which refers to the ltm-CSI-ResourceConfig-r18 IE in the RRC spec.

- The L1 Configuration IE is the L1 RS configuration per candidate PSCell, which refers to the ltm-SSB-Config-r18 IE in the RRC spec for the SSB based L1 measurement, or the ltm-NZP-CSI-RS-ResourceConfigToAddModList-r19 IE associated with the ltm-NZP-CSI-RS-ResourceSetToAddModList-r19 IE in the RRC spec for the CSI-RS based L1 measurement.

- For the avoidance of simultaneous configurations of inter-CU MCG LTM and inter-CU SCG LTM, no RAN3 impact is foreseen.

- The SP CSI-RS activation/(de)activation for the inter-CU LTM in DC is deprioritized in rel-19.

- TP for BLCR of TS37.340 (R3-253922) and TPs for BLCR of TS38.423 for DC (R3-253904, R3-253850) are agreed.

For intra-CU Conditional LTM:

- RAN3 confirms that the network can trigger an LTM Cell Switch Command MAC CE towards a candidate cell with C-LTM candidate configuration.

- The candidate (initial source) DU need to know the L1 C-LTM and the L3 C-LTM.

- The DU needs to know to which other candidate cell(s) to generate the L1 execution condition for L1 C-LTM.

- WA: To introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”.

- WA: To introduce a new IE with a list of candidate cells for L1 execution condition.

- It is acknowledged that the C-LTM indicator needs to be sent to the initial source DU for the very first C-LTM.

- The candidate DU generates the TAT for its own candidate cells.

- TP for BLCR of TS38.473 (R3-253960) and TPs for BLCR of TS38.401 (R3-253910) are agreed.

#### 2.3.2	Remaining Open issues

For inter-CU LTM:

- FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2:

  - Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

  - Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different.

- FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB.

- FFS on whether gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

- FFS on whether the TCI State/QCL-info List needs to be included in CSI-RS COORDINATION procedure.

- FFS on whether to include the SP CSI-RS and SSB mapping in the HANDOVER REQUEST ACKKNOLEDGE message and UE Context Modification Request message.

- FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP.

For inter-CU LTM in DC:

- FFS on whether to provide the SCG reference configuration by an implicit way in the SN Addition Request Acknowledge message from the source SN/candidate SN to the MN.

- FFS on the solutions on Rel-19 Set ID(s) assignment in DC scenario. Try to reuse the same solution for inter-CU LTM.

- FFS on whether to enhance XN-U ADDRESS INDICATION message for inter-SN LTM case.

- FFS on whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN.

- FFS on whether LTM modification/cancel related procedures are needed in DC scenario.

For intra-CU Conditional LTM:

- FFS on whether it is needed that for each candidate cell, the CU indicates to the source DU and the candidate DUs in the LTM Configuration ID Mapping List the need of generating L1 execution condition.

- Whether needs to indicate from CU to source DU and candidate DU when the cell is configured with L3 CLTM, separately from L1 CLTM.

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#114bis

Event triggered L1 measurement reporting

Issue 2-2: Event triggered reporting delay for L1 reporting

- Reuse the framework from L3 event triggered reporting. Discuss the details further.

Issue 2-5: whether and how to coordinate with R19 MIMO

- RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

Issue 2-6: L1 filtering assumption in event evaluation

- No L1 filter is assumed for requirements in event triggered L1 measurement reporting in mobility.

Issue 2-7: Impact on other LTM requirements

- For FR1:

  - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command,

    - Case 1: leaving condition of the event is configured and corresponding condition is not met,

      - the related cell switch delay and TCI state activation requirements are applicable

    - Case 2: leaving condition of the event is NOT configured, FFS

- For FR2: FFS

Issue 2-8: which result to report if UE has chance to update measurement after report is triggered and before report is actually sent

- Up to UE implementation.

CSI-RS based L1 measurement

Issue 4-1-1: Whether to use detectable or measurable?

- Confirm the RAN4#114 tentative agreement “Use measurable and specify the side condition for RS being measurable”

Issue 4-1-2: Side condition for CSI-RS based L1 measurement for neighbor cell

- Use legacy side condition of SINR=-3dB.

Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2

- Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting.

- Further discuss whether step 2 can be skipped for serving cell or not.

Issue 4-2-2: How to interpretate the UE capability (for FR2-1)

- For a UE not supporting the capability, Step 2 is necessary.

  - The CSI-RS resources in Step 3 should be at least Type-D QCL’ed with the SSB in Step 2.

  - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

- For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped:

  - The CSI-RS resources in Step 3 should be Type-D QCL’ed with the SSB in Step 1.

  - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

  - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor.

Issue 4-3: Definition of “inside/outside active BWP”

- RRM requirements (delay and accuracy) apply under the following condition:

  - At least 48 RBs of the CSI-RS configured for measurement is within the active BWP.

Issue 4-5: Measurement on CSI-RS with repetition ON/OFF

- In FR2, only consider CSI-RS with repetition OFF in RAN4 requirements.

Issue 4-6: measurement GAP and RTD assumption

- From RAN4 perspective,

  - Not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell.

  - Not to define RAN4 requirements for RTD > CP.

Issue 4-7: CSI-RS resource type

- Define RRM requirement for CSI-RS based L1 measurement requirement for periodic CSI-RS and semi-persistent CSI-RS.

Issue 4-10: Definition of frequency layer

- No need to introduce definition of frequency layer.

Issue 4-11: Whether to consider CSI-RS based L1-SINR measurement

- RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

Conditional LTM

Issue 1-2-1: General aspects of Conditional LTM delay

  - CHO requirements framework can be used as baseline for Conditional LTM.

Issue 1-2-2: The starting point of conditional LTM delay

  - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

Issue 1-2-4: Tinterrupt

  - Keep either TCLTM_execution or TCLTM-RRC-processing as one component for the conditional LTM delay.

    - Note: The legacy value(s) in previous releases will be reused.

  - TCLTM_execution or TCLTM-RRC-processing is not included in Tinterrupt.

  - Not include the components for TCI state activation and/or PDCCH order PRACH in C-LTM delay.

    - TEvent_DU can be updated based on RAN2 progress if needed.

  - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell.

    - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed.

      - Measurement latency for both serving and candidate cells need to be considered.

    - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO.

    - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19.

Issue 1-2-15: Other

- L1 measurement behaviour, which was introduced in R18, is not changed. CLTM requirements do not apply for the candidate cells without active TCI states after TCI state activation for at least one candidate cell in FR2.

Issue 1-3-2: The starting point of Subsequent conditional LTM delay

- The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

RAN4#115

Event triggered L1 measurement reporting

Issue 2-2: Event triggered reporting delay for L1 reporting

- The event triggered measurement reporting delay shall be no larger than the maximum of L1-RSRP measurement periods on the cells corresponding to the event.

  - If serving cell is involved in event evaluation, L1-RSRP measurement period of serving cell refers to TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS respectively for SSB based and CSI-RS based L1-RSRP measurement, as defined in 9.5, assuming TReport = 0

  - If neighbor cell is involved in event evaluation:

    - L1-RSRP measurement period of neighbor cell refers to TL1-RSRP_Measurement_Period_SSB_intra  for SSB based intra-frequency measurement, as defined in 9.14, assuming TReport = 0.

    - L1-RSRP measurement period of neighbor cell refers to TL1-RSRP_Measurement_Period_SSB_inter  for SSB based inter-frequency measurement, as defined in 9.15, assuming TReport = 0.

    - L1-RSRP measurement period of neighbor cell refers to [TL1-RSRP_Measurement_Period_CSI_intra] for CSI-RS based intra-frequency measurement, as defined in [9.x], assuming TReport = 0.

CSI-RS based L1 measurement

Issue 3-1: definition on CSI-RS based intra-frequency L1 measurement

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and

  - It is applied for SCS = 60KHz

- at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP.

Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement.

- RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19.

Issue 3-4: Scaling factor for Rx beam sweeping

- For neighbor cell CSI-RS based L1-RSRP measurement in FR2, if SSB based measurement is skipped, the measurement period for CSI-RS based L1-RSRP will be scaled by 8.

Issue 3-8: Requirement applicability

- For intra-f CSI-RS based L1-RSRP measurement on neighbor cell (when RTD<CP) in FR1, reuse legacy measurement period, measurement restriction and scheduling restriction requirements as baseline.

- For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define scheduling restriction.

RAN4 UE feature list for CSI-RS based L1 measurement

- Number of CSI-RS resources for L1-RSRP measurement within a slot

- Number of total CSI-RS resources to be measured

- Skip SSB based L1-RSRP measurement for [neighboring or candidate] cell CSI-RS-based L1-RSRP measurement

- FFS: Number of CSI-RS resources for L1-RSRP measurement per serving carrier

Conditional LTM

Issue 1-1-1: L1 measurement on deactivated Scell and UE based TA measurement

- Not define requirements for L1 measurement on deactivated Scell.

- Not define requirements for UE based TA measurement.

Issue 1-2-1: TRRC

- TRRC is the RRC procedure delay.

Issue 1-2-2: Tmeasure

In C-LTM delay, Tmeasure is measurement time. It is defined from the end of TEvent_DU until UE starts TCLTM-RRC-processing to a target cell.

For SSB based L1-RSRP measurement:

- Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed.

- Measurement latency for both serving and candidate cells need to be considered, and follow the definition of measurement delay in the event-triggered L1 reporting delay discussed under the corresponding agenda item.

For SSB based L3-RSRP measurement:

- Reuse Tmeasure in CHO.

For CSI-RS based L1-RSRP measurement:

- For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period.

- Further discuss the conditions for TCSI-RS_SFN_intra = 0.

- Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS.

For CSI-RS based L3-RSRP measurement:

- RAN4 does not define CLTM requirements for the case where CSI-RS based L3-RSRP measurements are used in the execution condition evaluation.

Issue 1-2-3: TLTM-RRC-processing /TCLTM_execution

  - TCLTM-RRC-processing is used in C-LTM delay, discuss the description of TCLTM-RRC-processing in draftCR directly.

Issue 1-2-4: General on Tfirst-RS and TRS-proc

  - [Tfirst-RS and TRS-proc] are included in Tinterrupt.

Issue 1-2-5: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0

  - Tfirst-RS = 0 and TRS-proc = 0 if the target TCI state is on the serving cell or LTM candidate cell active TCI state list or is associated with a TCI state on the active TCI state list.

    - Note: the limitation of 160/480 ms from Rel-18 still applies.

Issue 1-2-6: TLTM-processing

  - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

    - Reuse Rel-18 LTM values for TLTM-processing in the CLTM cell switch delay.

Issue 1-2-7: TLTM-IU

- TLTM-IU is the interruption uncertainty during LTM cell switch.

    - For RACH-based LTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell.

    - For RACH-less LTM cell switch, TLTM-IU_is the uncertainty on transmitting the first uplink transmission on the target cell.

Issue 1-3-1: General aspects of Subsequent Conditional LTM delay

  - Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

    - TRRC will not be considered in subsequent conditional LTM delay.

    - TEvent_DU needs to be updated

Issue 1-3-2: The components of subsequent conditional LTM delay

- Since issue 1-3-1 has been agreed, this issue is closed.

Issue 1-3-3: Other

- RAN4 specifies the initial CLTM and subsequent CLTM delay separately.

#### 2.4.2	Remaining Open issues

Event triggered L1 measurement reporting

Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category

Issue 2-4: Impact on other LTM requirements

Performance part

CSI-RS L1 RSRP measurement

Issue 3-2: whether step 2 is skipped for serving cell

Issue 3-3: Sharing factor for resource collision between serving and neighbour cell

Issue 3-6: Measurement restriction

Issue 3-7: Measurement with repetition ON/OFF

Issue 3-9: early CSI acquisition

Performance part

Conditional LTM

Performance part

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RP-250338	Status Report for Work Item: NR mobility enhancements Phase 4 (rapporteur: Apple)

RP-250339	Revised Work Item: NR mobility enhancements Phase 4

RAN1#120bis:

R1-2502627	Higher layer parameters for Rel-19 NR mobility enhancements Ph4	Rapporteur (Apple)

R1-2501756	Discussion on measurements related enhancements for LTM	LG Electronics

R1-2501784	Discussion on measurements related enhancements for LTM	ZTE Corporation, Sanechips

R1-2501821	Discussion on measurements related enhancements for LTM	vivo

R1-2501849	Discussion on LTM enhancements	TCL

R1-2501879	Discussion on measurements related enhancements for LTM	Spreadtrum, UNISOC

R1-2501956	Discussion on measurement related enhancements for LTM	Lekha Wireless Solutions

R1-2502006	Discusssions on measurements related enhancements for LTM	CATT

R1-2502087	FL plan for mobility enhancements in RAN1#120bis	Moderator (Fujitsu)

R1-2502088	FL summary 1 of Measurements related enhancements for LTM	Moderator (Fujitsu)

R1-2502090	FL summary 3 of Measurements related enhancements for LTM	Moderator (Fujitsu)

R1-2502091	Final FL summary of Measurements related enhancements for LTM	Moderator (Fujitsu)

R1-2502111	Measurements related enhancements for LTM	Lenovo

R1-2502132	Discussion on measurement related enhancements for LTM	Fujitsu

R1-2502172	Discussion on measurements related enhancements for LTM	CMCC

R1-2502201	Discussion on measurements related enhancements for LTM	NEC

Late submission

R1-2502206	Measurements related enhancements for LTM	InterDigital, Inc.

R1-2502214	Measurements related enhancements for LTM	Huawei, HiSilicon

R1-2502298	Discussions on measurement enhancement for LTM	OPPO

R1-2502329	Measurements related enhancements for LTM	Sony

R1-2502382	Views on Rel-19 measurement related enhancements for LTM	Samsung

R1-2502453	Discussion on measurements related enhancements for LTM	Xiaomi

R1-2502490	Discussion on measurements related enhancements for LTM	Sharp

R1-2502518	Discussion on measurements related enhancements for LTM	ETRI

R1-2502562	Measurement related enhancements for LTM	Ericsson

R1-2502628	Measurement related enhancements for LTM	Apple

R1-2502653	Measurement related enhancements for LTM	Nokia

R1-2502749	Discussion on measurements related enhancements for LTM	KDDI Corporation

R1-2502778	Discussion on measurement related enhancements for LTM	NTT DOCOMO, INC.

R1-2502853	Measurements related enhancement for LTM	Qualcomm Incorporated

R1-2502891	Discussion on measurements related enhancements for LTM	Google

R1-2502929	LTM measurements related enhancements	MediaTek

R1-2502088	FL summary 1 of Measurements related enhancements for LTM	Moderator (Fujitsu)

R1-2503076	[Draft] Reply LS on number of beam measurements in the measurement report MAC CE	Moderator (Fujitsu)

R1-2503077	Reply LS on number of beam measurements in the measurement report MAC CE	RAN1

R1-2503078	[Draft] LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells	Moderator (Fujitsu)

R1-2503079	LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells RAN1

R1-2502089	FL summary 2 of Measurements related enhancements for LTM	Moderator (Fujitsu)

R1-2502090	FL summary 3 of Measurements related enhancements for LTM	Moderator (Fujitsu)

RAN1#121:

R1-2503259	Measurements related enhancements for LTM	Huawei, HiSilicon

R1-2503376	Remaining issues on measurements related enhancements for LTM	vivo

R1-2503419	Discussion on measurements related enhancements for LTM	NEC

R1-2503422	Measurement related enhancements for LTM	Nokia

R1-2503428	Discussion on measurements related enhancements for LTM	LG Electronics

R1-2503447	Measurements related enhancements for LTM	TCL

R1-2503526	Discussion on measurements related enhancements for LTM	Spreadtrum, UNISOC

R1-2503580	Views on Rel-19 measurement related enhancements for LTM	Samsung

R1-2503672	Discussion on measurements related enhancements for LTM	ZTE Corporation, Sanechips

R1-2503713	Discussion on measurement related enhancements for LTM	Lekha Wireless Solutions

R1-2503809	Remaining issues on measurements related enhancements for LTM	CATT

R1-2503843	Discussion on measurements related enhancements for LTM	CMCC

R1-2503862	FL plan for mobility enhancements in RAN1#121	Moderator (Fujitsu)

R1-2503866	Final FL summary of Measurements related enhancements for LTM	Moderator (Fujitsu)

R1-2503894	Discussion on measurements related enhancements for LTM	Xiaomi

R1-2503916	Measurements related enhancements for LTM	Lenovo

R1-2503949	Measurements related enhancements for LTM	InterDigital, Inc.

R1-2504004	Discussion on measurement related enhancements for LTM	Fujitsu Limited

R1-2504148	Discussion on measurements related enhancements for LTM	ETRI

R1-2504184	Measurement related enhancements for LTM	Ericsson

R1-2504233	Discussions on measurement enhancement for LTM	OPPO

R1-2504300	Discussion on measurements related enhancements for LTM	Google

R1-2504340	Measurement related enhancements for LTM	Apple

R1-2504408	Measurements related enhancement for LTM	Qualcomm Incorporated

R1-2504450	Discussion on measurements related enhancements for LTM	KDDI Corporation

R1-2504513	Discussion on measurement related enhancements for LTM	NTT DOCOMO, INC.

R1-2504546	Discussion on measurements related enhancements for LTM	Sharp

R1-2504595	LTM measurements related enhancements	MediaTek Inc.

R1-2503863	FL summary 1 of Measurements related enhancements for LTM	Moderator (Fujitsu)

R1-2503864	FL summary 2 of Measurements related enhancements for LTM	Moderator (Fujitsu)

R1-2503865	FL summary 3 of Measurements related enhancements for LTM	Moderator (Fujitsu)

R1-2504889	FL summary 4 of Measurements related enhancements for LTM	Moderator (Fujitsu)

RAN2#129bis:

R2-2501709	LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells (R1- 2501500; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2	Cc:RAN3

R2-2501715	LS on L1filtering for LTM event triggered reporting (R1-2501577; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2

R2-2501756	Reply LS on security handling for inter-CU LTM in non-DC cases (S3-251124; contact: CATT)	SA3	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2, RAN3

R2-2502198	Observations on some topics related to Rel-19 Mobility Enh WI	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502197	Report of [POST129][106][Mob] Stage-2 CR 38.300 (Apple)	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502196	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple	draftCR	Rel-19	38.300	18.5.0	B	NR_Mob_Ph4-Core

R2-2503154	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple	draftCR	Rel-19	38.300	18.5.0	B	NR_Mob_Ph4-Core

R2-2502234	Introduction of NR mobility enhancements Phase 4 in TS 37.340	China Telecom	draftCR	Rel-19	37.340	18.5.0	B	NR_Mob_Ph4-Core

R2-2502534	Running RRC CR for inter-CU and conditional LTM	Ericsson	draftCR	Rel-19	38.331	18.5.1	B	NR_Mob_Ph4-Core

R2-2502535	List of FFSs for LTM and CLTM	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503001	Summary of offlines [AT129b][102][MOB] and [AT129b][110][MOB]	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502095	Summary of [POST128][108][MOB] RRC running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal1: Discuss whether ltm-CSI-RS-ReportConfigToAddModList should only be configured on SpCell.

R2-2502096	Draft running CR for event-triggered meas report for RRC	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.1	B	NR_Mob_Ph4-Core

R2-2503002	Summary of [AT129b][103][MOB] RRC running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502158	Running MAC CR for Mob Ph4	vivo (Rapporteur)	draftCR	Rel-19	38.321	18.5.0	B	NR_Mob_Ph4-Core

R2-2503003	Discussion report on [AT129bis][104][MOB] MAC open issues (vivo)	Vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501900	Report of [POST129][111][MOB] (CATT)	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503011	Report of [POST129][111][MOB] (CATT)	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502256	Draft 306 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.306	18.5.0	NR_Mob_Ph4-Core

R2-2502257	Draft 331 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.331	18.5.1	NR_Mob_Ph4-Core

R2-2501835	Further discussion on inter-CU LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501887	Discussion on open issues for inter-CU LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501897	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501929	Security discussion and other remaining issues of inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501977	Discussion on inter-CU LTM	Google	discussion	Rel-19

R2-2501995	Remaining issues of inter-CU LTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502017	Remaining Issues for Inter-CU LTM	Qualcomm Incorporated	discussion

R2-2502045	On remaining issues for Inter-CU LTM	Nokia	discussion

R2-2502066	Discussion on inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502120	Discussion on Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502121	Discussion on Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502122	Discussion on Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502135	Discussion on Inter-CU LTM security and fast recovery	InterDigital, Europe, Ltd.	discussion	Rel-19

R2-2502159	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502235	Discussion on inter-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502285	Discussion on inter-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502299	Discussion on inter-CU LTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502340	Discussion on inter-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502359	Remaining issues for Inter-CU LTM	Lenovo	discussion	Rel-19

R2-2502381	Discussion on issues for supporting inter-CU LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502423	Further discussion remaining issues of inter-CU LTM cell switch	Transsion Holdings	discussion	Rel-19

R2-2502444	Discussion on security for inter-CU LTM	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2502481	Discussion on inter-CU LTM	NTT DOCOMO, INC.	discussion	Rel-19

R2-2502490	LTM Resource consumption for the target cells	Sony	discussion	Rel-19	NR_Mob_Ph4

R2-2502509	Discussion on remaining issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502601	Discussion on open issues for inter-CU LTM	Ofinno, LLC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502683	Discussion on security handling for inter-CU LTM	KT Corp.	discussion

R2-2502727	Discussion on Inter-CU LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502852	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502944	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502965	Discussion on inter-CU LTM	DENSO CORPORATION	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502774	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501836	Discussion on measurement event evaluation and report	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501879	Remaining issues of L1 event triggered measurement reporting	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501888	Open issues for event triggered  L1 measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501898	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501930	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502019	L1 event-triggered measurement reporting for LTM	Qualcomm Incorporated	discussion

R2-2502032	Discussions on measurement related enhancements for LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502094	Discussion on event-triggered meas report	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502133	Discussion on L1 event triggered measurement reporting	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502160	Discussion on measurement event evaluation and reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502236	Discussion on L1 event triggered measurement reporting	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502311	Discussion on L1 event triggered  measurement	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502321	Details of MAC CE for event-triggered L1 measurement report	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502341	Discussion on L1 event triggered measurement reporting	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502360	Event based L1 measurement report	Lenovo	discussion	Rel-19

R2-2502382	Discussion on issues for supporting L1 event triggered measurement reporting	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502424	Discussion on L1 event triggered measurement reporting	Transsion Holdings	discussion	Rel-19

R2-2502433	Discussion on L1 event triggered measurement reporting	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502445	Discussion on event-based L1 measurements	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2502552	Further Details on L1 Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core	R2-2500332

R2-2502602	Discussion on L1 event triggered measurement report	Ofinno, LLC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502606	LTM event-triggered measurement reporting 	Fraunhofer HHI, Fraunhofer IIS	discussion

R2-2502721	Discussion on L1 event triggered measurement reporting	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502799	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502823	Discussion on MR MAC CE for LTM	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core	R2-2500417

R2-2502831	Discussion on L1 event triggered measurement reporting for LTM	KDDI Corporation	discussion	Rel-19

R2-2502950	L1 event triggered measurement report 	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

R2-2501826	Discussion on Conditional Intra CU LTM	Lekha Wireless Solutions	discussion	Rel-19

R2-2501837	Discussion on conditional LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501889	Discussion on conditional LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501899	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501931	Further discussion on Conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501996	Remaining issues of CLTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502018	Conditional intra-CU LTM	Qualcomm Incorporated	discussion

R2-2502033	Discussion on conditional Intra-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502067	Discussion on conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502118	Conditional LTM.	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502119	Remaining issues of Conditional LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502161	Discussion on conditional LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502237	Discussion on conditional intra-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502287	Remaining open issues for CLTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502289	Discussion on Conditional intra-CU LTM remaining issues	Baicells Technologies Co. Ltd	discussion

R2-2502312	Discussion on conditional Intra-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502342	Discussion on conditional intra-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502383	Discussion on issues for supporting conditional LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502396	CLTM Scenarios and remaining points	Lenovo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502399	Discussion on coexistence of conditional LTM and LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2502425	Discussion on Early TA handling	Transsion Holdings	discussion	Rel-19

R2-2502438	Discussion on conditional LTM	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502446	Discussion on Conditional LTM	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2502603	Discussion on open issues for conditoinal LTM	Ofinno, LLC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502728	Discussion on Conditional LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502748	Remaining issues on conditional LTM	ETRI	discussion	Rel-19

R2-2502756	Discussion on Conditional intra-CU LTM	ITL	discussion	Rel-19

R2-2502800	Remaining Issues of Conditional Intra-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502824	Discussion on early TAT handling for CLTM	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502846	Considerations on CLTM TAT handling 	Kyocera 	discussion	Rel-19

R2-2502878	On conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

R2-2502945	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

RAN2#130:

R2-2503310	Reply LS on number of beam measurements in the measurement report MAC CE (R1-2503077; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2

R2-2503311	LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells (R1-2503079; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2, RAN3

R2-2503320	Reply LS for Reply LS on security handling for inter-CU LTM in non-DC cases (R3-252449; contact: CATT)	RAN3	LS in	Rel-19	NR_Mob_Ph4-Core	To:SA3	Cc:RAN2

R2-2503412	Draft 306 running CR for UE capability for Mob Ph4	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503413	Draft 331 running CR for UE capability for Mob Ph4	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503616	Running MAC CR for Mob Ph4	vivo (Rapporteur)	draftCR	Rel-19	38.321	18.5.0	B	NR_Mob_Ph4-Core

R2-2503766	Discussion summary and list of MAC open issue for Mob Ph4	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504701	Discussion report on [AT130][101][MOB] MAC open issues (vivo)	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503785	Introduction of event-triggered meas report for RRC spec	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.1	B	NR_Mob_Ph4-Core

R2-2504125	Running RRC CR for inter-CU LTM and CLTM	Ericsson	draftCR	Rel-19	38.331	18.5.1	B	NR_Mob_Ph4-Core

R2-2503786	Summary of [POST128][116][MOB] RRC running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504126	RRC open issues list for mobility	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504702	Summary of [AT130][102][MOB]	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503732	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple	draftCR	Rel-19	38.300	18.5.0	B	NR_Mob_Ph4-Core

R2-2503733	Report of [POST129b][119][Mob] Stage-2 CR 38.300 (Apple)	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503679	Introduction of NR mobility enhancements Phase 4 in TS 37.340	China Telecom	draftCR	Rel-19	37.340	18.5.0	B	NR_Mob_Ph4-Core

R2-2503680	List of 37.340 open issues for R19 mobility	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503381	Some issues for Inter-CU LTM	Lenovo Information Technology	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503409	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503479	Fast recovery for inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503486	Discussion on open issues for inter-CU LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503532	Remaining issues of inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503617	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503681	Discussion on remaining issues of inter-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503744	Discussion on issues for supporting inter-CU LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503747	Discussion on inter-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503804	Discussion on inter-CU LTM	Qualcomm Incorporated	discussion

R2-2503822	Discussion on inter-CU LTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503858	Inter-CU LTM and Fast Recovery	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2503898	Discussion on inter-CU LTM	DENSO CORPORATION	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504027	Discussion on inter-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504043	Discussion LTM fast recover of inter-CU LTM cell switch	Transsion Holdings	discussion	Rel-19

R2-2504048	Remaining issues on inter-CU LTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504137	Further remaining issues for Inter-CU LTM	Nokia	discussion

R2-2504145	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504168	Remaining issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504194	Discussion on Inter-CU LTM fast recovery	InterDigital Washington DC	discussion	Rel-19

R2-2504262	Discussion on open issues of inter-CU LTM	Ofinno	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504330	Remaining issues of Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2504389	Discussion on Inter-CU LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504423	Remaining open issues on inter-CU LTM	ETRI	discussion	Rel-19

R2-2504516	Discussion on inter-CU LTM	NTT DOCOMO, INC.	discussion	Rel-19

R2-2504542	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503410	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503435	Remaining issues of L1 event triggered measurement reporting	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503469	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503484	Discussion on L1 measurement	Lenovo	discussion	NR_Mob_Ph4-Core

R2-2503487	Open issues for L1 event triggered measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503554	Discussions on L1 event triggered measurement reporting	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503618	Discussion on LTM measurement event evaluation and reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503682	Discussion on remaining issues of L1 event triggered measurement reporting	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503745	Discussion on issues for supporting L1 event triggered measurement reporting	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503772	Discussion on L1 event triggered measurement reporting	Baicells Technologies Co. Ltd	discussion

R2-2503802	Discussion on event triggered L1 measurement reporting for LTM 	Qualcomm Incorporated	discussion

R2-2503812	Remaining issues of LTM measurement	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503859	Discussion on L1 event-based measurements	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2503861	Discussion on truncated L1 measurement reporting	Panasonic	discussion

R2-2503989	Remaining issues on MR MAC CE	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504028	Discussion on L1 event triggered measurement reporting	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504044	Discussion on L1 event triggered measurement reporting	Transsion Holdings	discussion	Rel-19

R2-2504115	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504120	Discussion on the support of mTRP in event-triggered LTM	Huawei, HiSilicon, Nokia, NTT Docomo, OPPO, ZTE	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504135	Final Details on L1 Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504181	LCP for LTM measurement report MAC CE	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504263	Discussion on L1 event triggered measurement report	Ofinno	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504331	Remaining issues of L1 event triggered measurement reporting	Rakuten Mobile, Inc	discussion	Rel-19

R2-2504402	Discussion on L1 event triggered measurement reporting	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504408	Remaining issues of event-triggered L1 measurement reporting for LTM 	Kyocera 	discussion

R2-2504422	Discussion on L1 event triggered measurement reporting for LTM	KDDI Corporation	discussion	Rel-19

R2-2504445	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504472	Discussion on measurement event evaluation and report	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504495	Discussion on event-triggered L1 MR MAC CE	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504610	Discussion on LTM event-triggered measurement reporting 	Fraunhofer HHI, Fraunhofer IIS	discussion

R2-2504639	Resolving MAC open issues for LTM event triggered MR	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

R2-2503382	CLTM Scenarios and remaining points	Lenovo Information Technology	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503411	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503455	Discussion on Conditional Intra CU LTM	Lekha Wireless Solutions	discussion	Rel-19

R2-2503470	Further discussion on conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503488	Open issues for conditional LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503533	Discussion on conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503555	Discussion on conditional Intra-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503619	Discussion on conditional LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503683	Discussion on remaining issues of CLTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503746	Discussion on issues for supporting conditional LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503773	Discussion on Conditional intra-CU LTM remaining issues	Baicells Technologies Co. Ltd	discussion

R2-2503803	Discussion on intra-CU conditional LTM 	Qualcomm Incorporated	discussion

R2-2503813	Remaining issues of conditional LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503860	Open issues for Conditional LTM	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2503931	Remaining Open issues for CLTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503956	Discussion on conditional LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2504029	Discussion on conditional intra-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504045	Discussion on remaining issues of conditional intra-CU LTM	Transsion Holdings	discussion	Rel-19

R2-2504049	Remaining issues on CLTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504146	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504182	Conditional LTM.	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504269	Discussion on open issues of CLTM	Ofinno	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504283	Discussion on conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

R2-2504329	Remaining issues of Conditional LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2504390	Discussion on conditional LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504412	Remaining issue of Conditional intra-CU LTM 	Kyocera 	discussion	Rel-19

R2-2504424	Remaining open issues on conditional LTM	ETRI	discussion	Rel-19

R2-2504446	Remaining Issues of Conditional Intra-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504473	Discussion on conditional LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504496	Discussion on TA handling for C-LTM	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504531	Discussion on Conditional intra-CU LTM	ITL	discussion	Rel-19

RAN3 #127

- R3-251507	LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells	RAN1(Fujitsu)

- R3-251525	Reply LS on security handling for inter-CU LTM in non-DC cases	SA3(CATT)

- R3-251533	TP (BL CR TS 38.300) Inter-CU LTM Procedure	Nokia

- R3-251534	Discussion on Conditional LTM in disaggregated gNB architecture	Nokia

- R3-251553	(BL CR to 38.420) Support for Inter-CU LTM	ZTE Corporation, China Telecom, Samsung, Nokia, CATT, NEC, LG Electronics, Ericsson, Huawei, Lenovo

- R3-251554	(BL CR to 37.340) stage 2 for inter-CU LTM in NR-DC	CATT, China Telecom, Huawei, Nokia, LG Electronics, Google, Samsung, Ofinno, Ericsson, Lenovo, NEC, ZTE

- R3-251555	(BL CR to 37.483) Introducing Rel-19 Mobility enhancement	LG Electronics Inc., Nokia, China Telecom, Google, Ericsson, CATT, Qualcomm, Samsung, CMCC, ZTE, Huawei, NTT Docomo, Lenovo, NEC, Ofinno, Jio Platforms (JPL)

- R3-251556	(BL CR to 38.300) Support for Inter-CU LTM	Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung, Offino

- R3-251557	(BL CR to 38.401) on conditional intra-CU LTM	China Telecom, ZTE Corporation, NEC, Samsung, Nokia, Google, Huawei, Ericsson, LG Electronics

- R3-251558	(BL CR to 38.423) Xn support for inter-CU LTM	Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno

- R3-251559	(BL CR to 38.473) Inter-CU LTM	Huawei, Nokia, Samsung, Google, NEC, China Telecom, Ericsson, LG Electronics, CATT, Ofinno, ZTE, Lenovo

- R3-251582	Discussion on Inter-CU LTM security	Nokia

- R3-251629	Data forwarding for Inter-CU LTM DC scenarios	Ofinno, LLC

- R3-251630	Issues on SN initiated inter-CU SCG LTM	Ofinno, LLC

- R3-251633	Discussion on Inter-CU LTM	Rakuten Mobile, Inc

- R3-251670	(TP to BL CR for TS 38.423 on Inter-CU LTM) Rel-19 inter-CU LTM issues	NEC

- R3-251671	discussion Rel-19 inter-CU LTM in DC scenario	NEC

- R3-251672	(TP to BL CR for TS 38.401and 38.473 on conditional intra-CU LTM) Discussion of Rel-19 intra-CU Conditional LTM	NEC

- R3-251707	(TP to BL CR 38.423) Support subsequent mobility in inter-CU LTM	Google

- R3-251741	(TP for NR_Mob_Ph4 TS 38.473) Discussions on key distribution and NCC delivery for Inter-CU LTM	LG Electronics Inc.

- R3-251742	Discussions on key remaining aspects of Inter-CU LTM	LG Electronics Inc.

- R3-251743	(TP for TS 38.401 and TS 38.473) Discussions on RAN3 impacts from Conditional Intra-CU LTM	LG Electronics Inc.

- R3-251759	Signalling enhancements for Inter-CU LTM handover	Qualcomm Incorporated

- R3-251760	Signalling enhancements for Conditional Intra-CU LTM	Qualcomm Incorporated

- R3-251835	(TP for 38.473, 38.423) Discussion for security handling on non-DC case	CATT

- R3-251836	[Draft LS] Reply LS to SA3 on the security handling for inter-CU LTM in non-DC case	CATT

- R3-251837	(TP for 37.340) Discussion for Inter-CU LTM	CATT

- R3-251838	(TP to 38.473)Discussion for C-LTM	CATT

- R3-251862	Discussion on signaling enhancement for inter-CU LTM	China Telecom

- R3-251863	(TP to TS38.401) Support of activation or deactivation of SP CSI-RS for inter-DU LTM	China Telecom

- R3-251864	(TP for TS38.401) On support of intra-CU Conditional LTM	China Telecom

- R3-251881	(TP for 38.423, 38.300, 38.420, 38.470) Inter-CU LTM	ZTE Corporation

- R3-251882	(TP for LTM BLCR for TS38.401) Conditional intra-SN LTM	ZTE Corporation

- R3-251946	[TP to BLCR for TS 38.423] Inter-CU LTM	Lenovo

- R3-251947	[TP to BLCR for TS 38.423] Inter-CU LTM in DC	Lenovo

- R3-251948	[TP to BLCR for TS 38.401] Conditional intra-CU LTM	Lenovo

- R3-251963	Further discussion on inter-CU LTM	Ericsson

- R3-251964	(TP for LTM BL CR for TS 38.423, TS 38.473, TS 38.300, TS 37.340) – Support for inter-CU LTM	Ericsson

- R3-251965	(TP to CLTM BL CR for TS 38.473, TS 38.401) – Further discussion on Intra-CU Conditional LTM	Ericsson

- R3-251972	gNB-DU initiated LTM candidate cell reconfiguration	Rakuten Mobile, Qualcomm Inc

- R3-251986	Issues on source gNB initiated LTM cancel and UE association setup	Ofinno, LLC

- R3-252088	Support for Inter-CU L1/L2 Triggered Mobility (LTM) over Xn	Jio Platforms Ltd (JPL)

- R3-252089	Xn support for inter-CU L1/L2 Triggered Mobility	Jio Platforms Ltd (JPL)

- R3-252118	(TP for LTM BLCR for TS38.473, TS38.300, TS38.470): Further discussion on inter-CU LTM procedure	Huawei

- R3-252119	(TP for LTM BLCR for TS37.340, TS38.423): Discussion on inter-CU SCG LTM	Huawei

- R3-252120	(TP for LTM BLCR for TS38.473, TS38.401): Discussion on intra-CU conditional LTM	Huawei

- R3-252148	Discussion on inter-CU LTM	NTT DOCOMO INC..

- R3-252152	Discussion on inter-gNB-CU LTM	Samsung

- R3-252153	(TP to BLCR for TS38.423 and TS38.473) Inter-gNB-CU LTM	Samsung

- R3-252164	Discussion on Intra-CU Conditional LTM	Samsung

- R3-252228	CSI-RS network coordination	Ericsson, Nokia, LG Electronics

- R3-252229	(TP for TS 38.423) – Xn support for Semi-persistent CSI-RS transmission	Ericsson, Nokia, LG Electronics

- R3-252230	(TP for TS 38.473) – F1 support for Semi-persistent CSI-RS transmission	Ericsson, Nokia, LG Electronics

- R3-252240	(TP for 38.423, 37.340) SN initiate SCG LTM	ZTE Corporation

- R3-252261	Response to R3-251582 and R3-251881	vivo

- R3-252305	Summary of offline discussion on inter-CU LTM	China Telecom

- R3-252320	CB:MobilityENh_LTM	China Telecom

- R3-252321	CB:Mobility_CLTM	Nokia

- R3-252423	(TP for LTM BLCR for TS38.401) Conditional intra-SN LTM	ZTE Corporation, LG Electronics, Nokia, Lenovo, China Telecom, Huawei

- R3-252424	(TP to BL CR TS 38.300) Inter-CU LTM Procedure	Nokia

- R3-252431	(TP for LTM BL CR for TS 38.423) – Further agreements on inter-CU LTM	Ericsson, Nokia, Samsung, CATT, China Telecom, Ofinno, LG Electronics, Lenovo, Huawei

- R3-252433	(TP for LTM BLCR for TS38.473) Update on inter-CU LTM procedure	Huawei

- R3-252434	(TP for LTM BLCR for TS 38.470) Introduce inter-CU LTM procedure	Huawei, LG Electronics, Samsung, Ericsson, China Telecom, Nokia, ZTE Corporation, CATT

- R3-252435	(TP for LTM BLCR for TS38.473) Support of intra-CU conditional LTM	Huawei, Nokia, LG Electronics, Ericsson, Google, ZTE Corporation, China Telecom, Lenovo, NEC

- R3-252438	(TP to TS38.401) Support of activation or deactivation of SP CSI-RS for inter-DU LTM	China Telecom, Nokia, Samsung, Ericsson, ZTE, Huawei, CATT

- R3-252439	(TP for 37.340) Discussion for Inter-CU LTM	CATT

- R3-252440	Reply LS to SA3 on the security handling for inter-CU LTM in non-DC case	RAN3(CATT)

- R3-252444	(TP for BLCR to 38.420) Inter-CU LTM	ZTE Corporation, Nokia, Samsung, China Telecom, Ericsson, Huawei, LG Electronics, CATT

- R3-252445	[TP to BLCR for TS 38.423] Inter-CU LTM in DC	Lenovo

- R3-252449	Reply LS to SA3 on the security handling for inter-CU LTM in non-DC case	RAN3(CATT)

- R3-252457	(TP to BL CR TS 38.300) Inter-CU LTM Procedure	Nokia

- R3-252461	(TP to BL CR for 38.423 DC) Inter-CU LTM in DC	Lenovo, Ericsson, CATT, Huawei, Ofinno, Nokia, NEC, LGE, China Telecom, Google, ZTE, Samsung

- R3-252462	(TP for LTM BLCR for TS37.340): TP for Inter-CU LTM	CATT, China Telecom, Huawei, Nokia, LG Electronics, Google, Samsung, Ofinno, Ericsson, Lenovo, NEC, ZTE, Qualcomm

- R3-252463	(TP for LTM BLCR for TS38.473) Update on inter-CU LTM procedure	Huawei, LG Electronics, Samsung, Nokia, China Telecom, NEC, Ericsson, ZTE Corporation

- R3-252482	(TP to BL CR TS 38.300) Inter-CU LTM Procedure	Nokia, Samsung, Huawei, Ericsson, LG Electronics, CATT, Ofinno, ZTE, NEC, China Telecom, Qualcomm

- R3-252483	(TP for LTM BLCR for TS 38.470) Introduce inter-CU LTM procedure	Huawei, LG Electronics, Samsung, Ericsson, China Telecom, Nokia, ZTE Corporation, CATT

RAN3 #128

- R3-253008	LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells	RAN1(Fujitsu)

- R3-253009	LS on RAN2 agreements for security key handling in inter-CU LTM	RAN2(ZTE)

- R3-253084	(BL CR to 37.340) stage 2 for inter-CU LTM in NR-DC	CATT, China Telecom, Huawei, Nokia, LG Electronics, Google, Samsung, Ofinno, Ericsson, Lenovo, NEC, ZTE, Qualcomm

- R3-253085	(BL CR to 37.483) Introducing Rel-19 Mobility enhancement	LG Electronics Inc., Nokia, China Telecom, Google, Ericsson, CATT, Qualcomm, Samsung, CMCC, ZTE, Huawei, NTT Docomo, Lenovo, NEC, Ofinno, Jio Platforms (JPL)

- R3-253086	(BL CR to 38.300) Support for Inter-CU LTM	Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung, Ofinno

- R3-253087	(BL CR to 38.401) on conditional intra-CU LTM and intra-CU LTM	China Telecom, ZTE Corporation, NEC, Samsung, Nokia, Google, Huawei, Ericsson, LG Electronics, Qualcomm

- R3-253088	(BL CR to 38.420) Support for Inter-CU LTM	ZTE Corporation, China Telecom, Samsung, Nokia, CATT, NEC, LG Electronics, Ericsson, Huawei, Lenovo

- R3-253089	(BL CR to 38.423) Xn support for inter-CU LTM	Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno, Qualcomm

- R3-253090	(BL CR to 38.423 for DC) Xn support for inter-CU LTM in DC	Lenovo, Ericsson, CATT, Huawei, Ofinno, Nokia, NEC, LGE, China Telecom, Google, ZTE, Samsung

- R3-253091	(BL CR to 38.470) Support for Inter-CU LTM procedure	Samsung, Huawei, LG Electronics, Ericsson, China Telecom, Nokia, ZTE Corporation, CATT

- R3-253092	(BL CR to 38.473) Inter-CU LTM and intra-CU conditional LTM	Huawei, Nokia, Samsung, Google, NEC, China Telecom, Ericsson, LG Electronics, CATT, Ofinno, ZTE, Lenovo, Qualcomm Incorporated

- R3-253136	TP (BL CR TS 38.300, TS 38.401, TS 38.473, TS 38.423) Remaining issues on Inter-CU LTM procedure	Nokia

- R3-253137	Discussion on Inter-CU LTM with Dual Connectivity	Nokia

- R3-253138	TP (BL CR TS 38.473, BL CR TS 38.401) Conditional LTM in split architecture	Nokia

- R3-253149	(BL CR to 38.473) Inter-CU LTM and intra-CU conditional LTM	Huawei, Nokia, Samsung, Google, NEC, China Telecom, Ericsson, LG Electronics, CATT, Ofinno, ZTE, Lenovo, Qualcomm Incorporated

- R3-253174	(BL CR to 38.470) Support for inter-CU LTM procedure	Samsung, Huawei, LG Electronics, Ericsson, China Telecom, Nokia, ZTE Corporation, CATT

- R3-253184	(BL CR to 38.300) Support for Inter-CU LTM	Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung, Ofinno

- R3-253198	(TP to BL CR for TS 38.423 and 38.473 on Inter-CU LTM) Rel-19 inter-CU LTM issues	NEC

- R3-253199	Discussion Rel-19 inter-CU LTM in DC scenario	NEC

- R3-253200	(TP to BL CR for TS 38.401 and 38.473 on conditional intra-CU LTM) Discussion of Rel-19 intra-CU Conditional LTM	NEC

- R3-253249	(TP for 38.423, 38.473, 38.401, 38.300) Inter-CU LTM Inter-CU LTM	ZTE Corporation

- R3-253250	(TP for 38.423, 37.340) SN initiate SCG LTM	ZTE Corporation

- R3-253251	(TP for 38.473, 38.401) Conditional intra-CU LTM	ZTE Corporation

- R3-253324	[TP to BLCR for TS 38.401 and TS 38.423] Inter-CU LTM	Lenovo

- R3-253325	[TP to BLCR for TS 38.423] Inter-CU LTM in DC	Lenovo, Huawei, LG Electronics

- R3-253326	[TP to BLCR for TS 38.401] Conditional intra-CU LTM	Lenovo

- R3-253365	(TP for TS38.401) On support of inter-CU LTM	China Telecom

- R3-253366	Discussion on inter-CU LTM in DC scenario	China Telecom

- R3-253367	(TP to TS38.401) On support of intra-CU Conditional LTM	China Telecom

- R3-253468	Discussion for general issues in Inter-CU LTM	CATT

- R3-253469	(TP for 37.340) Discussion for Inter-CU LTM in DC	CATT

- R3-253470	(TP to 38.473)Discussion for C-LTM	CATT

- R3-253482	Further discussion on inter-CU LTM	Ericsson

- R3-253483	(TP for LTM BL CR for TS 38.423, TS 38.473, TS 38.300, TS 37.340) – Support for inter-CU LTM	Ericsson

- R3-253484	(TP to CLTM BL CR for TS 38.473, TS 38.401) – Further discussion on Intra-CU Conditional LTM	Ericsson

- R3-253518	Data forwarding for Inter-CU LTM DC scenarios	Ofinno, LLC

- R3-253519	(TP for TS 38.423) Xn Support for Data forwarding for Inter-CU LTM DC scenarios	Ofinno, LLC

- R3-253520	Open Issues on SN initiated inter-CU SCG LTM	Ofinno, LLC

- R3-253526	(TP to BL CRs 38.300 and 37.340) Clarification on RS configuration and CSI Resource Configuration	Google

- R3-253527	(TP to BL CR 38.423) Clarification on RS configuration and CSI Resource Configuration for MCG LTM	Google

- R3-253528	(TP to BL CR 38.423) Clarification on RS configuration and CSI Resource Configuration for SCG LTM	Google

- R3-253554	gNB-DU initiated LTM resource reconfiguration	Rakuten Mobile, Qualcomm Inc, NTT Docomo Inc

- R3-253561	Signalling enhancements for Inter-CU LTM handover	Qualcomm India Pvt Ltd

- R3-253562	Signalling enhancements for Intra-CU Conditional LTM	Qualcomm India Pvt Ltd

- R3-253599	(TP for LTM BLCR for TS38.473, TS38.300): Further discussion on inter-CU LTM procedure	Huawei

- R3-253600	(TP for LTM BLCR for TS38.423): Essential updates to XnAP BLCR	Huawei

- R3-253601	(TP for LTM BLCR for TS38.473, TS38.401): Discussion on intra-CU conditional LTM	Huawei

- R3-253654	Discussion on inter-gNB-CU LTM	Samsung

- R3-253655	(TP to BLCR for TS38.423 and TS38.473) Inter-gNB-CU LTM	Samsung

- R3-253656	"Continue	discussion on Intra-CU Conditional LTM"	Samsung

- R3-253659	(TP for NR_Mob_Ph4 TS 38.423 and TS 38.473) Discussions on remaining issues related to Inter-CU LTM security	LG Electronics Inc.

- R3-253660	Discussions on the remaining aspects of Inter-CU LTM	LG Electronics Inc.

- R3-253661	(TP for NR_Mob_Ph4 TS 38.473) Discussions on the remaining aspects of Conditional Intra-CU LTM	LG Electronics Inc.

- R3-253676	Discussion on inter-CU LTM for non-DC and DC cases	NTT DOCOMO INC..

- R3-253697	Discussion on inter-CU LTM	CMCC

- R3-253710	Support for Semi-persistent CSI-RS transmission	Ericsson, Nokia, LG Electronics, Google, Jio Platforms

- R3-253711	(TP to BL CR for TS 38.423) Xn support for Semi-persistent CSI-RS transmission	Ericsson, Nokia, LG Electronics, Google, Jio Platforms

- R3-253712	(TP to BL CR for TS 38.473) F1 support for Semi-persistent CSI-RS transmission	Ericsson, Nokia, LG Electronics, Google, Jio Platforms

- R3-253713	PRACH resources for RACH-less LTM	Ericsson, NTT DoCoMo, Lenovo, Jio Platforms, Verizon Wireless, Charter Communications

- R3-253714	(TP to BL CR for TS 38.423) PRACH Resources for RACH-less LTM	Ericsson, NTT DoCoMo, Lenovo, Jio Platforms, Verizon Wireless, Charter Communications

- R3-253715	(TP to BL CR for TS 38.473) PRACH Resources for RACH-less LTM	Ericsson, NTT DoCoMo, Lenovo, Jio Platforms, Verizon Wireless, Charter Communications

- R3-253786	CB:#MobilityEhn_LTM	China Telecom

- R3-253787	CB:#MobilityEhn_LTMDC	Lenovo

- R3-253788	CB:#MobilityEhn_CLTM	Huawei

- R3-253850	(TP for TS 38.423) Data Forwarding for LTM DC scenario	Ofinno, Ericsson, Nokia, Lenovo, Huawei, LG Electronics, ZTE, NEC, China Telecom, Samsung, CATT, Google

- R3-253895	(TP for LTM BLCR for TS38.473): Update on inter-CU LTM procedure	Huawei, Nokia, CATT, Ericsson, LG Electronics, China Telecom, NEC, Samsung, ZTE

- R3-253896	(TP for LTM BLCR for TS38.473): Update on intra-CU conditional LTM	Huawei

- R3-253898	(TP for TS38.401) On support of inter-CU LTM	China Telecom, Nokia, CATT, LG Electronics, Huawei, Google, NEC, Samsung

- R3-253904	[TP to BLCR for TS 38.423] Inter-CU LTM in DC	Lenovo, Huawei, LG Electronics, CATT, Ericsson, ZTE, Ofinno, China Telecom, Nokia, Samsung, Google, NEC

- R3-253910	(TP for 38.401) Conditional intra-gNB-CU LTM	ZTE Corporation, Nokia, LG Electronics, NEC, China Telecom, Ericsson, CATT, Huawei, Samsung, Google, Ofinno

- R3-253919	(TP for LTM BL CR for TS 38.423) - Support for Inter-CU LTM	Ericsson

- R3-253922	(TP for 37.340) stage2 CR for Inter-CU LTM in DC	CATT, China Telecom, Huawei, Nokia, LG Electronics, Google, Samsung, Ofinno, Ericsson, Lenovo, NEC, ZTE, Qualcomm

- R3-253926	(TP to BL CR TS 38.300) Remaining issues on Inter-CU LTM procedure	Nokia, Samsung, Huawei, Ericsson, LG Electronics, CATT, Ofinno, ZTE, NEC, China Telecom, Google

- R3-253951	(TP for LTM BLCR for TS38.473): Update on intra-CU conditional LTM	Huawei, Nokia, Ericsson, LG Electronics, NEC, China Telecom, ZTE, CATT

- R3-253959	(TP for LTM BL CR for TS 38.423) - Support for Inter-CU LTM	Ericsson, LG Electronics, China Telecom, Nokia, ZTE, NEC, Ofinno, Huawei, Samsung, CATT, Google

- R3-253960	(TP for LTM BLCR for TS38.473): Update on intra-CU conditional LTM	Huawei, Nokia, Ericsson, LG Electronics, NEC, China Telecom, ZTE, CATT

RAN4 #114bis:

R4-2503303	Discussion on Conditional Intra-CU LTM for NR mobility enhancements Phase 4 Xiaomi

R4-2503310	Discussion on event triggered L1 measurement reporting for mobility	Xiaomi

R4-2503311	Discussion on CSI-RS based L1 for mobility	Xiaomi

R4-2503346	On event triggered L1 measurement reporting	OPPO

R4-2503347	On CSI-RS based L1 measurement	OPPO

R4-2503433	Discussion on Event triggered L1 measurement reporting for Rel-19 LTM enhancements	CATT

R4-2503434	Discussion on CSI-RS based L1 measurement  for Rel-19 LTM enhancements CATT

R4-2503435	Discussion on conditional Intra-CU LTM for Rel-19 mobility enhancements	CATT

R4-2503572	Discussion on event triggered L1 measurement reporting	Apple

R4-2503573	Discussion on conditional Intra-CU LTM	Apple

R4-2503585	CSI-RS based L1-RSRP measurement	Apple

R4-2503603	Discussion on event triggered L1 report for LTM	MediaTek Inc.

R4-2503604	Discussion on CSI-RS L1 measurement for LTM	MediaTek Inc.

R4-2503605	Discussion on Conditional LTM	MediaTek Inc.

R4-2503628	Topic summary for [114bis][218] NR_Mob_Ph4_Part1	Moderator (Apple)

R4-2503629	Topic summary for [114bis][219] NR_Mob_Ph4_Part2	Moderator (China Telecom)

R4-2503752	Discussion on intra-CU conditional LTM	Ericsson

R4-2503757	Discussion on event triggered L1 measurement reporting	CMCC

R4-2503758	Discussion on CSI-RS based L1 measurement	CMCC

R4-2503759	Discussion on conditional intra-CU LTM	CMCC

R4-2503854	Discussion on event triggered L1 measurement reporting	ZTE Corporation, Sanechips

R4-2503855	Discussion on CSI-RS based L1-RSRP measurement	ZTE Corporation, Sanechips

R4-2503856	Discussion on Conditional Intra-CU LTM	ZTE Corporation, Sanechips

R4-2503883	Discussion on Event triggered L1 measurement reporting	Huawei, HiSilicon

R4-2503884	Discussion on CSI-RS based L1 measurement	Huawei, HiSilicon

R4-2503885	Discussion on conditional Intra-CU LTM	Huawei, HiSilicon

R4-2503936	Discussion on Event triggered L1 measurement reporting	Ericsson

R4-2503937	Discussion on CSI-RS based L1 measurement	Ericsson

R4-2503987	RRM Core requirements on event triggered L1 measurement reporting	China Telecom

R4-2503988	RRM Core requirements on CSI-RS based L1 measurement	China Telecom

R4-2503989	RRM Core requirements on conditional Intra-CU LTM	China Telecom

R4-2504030	On RRM requirements for LTM event-triggered L1-RSRP reporting	vivo

R4-2504031	On RRM requirements for LTM CSI-RS based L1 measurement	vivo

R4-2504052	Further discussion on Conditional Intra-CU LTM	vivo

R4-2504071	On event-triggered L1 reporting for LTM	Nokia

R4-2504072	DraftCR for LTM event-triggered L1 requirements	Nokia

R4-2504202	On CLTM RRM requirements	Nokia

R4-2504470	Discussion on CSI-RS based L1 measurements	Nokia

R4-2504527	CSI-RS based L1 measurement for LTM	Qualcomm Incorporated

R4-2504528	Conditional LTM	Qualcomm Incorporated

R4-2504910	WF on RRM requirements for NR_Mob_Ph4_Part2	China Telecom

R4-2504965	WF on RRM requirements for NR_Mob_Ph4_Part1	Apple

RAN4 #115

R4-2505473	RRM Core requirements on event triggered L1 measurement reporting	China Telecom

R4-2505474	RRM Core requirements on CSI-RS based L1 measurement	China Telecom

R4-2505475	RRM Core requirements on conditional Intra-CU LTM	China Telecom

R4-2505476	DraftCR to 38.133 on conditional Intra-CU LTM	China Telecom

R4-2505573	Topic summary for [115][224] NR_Mob_Ph4_Part1	Moderator (Apple)

R4-2505574	Topic summary for [115][225] NR_Mob_Ph4_Part2	Moderator (China Telecom)

R4-2505710	On event triggered L1 measurement reporting	OPPO

R4-2505711	draftCR on capabilities for Support of Event Triggering and Reporting Criteria	OPPO

R4-2505712	On CSI-RS based L1 measurements	OPPO

R4-2505731	On remaining LTM L1 event-triggered reporting requirements	Nokia

R4-2505732	DraftCR for LTM L1 event-triggered reporting	Nokia

R4-2505733	On performance requirements for NR mobility enhancements Ph4	Nokia

R4-2505768	Discussion on event triggered L1 measurement reporting for mobility	Xiaomi

R4-2505769	Discussion on CSI-RS based L1 for mobility	Xiaomi

R4-2505794	Discussion on Conditional Intra-CU LTM for NR mobility enhancements Phase 4	Xiaomi

R4-2505864	On UE feature list for CSI-RS based L1 measurement	Apple

R4-2505865	On open issue for CSI-RS based L1 measurement	Apple

R4-2505866	draft CR on CSI-RS based L1 measurement requirement	Apple

R4-2505875	Discussion on event triggered L1 measurement reporting	Apple

R4-2505876	Discussion on conditional Intra-CU LTM	Apple

R4-2505877	Discussion on RRM performance of NR mobility enhancements Phase 4	Apple

R4-2506104	Discussion on Event triggered L1 measurement reporting for Rel-19 LTM enhancements	CATT

R4-2506105	Discussion on CSI-RS based L1 measurement  for Rel-19 LTM enhancements	CATT

R4-2506106	Discussion on conditional Intra-CU LTM for Rel-19 mobility enhancements	CATT

R4-2506107	Draft CR to TS 38.133 on number of CSI-RS, number of cells and number of CSI-RS resources	CATT

R4-2506108	Discussion on Rel-19 NR mobility Phase4 RRM performance requirements	CATT

R4-2506139	Discussion on CSI-RS measurements core part	Nokia

R4-2506140	DraftCR to 38.133 on LTM CSI-RS applicability	Nokia

R4-2506458	On RRM requirements for LTM CSI-RS based L1 measurement	vivo

R4-2506459	On RRM requirements for LTM event-triggered L1-RSRP reporting	vivo

R4-2506460	On UE capability for mobility enhancement phase 4 WI	vivo

R4-2506461	Draft CR on measurement reporting requirements for CSI-RS based LTM	vivo

R4-2506462	On performance requirements for mobility enh phase 4	vivo

R4-2506500	Discussion on event triggered L1 report for LTM	MediaTek Inc.

R4-2506501	Discussion on CSI-RS L1 measurement for LTM	MediaTek Inc.

R4-2506502	Draft CR on introduction of CSI-RS L1-RSRP measurement	MediaTek Inc.

R4-2506503	Discussion on Conditional LTM	MediaTek Inc.

R4-2506504	Discussion on RRM performance requirements for R19 mobility	MediaTek Inc.

R4-2506555	Further discussion on Conditional Intra-CU LTM	vivo

R4-2506567	Discussion on Conditional Intra-CU LTM	ZTE Corporation, Sanechips

R4-2506568	Discussion on CSI-RS based L1-RSRP measurement	ZTE Corporation, Sanechips

R4-2506569	Discussion on Event triggered L1 measurement reporting	ZTE Corporation, Sanechips

R4-2506570	Discussion on performance requirements for LTM	ZTE Corporation, Sanechips

R4-2506577	draftCR on measurement restriction for CSI-RS based L1-RSRP	ZTE Corporation, Sanechips

R4-2506589	Discussion on event triggered L1 measurement reporting	CMCC

R4-2506590	Discussion on CSI-RS based L1 measurement	CMCC

R4-2506591	Discussion on conditional intra-CU LTM	CMCC

R4-2506592	Discussion on RRM performance requirements for NR mobility enhancements	CMCC

R4-2506617	DraftCR on event-triggered reporting for CSI-RS based L1 measurement	CMCC

R4-2506670	Discussion on intra-CU conditional LTM	Ericsson

R4-2506671	Discusion on performance requirement for Rel-19 mobility	Ericsson

R4-2506730	Discussion on Event triggered L1 measurement reporting	Huawei, HiSilicon

R4-2506731	Discussion on CSI-RS based L1 measurement	Huawei, HiSilicon

R4-2506732	Draft CR on scheduling availability of UE during CSI-RS based L1-RSRP measurement	Huawei, HiSilicon

R4-2506733	Discussion on conditional Intra-CU LTM	Huawei, HiSilicon

R4-2506734	Discussion on performance requirements of mobility enhancement phase 4	Huawei, HiSilicon

R4-2507015	Rel-19 RAN4 UE feature list for NR mobility enhancements Phase 4	Ericsson

R4-2507016	Discussion on measurements enhancements for purpose of supporting LTM	Ericsson

R4-2507017	Draft CR on event-Triggered Reporting for Intra-Frequency L1 Measurements on Neighboring Cells	Ericsson

R4-2507018	Discussion on CSI-RS based L1 measurement requirements	Ericsson

R4-2507134	Discussion on Conditional Intra-CU LTM	Nokia

R4-2507804	CSI-RS based L1 measurement for LTM	Qualcomm Incorporated

R4-2507805	Conditional LTM	Qualcomm Incorporated

R4-2505573	Topic summary for [115][224] NR_Mob_Ph4_Part1

R4-2508281	Ad-hoc minutes for NR_Mob_Ph4

R4-2508282	WF on RRM requirements for NR_Mob_Ph4_Part1

R4-2505574	Topic summary for [115][225] NR_Mob_Ph4_Part2

R4-2508283	WF on RRM requirements for NR_Mob_Ph4_Part2

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template