3GPP TSG RAN WG1 #108			R1-251198

Prague, CZ Jun 9th – 13th, 2025

Agenda item:	15.1.1

Source:	Tejas Networks Ltd.

Title:	Views on Scope for 5G-Advanced Release 20 – AI/ML for Air-Interface

Document for: 	Discussion and Decision

# Introduction

This document presents views from Tejas Networks on the scope for Release 20 (Rel-20) with respect to the use of Artificial Intelligence and Machine Learning (AI/ML) over the air interface, building upon the foundational studies conducted in Rel-19. The focus is primarily on enhancing CSI feedback through advanced AI/ML techniques.

# Background and Motivation

In 3GPP Rel-19, a study item on AI/ML for the air interface was introduced, covering key areas such as:

- AI/ML-based CSI compression and prediction.

- AI/ML-based beam prediction.

- AI/ML-based positioning enhancements.

This study highlighted promising directions for performance improvements in wireless systems, particularly around physical layer feedback and control signaling. For instance, in CSI feedback enhancement using AI/ML:

- A set of five sub-use cases was identified, with special attention on temporal dimension in use cases 2 and 3.

- Simulation results showed 20–30% throughput gains compared to traditional eType2 codebooks.

- Inter-vendor training collaboration challenges were extensively analyzed, leading to two consolidated solution paths

# Scope for Rel-20 – AI/ML CSI Feedback Enhancement Phase 2

The proposed scope for Rel-20 includes:

Performance vs. Complexity Optimization

Enhance existing AI/ML CSI compression techniques to:

- Extend beyond spatial/frequency compression to spatial/temporal/frequency compression.

- Explore cell-/site-specific models for CSI reconstruction.

- Combine CSI compression with prediction mechanisms.

Inter-Vendor Training Collaboration

Two primary approaches are proposed:

- Option 3a-1/3b: Specification of model structure and support for parameter exchange. (Also referred to as Direction C)

- Option 4-1: Exchange of datasets instead of model parameters.

Key considerations include:

- Scalable model structures with respect to the number of transmit antennas, feedback payload size, and system bandwidth.

- Specification enhancements to enable hardware-friendly implementation for CSI processing

Observation 1: Evaluations across Rel-18 and Rel-19 consistently show that AI/ML-based two-sided CSI compression achieves higher throughput and SGCS gains when compared to non-AIML benchmarks.

Observation 2: RAN1 has confirmed feasibility of the following inter-vendor collaboration paths:

- Sub-option 4-1: Dataset exchange with standardized format

- Sub-option 3a-1: Reference model structure + parameter exchange

- Direction C: Fully standardized reference model (structure + parameters)

Observation 3: Rel-19 investigations have produced scalable model structures suitable for standardization. Many RAN1 members agree that reference model definitions can be coordinated with RAN4 for performance testing purposes.

#### Proposal 1: Initiate normative specification of two-sided AI/ML-based CSI compression in Rel-20

- Treat Case 0 (spatial-frequency compression) as the highest priority for specification.

- Treat Case 2 (temporal-spatial-frequency) and Case 3 (compression + prediction) as lower priority extensions, subject to time availability.

#### Proposal 2: Enable inter-vendor collaboration via:

- Primary Option: Sub-option 4-1 – standardized dataset format + exchange

- Secondary Option: Sub-option 3a-1 – standardized model structure and parameter exchange (aligned with RAN4 encoder structures)

- Fallback Option: Direction C – fully standardized reference model, if feasible and aligned with RAN4 testability goals

#### Proposal 3: Define support for AIML CSI compression with the following:

- Dataset and parameter exchange mechanisms (OTA or alternative)

- Model pairing procedure (ID, applicability reporting)

- Training data collection:

  - UE-side: UE-triggered RS configuration

  - NW-side: Mechanisms for collecting ground-truth CSI via feedback

- Performance monitoring:

  - NW-side: Using legacy codebooks

  - UE-side: SGCS reporting (aligned with CSI prediction work)

- AI/ML lifecycle management (LCM) signaling and fallback mechanisms