3GPP TSG RAN Meeting #108										RP-251199

Prague, Czech Republic, June 9-13, 2025

Status Report to TSG

Agenda item:			9.3.1.4

| WI / SI Name | Low-power wake-up signal and receiver for NR (LP-WUS/WUR) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_LPWUS |  |  |  |  |
| Unique ID | 1020094 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241824 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: N/A |  |
| Overall Completion level | Study Item: n/a | Core part: 90% (RAN1: 100%) | Performance Part: 5% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN 1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Xueming Pan |
|  | Company | vivo |
|  | Email | panxueming@vivo.com |
| Rapporteur | Name | Shinya Kumagai |
|  | Company | NTT DOCOMO |
|  | Email | shinya.kumagai.yw@nttdocomo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

LP-WUS and LP-SS design

RAN1#120bis

Agreement

From specification design perspective, for both RRC idle/inactive and RRC connected mode, support up to 5 information bits and up to 32 codepoints for a LP-WUS.

Agreement

Support maximum length for a LP-WUS (actual OFDM symbols used) which is larger than one slot

- Non-consecutive OFDM symbols at least across multiple slots can be used for a LP-WUS transmission

Agreement

For RRC connected mode, maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell,

- support maximum 16 candidates overlaid sequences for M=1

- support maximum 8 candidates overlaid sequences for M=2

- support maximum 4 candidates overlaid sequences for M=4 (agreed in RAN1#119)

- For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value.

  - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip.

- FFS: Whether or not the candidate set of overlaid sequences change across OOK ON chips

Agreement

For M=1, the overlaid OFDM sequence length LZC=12*X, X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

- The agreement overrides previous WA

Agreement

Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1, M=2:

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

  - Additionally support LP-SS periodicity of 160ms

Working Assumption

At least for FR1, support maximum 2 roots to be used for overlaid OFDM sequences for M=1/2/4 (max 2 for each M value).

Different roots can be picked for different M values.

Agreement

For > 2 information bits with RM coding, support one of the following alternatives for rate matching after RM coding, when code block (coded bits) length after rate matching < 32

- Alt 1: Rate matching after RM coding is applied, as section 5.4.3 in TS 38.212.

Agreement

For the length L of LP-SS binary sequence, supports

- M=1, L= {6,8}

- M=2, L= {12,16}

- M=4, L= {16,32}

Note: For each value of M, UE supports both L values – no UE capability.

Note: There is no consensus in RAN1 that SNR of -3dB can be fulfilled with the smaller value of L for each M. Definition of the proper requirements for different values of L is up to RAN4.

Agreement

For LP-SS with M=4, supports set1 (balanced).

Agreement

For the root(s) used for overlaid OFDM sequences, gNB can configure any value from 1 ~ (Bzc -1), where Bzc is given by the largest prime number such that BZC<LZC, where  LZC is the overlaid OFDM sequence length.

Agreement

For WUS information carried by the overlaid OFDM sequence(s), down-select between Alt 1 and Alt2:

- Alt 1: Raw information bits are mapped to sequence(s)

  - N raw information bits are divided into K segments from MSB to LSB, where K= ceil (N/log2L), L is the number of candidate overlaid OFDM sequences for one OOK ON chip.

  - In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially, e.g., for a segment of 2 information bits, 00 is mapped to sequence #1, 01 is mapped to seq #2.

  - In case N/log2L is not an integer, Bit 0 as MSB is used for padding

Agreement

For > 2 information bits with RM coding, support candidate code block (coded bits) length after rate matching within the below range:

- Alt 1a: code block (coded bits) length: [3,4]~[32,64]

  - FFS whether all or a subset of lengths is selected.

  - FFS whether same range applies to all M values

  - FFS: above lengths do not consider repetition (if supported) after rate matching

Agreement

For RRC idle/inactive, support Alt 1 for both Redcap and non-redcap UEs:

- Alt 2: LP-WUS/LP-SS frequency resource can be out of initial DL BWP where UE receives paging but within the same carrier.

    - FFS whether to define the maximum frequency gap between LP-WUS/LP-SS and SSB

Working Assumption

Support 1 & 2 information bits for LP-WUS. No additional restriction is introduced on the supported number of subgroups/codepoints by LP-WUS.

- 2 information bits: coding is done as described in the first row in the table in section 5.3.3.2 of 38.212 (scrambling is not applied)

- 1 information bit: repetition is done as described in the first row in the table in section 5.3.3.1 of 38.212 (scrambling is not applied)

Agreement

For the M value for LP-WUS and LP-SS, supports:

- Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

  - At least for M values larger than 1 (applies to both LP-WUS and LP-SS)

  - FFS: If M=1 is configured for either of LP-WUS or LP-SS, M=1 is configured for both

RAN1#121

Agreement

For LP-WUS/LP-SS frequency resource configuration, the starting PRB index with reference to carrier boundary determined by offsetToCarrier corresponding to SCS of the active BWP or legacy DL initial BWP is configured by gNB, with range of 0~263. Starting PRB index configured for LP-WUS is same as that of LP-SS.

Agreement

Confirm the following working assumption with revision in red

Working Assumption At least for FR1, support maximum 2 roots to be used for overlaid OFDM sequences for M=1/2/4 (max 2 for each M value). Different roots can be picked for different M values. Above applies for LP-WUS

Agreement

Update the agreements as below:

Agreement For idle mode, regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip.

Agreement For RRC connected mode, maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - support maximum 4 candidates overlaid sequences for M=4 (agreed in RAN1#119) - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. - FFS: Whether or not the candidate set of overlaid sequences change across OOK ON chips

Note: The WUS information carried by overlaid OFDM sequence is based on codepoint i.e. raw information bit

Agreement

From RAN1 perspective, there is no restriction on the maximum frequency bandwidth covering both LP-WUS/LP-SS and SSB.

- It is up to RAN4 to introduce any restriction depending on their discussions

- Above applies for IDLE and INACTIVE mode UEs

Agreement

For code block (coded bits) length L after rate matching as defined in 38.212 section 7.4.2.1, support

- For 1 information bit, L is in 1~32

- For 2 information bits, L is in 2~32

- For 3 information bits, L is in 3~32

- For 4 information bits, L is in 5~32

- For 5 information bits, L is in 6~32

- For M=4, the code block (coded bits) length L is even number.

- Further restriction on the L value within above range is not precluded.

- Do not support repetition after rate matching

Agreement

For both FR1 and FR2, when M value for LP-WUS is 1, the M values for LP-SS can be configured as 1, 2, or 4.

Agreement

Confirm the following working assumption

Working Assumption Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs: - Codepoint based - The maximum number of codepoints checked per MO by a UE is up to 8 Depending on UE capability, a UE may support less than 8

Agreement

For mapping between a sequence index cm and root value (qcm) & CS value (ncs, cm).

qcm=qi ,   i=floor(cmP),  P=NseqNroot

ncs, cm=ncs, k,     where k=mod(cm, P)

where cyclic shift value ncs, kis determined by
ncs, k=k*floor BZC P, where k=0,…, P-1

Agreement

Support LP-WUS with M=4 for 30kHz SCS without separate UE capability.

- No additional sync signal for this case, LP-SS periodicity =320ms and 160ms

Agreement

For the overlaid OFDM sequence for LP-SS in one cell, single root is used and gNB can configure any value from 1 ~ (Bzc -1), where Bzc is given by the largest prime number such that BZC<LZC, and   LZC is the overlaid OFDM sequence length.

- cyclic shift value ncs=0.

- Note: Different roots can be used to differentiate overlaid OFDM sequences for LP-SS from different cells

Agreement

For LP-WUS in FR2, support M=1 for 120kHz in both RRC idle/inactive and RRC connected, and M=1, 2 for SCS=60kHz in RRC connected only.

Agreement

For LP-WUS in FR2, the number of roots and number of sequences are

- Max 2 roots and max 4 sequences for M =1 for SCS 60kHz

- Max 2 roots and max 2 sequences for M=2 for SCS 60kHz

- Max 2 roots and max 2 sequences for M=1 for SCS 120kHz

Agreement

Confirm the following working assumption

Working Assumption

Support 1 & 2 information bits for LP-WUS. No additional restriction is introduced on the supported number of subgroups/codepoints by LP-WUS.

- 2 information bits: coding is done as described in the first row in the table in section 5.3.3.2 of 38.212 (scrambling is not applied)

- 1 information bit: repetition is done as described in the first row in the table in section 5.3.3.1 of 38.212 (scrambling is not applied)

Agreement

For LP-WUS overlaid OFDM sequence configuration in a cell,

- gNB configures number of sequences Nseq, and one or two root values.

  - Alt1: Nseq is 2^n, where the value range of n is 0~4, 0~3, 0~2 for M=1, 2, 4, respectively.

    - No separate UE capability for different n values.

  - Nroot=2, when Nseq=16,8,4 for M=1,2,4, where Nroot is the number of root values.

    - Nroot=1 or 2 when Nseq is other value

- The number of CS Ncs is derived by Nseq/Nroot

LS on LP-WUS UE RF to RAN4 is endorsed in R1-2504943.

LP-WUS operation in IDLE/INACTIVE modes

RAN1#120bis

Agreement

Confirm the following working assumption with modifications in red:

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Agreement

Option A for MO configuration is supported.

Agreement

A LP-WUS MO can span across multiple slots.

- FFS the limitation on the maximum length for an LP-WUS MO

Agreement

Only R = 1 is supported for Option A.

Conclusion

For the offset value(s) between an LO and a reference PO/PF, do not support the configuration of 3 offset values.

Agreement

The maximum number of POs per LO is 4, and the number of POs per LO can be 1, 2 or 4.

Agreement

For Option 2, the maximum value of M (number of LP-WUS MOs per beam) in Option A for MO configuration is 4.

Agreement

For LP-WUS, the N * M LP-WUS MOs in an LO are indexed sequentially in time, from 1 to N*M, where N is the number of beams corresponding to LP-WUS, and M is the number of LP-WUS MOs for each beam.

- The (n*M+m+1)-th LP-WUS MO corresponds to the (n+1)-th beam, where m=0,1,…,M-1, n=0,1,2,…,N-1. (multiple MOs first, beam second)

Note: Above does not change the previous agreement on association between LP-WUS and SSB beams.

Agreement

For OFDM-based LP-WUR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LP-WUR.

FFS: Whether OFDM receiver can measure LP-SS if overlaid OFDM sequence is not configured (M=1).

Agreement

For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

- Send an LS to RAN4 to confirm the above agreement. Final LS in R1-2503103.

Agreement

LS on the RRM measurement metrics for OFDM-based LP-WUR is agreed. Final LS in R1-2503103.

Note: RAN1 understanding is existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications.

Agreement

For Option 2, at least one codepoint corresponding to each of the subgroups in each PO is supported.

- For codepoint corresponding to more than one subgroups:

  - Alt 2: One codepoint for each PO corresponding to all the subgroups in the PO

Agreement

For Option 2, a common codepoint per PO is always used and the maximum number of subgroups supported per PO is

- 7 for the case where 4 POs are mapped to one LO

- 15 for the case where 2 POs are mapped to one LO

Agreement

Regarding whether there is any restriction on mapping multiple POs to one LO, no additional constraint for mapping multiple POs to one LO

Agreement

UE determines whether a symbol is available for LP-WUS based on:

- Alt 1: Time-domain pattern configured by the gNB

  - Alt 1A: Periodic time-domain pattern

    - E.g. (1) 1-slot periodicity, the pattern indicates the available symbols in each slot; (2) multi-slot or frame-level periodicity with a bitmap indication; (3) Search space set-like pattern; (4) multi-level time-domain patterns; (5) reuse the mechanism of rate matching pattern

  - Alt 1B: Per-MO pattern, applicable for all Mos

- Alt 2: Information from existing configurations available for idle/inactive UEs such as [SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc].

- Alt 3: Combination of Alt 1 and Alt 2

- Alt 4: NW ensures LP-WUS configuration without collision with existing signal(s)

Agreement

For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO,

- A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO.

- The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point.

  - FFS slot-level or symbol-level offset

- The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives:

  - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs.

    - FFS slot-level or symbol-level offset

  - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO.

    - FFS additional configuration to control the subsequent MO locations, e.g.,

      - Alt 2A: configuration of a single gap between the end of the previous MO (or a set of previous MOs) and the start of the next MO

      - Alt 2B: configuration of candidate starting locations for MOs, similar to search space configuration

  - FFS restriction on MO locations, e.g. only on DL slots

  - FFS minimum gap is needed between two MOs to ensure LR processing time

Agreement

Terminology definition

- Nominal MO duration: this includes both the available and unavailable symbols

- Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission as assumed by the UE

Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives:

- Alt A: Nominal MO duration is configured. (e.g. in unit of slots)

  - Actual LP-WUS duration is the number of available symbols within the MO.

  - Actual LP-WUS duration can vary from one MO to another MO.

- Alt B: Actual LP-WUS duration is configured. (e.g. in unit of OFDM symbols, or M and L values)

  - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration.

    - FFS: Additional termination condition such as time window

- Alt C: Both nominal MO duration and actual LP-WUS duration are configured.

  - If the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, the MO is considered as invalid (no LP-WUS monitoring in this MO) and dropped/deferred.

    - FFS UE behavior if there is no valid MO for the beam(s) that the UE monitors

  - Note: the number of available OFDM symbols within a nominal MO duration can be different for different MOs.

- Alt D: Nominal MO duration is configured. The actual LP-WUS duration is determined based on the same pattern for the available symbols for all the MOs (e.g. by using a per-MO pattern), which is the same for all the MOs.

Agreement

On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms, consider the following alternatives for possible down-selection in RAN1#121.

- Alt 1: Do not report for SSB periodicities other than 20ms

  - Note: LP-WUS is not supported for SSB periodicities larger than 20ms

- Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns):

| SSB periodicity (ms) | Wake-up delay (ms) UE capability 1 | Wake-up delay (ms) UE capability 2 | Wake-up delay (ms) UE capability 3 |
| --- | --- | --- | --- |
| [5] | [x] | [x] | [x] |
| [10] | [x] | [x] | [x] |
| 20 | [70] | [500] | [900] |
| [40] | [x] | [x] | [x] |
| [80] | [x] | [x] | [x] |
| [160] | [x] | [x] | [x] |

- Alt 3: It is up to RAN4

Agreement

Each LP-SS transmission for each beam always occupies consecutive OFDM symbols.

RAN1#121

Agreement

It can be configured by the gNB in the LP-WUS/LP-SS configuration on whether LP-WUS/LP-SS transmission is present or not for each beam of the actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

- For the LP-WUS MO and LP-SS occasion determination,

  - Alt 1: S is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and LP-WUS MOs and LP-SS occasions are determined assuming S beams. gNB transmits LP-WUS/LP-SS in a subset of the S beams according to the LP-WUS/LP-SS configuration.

    - FFS: Additional details on Alt 1 to be finalized in RAN1#121

- If not configured, UE assumes LP-WUS/LP-SS transmission is present for all the actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1

Agreement

Each LP-SS transmission for each beam is contained within one slot.

Agreement

For LP-SS, the LP-SS occasions are indexed sequentially in time, and the n-th LP-SS occasion is associated with the beam of the n-th transmitted SSB, n = 1, 2, …, N, where N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

The RAN1#120bis agreement is updated as follows:

For LP-WUS, the N * M LP-WUS MOs in an LO are indexed sequentially in time, from 1 to N*M, where N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 the number of beams corresponding to LP-WUS, and M is the number of LP-WUS MOs for each beam.

- The (n*M+m+1)-th LP-WUS MO corresponds to the (n+1)-th beam, which is associated with the beam of the (n+1)-th transmitted SSB, where m=0,1,…,M-1, n=0,1,2,…,N-1. (multiple MOs first, beam second)

Agreement

For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns):

| SSB periodicity (ms) | Wake-up delay (ms) UE capability 1 | Wake-up delay (ms) UE capability 2 | Wake-up delay (ms) UE capability 3 |
| --- | --- | --- | --- |
| 5/10/20 | [70] | [500] | [900] |
| 40 | [x] | [x] | [x] |
| 80 | [x] | [x] | [x] |
| 160 | [x] | [x] | [x] |

Agreement

If the number of POs associated with a LO is less than Ns (the number of POs per PF), to determine the LP-WUS MOs for the multiple LOs with the same reference PF:

- Alt 1: additional frame-level offset(s) are configured.

  - Common slot-level or symbol level offsets are shared across all LOs

Agreement

For the configuration on whether LP-WUS/LP-SS transmission is present or not for each beam of the actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1:

- use bitmap, with one bit corresponding to each of the SSB beams (similar to existing indication for ssb-PositionsInBurst)

Agreement

For PO-to-LO association and codepoint determination, assume NPOLO is the number of POs associated with a LO, NSGPO is the number of subgroups per PO for LP-WUS, and  iSG is the subgroup ID of a UE (0≤iSG<NSGPO).

- The PO index within the LO is defined as

iPO=UE_ID mod N∙Ns+is mod NPOLO,

- The reference PF for the LO of a PO is provided by (SFN for PF) – floor(iPO/NS) * T/N, which is the first PF of the PF(s) associated with the LO.

- For the codepoints,

  - If NSGPO>1,

    - The number of information bits in LP-WUS is log2NPOLO*(NSGPO+1).

    - Alt 2: (the codepoints for each PO are consecutive)

      - The codepoint for subgroup iSG in PO iPO is iPO*(NSGPO+1)+iSG.

      - The codepoint for all the subgroups in PO iPO is (iPO+1)*NSGPO+1-1.

  - If NSGPO=1, the number of information bits in LP-WUS is log2NPOLO, and the codepoint for PO iPO is iPO.

- Here UE_ID (for LP-WUS), N, NS, is and T are defined in TS 38.304

Agreement

For UE to determine whether a symbol is available for LP-WUS, at least the following is supported:

- A unit-level bitmap with a periodicity 10, 20, or 40 units and a 14-bit or 28-bit symbol-level bitmap that covers 1 or 2 slots can be configured, where each unit is 1 or 2 slots for 14-bit or 28-bit symbol-level bitmap, respectively, with a maximum periodicity of 40ms.

  - ‘1’ in unit-level bitmap means the symbol level bitmap is applied to determine which symbols are unavailable in the unit for LP-WUS (‘0’ means unavailable).

  - ‘0’ in unit-level bitmap means: all the symbols in the unit are unavailable for LP-WUS

- If the slot-level bitmap is not configured, UE assumes all 1’s for the bitmap.

- If the symbol-level bitmap is not configured, UE assumes all 1’s for the bitmap.

Agreement

For the handling of SSB for IDLE mode, the SSB symbols are considered as unavailable for LP-WUS, if SSB and LP-WUS overlap in frequency domain

Agreement

Nominal MO duration (X1, in unit of OFDM symbols) and actual LP-WUS duration (X2, in unit of OFDM symbols) are configured. (Alt C)

- A LP-WUS MO spans the nominal MO duration (i.e., the LP-WUS MO duration is the same as the nominal MO duration.)

- If the number of available OFDM symbols within the nominal MO duration is no less than the actual LP-WUS duration, UE monitors LP-WUS on the first X2 available symbols within the LP-WUS MO.

- Otherwise, UE does not monitor LP-WUS in this MO (i.e., the MO is dropped).

- Note: Any symbols that are not defined as unavailable are available symbols for LP-WUS.

Agreement

The starting time location of the first LP-WUS MO in a LO is configured by an offset w.r.t. the reference point, where the offset is a symbol-level offset

Agreement

If LP-SS is configured, the periodicity is configured with the candidate value set of {160ms, 320ms}.

- A time offset is configured for the first LP-SS occasion with reference to SFN0

  - If the periodicity is 160ms, the candidate value set for the time offset is {0, 1, …, 159}ms.

  - If the periodicity is 320ms, the candidate value set for the time offset is {0, 1, …, 319}ms.

Agreement

The number of MOs per beam per LO is configured by gNB, with candidate value set of {1, 2, 3, 4}.

Agreement

The candidate value set for the frame-level offset(s) is {8, 9, …, 200} in unit of frames.

- The minimum value can be modified depending on relevant RAN4 outcome

Agreement

For UE to determine the LP-SS occasions,

- One or two start symbol locations within a slot are configured.

  - Candidate value range for each start symbol location: {0, 1, …, 10}.

  - If one value is configured, there is one LP-SS occasion in a slot.

  - If two values are configured, there are two LP-SS occasions in a slot.

- Starting from the slot for the first LP-SS occasion (determined from the periodicity/offset configuration for LP-SS), LP-SS occasions are present in this slot and the next (ceil(X/Y)-1) DL slots, where X is the number of SSBs determined according to ssb-PositionsInBurst in SIB1, and Y is the number of LP-SS occasions in a slot.

  - UE expects the slot for the first LP-SS occasion is a DL slot.

  - Note: a slot is a DL slot if all the symbols in the slot are indicated as DL symbols in tdd-UL-DL-ConfigurationCommon.

Note: UE follows the configured resources for LP-SS processing.

Agreement

In one LO, the start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO.

- No additional RRC configuration is provided.

- FFS whether/how to ensure a gap between two LP-WUS MOs

Agreement

The OFDM symbols configured for Type-0 CSS are considered as unavailable for LP-WUS if CORESET#0 and LP-WUS overlap in frequency domain.

Agreement

The UL symbols/slots configured in tdd-UL-DL-ConfigurationCommon are considered as unavailable for LP-WUS.

Agreement

If LP-SS overlap (including partial overlap) in time domain with the available symbols that may be used for LP-WUS transmission in a LP-WUS MO, UE does not monitor LP-WUS in this MO (i.e., the MO is dropped).

Agreement

For the EPRE ratio between LP-WUS/LP-SS and SSB (i.e. LP-WUS/LP-SS EPRE divided by SSB EPRE), separate configurations are provided for LP-WUS and LP-SS.

- The candidate value set for the EPRE ratio is {-3dB, 0 dB, 3 dB, 6 dB}.

- If M=1 for both LP-WUS and LP-SS, or M>1 for both LP-WUS and LP-SS, the difference between the two EPRE ratios shall be no larger than 3 dB.

- If M=1 for LP-WUS and M>1 for LP-SS, the ERPE ratio for LP-SS minus the EPRE ratio for LP-WUS should be within the range of -6 dB to 0 dB.

Note: EPRE refers to EPRE in one OFDM symbol with non-zero power (from baseband perspective) LP-WUS/LP-SS transmission.

LP-WUS operation in CONNECTED modes

RAN1#120bis

Agreement

- For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, LP-WUS can be configured only on PCell

- For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, LP-WUS can be configured only on PCell/PSCell per cell-group

Conclusion

From RAN1 perspective, when LP-WUS monitoring Option 1-2 is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers PDCCH monitoring according to the legacy UE behaviour

Conclusion

For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, there is no consensus in RAN1 which of the following options to support for this case in Rel-19.

- Alt1

  - Basic capability

    - LP-WUS can be configured only on PCell

    - LP-WUS indication is applicable to all serving cells of all DRX groups.

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - Advanced capability

    - LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group

    - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

    - This capability has prerequisite of the above basic capability

- Alt2

  - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group

  - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored

    - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - This capability has candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both}

Agreement

Update previous agreement in red:

Agreement

For LP-WUS MOs in connected mode for Option 1-1, support Approach 1:

- LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

- Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start

  - UE monitors the first Z LP-WUS MO(s) at or after Time offset2.

    - FFS: How to determine the value of Z is RRC configured

  - Time offset2 is RRC configured

UE is not required to monitor LP-WUS during the X [slots/symbols] prior to the beginning of a slot where the UE would start the drx-onDurationTimer. FFS: How X is determined given based on the minimum time gap reported by UE capability.

Agreement

- Update previous agreement in red:

Agreement

- For LP-WUS MOs in connected mode for Option 1-2, support Approach 1:

  - LP-WUS MOs, including a periodicity and a time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - FFS one or multiple MOs per periodicity

    - UE is configured with a number of MOs for the periodicity by RRC

  - UE monitors LP-WUS in the LP-WUS MOs and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS

    - FFS: Definition of time offset4

    - Alt1:

      - TheA time offset4 configured by the network indicating a time from the first LP-WUS MO per periodicity, after which the UE starts PDCCH monitoring via starting the new timer.

      - The time gap from the last LP-WUS MO per periodicity and the start of new timer is not smaller than the minimum time gap given based on UE capability

Agreement

For the UE capability report on the minimum time gap is between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring regardless of SCS, support X=3 candidate values corresponding to {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3.

- FFS: Whether to support different set of values for different receiver types

Conclusion

There is no consensus in RAN1 on whether to introduce any enhancements of PDCCH skipping indication for the interaction with LP-WUS in RRC connected mode in Rel-19.

Agreement

For RRC CONNECTED mode when LP-WUS is configured with Cell DTX, during Cell DTX inactive time, the UE is not expected to monitor LP-WUS both for Option 1-1 and 1-2

Agreement

The case where a UE does NOT supports Rel-17 unified TCI framework but supports LP-WUS is supported in Rel-19. For the TCI state of LP-WUS in RRC CONNECTED mode, select one of the following in RAN1#121 for this case:

- Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS

- Alt2: RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them

The case where a UE supports Rel-17 unified TCI framework and supports LP-WUS is supported in Rel-19. For the TCI state of LP-WUS in RRC CONNECTED mode,

- When Rel-17 unified TCI framework is configured, LP-WUS follows the activated/indicated TCI-state by MAC-CE/DCI, same as other DL channels/signals (no change to DCI format)

- When Rel-17 unified TCI framework is NOT configured, select one of the following in RAN1#121 for this case

  - Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS

  - Alt2: RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them

Same alternative to be chosen for both cases.

Agreement

Update previous agreement in red:

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable

RAN1#121

Agreement

For LP-WUS MOs in connected mode for Option 1-1/1-2, the time offset1/3 in previous agreement are configured relative to the start of SFN0 where the offset is a symbol-level offset

Agreement

As the initial reply to RAN2 LS in R1-2503616, RAN1 confirms that at least the collision with Active Time, measurement gap, and RAR window monitoring for BFR can be considered for the cases/scenarios on when the UE is not able to monitor LP-WUS.

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, when Rel-17 unified TCI framework is NOT configured or UE does NOT support Rel-17 unified TCI framework

- Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS

Conclusion

There is no consensus in RAN1 whether to specify UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS in RRC CONNECTED mode

Agreement

As the reply to RAN2 LS in R1-2503616, RAN1 assumes that UE is not able to operate LR and MR simultaneously in Rel-19. RAN1 understanding is that the terminology of LR and MR operations are for discussion purpose and will not be specified

- LR operation is the UE operation for LP-WUS monitoring

- MR operation is the UE operation for all other NR signals/channels transmissions/receptions in connected mode

Conclusion

From RAN1 perspective, for the case of potential collision (if any) in Option 1-2, when the UE is not able to monitor all the LP-WUS MO(s) in a LP-WUS periodicity,

- It is up to RAN2 to further discuss and finalize the specification support, if any.

Agreement

For the UE capability report on the minimum time gap between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring regardless of SCS, the same candidate values {V1, V2, V3} are supported for different receiver types

- V1=5ms

- V2=13ms

- V3=37ms

Agreement

For the UAI of preferred time offset for LP-WUS monitoring in RRC connected mode,

- The candidate values are same as those for the UE capability of minimum time gap, i.e., {V1=5, V2=13, V3=37}ms

- The reported UAI value is equal to or longer than the minimum time gap reported by UE capability for a UE

Note: There is no change to the RAN1 agreed definition for UAI

Final reply LS on LP-WUS in RRC_CONNECTED is endorsed in R1-2504888

Agreement

Nominal MO duration (X1, in unit of OFDM symbols) and actual LP-WUS duration (X2, in unit of OFDM symbols) are configured for LP-WUS in connected mode. (Alt C)

- A LP-WUS MO spans the nominal MO duration (i.e., the LP-WUS MO duration is the same as the nominal MO duration.)

- If the number of available OFDM symbols within the nominal MO duration is no less than the actual LP-WUS duration, UE monitors LP-WUS on the first X2 available symbols within the LP-WUS MO.

- Otherwise, UE does not monitor LP-WUS in this MO (i.e., the MO is dropped).

- Note: Any symbols that are not defined as unavailable are available symbols for LP-WUS.

- Further discuss possible introduction of UE capability to restrict configuration of LP-WUS MO and LP-WUS duration. For example,

  - A LP-WUS spans a number of consecutive OFDM symbols according to the configured LP-WUS duration

  - If there is at least one OFDM symbol unavailable for the LP-WUS MO within the OFDM symbols where the LP-WUS would span, the UE does not monitor the LP-WUS in the MO

Agreement

For UE to determine whether a symbol is available for LP-WUS in connected mode, at least the following is supported:

- A unit level bitmap with a periodicity 10, 20, or 40 units and a 14-bit or 28-bit symbol-level bitmap that covers 1 or 2 slots can be configured, where each unit is 1 or 2 slots for 14-bit or 28-bit symbol-level bitmap, respectively, with a maximum periodicity of 40ms.

  - ‘1’ in unit-level bitmap means the symbol level bitmap is applied to determine which symbols are unavailable in the unit for LP-WUS (‘0’ means unavailable).

  - ‘0’ in unit-level bitmap means: all the symbols in the unit are unavailable for LP-WUS

- If the slot-level bitmap is not configured, UE assumes all 1’s for the bitmap.

- If the symbol-level bitmap is not configured, UE assumes all 1’s for the bitmap.

Agreement

For the handling of at least PCell/PSCell SSB for connected mode, the SSB symbols are considered as unavailable for LP-WUS

Agreement

The UL symbols/slots configured in tdd-UL-DL-configurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as unavailable for LP-WUS in connected mode

2.1.2	Remaining Open issues

None

2.2	RAN2

2.2.1	Agreements

General

RAN2#130

R2-2503612	RRC Running CR for LP-WUS WUR	vivo (Rapporteur)	draftCR	Rel-19	38.331	18.5.1	B	NR_LPWUS-Core

Endorsed. Will be updated and reviewed in post meeting email discussion.

R2-2503657	38.304 Running CR for LP-WUS	CATT	draftCR	Rel-19	38.304	18.4.0	NR_LPWUS-Core

Endorsed. Will be updated and reviewed in post meeting email discussion.

R2-2503759	Running 37.340 CR for LP-WUS	ZTE Corporation, Sanechips	draftCR	Rel-19	37.340	18.5.0	B	NR_LPWUS-Core

Endorsed. Will be updated and reviewed in post meeting email discussion.

R2-2503807	Running MAC CR for LP-WUS	Apple (Rapporteur)	draftCR	Rel-19	38.321	18.5.0	B	NR_LPWUS-Core

Endorsed. Will be updated and reviewed in post meeting email discussion.

R2-2504578	Introduction of Low-Power Wake-Up Signal and Receiver for NR	Ericsson	draftCR	Rel-19	38.331	18.5.1	B	NR_LPWUS-Core

Endorsed. Will be updated and reviewed in post meeting email discussion.

On open issues for TS38.304

For UE_ID based subgrouping , X is 1048576, i.e., the largest UE ID range in all LP_WUS cases is be used for all LP-WUS monitoring cases.

UEs expecting MBS group notification should monitor its PO to receive the MBS group notification regardless of LP-WUS.

On open issues for TS38.331

RRM relaxation / offloading configuration is provided in SIB2.

RAN2 assumes the design of UAI reporting for preferred time offset is same as the legacy, e.g. including the configuration, procedure, as well as prohibit timer, etc.

On LP-WUS in MR-DC

LP-WUS, if supported by UE, can only be configured to be monitored on the PCell, if the MN is a gNB (i.e. for NE-DC and NR-DC) and/or with LP-WUS to be monitored on the PSCell, if the SN is a gNB (i.e. for EN-DC, NGEN-DC and NR-DC).

On UE capabilities

A UE indicating support of LP-WUS reception in IDLE/INACTIVE shall support UE-ID based subgrouping.

From R2 point of view, RRM measurement relaxation and RRM measurement fully offloading are defined as RAN2 capability without UE capability signalling.

UE supporting LP-WUS reception shall also support RRM measurement relaxation and RRM measurement fully offloading

Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

RAN2#129bis

LP-WUS is supported with eDRX, FFS on exact impact if any

Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID = 5G-S-TMSI mod X.

X is based on 32 subgrouping number. Details can be discussed in the running CR.

Send LS to RAN3 (CC SA2/SA3) to inform our agreements on UE ID based subgrouping.

Correct the typo as following for the previous agreed formula of UE_ID based subgrouping for LP-WUS:

        - Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1.

Confirm the principle for determining CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping. Details will be discussed in the running CR.

All the LP-WUS related configurations except for measurement configurations are provided in SIB1. FFS the details on measurement configurations.

Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

RAN2#130

The entry/exit condition for LP-WUS monitoring is mandatory in LP-WUS configuration, if the LP-WUS configuration is provided by the NW.

Full coverage for LP-WUS is not precluded, e.g., if there needs to be a threshold value so that the condition is always fulfilled for all LPWUS UEs.

It is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement based conditions are used for LP-WUS monitoring entry/exit condition, if UE support both measurement types.

RAN2 aim at supporting enabling/disabling LP-WUS monitoring in IDLE/INACTVE per UE, if the solution can be concluded in the August meeting.

RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

RAN2#129bis

RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both.

It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell.

The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ.

How to define LP-RSRP and LP-RSRQ is up to RAN1.

The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review.

Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

RAN2#130

The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation), if there is such configuration. Capture this in the field description. Details will be discussed in the running CR.

It is up to NW to configure the condition for LP-WUS monitoring and/or [R19 serving/ neighboring cell RRM relaxation /R19 serving cell RRM offloading], as in the current RRC running CR.

The threshold of the MR based entry condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, which is the maximum of {SIntraSearchP, SnonIntraSearchP}. Capture this in the field description. Details will be discussed in the running CR.

Procedures for LP-WUS in RRC_CONNECTED

RAN2#129bis

For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used.

Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used.

Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback.

Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously.

If configured, the UE can signal a preferred time offset via UAI signalling.

Ask RAN1 for further information regarding their conclusions.

FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group

For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells.

For NR-DC, the LP-WUS in MCG and SCG can be configured independently.

Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

RAN2#130

Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime.  When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

Check whether we need to capture in MAC that UE is not expected to monitor LP-WUS if not in Cell DTX active period

2.2.2	Remaining Open issues

- For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

  - Remaining details of RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

- For CONNECTED mode, remaining details of procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring

2.3	RAN3

2.3.1	Agreements

RAN3#127bis

The maximum number of subgroups is decided to 31.

Turn the working assumption into agreement and remove the FFS on the presence of the LP-WUS Subgrouping Support Indication IE per cell in the F1AP Paging message.

RAN3#128

| R3-253105 | (BL CR to 38.300) Introduction of low-power wake-up signal and receiver for NR (vivo, NTT DOCOMO INC., Nokia, Samsung, Ericsson, ZTE, CATT, Huawei) | draftCR Endorsed as BL CR |
| --- | --- | --- |
| R3-253106 | (BL CR to 38.413) Introduction of low-power wake-up signal and receiver for NR (ZTE Corporation, Nokia, Samsung, Ericsson, Huawei, CATT, NTT) | CR1261r2, TS 38.413 v18.5.0, Rel-19, Cat. B Endorsed as BL CR |
| R3-253107 | (BL CR to 38.423) Introduction of low-power wake-up signal and receiver for NR (Ericsson, Vivo, NTT Docomo, Huawei, ZTE, Nokia, Samsung, CATT) | CR1341r7, TS 38.423 v18.5.0, Rel-19, Cat. B Endorsed as BL CR |
| R3-253108 | (BL CR to 38.470) Introduction of low-power wake-up signal and receiver for NR (Nokia, Samsung, ZTE, Ericsson, Huawei, CATT) | CR0157r3, TS 38.470 v18.4.0, Rel-19, Cat. B Endorsed as BL CR |
| R3-253109 | (BL CR to 38.473) Introduction of low-power wake-up signal and receiver for NR (Huawei, Nokia, Samsung, Ericsson, CATT, ZTE, Vivo, NTT DOCOMO INC.) | CR1443r7, TS 38.473 v18.5.0, Rel-19, Cat. B Endorsed as BL CR |

2.3.2	Remaining Open issues

- For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

2.4	RAN4

2.4.1	Agreements

LP-WUS UE RRM

RAN4#114bis

Issue 1-1-10: Considerations on higher priority frequency layer

Agreement:

- Support relaxed higher priority frequency layer search at case 1

Issue 1-1-12: On LR measurement metrics LP-RSRP and LP-RSRQ

Agreement:

For frequency range 1, the reference point for the LP-RSRP and LP-RSRQ shall be the antenna connector of the UE

Spec impact and CR split consideration

Agreement:

|  | Clause | Impact Comment |
| --- | --- | --- |
| 3 | Definitions, symbols and abbreviations | Yes |
| 4 | SA: RRC_IDLE state mobility | Yes |
| 5 | SA: RRC_INACTIVE state mobility | Yes |
| 6 | RRC_CONNECTED state mobility | No |
| 7 | Timing | No |
| 8 | Signalling characteristics | FFS |
| 9 | Measurement Procedure | FFS |
| 10 | Measurement Performance requirements | No |
| Annex B | Conditions for RRM requirements applicability for operating bands | Yes |

Issue 1-2-1: General on LR accuracy requirement

Agreement:

No dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin relative to the threshold in the core requirement.

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

Agreement:

For FR1:

    - [3.5] dB can be used for core requirements for LP-RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for LP-RSRP accuracy, under the side conditions Ês/Iot [x, x = -3] dB

    - [3.5] dB can be used for core requirements of SSB based RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for SSB based RSRP accuracy, under the side conditions Ês/Iot [y, y = -3] dB

    - Note: [2 or 2.5] dB RF impairment margin is assumed

Issue 1-2-2: Periodicity for LP-WUR RRM requirements

Agreement:

Regardless of  whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Issue 1-2-4-1: On LR measurement filtering requirement

Agreement:

  - The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR.

Issue 1-2-5-2: On MR evaluation requirements

Agreement:

- Reuse legacy Nserv

- Note: the legacy values are Nserv = 4 when DRX = 0.32s and 0.64s, Nserv = 2 when DRX = 1.28 and 2.56s

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR and RAN1 LS R1-2501624

Issue 1-2-6-1: On the number of SSBs

Agreement: Aim to conclude the issue by Tuesday in RAN4 #115.

Issue 1-2-6-2: On SSBs number for different SSB periodicities

Agreement

Same SSB number for different SSB periodicity

Agreement

When SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay.

Note: This above agreement based on the assumption that the wake up duration is defined as the ramping time + SSB number * SSB periodicity

Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1

Agreement:

  - No need to define the LR-WUR based cell selection criterion evaluation in case 1.

RAN4#115

Issue 1-1-10: Higher priority frequency layer measurement requirements

Agreement:

For case 1:

  - MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60

    - Note: RAN4 assumes Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met for case 1.

For case 3:

  - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60

  - When the condition of Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is NOT met, the same requirement for higher priority, equal priority and lower priority carriers:

    - 16 times of Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter are applied

Issue 1-1-17 Reply RAN1 LS R1-2503103

Agreement on the reply LS to RAN1:

- Confirm to RAN1 that for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

- RAN4 agreed that: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Issue 1-2-4-1: On LR measurement filtering requirement

Agreement:

No need to define filtering requirement for LP-RSRP and LP-RSRQ measurements.

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

Agreement: This issue is closed.

Issue 1-2-4-2-3: On how to define LR evaluation requirements

Working assumption:

Option 1: using x1=2*x and y1=2*y for the evaluation requirement.

Option 2: The value x1 for LP-SS based and y1 for SSB based LR evaluation requirement could be defined by adding extra [1 or 2 ] samples

Issue 1-2-6-1: On the number of SSBs (for RAN1 LS R1-2501624)

Agreement:

For the 400ms and 800ms ramping up time: {5, 5} samples

For the 10ms ramping up time: 3 samples

Agreement:

The numbers of SSB samples agreed in the LS is based on the scenario that the MR waking up is triggered by LP-WUS.

Agreement

For the side conditions, RAN4 assume the side conditions is the side conditions of the LP-WUS monitoring used by RAN1

Issue 2-1-5: On ideal RSRP/RSRQ in simulation

Agreement:

Use method 1 in future simulation alignment

Issue 2-1-11: On I and Q branches in simulation

Agreement:

For RAN4 simulation alignment, RAN4 simulation should consider power from both I and Q branches

Issue 2-2-2: Summary on proposal/observations based on simulation results

Working assumption 1:

The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRP at -3 dB SINR is [from 1 to 3];

When LP-SS samples number is larger than 3 at -3 dB SINR, the extra gain on accuracy for LP-SS RSRP is smaller;

Working assumption 2:

The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRQ for AWGN and TDL-C channel at -3 dB SINR is [2 or 3];

LP-WUS UE RF

RAN4#114bis

The new version of TR 38.774 v0.3.0 is approved in [48];

The topic summary for LP-WUS RF is recorded in [78];

The WF on UE RF is approved in [88], with the following detailed agreements:

Issue 1-1-1: Any RF impacts in RAN2 LS R2-2501394

Agreement:

- No RF impact corresponding to RAN2 LS R2-2501394

Issue 1-1-3: Operation bands for LP-WUS in TR 38.774

Agreement:

  - n47, n46 ,n96 and n102 should be removed for LP-WUS.

Issue 2-1-1: SNR for OOK-based LP-WUS

- Proposals

  - Proposal 1: Conclude SNR for OOK-based LR.  (Moderator)

| FR1 OOK-based receiver | SNR | RM coding | Length of information bits | ADC | CRC | Note | REFSENS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Huawei | -4dB for both | 8/16/32 | 5bit? | 4-bit | No | SNR: -2.15 dB (8 bit), -4.63 dB (16 bit), -5.94 dB (32 bit) |  |
| Sony | -7 to -3dB |  |  |  |  |  |  |
| Spread trum | -3 |  |  |  |  | SNR last meeting |  |
| vivo | -6.5dB | 16/20 | 5bit | 4-bit | No | SNR: -6.5dB for 16bit RM coding without Te or 20bit RM coding with Te |  |
| CATT | -3.3dB |  |  |  |  | SNR last meeting | -96.3/-93.3 |
| CMCC | -2.5dB |  |  |  |  | SNR last meeting |  |
| ZTE | -1.5dB | in TS38.212 Clause 5.3.3.3 | 5bit | 4-bit | No | SNR -1.5dB (without ML), -7.5dB (with ML) | 9dB higher than MR |
| Nokia | >RAN1 SNR |  |  |  |  | SNR last meeting |  |
| E/// | -1.5 dB |  |  |  |  | SNR last meeting |  |
| QC | -4.0 dB |  |  |  |  | SNR as side condition. Assume RAN1: NF + IM = 17.0 dB, and 14.0 dB? |  |
| LGE | -7 to 8 dB |  |  | 4bit |  | SNR last meeting |  |

Agreements:

- Companies are encouraged to provide the simulation results with the assumption of the RM coding of 8bit, 16bit and 32bit to align simulations in RAN4. It is targeted to conclude SNR value for envelope detection and OFDM sequence detection next RAN4 meeting.

Issue 2-1-5: How to map two LP-WUR types with UE capability

Agreement:

  - RAN4 define two LP-WUR types (Type 1 and Type 2), no need 1-to-1 mapping between waveform and LP-WUR type.

  - Requirement applicability should be clearly defined.

  - RAN4 need to further discuss how to design the RMC for each type, how to ensure the performance of each type properly.

Issue 2-2-2: Diversity gain value for FR2 LP-WUR

Agreement:

- [-3]dB diversity gain is assumed for FR2 LP-WUR REFSENS derivation

Issue 2-2-3: NF+IM for FR2 LP-WUR

Agreements:

  - RAN4 further discuss FR2 LPWUR, NF+IM value.

Issue 2-2-4: Spherical coverage for FR2 LP-WUR

Agreement:

- The coverage matching requires the same spherical coverage percentile of WUR and MR for different FR2 power classes.

- FFS on whether to skip the test of FR2 LP-WUR for spherical coverage.

Issue 2-3-1: ASCS requirements for OOK-based receiver

Agreement:

- For Type1 UE requirements, which are derived based on the assumption that demodulation of OOK content of LP-WUS is targeted

  - 1 RB guard for ASCS and 0.5dB de-sens

- For Type2 UE requirements, which are derived based on the assumption that demodulation of OFDM content of LP-WUS is targeted

  - 0 RB guard for ASCS and 0 dB de-sens

- For Type 1 and Type2 UE are not limited to specific LP-WUS waveform.

- FFS how to define test case and requirement applicability for Type 1 and Type 2

  - FFS how type 1 and type 2 capability related to RAN1 UE feature

- A UE can only support either Type 1 or Type 2 but not both for LP-WUR

Issue 2-3-3: Channel bandwidth for ASCS Test case

Agreements:

  - ASCS requirement should be applied to all supported UE CBW. Conformance test case and potential down-selection will be defined by RAN5.

  - For conformance testing, it is expected that not all CBW need to be tested.

Issue 2-3-4: whether high in-band signal (both LP-WUS and NR) case is considered for alternative ASCS requirements

Agreements:

  - FFS For ASCS, whether additional test case that the wanted signal is 14dB higher than LP-WUS REFSENS, i.e., REFSENSLP_WUR + 14 dBm, should be considered or not.

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB for SET-1 (worse NF) LP-WUR

- Proposals

  - Proposal 1: check the following input from companies and converge the range for set-1 LR.  (Moderator)

|  | vivo | CMCC | ZTE | Huawei | Nokia | E/// | Sony | QC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACS (dB) | 28 | 22-24 | 24 | 11 (Note1) | 22 | Note 2 | Higher wanted signal (Note3) | Higher wanted signal Note4 |
| Guard RB (as test parameter, not requirements) | 1RB | 1RB | 1RB | Less RB | 2RBs |  |  |  |

Note 1: For LP-WUR type aimed at moderate power consumption, the guard RBs could be derived with better phase noise profile and the interferer level could similar to that of MR.

Note 2: The same set of the inteference signal and wanted signal defined for both two set of the LP-WUS requirments. Further discuss the number of guard RB for two set of requirment.

Note 3: A higher wanted signal can be specified for OOK type LP-WUR ACS requirement to balance the power consumption and interference mitigation capability.

Note 4: LPWUS ACS is specified as (assuming higher wanted signal is higher than refsens by 14 dB). DgMR is the diversity gain used for legacy PDSCH REFSENS derivation
ACSLPWUS=  NFlegacy-NFLPWUS+10logCBWBWLPWUS+ACSlegacy-DgMR

  - Proposal 2: Whether consider a higher the wanted signal for set-1 ACS requirements, e.g., LP REFSENS+ >14dB.  (Moderator)

WF:

- Encourage companies to update the simulation analysis based on latest agreements. Target to conclude ACS requirements and test parameters next meeting.

Issue 2-4-2: Whether adopt the same wanted power level for Set-2 LR?

Agreement:

- The same set of the interference signal level defined for both two types of the LP-WUS requirements.

Issue 2-4-4: Case 1 ACS test parameter

Agreements:

Specify case 1 and case 2 ACS test parameters for Type 1 and Type 2.

  - Define interferer as BWinterferer of 5 MHz.

Issue 3-1-1: IBB, OBB requirements

Agreements:

- Same interference level as MR. further discuss and decide next meeting on relaxed wanted signal.

Issue 4-1-1: Whether LR can be tested directly

Agreements:

- Down select below two approaches for LP-WUR UE RF requirements:

- Option 1: stick to 1% MDR LP-WUS as performance metric, but with a note in TS that LP-WUR ON is not mandated.

  - The note depends on RAN1 conclusion

- Option 2: update it to 1% MDR of PDCCH triggered by LP-WUR

Issue 4-1-4: UE test mode for LP-WUR

Agreements:

- Test mode design as well as the details of test mode for LP-WUS verification can be left to RAN5 with RAN4 inputs.

Issue 5-1-1: CR work splitting

TS 38.101-1

| Requirements | Title | Leading company |
| --- | --- | --- |
| General aspects | Clause 2, References Clause 3, Definitions, symbols and abbreviations Clause 4, General | vivo |
| Operation band | 5.2M Operating bands for LP-WUS/WUR | CATT |
| Bandwith (RBs) | 5.3A UE channel bandwidth for LP-WUS/WUR | CATT |
| REFSENS | 7.3M Reference sensitivity for LP-WUS/WUR General REFSENS | Nokia, Huawei |
| Max input level | 7.4M Maximum input level for LP-WUS/WUR | E/// |
| ACS and ASCS | 7.5M Adjacent channel and subcarrier selectivity for LP-WUS/WUR ACS ASCS | QC, E///, Nokia, Huawei |
| Blocking | 7.6M Blocking characteristics for LP-WUS/WUR In-band blocking Out-of-band blocking Narrow band blocking | Huawei |
| Spurious response | 7.7M Spurious response for LP-WUS/WUR | ZTE |
| Intermodulation | 7.8M Intermodulation characteristics | ZTE, CMCC |
| Spurious emissions | 7.9M Spurious emissions for LP-WUS/WUR | ZTE, OPPO |
| Annex A Measurement channels | DL reference measurement channels | Nokia |

TR 38.774 TP owners for detailed sub-clauses

| Requirements | Title | Leading company |
| --- | --- | --- |
| General aspects |  | vivo |
| System parameters | Operation band Channel raster | CATT |
| Clause 6 Simulations (including typical simulation results and observations) | 6.3.1 General 6.3.2 SNR simulations | vivo |
|  | 6.3.3 ASCS simulations | vivo |
|  | 6.3.4 ACS simulations | vivo |
|  | 6.3.5 other aspects | vivo |
| RF requirements (including RAN4 technically discussions/considerations and final conclusion) | 7.1.1 General 7.1.2 Rx SNR evaluations | Nokia, Huawei |
|  | 7.1.3 Architecture and NF considerations | Nokia, Huawei |
|  | 7.1.4 REFSENS requirements | Nokia, Huawei |
|  | 7.1.5 ASCS requirements | QC, E///, Nokia, Huawei |
|  | 7.1.6 ACS requirements | QC, E///, Nokia, Huawei |
|  | 7.1.7 IBB requirements | Huawei |
|  | 7.1.8 OBB requirements | Huawei |
|  | 7.1.9 other Rx requirements | ZTE, CMCC |
| Clause 8 Testability | 8.1.1 General Framework | Nokia, Samsung |
|  | 8.1.2 other | Nokia, Samsung |
| Others? | Other clause needed ? | vivo |

RAN4#115

The new version of TR 38.774 v0.4.0 is approved in [83];

The topic summary for LP-WUS RF is recorded in [118];

The draft Big CR for LP-WUS UE RF is endorsed in [84];

The WF on UE RF is approved in [TBA], with the following detailed agreements:

Issue 1-1-3: Maximum Transmission bandwidth

Agreement:

  - Maximum Transmission bandwidth should be defined in table below

Table 5.3.2x-1: Maximum transmission bandwidth configuration NRB_WUR for WUR

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR | NRB_WUR |
| 15 | N/A | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 |
| 30 | N/A | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 |
| 60 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Issue 2-1-1: SNR for LP-WUR two types

Agreements:

- There is no repetition of information bits to derive OFDM-based receiver SNR.

- SNR=-4.5dB for envelope detection and OFDM-based receiver.

Issue 2-1-2: Whether update the equation for REFSENS

Agreements:

- Use legacy REFSENS approach to derive requirements in spec. For TR, some statement of this issue may be presented.

Issue 2-1-3: RMC for  LP-WUR requirements

Agreements:

- In the FR1 spec, there should be two tables for 15kHz and 30kHz;

- The following parameter is agreed. Further adding other parameter (if necessary) is not precluded.

- Table A.3.4.2-1 Fixed reference channel for LP-WUR requirements

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LP-WUS bandwidth | RB | 11 |
| Subcarrier spacing | kHz | 15/30kHz |
| RM coding | bits | 16bits |
| CRC |  | No CRC |
| Chip rate |  | M=4 (4 chips in an OFDM symbol) |
| overlaid OFDM sequence |  | Length 33: generated by 31-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | 4 |
| WUS duration for OOK |  | 8 OFDM symbols |
| WUS duration for OFDM |  | 2 OFDM symbols |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits |  | 5bits |

Issue 2-1-6: UE feature for LP-WUS

WF:

- FFS if RAN4 will specify capability (or capabilities) for the two different types of LP-WUR.

Issue 2-2-1: Combining strategy for FR2 LP-WUR

Agreements:

- The combining strategy of FR2 LP-WUR is selection-based diversity for the two Rx chains with orthogonally polarized antennas, just for requirement development perspective.

Issue 2-2-2: Diversity gain value for FR2 LP-WUR

Agreements:

- -3dB for diversity gain.

- FFS: 1% MDR REFSENS requirement should be satisfied for each orthogonal link polarization respectively.

Issue 2-2-3: Peak direction for FR2 LP-WUR

Agreements:

- All FR2 directional LPWUS requirements apply in the same direction as the corresponding legacy Rx requirement.

Issue 2-2-4: NF+IM for FR2 LP-WUR

Agreements:

- For FR2, NF+IM is [12] dB for at least type-2 LP-WUR.

- FFS two values for two types.

Issue 2-2-5: Spherical coverage for FR2 LP-WUR

WF:

- FFS FR2 LP-WUR spherical coverage whether can be skipped.

Issue 2-2-6: other Rx requirements for FR2 LP-WUR

WF:

- FFS whether skip out-of-band blocking and spurious response requirements for FR2 LP-WUR.

Issue 2-3-1: proposals to update the agreed ASCS requirements for LP-WUR last meeting

WF:

- Prepare Draft CR directly based on last meeting agreement of ASCS.

Issue 2-3-2: whether high in-band signal (both LP-WUS and NR) case is considered for ASCS requirements

Agreements:

- Keep using low power level for ASCS, but use Max input power requirement to verify CBW with full NR+LP-WUS RBs.

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB for LP-WUR

Agreement:

- 0 guard RB is assumed for deriving both type-1 and type-2 ACS requirement.

Issue 2-4-2: case 1 ACS test parameter

WF:

- Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting.

Issue 2-4-3: case 2 ACS test parameter

WF:

- Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting.

Issue 3-1-4: detailed requirements and table for test parameters of IBB, OBB, NBB, Intermodulation test case

WF:

  - Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting.

Issue 3-2-1: spurious response requirements for FR1 LP-WUR

WF:

- Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting.

Issue 3-2-2: spurious emission requirements for FR1 LP-WUR

Agreements:

- For FR1, the spurious emission requirements for LP-WUR will be considered same as MR.

- For FR2, the spurious emission requirements for LP-WUR will be considered same as MR.

Issue 3-2-3: Maximum input level requirements for FR1 LP-WUR

Agreements:

- In spec, for requirements using -25dBm as single value.

- The max. input power level is specified for all channel bandwidths. FFS the side condition

- in the presence of legacy RBs with the same PSD as the LPWUS RBs.

- And  no NR RBs in the CBW

Issue 3-2-5: detailed table for test parameters of spurious response, spurious emission, Maximum input level, and other test cases

WF:

- Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting.

Issue 4-1-1: How to verify LP-WUR

Agreement:

Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods)

- The ultimate test method including the potential test mode will be decided by RAN5

Issue 4-1-2: FR2 skipping rule

WF:

- Further check whether FR2 LP-WUR spherical coverage can be skipped.

Issue 4-1-3: skipping rule for LP-WUR different mode

Agreements:

- For idle-mode only or connected-mode only LP-WUR, the test should be done accordingly.

- FFS For LP-WUR support both.

LP-WUS BS RF

RAN4#114bis

The topic summary for LP-WUS BS RF is recorded in [77];

The WF for LP-WUS BS RF is endorsed in [81];

Issue 1-1: How to specify the BS manufacturer declaration for LP-WUS

- WF

  - Manufacturer shall declare the support of LP-WUS for each supported band, [frequency range] and channel bandwidth.

  - If the BS supports LP-WUS power boosting, manufacturer shall declare the LP-WUS power boosting level it could support with a minimum of +[X]dB.

    - X can be different for different channel bandwidth.

    - FFS on the value of X and use 3 as starting point.

Issue 1-2: How to comprehend the de-boost on the NR transmission power

- Proposals

  - How to attend the required power for LP-WUS power boosting should be treated as implementation issue and thus there is no need to reflect it in the specification. (Nokia)

- Agreement in AH

  - There is no need to reflect NR power de-boosting in the specification.

Issue 1-3: Whether to define EVM requirements for LP-WUS

- Options

  - Option 1: No need to define EVM requirements for LP-WUS. (Huawei)

    - Because simulation results show that the impact on missed detection error (MDR) is minor for EVM up to 30%, as such the EVM requirements do not seem relevant for LP-WUS based on RAN1 assumptions for LP-WUS operation (in low SNR) and design. (Nokia)

    - Because frequency-domain EVM on LP-WUS “constellations” has minor impact on the receiving MDR. (ZTE)

  - Option 2: EVM requirements for LP-WUS is needed. (CMCC)

    - Option 2-1 (CATT):

      - RAN4 to specify EVM requirement only for “ON” chips of OOK symbols.

      - RAN4 to specify different EVM requirements for the cases with or without overlaid OFDM sequence.

      - RAN4 to specify EVM requirements for ON chips of OOK symbols without overlaid OFDM sequence as 17.5%.

      - RAN4 can wait until RAN1 finalizes the overlaid OFDM sequence design before deciding how to specify the EVM requirement for ON chips of OOK symbols with overlaid OFDM sequence.

    - Option 2-2 (Ericsson):

      - Define the EVM levels of each NR carrier for OOK4 modulation scheme on LP-WUS as 8%.

      - Wait further RAN1 progress on PAPR discussion.

- WF

  - Further check in next meeting whether EVM for LP-WUS needs to be specified.

Issue 1-4: Whether/how to verify EVM requirements for LP-WUS

- Options

  - Option 1: There is no need to do the EVM verification e.g. utilizing a higher-order EVM test (64QAM) for NR with embedded LP-WUS. (Nokia)

  - Option 2: Without specific EVM requirement defined for LP-WUS, just use a higher-order EVM test for NR with embedded LP-WUS as the verification metric. (Huawei)

  - Option 3: The OOK4 signal quality test can be dependent on the 64QAM EVM test. (Ericsson)

  - Option 4: Others.

- WF

  - Consider together with EVM requirement for LP-WUS

    - If no EVM requirement is defined, no verification is needed

    - If EVM requirement is defined, FFS on whether verification is needed.

Issue 1-5: Submitted TP

- Proposals

  - Proposal 1: R4-2503637 TP to TR 38.774 on LP-WUS power boosting (Clause 7.2.2). (ZTE)

  - Proposal 2: R4-2504392 TP on TX signal quality (clause 7.2.4, Ericsson)

- WF

  - Postpone R4-2504392 to next meeting

Work split for LP-WUS TR/TS BS part

- Proposals

  - TPs for TR 38.774 BS part, and draft CRs for TS 38.104/38.141-1 are split among companies as below:

| TPs for TR 38.774 | Volunteer companies |
| --- | --- |
| 7.2.1 General | ZTE |
| 7.2.2 LP-WUS power boosting | ZTE |
| 7.2.3 Regulation relevant spectrum requirements | Already done |
| 7.2.4 Transmitted signal quality | CATT |
| 7.2.5 Other RF requirements | Nokia |
| Draft CR for TS 38.104 | - Power boosting definition: Huawei - Transmitter signal quality (if needed): |
| Draft CR for TS 38.141-1 | - Manufacturer declaration: CATT - Transmitter signal quality (if needed): |

RAN4#115

The topic summary for LP-WUS BS RF is recorded in [121];

The WF for LP-WUS BS RF is recorded in [158];

Issue 1-1: How to specify the BS manufacturer declaration for LP-WUS

- WF

  - Manufacturer may declare the LP-WUS power boosting level it can support with a minimum of +3dB which is applicable to CBWs equal to or larger than 50 MHz, different boosting levels may be declared for smaller CBWs.

Issue 1-2: Whether/how to define EVM requirements for LP-WUS

- Options

  - Option 1:

    - No BS LP-WUS EVM requirement is specified.

      - The background and consideration of LP-WUS transmitted signal quality are captured in TR 38.774.

  - Option 2:

    - BS LP-WUS EVM requirement as follows, no conformance test for this requirement.

- For BS type 1-C and 1-H, the EVM levels of each NR carrier for OOK-4 modulation scheme on LP-WUS is specified in table 6.5.2.2-2.

Table 6.5.2.2-2: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for LP-WUS | Required EVM |
| --- | --- |
| OOK-4 | [FFS] % |

- WF

  - Down select the options in next meeting

Issue 1-4: Restriction on the applicability of LP-WUS operation

- WF

  - Option 1:

    - It is proposed to restrict in-band LP-WUS operation to cases where the NR signal employs a maximum modulation order of 64QAM. This limitation should be explicitly specified to ensure robust system performance.

  - Option 2

    - No restriction of LP-WUS operation, this is BS implementation issue.

- WF

  - Down select the options in next meeting

LP-WUS Demodulation

RAN4#115 (first meeting)

The topic summary for LP-WUS Demod is recorded in [122];

The WF for LP-WUS Demod is endorsed in [155];

Issue 1-1: Work plan for Rel-19 LP-WUS demodulation performance part

- Agreement

- Table 1: Work plan for performance requirements of LP-WUS/WUR for NR

| - Meeting | - Target |
| --- | --- |
| - RAN4 115 (May 2025) | - Discuss work scope and feasibility of test cases |
| - RAN4 116 (Aug 2025) | - Further discuss general test cases and simulation assumptions for each test case. - Agree the test cases to be defined. |
| - RAN4 116-bis (Oct 2025) | - Companies provide initial simulation results. - Work on simulation alignment and discuss on CR split. |
| - RAN4 117 (Nov 2025) | - Companies provide further simulation results and draft CRs. |
| - RAN4 118 (Feb 2026) | - CR review and agree on the formal CR |

Issue 2-1-1: Whether to define a test mode for LP-WUS/WUR

- Agreement

- Take RF approach as a starting point to test MDR for demodulation requirements.

RF agreement

Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods)

- FFS whether test mode is recommended for MDR testing.

Issue 2-1-2: Whether to consider shortening the test duration (if a test is introduced)

- Agreement:

- FFS whether to introduce FAR test and further study feasibility of introducing FAR test

- FFS whether test mode is recommended for FAR testing.

Issue 2-1-3: Whether to define separate test cases for idle/inactive and connected modes

- WF

  - Option 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different. If both IDLE/INACTIVE mode and CONNECTED mode are supported by the LP-WUR, which mode is selected for test should be up to UE declaration.

Issue 3-1-1: Whether to introduce demodulation requirements for LP-WUS

- Agreement

  - Introduce demodulation requirements for LP-WUS

Issue 3-1-2: Whether to introduce MDR test

- Agreement

  - Introduce MDR test for LP-WUS

Issue 3-1-3: Whether to introduce FAR test

- WF

  - Option 1: Yes.

  - Option 2: No.

Issue 3-1-4: Test metric for MDR test

- Agreement

  - 1% MDR

Issue 3-1-5: Test metric for FAR test (if introduced)

- WF

  - Option 1: 1% FAR

  - Option 2: 0.1% FAR

Issue 3-1-6: LP-WUS waveform

- WF

Option 1: Consider both OOK-1 and OOK-4

Issue 3-1-7: Receiver type

- WF

  - Option 1: Consider both OOK and OFDM receivers

Issue 3-1-8: Frequency range

- WF

  - Option 1: Consider both FR1 and FR2.

  - Option 2: Prioritize FR1; FFS FR2.

Issue 3-1-9: Duplex mode

- WF

  - Option 1: Consider both TDD and FDD.

Issue 3-1-10: SCS

- WF

- Option 1: 15kHz for FDD FR1 and 30kHz for TDD FR1.

- Option 2: 15kHz for FDD FR1, 30kHz for TDD FR1 and 120KHz for FR2

Issue 3-1-11: Bandwidth

- WF

  - Option 1: For the channel bandwidth and SCS, take below assumptions for the starting points

    - 10MHz CHBW with SCS 15KHz for FR1 FDD

    - 40MHz CHBW with SCS 30KHz for FR1 TDD

    - 100MHz CHBW with SCS 120KHz for FR2

Issue 3-1-12: Number of RBs

- WF

  - Option 1: 11RB

Issue 3-1-13: Antenna configuration

- WF

  - Option 1: 1T1R.

Issue 3-1-14: Channel model

- WF

  - Option 1: TDLA30-10 only

  - Option 2: TDLC300-100 only

  - Option 3: Both TDLC300-100 and TDLA30-10

Issue 3-1-15: Time offsets

- WF

  - Option 1: T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

Issue 3-1-16: Frequency offsets

- WF

  - Option 1: For OFDM based receiver [5] ppm and for OOK based receiver [10] ppm

  - Option 1: up to 20 ppm

Issue 3-1-17: Whether to define performance requirements for LP-SS

- WF

  - Option 1: No

  - Option 2: Postpone further study on LP-SS until decided if performance requirements are to be introduced for LP-WUR/LP-WUS (OOK receiver)

Issue 3-1-18: Whether to define single requirement for Type 1 and Type 2 receivers

- WF

  - Option 1: Further discuss based on simulation results if one or two sets of requirements are to be defined

Issue 3-1-19: Coding/decoding algorithms for LP-WUS

- WF

  - Option 1: 1 or 2 information bits for small block coding and 3 or more information bits for RM coding

Issue 3-1-20: Whether to introduce dedicated PDSCH, PBCH and CSI reporting requirements for the main receiver

- Agreement

  - Do not introduce dedicated PDSCH, PBCH and CSI reporting requirements for the main receiver

2.4.2	Remaining Open issues

- RRM：For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring.

  - Remaining details of LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell.

  - Remaining details of requirements for further RRM relaxation of UE MR for both serving and neighbor cell measurements, and requirements for UE serving cell RRM measurement offloaded from MR to LP-WUR.

- RF：

  - CR for UE RF requirements. TR editing. At least consider the following aspects:

    - REFSENS

    - Maximum input level

    - ACS

    - ASCS

    - In-band blocking

    - Out-of-band blocking

    - Narrow band blocking

    - Spurious response

    - Wide band Intermodulation

    - Spurious emissions

  - BS RF requirements.

- UE Demod：

  - Demodulation requirements

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

### RAN1#120bis

- R1-2501710	Discussion on LP-WUS and LP-SS Design	FUTUREWEI

- R1-2501737	Discussion on LP-WUS and LP-SS Design	EURECOM

- R1-2501767	Discussion on LP-WUS design	ZTE Corporation, Sanechips

- R1-2501814	Remaining issues on LP-WUS and LP-SS design	vivo

- R1-2501845	Discussion on LP-WUS and LP-SS Design 	TCL

- R1-2501875	Discussion on LP-WUS and LP-SS design	Spreadtrum, UNISOC

- R1-2501999	Design of LP-WUS and LP-SS	CATT

- R1-2502007	LP-WUS and LP-SS design	Nokia

- R1-2502075	Discussion on LP-WUS and LP-SS design	NEC

- R1-2502167	Discussion on LP-WUS and LP-SS design	CMCC

- R1-2502237	Signal Design of LP-WUS and LP-SS	Huawei, HiSilicon

- R1-2502257	Signal design for LP-WUS and LP-SS	OPPO

- R1-2502302	Discussion on LP-WUS and LP-SS design	InterDigital, Inc.

- R1-2502322	LP-WUS and LP-SS design	Sony

- R1-2502375	Discussion on LP-WUS and LP-SS design	Samsung

- R1-2502448	Discussion on LP-WUS and LP-SS design	Xiaomi

- R1-2502481	Discussion on LP-WUS and LP-SS design	LG Electronics

- R1-2502580	Discussion on the LP-WUS and LP-SS design	Panasonic

- R1-2502615	LP-WUS and LP-SS design	Apple

- R1-2502675	Discussion on LP-WUS and LP-SS design	Sharp

- R1-2502694	Discussion on LP-WUS and LP-SS design	HONOR

- R1-2502711	LP-WUS and LP-SS design	MediaTek Inc.

- R1-2502773	Discussion on LP-WUS and LP-SS design	NTT DOCOMO, INC.

- R1-2502805	LP-WUS and LP-SS design	Ericsson

- R1-2502846	LP-WUS and LP-SS design	Qualcomm Incorporated

- R1-2502880	On LP-WUS and LP-SS design	Nordic Semiconductor ASA

- R1-2502908	Discussion on LP-WUS and LP-SS design	Lenovo

- R1-2503014	Summary #1 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2503015	Summary #2 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2503082	Summary #3 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2503083	Summary #4 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2503097	Summary #5 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2503098	Summary #6 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2503098	Summary #6 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2503099   Final summary of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2501711	Discussion on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

- R1-2501768	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE Corporation, Sanechips

- R1-2501815	Remaining issues on LP-WUS operation in IDLE/INACTIVE modes	vivo

- R1-2501846	Discussion on LP-WUS Operation in IDLE/INACTIVE state 	TCL

- R1-2501876	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum, UNISOC

- R1-2502000	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

- R1-2502008	LP-WUS operation in IDLE/Inactive mode	Nokia

- R1-2502076	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

- R1-2502168	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

- R1-2502238	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

- R1-2502258	Further consideration on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- R1-2502303	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

- R1-2502323	LP-WUS operation in IDLE / INACTIVE modes	Sony

- R1-2502376	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

- R1-2502449	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

- R1-2502482	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

- R1-2502516	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

- R1-2502581	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

- R1-2502616	LP-WUS operation in IDLE/INACTIVE modes	Apple

- R1-2502676	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Sharp

- R1-2502712	LP-WUS operation in IDLE/INACTIVE modes	MediaTek Inc.

- R1-2502774	Discussion on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

- R1-2502806	LP-WUS operation in IDLE and INACTIVE modes	Ericsson

- R1-2502847	LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

- R1-2502879	On LP-WUS operation in IDLE/Inactive	Nordic Semiconductor ASA

- R1-2502907	Discussion on LP-WUS operation in Idle/Inactive modes	Lenovo

- R1-2502618	Summary #1 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2502619	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2503087	Draft LS on the RRM measurement metrics for OFDM-based LP-WUR	Moderator (Apple)

- R1-2503103	LS on the RRM measurement metrics for OFDM-based LP-WUR

- R1-2502620	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2502621	Summary #4 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2502622	Summary #5 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2503147	Summary #6 on LP-WUS operation in IDLE/INACTIVE mode

- R1-2501769	Discussion on LP-WUS operation in CONNECTED mode	ZTE Corporation, Sanechips

- R1-2501816	Remaining issues on LP-WUS operation in CONNECTED modes	vivo

- R1-2501877	Discussion on LP-WUS operation in CONNECTED modes	Spreadtrum, UNISOC

- R1-2502001	System design and procedure of LP-WUS operation for UE in CONNECTED Modes	CATT

- R1-2502009	LP-WUS operation in CONNECTED mode	Nokia

- R1-2502060	Discussion on LP-WUS operation in CONNECTED mode	Panasonic

- R1-2502077	Discussion on LP-WUS operation in RRC CONNECTED mode	NEC

- R1-2502169	Discussion on LP-WUS operation in CONNECTED mode	CMCC

- R1-2502239	Procedures and functionalities of LP-WUS in CONNECTED mode	Huawei, HiSilicon

- R1-2502259	Further consideration on LP-WUS operation in connected mode	OPPO

- R1-2502304	Discussion on RRC CONNECTED mode LP-WUS monitoring	InterDigital, Inc.

- R1-2502324	LP-WUS operation in CONNECTED mode	Sony

- R1-2502377	Discussion on LP-WUS operation in CONNECTED modes	Samsung

- R1-2502450	Discussion on LP-WUS operation in Connected mode	Xiaomi

- R1-2502483	Discussion on LP-WUS operation in CONNECTED modes	LG Electronics

- R1-2502517	Discussion on LP-WUS operation in CONNECTED modes	ETRI

- R1-2502576	Discussion on LP-WUS procedures in Connected mode	TCL

- R1-2502617	LP-WUS operation in CONNECTED modes	Apple

- R1-2502677	Discussion on LP-WUS operation in CONNECTED modes	Sharp

- R1-2502713	LP-WUS operation in CONNECTED modes	MediaTek Inc.

- R1-2502775	Discussion on LP-WUS operation in CONNECTED mode	NTT DOCOMO, INC.

- R1-2502807	LP-WUS operation in CONNECTED mode	Ericsson

- R1-2502848	LP-WUR operation in connected mode	Qualcomm Incorporated

- R1-2502997	FL summary #1 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2503055	FL summary #2 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2503088	FL summary #3 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2503102	FL summary #4 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2503120	FL summary #5 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2503121	Final summary on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2501813	Updated RRC parameters list for Rel-19 LP-WUS/WUR	vivo

- R1-2503100	Summary of RAN1 agreements on LP-WUS/WUR for NR	Rapporteur (vivo)

- R1-2503137	Summary of discussion on RRC parameters for Rel-19 LP-WUS/WUR	vivo (WI rapporteur)

### RAN1#121

- R1-2503228	Remaining Issues on LP-WUS and LP-SS Design	FUTUREWEI

- R1-2503290	Signal Design of LP-WUS and LP-SS	Huawei, HiSilicon

- R1-2503318	Discussion on LP-WUS design	ZTE Corporation, Sanechips

- R1-2503369	Remaining issues on LP-WUS and LP-SS design	vivo

- R1-2503522	Discussion on LP-WUS and LP-SS design	Spreadtrum, UNISOC

- R1-2503573	Discussion on LP-WUS and LP-SS design	Samsung

- R1-2503631	Discussion on LP-WUS and LP-SS Design	TCL

- R1-2503694	LP-WUS and LP-SS design	Nokia

- R1-2503724	Discussion on LP-WUS and LP-SS Design	EURECOM

- R1-2503800	Design of LP-WUS and LP-SS	CATT

- R1-2503838	Discussion on LP-WUS and LP-SS design	CMCC

- R1-2503889	Discussion on LP-WUS and LP-SS design	Xiaomi

- R1-2503938	Discussion on LP-WUS and LP-SS design	NEC

- R1-2503964	Discussion on LP-WUS and LP-SS design	InterDigital, Inc.

- R1-2504018	LP-WUS and LP-SS design	Ericsson

- R1-2504066	LP-WUS and LP-SS design	Sony

- R1-2504102	Discussion on LP-WUS and LP-SS design	HONOR

- R1-2504187	Signal design for LP-WUS and LP-SS	OPPO

- R1-2504237	Discussion on the LP-WUS and LP-SS design	Panasonic

- R1-2504250	Discussion on LP-WUS and LP-SS design	LG Electronics

- R1-2504265	LP-WUS and LP-SS design	MediaTek Inc.

- R1-2504328	LP-WUS and LP-SS design	Apple

- R1-2504401	LP-WUS and LP-SS design	Qualcomm Incorporated

- R1-2504437	Discussion on LP-WUS and LP-SS design	Sharp

- R1-2504508	Discussion on LP-WUS and LP-SS design	NTT DOCOMO, INC.

- R1-2504575	On LP-WUS and LP-SS design	Nordic Semiconductor ASA

- R1-2504723	Summary #1 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2504724	Summary #2 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2504831	Summary #3 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2504832	Summary #4 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2504906	Summary #4 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2504908	Final summary of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2504907	Reply LS on LP-WUS UE RF	RAN1, vivo

- R1-2503229	Remaining Issues on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

- R1-2503291	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

- R1-2503319	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE Corporation, Sanechips

- R1-2503370	Remaining issues on LP-WUS operation in IDLE/INACTIVE modes	vivo

- R1-2503523	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum, UNISOC

- R1-2503574	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

- R1-2503643	Discussion on LP-WUS Operation in IDLE/INACTIVE modes	TCL

- R1-2503695	LP-WUS operation in IDLE/Inactive mode	Nokia

- R1-2503801	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

- R1-2503839	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

- R1-2503890	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

- R1-2503939	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

- R1-2503965	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

- R1-2504019	LP-WUS operation in IDLE and INACTIVE modes	Ericsson

- R1-2504067	LP-WUS operation in IDLE / INACTIVE modes	Sony

- R1-2504144	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

- R1-2504188	Further consideration on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- R1-2504238	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

- R1-2504251	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

- R1-2504266	LP-WUS operation in IDLE/INACTIVE modes	MediaTek Inc.

- R1-2504329	LP-WUS operation in IDLE/INACTIVE modes	Apple

- R1-2504335	Summary #5 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2504402	LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

- R1-2504438	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Sharp

- R1-2504509	Discussion on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

- R1-2504574	On LP-WUS operation in IDLE/Inactive	Nordic Semiconductor ASA

- R1-2504331	Summary #1 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2504332	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2504333	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2504334	Summary #4 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2504335	Summary #5 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2504891	Summary #6 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2504892	Summary #7 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2503292	Procedures and functionalities of LP-WUS in CONNECTED mode	Huawei, HiSilicon

- R1-2503320	Discussion on LP-WUS operation in CONNECTED mode	ZTE Corporation, Sanechips

- R1-2503371	Remaining issues on LP-WUS operation in CONNECTED modes	vivo

- R1-2503524	Discussion on LP-WUS operation in CONNECTED modes	Spreadtrum, UNISOC

- R1-2503575	Discussion on LP-WUS operation in CONNECTED modes	Samsung

- R1-2503696	LP-WUS operation in CONNECTED mode	Nokia

- R1-2503748	Discussion on LP-WUS operation in CONNECTED mode	Panasonic

- R1-2503802	System design and procedure of LP-WUS operation for UE in CONNECTED Modes	CATT

- R1-2503840	Discussion on LP-WUS operation in CONNECTED mode	CMCC

- R1-2503891	Discussion on LP-WUS operation in Connected mode	Xiaomi

- R1-2503940	Discussion on LP-WUS operation in RRC CONNECTED mode	NEC

- R1-2503966	Discussion on RRC CONNECTED mode LP-WUS monitoring	InterDigital, Inc.

- R1-2504020	LP-WUS operation in CONNECTED mode	Ericsson

- R1-2504145	Discussion on LP-WUS operation in CONNECTED modes	ETRI

- R1-2504189	Further consideration on LP-WUS operation in connected mode	OPPO

- R1-2504252	Discussion on LP-WUS operation in CONNECTED modes	LG Electronics

- R1-2504267	LP-WUS operation in CONNECTED modes	MediaTek Inc.

- R1-2504330	LP-WUS operation in CONNECTED modes	Apple

- R1-2504403	LP-WUR operation in connected mode	Qualcomm Incorporated

- R1-2504439	Discussion on LP-WUS operation in CONNECTED modes	Sharp

- R1-2504510	Discussion on LP-WUS operation in CONNECTED mode	NTT DOCOMO, INC.

- R1-2504588	Discussion on LP-WUS procedures in Connected mode	TCL

- R1-2504713	FL summary #1 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2504805	FL summary #2 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2504833	FL summary #3 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2504886	FL summary #4 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2504940	Final summary on LP-WUS operation in CONNECTED mode 	Moderator (NTT DOCOMO)

- R1-2504887	Draft reply LS on LP-WUS in RRC_CONNECTED	Moderator (NTT DOCOMO)

- R1-2504888	reply LS on LP-WUS in RRC_CONNECTED	Moderator (NTT DOCOMO)

- R1-2503368	Updated RRC parameters for Rel-19 LP-WUS/WUR	vivo

- R1-2504909	Summary of RAN1 agreements on LP-WUS/WUR for NR Rapporteur (vivo)

- R1-2504956	Summary of discussion on RRC parameters for Rel-19 LP-WUS/WUR	vivo (WI rapporteur)

### RAN2#129bis

- R2-2501743	LS on LP-WUS UE RF (R4-2503003; contact: vivo)	RAN4

- R2-2501769	Discussing on LP-WUS monitoring in Connected mode	Xiaomi

- R2-2501831	LP-WUS in RRC_IDLE/INACTIVE	HONOR

- R2-2501832	Procedures for LP-WUS in RRC_CONNECTED	HONOR

- R2-2501893	Discussion on LP-WUS in RRC_IDLE INACTIVE	NEC

- R2-2501894	Discussion on LP-WUS RRM	NEC

- R2-2501895	Discussion on LP-WUS in RRC_CONNECTED	NEC

- R2-2501954	Running 37.340 CR for LP-WUS	ZTE Corporation, Sanechips

- R2-2501955	Summary of [Post129][213] LP-WUS in MR-DC	ZTE Corporation, Sanechips

- R2-2501960	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips

- R2-2501961	Procedures for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips

- R2-2501967	Further discussion on the criteria for RRM measurement relaxation and offloading	Huawei, HiSilicon

- R2-2501992	Remainng issues on LP-WUS in RRC_CONNECTED	LG Electronics Inc.

- R2-2501997	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung

- R2-2501998	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung

- R2-2501999	Procedures for LP-WUS in RRC Connected Mode	Samsung

- R2-2502005	Remaining issues on LP-WUS paging monitoring	Xiaomi Communications

- R2-2502006	Discussion on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications

- R2-2502014	IDLE/INACTIVE mode procedures for supporting LP-WUS	Tejas Network Limited

- R2-2502016	LP-WUS operation in Connected mode	Tejas Network Limited

- R2-2502097	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon

- R2-2502098	Discussion on UE capability for LP-WUS	Huawei, HiSilicon

- R2-2502141	38.304 Running CR for LP-WUS (CATT)	CATT

- R2-2502142	Summary of [Post129][212][LPWUS] Running CR for TS 38.304 (CATT)	CATT

- R2-2502143	LP-WUS in RRC_IDLE/INACTIVE	CATT

- R2-2502144	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT

- R2-2502145	Analysis on LP-WUS for RRC_CONNECTED	CATT

- R2-2502153	RRC Running CR for LP-WUS WUR	vivo (Rapporteur)

- R2-2502154	Whether/How to reduce the number of thresholds for LP-WUS monitoring and RRM relaxation/offloading	vivo

- R2-2502155	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo

- R2-2502156	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo

- R2-2502157	Discussion on LP-WUS WUR in RRC_Connected	vivo

- R2-2502212	LP-WUS in IDLE and INACTIVE	Nokia

- R2-2502213	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia

- R2-2502227	Remaining issues on LP-WUS in RRC IDLE or INACTIVE	LG Electronics Inc.

- R2-2502228	Remaining issues on measurement offloading and relaxation	LG Electronics Inc.

- R2-2502307	Running MAC CR for LP-WUS	Apple

- R2-2502308	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple

- R2-2502309	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Apple

- R2-2502310	Procedures for LP-WUS in RRC_CONNECTED	Apple

- R2-2502324	Discussion on LP-WUS procedure and configuration	OPPO

- R2-2502325	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO

- R2-2502326	Discussion on LP-WUS in RRC_CONNECTED	OPPO

- R2-2502351	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo

- R2-2502447	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated

- R2-2502448	LP-WUS operation in CONNECTED state	Qualcomm Incorporated

- R2-2502449	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated

- R2-2502471	Further discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon

- R2-2502477	LP-WUS in CONNECTED mode	InterDigital

- R2-2502486	RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode	Sony

- R2-2502597	Procedure and Configuration of LP-WUS in RRC IDLE/INACTIVE	Lenovo

- R2-2502598	LP-WUS in RRC Connected Mode	Lenovo

- R2-2502659	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.

- R2-2502660	Discussion on RRM measurement relaxation and offloading	InterDigital, Inc.

- R2-2502722	Discussion on RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC

- R2-2502723	Discussion on LP-WUS operation in CONNECTED mode	CMCC

- R2-2502743	Discussion on LP-WUS in RRC_IDLE and RRC_INACTIVE	Sharp

- R2-2502744	Discussion on RRM measurement relaxation and offloading	Sharp

- R2-2502757	Discussion on neighboring cell measurement with LR	InterDigital, Ericsson, Nokia, Sony, Vodafone, KT

- R2-2502760	Discussion on RRM measurement relaxation and offloading for RRC_IDLE and INACTIVE	China Telecom

- R2-2502882	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell

- R2-2502901	Further Consideration on LP-WUS operation in IDLE/INACTIVE	CMCC

- R2-2502910	LP-WUS in idle and inactive	Ericsson

- R2-2502911	LP-WUS and RRM measurements	Ericsson

- R2-2502912	LP-WUS in connected	Ericsson

- R2-2502913	Introduction of Low-Power Wake-Up Signal and Receiver for NR	Ericsson

- R2-2502931	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	ZTE Corporation, Sanechips

- R2-2502976	Discussion on the LP-WUS handling for Emergency call back	NTT DOCOMO INC..

- R2-2502977	Discussion on the LP-WUS capability issue within non-homogeneous deployment	NTT DOCOMO INC..

- R2-2502995	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated

- R2-2503031	[AT129bis][201][LPWUS] Proposals for UE ID based sub-grouping (CATT)	CATT

- R2-2503032	Discussion report on [AT129bis][202][LPWUS]	vivo

### RAN2#130

- R2-2503313	LS on the RRM measurement metrics for OFDM-based LP-WUR (R1-2503103; contact: Apple)	RAN1

- R2-2503349	Discussing on LP-WUS monitoring in Connected mode	Xiaomi

- R2-2503568	Discussion on LP-WUS in RRC_IDLE INACTIVE	NEC

- R2-2503569	Discussion on LP-WUS RRM	NEC

- R2-2503570	Discussion on LP-WUS in RRC_CONNECTED	NEC

- R2-2503603	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung

- R2-2503604	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung

- R2-2503605	Procedures for LP-WUS in RRC Connected Mode	Samsung

- R2-2503612	RRC Running CR for LP-WUS WUR	vivo (Rapporteur)

- R2-2503613	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo

- R2-2503614	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo

- R2-2503615	Discussion on LP-WUS WUR in RRC_Connected	vivo

- R2-2503651	Remaining issues on LP-WUS paging monitoring	Xiaomi Communications

- R2-2503652	Remaining issues on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications

- R2-2503657	38.304 Running CR for LP-WUS	CATT

- R2-2503658	Summary of [Post129bis][208][LPWUS] Running CR for 38.304 (CATT)	CATT

- R2-2503659	Remaining issues on LP-WUS in IDLE and INACTIVE	CATT

- R2-2503660	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT

- R2-2503661	Analysis on LP-WUS for RRC_CONNECTED	CATT

- R2-2503758	Summary of [Post129bis][206] Remaining issue of LP-WUS in MR-DC	ZTE Corporation, Sanechips

- R2-2503759	Running 37.340 CR for LP-WUS	ZTE Corporation, Sanechips

- R2-2503763	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips

- R2-2503764	Procedures for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips

- R2-2503765	Discussion summary and list of RRC open issue for LP-WUS WUR	vivo

- R2-2503790	Discussion on LP-WUS in RRC_IDLE and INACTIVE	China Telecom

- R2-2503791	Discussion on RRM measurement relaxation and offloading in RRC_IDLE and INACTIVE	China Telecom

- R2-2503807	Running MAC CR for LP-WUS	Apple (Rapporteur)

- R2-2503808	Report of [Post129bis][210][LPWUS] Running CR for TS 38.321 (Apple)	Apple (Rapporteur)

- R2-2503809	Remaining issues of LP-WUS in RRC_IDLE/INACTIVE	Apple

- R2-2503810	Remaining issues of LP-WUS RRM Measurement	Apple

- R2-2503811	Remaining issues of LP-WUS in RRC_CONNECTED	Apple

- R2-2503819	Remainng issues on LP-WUS in RRC_CONNECTED	LG Electronics Inc.

- R2-2503833	LP-WUS in CONNECTED mode	InterDigital

- R2-2503837	Remaining issues on LP-WUS in RRC IDLE or INACTIVE	LG Electronics Inc.

- R2-2503838	Remaining issues on measurement offloading and relaxation	LG Electronics Inc.

- R2-2503867	Open issues on LP-WUS in IDLE/INACTIVE mode	Tejas Network Limited

- R2-2503869	Open issues on LP-WUS operation in CONNECTED mode	Tejas Network Limited

- R2-2503881	Discussion on LP-WUS in RRC_IDLE and RRC_INACTIVE	Sharp

- R2-2503882	Discussion on RRM measurement relaxation and offloading	Sharp

- R2-2503899	Summary of [Post129bis][218][LPWUS] UE Capabilities (Huawei)	Huawei, HiSilicon

- R2-2503900	Further discussion on the LP-WUS in RRC_IDLE/INACTIVE mode	Huawei, HiSilicon

- R2-2503902	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo

- R2-2503953	Discussion on prioritizing the frequencies supporting LP-WUS	Huawei, HiSilicon, vivo, CMCC, CATT, Nokia, Samsung, LG Electronics Inc., Apple, Ericsson, OPPO, Sharp, NEC

- R2-2503954	Further discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon

- R2-2504003	Discussion on LP-WUS procedure and configuration	OPPO

- R2-2504004	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO

- R2-2504005	Discussion on support of dual DRX group with LP-WUS in RRC_CONNECTED	OPPO

- R2-2504066	Further discussion on the criteria for RRM measurement relaxation and offloading	Huawei, HiSilicon

- R2-2504264	LP-WUS in IDLE and INACTIVE	Nokia

- R2-2504265	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia

- R2-2504288	LP-WUS in Idle and Inactive	Ericsson

- R2-2504289	LP-WUS and RRM measurements	Ericsson

- R2-2504290	LP-WUS in Connected	Ericsson

- R2-2504291	LP-WUS and secondary DRX	Ericsson, Huawei, HiSilicon, NEC, LGE, Apple, InterDigital, CATT

- R2-2504363	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.

- R2-2504364	Discussion on RRM measurement relaxation and offloading	InterDigital, Inc.

- R2-2504365	Discussion on neighboring cell measurement with LR	InterDigital, Ericsson, Nokia, Sony, Vodafone, KT, Turkcell

- R2-2504385	Discussion on LP-WUS operation in CONNECTED mode	CMCC

- R2-2504403	Discussion on RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC

- R2-2504404	Further Consideration on LP-WUS operation in IDLE/INACTIVE	CMCC

- R2-2504468	Discussion on LP-WUS in RRC_IDLE/INACTIVE	HONOR

- R2-2504469	Discussion on LP-WUS in RRC_CONNECTED	HONOR

- R2-2504514	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell

- R2-2504555	Open issues on LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated

- R2-2504556	Open issues on LP-WUS operation in CONNECTED state	Qualcomm Incorporated

- R2-2504557	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated

- R2-2504565	RRM measurement relaxation and offloading in RRC IDLEINACTIVE for LP-WUS/WUR	Panasonic

- R2-2504578	Introduction of Low-Power Wake-Up Signal and Receiver for NR	Ericsson

- R2-2504601	LP-WUS in RRC Connected Mode	Lenovo

- R2-2504603	Procedure and Configuration of LP-WUS in RRC Idle/ Inactive	Lenovo

- R2-2504623	Remaining issues for LP-WUS RRM	ZTE Corporation, Sanechips

- R2-2504643	Discussion on the LP-WUS deactivation with RRC dedicated message	NTT DOCOMO INC..

- R2-2504677	Discussion on LP-WUS procedure and configuration	OPPO

- R2-2504678	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO

- R2-2504679	Discussion on support of dual DRX group with LP-WUS in RRC_CONNECTED	OPPO

- R2-2504738	Summary of [AT130][204][LPWUS] Proposals on whether/how to enable/disable LP-WUS, e.g. by RRC/NAS (Huawei)	Huawei, HiSilicon

- R2-2504739	Discussion report on [AT130][205][LPWUS]	vivo

### RAN3#127bis

- R3-251515	Reply LS on LP-WUS subgrouping	RAN2(Huawei)

- R3-251568	(BL CR to 38.470) Introduction of low-power wake-up signal and receiver for NR	Nokia, Samsung, ZTE, Ericsson, Huawei, CATT

- R3-251569	(BL CR to 38.473) Introduction of low-power wake-up signal and receiver for NR	Huawei, Nokia, Samsung, Ericsson, CATT, ZTE, Vivo, NTT DOCOMO INC.

- R3-251570	(BL CR to 38.300) Introduction of low-power wake-up signal and receiver for NR	vivo, NTT DOCOMO INC., Nokia, Samsung, Ericsson, ZTE, CATT, Huawei

- R3-251571	(BL CR to 38.423) Introduction of low-power wake-up signal and receiver for NR	Ericsson, Vivo, NTT Docomo, Huawei, ZTE, Nokia, Samsung, CATT

- R3-251572	(BL CR to 38.413) Introduction of low-power wake-up signal and receiver for NR	ZTE Corporation, Nokia, Samsung, Ericsson, Huawei, CATT, NTT

- R3-251625	Support LP-WUS subgrouping in RRC_IDLE/RRC_INACTIVE	Qualcomm Inc.

- R3-251644	(TP to BL CR for TS 38.410) Potential impacts of LP-WUS	CATT

- R3-251645	(draft) LS on LP-WUS subgrouping	CATT

- R3-251664	Conclusions on RAN3 Impacts of LP-WUS	Nokia

- R3-251665	[TP for BL CR LP-WUS TS 38.473] Conclusions for LP-WUS	Nokia

- R3-251693	Discussion on LP-WUS UE-ID	NEC

- R3-251698	Discussion on remaining issues for LP-WUS	ZTE Corporation

- R3-251699	(TP to TS 38.413) on support of LP-WUS	ZTE Corporation

- R3-251754	(TP to BLCR for TS 38.413 and TS 38.423 and TS 38.300) Discussion on Low-power wake-up signal and receiver for NR	Huawei

- R3-251755	(TP to BLCR for TS 38.473) Discussion on Low-power wake-up signal and receiver for NR	Huawei

- R3-251756	[draft] Reply LS on LP-WUS subgrouping	Huawei

- R3-252023	Support of Low power radio WUS	Ericsson

- R3-252024	(TP for LP-WUS BL CR for TS38.423) Removal of FFS	Ericsson

- R3-252147	Discussion on LP-WUS	NTT DOCOMO INC..

- R3-252181	(TP to 38.473)Discussion on supporting LP-WUS Indicating Paging Monitoring	CMCC

- R3-252209	Remaining Issues on LP-WUS subgrouping	Samsung

- R3-252340	CB:#LP_WUS	NTTDoCoMo

- R3-252342	[TP for BL CR LP-WUS TS 38.473] Conclusions for LP-WUS	Nokia, Huawei

- R3-252362	(TP to BLCR for TS 38.300) Discussion on Low-power wake-up signal and receiver for NR	Huawei, Ericsson, ZTE, Nokia, CATT

- R3-252514	(BL CR to 38.300) Introduction of low-power wake-up signal and receiver for NR	vivo, NTT DOCOMO INC., Nokia, Samsung, Ericsson, ZTE, CATT, Huawei

- R3-252515	(BL CR to 38.473) Introduction of low-power wake-up signal and receiver for NR	Huawei, Nokia, Samsung, Ericsson, CATT, ZTE, Vivo, NTT DOCOMO INC.

### RAN3#128

- R3-253012	Reply LS on paging capability loss issue	RAN2(Huawei)

- R3-253026	LS on LP-WUS subgrouping progress	RAN2(NEC)

- R3-253105	(BL CR to 38.300) Introduction of low-power wake-up signal and receiver for NR	vivo, NTT DOCOMO INC., Nokia, Samsung, Ericsson, ZTE, CATT, Huawei

- R3-253106	(BL CR to 38.413) Introduction of low-power wake-up signal and receiver for NR	ZTE Corporation, Nokia, Samsung, Ericsson, Huawei, CATT, NTT

- R3-253107	(BL CR to 38.423) Introduction of low-power wake-up signal and receiver for NR	Ericsson, Vivo, NTT Docomo, Huawei, ZTE, Nokia, Samsung, CATT

- R3-253108	(BL CR to 38.470) Introduction of low-power wake-up signal and receiver for NR	Nokia, Samsung, ZTE, Ericsson, Huawei, CATT

- R3-253109	(BL CR to 38.473) Introduction of low-power wake-up signal and receiver for NR	Huawei, Nokia, Samsung, Ericsson, CATT, ZTE, Vivo, NTT DOCOMO INC.

- R3-253173	Support LP-WUS subgrouping in RRC_IDLE/RRC_INACTIVE	Qualcomm Inc.

- R3-253215	Discussion on LP-WUS UE-ID	NEC

- R3-253216	Reply LS on LP-WUS subgrouping progress	NEC

- R3-253231	Conclusions for LP-WUS open points	Nokia

- R3-253232	[TP for BL CR LP-WUS TS 38.423] Further Extension of UE Identity Index Value for LP-WUS	Nokia, Ericsson, Huawei, ZTE, Vivo, CATT, NTT Docomo

- R3-253233	[TP for BL CR LP-WUS TS 38.423] Resolution of LP-WUS open points	Nokia

- R3-253255	(TP to BLCR for TS 38.413, TS 38.423 and TS 38.300) Discussion on Low-power wake-up signal and receiver for NR	Huawei

- R3-253256	(TP to BLCR for TS 38.473) Discussion on Low-power wake-up signal and receiver for NR	Huawei

- R3-253257	(TP to BLCR for TS 38.473) Addition of Further Extended UE Identity Index Value	Huawei, Ericsson, Nokia, ZTE, vivo, CATT, Qualcomm Inc, NTTDOCOMO, INC.

- R3-253308	Potential impacts of LP-WUS	CATT

- R3-253309	(TP for BLCR to 38.410) Introduction of LP-WUS	CATT, Nokia, Ericsson, Huawei, Vivo, ZTE, Qualcomm, NTT Docomo

- R3-253400	Discussion on remaining issues for LP-WUS	ZTE Corporation

- R3-253401	[draft] Reply LS on LP-WUS subgrouping	ZTE Corporation, Huawei, Nokia, Ericsson, vivo, CATT

- R3-253457	Discussion on remaining aspects on LP-WUS	Ericsson

- R3-253458	(TP for LP-WUS BL CR for TS38.413) Addition of Further Extended UE Identity Index Value	Ericsson, Huawei, Nokia, ZTE, Vivo, CATT, Qualcomm Inc., NTT DOCOMO

- R3-253643	[draft] Reply LS on LP-WUS subgrouping	Huawei

- R3-253675	Further discussion on LP-WUS	NTT DOCOMO INC..

- R3-253690	Discussion on supporting LP-WUS Indicating Paging Monitoring	CMCC

- R3-253739	(TP to TS 38.413, 38.420) on support of LP-WUS	ZTE Corporation

- R3-253746	Response to R3-253255	Ericsson

- R3-253804	CB:#LPWUS	NTTDoCoMo

- R3-253809	[TP for BL CR LP-WUS TS 38.423] Further Extension of UE Identity Index Value for LP-WUS	Nokia, Ericsson, Huawei, ZTE, Vivo, CATT, NTT Docomo

- R3-253830	(TP for LP-WUS BL CR for TS38.413) Addition of Further Extended UE Identity Index Value	Ericsson, Huawei, Nokia, ZTE, Vivo, CATT, Qualcomm Inc., NTT DOCOMO

- R3-253831	(TP to TS 38.300): Clarification on homogenous deployment	Ericsson, NTT DOCOMO

- R3-253846	Reply LS on LP-WUS subgrouping progress	RAN3(NEC)

- R3-253861	(TP to BLCR for TS 38.473) Addition of Further Extended UE Identity Index Value	Huawei, Ericsson, Nokia, ZTE, vivo, CATT, Qualcomm Inc, NTTDOCOMO, INC.

- R3-253862	Reply LS on LP-WUS subgrouping	RAN3(Huawei)

- R3-253897	(TP to TS 38.420) Introduction of LP-WUS	ZTE Corporation, Nokia, Ericsson, Qualcomm

- R3-253931	(TP to TS 38.420) Introduction of LP-WUS	ZTE Corporation, Nokia, Ericsson, Qualcomm

- R3-253932	[TP for BL CR LP-WUS TS 38.423] Further Extension of UE Identity Index Value for LP-WUS	Nokia, Ericsson, Huawei, ZTE, Vivo, CATT, NTT Docomo

- R3-253933	(TP to BLCR for TS 38.473) Addition of Further Extended UE Identity Index Value	Huawei, Ericsson, Nokia, ZTE, vivo, CATT, Qualcomm Inc, NTTDOCOMO, INC.

### RAN4#114bis

- R4-2503168	BS RF requirements for low-power wake-up signal for NR	Nokia

- R4-2503170	On UE RF requirements for the LPWUR	Qualcomm Incorporated

- R4-2503171	On test method for UE RF requirements for the LPWUR	Qualcomm Incorporated

- R4-2503191	On LP-WUR Rx requirements	Apple

- R4-2503296	Further discussion on simulation assumptions for LP-WUR	vivo

- R4-2503297	Simulation results for LP-WUR	vivo

- R4-2503298	Consideration on remaining RRM issues for LP-WUR	vivo

- R4-2503301	Discussion on simulation assumptions and results for LP-WUS/WUR	Xiaomi

- R4-2503302	Discussion on RRM core requirements for LP-WUS/WUR	Xiaomi

- R4-2503354	Simulation assumption and results for LP-WUSWUR	OPPO

- R4-2503355	Discussion on RRM requirements for LP-WUSWUR	OPPO

- R4-2503383	Simulation assumption and results for LP-WUR	CMCC

- R4-2503384	Discussion on RRM impact of LP-WUR	CMCC

- R4-2503398	Discussion on LP-WUS BS RF requirements	CMCC

- R4-2503399	Discussion on LP-WUS UE RF Rx requirements of REFSENS, ASCS and ACS	CMCC

- R4-2503400	Discussion on LP-WUS UE system parameters requirements	CMCC

- R4-2503401	Discussion on LP-WUS UE RF Rx requirements of IBB, OBB, intermodulation, spurious emissions and others	CMCC

- R4-2503432	Discussion on RRM requirements for LP-WUS/WUR	CATT

- R4-2503483	TP to TR 38.774 on Clause 5 (System parameters)	CATT

- R4-2503484	Further discussion on REFSENS for LP-WUS	CATT

- R4-2503485	Further discussion on other RF requirements for LP-WUS	CATT

- R4-2503486	Further discussion on Testability for UE RF requirements for LP-WUS	CATT

- R4-2503487	Further discussion on BS RF requirements for LP-WUS	CATT

- R4-2503488	draftCR to TS 38.141-1 on declaration of support for LP-WUS	CATT

- R4-2503489	draftCR to TS 38.141-2 on declaration of support for LP-WUS	CATT

- R4-2503527	Discussion of Simulation Assumptions for LP-WUR RRM Requirements	Apple

- R4-2503528	Summary of Simulation Results for LP-WUR RRM Requirements	Apple

- R4-2503533	On RRM core requirements for LP-WUR	Apple

- R4-2503534	Reply LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode	Apple

- R4-2503580	Discussion on simulation assumptions and results for LP-WUS/WUR	LG Electronics Inc.

- R4-2503581	Discussion on RRM core requirements for LP-WUS/WUR	LG Electronics Inc.

- R4-2503627	Topic summary for [114bis][217] NR_LPWUS	Moderator (vivo)

- R4-2503636	Discussion on BS RF requirements for LP-WUS/WUR	ZTE Corporation, Sanechips

- R4-2503637	TP to TR 38.774 on LP-WUS power boosting (Clause 7.2.2)	ZTE Corporation, Sanechips

- R4-2503638	Discussion on REFSENS, ASCS, ACS for LP-WUR	ZTE Corporation, Sanechips

- R4-2503639	Discussion on receiver characteristics for LP-WUR	ZTE Corporation, Sanechips

- R4-2503640	Discussion on testability of LP-WUR	ZTE Corporation, Sanechips

- R4-2503648	Discussion on UE RF requirements for FR2 LP-WUR	Samsung

- R4-2503649	Discussion on testability for LP-WUR	Samsung

- R4-2503792	Discussion on LP-WUS for core part	ZTECorporation,Sanechips

- R4-2503796	Discussion on OOK based LP-WUR simulation	ZTECorporation,Sanechips

- R4-2503803	Draft reply LS to RAN1 on wake-up delay for LP-WUS operation in IDLE/INACTIVE mode	ZTECorporation,Sanechips

- R4-2503804	Further consideration on general aspects of LP-WUS	Huawei, HiSilicon

- R4-2503805	Further consideration on UE REFSENS, ACS requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2503806	Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2503807	Further consideration on UE RF testability issues for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2503808	Further consideration on BS RF for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2503830	TR 38.774 v0.3.0	vivo

- R4-2503831	Draft Big CR for LP-WUS UE RF requirements in Rel-19	vivo

- R4-2503832	TP to TR 38.774 on LP-WUS	vivo

- R4-2503833	Updates on LP-WUS REFSENS, ASCS and ACS	vivo

- R4-2503834	draft CR to 38.101-1 on LP-WUS UE RF	vivo

- R4-2503899	Simulation assumption and additional simulation results for LP-WUR measurement	Huawei, HiSilicon

- R4-2503900	Discussion on RRM requirements for LP-WUR	Huawei, HiSilicon

- R4-2503919	Discussion on RRM core requirements for LP-WUS/WUR	China Telecom

- R4-2504065	views on requirements of the low-power wake-up receiver	Sony

- R4-2504222	Discussion on LP-WUS RRM requirement	Ericsson

- R4-2504223	Simulation assumption on LP-WUS RRM measurement	Ericsson

- R4-2504224	Simulation on LP-WUS RRM measurement	Ericsson

- R4-2504392	TP to TR38.774: BS RF requirement overview for LP-WUS	Ericsson

- R4-2504393	On general issues for WUR	Ericsson

- R4-2504394	On system parameter for WUR	Ericsson

- R4-2504395	On WUR RF requirement testability	Ericsson

- R4-2504396	WUR RF requirement other than REFSENS	Ericsson

- R4-2504397	WUR RF requirement REFSESN ACS ASCS	Ericsson

- R4-2504413	System parameters for LP-WUR	Nokia

- R4-2504414	Fundamental RX requirements for LP-WUR	Nokia

- R4-2504415	Other RX requirements for LP-WUR	Nokia

- R4-2504416	Testability aspects of LP-WUR	Nokia

- R4-2504417	TP for TR 38.774 on LP-WUS	Nokia

- R4-2504466	Simulation results for RRM LP-SS simulation campaign	Nokia

- R4-2504467	Discussion on LP-WUS RRM Simulations	Nokia

- R4-2504468	Discussion on LP-WUS core requirements	Nokia

- R4-2504598	Simulation assumptions for R19 LP-WUS	MediaTek inc.

- R4-2504599	Discussion on the RRM core requirements for LP-WUS	MediaTek inc.

- R4-2504605	RRM requirements for LP-WUR	Qualcomm Incorporated

- R4-2504633	Topic summary for [114bis][308] NR_LPWUS	Moderator (Huawei)

- R4-2504689	Topic summary for [114bis][131] NR_LPWUS_UERF	Moderator(Vivo)

- R4-2504742	Ad-hoc meeting minutes for [114bis][308] NR_LPWUS	Huawei

- R4-2504743	TP to TR 38.774 on LP-WUS power boosting (Clause 7.2.2)	ZTE Corporation, Sanechips, Ericsson, Huawei, Nokia and CATT

- R4-2504744	Way Forward for [114bis][308] NR_LPWU	Huawei

- R4-2504908	WF on RRM requirements for NR_LPWUS	vivo

- R4-2504921	Ad-hoc minutes for NR_LPWUS	vivo

- R4-2505104	WF on UE RF requirements for LP-WUS	Vivo

- R4-2505148	TP to TR 38.774 on Clause 5 (System parameters)	CATT, Ericsson

- R4-2505149	TP to TR 38.774 on LP-WUS	vivo, Ericsson

- R4-2505150	TP for TR 38.774 on LP-WUS	Nokia

- R4-2505234	WF on UE RF requirements for LP-WUS	Vivo

- R4-2505236	TP for TR 38.774 on LP-WUS	Nokia

### RAN4#115

- R4-2505368	On UE RF requirements for the LPWUR	Qualcomm Incorporated

- R4-2505369	Some test skipping proposals for LPWUS	Qualcomm Incorporated

- R4-2505370	dCR to 38.101-1, LPWUS ACS	Qualcomm, Nokia, Ericsson

- R4-2505433	On LP-WUR Rx requirements	Apple

- R4-2505518	Further consideration on UE REFSENS, ACS requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2505519	Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2505520	Further consideration on UE RF testability issues for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2505521	Further consideration on BS RF for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2505522	draft CR to TS 38.101-1 on Blocking characteristics for LP-WUR	Huawei, HiSilicon

- R4-2505523	TP for 38.774: Blocking characteristics for LP-WUR	Huawei, HiSilicon

- R4-2505524	draft CR to TS 38.104 on introduction of general part and power boosting definition for LP-WUS	Huawei, HiSilicon

- R4-2505572	Topic summary for [115][223] NR_LPWUS	Moderator (vivo)

- R4-2505665	Remaining issues for simulation for LP-WUR	vivo

- R4-2505666	Further simulation results for LP-WUR	vivo

- R4-2505667	On remaining RRM issues for LP-WUR	vivo

- R4-2505668	Reply LS on LS on the wake-up delay for LP-WUS operation in IDLEINACTIVE mode	vivo

- R4-2505669	Work plan for performance part for LP-WUSWUR	vivo

- R4-2505670	Initial consideration on performance for LP-WUR	vivo

- R4-2505673	Draft CR for inactive state LP-WUR serving cell measurement and evaluation requirements	vivo

- R4-2505707	Discussion on RRM core requirements for LP-WUSWUR	OPPO

- R4-2505708	DraftCR on measurements of intra-frequency NR cells for UE with LP-WUR in IDLE and INACTIVE state	OPPO

- R4-2505709	Discussion on RRM performance requirements for LP-WUSWUR	OPPO

- R4-2505791	Discussion on simulation assumptions and results for LP-WUS/WUR	Xiaomi

- R4-2505792	Discussion on RRM core requirements for LP-WUS/WUR	Xiaomi

- R4-2505793	draftCR on 5.X.2.3 MR serving cell measurement and evaluation requirements	Xiaomi

- R4-2505856	Discussion of Simulation Assumptions for LP-WUR RRM Requirements	Apple

- R4-2505857	Summary of Simulation Results for LP-WUR RRM Requirements	Apple

- R4-2505858	On Demodulation Requirements for LP-WUS	Apple

- R4-2505878	Discussion of RRM impact of XR for NR Phase 3	Apple

- R4-2505879	Inter-RAT measurements E-UTRAN TDD measurements	Apple

- R4-2505929	On RRM core requirements for LP-WUR	Apple

- R4-2505930	Reply LS on the RRM measurement metrics for OFDM-based LP-WUR	Apple

- R4-2505931	Reply LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode	Apple

- R4-2506049	draftCR to TS 38.101-1 on operating bands for LP-WUS/WUR (sub-clause 5.2M)	CATT

- R4-2506050	draftCR to TS 38.101-1 on UE channel bandwidth for LP-WUS/WUR (sub-clause 5.3M)	CATT

- R4-2506051	draftCR to TS 38.141-1 on declaration of support for LP-WUS	CATT

- R4-2506052	draftCR to TS 38.141-2 on declaration of support for LP-WUS	CATT

- R4-2506053	Further discussion on REFSENS for LP-WUS	CATT

- R4-2506054	Further discussion on other RF requirements for LP-WUS	CATT

- R4-2506055	Further discussion on Testability for UE RF requirements for LP-WUS	CATT

- R4-2506056	Further discussion on BS RF requirements for LP-WUS	CATT

- R4-2506102	Draft CR to TS 38.133 on definitions, symbols and abbreviations	CATT

- R4-2506103	Draft CR to TS 38.133 on measurements of inter-RAT E-UTRAN cells for UE with LP-WUR	CATT

- R4-2506113	TP to TR 38.774 on transmitted signal quality (clause 7.2.4) for LP-WUS	CATT

- R4-2506114	System parameters for LP-WUR	Nokia

- R4-2506115	Fundamental RX requirements for LP-WUR	Nokia

- R4-2506116	Other RX requirements for LP-WUR	Nokia

- R4-2506117	Testability aspects of LP-WUR	Nokia

- R4-2506118	TP for TR 38.774 on LP-WUS	Nokia

- R4-2506119	Draft CR to add new LP-WUS requirements	Nokia

- R4-2506134	Discussion on LP-WUR Simulation assumptions and results	Nokia

- R4-2506135	LP-SS Simulation results	Nokia

- R4-2506136	Discussion on LP-WUS-WUR RRM core requirements	Nokia

- R4-2506137	DraftCR to TS 38.133  on LP-WUR serving cell measurement and evaluation requirements	Nokia, Vivo

- R4-2506138	Discussion on RRM performance requirements	Nokia

- R4-2506168	Discussions on demodulation performance requirements of LP-WUS/WUR for NR	QUALCOMM Europe Inc. - Spain

- R4-2506169	Discussions on work plan for demodulation performance requirements of LP-WUS/WUR for NR	QUALCOMM Europe Inc. - Spain

- R4-2506303	Discussion on simulation assumptions and results for LP-WUS/WUR	LG Electronics Inc.

- R4-2506304	Discussion on RRM core requirements for LP-WUS/WUR	LG Electronics Inc.

- R4-2506305	Draft CR for Measurements of inter-frequency NR cells for UE with LP-WUR	LG Electronics Inc.

- R4-2506311	Discussion for LP-WUS/WUR REFSENS	LG Electronics France

- R4-2506313	Discussion on BS RF requirements for LP-WUS/WUR	ZTE Corporation, Sanechips

- R4-2506314	TP to TR 38.774 on LP-WUS BS RF general part (Clause 7.2.1)	ZTE Corporation, Sanechips

- R4-2506315	Discussion on REFSENS, ASCS, ACS for LP-WUR	ZTE Corporation, Sanechips

- R4-2506316	Discussion on receiver characteristics for LP-WUR	ZTE Corporation, Sanechips

- R4-2506317	Draft CR to Rel-19 TS38.101-1 on receiver characteristics for LP-WUR	ZTE Corporation, Sanechips

- R4-2506345	Discussion on RRM core requirements for LP-WUS/WUR	China Telecom

- R4-2506346	Discussion on RRM performance requirements for LP-WUS/WUR	China Telecom

- R4-2506368	Draft CR for 38.101-1 to Introduce R19 LPWUS UE intermodulation requirements	CMCC

- R4-2506369	Discussion on LP-WUS UE RF Rx requirements of REFSENS, ASCS and ACS	CMCC

- R4-2506370	Discussion on LP-WUS UE RF Rx requirements of IBB, OBB, intermodulation, spurious emissions and others	CMCC

- R4-2506371	Discussion on LP-WUS BS RF requirements	CMCC

- R4-2506427	Simulation assumption and results for LP-WUR	CMCC

- R4-2506428	Discussion on RRM impact of LP-WUR	CMCC

- R4-2506429	draftCR Introduce inter-frequency and inter-RAT cell re-selection requirements for UE with LP-WUR in RRC_Inactive	CMCC

- R4-2506430	Discussion on test cases for LP-WUS	CMCC

- R4-2506443	Discussion on UE demodulation performance requirements for LPWUS	Nokia

- R4-2506684	Discussion on UE demodulation requirements for NR LP-WUS and receiver	Samsung

- R4-2506789	Simulation assumption and additional simulation results for LP-WUR measurement	Huawei, HiSilicon

- R4-2506790	Discussion on RRM requirements for LP-WUR	Huawei, HiSilicon

- R4-2506791	draftCR on MR measurement requirements for serving cell in IDLE	Huawei, HiSilicon, vivo

- R4-2506792	Discussion on test case for LP-WUR	Huawei, HiSilicon

- R4-2506881	TR 38.774 v0.4.0	vivo

- R4-2506882	Draft Big CR for LP-WUS UE RF requirements in Rel-19	vivo

- R4-2506883	TP to TR 38.774 on LP-WUS UE RF Simulations	vivo

- R4-2506884	Discussions on LP-WUS REFSENS, ASCS, ACS and RMC	vivo

- R4-2506885	draft CR to 38.101-1 on LP-WUS UE RF	vivo

- R4-2506886	Discussions on LP-WUS Testability	vivo

- R4-2506900	Discussion on LP-WUS RRM requirement	Ericsson

- R4-2506901	Simulation on LP-WUS RRM measurement	Ericsson

- R4-2506902	draftCR on LP-WUS introduction	Ericsson

- R4-2506903	Discussion on performance requirements for LP-WUR	Ericsson

- R4-2506956	Discussion on UE demodulation performance requirements for LP-WUS	ZTE Corporation, Sanechips

- R4-2506969	Discussion on UE RF requirements for LP-WUR	Samsung

- R4-2506970	Discussion on testability for LP-WUR	Samsung

- R4-2506971	TP to TR 38.774 on testability of LP-WUR	Samsung

- R4-2507039	Discussion on demodulation performance requirements for R19 LP-WUS	Huawei, HiSilicon

- R4-2507062	Draft CR for TS 38.101-1 to Introduce R19 LPWUS UE intermodulation requirements	CMCC

- R4-2507085	BS RF requirements for low-power wake-up signal for NR	Nokia

- R4-2507086	TP to TR 38.774: Other BS RF requirements for Low-Power Wake-up Signal and Receiver	Nokia

- R4-2507087	Draft CR to TS 38.104 on introduction of LP-WUS into unwanted emissions and transmitter intermodulaton clauses	Nokia

- R4-2507109	UE demodulation requirements for LP-WUS/WUR	Ericsson

- R4-2507115	Discussions on demodulation requirements for R19 LP-WUS/WUR	MediaTek inc.

- R4-2507130	views on requirements of the low-power wake-up receiver	Sony

- R4-2507192	Discussion on RRM core requirements on LPWUS	ZTECorporation,Sanechips

- R4-2507193	Discussion on RRM performance requirements on LPWUS	ZTECorporation,Sanechips

- R4-2507194	Discussion on OOK-based LP-WUR simulation	ZTECorporation,Sanechips

- R4-2507197	Draft LS to RAN1 on LP-SSS-RSRP/RSSI measurement	ZTECorporation,Sanechips

- R4-2507198	Draft LS to RAN1 on wake-up delay for LP-WUS operation in IDLE/INACTIVE	ZTECorporation,Sanechips

- R4-2507203	(NR_LPWUS-Core) LPWUR and MR measurement capapbility in IDLE state	ZTECorporation,Sanechips

- R4-2507204	(NR_LPWUS-Core) LPWUR and MR measurement capapbility in INACTIVE state	ZTECorporation,Sanechips

- R4-2507489	TP to TR38.774: BS RF requirement overview for LP-WUS	Ericsson

- R4-2507490	On general issues for WUR	Ericsson

- R4-2507491	On system parameter for WUR	Ericsson

- R4-2507492	On WUR RF requirement testability	Ericsson

- R4-2507493	WUR RF requirement other than REFSENS	Ericsson

- R4-2507494	WUR RF requirement REFSESN ACS ASCS	Ericsson

- R4-2507575	Topic summary for [115][136] NR_LPWUS_UERF	Moderator( Vivo)

- R4-2507812	Simulation assumptions for R19 LP-WUS	MediaTek inc.

- R4-2507813	Discussion on the RRM core requirements for LP-WUS	MediaTek inc.

- R4-2507840	Topic summary for [115][308] NR_LPWUS	Moderator (Huawei)

- R4-2507864	Topic summary for [115][332] NR_LPWUS_demod	Moderator (Qualcomm)

- R4-2507875	RRM core requirements for LP-WUR	Qualcomm Incorporated

- R4-2507876	RRM performance requirements for LP-WUR	Qualcomm Incorporated

- R4-2508089	TP for TR 38.774 on LP-WUS	Nokia

- R4-2508090	TP to TR 38.774 on testability of LP-WUR	Samsung

- R4-2508091	draftCR to TS 38.101-1 on operating bands for LP-WUS/WUR (sub-clause 5.2M)	CATT

- R4-2508092	draftCR to TS 38.101-1 on UE channel bandwidth for LP-WUS/WUR (sub-clause 5.3M)	CATT

- R4-2508093	dCR to 38.101-1, LPWUS ACS	Qualcomm, Nokia, Ericsson

- R4-2508094	dCR to 38.101-1, LPWUS ASCS	Qualcomm

- R4-2508095	Draft CR to add new LP-WUS requirements	Nokia

- R4-2508096	TP for 38.774: Blocking characteristics for LP-WUR	Huawei, HiSilicon

- R4-2508097	draft CR to 38.101-1 on LP-WUS UE RF	vivo

- R4-2508098	Draft CR to Rel-19 TS38.101-1 on receiver characteristics for LP-WUR	ZTE Corporation, Sanechips

- R4-2508099	draft CR to TS 38.101-1 on Blocking characteristics for LP-WUR	Huawei, HiSilicon

- R4-2508117	TP to TR 38.774 on LP-WUS UE RF Simulations	vivo

- R4-2508279	Ad-hoc minutes for NR_LPWUS	Vivo

- R4-2508280	WF on RRM requirements for NR_LPWUS	Vivo

- R4-2508334	Reply LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode	vivo, MediaTek

- R4-2508335	Reply LS on the RRM measurement metrics for OFDM-based LP-WUR	OPPO

- R4-2508336	Draft CR for inactive state LP-WUR serving cell measurement and evaluation requirements	vivo

- R4-2508337	DraftCR on measurements of intra-frequency NR cells for UE with LP-WUR in IDLE and INACTIVE state	OPPO

- R4-2508338	draftCR on 5.X.2.3 MR serving cell measurement and evaluation requirements	Xiaomi

- R4-2508339	Draft CR to TS 38.133 on definitions, symbols and abbreviations	CATT

- R4-2508340	Draft CR to TS 38.133 on measurements of inter-RAT E-UTRAN cells for UE with LP-WUR	CATT

- R4-2508341	DraftCR to TS 38.133  on LP-WUR serving cell measurement and evaluation requirements	Nokia, Vivo

- R4-2508342	Draft CR for Measurements of inter-frequency NR cells for UE with LP-WUR	LG Electronics Inc.

- R4-2508343	draftCR Introduce inter-frequency and inter-RAT cell re-selection requirements for UE with LP-WUR in RRC_Inactive	CMCC

- R4-2508344	draftCR on MR measurement requirements for serving cell in IDLE	Huawei, HiSilicon, vivo

- R4-2508345	draftCR on LP-WUS introduction	Ericsson

- R4-2508346	(NR_LPWUS-Core) LPWUR and MR measurement capapbility in IDLE state	ZTECorporation,Sanechips

- R4-2508347	(NR_LPWUS-Core) LPWUR and MR measurement capapbility in INACTIVE state	ZTECorporation,Sanechips

- R4-2508420	Inter-RAT measurements E-UTRAN TDD measurements	Apple

- R4-2508442	Reply LS on the RRM measurement metrics for OFDM-based LP-WUR	OPPO

- R4-2508626	Way Forward for [115][332] NR_LPWUS_demod	Qualcomm

- R4-2508666	draftCR to TS 38.141-1 on declaration of support for LP-WUS	CATT

- R4-2508667	draftCR to TS 38.141-2 on declaration of support for LP-WUS	CATT

- R4-2508668	Way Forward for [115][308] NR_LPWUS	Huawei

- R4-2508739	TP to TR 38.774 on LP-WUS BS RF general part (Clause 7.2.1)	ZTE Corporation, Sanechips, Ericsson, Huawei, Nokia

- R4-2508769	TP to TR 38.774: Other BS RF requirements for Low-Power Wake-up Signal and Receiver	Nokia

- R4-2508770	TP to TR38.774: BS RF requirement overview for LP-WUS	Ericsson, Nokia

- R4-2508772	Draft CR to TS 38.104 on introduction of LP-WUS into unwanted emissions and transmitter intermodulaton clauses	Nokia

- R4-2508777	draft CR to TS 38.104 on introduction of general part and power boosting definition for LP-WUS	Huawei, HiSilicon

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template