3GPP TSG RAN Meeting #108									RP-251201

Prague, Czech Republic, June 9-13, 2025

Status Report to TSG

Agenda item:		9.3.4.8

| WI / SI Name | WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | TRP_TRS_MIMO_OTA_Ph3 |  |  |  |  |
| Unique ID | 1030085 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP‑250197 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Sep/2025 | Performance part: Mar/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 90% | Performance Part: 50% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Wang, Ruixin |
|  | Company | vivo |
|  | Email | ruixin.wang@vivo.com |
| Rapporteur | Name | Zhu, Siting |
|  | Company | CAICT |
|  | Email | zhusiting@caict.ac.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.4	RAN4

2.4.1	Agreements

RAN4 #114bis

General

- Topic summary for WI in [33].

- WF for WI was approved in [39].

- Ad-hoc meeting minutes for WI was captured in [37].

- TR 38.762 v0.3.0 was approved in [20].

Detailed Agreements for WI:

    - Sub-topic 1-2 Test method and Performance metric for XR devices

Issue 1-2-1: General XR TRP test procedure

Agreements:

- For XR glasses TRP testing, use a grid size of 15 degrees as baseline.

Issue 1-2-2: General XR TRS test procedure

Agreements:

- For XR glasses TRS testing, use a grid size of 30 degrees as baseline.

    - Sub-topic 1-3 Alternative test solutions

Issue 1-3-3: Alternative split measurement grids method for XR to resolve battery issue

Agreements:

- The measurement data for smartphone will be provided next meeting.

    - Sub-topic 2-1 UE type, usage scenarios and performance metric for NTN (NR-NTN and IoT-NTN)

Issue 2-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2

Agreements:

o   For IoT-NTN and NR-NTN, typical usage scenarios are:

§  Scenario 1: both head+hand mode and hand only mode.

§  Scenario 2: hand only mode targeted for specific UE orientation usage

§  Scenario 3: hand only mode targeted for arbitrary UE orientation usage

o   UE shall declare support for only one scenario per band.  The declaration details of each scenario should be further discussed.

o   For IoT-NTN and NR-NTN handheld UEs declared Scenario 2, the performance metrics are partial sphere metric. FFS details

o   For IoT-NTN and NR-NTN handheld UEs declared Scenario 3, the performance metrics are full sphere TRP and TRS

    - Sub-topic 3-1 Dynamic channel model generation and validation

Issue 3-1-1: Dynamic channel model re-naming in 3GPP

Agreements:

- Update the dynamic channel model name as UMidyn and UMadyn in TR 38.762.

Issue 3-1-3: Dynamic channel model validation method

Agreements:

- Reuse the principle of Rel-17 FR1 static channel model validation pass/fail limits in dynamic channel model validation for PDP, TCF, SCF, and XPO.

Issue 3-1-4: Dynamic channel model validation alignment among companies

Agreements:

- One more meeting to further confirm UMi channel model reference targets.

    - Sub-topic 4-1 Lab alignment activity

Issue 4-1-2: latest status of Lab Alignment Devices (LADs) for 2Tx lab alignment activity

Agreements:

- Removing n78, and move forward on 2Tx lab alignment activity with 1 LAD.

- Additional one device for n41 is expected.

Issue 4-1-3: Working state confirmation of 2Tx non-coherent UE

Agreements:

- DUT providers/test labs pay attention to troubleshooting related issues in subsequent activities to ensure that the tested devices operates in the expected working state.

    - Sub-topic 4-3 Static MIMO OTA requirements

Issue 4-3-1: bands for Rel-19 static MIMO OTA requirements

Agreements:

- n3, n7, n77 and n8 for Rel-19 static MIMO OTA requirements.

Issue 4-3-2: Update spec scope of TS 38.151

Agreements:

- No need.

Working procedure for Rel-19 AC 2Tx Lab Alignment Campaign

- The purpose of 2Tx (non-coherent UL-MIMO) Lab Alignment Campaign is to ensure there is no unexpected lab deviation and establish full trust and confidence on the results of UE with 2Tx configuration.

- Test labs are invited to participate to the lab alignment and test campaign, the following conditions should be fulfilled:

  - Participating lab should be accredited under ISO 17025 (ISO 17025 accredited labs) and have any of 3GPP TS 37.544, CCSA YD/T 1484.6, and CTIA OTA Test Plan listed on its accreditation scope.

  - Participating lab should have anechoic chamber(s) ready to support 1Tx and 2Tx TRP TRS testing based on 3GPP TR 38.870.

  - Participating lab should have sufficient test resource to provide the on-time measurement results without delay.

- Test methodology:

  - Test plan: 3GPP TR 38.870 v18.2.0;

- Test cases for Lab Alignment Campaign:

  - Test bands: n41. For new labs only join 2Tx lab alignment, the additional lab alignment work of 1Tx is not needed

  - Number of test cases per band: TRP and TRS; BHHL and BHHR at low/mid/high channel;

  - Use scenario: Head and Hand phantom (Talk mode)

  - Hand Phantom: Wide Grip hand

  - Operation mode: NR Standalone (SA)

  - Number of Tx chain: UE with 2Tx (non-coherent UL MIMO)

- Lab Alignment Device (LAD): in total 2 devices

  - 1 New devices (LAD 5) for band n41 non-coherent UL-MIMO;

  - UE information will be further added:

    - Smartphone DUT size: width >72mm and ≤92mm (Size 1)

    - DUT capability: support for all the prioritized bands listed in the WID is preferred, but devices supporting only a subset of the above bands can equally be used in the lab alignment campaign (also pending further prioritization of the bands in the WID)

  - Power Class: PC2 (n41)

  - Only focusing on UEs supporting non-coherent UL-MIMO in Rel-19.

- Test results submitting:

  - Using the same worksheet template in R4-2418214 to submit the measurement results

  - The measurement results of LADs should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - Results shall not be shared between labs before submitting to RAN4 meetings or sharing in the RAN4 reflector. Comparison and lab alignment analysis should only be done in RAN4 meetings/discussions

  - Sharing of the measurements results of a specific device from test labs is permitted only towards the 3GPP member that provided such device (i.e., device’s owner) before submitting to RAN4

- Lab alignment criteria:

  - The pass/fail criteria are defined as the maximum deviation between the measurement result and the reference value

  - Confirm the reference value derived based on the per-band per-PC averaging approach (linear average with dBm) of lab alignment data pool from ≥3 labs submitted before end of [RAN4#116] as baseline.

  - Apparent outliers will not be considered in averaging process. The value deviates over 1.5*MU from all the other lab’s results should be identified as apparent outlier.

  - Pass/fail limit for lab alignment should be defined as 0.75*MU as baseline. MU value is the expanded MU for 2Tx condition (need confirmation whether same as 1Tx).

  - The summation form for TRP and TRS lab alignment should keep consistent during the calculation process of TRP TRS lab alignment from each company i.e. sin weights approach. Only traditional approach (15-degrees TRP and 30-degrees TRS) should be used during lab alignment activity to reduce unnecessary uncertainty.

  - The timeline of 2Tx AC alignment activity is in [R4-2416590]. How to treat late submission results and confirm the alignment: TBD

- Test lab procedures:

  - LAD delivery scheme

    - Decide LAD delivery scheme after all the test lab and LAD information being confirmed (after the confirmation of volunteered labs and LADs).

  - LAD measurement time in each test lab: finalize LAD measurement within [5] workdays, and deliver to next lab ASAP with LAD delivery In/Out information shared in reflector.

  - Encourage test labs to share resulting combined MU based on their own systems.

  - DUT providers and test labs could work together to ensure that the tested devices operates in the expected working state.

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: [4 bands for 1Tx from n3, n5, n7, n8, and n20; 2Tx at bands n41;]

  - Use scenarios:

    - For 1Tx, consider both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR;

    - For 2Tx, only consider Browsing mode (Hand phantom only) requirements in Rel-19;

  - Hand Phantom: Wide Grip Hand as first priority, PDA hand is not precluded if sufficient devices are available

  - Operation mode: SA with 1Tx  as phase 1. SA with 2Tx (non-coherent UL-MIMO) as phase 2 after 2Tx lab alignment activity.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: Size 1(width >72mm and ≤92mm)

  - DUT capability: support for Bands those listed in the latest WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - The following selection criteria can also be considered:

- Year of production: [2022-2025]

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3. For bands support both PC2 and PC3, focus on PC2 measurements, but both PC2 and PC3 requirements are needed. For PC3 bands, only PC3 requirements.

- Devices provisioning:

  - Any 3GPP member can work with test labs to provide devices to the aligned test labs (each test lab shall provide a single measurement result set for repeated UE model into RAN4 measurement campaign data pool)

    - FFS for the same UE model with supporting different set of bands

  - Logistical aspects for devices provisioning to the labs are TBD

- Test results submitting:

  - UE information disclosure: the following rules should be considered

    - UE model information based on template [Rel-18 R4-2408104 as starting point, update is needed] can be shared to MCC. RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure.

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

    - laboratories use the same template to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

      - Information of the devices that are going to be measured shall be shared with the RAN4 Secretary as soon as available (i.e., before the measurement activity on such devices starts)

      - The RAN4 Secretary updates the summary of statistical information  and publishes it to 3GPP RAN4 (i.e., living document) in order to monitor the achievement of the thresholds in a timely manner and take the proper actions if these are not met

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

    - Statistics of UE information can be shared to RAN4 reflector after TBD meeting by RAN4 secretary after post-processing

    - Consider following aspects as thresholds related to the devices pool for Rel-19

      - Minimum number of devices for each band, each device size, each power class: TBD

      - Minimum number of device models: TBD

      - Minimum number of devices' vendors: TBD

      - Percentage of devices from second-half 2022 to 2025: TBD

      - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: TBD

  - Laboratories use the same worksheet template in [Rel-18 R4-2315844 as starting point, update is needed] to submit the measurement results for Rel-19 3GPP TRP TRS performance data pool.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - The allowed maximum number of submitted devices from each lab is [15] (depends on how many test labs will join the activity) with a minimum of 3 devices.

  - Only the results from aligned labs will be considered for specifying requirements

  - The progress in each lab are encouraged to share on the RAN4 reflector (for example - how many devices have been measured and on which bands)

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

- Specify TRP TRS requirements:

  - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): [30]

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-19.

  - Method of limits derivation: per-band Data driven approach, working procedure in Rel-18 on how to specify the requirement is baseline.

  - The value at [80%] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - For a band supporting both PC2 and PC3, specify PC3 requirement based on finalized PC2 requirements, for TDD bands 3dB offset, for FDD bands TBD.

  - How to specify PDA hand related TRP/TRS requirements (Size2) in legacy band of Rel-18: FFS.

- Test lab procedures for 1Tx

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - For UE support PC2 at one band, PC3 testing is not needed.

  - For new labs of 1Tx requirements work, those not declared aligned in Rel-17 and Rel-18 also need to complete 1Tx lab alignment on at least n78 (high band) and n28 (low band).

  - Only legacy grid, i.e., 15degree TRP and 30 degree TRS, is used for this measurements campaign

- Test lab procedures for 2Tx

  - FFS

RAN4 #115

- Topic summary for WI in [44].

- WF for WI was approved in [53].

- Ad-hoc meeting minutes for WI was captured in [49].

- TR 38.762 v0.4.0 was approved in [29].

Detailed Agreements for WI:

    - Sub-topic 1-1 General aspect of AC (measurement distance)

Issue 1-1-4: MU for Minimum measurement distance for 50cm QZ

Agreements:

- RAN4 should further study the MU impact, if the measurement distance is smaller than the min measurement distance of 50cm QZ. FFS the maximum allowed MU for this issue.

    - Sub-topic 1-2 Test method and Performance metric for XR devices

Issue 1-2-2: MU aspects other than phantom and positioning

Agreements:

- No other MU update is needed for XR OTA.

Issue 1-2-3: New performance metric for XR devices

Agreements:

RAN4 further study new metric and corresponding test procedure/data processing for XR devices, e.g.,.

⦁	Weighted TRP & TRS

⦁	Near horizontal part radiated power (NHPRP) and Near horizontal part Isotropic Sensitivity (NHPIS)

⦁	Others

If no consensus reached, the WI can also be closed with agreed TRP/TRS metric for XR devices.

    - Sub-topic 2-1 UE type, usage scenarios and performance metric for NTN (NR-NTN and IoT-NTN)

Issue 2-1-1: Details on NTN usage scenario declaration

Agreements:

- RAN4 to adopt the following rules for scenario declaration in NTN OTA testing:

  - DUT shall declare to support only one scenario

  - The declaration of scenario applicability shall be consistent with product manual usage scenario

  - The declaration of scenario shall be clearly stated in the test report

Issue 2-1-2: alignment of understanding on UE positioning guideline for Scenario 2

Agreements:

- For Scenario 2 UEs:

  - The DUT shall be mounted in a suitable hand phantom

  - The elevation tilt of the default position of the DUT can be:

    - 0 degree (similar to Figure 6.1-1 of TR 38.870, but with hand phantom) or as shown below

    - [±15, ±30, ±45] degrees; Other tilt angles are not precluded

    - NOTE 1: the granularity of the elevation tilt to be further discussed next meeting

  - The elevation tilt of the hand phantom & DUT is determined based on OEM declaration

Issue 2-1-4: Whether the partial sphere is using the =0° (top pole) as a center

Agreements:

the partial sphere metric is using the  =0° (top pole) as a center

Issue 2-1-5: What is the angle range of the partial sphere-after concluding center of the sphere

Agreements:

- RAN4 further discuss the theta angle range of partial sphere metric, from ±30°, ±60°, ±90°.

  - Option 1:  single range is selected and captured in TR

  - Option 2:  multi-range in TR but based on UE declaration, details are FFS

- Note1: the phi range should be 360 degrees

- Note2: if multiple ranges are captured in TR, the metric applicability rule should also be defined. The target is selecting one for NTN scenario 2.

Issue 2-1-6: Whether Using integration of EIRP/EIS or CDF?

Agreements:

- Integration is baseline.

- If different range is declared in issue 2-1-5, how to ensure the metric comparable should be discussed and resolved.

    - Sub-topic 2-2 NTN OTA test methodologies

Issue 2-2-2: Whether Dwell time is considered during NTN OTA testing

Agreements:

- RAN4 further discuss and align understanding of NTN antenna tuning, if any.

- RAN4 further discuss whether Dwell time for NTN OTA test case is needed or not. a decision in RAN4 is needed.

Issue 2-2-3: NTN MU

Agreements:

- For NTN OTA, there is no MU update identified. Partial sphere metric related MU may be further discussed, if needed.

    - Sub-topic 3-1 Dynamic channel model generation and validation

Issue 3-1-1: TP to TR on DynMIMO CM Validation

Agreements:

The TP R4-2505845 can be agreed.

Issue 3-1-2: dynamic channel model pass/fail limits

Agreements:

  - Reuse the principle of Rel-17 FR1 static channel model validation pass/fail limits in dynamic channel model validation for PDP, TCF, SCF, i.e.,

- For PDP: Define the detailed pass/fail limits for each cluster of CDL-C UMadyn and CDL-C UMidyn as below.

|  | Power Tolerance | Delay Tolerance |
| --- | --- | --- |
| Paths from 0dB to 10dB | ±1dB | ±6ns |
| Paths from 10dB to 20dB | ±2.5dB | ±6ns |
| Paths from 20dB to 30dB | ±5dB | ±6ns |
| Paths from 30dB to 40dB | ±10dB | ±6ns |

- For TCF: The pass/fail limits for theoretical temporal correlation above 0.3 are formed as bands of ±0.1 of correlation capped at 1 at the high end. Additionally, when the theoretical temporal correlation drops below 0.3, the limits are formed at bands of ±0.3 of correlation capped at 0 at the low end.

- For SCF: The pass/fail limits for spatial correlation are formed as bands of ±10% of correlation capped at 100% for the upper limit for target correlation of 35% (for CDL-C UMa @3600MHz, this value is 65%) and above. For target correlations below 35% (for CDL-C UMa @3600MHz, this value is 65%), the band is widened to ±20% capped at 0%.

- For XPO, pass/fail limit as ±2dB.

    - Sub-topic 3-2 Test method and Performance metric for dynamic MIMO OTA

Issue 3-2-1: TP to 38.762 on General parts

Agreements:

- The TP R4-2506285 is agreeable.

Issue 3-2-2: device rotations per orientation

Agreements:

- Include the 4 device rotations per orientation in the definitions of the TMTs.

Issue 3-2-4: Performance metric for UE dynamic MIMO OTA

WF:

- RAN4 further discuss and performance metric for dynamic MIMO OTA.

    - Sub-topic 4-1 2Tx Lab alignment activity

Issue 4-1-1: lab alignment outcome for 2Tx lab alignment activity

Agreements:

- Based on the analysis in R4-2508761. The three labs in contribution R4-2506887, R4-2506932, and R4-2508912 are well aligned for 2Tx lab alignment.

Issue 4-1-2: CR for UL MIMO

Agreements:

The CR R4-2505441 is agreeable.

Issue 4-1-3: update the lab alignment template

Agreements:

- Update the lab alignment template to incoporate the reference to test parameters in TR 38.870 clause 4.3.

    - Sub-topic 4-2 1Tx FR1 TRP TRS requirements

Issue 4-2-1: offset between Size 1 and Size 2 device TRP/TRS requirements

Agreements:

- For already defined requirements:

    - - n1: 0.5 dB tighten

    - - n28, n41, n78: 0 dB

- For non-defined requirements, Single set of requirements for device width ≥56mm and ≤92mm.

  - Note: both device size results can be used for data pool to generate CDF.

Issue 4-2-2: Working procedure for 1Tx requirements related performance measurement campaign

Agreements:

  - Approve the following update based on agreed framework in R4-2504763.

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: n3, n5, n7, and n8 [2Tx at bands n41;]

  - Use scenarios:

    - For 1Tx, consider both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR;

    - For 2Tx, only consider Browsing mode (Hand phantom only) requirements in Rel-19;

  - Hand Phantom: Wide Grip Hand and PDA hand

  - Operation mode: SA with 1Tx  as phase 1. SA with 2Tx (non-coherent UL-MIMO) as phase 2 after 2Tx lab alignment activity.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: UE (width >56mm and ≤92mm)

  - DUT capability: support for Bands those listed in the latest WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - The following selection criteria can also be considered:

- Year of production: [2023-2025]

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3. For bands support both PC2 and PC3, focus on PC2 measurements, but both PC2 and PC3 requirements are needed. For PC3 bands, only PC3 requirements.

- Devices provisioning:

  - Any 3GPP member can work with test labs to provide devices to the aligned test labs (each test lab shall provide a single measurement result set for repeated UE model into RAN4 measurement campaign data pool)

    - FFS for the same UE model with supporting different set of bands

  - Logistical aspects for devices provisioning to the labs are TBD

    - Sub-topic 4-3 Static MIMO OTA requirements

Issue 4-3-1: Update framework for MIMO OTA

Agreements:

- R4-2506286 can be agreed.

Issue 4-3-2: Template for MIMO OTA measurement campaign

Agreements:

- R4-2506287 can be agreed.

Issue 4-3-3: Check the channel model validation results

Agreements:

- The channel model validation results in R4-2505434, R4-2505752, and R4-2508761 are reviewed. No issue identified.

2.4.2	Remaining Open issues

- Core part

    - Finalization of XR OTA

    - Finalization of NTN OTA

    - Finalization of FR1 dynamic MIMO OTA

    - Finalization of TR drafting for FR1 dynamic MIMO OTA

- Performance part

    - Finalization of 2Tx lab alignment activity for TRP TRS

    - Measurement campaign for 1Tx TRP TRS and static MIMO OTA

    - Final Requirements and recommended TT for 1Tx TRP TRS and static MIMO OTA

2.5	RAN5

2.5.1	Agreements

RAN5 #107

- No discussion this meeting

2.4.2	Remaining Open issues

- Core part

  - MU assessment

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

4.	References

RAN4 #114bis

- R4-2503136	Further discussion on XR test method	Ericsson-LG Co., LTD

- R4-2503137	Discussion on Rel-19 TRP TRS requirements	Ericsson-LG Co., LTD

- R4-2503332	Discussion on OTA test for FR1 NTN devices	Xiaomi

- R4-2503495	on MIMO OTA tests	Huawei, HiSilicon

- R4-2503496	on NTN OTA tests	Huawei, HiSilicon

- R4-2503497	on TRP TRS OTA tests	Huawei, HiSilicon

- R4-2503498	on XR OTA tests	Huawei, HiSilicon

- R4-2503551	On NTN OTA test methodology	Apple

- R4-2503552	draftCR to TR38.870 on NTN DUT usage scenario applicability	Apple

- R4-2503650	On OTA performance metric for handheld NTN UE	Samsung

- R4-2503651	Draft CR to TR 38.870 on NTN usage scenario and performance metric	Samsung

- R4-2503652	On performance metric for FR1 dynamic MIMO	Samsung

- R4-2503653	On TRP TRS spec for narrow width UE	Samsung

- R4-2503809	On FR1 MIMO OTA Dynamic Channel Model	Apple

- R4-2503810	CR to TS38.161 for an alternate TRS and TRP test procedure for XR devices	Apple

- R4-2503811	Views on Rel-19 TRP TRS Performance Requirements	Apple

- R4-2503828	draft CR to 38.870 on NTN OTA performance metric and test method	vivo

- R4-2503829	Updated working procedure for Rel-19 TRP TRS performance part	vivo

- R4-2503990	On Rel-19 MIMO OTA	CAICT

- R4-2503991	TR 38.762 V0.3.0 on MIMO OTA dynamic test methodology for FR1 UEs	CAICT

- R4-2504058	Discussion on FR1 NTN devices OTA	Qualcomm Incorporated

- R4-2504081	On dynamic MIMO OTA channel models reference target values	CMCC

- R4-2504150	Discussion on TRP TRS requirements	CAICT

- R4-2504182	Draft CR for split measurement grids method for TRP/TRS measurements	Nokia, Keysight Technologies UK Ltd

- R4-2504425	Draft CR for split measurement grids method for TRP/TRS measurements	Nokia, Keysight Technologies UK Ltd

- R4-2504440	Draft CR for split measurement grids method for TRP/TRS measurements	Nokia, Keysight Technologies UK Ltd

- R4-2504566	TP for TR 38.762: Clarifications of Channel Models and Applicability	Keysight Technologies UK Ltd, Spirent Communications, ETS-Lindgren, CAICT

- R4-2504568	DraftCR to add min measurement distance for 50cm QZ	Keysight Technologies UK Ltd

- R4-2504577	Proposals for Rel-19 TRP TRS requirements topic	Telecom Italia S.p.A., Vodafone

- R4-2504578	On NTN OTA test methodology	Apple

- R4-2504579	draftCR to TR38.870 on NTN DUT usage scenario applicability	Apple

- R4-2504621	Discussion on OTA Test for XR Devices	Meta

- R4-2504653	Topic summary for [114bis][328] TRP_TRS_MIMO_OTA_Ph3	Moderator (vivo)

- R4-2504728	draft CR to 38.870 on NTN OTA performance metric and test method	vivo

- R4-2504729	TP for TR 38.762: Clarifications of Channel Models and Applicability	Keysight Technologies UK Ltd, Spirent Communications, ETS-Lindgren, CAICT

- R4-2504730	Way Forward for [114bis][328] TRP_TRS_MIMO_OTA_Ph3	Vivo

- R4-2504731	Ad-hoc meeting minutes for [114bis][328] TRP_TRS_MIMO_OTA_Ph3	Vivo

- R4-2504758	DraftCR to add min measurement distance for 50cm QZ	Keysight Technologies UK Ltd

- R4-2504763	Way Forward for [114bis][328] TRP_TRS_MIMO_OTA_Ph3	Vivo

RAN4 #115

- R4-2505396	On MIMO dynamic OTA tests	Huawei, HiSilicon

- R4-2505397	on NTN OTA tests	Huawei, HiSilicon

- R4-2505398	on TRP TRS OTA tests	Huawei, HiSilicon

- R4-2505399	on XR OTA tests	Huawei, HiSilicon

- R4-2505425	CR to TS38.161 for an alternate TRS and TRP test procedure for XR devices	Apple

- R4-2505426	On NTN TRP TRS test methodology	Apple, GIST, NRRA

- R4-2505434	On FR1 MIMO OTA n3 Channel Model Validation	Apple

- R4-2505435	On performance metric for UE dynamic MIMO OTA	Apple

- R4-2505440	Views on TRP TRS Performance Part	Apple

- R4-2505441	CR to TR 38.870 on clarification of UE configuration for UL-MIMO test	Apple

- R4-2505472	Discussion on Rel-19 TRP TRS requirements	Ericsson Korea Partners Co Ltd

- R4-2505743	On FR1 NTN devices OTA	Qualcomm Incorporated

- R4-2505750	Discussion on test methodology and radiated performance metric for FR1 NTN devices	Xiaomi

- R4-2505751	Discussion on TRP and TRS requirements for Size 2 Ues	Xiaomi

- R4-2505752	Channel model validation results for Band 3, n7, n8 and n77	Xiaomi

- R4-2505810	Proposals for Rel-19 TRP TRS requirements working procedure	Telecom Italia S.p.A., Vodafone, Deutsche Telekom, Orange

- R4-2505842	CR to add min measurement distance for 50cm QZ	Keysight Technologies UK Ltd, Huawei, Ericsson, Rohde&Schwarz

- R4-2505843	On Inadequate Range Length MU	Keysight Technologies UK Ltd

- R4-2505844	On TMT and CTMT metric definitions	Keysight Technologies UK Ltd

- R4-2505845	TP to TR 38.762: DynMIMO CM Validation Details, Clarifications, and Corrections	Keysight Technologies UK Ltd, CMCC, CAICT

- R4-2505846	TP to TR 38.762: on performance metrics	Keysight Technologies UK Ltd

- R4-2505849	On Partial Radiated Surfaces	Keysight Technologies UK Ltd

- R4-2505850	On Split TRP/TRS Grids for Devices with Limited Battery Capacity	Keysight Technologies UK Ltd, Nokia

- R4-2506284	On Rel-19 dynamic MIMO OTA	CAICT

- R4-2506285	TP to 38.762 on General parts	CAICT, SAICT

- R4-2506286	Framework and time plan for Rel-19 MIMO OTA performance requirements development (May 2025)	CAICT

- R4-2506287	Template for Rel-19 FR1 MIMO OTA Measurement Campaign	CAICT

- R4-2506288	Channel model validation results for band n3	CAICT

- R4-2506289	TR 38.762 V0.4.0 on MIMO OTA dynamic test methodology for FR1 UEs	CAICT

- R4-2506876	CR to TR 38.870 on testing bands	vivo

- R4-2506877	Remaining issues for XR OTA test method	vivo

- R4-2506878	Remaining issues for NTN OTA test method	vivo

- R4-2506879	Discussions on Rel-19 OTA performance part	vivo

- R4-2506880	draft CR to 38.870 on NTN OTA performance metric and test method	vivo

- R4-2506887	Measurement results for Rel-19 TRP TRS 2Tx AC lab alignment activity	vivo

- R4-2506932	Measurement results for Rel-19 TRP TRS 2Tx AC lab alignment activity - CAICT	CAICT.

- R4-2506972	Discussion on partial sphere metric	Samsung

- R4-2506973	On performance metric for FR1 dynamic MIMO OTA	Samsung

- R4-2507005	Discussion on OTA requirements for XR Headworn Devices	Vodafone, Telecom Italia S.p.A., Deutsche Telekom

- R4-2507094	Draft CR for split measurement grids method for TRP/TRS measurements	Nokia, Keysight Technologies UK Ltd

- R4-2507095	Discussion on Near-Field to Far-Field Transform Method for OTA TRP Measurement	GIST, RRA, CSU

- R4-2507097	Draft CR for 38.870 on Abbreviations Section	GIST, RRA, CSU

- R4-2507366	CR for split TRP/TRS measurement grids method	Nokia, Keysight Technologies UK Ltd

- R4-2507869	Topic summary for [115][337] TRP_TRS_MIMO_OTA_Ph3	Moderator (vivo)

- R4-2508607	Channel model validation results for Band 3, n7, n8 and n77	Xiaomi

- R4-2508659	CR to TS38.161 for an alternate TRS and TRP test procedure for XR devices	Apple

- R4-2508660	CR for split TRP/TRS measurement grids method	Nokia, Keysight Technologies UK Ltd

- R4-2508661	Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3	vivo

- R4-2508662	Ad-hoc meeting minutes for [115][337] TRP_TRS_MIMO_OTA_Ph3	vivo

- R4-2508760	Channel model validation results for band n3	CAICT

- R4-2508779	draft CR to 38.870 on NTN OTA performance metric and test method	vivo

- R4-2508780	Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3	vivo

- R4-2508782	Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3	vivo

- R4-2508911	CR for 38.870 on Abbreviations Section	GIST, RRA

- R4-2508912	Measurement results for Rel-19 TRP TRS 2Tx AC lab alignment activity	SGS Wireless

RAN5 #107

N/A

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template