3GPP TSG RAN Meeting #108										RP-251255

Prague, Czech Republic, June 9-13, 2025

Source:	OPPO

Title:	Consideration on Lower Layer Security and Privacy in 6G RAN

Agenda Item:	16.1

Document for:	Discussion

- Introduction

It is proposed to discuss and consider to study potential security/privacy protection schemes for PHY layer and MAC layer in 6G RAN, in alignment with SA3 threats analysis and security requirements.

- Discussions

In 6G, with threats analysis from 3GPP SA3 and others sources, lower layer security improvement is a potential area of consideration in RAN related WGs and SA3. Especially with enhanced AI capabilities for attackers in 6G, some of the parameters that were once considered not privacy-sensitive may become exposed to privacy risks. This paper provides some security considerations for PHY layer and MAC layer, also for the combination of the above two.

  - PHY Layer Security Consideration

PHY layer signaling and PHY layer channel information leakages may lead to user privacy risks or UE access failure. For example:

- Sensing signal

Analysis: In 6G, sensing is one of the important use cases. By the nature of sensing, sensing data are usually extracted or processed base on the sensing signals, e.g., sensing reference signal, at the PHY layer. An attacker can extract sensing data from periodically transmitted sensing signals through some PHY layer methods. As described in 4.3 and 5.2.4 of TS 22.137 [1] sensing data security protection is needed, and privacy consideration is necessary for human related sensing scenarios. Scenarios with potential privacy risk concerns mainly include human-related scenarios such as Use Case on Environment Object Reconstruction, human health monitoring, and etc. [2]. In the human health monitoring scenario, by eavesdropping micro doppler effect by ISAC caused by chest rise/fall during sleeping, an attacker may be able to predict the health status of the target person.

- MIB/SIB

Analysis: The attacker can first listen to the broadcast information blocks, e.g., the MIB carried by the PBCH or the SIB transmitted in the PDSCH, to achieve the time-frequency synchronization with the gNB. Specifically, the attacker first determines the SSB burst based on the envelope detection and short-time Fourier Transform, and use the signaling of the 	MIB to obtain the Control Resource SET (CORESET) quickly. The attacker then analyzes the 	DCI in CORESET to locate the time-frequency position of the PDSCH carrying SIB, and further extracts the code stream and signaling of SIB through demodulating and decoding.

Based on the time-frequency synchronization, the attacker then decodes additional unprotected information from the other physical channels, such as PUCCH and PDCCH. Base on this information, the attacker can also launch some active attacks. For example, the attacker can prevent the UE from accessing to a cell by spoofing or tampering the MIB/SIB messages.

Observation on potential PHY Spec impacts: Taking security into consideration at the beginning of 6G reference signal or channel design can effectively prevent potential PHY layer security and privacy threats.

  - MAC CE Security Consideration

Spoofing MAC CEs may lead to RAN service failure and UE access failure, and the following are some examples.

- PCI or NCC

Analysis: In the security and privacy working group SA3, MAC CE security threats have been identified for the LTM (Lower-layer Triggered Mobility) procedure, as spoofing MAC CEs such as PCI or NCC could cause LTM procedure to fail. Addressing these threats and vulnerabilities is one of the priorities for R20. [4][5]

- Serving Cell ID

Analysis: If spoofing attack is possible, further security consideration for other MAC CEs could also possibly lead to RAN procedure failure. Serving Cell ID indicates the identifier of the cell the user is currently connected to. If an attacker spoofs it, it may cause the UE to attempt access to an incorrect cell, resulting in failed network access.

Some unprotected MAC CEs would lead to potential privacy leakage for UE, especially with enhanced AI capabilities for attackers in 6G, some of the parameters that were once considered not privacy-sensitive may become exposed to privacy risks. The following are some examples.

- C-RNTI + Serving Cell ID

Analysis: C-RNTI is related to a certain UE within the cell in a period of time, and Serving Cell ID indicates the identity of the primary serving cell to which the UE is currently connected. If the C-RNTI remains unchanged for an extended period within the serving cell, an attacker could continuously track the UE’s location information in that cell by monitoring the binding relationship between the C-RNTI and Serving Cell ID.

- C-RNTI + Serving Cell ID + TA Command + Beam index ID

Analysis: The TA value in TA Command reflects the signal propagation delay between the user equipment and the base station, which is proportional to the distance between the user equipment and the base station. By using the TA value, the user’s location can be narrowed down into a certain distant to the base station. Combined with the Beam index ID, an attacker can further determine the range of the user relative location to the base station (e.g., northeast direction), further reducing the positioning range to a sector-shaped area. If multiple historical TA/Beam sequences associated with the C-RNTI are also considered, the attacker may even reconstruct the location trajectory of a certain user corresponding to a certain C-RNTI in a period of time.

Observation on potential MAC Spec impacts: Potentially, for spoofing attacks, integrity protection could prevent tempering transmissions, and for leakage of sensitive information such as privacy during transmission, encryption could prevent attacker from theft transmission information.

  - Combination of PHY and MAC CE Security Consideration

The attackers can also combine the PHY layer and MAC CE to carry out attacks.The following are some examples.

- MAC CE and UCI

Analysis: Based on the time-frequency synchronization described above, an attacker can decode additional unprotected information from the other physical channels, such as PUCCH and PDCCH. For PUCCH channel, there is a typical attack model related to user privacy [3]. The attacker can spoof MAC CE naming CSI reporting on PUCCH Activation/Deactivation to deceive users into transmitting or stop transmitting CSI reports information, which contains a pair of values, i.e., beam index and RSRP, and also RNTI of the UE. Since the CSI reports information are transmitted unprotected in PUCCH, the attacker can obtain the CSI reports information to further track the UE path based on some path inference algorithm. As described in [3], the results show that the attacker is able to recover the real path with a maximum deviation below 14 meters in 90.32% , and the average error across paths is 5.34 meters.

Figure 2.3.1 Comparison of the real path and the estimated path by the attacker [3]

- MAC CE and DCI

Analysis: An attacker can also combine the PHY layer and MAC CE to carry out attacks. PDCCH Order (PO) is a special DCI used to indicate the UE to initiate a RA procedure to re-establish synchronization in the UL. On one hand, the attacker can inject POs to multiple users to initiate the RA procedures, which not only amplifies the collision effect during RA but also wastes the battery consumption of the UEs.

On the other hand, the attacker can combine POs and other vulnerabilities to violate users' privacy.  The attacker can perform a PO spoofing attack to trigger the RA procedure, and further estimate user location by using a single-device fingerprinting-based approach [3]. Specifically, the attacker can use the deduced beam and TA value that were monitored during RA to estimate the azimuth of the UE and the distance from the Base Station respectively, resulting in an estimation of the UE location.

These above examples are just scratching the surface of what is needed to be analyzed. The above examples with other additional unprotected parameters may prove more vulnerability of privacy exposure and security attacks. Therefore, it is beneficial that RAN can include PHY layer and MAC layer security and privacy considerations and incorporate security requirements/mechanisms with SA3.

- Conclusion

Proposal 1: Discuss and consider to study potential security/privacy protection schemes for PHY layer and MAC layer in 6G RAN, in alignment with SA3 threat analysis and security requirements.

- References

[1]	3GPP TS 22.137: ‘Service Requirements for Integrated Sensing and Communication’.

[2]	3GPP TR 22870: ‘ Study on 6G Use Cases and Service Requirements’.

[3]	Ludant N, Vomvas M, Noubir G. Unprotected 4G/5G Control Procedures at Low Layers Considered Dangerous[J]. arXiv preprint arXiv:2403.06717, 2024.

[4]	S3-251124:	‘Reply LS on security handling for inter-CU LTM in non-DC cases Feasibility’.

[5]	S3-252285 : ‘Living document on NR mobility enhancement’.