3GPP TSG RAN Meeting #108	RP-251263

Prague, CZ, June 9th – 13th, 2025

Agenda Item:	15.3.2

Source:	Tejas Networks

Title:	Views on Scope for R20 “Sensing/ISAC for 5G-Advance and 6G”

Document for:	Discussion and Decision

Introduction

The study on channel modeling for Integrated Sensing and Communication (ISAC) in NR was initiated at RAN#102 [1], encompassing the following use cases and sensing modes targeted for Release 19:

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic).

Furthermore, in RAN-107 3GPP agreed to discuss and check the following aspects in RAN#108:

- The discussion on sensing/ISAC in 5GA and 6G are independent. - The objectives of Sensing/ISAC in Rel-20 are listed below: - Study on gNB mono-static mode for UAV use case. [RAN3, RAN1] - Study the procedures and signaling support between RAN and CN, and the corresponding sensing measurements. [RAN3, RAN1] - Performance evaluation for gNB mono-static sensing. [RAN1] - Note: No new waveform/RS to be specified in RAN1 and no UE impact is expected. - Note: Coordination with SA2 when needed. - It’s assumed 9 months for the Rel-20 Study Item on Sensing/ISAC, followed by 9 months for the subsequent Work Item based on the conclusion of SI.

This contribution provides our detailed views on scope of ISAC work for release-20 in 5G advanced and 6G track.

Study on ISAC/Sensing in Release-20

The ISAC system architecture work will be completed by 3GPP SA1 in release-19 which focused on many use-cases including object and target detection and tracking, such as unmanned aerial vehicles (UAVs), automotive vehicles, and human beings, as well as environmental monitoring applications like rainfall and flood detection. Currently, 3GPP RAN1 is in the final stage of ISAC channel modelling efforts. Most of the work related to ISAC deployment scenarios have been completed and ISAC channel modelling is expected to be completed as soon a few important aspects related to channel concatenation and channel power normalization are resolved. This channel is designed to support all the use-cases defined by 3GPP-SA1 in 22.837 for sensing. The channel modeling and calibration work is expected to be completed before 3GPP RAN1#122 in Release-19. During RAN#107, companies agreed to keep study of ISAC in 5G advance and 6G independent. It was agreed that the ISAC study would likely be led by RAN3 with focus on monostatic sensing and possible extension to bistatic sensing for detection of UAV use-case with scope to:

- Study on gNB mono-static mode and possibly bi-static mode for UAV use case. [RAN3, RAN1]

- Study the procedures and signaling support between RAN and CN, and the corresponding sensing measurements. [RAN3, RAN1]

- Performance evaluation for gNB sensing. [RAN1]

- Note: No new waveform/RS to be specified in RAN1 and no UE impact is expected.

- Note: Coordination with SA2 when needed.

## gNB-gNB Bistatic Sensing

The sensing can be performed based on one or multiple of the following sensing modes:

| Sensing Mode | Merit | Limitation |
| --- | --- | --- |
| gNB monostatic | Very low overhead | Self-interference cancellation is very difficult |
| gNB-gNB bistatic | Low overhead | New SFI formats required |
| gNB-UE bistatic* | Low standards impact | High reporting overhead |
| UE monostatic | Low overhead | Self-interference cancellation is difficult. Poor sensing performance |
| UE-UE bistatic |  | Localized sensing Synchronization and calibration issues |
| Note: * 1. One or multiple gNB(s) transmits and one or multiple UE(s) receives the sensing signal in DL or, 2. One or multiple UE(s) transmits, and one or multiple gNB(s) receives the sensing signal in UL. |  |  |

Sensing

Communication

Figure 1: Sensing aerial target based on gNB-gNB multi-static sensing.

We believe that gNB based monostatic sensing is a good starting point for ISAC standardization in 3GPP as it has little impact on RAN standards. However, it is very challenging to cancel the in-band self-interference in monostatic sensing and this phenomenon has not been studied in 3GPP in detail before. This cancellation could be further complicated by the nature of OFDM waveform. In view of the above challenges, we also opine that gNB-to-gNB bistatic sensing mode is the most reasonable and practical option and should be prioritized in the ISAC study by 3GPP RAN for all its works. Moreover, this aligns with views of the majority stakeholders in SA2#167 prioritizing gNB based sensing. Therefore, we propose that the gNB-to-gNB bistatic sensing mode should be prioritized. It has following advantages over the other use-cases:

- Low reporting overhead from gNB to Sensing Function,

- We believe this case has the limited impact on existing standards,

- High sensing accuracy due to large bandwidth and multiple Tx-Rx antennas at gNB,

- Excellent LOS probability with targets like UAVs in gNB-to-gNB sensing,

- Superior sensing coverage and tracking with advanced transmitters and receivers,

- Reduced network overhead (e.g., fewer handovers) compared to other sensing modes

- Static nature of gNBs, with precisely known location, simplifies initial implementation compared to bistatic scenarios with UEs.

Observation-1: Its challenging to cancel the in band self-interference caused by monostatic sensing.

Observation-2: The gNB-to-gNB bistatic mode is the most viable and efficient option for implementing sensing in existing networks.

Observation-3: gNB-gNB bistatic has following advantages over the other use-cases:

- Low reporting overhead from gNB to Sensing Function,

- We believe this case has the limited impact on existing standards,

- High sensing accuracy due to large bandwidth and multiple Tx-Rx antennas at gNB,

- Excellent LOS probability with targets like UAVs in gNB-to-gNB sensing,

- Superior sensing coverage and tracking with advanced transmitters and receivers,

- Reduced network overhead (e.g., fewer handovers) compared to other sensing modes

- Static nature of gNBs, with precisely known location, simplifies initial implementation compared to bistatic scenarios with UEs.

Proposal-1: gNB monostatic and gNB-gNB bistatic mode should be studied in Rel-20 for UAV sensing use-case.

Proposal-2: Considering the practical deployments in mind, the gNB-to-gNB bistatic should be prioritized equally with the monostatic mode.

It is unusual for RAN3 to lead a study item when there is a significant RAN1 impact. However, as RAN1 is constrained due to limited time units, we are okay with RAN3 leading the study item, followed by a feasibility study carried out by RAN1 as soon as RAN3 makes enough progress on the RAN architecture, signalling, and procedures part.

Observation-4: It is unusual for RAN-3 to lead a study item when there is a significant RAN1 impact.

Proposal-3: We support RAN3 leading the study item, followed by feasibility study carried out by RAN1 as soon as enough progress is made on the RAN architecture, signalling and procedures part.

Proposals and Conclusions

This section captures all the observations and proposals made in this document.

## Conclusions

Observation-1: Its challenging to cancel the in band self-interference caused by monostatic sensing.

Observation-2: The gNB-to-gNB bistatic mode is the most viable and efficient option for implementing sensing in existing networks.

Observation-3: gNB-gNB bistatic has following advantages over the other use-cases:

- Low reporting overhead from gNB to Sensing Function,

- We believe this case has the limited impact on existing standards,

- High sensing accuracy due to large bandwidth and multiple Tx-Rx antennas at gNB,

- Excellent LOS probability with targets like UAVs in gNB-to-gNB sensing,

- Superior sensing coverage and tracking with advanced transmitters and receivers,

- Reduced network overhead (e.g., fewer handovers) compared to other sensing modes

- Static nature of gNBs, with precisely known location, simplifies initial implementation compared to bistatic scenarios with UEs.

Observation-4: It is unusual for RAN-3 to lead a study item when there is a significant RAN1 impact.

## Proposals

Proposal-1: gNB monostatic and gNB-gNB bistatic mode should be studied in Rel-20 for UAV sensing use-case.

Proposal-2: Considering the practical deployments in mind, the gNB-to-gNB bistatic should be prioritized equally with the monostatic mode.

Proposal-3: We support RAN3 leading the study item, followed by feasibility study carried out by RAN1 as soon as enough progress is made on the RAN architecture, signalling and procedures part.

References

- RP-234069, New SID: Study on channel modelling for Integrated Sensing and Communication (ISAC) for NR.

- RP-250812, RAN Chair’s Summary of 5G-Advanced in Rel-20.

- TR 22.837, Feasibility Study on Integrated Sensing and Communication.