3GPP TSG RAN meeting #108								     		RP-251271

Prague, Czech Republic, June 9th - 13th, 2025

## Status Report to TSG

Agenda item:			9.4.1.3

| WI / SI Name | Introduction of Ku Bands for NR NTN |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_Ku_bands |  |  |  |  |
| Unique ID | 1040135 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP‑250799 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 65% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Manook Soghomonian |
|  | Company | Intelsat |
|  | Email | Manook.soghomonian@intelsat.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN4

Summary

During RAN#107, the checkpoint to check the feedbacks on specifying both FR1-NTN numerology and FR2-NTN numerology was removed, and in addition, the 15kHz SCS support for 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidth is added.

Based on the updated WID, RAN4 had agreed the system parameters for the priority 1 bands, including the needed update on the transmission bandwidths, minimum guard bands, ARFCN, sync raster design, asymmetric channel bandwidth support and the flexible TX-RX frequency separation support. Considering the workload, the group had agreed that priority 2 bands will not be treated until we reach a level of completion for priority 1 bands.

Then the group had discussed the simulation scenarios and the assumptions for the coexistence study, and the initial ACIR results had been provided for further discussion.

Regarding the UE and SAN RF requirements, the group discussed the general impacts of RF requirements and reached a consensus that certain existing FR2-like general requirements could be applied to both FR1-NTN and FR2-NTN Ku bands. However, the transient period length should continue to be handled separately for FR1-NTN and FR2-NTN. Then the group had agreed on the handling of FCC & CEPT regulations, and several related requirements had been identified. In addition, the group had also agreed the required minimum peak EIRP for VSAT communicating with GSO and LEO, however there are still remaining requirements to be discussed.

RAN4 has started the RRM part of the work, and several impacts on the current RRM requirements have been identified.

#### 2.1.1	Agreements

##### Core part:

###### RAN WG4 Meeting #114bis, Wuhan, China, 07th – 11th April, 2025:

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_General in [49] were approved:

WF for NTN Ku band General and work plan

Sub-topic 1-1: Draft CR for baseline band plan

The baseline decisions from RAN4 #114 are incorporated in the draft CR in R4-2504721.

Sub-topic 1-2: Corrections to priority 1 band plan agreements

Add to Table 5.2.2-1 and 5.2.3-1 the following note to fully specify the bands for Region 2:

NOTE :	For Region 2, the downlink is limited to 10700 – 12700 MHz.

Sub-topic 1-3: Priority 2 band plan

Priority 2 bands will not be treated until we reach a level of completion for priority 1 bands, say 90%. We will assess the progress at the end of each meeting to decide whether priority 2 can be started at the next meeting.

WF for NTN Ku band System parameters

Sub-topic 2-1: Transmission bandwidth configuration

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB for FR1-NTN

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 270 | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 133 | 189 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 65 | 93 | 135 |

Sub-topic 2-2: Minimum guard band

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz) for FR1-NTN

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 692.5 | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 1045 | 965 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1570 | 1490 | 1370 |

Sub-topic 2-3: UE channel bandwidth per operating band

Table 5.3.5-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
| n248 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.5-2: Channel bandwidths for each NTN satellite band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n509 | 120 | 50 | 100 | 200 | 400 |
| n508 | 120 | 50 | 100 | 200 | 400 |

Sub-topic 2-4: Asymmetric channel bandwidth support

Table 4: FDD asymmetric UL and DL channel bandwidth combinations proposed for Rel.19 NTN Ku band

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n248, n247 | 10 | 15, 25, 35, 50 | 0 |
|  | 15 | 10, 25, 35, 50 | 0 |
|  | 25 | 10, 15, 35, 50 | 0 |
|  | 35 | 10, 15, 25, 50 | 0 |
|  | 50 | 10, 15, 25, 35 | 0 |
|  | 10 | 20, 25, 35, 50, 70 | 1 |
|  | 20 | 10, 25, 35, 50, 70 | 1 |
|  | 25 | 10, 20, 35, 50, 70 | 1 |
|  | 35 | 10, 20, 25, 50, 70 | 1 |
|  | 50 | 10, 20, 25, 35, 70 | 1 |
|  | 70 | 10, 20, 25, 35, 50 | 1 |
|  | 10 | 20, 25, 35, 50, 70, 100 | 2 |
|  | 20 | 10, 25, 35, 50, 70, 100 | 2 |
|  | 25 | 10, 20, 35, 50, 70, 100 | 2 |
|  | 35 | 10, 20, 25, 50, 70, 100 | 2 |
|  | 50 | 10, 20, 25, 35, 70, 100 | 2 |
|  | 70 | 10, 20, 25, 35, 50, 100 | 2 |
|  | 100 | 10, 20, 25, 35, 50, 70 | 2 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [11], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

Sub-topic 2-5: SAN channel bandwidth

Define the SAN channel bandwidth per operating band in Table 2-3 for n248, n247, and SAN channel bandwidth per operating band in Table 2-4 for n509, n508.

Sub-topic 2-6: FR1-NTN ARFCN definitions

Adopt the following for bands n248 and n247

| Band | SCS | Uplink |  |  | Downlink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n248 | 15 | 1333334 | <1> | 1366666 | 1113334 | <1> | 1250000 |
| n247 | 15 | 1316667 | <1> | 1333333 | 1113334 | <1> | 1250000 |

Sub-topic 2-7: FR2-NTN ARFCN definitions

Decisions on further FR2-NTN ARFCN definitions for priority 2 bands are deferred as per sub-topic 1-3.

Sub-topic 2-8: FR1-NTN GSCN definitions

Adopt the following for bands n248 and n247

| Band | SCS | Case | Lower | Step | Upper |
| --- | --- | --- | --- | --- | --- |
| n248 | 15 | Case A | 12848 | <4> | 14268 |
| n247 | 15 | Case A | 12848 | <4> | 14268 |

Sub-topic 2-9: FR2-NTN GSCN definitions

Decisions on further FR2-NTN ARFCN definitions for priority 2 bands are deferred as per sub-topic 1-3.

Note that a draft CR was endorsed [2] to capture the agreements for the general part.

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_Coexistence in [48] were approved:

Agreed Way Forward for the Ku band coexistence

The following parameters were agreed:

To focus on 4 worst case scenarios

1. NTN UL interfering TN UL

2. NTN DL interfering TN DL

3. TN DL interfering NTN DL

4. TN DL interfering NTN UL

Simulate 90 and 35-degree elevation angles in line with ETSI standards

Use the following Table 1 for the simulation matrix

The following Table 1 illustrates agreed simulation matrix:

Table 1 Simulation Matrix

| VSAT model SAN | 45 x 45 Phased Array (1st step for coexistence) | 60 cm Parabolic (1st step for coexistence) | 78 x 78 Phased Array |
| --- | --- | --- | --- |
| GEO | X | X | X |
| LEO 600km | X | X |  |

Following Table 2 illustrates agreed LEO and GEO Satellite parameters:

Table 2 Satellite Parameters

| Satellite orbit |  |  | GEO | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 600 km |
| Satellite antenna pattern |  |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) |  | Ku-band (i.e. 11 GHz for DL) | 4 m | 0.91 m |
| Satellite EIRP density |  |  | 43 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  |  | 43 dBi | 38.5 dBi |
| 3dB beamwidth |  |  | 0.400 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  |  | 250 km | 19 km |
|  | Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) |  | Ku-band (i.e. 14 GHz for UL) | 4 m | 0.714 m |
| G/T |  |  | 16.5 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  |  | 44 dBi | 38.5 dBi |
|  | NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |

And the following Table 3 illustrates the agreed Phased Array and Parabolic Antenna parameters:

Table 3 Phase Array and Parabolic Antenna Parameters

| 1 | VSAT Antenna Characteristics |  |  |  |
| --- | --- | --- | --- | --- |
| 1.1 | Antenna pattern | TR 38.921, TR 38.922 | TR 38.921, TR 38.922 | Circular Aperture TR 38.863 |
| 1.2 | Element gain (dBi) (Note 1) | 3.5 | 3.5 | N/A |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis | 90° for Y axis 90° for Z axis | N/A |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB | 30 dB | N/A |
| 1.5 | Antenna polarization (Note 5) | Circular (RHCP or LHCP) | Circular (RHCP or LHCP) | Circular (RHCP or LHCP) |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 3) | 45x45 elements | 78x78 elements | 60 cm diameter aperture |
| 1.7 | Number of supported polarizations, P | 1 | 1 | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda | 0.5 lambda | N/A |
| 1.9 | Array Ohmic loss (dB) (Note 1) | 2 | 2 | N/A |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2) | 6.9 | 9.1 | N/A |
| 1.11 | Tx Transmit power | 10 Watts for GEO and for LEO | 50 Watt for GEO | 10 Watts for GEO and for LEO |
| 1.12 | Maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees | 0~360 degrees | 0~360 degrees |
| 1.13 | Vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees | 0~60 degrees | 0~60 degrees |
| 1.14 | Normal direction | Toward X+ axis | Toward X+ axis | Toward X+ axis |
| 1.15 | Max EIRP Value | 76.6 dBm | 88.2 dBm | 76.6 dBm |
| 1.16 | Noise Figure | 6 dB | 6.0 dB for GEO | 2.5 dB for both LEO and GEO |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Note 5: Antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization evaluations are conducted by converting to linear polarization |  |  |  |  |

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_UE_SAN_RF in [47] were approved:

WF for NTN Ku band UE RF

Sub-topic 1-1: General issues for Ku band UE RF requirements

Issue 1-1-1: Limitation on the mobile VSAT type 4/5

The following changes (in red) can be considered for mobile VSAT 4/5 (updated from WF in R4-2502288)

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52,3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to n512, n511, n510, n509, n508 and n248, n247; LEO support is only defined for n509, n508 and n248, n247 |  |  |

Issue 1-1-2: Needed update for the requirement set applicability

Agreement:

The updated requirement set applicability due to the introduction of NTN Ku bands:

| Requirement | Requirement set |  |  |
| --- | --- | --- | --- |
|  | FR1-NTN bands |  | FR2-NTN bands |
|  | Terminal with antenna connector for bands < 10 GHz | VSAT types terminal for bands > 10 Ghz | VSAT types terminal |
| Conducted transmitter characteristics | Clause 6 | NA | NA |
| Conducted receiver characteristics | Clause 7 | NA | NA |
| Conducted performance requirements | Clause 8 | NA | NA |
| Radiated transmitter characteristics | NA | Clause 9 | Clause 9 |
| Radiated receiver characteristics | NA | Clause 10 | Clause 10 |
| Demodulation performance requirements (Radiated requirements) | NA | Clause 11 | Clause 11 |

Issue 1-1-3: Consideration on the VSAT RF requirements as per satellite orbit

The VSAT RF requirements for the Ku band can follow the approach for the Ka-band as the baseline, i.e., the Ku VSAT requirement will be defined agnostic to the SAN orbit except the following aspect

Additional regional requirements

FFS on other potential exception (ex: minimum peak EIRP proposal in issue 1-2-1)

Issue 1-1-4: General RF requirements impacts on numerologies dependency

The following aspects are considered when defining the general RF requirements for the Ku bands.

No update needed on 9.3.1 Minimum output power, 9.3.2 transmit off power, 9.3.4 Power control, and 9.4 Transmitter signal quality for supporting Ku bands.

For 9.3.3 Transmit ON/OFF time mask,

The time masks and transient period times will be re-used for FR2-NTN Ku bands

FFS on FR1-NTN Ku bands

Issue 1-1-5: Handling of FCC, ISED, CEPT regulation related requirement

New NSs will be defined for the Ku band concerning different regulations

FFS on whether different NS numbers will be used for FR1-NTN Ku bands and FR2-NTN Ku bands

Different NS numbers will be used to handle FCC and CEPT regulation at least

FFS on how to handle FCC and ISED requirements in TS 38.101-5

Option 1: capture the most stringent requirement with same NSs

Option 2: separate NSs

Issue 1-1-6: Handling of other regulation related requirement

Interested companies are encouraged to further study the requirements of other regulations.

Sub-topic 1-2: Tx UE requirements for the NTN Ku band

Issue 1-2-1: Minimum EIRP requirements

Consider the following as the starting point

| Operating band | UE Type | Min peak EIRP (dBm) |  |
| --- | --- | --- | --- |
|  |  | [When communicating with LEO] | [When communicating with GSO] |
| n248, n247, n509, n508 | 4 | [64] | [70] |
|  | 5 | [64] | [70] |

FFS on whether further clarification is needed

Issue 1-2-3: Off-axis EIRP requirements for the CEPT Ku band

Agreement:

Adopt the following off-axis EIRP requirements for the CEPT Ku band n248 and n509

For co-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.5° ≤ θ ≤ 7° | 63 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 42 – K | 40 |
| 9.2° ≤ θ ≤ 48° | 66 – 25log10(θ) – K | 40 |
| 48° < θ | 24 – K | 40 |
| NOTE 1: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.3.2 in EN 301 428. The manufacturer shall declare the value of N. NOTE 2: For θ> 70° the values given above may be increased to 4 - K dBW over the range of angles for which the particular feed system may give rise to relatively high levels of spillover. |  |  |

For cross-polarized transmissions

| θ value | Maximum Off-axis EIRP (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2.5° ≤ θ ≤ 7° | 53 – 25log10(θ) – K | 40 |
| 7° ≤ θ ≤ 9.2° | 32 – K | 40 |
| NOTE: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.3.2 in EN 301 428. The manufacturer shall declare the value of N. |  |  |

Issue 1-2-4: On-axis spurious requirements for the CEPT Ku band

Agreement:

Adopt the following on-axis EIRP requirements for the CEPT Ku band n248 and n509

For the “emissions disabled” and “carrier-off” states

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 9 | 100 |

For the “Carrier-on” state

Table 1-2-4.1: On-axis spurious limits in “Carrier-on” state - outside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- | --- |
| 14.0 – 14.5 | 4, 5 | 34 - K (NOTE) | 100 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in EN 302 977. The manufacturer shall declare the value of N. |  |  |  |

Table 1-2-4.2: On-axis spurious limits in “Carrier-on” state - inside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- | --- |
| 14.0 – 14.5 | 4, 5 | 48 - K (NOTE) | 100 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in EN 302 977. The manufacturer shall declare the value of N. |  |  |  |

Issue 1-2-5: Off-axis spurious requirements for the CEPT Ku band

Agreement:

Adopt the off-axis spurious requirements for the CEPT Ku band n248 and n509 based on the stringent requirement among EN 302 977, EN 303 980, EN 303 981 for both GSO and NGSO as the baseline, and align those requirement with 100kHz measurement bandwidth.

The following two tables are the baseline

Table 1-2-5.2: Limits of off-axis spurious EIRP - "Transmission disabled" state

| Frequency band | EIRP limit | EIRP limit (to be removed) | Measurement bandwidth |
| --- | --- | --- | --- |
| 1,0 GHz to 2,0 GHz | -38 dBm | 42 dBpW | 100 kHz |
| 1,0 GHz to 10,7 GHz | -42 dBm | 48 dBpW | 100 kHz |
| 10,7 GHz to 21,2 GHz | -36 dBm | 54 dBpW | 100 kHz |
| 21,2 GHz to 40,0 GHz | -30 dBm | 60 dBpW | 100 kHz |

Table 1-2-5.3: Limits of off-axis spurious EIRP “carrier-on” and “carrier-off” state

| Frequency band | EIRP limit | EIRP limit (to be removed) | Measurement bandwidth |
| --- | --- | --- | --- |
| 1,0 GHz to 2,0 GHz | -47 dBm | 43 dBpW | 100 kHz |
| 2,0 GHz to 3,4 GHz | -41 dBm | 49 dBpW | 100 kHz |
| 3,4 GHz to 10,7 GHz | -35 dBm | 55 dBpW | 100 kHz |
| 10,7 GHz to 13,75 GHz | -29 dBm | 61 dBpW | 100 kHz |
| 13,75 GHz to 14,00 GHz | 5 dBm (see note) | 95 dBpW (see note) | 10 MHz |
| 14,50 GHz to 14,75 GHz | 5 dBm (see note) | 95 dBpW (see note) | 10 MHz |
| 14,75 GHz to 21,2 GHz | -29 dBm | 61 dBpW | 100 kHz |
| 21,2 GHz to 60,00 GHz | -23 dBm | 67 dBpW | 100 kHz |
| NOTE: This limit may be exceeded in a frequency band which shall not exceed 50 MHz, centred on the carrier frequency, provided that the on-axis EIRP density at the considered frequency is 50 dB below the maximum on-axis EIRP density of the signal (within the nominated bandwidth) expressed in dBW/100 kHz. |  |  |  |

Issue 1-2-5: Off-axis spurious requirements for the CEPT Ku band

Sub-topic 1-3: Rx UE requirements for the NTN Ku band

Issue 1-3-1: EIS requirement

Agreement:

FFS on the following two options

Option 1:

Defining EISREFSENS_10MHz for FR1-NTN Ku bands, and define EISREFSENS_50MHz for FR2-NTN Ku bands

Option 2:

Consider define the common EIS requirement for both FR1-NTN Ku bands and FR2-NTN Ku bands

FSS on the value of the EISREFSENS_10MHz and EISREFSENS_50MHz

The requirement for other channel bandwidth will be scaled based on PRB based

Issue 1-3-2: Receiver antenna off-axis gain requirements for the CEPT Ku band

Proposals:

Proposal 1 (ZTE): It is proposed to define the receiver antenna off-axis gain of each co-polarized and cross-polarized components as Table 1-3-2.1 and Table 1-3-2.2 respectively for CEPT Ku band.

Table 1-3-2.1: Off-axis Co-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φmin ≤ φ < 48° | 32 – 25log10(φ) |
| 48° ≤ φ ≤ 85° | -10 |
| 85° ≤ φ ≤ 180° | -2 |
| NOTE: φmin = 1o or 100λ/D (degrees) whichever is the greater, for D/λ≥ 50. φmin = 2o or 114(D/ λ)-1.09 (degrees) whichever is the greater, for D/λ 50. where D is the nominal diameter of the antenna |  |

Table 1-3-2.2: Off-axis Cross-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φr ≤ φ ≤ 7° | 23 – 20log10(φ) |
| 7° < φ ≤ 32.3° | 20.2 – 16.7log10(φ) |
| 32.3° < φ ≤ 75° | -5 |
| 75° < φ ≤ 180° | -2 |
| NOTE: φr = 1o or 100λ/D (degrees) whichever is the greater where D is the nominal diameter of the antenna |  |

Agreement:

Agree proposal 1

Sub-topic 1-4: Work split for the NTN Ku band

Issue 1-4-1: Work split for the UE RF specification 38.101-5

Agree on the worksplit based on Table 1-4-1.1 in [47]

WF for NTN Ku band SAN RF

Sub-topic 2-1: General issues for Ku band SAN RF requirements

Issue 2-1-1: General issues for Ku band SAN RF requirements

Agreement:

FFS on whether to introduce a new SAN type for FR1-NTN Ku bands which is based on SAN type 2-O RF requirements.

Sub-topic 2-2: Specific topics for Ku band SAN RF requirements

Issue 2-2-1: The tolerance for the radiated transmit power and SAN output power

Agreement:

For the tolerance of the radiated transmit power and SAN output power for both the FR1-NTN and FR2-NTN Ku bands

[+-3.4] dB EIRP accuracy,  [+-3] dB TRP accuracy (re-using FR2-NTN) as baseline

Other options are not precluded

Other NTN bands will not be impacted

Issue 2-2-2: OTA RE power control dynamic range for the Ku band SAN

Agreement

The OTA RE power control dynamic range is not required for FR2-NTN Ku-band SAN.

Further discuss whether OTA RE power control dynamic range (i.e. RE power boosting) is needed for FR1-NTN Ku-band SAN for the smaller channel bandwidth.

Issue 2-2-3:  OTA total power dynamic range for the Ku band SAN

Agreements:

Agree on adding the following OTA SAN total power dynamic range requirement for new channel bandwidths of FR1-NTN Ku bands.

Table 2-2-3.3

| SAN channel | Total power dynamic range (dB) |  |
| --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS |
| 25 | 21.2 | 18.1 |
| 35 | 22.7 | 19.6 |
| 50 | 24.3 | 21.2 |
| 70 | N/A | 22.7 |
| 100 | N/A | 24.3 |

Issue 2-2-5:  OTA sensitivity for the Ku band SAN

Agreement:

Proposal 1 (ZTE): It is proposed not to specify OTA SAN sensitivity for NTN Ku bands.

Issue 2-2-7:  OTA dynamic range for the Ku band SAN

Agreement:

Proposal 1 (ZTE): Not to specify OTA SAN dynamic range for NTN Ku bands.

The following agreements documented in the Way Forward for NR_NTN_Ku_Bands (RRM part) in [50] were approved:

WF for RRM requirements for NR_NTN_Ku_bands

Issue 1-1: RRM scope for NR NTN on Ku bands

Agreement:

Define RRM requirements for added 15kHz SCS in FR1-NTN numerology for GSO in Ku-band

Issue 1-6: NR SAN FR2-NTN – NR SAN FR2-NTN Handover (section 6.1C.1.3)

Agreement:

Reuse the same value of T_sat_beam as in Ka band for Ku band

For VSAT UEs operating in FR1-NTN numerology, not specify the RRM requirements for TN-NTN mobility, inter-RAT mobility [or satellite switching with re-sync].

Issue 1-7: timing accuracy requirements for case 1 and 2 (section 7.1C)

Agreement:

Both 15kHz and 30kHz SCS for timing error requirements should be considered for Ku-band

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  | 30 | 30 | 22*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |

Issue 1-8: timing accuracy requirements for case 3 (section 7.1C)

Agreement:

7.5*64*Tc with the same side condition as agreed for Rel-18 Ka band.

Issue 1-9: timing accuracy requirements for case 4 (section 7.1C)

Agreement:

The same Te_NTN requirement applies to case 4. On top of the side conditions agreed for Rel-18 Ka band, RAN4 to add the following side condition to Te_NTN requirements for Case 4

Option 1: Capture it as a NOTE on the test case in the core requirement: The interval between adjacent epoch times should not exceed [2.56 or 5.12] sec in the core corresponding test case.

Option 2: Capture it as a NOTE in core requirement: The interval between adjacent epoch times should not exceed [2.56 or 5.12] sec.

Issue 1-10: timing accuracy requirements applicability (section 7.1C)

Agreement:

The timing accuracy requirements for mobile VSAT served by NGSO are applicable for operation on Ku bands only. FFS how to capture it in the specification.

Issue 1-11: NTAoffset (section 7.1C.2)

Agreement:

If UE is not provided with the information n-TimingAdvanceOffset, the default value of N_"TA offset" is set as 0 for FR1-NTN numerology and FR2-NTN numerology.

Issue 1-14: CR split

| Section Number | Section/ sub-section title | Leading company |
| --- | --- | --- |
| 3 | Definitions, symbols and abbreviations |  |
| 4 | SA: RRC_IDLE state mobility | ZTE |
| 6 | RRC_CONNECTED state mobility | Huawei |
| 7 | Timing | MTK |
| 8 | Signalling characteristics |  |
| 9 | Measurement Procedure | Nokia |

Endorsed draft CRs in RAN4#114bis:

[2] R4-2504721, Draft CR for Operating bands including priority 2 cases	Eutelsat Group

###### RAN WG4 Meeting #115, Malta, Malta, 19th – 23th April, 2025:

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_General in [57] were approved:

WF for NTN Ku band General and work plan

Sub-topic 1-1: Numbering of the Ku bands

The existing band numbering will remain. Further discussion on optimizing band numbering is FFS.

WF for NTN Ku band System parameters

Sub-topic 2-1: Addition of requirement that SSB SCS and data channel SCS are the same

Add “NOTE 2:    SS Block SCS is same with SCS for data channels.” for band n248 and n247 for applicable SS raster in Table 5.4.3.3-1 as below:

Sub-topic 2-2: Definition of channel rasters

Modify the draft CR for channel raster to align with 38.104 wording:

Sub-topic 2-3: Tx-Rx frequency separation

Agreement:

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n248 | -3 795 to -1 255 MHz |
| n247 | -3 295 to -1 005 MHz |

Table 5.4.4-2: UE TX-RX frequency separation (FR2-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n509 | -3 750 to -1 300 MHz |
| n508 | -3 250 to -1 050 MHz |

Sub-topic 2-4: Add requirement to preclude simultaneous operation of FR1-NTN and FR2-NTN in the UE

Agreement

Further discuss whether and how to capture the note in UE or SAN spec.

“Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE.

Sub-topic 2-5: SAN channel bandwidth

Agree the change to not require support of the 7.5 kHz frequency shift for bands n247 and n248.

Sub-topic 2-6: Corrections to channel raster definition

Agree corrections to the channel raster definition.

Note that a draft CR was endorsed [77] to capture the agreement in sub-topic 2-2, 2-5, 2-6.

Note that a running draft CR for 38.101-5 capturing changes in [2] and [77] was approved [52].

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_Coexistence in [59] were approved:

Agreements for the Ku band coexistence:

Following discussion during online and at ad-hoc sessions the following were agreed.

2.1 Practical considerations

For the purpose of writing the TR, at the time of this 3GPP co-existence study, there is no TN band defined near the simulated frequencies.

2.2 On the Throughput Performance

Use 5% percentile for the first preference and if no data is available then use average throughput loss results.

Note: in real satellite networks, the cell size is not well defined, and beams are overlapping

2.3 On the guard band between the terrestrial and the non-terrestrial Ku band in simulations:

Guard band was not considered, and it is not necessary for the co-existence analysis.

2.4 On the Calibration Data

Additional calibration data for the phase array would be needed as the current data is spread-out.

Calibration Assumptions:

For the calibration matrix, to use the assumptions from TR 38.863 Annex A.1 Table A.1-1 and Annex B.1 Table B.1-1 with further modifications based on the current parameters.

NTN assumptions

Re-use from Annex A.1 Table A.1-1 from TR 38.863 with BW 100 MHz

#UE – 1 DL/ 10 UL

Elevation angle – 35 degrees for both GEO and LEO

Urban scenario

VSAT phased array 45x45 assumptions:

VSAT NF of 6 dB

TN AAS assumptions

TN antenna parameters for 11 GHz and 14 GHz

#UE – 1 DL/ 3 UL

Urban scenario

100% outdoor

ISD is 0.45 km

Calibration Metrics:

UL - SINR and Coupling Loss distributions at 5%, 50% and 95%iles on the CDF.

DL - SINR, coupling loss and transmit power distributions at 5%, 50% and 95%iles on the CDF.

Prioritize the simulation results for the parabolic and then tackle the calibration for the phase array case.

Note: For the parabolic antenna, the actual simulation results presented are also spread out and would need additional data

2.5 On the FCC and CEPT Mask Compliance

On the FCC and CEPT mask for the co-existence study the consensus is to not to use the masks

Way Forward for the Ku band coexistence:

1. Companies are invited to provide additional calibration data for the parabolic antenna and phase array VSATs.

2. Companies are invited to provide co-existence simulation results based on the agreed simulation assumptions.

3. Companies are invited to provide mitigation techniques to provide better protection to NTN based on the VSAT ACS and SAN ACS co-existence results.

4. Companies are invited to provide analysis with considerations of NTN SAN/ VSAT ACLR and ACS values in order to ensure the coexistence between NTN and TN.

5. For the 78x78 phased array antenna configurations, companies are invited to prioritize scenario 1 for ACLR value and scenario 5 for the ACS value in the co-existence simulations.

Note that a running document containing Ku band co-existence simulation assumptions was approved [70].

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_UE_SAN_RF in [60] were approved:

WF for NTN Ku band UE RF

Issue 1-1-1: Transmit ON/OFF time mask for the FR1-NTN Ku bands

Agreement:

Apply 10 us transient period to the FR1-NTN Ku bands

Issue 1-1-2: Handling of FCC, CEPT regulation for n248, n509

Agreement:

Apply the same NS number for FR1-NTN Ku bands and FR2-NTN Ku bands with the same frequency range.

Consider the following NS values for bands n509 and n248:

NS_203N: to handle FCC Regulation for GSO.

NS_204N: to handle FCC Regulation for LEO.

NS_205N: to handle CEPT Regulation for GSO.

NS_206N: to handle CEPT Regulation for LEO.

Issue 1-1-3: Handling of ISED regulation for n248, n509

Agreement:

Assume additional NS values can be considered for ISED regulation as the starting point.

Companies are encouraged to provide the analysis on the ISED requirements in the next RAN4 meeting.

Issue 1-1-4: Handling of regulation for uplink 13.75 – 14.0 GHz band (n247, n508)

Agreement:

For Ku band with uplink 13.75 – 14.0 GHz, n247 and n508, consider two NS values (one for GSO operation, the other for LEO operation) concerning the regulation from Taiwan (which is referred and amended from the FCC regulation) as a starting point.

Encourage companies to study other available regulations for 13.75 – 14.0 GHz.

Issue 1-1-6: Dependency on different requirements

Agreement:

The maximum EIRP and off-axis EIRP mask requirements that have already been agreed upon will remain unaffected by the ACLR requirements.

Sub-topic 1-2: Tx UE requirements for the NTN Ku band

Issue 1-2-1: Clarification on the minimum peak EIRP requirements

Agreement:

- Currently the VSAT type 4 & type 5 covers both GSO and LEO, but companies show strong need on allowing VSAT support LEO only in Rel.19.

- Consider a new type for LEO only based on the same VSAT antenna size assumption as type 4 & 5 as a working assumption

Issue 1-2-2: Minimum peak EIRP requirements for GSO

Agreement:

- Confirm minimum peak EIRP at 70 dBm for Rel.19 Ku band VSAT communicating with GEO.

Issue 1-2-3: Minimum peak EIRP requirements for LEO

Agreement:

- Confirm minimum peak EIRP at 60 dBm as min peak EIRP for both electronically steering VSAT and mechanical steering VSAT connecting to LEO.

Issue 1-2-4: RB configuration for EIRP requirement

Agreement:

- Consider both minimum peak & maximum EIRP to be defined using RB allocation around 10~20MHz for the NTN Ku band VSAT as the starting point

- Consider the RB allocation number to be suitable for DFT-s-OFDM at least

Issue 1-2-6: PFD limits for the Ku band

Wayforward:

Encourage companies to further check the PFD requirements, whether it is based on the measurement or analysis, before capture it to the specification.

Issue 1-2-7: SEM requirements for uplink 13.75 – 14.0 GHz

Agreement:

- For the Ku band with uplink 13.75 – 14.0 GHz (Earth-to-Space), consider the same additional spectrum emission mask requirement as for n511, n510 as the starting point.

Issue 1-2-8: Off-axis and antenna requirements for uplink 13.75 – 14.0 GHz

Proposals:

Proposal 1 (CHTTL): For the Ku band with uplink 13.75 – 14.0 GHz (Earth-to-Space), the co-polarization & off-axis cross-polarization gain limit in Table 4 ~ 7 should be specified in 38.101-5.

Table 4: Co-polarization gain limit in the plane tangent to the GSO arc other than conventional Ku band

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 1.5° ≤ θ ≤ 7° | 29 – 25log(θ) |
| 7° ≤ θ ≤ 9.2° | 8 |
| 9.2° ≤ θ ≤ 19.1° | 32 – 25log(θ) |
| 48° < θ ≤ 180° | -10 |

Table 5: Co-polarization gain limit in the plane perpendicular to the GSO arc other than conventional Ku band

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 3° ≤ θ ≤ 48° | 32 – 25log(θ) |
| 48° < θ ≤ 180° | -10 |

Table 6: Cross-polarization gain limit in the plane tangent to the GSO arc for the Ku band

| θ value | Cross-polarization gain (dBi) |
| --- | --- |
| 1.8° ≤ θ ≤ 7° | 19 – 25log(θ) |

Table 7: Cross-polarization gain limit in the plane perpendicular to the GSO arc for the Ku band

| θ value | Cross-polarization gain (dBi) |
| --- | --- |
| 3° ≤ θ ≤ 7° | 19 – 25log(θ) |

Proposal 2 (CHTTL): For the Ku band with uplink 13.75 – 14.0 GHz (Earth-to-Space), the off-axis EIRP density limits requirement in Table 8 ~ 10 should be specified in 38.101-5.

Table 8: Off-axis EIRP density limits for co-polarized transmissions in the plane tangent to the GSO arc for the extended Ku-band

| θ value | Maximum Off-axis EIRP (dBW) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 1.5° ≤ θ ≤ 7° | 15 – 25log(θ) | 4 |
| 7° ≤ θ ≤ 9.2° | -6 | 4 |
| 9.2° ≤ θ ≤ 48° | 18 – 25log(θ) | 4 |
| 48° < θ ≤ 180° | -24 | 4 |

Table 9: Off-axis EIRP density limits for co-polarized transmissions in the plane perpendicular to the GSO arc for the extended Ku-band

| θ value | Maximum Off-axis EIRP (dBW) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 3° ≤ θ ≤ 48° | 18 – 25log(θ) | 4 |
| 48° < θ ≤ 85° | -24 | 4 |

Table 10: Off-axis EIRP density limits for cross-polarized transmissions in the plane tangent to the GSO arc and in the plane perpendicular to the GSO arc for the extended Ku-band

| θ value | Maximum Off-axis EIRP (dBW) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 1.5° ≤ θ ≤ 7° | 5 – 25log(θ) | 4 |

Agreement:

- Consider the proposal 1 and proposal 2 as the starting point.

Sub-topic 1-3: Rx UE requirements for the NTN Ku band

Issue 1-3-1: EIS requirement

Wayforward:

- Consider calculate the EIS requirement for 50MHz channel bandwidth based on the DL SNR target -1 dB (MCS for QPSK 1/3)

- At least 2dB implementation margin and will be taken into account.

- At least the beam gain reduction around [3.5dB] at the 30~35 degree elevation angle can be taken into account.

- Companies are encourage to provide views on the assumed VSAT G/T value for the EIS requirement in the next RAN4 meeting

Wayforward:

- Companies are encouraged to further check whether the OCNG Pattern is needed for the NTN EIS requirement in the next RAN4 meeting.

Issue 1-3-2: Maximum input level requirement

Agreement:

- RAN4 will define the maximum input level requirement for the NTN Ku band VSAT.

WF for NTN Ku band SAN RF

Sub-topic 2-1: General issues for Ku band SAN RF requirements

Issue 2-1-1: General issues for Ku band SAN RF requirements

Wayforward:

Consider to endorse the draft CRs to 38.108 for the Ku band SAN RF requirements based on the agreements in RAN4#114

Companies can provide a draft CR to show the benefits on introducing new SAN type for the Ku band NTN.

Sub-topic 2-2: Specific topics for Ku band SAN RF requirements

Issue 2-2-1: The tolerance for the radiated transmit power and SAN output power

Wayforward:

For the tolerance of the radiated transmit power and SAN output power for both the FR1-NTN and FR2-NTN Ku bands, down select to the following two options:

Option 1: [+-2.5] dB EIRP accuracy,  [+-2.2] dB TRP accuracy

Option 2: [+-3.4] dB EIRP accuracy,  [+-3] dB TRP accuracy (re-using FR2-NTN)

Issue 2-2-2: OTA RE power control dynamic range for the Ku band SAN

Agreement:

Consider not to specify OTA RE power control dynamic range for the FR1-NTN Ku bands at this stage, unless there is company showing strong interests.

Issue 2-2-3:  Spurious frequency range for the Ku band SAN

Proposals:

Proposal 1 (ZTE): It is proposed to define the upper limit of spurious frequency range as 26 GHz for both FR1-NTN Ku bands and FR2-NTN Ku bands.

Wayforward:

- Consider proposal 1 as the starting point, companies can further check in the next RAN4 meeting.

Issue 2-2-4:  OTA reference sensitivity level for the Ku band SAN

Wayforward:

- Consider calculate the SAN EIS requirement for 50MHz channel bandwidth based on the UL SNR target -1 dB (MCS for QPSK 1/3)

- At least 2dB implementation margin and will be taken into account.

- SAN parameters for GEO

Option 1: NF = 3, Rx gain = [36-44]

Option 2: G/T = 7.5~16.5

Option 3: G/T = 12.5~21.5

Other options are not precluded

- SAN parameters for LEO

Option 1: NF = 3, Rx gain = [30.5-38.5]

Option 2: G/T = 1.9~10.9

Other options are not precluded

Issue 2-2-5:  OTA out-of-band blocking for the Ku band SAN

Wayforward:

Option 1:

Consider ΔfOOB = 100 MHz for both FR1-NTN Ku bands and FR2-NTN Ku bands

Consider 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz is agreeable for both FR1-NTN Ku bands and FR2-NTN Ku bands

Option 2

Reuse the parameters for the Ka band

The following agreements documented in the Way Forward for NR_NTN_Ku_Bands (RRM part) in [55] were approved:

RRM impact of NR NTN on Ku bands

Issue 1-1: applicability update (section 3.6.12).

Agreement:

Clarify in clause 3.6.12 that “Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN”.

Issue 1-2: Assumption for defining RRM requirements in Ku-band NTN

Candidate options:

Option 1: (LGE)

The same assumptions considered for Rel-18 Ka-band RRM requirements should apply to Ku-band RRM requirements for both FR1-NTN and FR2-NTN numerologies;

Single SAN Tx beam per radio cell in DL

For inter satellite HO, only blind HO and no neighbor cell measurement

Intra satellite neighbor cell measurement is needed

Same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite (i.e., no Rx beam sweeping)

Option 2: other options are not precluded.

Issue 1-3: satellite switch requirements

Agreement:

Introduce RRM requirement for satellite switching with re-sync for Ku band with VSAT UE for FR1 and FR2 numerologies

Define requirement for hard satellite switch,

Tinterrupt = Tsat_beam + Tsearch + Tprocessing + T∆ + Tmargin ms

Further discuss the details on the definition of each component.

FFS whether to define requirements for soft satellite switch

FFS only switching delay requirement or both switching delay requirement and scheduling restriction requirement

Option for the delay requirement:

Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

Issue 1-4: Measurements of inter-frequency NR cells with NTN carrier (section 4.2.2.12).

Agreement:

Do not extend the applicability of clause 4.2.2.12 to VSAT.

Issue 1-5: inter-satellite cell reselection requirements (section 4.2C).

Candidate options:

Option 1: RAN4 to create inter-satellite cell reselection requirements for VSAT UEs with electronically steered beams in Rel-19 (Nokia)

Option 2: Do not define inter-satellite cell reselection requirements in this WI. (HW, ZTE, E///)

Option 2a: RAN4 to consider inter-satellite measurement related discussion for Ku-band NTN in a future release (e.g., Rel-20). (LGE)

Issue 1-6: timing accuracy requirements for case 4 (section 7.1C)

Agreement:

For timing requirement in Case 4, capture it as a NOTE in core requirement: The requirement applies when the interval between adjacent epoch times does not exceed 5.12 sec.

Issue 1-7: UE measurement capability (section 4.2C.2.1).

Agreement:

Re-use existing measurement capability requirements for VSAT in Ku bands.

Issue 1-8: RLM (section 8.1C)

Candidate options:

Option 1: LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology.

Option 2: LMax is equal to 8 for both FR1-NTN numerology and FR2-NTN numerology.

Issue 1-9: terminology update

Agreement:

For VSAT UE in Ku-band with FR1-NTN numerology, use the terminology of ‘VSAT UE in FR1-NTN.

Endorsed draft CRs in RAN4#115:

[52]   R4-2505737,	Running CR for 38.101-5 Ku bands	Eutelsat Group

[77]   R4-2508771, draft CR for updates to NTN Ku-band channel raster sections	CHTTL

[110] R4-2506784, draftCR on applicability of requirements for Ku band NTN		Huawei, HiSilicon

[113] R4-2507201, (NR_NTN_Ku_bands-Core) RRC IDLE state mobility	ZTE Corporation, Sanechips

[117] R4-2508383, DraftCR on uplink timing requirements for mobile VSAT served by NGSO	MediaTek inc.

[118] R4-2508387, DraftCR to TS 38.133 for including measurement procedures for VSAT UEs operating in KU band	Nokia

##### Performance part:

Not started.

#### 2.1.2	Remaining Open issues

##### Core part:

Define additional bands for Ku (discussion on the Priority 2 Ku band #2a and #2b in RP‑250799)

Completion of co-existence study

Define system parameters

System parameters for Priority 2 Ku bands

Specify RF requirements

Remaining SAN RF requirements including ACS, ACLR, the tolerance for the radiated transmit power and SAN output power, spurious frequency range, OTA reference sensitivity level, out-of-band blocking at least

Remaining UE RF Requirements including ACS, ACLR, RB configuration for EIRP requirements, EIS, maximum input level requirement at least

Specify RRM requirements

##### Performance part:

RRM performance requirements and test cases

UE demodulation and CSI reporting requirements

Satellite access node demodulation requirements

Satellite access node conformance tests

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

#### Not applicable.

## 4.	References

Contributions submitted to RAN4#114bis:

| Ref | Tdoc | Subject | Source | Agenda | Status |
| --- | --- | --- | --- | --- | --- |
| [1] | R4-2503940 | NTN Ku-band - General aspects - bands | Ericsson | 7.11.1 | noted |
| [2] | R4-2504721 | Draft CR for Operating bands including priority 2 cases | Eutelsat Group | 7.11.1 | endorsed |
| [3] | R4-2503320 | Discussion the assumptions and parameters for NR NTN Ku Band coexistence study | xiaomi | 7.11.2 | noted |
| [4] | R4-2503454 | Further discussion on coexistence evaluations for Ku-band for NR NTN | CATT | 7.11.2 | noted |
| [5] | R4-2504263 | On co-existence simulation assumptions for NR NTN Ku band | Ericsson | 7.11.2 | noted |
| [6] | R4-2504264 | On co-existence simulation results for NR NTN Ku band | Ericsson | 7.11.2 | noted |
| [7] | R4-2504338 | On Ku-band coexsitence studies | Samsung | 7.11.2 | noted |
| [8] | R4-2504477 | Discussion on Ku-band co-existence study | SKY Perfect JSAT Corporation | 7.11.2 | noted |
| [9] | R4-2504559 | Discussion on coexistence simulation parameters for GEO and LEO VSAT | SHARP Corporation | 7.11.2 | noted |
| [10] | R4-2504658 | Considerations for reducing Ku-band coexistence configurations | THALES | 7.11.2 | noted |
| [11] | R4-2503231 | Ku Band: Further System Parameters incorporating 15 kHz Sub-carrier Spacing and lower Uplink A Band (Priority 2 Band Combination) | Intelsat | 7.11.3 | noted |
| [12] | R4-2503334 | System parameters for Ku band(s) | Xiaomi | 7.11.3 | noted |
| [13] | R4-2503455 | Further discussion on system parameters for Ku-band for NR NTN | CATT | 7.11.3 | noted |
| [14] | R4-2503740 | Discussion on system parameters for NTN Ku band | ZTE Corporation, Sanechips | 7.11.3 | noted |
| [15] | R4-2503941 | NTN Ku-band - System parameters | Ericsson | 7.11.3 | noted |
| [16] | R4-2504183 | On Ku-band general requirements | LG Electronics Finland | 7.11.3 | noted |
| [17] | R4-2504278 | Discussion on the asymmetric channel bandwidths support for NR NTN Ku band | CHTTL, SKY Perfect JSAT Corporation, Intelsat | 7.11.3 | noted |
| [18] | R4-2504280 | Further discussion on the remaining system parameters for NR NTN Ku band with the inclusion of 15 kHz SCS | CHTTL | 7.11.3 | noted |
| [19] | R4-2504427 | Draft CR for channel arrangement | Eutelsat Group | 7.11.3 | not pursued |
| [20] | R4-2504473 | NR NTN system parameters for Ku band | Nokia | 7.11.3 | noted |
| [21] | R4-2503335 | Requirements for VSAT on Ku band(s) | Xiaomi | 7.11.4 | noted |
| [22] | R4-2503698 | Discussion on VSAT requirements for NR NTN Ku band | Huawei, HiSilicon | 7.11.4 | noted |
| [23] | R4-2503741 | Discussion on UE RF requirements for NTN Ku band | ZTE Corporation, Sanechips | 7.11.4 | noted |
| [24] | R4-2503942 | NTN Ku-band - NTN VSAT RF requirements | Ericsson | 7.11.4 | noted |
| [25] | R4-2503944 | Draft CR to 38101-5 - Needed updates to support NTN VSAT in FR1-NTN | Ericsson | 7.11.4 | postponed |
| [26] | R4-2504212 | On Ku-band NTN VSAT requirements | LG Electronics Finland | 7.11.4 | noted |
| [27] | R4-2504279 | Discussion on NTN Ku band UE RF requirements | MediaTek (Wuhan) Inc. | 7.11.4 | noted |
| [28] | R4-2504281 | Discussion on the minimum peak EIRP, EIS and the maximum input level requirement for the Rel.19 NTN Ku band VSAT | CHTTL | 7.11.4 | noted |
| [29] | R4-2503456 | Further discussion on SAN RF core requirements for Ku-band for NR NTN | CATT | 7.11.5 | noted |
| [30] | R4-2503742 | Discussion on SAN RF requirements for NTN Ku band | ZTE Corporation, Sanechips | 7.11.5 | noted |
| [31] | R4-2503743 | DraftCR to TS38.108 Add 10.4 and 10.5 requirements for NTN Ku bands | ZTE Corporation, Sanechips | 7.11.5 | postponed |
| [32] | R4-2503943 | NTN Ku-band - SAN requirements | Ericsson | 7.11.5 | noted |
| [33] | R4-2503945 | Draft CR to 38108 - Needed updates to add a new SAN type in FR1-NTN | Ericsson | 7.11.5 | postponed |
| [34] | R4-2504474 | Discussion on NR NTN SAN requirement for Ku band | Nokia | 7.11.5 | noted |
| [35] | R4-2503129 | Discussion on RRM requirements for Ku bands for NR NTN | MediaTek inc. | 7.11.6 | noted |
| [36] | R4-2503139 | Draft CR to 38.133 to modify uplink timing requirements to enable mobile VSAT served by NGSO | Eutelsat Group, Thales, European Space Agency, Sharp, CHTTL, Airbus | 7.11.6 | postponed |
| [37] | R4-2503508 | Ku Band RRM updates required to support FR1-NTN and FR2-NTN numerology | Eutelsat Group | 7.11.6 | noted |
| [38] | R4-2503706 | Discussion on RRM requirements for Ku-band NTN | LG Electronics Inc. | 7.11.6 | noted |
| [39] | R4-2503794 | Discussion on Ku bands for NR NTN | ZTECorporation,Sanechips | 7.11.6 | noted |
| [40] | R4-2503906 | Discussion on RRM requirements for NTN in Ku band | Huawei, HiSilicon | 7.11.6 | noted |
| [41] | R4-2504449 | Discussing on the adoption of VSAT Requirements for FR1-NTN | Nokia | 7.11.6 | noted |
| [42] | R4-2504532 | RRM requirements for VSAT UE in Ku band | Qualcomm Incorporated | 7.11.6 | noted |
| [43] | R4-2503616 | Topic summary for [114bis][206] NR_NTN_Ku_bands | Moderator (Apple) | 7.11.7 | noted |
| [44] | R4-2504637 | Topic summary for [114bis][312] NR_NTN_Ku_Band_General | Moderator (Eutelsat) | 7.11.7 | noted |
| [45] | R4-2504638 | Topic summary for [114bis][313] NR_NTN_Ku_Band_Coexistence | Moderator (Intelsat) | 7.11.7 | noted |
| [46] | R4-2504639 | Topic summary for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF | Moderator (CHTTL) | 7.11.7 | noted |
| [47] | R4-2504720 | Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF | CHTTL | 7.11.7 | approved |
| [48] | R4-2504723 | Way Forward for [114bis][313] NR_NTN_Ku_Band_Coexistence | Intelsat | 7.11.7 | approved |
| [49] | R4-2504761 | Way Forward for [114bis][312] NR_NTN_Ku_Band_General | Eutelsat | 7.11.7 | approved |
| [50] | R4-2504897 | WF on RRM requirements for NR_NTN_Ku_bands | Apple | 7.11.7 | approved |

Contributions submitted to RAN4#115:

| Ref | Tdoc | Subject | Source | Agenda | Status |
| --- | --- | --- | --- | --- | --- |
| [51] | R4-2505559 | Topic summary for [115][210] NR_NTN_Ku_bands | Moderator (Apple) | 7.11.1 | noted |
| [52] | R4-2505737 | Running CR for 38.101-5 Ku bands | Eutelsat Group | 7.11.1 | endorsed |
| [53] | R4-2507844 | Topic summary for [115][312] NR_NTN_Ku_Band_General | Moderator (Eutelsat) | 7.11.1 | noted |
| [54] | R4-2507846 | Topic summary for [115][314] NR_NTN_Ku_Band_UE_SAN_RF | Moderator (CHTTL) | 7.11.1 | noted |
| [55] | R4-2508262 | WF on RRM requirements for NR_NTN_Ku_bands | Apple | 7.11.1 | approved |
| [56] | R4-2508630 | Ad-hoc meeting minutes for [115][314] NR_NTN_Ku_Band_UE_SAN_RF | CHTTL | 7.11.1 | noted |
| [57] | R4-2508640 | Way Forward for [115][312] NR_NTN_Ku_Band_General | Eutelsat | 7.11.1 | approved |
| [58] | R4-2508641 | Topic summary for [115][313] NR_NTN_Ku_Band_Coexistence | Moderator (Intelsat) | 7.11.1 | noted |
| [59] | R4-2508773 | Way Forward for [115][313] NR_NTN_Ku_Band_Coexistence | Intelsat | 7.11.1 | approved |
| [60] | R4-2508778 | Way Forward for [115][314] NR_NTN_Ku_Band_UE_SAN_RF | CHTTL | 7.11.1 | approved |
| [61] | R4-2506142 | NTN Ku-band - bands numbering | Ericsson | 7.11.2 | noted |
| [62] | R4-2505718 | Ku Band Co-existence: Performance of 45x45 and 78x78 Element Phase Array Antennas | Intelsat | 7.11.3 | noted |
| [63] | R4-2505747 | Coexistence simulation results between TN and NTN for Ku band | Qualcomm Incorporated | 7.11.3 | noted |
| [64] | R4-2505762 | Temporary calibration results and simulation results for the Ku-band NTN coexistence study | xiaomi | 7.11.3 | noted |
| [65] | R4-2506020 | Further discussion on coexistence evaluations for Ku-band for NR NTN | CATT | 7.11.3 | noted |
| [66] | R4-2506258 | Discussion on coexistence study for NR NTN Ku band | ZTE Corporation, Sanechips | 7.11.3 | noted |
| [67] | R4-2507169 | Discussion on Ku-band coexsitence studies | Samsung | 7.11.3 | noted |
| [68] | R4-2507672 | On co-existence simulation assumptions for NR NTN Ku band | Ericsson | 7.11.3 | noted |
| [69] | R4-2507673 | On co-existence simulation results for NR NTN Ku band | Ericsson | 7.11.3 | noted |
| [70] | R4-2508774 | Ku band co-existence simulation assumptions (running document) | Ericsson | 7.11.3 | approved |
| [71] | R4-2506021 | Further discussion on system parameters for Ku-band for NR NTN | CATT | 7.11.4 | noted |
| [72] | R4-2506143 | NTN Ku-band - system parameters aspect | Ericsson | 7.11.4 | noted |
| [73] | R4-2506259 | Discussion on system parameters for NR NTN Ku band | ZTE Corporation, Sanechips | 7.11.4 | noted |
| [74] | R4-2506869 | Draft CR to 38.101-5 on restriction of simultaneous operation of FR1-NTN and FR2-NTN numerology for Ku band | vivo | 7.11.4 | postponed |
| [75] | R4-2506931 | Discussion on Ku-band VSAT Variable Tx-Rx frequency | LG Electronics Finland | 7.11.4 | noted |
| [76] | R4-2507160 | Discussion on the remaining system parameters for NR NTN Ku band | CHTTL | 7.11.4 | noted |
| [77] | R4-2508771 | draft CR for updates to NTN Ku-band channel raster sections | CHTTL | 7.11.4 | endorsed |
| [78] | R4-2505535 | Discussion on VSAT requirements for NR NTN Ku band | Huawei, HiSilicon | 7.11.5 | noted |
| [79] | R4-2505536 | Draft CR for TS 38.101-5 to introduce ACS and blocking characteristics for Ku band | Huawei, HiSilicon | 7.11.5 | postponed |
| [80] | R4-2506144 | NTN Ku-band - NTN VSAT RF requirements | Ericsson | 7.11.5 | noted |
| [81] | R4-2506146 | Draft CR to 38101-5 - Off axis EIRP emissions density | Ericsson | 7.11.5 | postponed |
| [82] | R4-2506147 | Draft CR to 38101-5 - Receiver antenna off-axis performance | Ericsson | 7.11.5 | postponed |
| [83] | R4-2506260 | Discussion on UE RF requirements for NR NTN Ku band | ZTE Corporation, Sanechips | 7.11.5 | noted |
| [84] | R4-2506261 | DraftCR to TS38.101-5 Add 9.5.1, 9.5.2 and 9.7 requirements for NTN Ku bands | ZTE Corporation, CHTTL | 7.11.5 | postponed |
| [85] | R4-2506262 | DraftCR to TS38.101-5 Add 9.5.3 requirement for NTN Ku bands | ZTE Corporation, Sanechips | 7.11.5 | postponed |
| [86] | R4-2506290 | Consideration of beam angle on beam gain degradation for minimum EIRP discussion | Sharp | 7.11.5 | noted |
| [87] | R4-2506529 | Discussion on NTN Ku band VSAT RF requirements | MediaTek (Wuhan) Inc. | 7.11.5 | noted |
| [88] | R4-2506585 | Discussion on UE RF requirements for NR NTN Ku band | SKY Perfect JSAT Corporation | 7.11.5 | noted |
| [89] | R4-2506662 | Draft CR to TS 38.101-5 adding NTN Ku band VSAT maximum output power requirement | SKY Perfect JSAT Corporation, CHTTL | 7.11.5 | postponed |
| [90] | R4-2507090 | On Ku-band VSAT reference sensitivity and maximum input level | LG Electronics Finland | 7.11.5 | noted |
| [91] | R4-2507099 | Draft CR for Ku-band VSAT OTA Reference Sensitivity and Maximum Input Level | LG Electronics Finland | 7.11.5 | postponed |
| [92] | R4-2507147 | Discussion on the regulations related VSAT requirements for the uplink 13.75~14.0 GHz band | CHTTL, SGS Wireless | 7.11.5 | noted |
| [93] | R4-2507172 | Further discussion on the EIRP, EIS and the maximum input level requirement for the Rel.19 NTN Ku band VSAT | CHTTL | 7.11.5 | noted |
| [94] | R4-2507716 | Discussion on NR NTN UE requirement for Ku bands | Nokia | 7.11.5 | noted |
| [95] | R4-2507873 | On VSAT UE RF requirements for Ku-band | THALES | 7.11.5 | noted |
| [96] | R4-2506022 | Further discussion on SAN RF core requirements for Ku-band for NR NTN | CATT | 7.11.6 | noted |
| [97] | R4-2506145 | NTN Ku-band - SAN requirements | Ericsson | 7.11.6 | noted |
| [98] | R4-2506263 | Discussion on SAN RF requirements for NR NTN Ku band | ZTE Corporation, Sanechips | 7.11.6 | noted |
| [99] | R4-2506264 | DraftCR to TS38.108 Add 10.4 and 10.5 requirements for NTN Ku bands | ZTE Corporation, Sanechips | 7.11.6 | postponed |
| [100] | R4-2506293 | Discussion on SAN EIS requirements for NR NTN Ku band | LG Electronics Inc. | 7.11.6 | noted |
| [101] | R4-2507170 | Discussion on Ku-band SAN RF requirements | Samsung | 7.11.6 | noted |
| [102] | R4-2507171 | Draft CR to TS 38.108 to add 10.6 and 10.9 requirements for Ku-band NTN | Samsung | 7.11.6 | postponed |
| [103] | R4-2507717 | Discussion on NR NTN SAN requirement for Ku bands | Nokia | 7.11.7 | noted |
| [104] | R4-2505851 | RRM performance requirements for UEs with directional antenna | Eutelsat Group | 7.11.7 | noted |
| [105] | R4-2506237 | Discussion on RRM requirements for Ku-band NTN | LG Electronics Inc. | 7.11.7 | noted |
| [106] | R4-2506352 | Discussion on RRM requirements for Ku bands for NR NTN | Ericsson | 7.11.7 | noted |
| [107] | R4-2506353 | (NR_NTN_Ku_bands-Core) Draft CR on signalling characteristics for NR_NTN_Ku_Band | Ericsson | 7.11.7 | merged |
| [108] | R4-2506480 | Discussion on RRM requirements for Ku bands for NR NTN | MediaTek inc. | 7.11.7 | noted |
| [109] | R4-2506783 | Discussion on RRM requirements for NTN in Ku band | Huawei, HiSilicon | 7.11.7 | noted |
| [110] | R4-2506784 | draftCR on applicability of requirements for Ku band NTN | Huawei, HiSilicon | 7.11.7 | endorsed |
| [111] | R4-2507190 | Discussion on RRM requirements for Ku band NTN | ZTE Corporation, Sanechips | 7.11.7 | noted |
| [112] | R4-2507200 | (NR_NTN_Ku_bands-Core) Definitions,symbols and abbreviations | ZTE Corporation, Sanechips | 7.11.7 | postponed |
| [113] | R4-2507201 | (NR_NTN_Ku_bands-Core) RRC IDLE state mobility | ZTE Corporation, Sanechips | 7.11.7 | endorsed |
| [114] | R4-2507202 | (NR_NTN_Ku_bands-Core) Signalling characteristics | ZTE Corporation, Sanechips | 7.11.7 | postponed |
| [115] | R4-2507360 | Requirements for NTN UEs on Ku band operation | Nokia | 7.11.7 | noted |
| [116] | R4-2507810 | RRM requirements for VSAT UE in Ku band | Qualcomm Incorporated | 7.11.7 | noted |
| [117] | R4-2508383 | DraftCR on uplink timing requirements for mobile VSAT served by NGSO | MediaTek inc. | 7.11.7 | endorsed |
| [118] | R4-2508387 | DraftCR to TS 38.133 for including measurement procedures for VSAT UEs operating in KU band | Nokia | 7.11.7 | endorsed |

12.08.2024		minor adaptations for RAN #105

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v01	13.11.2009		First version of the template