3GPP TSG RAN meeting #108								                      RP-251287

Prague, Czech Republic, 9-13 June, 2025

Status Report to TSG

Agenda item:			9.2.7

| WI / SI Name | Study on NR FR1 DL fragmented carriers |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_FR1_DL_Frag_Carrier |  |  |  |  |
| Unique ID | 1040101 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-243002 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: Jun. 2025 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 100% | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Primary: Huanren (Henry) Fu Secondary: |
|  | Company | MediaTek Inc. |
|  | Email | huanren.fu@mediatek.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#115

TPs for TR38.755 are agreed:

R4-2507938	TP for triggering condition option1 in FS_NR_DL_Frag_Carrier

R4-2507939	TP for RRM aspect of FS_NR_DL_Frag_Carrier

R4-2508113	TP for TR signaling indication for Option 2 new notations

R4-2508114	TP for TR indication of fragmented CA alternative 1

R4-2506065	TP to TR 38.755 on Clause 6

R4-2507950	TP to TR 38.755 on fallback behaviour

R4-2507951	TP for TR38.755: Summary for NR FR1 DL fragment carriers study

R4-2507948	TPs to TR 38.755 with aspects of Fragmented CA

R4-2507949	TP to TR 38.755 on UE indication to the network

R4-2507947	TP for TR Discussion on signalling aspects

R4-2505783	TP to TR 38.755 on RF requirements evaluation

R4-2507953	TP to TR 38.755: on UE indication of supporting FR1 fragmented carriers

R4-2506270	TP for TR 38.755 on RF requirements evaluation for fragmented carriers from company S

R4-2506376	TP to TR 38.755 on UE RF requirements evaluation for fragmented carriers

R4-2506378	TP to TR38.755 on UE RF requirements impact for fragmented carriers

R4-2507952	TP for TR Summary on the evaluation results and further discussion on the one Rx RF chain mode applicability

R4-2507477	TP for TR 38.755 on RF performance

RAN4#114-bis

WF was approved in R4-2505109

Triggering condition to enable ”One Rx RF chain”

RAN4 agree there are two aspects regarding the triggering condition enable/disable the one Rx RF chain mode:

- Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed

- Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified

- Note: detailed conditions to enable/disable can be further discussed and decided separately

How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers

- Consider the following alternatives

  - Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication

    - Alternative 1 has no modification of CA configurations in RAN4

  - Alternative 1a: use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers.

    - Alternative 1a has no modification of CA configurations in RAN4

  - Alternative 2: use the new notation for CA configurations with single Rx RF Chain

  - Alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

Discussion on semi-statically switch hardware resources procedure and fallback considerations

- Further discussions on

  - CA fallback with/without UE architecture switching

    - Consider the fall back

      - from high order CA to lower order CA

      - from CA to single carrier mode

    - Decided by network

  - UE architecture switching:

    - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA

    - FFS on whether it is decided by UE itself or decided by network

      - FFS on whether the additional indication from UE side is needed

Discussion on applicability for PCell and SCell

A more detailed proposal based on the tentative agreement in online discussion:

- To address the potential impacts on the DL performance for PCell, RAN4 agrees to further discuss the following aspects:

  - To limit the use of this feature to CA configurations where the impact is limited

  - In the CA configurations where there is limited impact, how to further mitigate the impact by means including restricting the PCC and SCC frequency location, or restricting UE locations (e.g., the feature is not enabled when the UE is located at cell edge where wanted signal level is close to receiver sensitivity)

RAN4#114

WF was approved in R4-2420519

Discussion on maximum input level for fragmented carriers

Maximum power level including all RF signals and interference at the UE input does not exceed -25 dBm.

Discussion of impacts on DL performance for UE enabling one Rx RF chain mode

- Relaxation on REFSENS for both PCC and SCC will be evaluated and captured in the study outcome report.

  - DL performance degradation can be considered based on configurations

  - Configurations leading to no relaxation are not precluded pending on the evaluation results. The exact values and applicable conditions shall be decided in the WI phase if there is corresponding WI

- Maintain the table format agreed in previous RAN4 meeting. Detailed evaluation assumptions such as imbalanced PRx/DRx noise level and other assumptions are encouraged to be provided from companies evaluation TPs

Discussion on in-gap interference requirements

- For image of in-gap interference not overlap with wanted CC: Evaluate whether relaxation on ACS/IBB is needed  based on evaluated ΔRIBNC on PCC and SCC

- For image of in-gap interference(ACS/IBB) overlap with wanted CC: companies are encouraged to provide the impact of interference to wanted signal power ratio of [15-26]dB to the ΔRIBNC for PCC and SCC for

Discussion on DL fragmented carriers in TDD bands

- Option 2: Separate LOs are assumed for UL and DL for TDD band

TP for TR38.755: R4-2502925, R4-2502949 were approved

RAN4#113

WF was approved in R4-2420519

Discussion on baseband assumptions

Agreement:

- Consider only same SCS for both CCs for one Rx RF chain architecture in this SI

  - 15KHz SCS is used as the starting point

- Single FFT or separate FFT can be left to UE implementation, and however the condition of single FFT compared to separate FFT can be captured in the TR.

Triggering condition to enable ”One RF Rx chain”

RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled

- Further study whether the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled

  - Other determining factors are not precluded to be studied

- Study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2

Discussion of impacts on DL performance for UE enabling fragmented carrier mode

- Companies are encouraged to bring analysis with their own assumptions following the table format

- A separate table would be needed for Tx leakage + blocker and individual CA channel bandwidth combination in Table 7.3A.2.2-1 of TS 38.101-1

Table 1. Link budget of Self-interference for 1st configuration on band n3

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise (11 bit): -68.0dBFs (6.02N+1.76) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to 0dBFs (PAPR is not considered) | Tx leakage mapping to -12dBFS (PAPR is considered) |
| PA out | 27 | - | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - | - |
| Rx Antenna input | - | -97 | -92.3 | - | - | - |
| FE loss | - | 4 | 4 | - | - | - |
| Duplexer output | -23 | -101 | -96.3 | - | - | - |
| LPF attenuation | 13.4 | 1 | 1 | - | - | - |
| LPF output (dBm) | -36.4 | -102 | -97.3 | - | - | - |
| Noise floor for basic REFSENS | - | - | - | -101 | - | - |
| NF increase due to AGC adjustment | - | - | - | 10 | - | - |
| Noise floor | - | - | - | -91 | - | - |
| In-band noise (Derived based on MSD and SNR = -1dB) | - | - | -96.3 | - | - | - |
| ADC output (dBm) | - | - | - | - | -104.4 | -92.4 |
| SNR after ADC (dB) | - | - | - | - | -11.2(PCC) | -13.4(PCC) |
|  |  |  |  |  | -7.6(SCC) | -9.4(SCC) |

Discussion on in-gap requirements

RAN4 evaluate performance impact on the cases:

- image of in-gap interference overlaps with wanted carrier

- For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC

- image of in-gap blocking interference does not overlap with wanted carrier

Signaling related

WF:

Companies are encouraged to provide analysis with pros and cons on preferred signalling solutions so that they can be discussed in next meeting.

Discussion on evaluation priority of example bands

WF:

It would be up to companies’ preference and contribution driven on the example bands evaluation. The study needs to cover evaluation on both FDD and TDD bands and focus on general requirements. Band specific requirements can be left for WI phase.

RAN4#112bis

WF was approved in R4-2417201

Study scope clarification

Agreement:

    - Frequency span ≤ 100 MHz containing two non-contiguous CCs means the frequency separation between lower edge of lowest CC and upper edge of highest CC in downlink of a frequency band is less than or equal to 100MHz as illustrated below:

    - Any update of scope needs be decided by RAN

Discussion on assumptions on test configurations for evaluation

Agreement:

(a)	RAN4 agree on the general assumptions that are same for legacy NR RF performance evaluation:

o	UE self-band uplink Tx re-use same assumptions for NR PC3 PA i.e., ACLR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB

o	Reuse the Rx chain image rejection, which is 25dB, as the starting point.

Agreement:

- The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, for ΔRIBNC, REFSENS, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example combos

Agreement:

- Priority to one uplink carrier

Agreement:

  - Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc

  - The example bands should be in the scope of existing intra-band non-contiguous CA operating bands

  - No relaxation on any Tx RF requirements should be assumed in this study item

  - Both PC3 and PC2 should be considered

    - Prioritize PC3

  - The minimum Wgap for ACS, narrow band blocking and in-band blocking verification should meet:

    - Wgap ≥ 2∙|FInterferer (offset),j|  – BWChannel(j)

    - It is the condition to apply the requirements

  - Both the symmetric CC scenario and the asymmetric CC scenario should be considered in the FC SI, and companies are encouraged, but not limited, to study the CC scenarios shared by the operators

  - [Adjusted requirements for fragmented carriers, if any, shall be separated from the existing requirements]

  - RAN4 shall not re-discuss the power imbalance assumption of the wanted carriers further in relation to the SI on fragmented carriers

Example band consideration for evaluation

Agreement:

  - RAN4 doesn’t consider 3MHz channel bandwidth for fragmented carriers

  - The example band(s) should already have NC DLCA specified

    - Prioritize the frequency bands of ≤2700MHz

  - Remove n39 and include n26 in the example bands: Consider n2/n25, n3, n7, n66, n41, n39, n26 as the example band

Merge issue 2-0/2-1 discussion:

Agreement:

The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter

  - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers

  - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823)

Study on power spectral density difference between carriers of co-located adjacent channel operators

Agreement:

To evaluate acceptable [PSD] difference from UE side between wanted signal and un-wanted signal based on the deployment where BSs from different operators are collocated but their radio units (RUs) are not shared.

  - The [PSD] difference means the difference between unwanted signal and the wanted carrier with the lower PSD

  - Start evaluation with wanted signal level currently existing ACS requirements as baseline and to see the performance difference

RAN4#112

Study Item:

- WF was approved in R4-2414457.

- The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain

  - The two non-contiguous CCs is still under the current DL non-contiguous CA framework

    - Each fragment is treated as an individual CC

  - The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz.

  - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands

    - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands.

      - Provide the details related to gap between fragmented DL blocks]

- Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements

  - Proposal 1: put off the signalling related discussions until the UE performance aspects are clear

2.4.2	Remaining Open issues

RAN4#115

Complete All objectives in SID. There are no open issues in SID

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#115

- R4-2505783	TP to TR 38.755 on RF requirements evaluation	Xiaomi

- R4-2505784	Methods for reducing the number of UE Rx chains	Xiaomi

- R4-2505785	Impacts on UE RF requirements and DL performance	Xiaomi

- R4-2505885	On methods for reducing the number of UE Rx chains for fragmented carriers	Apple

- R4-2505886	On UE RF requirements for fragmented carriers	Apple

- R4-2506065	TP to TR 38.755 on Clause 6	CATT

- R4-2506165	UE architecture switching process	Qualcomm Incorporated

- R4-2506270	TP for TR 38.755 on RF requirements evaluation for fragmented carriers from company S	Samsung

- R4-2506376	TP to TR 38.755 on UE RF requirements evaluation for fragmented carriers	Spreadtrum,UNISOC

- R4-2506377	View the methods for reducing the number of UE Rx chain	ZTE Corporation, Sanechips

- R4-2506378	TP to TR38.755 on UE RF requirements impact for fragmented carriers	ZTE Corporation, Sanechips

- R4-2506543	Discussion on methods for reducing the number of UE RX chains	Spreadtrum,UNISOC

- R4-2506835	TP to TR 38.755 on fallback behaviour	vivo

- R4-2506836	Discussion on methods for reducing the number of UE RX chains	vivo

- R4-2506984	draft update TR38755-030	MediaTek Inc.

- R4-2506986	TP for TR38.755: Summary for NR FR1 DL fragment carriers study	MediaTek Inc.

- R4-2506994	TP for TR 38.755 on signalling aspects	MediaTek Inc.

- R4-2506996	TP for TR 38.755 on summary on the evaluation results and further discussion on the one Rx RF chain mode applicability	MediaTek Inc.

- R4-2507004	Discussion on methods for reducing the number of UE Rx chains for fragmented carriers	Google Korea LLC

- R4-2507219	TP to TR 38.755: on UE indication of supporting FR1 fragmented carriers	CHTTL

- R4-2507264	Discussion on methods for reducing the number of UE Rx RF chains	Nokia

- R4-2507320	Discussion on reducing the number of UE Rx chains	OPPO

- R4-2507321	Discussion on impacts on UE RF requirements	OPPO

- R4-2507375	Discussion on RF requirements for Fragmented CA	Nokia

- R4-2507376	TPs to TR 38.755 with aspects of Fragmented CA	Nokia

- R4-2507391	TP to TR 38.755 on UE indication to the network	Ericsson

- R4-2507392	Discussion on signaling aspects of fragmented carriers	Ericsson

- R4-2507393	Discusson on impact on UE RF requirement of fragmented carriers	Ericsson

- R4-2507475	TP for TR 38.755 on UE's fallback behavior	Huawei, HiSilicon

- R4-2507476	TP for TR 38.755 on triggering condition	Huawei, HiSilicon

- R4-2507477	TP for TR 38.755 on RF performance	Huawei, HiSilicon

- R4-2507563	Topic summary for [115][124] FS_NR_DL_Frag_Carrier	Moderator (Mediatek)

- R4-2507937	AH meeting summary for [115][124] FS_NR_DL_Frag_Carrier	MediaTek

- R4-2507938	TP for triggering condition option1 in FS_NR_DL_Frag_Carrier	OPPO

- R4-2507939	TP for RRM aspect of FS_NR_DL_Frag_Carrier	Nokia

- R4-2507947	TP for TR 38.755 on signalling aspects	MediaTek Inc.

- R4-2507948	TPs to TR 38.755 with aspects of Fragmented CA	Nokia

- R4-2507949	TP to TR 38.755 on UE indication to the network	Ericsson, Xiaomi

- R4-2507950	TP to TR 38.755 on fallback behaviour	Huawei, vivo, OPPO

- R4-2507951	TP for TR38.755: Summary for NR FR1 DL fragment carriers study	MediaTek Inc.

- R4-2507952	TP for TR 38.755 on summary on the evaluation results and further discussion on the one Rx RF chain mode applicability	MediaTek Inc.

- R4-2507953	TP to TR 38.755: on UE indication of supporting FR1 fragmented carriers	CHTTL

- R4-2507956	TP for TR signaling indication for Option 2 new notations	vivo

- R4-2507971	TP for TR indication of fragmented CA alternative 1	vivo

RAN4#114-bis

- R4-2503337	Methods for reducing the number of UE Rx chains	Xiaomi

- R4-2503338	Impacts on UE RF requirements and DL performance	Xiaomi

- R4-2503366	On fallback behavior and signalling aspect for Fragmented carriers	Samsung

- R4-2503479	TP to TR 38.755 on Clause 5	CATT

- R4-2503556	Procedures for fragmented carriers	Qualcomm Incorporated

- R4-2503557	UE performance in one RX mode	Qualcomm Incorporated

- R4-2503558	TP for TR38.755: Company C evaluation results	Qualcomm Incorporated

- R4-2503835	Discussion on methods for reducing the number of UE RX chains	vivo

- R4-2503836	Impacts on UE RF requirements and DL performance	vivo

- R4-2503963	Discussion the methods for reducing the number of UE Rx chain	ZTE Corporation, Sanechips

- R4-2503964	Discussion on the RF requirement for fragmented carrier	ZTE Corporation, Sanechips

- R4-2504036	Discussion on UE capability indication for single Rx chain	Ericsson

- R4-2504037	Discussion on the DL PCell REFSENS degradation of fragmented carriers	Ericsson

- R4-2504102	Discussion on methods for reducing the number of UE Rx chains for fragmented carriers	Google

- R4-2504105	draft update TR 38.755 FR1 Fragmented carriers_rm	MediaTek Inc.

- R4-2504128	Discussion on RF requirements for Fragmented CA	Nokia

- R4-2504129	General aspects of Fragmented CA	Nokia

- R4-2504153	Discussion on reducing the number of UE Rx chains	OPPO

- R4-2504154	Discussion on impacts on UE RF requirements	OPPO

- R4-2504173	Discussion on methods for reducing the number of UE Rx chains	Nokia

- R4-2504179	Discussion on methods for reducing the number of UE RX chains	Spreadtrum,UNISOC

- R4-2504211	draft update TR 38.755 FR1 Fragmented carriers	MediaTek Inc.

- R4-2504213	Discussion on one Rx RF chain mode signalling aspects	MediaTek Inc.

- R4-2504214	Discussion of impacts on UE RF requirements and DL performance	MediaTek Inc.

- R4-2504239	Discussion of impacts on UE RF requirements and DL performance	Spreadtrum,UNISOC

- R4-2504260	Views on the usage of the FR1 fragmented carriers	CHTTL

- R4-2504283	On signalling aspect of fragmented carriers	Huawei, HiSilicon

- R4-2504284	On RF requirements of fragmented carriers	Huawei, HiSilicon

- R4-2504426	Link budget calculation results for fragmented carrier	Samsung

- R4-2504450	On methods for reducing the number of UE Rx chains for fragmented carriers	Apple

- R4-2504451	On UE RF requirements for fragmented carriers	Apple

- R4-2504452	TP on UE RF requirements impact for fragmented carriers	Apple

- R4-2504453	Draft LS on signaling aspects for fragmented carriers	Apple

RAN4#114

- R4-2500039	Comments on Fragmented Carrier Signalling	Rogers Communications Canada, CableLabs

- R4-2500118	Discussion on UE RF requirement impact for DL fragmented carriers	Murata Manufacturing Co Ltd.

- R4-2500219	In-Gap Blocking for Fragmented Carrier	Murata Manufacturing Co Ltd.

- R4-2500243	On methods for reducing the number of UE Rx chains for fragmented carriers	Apple

- R4-2500244	On UE RF requirements for fragmented carriers	Apple

- R4-2500613	UE mechanism to trigger one RF RX chain mode	Qualcomm Incorporated

- R4-2500650	Methods for reducing the number of UE Rx chains	Xiaomi

- R4-2500676	Topic summary for [114][124] FS_NR_DL_Frag_Carrier	Moderator(Mediatek)

- R4-2500726	Discussion on methods for reducing the number of UE RX chains	vivo

- R4-2500727	Impacts on UE RF requirements and DL performance	vivo

- R4-2500808	Discussion on general aspects for fragmented carriers	Samsung

- R4-2500809	Link budget calculation for DL performance with shared Rx RF chain	Samsung

- R4-2500941	Discussion the methods for reducing the number of UE Rx chain	ZTE Corporation, Sanechips

- R4-2500942	Discussion on the RF requirement for fragmented carrier	ZTE Corporation, Sanechips

- R4-2501032	Discussion on fragmented DL carriers	CATT

- R4-2501150	Discussion on remaining signalling aspects for DL fragmented carriers study	MediaTek Inc.

- R4-2501189	Discussion on methods for reducing the number of UE Rx chains for fragmented carriers	Google

- R4-2501196	Disscussion on methods for reducing the number of UE RX chains	Spreadtrum,UNISOC

- R4-2501295	Views on enabling condition on the FR1 fragmented carriers	CHTTL

- R4-2501472	Discussion on reducing the number of UE Rx chains	OPPO

- R4-2501473	Discussion on impacts on UE RF requirements	OPPO

- R4-2501662	Discussion on requirements for Fragmented CA	Nokia

- R4-2501802	On signalling aspect of fragmented carriers	Huawei, HiSilicon

- R4-2501803	On RF requirements of fragmented carriers	Huawei, HiSilicon

- R4-2501896	Discussion on methods for reducing the number of UE Rx chains	Nokia

- R4-2501936	Discussion on UE signaling of Fragmented Carriers	Ericsson

- R4-2501937	Discusson on impact on UE RF requirement of fragmented carriers	Ericsson

- R4-2502872	Ad hoc minutes on fragmented DL	MediaTek

- R4-2502873	WF on fragmented DL	MediaTek

- R4-2502925	TP for TR38.755	MediaTek Inc., Nokia, [Apple, Ericsson, Huawei, Skyworks,…]

- R4-2502949	Impacts on UE RF requirements and DL performance impacts	MediaTek Inc.

RAN4#113

- R4-2418089	NC DL CA vs single CC ACS	Qualcomm Incorporated

- R4-2418146	Topic summary for [113][124] FS_NR_DL_Frag_Carrier	Moderator (Mediatek)

- R4-2418168	Discussion on methods for reducing the number of UE RX chains	vivo

- R4-2418169	Impacts on UE RF requirements and DL performance	vivo

- R4-2418301	Discussion on fragmented carriers	Samsung

- R4-2418531	On methods for reducing the number of UE Rx chains for fragmented carriers	Apple

- R4-2418532	On UE RF requirements for fragmented carriers	Apple

- R4-2418615	Discussion on methods for reducing the number of UE RX chains	Spreadtrum Communications

- R4-2418695	View on Fragmented carrier RF requirements	ZTE Corporation, Sanechips

- R4-2418717	On methods for reducing the number of UE Rx chain	Huawei, HiSilicon

- R4-2418718	On RF requirements of fragmented carriers	Huawei, HiSilicon

- R4-2418841	Discussion on methods for reducing the number of UE Rx chains for fragmented carriers	Google Ireland Limited

- R4-2418854	Discussion on remaining open issues for DL fragmented carriers study	MediaTek Inc.

- R4-2418862	Discussion of impacts on UE RF requirements and DL performance impacts	MediaTek Inc.

- R4-2419013	Discussion on reducing the number of UE Rx chains	OPPO

- R4-2419014	Discussion on impacts on UE RF requirements	OPPO

- R4-2419453	Discussion on Fragmented Carrier Study Item Description	Nokia

- R4-2419454	Discussion on Fragmented Carrier reduction of Rx RF chains	Nokia

- R4-2419455	Discussion on Fragmented Carrier impacts on UE RF requirements	Nokia

- R4-2419464	In-band In-gap Blocking for Fragmented Carriers	Murata Manufacturing Co Ltd.

- R4-2419551	Discussion on UE Rx chains of Fragmented Carriers	Ericsson

- R4-2419552	Discusson on impact on UE RF requirement of fragmented carriers	Ericsson

- R4-2419618	Fragmented carrier in-gap and adjacent assumptions to assess PSD imbalance	Skyworks Solutions Inc.

- R4-2420337	WF on fragmented DL study	MediaTek

- R4-2420512	Ad hoc meeting minute on NR FR1 DL fragmented carriers study	MediaTek

- R4-2420519	WF on NR FR1 DL fragmented carriers study	MediaTek

RAN4#112bis

- R4-2414946	Views on UE architecture and PSD difference for fragmented carriers	Samsung, TELUS, Bell mobility

- R4-2414947	Views on UE RF requirements for fragmented carriers	Samsung, TELUS, Bell mobility

- R4-2415037	Discussion on reducing the number of UE Rx chains	OPPO

- R4-2415038	Discussion on impacts on UE RF requirements and DL performance	OPPO

- R4-2415125	Scope clarification on DL fragmented carriers	CATT

- R4-2415126	Discussion on methods for reducing the number of UE Rx chains	CATT

- R4-2415127	Discussion on impacts on UE RF requirements and DL performance	CATT

- R4-2415226	Topic summary for [112bis][122] FS_NR_DL_Frag_Carrier	Moderator(Mediatek)

- R4-2415360	Discussion on UE RF requirement impact for DL fragmented carriers	Murata Manufacturing Co Ltd.

- R4-2415390	Discussion on methods for reducing the number of UE Rx RF chains for Fragmented Carriers	Nokia

- R4-2415391	Discussion on impacts on UE RF requirements and DL performance for Fragmented Carriers	Nokia

- R4-2415510	On methods for reducing the number of UE Rx chains for fragmented carriers	Apple

- R4-2415511	On UE RF requirements for fragmented carriers	Apple

- R4-2415533	On in-gap and adjacent blockers level for fragmented spectrum reception in a single Rx path	Skyworks Solutions Inc.

- R4-2415592	Discussion on methods for reducing the number of UE RX chains	Spreadtrum Communications

- R4-2415758	Discussion on methods for reducing the number of UE RX chains	vivo

- R4-2415759	Discussion on impacts on UE RF requirements and DL performance	vivo

- R4-2415804	On methods for reducing the number of UE Rx chain	Huawei, HiSilicon

- R4-2415805	On RF requirements of fragmented carriers	Huawei, HiSilicon

- R4-2415910	View on methods for reducing the number of UE Rx chains	ZTE Corporation, Sanechips

- R4-2415911	View on Fragmented carrier general part and RF requirements	ZTE Corporation, Sanechips

- R4-2415992	Discussion on UE Rx chains of Fragmented Carriers	Ericsson

- R4-2415993	Discusson on impact on UE RF requirement of fragmented carriers	Ericsson

- R4-2416040	Discussion on the scenarios and the general aspects on the FR1 fragmented carriers study	CHTTL

- R4-2416062	Views on the RF impact on the FR1 fragmented carriers	CHTTL

- R4-2416132	Update of TR for Study on NR FR1 DL Fragmented Carriers	MediaTek Inc.

- R4-2416138	Discussion on UE Rx chain architecture and network assumption	MediaTek Inc.

- R4-2416164	Discussion of impacts on UE RF requirements and DL performance impacts	MediaTek Inc., Telstra

- R4-2416287	Views on UE architecture and PSD difference for fragmented carriers_rev	Samsung, TELUS, Bell mobility

- R4-2417201	WF on fragmented DL	MediaTek

- R4-2417202	Ad hoc minutes on fragemented DL	MediaTek

RAN4#112

- R4-2412827	Topic summary for [112][125] FS_NR_DL_Frag_Carrier	Moderator(MediaTek Inc.)

- R4-2414457	WF on fragmented DL	MediaTek Inc.

- R4-2411554	Work plan for fragment carriers study	MediaTek Inc.

- R4-2411553	TR 38.755 skeleton FR1 Fragmented carriers 	MediaTek Inc.

- R4-2411310	Views on UE RF architecture and NW deployment assumption for fragmented carriers	Samsung, TELUS, Bell mobility

- R4-2411404	On methods for reducing the number of UE Rx chains for fragmented carriers	Apple

- R4-2411555	Discussion on methods for reducing the number of UE Rx chains	MediaTek Inc.

- R4-2411691	On general aspects of fragmented carriers	Huawei, HiSilicon

- R4-2412087	Discussion on methods for reducing the number of UE RX chains	vivo

- R4-2412274	Discussion on methods for reducing the number of UE RX chains	Spreadtrum Communications

- R4-2412278	Operator’s initial views on FR1 fragmented carriers study	CHTTL

- R4-2413031	On architecture options for fragmented spectrum reception	Skyworks Solutions Inc.

- R4-2413270	Discussion on UE Rx chains of Fragmented Carriers	Ericsson

- R4-2413339	Discussion on methods for reducing the number of UE Rx chains for Fragmented Carriers Nokia

- R4-2411114	Discussion on impacts on UE RF requirements and DL performance for fragmented carriers	CATT

- R4-2411311	Views on UE RF requirements for fragmented carriers	Samsung, TELUS, Bell mobility

- R4-2411405	On UE RF requirements for fragmented carriers	Apple

- R4-2411556	Discussion on UE RF requirements and DL performance impacts	MediaTek Inc.

- R4-2411692	On RF requirements of fragmented carriers	Huawei, HiSilicon

- R4-2411886	View on Fragmented carrier	ZTE Corporation, Sanechips

- R4-2412088	Discussion on impacts on UE RF requirements and DL performance	vivo

- R4-2413271	Discusson on impact on UE RF requirement of fragmented carriers	Ericsson

- R4-2413340	Discussion on impacts on UE RF requirements and DL performance for Fragmented Carriers	Nokia

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template