3GPP TSG RAN meeting #108										RP-251316

Prague, Czechia, June 9-13, 2025

## Status Report to TSG

Agenda item:			9.3.4.11

| WI / SI Name | Low NR band carrier aggregation via switching |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_LBCA_Sw |  |  |  |  |
| Unique ID | 1060082 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-250247 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 60% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Ivo Maljevic |
|  | Company | TELUS |
|  | Email | ivo.maljevic@telus.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #120bis

General aspects

-	The feature lead summaries were prepared in [16], [17], [18], [22]

Agreements

-	The following agreements were achieved:

Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases, - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell. - SCS 15KHz on both carriers. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2. - The pattern starts from the beginning of SFN 0 of PCell. - Restriction of maximum X switch(es) within Y slot(s). - FFS: detailed value and relation of X and Y. - FFS: other restriction. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, down-select one of the following solutions: - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier. - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier. - Alt 3: Switching gap is always assumed at the SDL carrier. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed). - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL. - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA. - FFS: whether it has specification impact.

RAN1 #121

General aspects

-	The feature lead summaries were prepared in [38], [39], [40]

-	The company proposals on UE capabilities were summarized in [58], [59]

Agreements

-	The following agreements were achieved:

Agreement For Rel-19 low NR band carrier aggregation via switching, support configuration of only one switching pattern to a UE based on UE-specific RRC configuration. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier. - Switching gap ends at the end of the last slot on the “switch from” carrier Agreement Draft LS R1-2504868 is endorsed in principle. Agreement Final LS R1-2504869 is endorsed. Agreement - For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling - The following cases for collision handling are specified where UL TA is assumed to be 0 for the determination of overlapping. The UL TA =0 assumption only applies to the UL transmission configured by higher layer - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 3: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 4: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - Collision 5: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 6: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision - UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA

Liaison

-	A liaison to RAN2 and RAN4 with the above agreements was endorsed in [60]

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #114bis – RF (Core)

General aspects

-	The topic summary was prepared in [91]

-	An update of TR 38.768 was approved in [94]

UE RF aspects

-	The following agreements were captured in [96]:

Issue 1: Switching period capability - The switching period capability can be defined per band pair per band combination. Issue 2: Switching period location - Await outcome of RAN1 design Issue 3: Time mask for switching - Define the Tx on-off time mask requirements - FFS on how to define the Rx switching requirements to reflect where the switching period occurs - RAN4 has understanding that RAN1 will take switching period and timing advance into account for RAN1 scheduling pattern for low band CA via switching Issue 4: Considerations related to the Tx on-off time mask - Option 1: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL) - Option 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier - Other options are not precluded Issue 5: Test methodology - Companies are encouraged to contribute proposals related to the testability of the requirement on the time mask for switching

RAN4 #114bis – RRM (Core)

General aspects

-	The topic summary was prepared in [74]

RRM aspects

-	The following agreements were captured in [92]:

Issue 1-1-1: SSB assumptions for R19 LB CA via switching Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-1-2: MRTD assumptions for R19 LB CA via switching - FFS: - Option 1 (OPPO, vivo, QC, ZTE, Nokia, Xiaomi, CATT, Samsung, HW, MTK, LGE): The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us. - Option 3 (Ericsson): MRTD = CP for in LB-LB CA with switching. Issue 1-1-3: switching pattern applicability for RRM requirement in R19 LB CA via switching Agreement: - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirements for deactivated SDL SCell measurement. - Note: the above FFS parts will be discussed and decided in RAN4. Issue 1-1-4: RRM requirement applicability for R19 LB CA via switching Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-2-1A: Necessity of interruption requirement in RRM between FDD CC and SDL CC Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-2-1B: Necessity of interruption requirement in RRM when more carriers are considered Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-2-2: Interruption requirement (if needed) for carrier switching between FDD CC and SDL CC Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-3-1: Activated SDL SCell L3 measurement and corresponding interruption Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-3-2: Deactivated SDL SCell measurement and corresponding interruption Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-3-3: SDL SCell activation/deactivation Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-3-4: BFD/CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for SDL SCell Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-3-5: others for SDL SCell Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-4-1: Impact on timing requirement for PCell Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614. Issue 1-4-2: others RRM requirement of FDD PCell in LB CA via switching Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614.

Liaison to other working groups

-	A liaison to RAN1 (with RAN2 in CC) was approved with the following content [93]:

# 1 Overall description RAN4 discussed the applicability of switching pattern for SDL SCell for RRM requirements and would like to share the following information to RAN1. - For the SCell which has been activated (i.e., after UE reporting valid CQI for SCell activation, e.g., in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - RAN4 will also discuss potential scheduling restriction/interruption on PCell during the SCell activation procedure for receiving the above mentioned RS occasions for the SCell activation - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - RAN4 will discuss and decide whether/how UE to perform the deactivated SDL SCell measurement Note: The switching pattern for SDL SCell referred above is the RAN1 defined semi-static switching pattern configured through RRC for the FDD PCell/SDL SCell switching. # 2 Actions To RAN1: ACTION: RAN4 respectfully asks RAN1 to take the above into account in their future work.

RAN4 #115 – RF (Core)

General aspects

-	The topic summary was prepared in [130]

-	A text proposal to TR 38.768 with corrections to the operator provided scenarios was approved in [113]

-	A text proposal to TR 38.768 with time masks for switching was approved in [134]

-	Version 0.0.4 of TR 38.768 was approved via email discussion in [136]

UE RF aspects

-	The following agreements were captured in [132]:

Issue 1: Agreements related to the time mask requirement - A text proposal to TR38.768 in R4-2507932 captures the relevant agreements - How to specify the DL time mask: - The UE DL behaviour following the switch operation can be discussed in the RRM session - Considering that only one RAN4 meeting is left before the conclusion of the work item, companies are encouraged to exchange views via the RAN4 email reflector in advance of RAN4 #116 to converge on the technical content of the CR to 38.101-1 - RAN4 to further discuss whether the time mask should show the switching period/gap location and whether the diagram similar to Figure 5.2.2-1 and 5.2.3-1 in TR38.768 should be considered. Issue 2: Agreements related to UE capabilities - A new feature group is included in the Rel-19 feature list for NR_LBCA_Sw - Related to concurrent Rx operation, the following aspect is included in the feature group: - [The con-current reception on FDD band and SDL band is not supported. The con-current operation between FDD UL and SDL is not supported.] - Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID. - Support 70us as a candidate value of switching gap for 15kHz SCS in addition to 35us and 140us

RAN4 #115 – RRM (Core)

General aspects

-	The topic summary was prepared in [102]

-	A text proposal to TR 38.768 on RRM requirements for low band aggregation via switching was approved in [113]

RRM aspects

-	The following agreements were captured in [133]:

Agreement in AH: Table 1. Necessity of requirement on SDL CC Requirement; Whether or not RAN4 will have requirement; Note. interruption requirement for CC switching; No (can be revisited if not aligned with RAN1 further agreement.); Issue 1-2-1, Agreed on Tuesday online session. Activated SDL SCell L3 measurement; Yes; Issue 1-3-1, Agreed on Tuesday online session. Deactivated SDL SCell measurement [and corresponding interruption]; Yes; Issue 1-3-3, Agreed on Tuesday online session. SDL SCell activation/deactivation and corresponding interruption; Yes. [BFD/CBD]; Yes; Related with issue 1-3-10. TCI state switch delay; Yes; Related with issue 1-3-11. PL-RS switch delay; No; Related with issue 1-3-12. L1-RSRP/SINR; Yes; Related with issue 1-3-13. Issue 1-1-1: SSB assumptions for R19 LB CA via switching Agreement on Monday AH: - The baseline assumption is: - SDL SCell is transmitting SSB - RAN4 to use SSB based requirement as baseline for Rel-19 low band CA via switching - requirement of PCell will still use the existing principle as baseline but the impact of carrier switching in LB CA will be considered Issue 1-1-2: MRTD assumptions for R19 LB CA via switching Agreement on Monday AH: - The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us. - Whether or not above RTD can be used as side condition for SCell activation will be discussed in another issue. Issue 1-1-3: RRM requirement applicability for R19 LB CA via switching [Wayforward]: FFS: options in topic summary R4-2505556. Issue 1-1-4: others for general part [Wayforward]: FFS: options in topic summary R4-2505556. ### Issue 1-2: Interruption or scheduling restriction requirement Issue 1-2-1: Necessity of interruption requirement in RRM between FDD CC and SDL CC Online Agreement on Tuesday: - For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier. - Note: the above agreement can be revisited if not aligned with RAN1 further agreement. Issue 1-2-2: Necessity of interruption requirement in RRM when more carriers are considered [Wayforward]: FFS: options in topic summary R4-2505556. Issue 1-2-3: Interruption or scheduling restriction requirement (if needed) for carrier switching between FDD CC and SDL CC [Wayforward]: this issue is depending on issue 1-2-1. ### Issue 1-3: SDL SCell related requirement #### Activated SDL SCell L3 measurement Issue 1-3-1: Necessity of activated SDL SCell L3 measurement and corresponding interruption requirement Online Agreement on Tuesday: - RAN4 to define the requirement for activated SDL SCell L3 measurement. Issue 1-3-2: Requirement design of activated SDL SCell L3 measurement Online Agreement on Tuesday: - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. FFS: - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply (from Nokia proposal) - FFS on details of requirement design. #### Deactivated SDL SCell L3 measurement Issue 1-3-3: Necessity of deactivated SDL SCell measurement and corresponding interruption requirement Online agreement on Tuesday: - RAN4 to define the requirement for deactivated SDL SCell L3 measurement. Issue 1-3-4: Requirement design of deactivated SDL SCell measurement and corresponding interruption Agreement on Thursday: - The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement. Agreement on Thursday: - For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. - FFS: X, and X can be same or different for different SDL SCC measurement cycle (i.e., configured measCycleSCell). #### SDL SCell activation/deactivation Issue 1-3-5: Scope of SDL SCell activation/deactivation Online agreement on Tuesday: - PUCCH SCell activation, Rel-19 fast SCell activation through EMR, and activation of multiple SCells is not in the scope of the WI. Issue 1-3-6: side condition for SDL SCell activation/deactivation requirement Agreement on Thursday: - Follow the legacy case and side conditions for SSB based inter-band FR1 SCell activation in existing spec. Issue 1-3-7: SDL SCell activation/deactivation delay requirement - Agreement on Thursday: - The R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching if UE supports corresponding feature. - R16 Direct SCell activation with HO is not considered for R19 LB CA. - R17 PUCCH SCell activation is not considered for R19 LB CA. - R18 SSB-less SCell activation for NES is not considered for R19 LB CA. - R17 Fast SCell activation is not considered for R19 LB CA. - Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation. - Known/unknown conditions are reused from existing SCell activation requirements - FFS: - RAN4 to discuss the UE behavior if the CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation. (Apple) - The starting point and ending point of CSI measurement for the sake of CSI reporting should be determined, for both SSB based and SSB-less SCell activation.(ZTE) - Amend the previous agreement with the following - The UE shall continuously monitor PCell disregarding the configured switching pattern. (Nokia) Issue 1-3-8: interruption or scheduling restriction during SDL SCell activation/deactivation [Wayforward]: FFS: options in topic summary R4-2505556. Issue 1-3-9: Activation of Switching Pattern [Wayforward]: FFS: options in topic summary R4-2505556. #### SDL SCell L1 requirement Issue 1-3-10: BFD/CBD requirements for SDL SCell Agreement: - If RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching, the BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern. Issue 1-3-11: TCI state switch delay requirement for SDL SCell Agreement: - The L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern. Issue 1-3-12: PL-RS switch delay requirement for SDL SCell Agreement in AH (based on the summary table 1): - Not define requirement for PL-RS switch delay for SDL SCell in LB CA via switching. Issue 1-3-13: L1-RSRP/SINR measurement requirements for SDL SCell Agreement in AH: - For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern. #### Others for SDL SCell Issue 1-3-14: others for SDL SCell [Wayforward]: FFS: options in topic summary R4-2505556. ### Issue 1-4: Impact on FDD PCell RRM Issue 1-4-1: Impact on timing requirement for PCell - Agreement in AH: - the existing Tx timing requirement for PCell with the same applicability condition (SSB is available in last 160ms) can apply for R19 LB CA via switching. Issue 1-4-2: others RRM requirement of FDD PSCell in LB CA via switching [Wayforward]: FFS: options in topic summary R4-2505556. CR work split: Index; Requirement; note; Company. 1; Activated SDL SCell L3 measurement; Activated and deactivated SCell measurement delay requirement can be in the same section. All details are depending on the discussion.; OPPO. Deactivated SDL SCell measurement. 2; [Interruption for deactivated SDL SCell measurement]; Xiaomi. 3; SDL SCell activation/deactivation and corresponding interruption; All scope and details are depending on the discussion.; Nokia. 4; [BFD/CBD] for SDL SCell and PCell (if needed); (1) (If needed) BFD/CBD requirement for SDL SCell. (2) (If needed) The impact of switching pattern to PCell BFD/CBD requirement; Huawei. 5; TCI state switch delay for SDL SCell and PCell (if needed); (1) TCI state switch delay requirement for SDL SCell. (2) (If needed) The impact of switching pattern to PCell TCI state switch delay requirement; LGE. 6; PL-RS switch delay for PCell (if needed); (If needed) The impact of switching pattern to PCell PL-RS switching requirement; ZTE. 7; L1-RSRP/SINR for SDL SCell and PCell (if needed); (1)L1-RSRP/SINR measurement requirement for SDL SCell. (2) (If needed) The impact of switching pattern to PCell L1-RSRP/SINR measurement requirement; Samsung. 8; PCC L3 measurement delay requirement (if needed); (1) PCC L3 requirement impact can also be captured if needed. (2) PCell RLM requirement impact can also be captured if needed.; Ericsson. RLM requirement for PCell (if needed).

#### 2.4.2	Remaining Open issues

Core part:

-	Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2}

-	Specify the switching delay and time mask for carrier switching [RAN4]

-	Specify necessary RRM requirements [RAN4]

-	Define the corresponding UE capabilities [RAN4, RAN2, RAN1]

-	Specify requirements for the example band combination (CA_n5A-n29A) [RAN4]

Performance part:

-	Specify RRM test cases [RAN4]

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

RAN1 #120bis

- R1-2501828	vivo	Discussion on Low band carrier aggregation via switching

- R1-2501886	Spreadtrum, UNISOC	Discussion on low band carrier aggregation via switching

- R1-2501893	Lenovo	Discussion on NR low band CA via switching

- R1-2501911	ZTE	Discussion on low band carrier aggregation via switching

- R1-2501978	CATT	Discussion on low band carrier aggregation via switching

- R1-2502046	Ofinno	Discussion on a low-band CA with switching

- R1-2502203	NEC	Discussion on low band carrier aggregation via switching

- R1-2502216	Huawei, HiSilicon	Discussion on low band CA via switching

- R1-2502283	OPPO	Discussion of low-band CA via switching

- R1-2502389	Samsung	Discussion on low band carrier aggregation via switching

- R1-2502426	Ericsson	Low band carrier aggregation through switching

- R1-2502485	LG Electronics	Discussion on low band CA operation via switching for Rel-19

- R1-2502496	Nokia	Principles for low-band CA via switching

- R1-2502575	TCL	Low band carrier aggregation via switching

- R1-2502634	Apple	On low-band carrier aggregation via switching

- R1-2502635	Moderator (Apple)	FL summary #1 of Low band carrier aggregation via switching

- R1-2502636	Moderator (Apple)	FL summary #2 of Low band carrier aggregation via switching

- R1-2502637	Moderator (Apple)	FL summary #3 of Low band carrier aggregation via switching

- R1-2502721	MediaTek Inc.	Low band carrier aggregation via switching

- R1-2502784	NTT DOCOMO, INC.	Discussion on Low band carrier aggregation via switching

- R1-2502860	Qualcomm Incorporated	Low band carrier aggregation via switching

- R1-2503110	Moderator (Apple)	FL summary #4 of Low band carrier aggregation via switching

RAN1 #121

- R1-2503273	Huawei, HiSilicon	Discussion on low band CA via switching

- R1-2503331	ZTE Corporation, Sanechips	Discussion on low band carrier aggregation via switching

- R1-2503384	vivo	Remaining issues on Low band carrier aggregation via switching

- R1-2503421	NEC	Discussion on low band carrier aggregation via switching

- R1-2503430	Nokia	Principles for low-band CA via switching

- R1-2503532	Spreadtrum, UNISOC	Discussion on low band carrier aggregation via switching

- R1-2503588	Samsung	Discussion on low band carrier aggregation via switching

- R1-2503657	Lenovo	Discussion on carrier switching pattern for low band carrier aggregation via switching

- R1-2503739	Ofinno	Discussion on low-band CA with switching

- R1-2503780	CATT	Discussion on low band carrier aggregation via switching

- R1-2504153	ETRI	Discussion on low band carrier aggregation via switching

- R1-2504217	OPPO	Discussion of low-band CA via switching

- R1-2504255	LG Electronics	Discussion on low band CA operation via switching for Rel-19

- R1-2504275	MediaTek Inc.	Low band carrier aggregation via switching

- R1-2504347	Apple	On low-band carrier aggregation via switching

- R1-2504348	Moderator (Apple)	FL summary #1 of Low band carrier aggregation via switching

- R1-2504349	Moderator (Apple)	FL summary #2 of Low band carrier aggregation via switching

- R1-2504350	Moderator (Apple)	FL summary #3 of Low band carrier aggregation via switching

- R1-2504416	Qualcomm Incorporated	Low band carrier aggregation via switching

- R1-2504440	Ericsson	Low band carrier aggregation through switching

- R1-2504520	NTT DOCOMO, INC.	Discussion on Low band carrier aggregation via switching

- R1-2504606	Google	On Low band carrier aggregation via switching

- R1-2503274	Huawei, HiSilicon	UE features for low band CA via switching

- R1-2503326	TELUS	UE features for low band CA via switching

- R1-2503335	ZTE Corporation, Sanechips	Discussion on UE features for Rel-19 low band carrier via switching

- R1-2503396	vivo	Discussion on UE features for Low band carrier aggregation via switching

- R1-2503409	Nokia	LB-CA switching UE features

- R1-2503602	Samsung	UE features for low band CA via switching

- R1-2503659	Lenovo	Discussion on UE feature for low band CA via switching

- R1-2504219	OPPO	Discussion on UE feature for low-band CA via switching

- R1-2504361	Apple	On UE features for Rel-19 low band carrier aggregation via switching

- R1-2504430	Qualcomm	UE features for low band CA via switching

- R1-2504441	Ericsson	UE features for low band CA via switching

- R1-2504484	MediaTek Inc.	UE features for low band CA via switching

- R1-2504533	NTT DOCOMO, INC.	Discussion on UE features for Low band carrier aggregation via switching

- R1-2504671	Moderator (NTT DOCOMO, INC.)	Summary of discussion on Rel-19 NR UE features for low band CA via switching

- R1-2504672	Ad-Hoc Chair (NTT DOCOMO, INC.)	Session Notes of AI 9.15.12

- R1-2504869	LS on Low NR band carrier aggregation via switching

RAN4 #114bis

- R4-2503152	Huawei, HiSilicon	Discussion on LB-LB switching

- R4-2503194	Apple	UE RF requirements for LB CA via switching

- R4-2503244	Skyworks Solutions, Inc.	PA transients for LB-LB switching

- R4-2503256	Qualcomm Incorporated	RRM requirements for low band carrier aggregation via switching

- R4-2503329	Xiaomi	Discussion on LB-CA RRM requirements

- R4-2503336	Xiaomi	LB (FDD) LB (SDL) CA with switching

- R4-2503359	OPPO	Discussion on RRM requirements for low band carrier aggregation via switching

- R4-2503421	CATT	Discussion on the impacts of low band CA via switching on RRM requirements

- R4-2503477	CATT	TP to TR 38.768 on switching periods

- R4-2503478	CATT	On UE RF requirements and UE capability for low band CA via switching

- R4-2503532	Apple	On RRM requirement for R19 LB CA

- R4-2503561	Qualcomm Incorporated	RX Switching time, frequency and location

- R4-2503599	LG Electronics	UE RF requirements of LB carrier aggregation

- R4-2503614	Moderator (Apple)	Topic summary for [114bis][204] NR_LBCA_Sw

- R4-2503702	LG Electronics	Discussion on RRM requirements for Low Band CA operation

- R4-2503731	Spreadtrum, UNISOC	Discussion on UE RF requirements for LB-LB CA switching

- R4-2503845	vivo	Discussion of UE RF impact for LB-LB switching

- R4-2503890	Huawei, HiSilicon	Discussion on low band carrier aggregation via switching

- R4-2503933	Ericsson	RRM requirements for low band carrier aggregation via switching

- R4-2503962	ZTE Corporation, Sanechips	View on low-low band CA via switching

- R4-2503994	Nokia	NR_LBCA_Sw Considerations

- R4-2504028	vivo	Further discussion on RRM requirements for LB CA via switching

- R4-2504101	MediaTek Inc.	Discussion on low-band CA switching

- R4-2504104	Google	Discussion on low band carrier aggregation via switching

- R4-2504190	Ericsson	RF requirements for low-band CA via switching

- R4-2504253	ZTE Corporation, Sanechips	Discussion on RRM aspects of R19 Low band carrier aggregation

- R4-2504285	Samsung	General discussion on RRM requirements for LB CA switching

- R4-2504388	Samsung	UE RF requirements for LB CA via switching

- R4-2504478	MediaTek inc.	Discussion on low-band CA switching

- R4-2504499	Nokia	On LBCA RRM requirement impact

- R4-2504679	Moderator (Apple)	Topic summary for [114bis][121] NR_LBCA_Sw

- R4-2504895	Moderator (Apple)	WF on RRM requirements for NR_LBCA_Sw

- R4-2504969	Apple	LS of RAN4 RRM agreements on LB CA via switching

- R4-2505146	Apple	TR38.768 for low band aggregation via switching

- R4-2505147	Apple	TP to TR38.768 on operator provided scenarios for low band aggregation via switching

- R4-2505223	Apple	WF on UE RF requirements for LB CA via switching

RAN4 #115

- R4-2505322	Huawei, HiSilicon	Discussion on LB-LB switching

- R4-2505323	Huawei, Hisilicon	draftCR to 38.101-1: Time mask for LB-LB switching

- R4-2505344	TELUS	UE features for low band CA via switching

- R4-2505439	Apple	UE RF requirements for LB CA via switching

- R4-2505512	Qualcomm Incorporated	RRM requirements for low band carrier aggregation via switching

- R4-2505556	Moderator (Apple)	Topic summary for [115][207] NR_LBCA_Sw

- R4-2505675	Nokia	Considerations for Tx on-off time mask for NR_LBCA_Sw

- R4-2505716	OPPO	Discussion on RRM requirements for low band carrier aggregation via switching

- R4-2505781	Xiaomi	Discussion on LB-CA RRM requirements

- R4-2505786	Xiaomi	LB (FDD) LB (SDL) CA with switching

- R4-2505787	Xiaomi	TP to TR 38.768 on Time mask for switching

- R4-2505841	Skyworks Solutions, Inc.	On CA_n5-n8 switching between 1UL2DL and 1DL2UL

- R4-2505927	Apple	On RRM requirement for R19 LB CA

- R4-2505928	Apple	TP to TR38.768 on RRM requirements for low band aggregation via switching

- R4-2506000	CATT	Discussion on the impacts of low band CA via switching on RRM requirements

- R4-2506066	CATT	Further discussion on UE RF requirements for low band CA via switching

- R4-2506072	CATT	TP to TR 38.768 on clause 4 (Scenarios)

- R4-2506166	Qualcomm Incorporated	Switching time diagram

- R4-2506196	Ericsson	TP for 38.768: time masks for DL-only CA via switching

- R4-2506197	Ericsson	Draft CR for 38.101.1: time masks for DL-only CA via switching

- R4-2506201	LG Electronics	UE RF requirements of LB carrier aggregation

- R4-2506236	LG Electronics	Discussion on RRM requirements for Low Band CA operation

- R4-2506382	ZTE Corporation, Sanechips	View on low-low band CA via switching

- R4-2506448	Spreadtrum, UNISOC	Discussion on UE RF requirements for LB-LB CA switching

- R4-2506454	vivo	Further discussion on RRM requirements for LB CA via switching

- R4-2506739	Huawei, HiSilicon	Discussion on low band carrier aggregation via switching

- R4-2506864	vivo	Discussion of UE RF impact for LB-LB switching

- R4-2507056	Ericsson	RRM requirements for low band carrier aggregation via switching

- R4-2507060	MediaTek Inc.	Discussion on low-band CA switching time mask and fallback applicapability

- R4-2507249	Samsung	Discussion on RRM requirements for LB CA switching

- R4-2507293	ZTE Corporation, Sanechips	Discussion on RRM aspects of R19 Low band carrier aggregation

- R4-2507385	Nokia	Discussion on LBCA RRM requirements

- R4-2507527	Samsung	Views on UE RF aspects for LB CA via switching

- R4-2507565	Moderator (Apple)	Topic summary for [115][126] NR_LBCA_Sw

- R4-2507578	MediaTek inc.	Discussion on low-band CA switching

- R4-2508118	Apple	WF on UE RF requirements for LB CA via switching

- R4-2508259	Moderator (Apple)	WF on RRM requirements for NR_LBCA_Sw

- R4-2507932	Ericsson, Xiaomi, MediaTek, Apple	TP for 38.768: time masks for DL-only CA via switching

- R4-2508434	Apple	TP to TR38.768 on RRM requirements for low band aggregation via switching

- R4-2508119	Apple	Draft TR for 38.768 LB-CA via switching

04.02.2025		minor adaptations for RAN #107

17.10.2024		minor adaptations for RAN #106 and 2.6 for RAN6 removed

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template