3GPP TSG RAN Meeting #108				RP-251324

Prague, Czech Republic, June 9-13, 2025

Source:	Skyworks Solutions, Inc.

Title:	On HPUE for 6G

Agenda Item:	16.1

Document for:	Discussion

# Introduction

The study item on 6G scenarios and requirements was approved in RAN#107 in [1] and covers two parts:

- 3GPP inputs to ITU-R IMT-2030 TPR;

- 3GPP requirements for 6G Radio.

In this paper, we share our views on required improvements for High Power UE for 6G Radio.

# HPUE for 6G

## Study Objectives Relevant to HPUE

The study item [1] lists several objectives targeting the delivery of both the input towards ITU TPRs and Study 6G attributes for 3GPP practical deployment scenarios. Within this list, a number of parameters point to the need for High Power UEs (HPUE), the most representative items are copied below:

- Enhanced overall coverage, focus on cell-edge performance and UL coverage;

- Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz; and

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

For all of the above objectives, higher power capability on the UE side is needed while UL is seen more often as the bottleneck for improved link performance:

- Higher UL power capability is needed to improve both the DL and UL throughput.

- Higher UL power allows both larger coverage and wider transmit bandwidths in UL and DL.

  - 200MHz in FR1 will require wider bandwidth in UL and to maintain PSD UE power need to increase.

  - Even more for 200MHz or 400MHz in 7GHz to 15GHz frequency range.

## HPUE options for 6G

Within the current NR specification there are already several HPUE cases that are covered:

- Single band and single CC power classes:

  - PC5 1Tx (20dBm) for NRU

  - PC3 default power class (23dBm) for FDD/TDD TN and NTN

    - 1Tx for NR with optional power boosting

    - 2Tx for NRU

  - PC2 (26dBm) for FDD/TDD TN (+NTN in Rel-19)

    - 1Tx and 2Tx for NR with optional power boosting

  - PC1.5 (29dBm) for TDD TN

    - 2Tx for NR (+NR-NTN in Rel-19)

    - 4Tx for FWA

  - PC1 (31dBm) for NR (FWA+IoT NTN in Rel-19)

  - Rel-19 is introducing means to decrease Minimum Power Reduction (MPR) in some circumstances

- Intra-band contiguous and non-contiguous CC power classes:

  - PC3 default power class (23dBm) for 1Tx and 2Tx FDD/TDD TN

  - PC2 (26dBm) for FDD/TDD TN

    - 1Tx and 2Tx

  - PC1.5 (29dBm) introduced in Rel-19

    - 1Tx and 2Tx for contiguous case

    - 1Tx for non-contiguous case

  - Rel-19 is introducing means to decrease Minimum Power Reduction (MPR) in some circumstances

- Inter-band power classes:

  - PC3 default power class (23dBm) for 1/2/3Tx with PC3 per band (PC5 for NR-U bands)

  - PC2 (26dBm) for 1/2/3Tx with PC3/PC2 per band

  - PC1.5 (29dBm) 1/2/3Tx with PC3/PC2/PC1.5 per band

  - With increased power option when the sum of the band power capability is higher than the inter-band power class.

As seen on the above list, there are already a number of cases that are specified in 5GA. However, the specification framework is very complex with:

- 5 power classes;

- MPR and power boosting and reduced MPR options applying offsets to both the upper and lower bound of the power class;

- A large number of inter-band cases with permutations of PC3/PC2/PC1.5 bands for 1UL or 2UL 2/3Tx and up to 3CC;

- PC3 as the default power class for two UL inter-band cases where each band is already PC3 capable; and

- MPR and A-MPR studies require to explore a large set of RB allocation.

For 6G where UL performance and HPUE capability drive several important UL and DL performance metrics, it is essential that the UEs are always operating at their maximum feasible RF output power across all UL configurations:

- Single band, intra-band and/or inter-band combinations; and

- RB allocations (possibly limiting allocation cases resulting in high power back-off).

We also should learn from our 5G experience where Rel-20 will be the fifth release with HPUE content and reduce the number of FR1 power classes, remove unnecessary power limitations and facilitate the definition of a power class from single UL band single CC up to two UL band with four CC.

Although we understand it might be early to have a detailed discussion for power classes in 6G, we think it is important to propose a few principles to allow a more straightforward HPUE framework in 6G.

Proposal 1: For 6G, define PC2 (26dBm) power class as the default power class for the 0.4-15GHz range. Number of supported Tx is FFS depending on the frequency.

Proposal 2: For 6G, study power class requirements to avoid unnecessary output power limitation.

For example:

- Two UL inter-band power class always allows each band to reach its maximum power;

- The UE is always allowed to boost some allocations (lower PAPR, In-band limitation only, improved linearity/impairments).

Proposal 3: For 6G, study whether allocations resulting in large power back-off (MPR) should be avoided.

# References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025.