3GPP TSG RAN Meeting #108								          RP-251343

Prague, Czech Republic, June 9-13, 2025

## Status Report to TSG

Agenda item:			9.3.4.3

| WI / SI Name | NR Radio Resource Management (RRM) Phase 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_Ph5 |  |  |  |  |
| Unique ID | 1031079 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242380 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall:90% RAN4: 90% | Performance Part: 20% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Aijun Cao |
|  | Company | CATT |
|  | Email | ajcao@catt.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#114bis meeting

The following tdocs have been approved/endorsed in RAN4#114bis meeting for core part:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2504916 | Ad-hoc minutes for NR_RRM_Ph5 WI | Apple |
| R4-2504899 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| R4-2503530 | Test case (performance part) plan for R19 RRM Phase 5 WI | Apple |
| R4-2504900 | WF on RRM requirements for NR_RRM_Ph5_Part2 | CATT |
| R4-2504962 | LS on Rx BSF optimization for NR RRM Ph5 | CATT |

Issue 0: Work plan of test case (performance part) for NR_RRM_Ph5

Agreement in online session: Approve the work plan in R4-2503530.

Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF

Agreement in RRM session (Monday):

    - For the baseline scenario agreed in RAN4 #112, limit the serving cell to a FR2 serving cell.

    - Leave the decision to RAN2. Further discuss the details for the LS to RAN2, and the LS can only be approved if the content is agreeable.

      - Details of the LS can refer to R4-2504962.

Issue 1-2-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Agreement in the ad-hoc:

- Introduce the UE capability of reduced N, the values of which are {2, 4 and 6}. And UE chooses one value to report according to its capability. FFS the granularity of the UE capability in issue 1-2-4.

Issue 1-2-3: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF

Agreement in the ad-hoc:

  - For each scenario for PC3 UE, the described factors in the table should be replaced with capability value.

| Scenarios | Factors |
| --- | --- |
| 1 | Mpss/sss_sync_w/o_gaps = [reduced N]*3 Mmeas_period_w/o_gaps = [reduced N]*3 |
| 2 | Mpss/sss_sync_with_gaps = [reduced N]*3 Mmeas_period_with_gaps = [reduced N]*3 |
| 3 | Mpss/sss_sync_inter = [reduced N]*3 Mmeas_period_inter = [reduced N]*3 MSSB_index_inter = [reduced N]*3 |
| 4 | Mpss/sss_sync_inter = [reduced N]*5 Mmeas_period_inter = [reduced N]*5 MSSB_index_inter = [reduced N]*3 |
| 5 | If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then N in the HO requirement will be replaced by [reduced N] when the target cell is in FR2-1 and the PC3 UE supports [Rel-19 Rx FBS]. |
| 7 | RRC Re-establishment/redirection Can be discussed directly in the draft CR. |

Topic #2: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Issue 1-1: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

Agreement:

- the CSSF optimization can be applied to the CSSF tables for the related CA/DC scenarios.

  - For solution 1: CSSF tables for CA, NR-DC, EN-DC and NE-DC

  - For solution 3: CSSF tables for CA, NR-DC, EN-DC with FR1 NR only

- Details of whether/how the CSSF factor is modified can be left to CR discussion.

Issue 1-3-1: whether solution 3 includes NR-DC scenario or not?

Agreement in online session:

- Solution 3 includes NR-DC of FR1 and FR1 + FR2

Issue 1-3-3: detailed CSSF table design for solution 3

Agreement in online session:

- This requirement applies regardless of whether measurement gap is configured or not.

Issue 1-4-1: UE capability for CSSF enhancement

Agreement:

- UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR.

- UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

Issue 2-1-1: TC list for Rx BSF optimization

Agreement in online session: Further discuss based on the following table:

| Index | Scenario | Need or not | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 or 2 TCs | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | CMCC |
| 3 | Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | CATT |
| 2 | Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 or 2 TCs | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | Huawei |
| 4 | Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | Samsung |
| 5 | Scenario 5: Handover | 1 or 2 TCs | TBA(e.g., Intra-frequency or inter-frequency, unknown) | OPPO |
| 6 | Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection | 1 or 2 TCs | TBA(e.g., re-establishment or RRC redirection, Intra-frequency or inter-frequency, [or known] or unknown) | Intel |

Issue 2-2-1: TC list for CSSF optimization

- TC for solution 1:

Agreement in online session:

- Define the test cases for solution 1 of CSSF enhancement following the table below:

| TC index | Scenario | Need or not | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | Nokia |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | Ericsson |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | ZTE |

RAN4#115 meeting

The following tdocs have been approved/endorsed in RAN4#114bis meeting for core part:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2508265 | Ad-hoc minutes for NR_RRM_Ph5 WI | Apple |
| R4-2508264 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| R4-2508266 | WF on RRM requirements for NR_RRM_Ph5_Part2 | CATT |

Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Agreement:

For the quitting condition of FBS:

- Introduce a new UAI indicating “overheating” issue:

  - RAN4 will not discuss the UE behaviour after reporting UAI

- Introduce a timer-based solution:

  - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition

    - Further discuss the candidate numbers for T1.

Issue 1-1-2: Status report of fast beam sweeping

Agreement:

  - No need to define UE status report of fast beam sweeping.

Issue 1-1-3: UE capability for Rx BSF optimization

Agreement:

For FBS capability design:

  - UE capability is per-band and optional

  - TDD only

  - FR2-1 only

  - FFS: in the same FG, whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency)

Issue 2-1-1: TC list for Rx BSF optimization (update based on last meeting agreement)

- Agreement:

| Index | Scenario | Number of TC | Configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 | - Non-DRX - In TC1/2/3/4, UE is only required to pass TC1+TC4 or TC2+TC3. | CMCC |
| 3 | Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter | 1 |  | CATT |
| 2 | Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 | - With DRX, DRX.1 for sub-test1 and DRX.7 for sub-test2 (same as in table A.7.6.1.2.1-2) - In TC1/2/3/4, UE is only required to pass TC1+TC4, or TC2+TC3. | Huawei |
| 4 | Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter | 1 |  | Samsung |
| 5 | Scenario 5: Handover | 1 | - Intra-frequency handover from FR2 to FR2 - Unknown target cell | OPPO |
|  |  | 1 | - Inter-frequency handover from FR2 to FR2 - Unknown target cell |  |
| 6 | Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection | 1 | - Inter-frequency RRC Re-establishment; - Unknown target cell - FR2 to FR2 | Intel |
|  |  | 1 | - RRC Connection release with redirection - FR2 to FR2 |  |

Issue 2-1-2: baseline setup for BSF test case

- Agreement:

  - For BSF optimization, for scenario 1/2/3/4, the existing test configurations in A.7.6.1 and A.7.6.2 can be used as baseline.

  - For BSF optimization, for scenario 5/7, the existing test configurations in A.7.3.1 and A.7.3.2 can be reused except that the test requirements need to be updated based on the reduced beam sweeping factor reported by UE.

Issue 2-1-3: whether or not to test FBS quitting

Agreement on Monday:

No need to test UE not using FBS for scenario 1~4 if the FBS condition is not met.

Topic #2: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Issue 1-2-1: detailed design for solution 3 (by-default option 3 vs. need network indication for specific SCC)

Agreement on Monday:

  - For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing.

Issue 1-2-2: applicability limitation for solution 3

Agreement in AdHoc:

  - For UE supporting both per-FR measurement gap and new 3-searcher capability,

    - If network configures per-FR measurement gap, UE will use legacy requirement (i.e., 2-searcher requirement) when at least one SMTC occasion of outside gap measurement objects in one FR are overlapped by per-FR measurement GAP in another FR. Otherwise, the 3-searcher requirement applies.

  - For UE supporting new 3-searcher capability,

    - If network configures per-UE measurement gap and does not configure per-FR measurement gap, the 3-searcher requirement always applies.

Issue 1-3-1: UE capability for CSSF enhancement option 1 (measure one carrier per band)

Agreement in AdHoc:

  - For CSSF enhancement solution 1 in Rel-19, a new optional per-UE capability, apply for FR2-1 only.

Note: the UE capability can be revisited if this feature is expanded to FR1 case.

Issue 1-3-2: detailed UE capability for CSSF enhancement

Please refer to the R4-2508264.

- General for solution 1 and 3

Issue 2-2-1: General assumption for TCs of solution 1 and 3

Agreement on Monday:

Not to define additional test cases for MG related features (i.e., NCSG with nogap-noncsg, Rel-18 Needforgaps with no-gap-no-interruption or no-gap-with-interruption) with solution 1 or solution 3 of CSSF optimization.

- TC for solution 1 (issue 2-2-2/3/4):

Issue 2-2-2: TC list for CSSF optimization solution 1

- Agreement

| TC index | Scenario | CA/DC Operation mode | Configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them | - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC | Nokia |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them | - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC | Ericsson |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them | - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC | ZTE |

Note: network indication is up to the other RAN2/4 core part discussion. In case network indication for a specific SCC measurement is replaced by other new solution(e.g., SCC selected by UE implementation), use the new solution for CA mode test.

Issue 2-2-3: baseline TC setup for CSSF optimization solution 1

- Agreement

  - For solution 1 of CSSF optimization, the legacy test configurations in A.7.6.1 and A.7.6.2 can be used as baseline.

Issue 2-2-4: other detailed TC design for CSSF optimization solution 1

- discuss this issue directly in draft CRs.

- TC for solution 3 (issue 2-2-5/6/7):

Issue 2-2-5: TC list for CSSF optimization solution 3

Agreement on Monday:

| TC index | Aspects | CA/DC mode | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them. | Without DRX and without SSB index reporting | Ericsson |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them. | Without DRX and without SSB index reporting | OPPO |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them. | Without DRX and without SSB index reporting | Huawei |

Issue 2-2-6: baseline TC setup for CSSF optimization solution 3

Agreement on Monday:

  - For FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, for solution 3, existing L3 intra-frequency/inter-frequency measurement test cases for SA and EN-DC modes, e.g., A.5.6.1 and A.5.6.2, A.7.6.1 and A.7.6.2, can be used as the baseline for FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC.

Topic #3: Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-2-1: Test case list

Agreement:

  - Scenario:

    - Normal SCell activation for FR1 and FR2

    - Direct SCell activation at SCell addition for FR1

    - PUCCH SCell activation for FR1 and FR2

    - FFS:

      - Direct SCell activation at SCell addition for FR2

  - UE capabilities and network configurations:

    - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

    - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

    - FFS:

      - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

#### 2.4.2	Remaining Open issues

As captured in WF R4-2508264 and R4-2508266.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#114bis meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2503250 | Enhancement on FR2 CSSF | Qualcomm Incorporated |
| R4-2503251 | FR2-1 L3 measurement delay reduction by optimizing RX beam sweeping factor | Qualcomm Incorporated |
| R4-2503288 | Remaining issues on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | vivo |
| R4-2503289 | Remaining issues on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | vivo |
| R4-2503340 | On optimizing CSSF for FR2-1 L3 measurement delay | OPPO |
| R4-2503356 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | OPPO |
| R4-2503357 | DraftCR on Multi-Rx based RRC Connection Release with Redirection requirements | OPPO |
| R4-2503358 | Discussion on performance requirements for fast SCell activation for UE supporting Rel-18 EMR | OPPO |
| R4-2503416 | Discussion on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor | CATT |
| R4-2503417 | Discussion on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CADC | CATT |
| R4-2503418 | Discussion on performance requirements for FR2-1 L3 measurement delay reduction | CATT |
| R4-2503419 | Discussion on performance requirements of fast SCell activation for UE supporting R18 EMR | CATT |
| R4-2503420 | DraftCR on SSB-based intra-frequency measurement without gap delay reduction by optimizing Rx BSF | CATT |
| R4-2503503 | FR2-1 SSB based L3 measurement delay reduction with CSSF optimization | Nokia, Nokia Shanghai Bell |
| R4-2503504 | draftCR on CSSF optimization solutions | Nokia, Nokia Shanghai Bell |
| R4-2503505 | draftCR on test case for intra-frequency measurement with CSSF solution 1 | Nokia, Nokia Shanghai Bell |
| R4-2503529 | draft CR for Rel-19 CSSF enhancement solution 1 (one CC measurement per band) | Apple, CATT, Huawei, HiSilicon, Qualcomm, Ericsson, ZTE, MediaTek, Samsung |
| R4-2503530 | Test case (performance part) plan for R19 RRM Phase 5 WI | Apple, CATT |
| R4-2503531 | On FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | Apple |
| R4-2503568 | draft CR for fast SCell activation for UE supporting Rel-18 EMR | Apple |
| R4-2503618 | Topic summary for [114bis][208] NR_RRM_Ph5_Part1 | Moderator (Apple) |
| R4-2503619 | Topic summary for [114bis][209] NR_RRM_Ph5_Part2 | Moderator (CATT) |
| R4-2503646 | Discussion on performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode | OPPO |
| R4-2503707 | Discussion on RRM requirements for FR2-1 L3 measurement delay reduction | LG Electronics Inc. |
| R4-2503753 | Discusssion for fast scell activation for UE supporting Rel-18 EMR | Ericsson |
| R4-2503754 | Discussion on performance part of the fast Scell activation for UE supporting Rel-18 EMR | Ericsson |
| R4-2503763 | Discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor | CMCC |
| R4-2503764 | Discussion on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC | CMCC |
| R4-2503765 | Discussion on fast SCell activation for UE supporting Rel-18 EMR | CMCC |
| R4-2503766 | Discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode | CMCC |
| R4-2503767 | Discussion on RRM performance requirements for fast SCell activation for UE supporting Rel-18 EMR | CMCC |
| R4-2503779 | DraftCR for optimizing Rx beam sweeping factor on SSB based Inter-frequency measurement with MG | CMCC |
| R4-2503785 | Discussions on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor | NTT DOCOMO, INC. |
| R4-2503848 | Discussion on the fast SCell activation for UE supporting Rel-18 EMR | ZTE Corporation, Sanechips |
| R4-2503849 | Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR | ZTE Corporation, Sanechips |
| R4-2503862 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Huawei, HiSilicon |
| R4-2503863 | Discussion on maintenance for fast SCell activation based on EMR | Huawei, HiSilicon |
| R4-2503864 | Draft CR on fast SCell activation for UE supporting Rel-18 EMR | Huawei, HiSilicon |
| R4-2503865 | Discussion on performance part for Fast SCell activation for UE supporting Rel-18 EMR | Huawei, HiSilicon |
| R4-2503874 | Discussion on FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC | Huawei, HiSilicon |
| R4-2503875 | Discussion Discussion on performance part for FR2-1 SSB based L3 measurement delay reduction for connected mode | Huawei, HiSilicon |
| R4-2503911 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | China Telecom |
| R4-2503912 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | China Telecom |
| R4-2503913 | Discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode | China Telecom |
| R4-2503914 | Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR | China Telecom |
| R4-2504020 | Discussion on delay reduction by optimizing Rx beam sweeping factors | Intel Corporation |
| R4-2504021 | Discussion on delay reduction by optimizing CSSF outside gap | Intel Corporation |
| R4-2504075 | Draft CR HO requirements and FBS requirement applicability | Nokia |
| R4-2504076 | On performance part of L3 measurement delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2504113 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Ericsson |
| R4-2504114 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | Ericsson |
| R4-2504115 | Discussion on performance requirements for FR2-1 L3 measurement enhancement | Ericsson |
| R4-2504242 | Discussion on CSSF optimization of R19 RRM enhancements | ZTE Corporation, Sanechips |
| R4-2504243 | Discussion on Rx beam factor optimization of R19 RRM enhancements | ZTE Corporation, Sanechips |
| R4-2504256 | Draft CR on FBS based RRC Re-establishment for R19 RRM enh | ZTE Corporation, Sanechips |
| R4-2504456 | On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Apple |
| R4-2504457 | Draft CR on SSB based Intra-frequency measurement with MG | Apple |
| R4-2504500 | On L3 FBS requirements | Nokia, Nokia Shanghai Bell |
| R4-2504518 | Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR | Nokia |
| R4-2504565 | Discussion on L3 measurement enhancement by optimizing Rx BSF | Samsung |
| R4-2504600 | Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor | MediaTek inc. |
| R4-2504601 | Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSF | MediaTek inc. |
| R4-2504657 | draft CR for Rx beam sweeping optimization for SSB based Intra-frequency measurement with NCSG | MediaTek Germany GmbH |
| R4-2504899 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| R4-2504900 | WF on RRM requirements for NR_RRM_Ph5_Part2 | CATT |
| R4-2504916 | Ad-hoc minutes for NR_RRM_Ph5 | Apple |
| R4-2504940 | Reply LS to RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18 | MediaTek |
| R4-2504957 | DraftCR on SSB-based intra-frequency measurement without gap delay reduction by optimizing Rx BSF | CATT |
| R4-2504958 | DraftCR for optimizing Rx beam sweeping factor on SSB based Inter-frequency measurement with MG | CMCC |
| R4-2504959 | Draft CR HO requirements and FBS requirement applicability | Nokia |
| R4-2504960 | Draft CR on FBS based RRC Re-establishment for R19 RRM enh | ZTE Corporation, Sanechips |
| R4-2504961 | Draft CR on fast SCell activation for UE supporting Rel-18 EMR | Huawei, HiSilicon |
| R4-2504962 | LS on Rx BSF optimization for NR RRM Ph5 | CATT |
| R4-2504970 | Draft CR on SSB based Intra-frequency measurement with MG | Apple |
| R4-2504971 | draft CR for Rx beam sweeping optimization for SSB based Intra-frequency measurement with NCSG | MediaTek Germany GmbH |
| R4-2504973 | draft bigCR on Rx BSF optimization for NR RRM ph5 | CATT |

RAN4#115 meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2505506 | Enhancement on FR2 CSSF | Qualcomm Incorporated |
| R4-2505507 | FR2-1 L3 measurement delay reduction by optimizing RX beam sweeping factor | Qualcomm Incorporated |
| R4-2505561 | Topic summary for [115][212] NR_RRM_Ph5_Part1 | Moderator (Apple) |
| R4-2505562 | Topic summary for [115][213] NR_RRM_Ph5_Part2 | Moderator (CATT) |
| R4-2505657 | On remaining issues on FR 2-1 L3 measurement delay by optimizaing CSSF outside gap in CADC | vivo |
| R4-2505658 | On remanining issues on FR 2-1 L3 measurement delay by optimizing Rx beam sweeping factor | vivo |
| R4-2505688 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | OPPO |
| R4-2505689 | DraftCR on Multi-Rx based RRC Connection Release with Redirection requirements | OPPO |
| R4-2505690 | On optimizing CSSF for FR2-1 L3 measurement delay | OPPO |
| R4-2505691 | Discussion on performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode | OPPO |
| R4-2505692 | DraftCR on test cases for Multi-Rx based handover | OPPO |
| R4-2505693 | Discussion on performance requirements for fast SCell activation for UE supporting Rel-18 EMR | OPPO |
| R4-2505726 | On remaining FBS requirements | Nokia |
| R4-2505727 | DraftCR for FBS requirement maintenance | Nokia |
| R4-2505728 | On performance requirements of L3 measurement delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2505869 | Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR | Apple |
| R4-2505889 | On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Apple |
| R4-2505935 | On feature list for RRM phase 5 WI | Apple |
| R4-2505936 | On FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | Apple |
| R4-2505937 | draft CR for Rel-19 CSSF enhancement solution 1 (one CC measurement per band) | Apple, CATT, Ericsson |
| R4-2505938 | On testing for CSSF enhancement in CA/DC | Apple |
| R4-2505992 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | CATT |
| R4-2505993 | draft bigCR on Rx BSF optimization for NR RRM ph5 | CATT |
| R4-2505994 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CADC | CATT |
| R4-2505995 | DraftCR on Multi-Rx based inter-frequency measurement with NCSG | CATT, OPPO |
| R4-2505996 | CR for fast SCell activation for UE supporting Rel-18 EMR | CATT, Apple, Huawei, HiSilicon |
| R4-2505997 | Discussion on performance requirements for FR2-1 L3 measurement delay reduction | CATT |
| R4-2505998 | Discussion on performance requirements of fase SCell activation for UE supporting R18 EMR | CATT |
| R4-2505999 | DraftCR to define test case for SSB based Inter-frequency measurement delay reduction without MG with fast beam sweeping | CATT |
| R4-2506214 | FR2-1 SSB based L3 measurement delay reduction with CSSF optimization | Nokia, Nokia Shanghai Bell |
| R4-2506215 | draftCR on CSSF optimization solutions | Nokia, Nokia Shanghai Bell |
| R4-2506216 | draftCR TC#1 SSB-based intra-f measurement without MG for solution 1 | Nokia, Nokia Shanghai Bell |
| R4-2506227 | Discussions on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor | NTT DOCOMO, INC. |
| R4-2506241 | Discussion on RRM requirements for FR2-1 L3 measurement delay reduction | LG Electronics Inc. |
| R4-2506242 | Discussion on performance requirements for FR2-1 L3 measurement delay reduction | LG Electronics Inc. |
| R4-2506331 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | China Telecom |
| R4-2506332 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | China Telecom |
| R4-2506333 | Discussion on Fast SCell activation for UE supporting Rel-18 EMR | China Telecom |
| R4-2506334 | Discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode | China Telecom |
| R4-2506335 | Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR | China Telecom |
| R4-2506359 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Ericsson |
| R4-2506360 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | Ericsson |
| R4-2506361 | Discussion on performance requirements for FR2-1 L3 measurement enhancement | Ericsson |
| R4-2506362 | (NR_RRM_Ph5-Core) draft CR on Rx BSF optimization for NR RRM ph5 | Ericsson |
| R4-2506363 | (NR_RRM_Ph5-Perf) Draft CR on SSB based Inter-frequency measurement without MG | Ericsson |
| R4-2506556 | Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR | vivo |
| R4-2506571 | Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR | ZTE Corporation, Sanechips |
| R4-2506572 | Discussion on the fast SCell activation for UE supporting Rel-18 EMR | ZTE Corporation, Sanechips |
| R4-2506597 | Discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor | CMCC |
| R4-2506598 | Discussion on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC | CMCC |
| R4-2506599 | Discussion on fast SCell activation for UE supporting Rel-18 EMR | CMCC |
| R4-2506600 | Discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode | CMCC |
| R4-2506601 | Discussion on RRM performance requirements for fast SCell activation for UE supporting Rel-18 EMR | CMCC |
| R4-2506615 | DraftCR for optimizing Rx beam sweeping factor on RRC connection release with redirection to NR | CMCC |
| R4-2506618 | DraftCR on TC on Rx BSF optimization for SSB based Intra-frequency measurement without MG (TC#1) | CMCC |
| R4-2506663 | Discussion on RRM core requirements of Fast SCell activation for UE supporting Rel-18 EMR | Ercisson |
| R4-2506664 | Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR | Ercisson |
| R4-2506665 | CR on Scell activation delay requirements for Deactivated PUCCH Scell | Ericsson |
| R4-2506702 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Huawei, HiSilicon |
| R4-2506703 | DraftCR on FBS L3 measurement for inter-frequency without gap | Huawei, HiSilicon |
| R4-2506704 | Discussion on performance part for Fast SCell activation for UE supporting Rel-18 EMR | Huawei, HiSilicon |
| R4-2506797 | Discussion on FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC | Huawei, HiSilicon |
| R4-2506798 | Discussion on performance part for RRM Phase 5 | Huawei, HiSilicon |
| R4-2506807 | CR on L3 measurement delay by utilising 3-searcher solution to optimise CSSF outside gap for Rel-19 | Huawei, HiSilicon |
| R4-2506826 | Test cases requirements for RRC re-establishment and release with redirection | Intel Corporation |
| R4-2506828 | Views on RRM enhancements ph5 RAN4 UE features in Rel-19 | Intel Corporation |
| R4-2507221 | Discussion on RRM core requirements of Fast SCell activation for UE supporting Rel-18 EMR | Nokia |
| R4-2507222 | draftCR to 38.133 on Fast SCell activation with EMR | Nokia |
| R4-2507223 | Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR | Nokia |
| R4-2507256 | Draft CR on measurement delay reduction for SSB based Inter-frequency measurement | Samsung |
| R4-2507257 | Test case on measurement delay reduction for SSB based Inter-frequency measurement | Samsung |
| R4-2507282 | Discussion on CSSF optimization of R19 RRM enhancements | ZTE Corporation, Sanechips |
| R4-2507283 | Discussion on Rx beam factor optimization of R19 RRM enhancements | ZTE Corporation, Sanechips |
| R4-2507296 | Discussion on Perf part of Rx beam factor optimization of R19 RRM enhancements | ZTE Corporation, Sanechips |
| R4-2507301 | Draft CR on Test case of Inter-RAT SSB measurement without MG of R19 CSSF enh | ZTE Corporation, Sanechips |
| R4-2507814 | Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor | MediaTek inc. |
| R4-2507815 | Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSF | MediaTek inc. |

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template