3GPP TSG RAN Meeting #108												RP-251365

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source:	Huawei, HiSilicon

Title:	Discussion on 6G deployment scenarios

Document for:	Discussion and decision

Introduction

A RAN-level study on “6G Scenarios and requirements” was approved in RAN#107[1]. Regarding the scenario and relevant key assumptions in the scenario, the objective of this SID includes:

This study item aims to

- investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the suTRP equent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- …...

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure suTRP tantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

…..

In this contribution, the considerations and views on some of the 6G scenarios and assumptions are discussed. Section 2 provides principles on how to address the typical and practical deployment scenarios for the further 6G. Section 3 for discussion on each specific 6G scenario. Annex provides the text proposal to the section of 6G deployment scenarios.

Principles of deployment scenario consideration

The section discusses the general principles for deployment scenarios and evaluation.

Carrier Frequency

There is different spectrum situation for 6G in different regions and countries. In some countries it seems promising to have new spectrum allocated in the coming years, while in some other countries existing spectrum is likely to be deployed in 6G Day 1. In addition, the value and techniques for 6G should be applied not to only new spectrum, but also to existing spectrum, to fully unleash the potential of a new generation. In this case, the evaluation shall consider both new spectrum and existing spectrum.

Proposal 1: both new and existing spectrum shall be considered for evaluation.

During the LTE-A and NR stage, the system level evaluation method uses single carrier in a single layer to evaluate spectrum efficiency and user experience data rates. This is seen useful to also apply to 6G, considering the case that operators usually would have one major band for initial 6G standalone deployment, with wide and continuous coverage. Therefore 6G should also consider single carrier in single layer for evaluation.

Other methods, such as multiple carrier simulation could also be considered for some KPIs evaluation, e.g. user experience data rate, energy saving etc. This may need further study considering other assumptions and methodologies for energy saving.

Proposal 2: Single carrier in single layer shall be considered as one method for evaluation.

For specific bands used for evaluation, it might be reasonable to have multiple candidate bands considering the situation from different regions. Regarding new spectrum, according to [2], the LS to ITU-R, the 7GHz with at least channel bandwidth of 200MHz from system perspective has already been agreed upon in the RAN#106 meeting. 3GPP document for ITU-R IMT-2030 with at least 400MHz system bandwidth was also approved in RAN#107 [3]. Therefore, 7GHz band can serve as one frequency band for future evaluation of 6G performance.

For existing bands, it is better to first understand operators’ major considerations from different areas, and select the most typical bands for evaluation. The consideration is not only about the frequency range, but also the duplex mode, e.g. TDD and FDD.

Proposal 3: For new spectrum, ~7GHz with at least 400MHz system bandwidth shall be evaluated.

Proposal 4: For existing spectrum, it is suggested to decide the candidate frequency bands, according to operators’ requirements.

User distribution and density

The major evaluation assumption is 10 users per cell in the technology study stage of LTE-A and NR. However, it is expected that 6G will support more services, e.g. various AI applications, more advanced immersive communications etc. The number of users using these emerging services is also expected to be increased, once these applications become more and more popular in the coming 10 years. Therefore reusing 10 users per cell may not be sufficient for future, and it is suggested to consider adding other options, e.g. 20, 30 and 50 users per cell for 6G evaluation.

Proposal 5: Multi-level of user number per cell shall be considered for 6G evaluation [10, 20, 30, 50].

Traffic model

During the NR stage, we explored many different traffic types for performance evaluation. For example, eMBB traffic has a large data rate, URLLC traffic requires high reliability and low latency, and mMTC traffic has a low data rate but occurs under conditions of high user density.

According to [4], the usage scenarios for IMT-2030 are extended to immersive communications, massive communications, hyper-reliable and low-latency communications, integrated sensing and communications, AI and communications, and ubiquitous connectivity. The potential traffic types are expected to be much more diverse than before, which brings various service requirements to satisfy user experience.

For communication traffic, there are two typical categories: one is characterized by packet size and arrival rate without latency requirements, such as Full buffer or FTP1, FTP2, FTP3 burst traffic models. In this case, the requirement of user experience does not take the latency into account, i.e., the traffic is less sensitive to latency. The other category involves packet size and arrival rate with latency requirements, such as XR burst traffic models, online gaming, or interactive mobile AI traffic. In this case, latency is a key factor affecting user experience. Additionally, the latency requirement can be different for different flows or packets. Therefore, it is important to consider whether and how latency requirements are included according to different traffic, to satisfy different service requirements in 6G technology evaluations.

Proposal 6: The traffic model should be based on the requirement of the service to consider with/without latency requirement, e.g. packet delay budget.

TRP antenna elements and UE antenna elements

In IMT-2020(5G), the TRP antenna elements in sub6G bandwidth is defined as up to 256. Besides, in Rel-19, 3GPP studies the channel model in 7-24GHz FR3 bandwidth [5], and the conclusions related to BS antenna element is agreed as below:

Agreement in RAN1#118 Previous agreement made in RAN1#117 is updated as: For the assumption on the aperture size of antenna array, the following is considered for near-field and spatial non-stationarity channel model study, e.g., simulation/measurement and calibration: - Up to 1.5 m for UMa with maximum antenna elements in the array is 5k for single Polarization. - Up to 1 m for UMi with maximum antenna elements in the array is 2.22k for single Polarization. - Up to 0.71 m for Indoor factory with maximum antenna elements in the array is 1.12k for single Polarization. - Up to 0.25 (for rectangular antenna array), 0.5 (for linear antenna array) m for Indoor office with maximum antenna elements in the array is 256, 80 for single Polarization, respectively.

In general it is expected that both network and UE should have more advanced capabilities for 6G, e.g. to have more antenna element number and Tx/Rx chains, to provide significant gains on spectral efficiency, user experience data rate, coverage and energy efficiency etc. It can also enable precise beamforming and spatial resolution, which supports high-capacity (via MU-MIMO), low-latency communication (via MU-MIMO and high SINR) and new applications like integrated sensing and communication (ISAC).

From network perspective, the antenna elements used in sub6G may be largely reused. For new spectrum, based on the channel model and evaluation assumptions in the current 3GPP release, the antenna element number for ~7 GHz for the base station (BS) need to be defined. Assuming the TRP installation l of a windward area is smaller than 0.8 m², up to 3072 antenna elements in 7GHz can be considered for the BS to alleviate performance constraints. Examples of 3072 antenna configuration could be (48, 32, 2) or (64, 24, 2) with in total 0.7m² @0.5λ as below.

It is worth mentioning that the 3072 is the maximum value of antenna elements for ~7GHz, while the specific antenna element number may depend on specific deployment scenarios and frequency bands, e.g. the antenna element number in indoor scenario is expected to be smaller than outdoor scenario.

Figure 2.5.1

One example for 3072 BS antenna element, ~0.7m² @0.5λ

Proposal 7: The max number of TRP antenna elements is up to 3072 for ~7GHz, and the specific number used for evaluation can be different according to specific deployment scenarios.

From UE perspective, the UE antenna elements in sub6G bandwidth are defined as up to 8 Tx/Rx in TR 38.913 [6]. Also, in Rel-19, 3GPP studies the channel model in 7-24GHz FR3 bandwidth, and the conclusions related to UE antenna element are agreed as below:

Agreement in RAN1#121 Agreement in RAN1#120b For CPE devices adopt the following device dimensions for UE antenna modeling: - (x cm × y cm × z cm) = (0 cm × 20 cm × 20 cm) - Total 9 candidate antenna locations in 4 corners of the vertical square plane, 4 center of the edges of the vertical square plane, and center of the vertical square plane. The figure below captures the side view on the device. - Note: a candidate antenna location (e.g., 9) can be used for multiple antennas

Based on the channel model and evaluation assumptions in the current 3GPP release, the UE antenna element in ~7GHz can support at least 16 antenna elements. Other candidate number of antenna elements in practical ~7 GHz deployment can also be considered according to different device types, such as 4, 8, 16 or 32. For example, the number of antenna elements of handheld devices is expected to be smaller than that of large size device, like CPEs, robots, etc, but still much larger than IoT devices. The number of UE Tx or Rx ports is usually smaller than the number of antenna elements, and can be further discussed in the evaluation stage, e.g. from 2T4R to 32T32R considering different device types and the way of antenna distribution within the devices. Considering there are different UE implementations, other distribution of UE antenna elements may need to be further considered.

Proposal 8: For ~7GHz carrier frequency, UE antenna element number is up to 16 or more, and specific UE antenna element number used in evaluation can be different according to device types.

Consideration on 6G scenario

Deployment scenarios should be considered for all 6G usage scenarios in this document, namely immersive communication (IC), massive communication (MC), hyper reliable and low latency communication (HRLLC), integrated sensing and communication (ISAC), AI and communication (AIC) and ubiquitous connectivity (UC).

Indoor hotspot

Indoor hotspot is an important scenario in previous generation study. In this deployment scenario, many key characteristics of this deployment scenario could be re-used to evaluate 6G, e.g. high capacity, high user density, user experience indoor, positioning performance, sensing-related performance.

The Indoor Hotspot-IC test environment consists of one floor of a building. The height of the floor is 3 m. The floor has a surface of 120 m × 50 m and 12 BSs/sites which are placed in 20 meters spacing as shown in Fig. 3.1.1, with a LOS probability as defined by channel model in [7]. In Fig. 3.1.1, internal walls are not explicitly shown but are modeled via the stochastic LOS probability model. The type of site deployed (e.g. one TRxP per site or 3 TRxPs per site) is not defined and should be reported by the proponent.

Figure 3.1.1

Indoor Hotspot sites layout

Therefore this scenario deployment can be reused for 6G and the values or assumptions are described as Table 3.1-1.

Table 3.1-1: Attributes for Indoor hotspot scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | At least 7GHz. FFS for other frequency. |
| System bandwidth | Around 7GHz: e.g. 400MHz NOTE1,2 |
| Layout | Single layer: - Indoor floor (Open office) |
| ISD | 20m (Equivalent to 12TRxPs per 120m x 50m) |
| TRP antenna elements | Around 7GHz: Up to 768 Tx and Rx antenna elements |
| UE antenna elements | Around 7GHz: Up to [16] Tx and Rx antenna elements |
| User distribution and UE speed | 100% Indoor, 3km/h, [10] users per TRxP, |
| Sensing target distribution and Sensing target speed | 100% Indoor, 3km/h [1] sensing target per TRxP. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.  The specific typical antenna configurations may be different for different device types and need further study.

NOTE4:	[10] users per TRxP is the baseline, other number of users per TRxP (e.g. 30) is not precluded.

NOTE5:	Sensing target number per TRxP depends on the use cases and sensing target type, [1] sensing target per TRxP are starting point, other number of sensing targets per TRxP (e.g., 5 or others) is not precluded.

Proposal 9:  Indoor hotspot should be 6G scenario. The values or assumptions in table 3.1-1 and notes are starting point for further discussion.

Dense urban

Dense urban is an important scenario in previous generation study. In this deployment scenario, many key characteristics of this deployment scenario could be reused to evaluate 6G, e.g. high data traffic loads, immersive capacity, outdoor and outdoor-to-indoor coverage, energy saving, positioning performance, sensing-related performance.

The Dense Urban test environment consists of one macro layer. The macro-layer base stations are placed in a regular grid, following hexagonal layout with three TRxPs each, as shown in Fig. 3.2.1 below.

Figure 3.2.1

Sketch of hexagonal site layout

Therefore this scenario deployment can be reused for 6Gand the values or assumptions are described as Table 3.2-1.

Table 3.2-1: Attributes for Dense Urban scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | At least 7GHz. FFS for other frequency. |
| System bandwidth | Around 7GHz: e.g. 400MHz NOTE1,2 |
| Layout | Single layer |
| ISD | 200m NOTE3, |
| TRP antenna elements | Around 7GHz: Up to 3072 Tx and Rx antenna elements |
| UE antenna elements | Around 7GHz: Up to [16] Tx and Rx antenna elements |
| User distribution and UE speed | [10, 30] users per TRxP, 80% indoor (3km/h), 20% outdoor (30km/h) |
| Sensing target distribution and Sensing target speed | [10] sensing targets per TRxP UAV is up to 160km/h, Vehicle is up to 60km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	ISD 200m is the baseline, other number of ISD (e.g., 250 m) is not precluded.

NOTE4:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE5:	[10, 30] users per TRxP is the baseline, other number of users per TRxP (e.g., 20, 50) is not precluded.

NOTE6:	Sensing target number per TRxP depends on the use cases and sensing target type, [10] sensing targets per TRxP are starting point, other number of sensing targets per TRxP (e.g. 20 or others) is not precluded.

Proposal 10:  Dense Urban should be 6G scenario. The values or assumptions in table 3.2-1 and notes are starting point for further discussion.

Rural

Rural is an important scenario in previous generation study. IIn this deployment scenario, many key characteristics of this deployment scenario could be reused to evaluate 6G, e.g. continuous wide area coverage supporting high speed vehicles, energy saving, positioning performance, sensing-related performance.

This scenario will be noise-limited and/or interference-limited, using macro TRxPs. In Rural test environment, the BSs/sites are placed in a grid with sparsity, following prescribed layout with three TRxPs each.

Figure 3.3.1

Sketch of hexagonal site layout

Therefore this scenario deployment can be reused for 6G and values or assumptions are described as Table 3.3-1.

Table 3.3-1: Attributes for Rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | At least 7GHz. FFS for other frequency. |
| System bandwidth | Around 7GHz: e.g. 400MHz NOTE1,2 |
| Layout | Single layer |
| ISD | 1732m NOTE3, |
| TRP antenna elements | Around 7GHz: Up to 3072 Tx and Rx antenna elements |
| UE antenna elements | Around 7GHz: Up to [16] Tx and Rx antenna elements |
| User distribution and UE speed | [10] users per TRxP, 50% outdoor vehicles (120km/h) and 50% indoor (3km/h), |
| Sensing target distribution and Sensing target speed | [5] sensing targets per TRxP UAV is up to 160km/h, Vehicle is up to 120km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	ISD 1732m is the baseline, other number of ISD (e.g., 6000 m) is not precluded.

NOTE4:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE5:	[10] users per TRxP is the baseline, other number of users per TRxP (e.g., 30) is not precluded.

NOTE6:	Sensing target number per TRxP depends on the use cases and sensing target type, [5] sensing targets per TRxP are starting point, other number of sensing targets per TRxP (e.g., 10 or others) is not precluded.

Proposal 11:  Rural should be 6G scenario. The values or assumptions in table 3.3-1 and notes are starting point for further discussion.

Urban macro

Urban macro is an important scenario in previous generation study. In this deployment scenario, many key characteristics of this deployment scenario could be reused to evaluate 6G, e.g. continuous and ubiquitous coverage, energy saving, positioning performance, sensing-related performance.

This scenario will be noise-limited and/or interference-limited, using macro TRxPs. In Rural test environment, the BSs/sites are placed in a grid with sparsity, following prescribed layout with three TRxPs each.

Figure 3.3.1

Sketch of hexagonal site layout

Therefore this scenario deployment can be reused for 6G and the values or assumptions are described as Table 3.4-1.

Table 3.4-1: Attributes for Urban Marco scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | At least 7GHz. FFS for other frequency. |
| System bandwidth | Around 7GHz: e.g. 400MHz NOTE1,2 |
| Layout | Single layer |
| ISD | 500m |
| TRP antenna elements | Around 7GHz: Up to 3072 Tx and Rx antenna elements |
| UE antenna elements | Around 7GHz: Up to [16] Tx and Rx antenna elements |
| User distribution and UE speed | [10] users per TRxP, 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h |
| Sensing target distribution and Sensing target speed | [5] sensing targets per TRxP UAV is up to 160km/h, Vehicle is up to 120km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE4:	[10] users per TRxP is the baseline, other number of users per TRxP (e.g., 30) is not precluded.

NOTE5:	Sensing target number per TRxP depends on the use cases and sensing target type, [5] sensing targets per TRxP are the baseline, other number of sensing targets per TRxP (e.g., 10 or others) is not precluded.

Proposal 12:  Urban macro should be 6G scenario. The values or assumptions in table 3.4-1 and notes are starting point for further discussion.

Sub-Urban macro

In RAN1#116-bis, the conclusion is to study 7-24GHz channel modeling in SMa deployment scenario.

The sub urban deployment scenario focuses on large and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 3.5-1.

Table 3.5-1: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | At least 7GHz. FFS for other frequency. |
| System bandwidth | Around 7GHz: e.g. 400MHz NOTE1,2 |
| Layout | Single layer |
| ISD | 1299m, 1732m |
| TRP antenna elements | Around 7GHz: Up to 3072 Tx and Rx antenna elements |
| UE antenna elements | Around 7GHz: Up to [16] Tx and Rx antenna elements |
| User distribution and UE speed | [10] users per TRxP, 20% Outdoor in cars: 40km/h, 80% Indoor in houses: 3km/h |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the  system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE4:	10 users per TRxP is the baseline, other number of users per TRxP (e.g., 30) is not precluded.

Proposal 13:  Sub-Urban macro should be 6G scenario. The values or assumptions in table 3.5-1 and notes are starting point for further discussion.

Urban Grid

The urban macro deployment scenario focuses on scenario of highly densely deployed vehicles placed in urban area. It could cover a scenario where freeways lead through an urban grid. The main KPI evaluated under this scenario are continuous and ubiquitous coverage, energy saving, positioning performance, sensing-related performance and AI-related performance in high network load and high UE density scenarios.

This scenario will be noise-limited and/or interference-limited, using macro TRxPs. For example, the macro BSs are deployed for Urban case, ISD of macro BS could be 250m or 500 m and the wrap around model as below.

Figure 3.6.1

Sketch of hexagonal site layout with urban grid in 500m ISD

Figure 3.6.2

Sketch of hexagonal site layout with urban grid in 250m ISD

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 3.6-1.

Table 3.6-1: Attributes for Urban Grid scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | At least 7GHz. FFS for other frequency. |
| System bandwidth | Around 7GHz: e.g. 400MHz NOTE1,2 |
| Layout | Single layer |
| ISD | 250m, 500m |
| TRP antenna elements | Around 7GHz: Up to 3072 Tx and Rx antenna elements |
| UE antenna elements | Around 7GHz: Up to [16] Tx and Rx antenna elements |
| User distribution and UE speed | - Pedestrian UE dropping using equally spaced along the sidewalk with a fixed inter-pedestrian X m dropped - Total number of pedestrian UEs is 500 - Pedestrian UE is in the middle of the sidewalk - The inter-pedestrian UE distance (m) (i.e., X) is calculated by ‘A/500’, where ‘A’ is the total length of sidewalk where the pedestrian UEs are dropped under the assumption of ‘N’ road grids (i.e., ‘{(250m – 17m) + (433m – 17m)} * 2 * N’). For example, if the pedestrian UEs are dropped in ‘14’ road grids, the inter-pedestrian UE distance (m) is ‘36.344’. - Companies should explain how many road grids (i.e., ‘N”) are assumed in the evaluation. - Pedestrian UE speed is 3 km/h |
| Sensing target distribution and Sensing target speed | Vehicle is up to 120km/h. how many sensing targets from vehicle distribution per TR 37.885 depend on the evaluation methodology. |
| Environment Objects when sensing enable | At least including 4 walls modelled per building of size 413m x 230m x 20m and/or other sizes, e.g. 40m x 20m x 30m. How many buildings and building types depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE4:	The vehicle type is type 1/2 (passenger vehicle) and/or type 3 (truck/bus)

Proposal 14:  Urban Grid should be 6G scenario. The values or assumptions in table 3.6-1 and notes are starting point for further discussion.

Non-Terrestrial Network scenario

This deployment scenario is defined for non-Terrestrial network. Satellite acts as a fill-in especially on roadways and rural/disaster areas where the terrestrial service isn’t available.

Some of its attributes according to [8] are listed in Table 3.7-1.

Table 3.7-1: Examples for Satellite Deployment

| Attributes | Deployment-1 | Deployment-2 |
| --- | --- | --- |
| Carrier Frequency | Around 2 GHz for both DL and UL | Around 20 GHz for DL Around 30 GHz for UL |
| Duplexing | FDD | FDD/HD-FDD |
| Satellite architecture | Bent-pipe On-board Processing | Bent-pipe, On-Board Processing |
| Typical satellite system positioning in the 6G architecture | Access network | Access network Backhaul network |
| System Bandwidth (DL + UL) | Up to 2*30 MHz | Up to 2*400 MHz |
| Satellite Orbit | GEO, MEO, LEO | GEO, MEO, LEO |
| UE Distribution | 100% Outdoors | 100% Outdoors |
| UE Mobility | Fixed, Portable, Mobile | Fixed, Portable, Mobile |

Proposal 15:  NTN scenario should be supplemental scenario of terrestrial 6G scenario. The values or assumptions in table 3.7-1 are starting point for further discussion.

Summary

We have discussed the principles of deployment scenarios, the proposals are as below:

Proposal 1: both new and existing spectrum shall be considered for evaluation.

Proposal 2: Single carrier in single layer shall be considered as one method for evaluation.

Proposal 3: For new spectrum, ~7GHz with at least 400MHz system bandwidth shall be evaluated.

Proposal 4: For existing spectrum, it is suggested to decide candidate frequency bands, according to operators’ requirements.

Proposal 5: Multi-level of user number per cell shall be considered for 6G evaluation [10, 20, 30, 50].

Proposal 6: The traffic model should be based on the requirement of the service to consider with/without latency requirement, e.g. packet delay budget.

Proposal 7: The max number of TRP antenna element number is up to 3072 for ~7GHz, and the specific TPR antenna element number can be different according to specific deployment scenarios.

Proposal 8: For ~7GHz carrier frequency, UE antenna element number is up to 16 or more, and specific UE antenna element number used in evaluation can be different according to device types.

The potential specific 6G deployment scenarios are also discussed and the proposals are as below:

Proposal 9:  Indoor hotspot should be 6G scenario. The values or assumptions in table 3.1-1 and notes are starting point for further discussion.

Proposal 10:  Dense Urban should be 6G scenario. The values or assumptions in table 3.2-1 and notes are starting point for further discussion.

Proposal 11:  Rural should be 6G scenario. The values or assumptions in table 3.3-1 and notes are starting point for further discussion.

Proposal 12:  Urban macro should be 6G scenario. The values or assumptions in table 3.4-1 and notes are starting point for further discussion.

Proposal 13:  Sub-Urban macro should be 6G scenario. The values or assumptions in table 3.5-1 and notes are starting point for further discussion.

Proposal 14:  Urban Grid should be 6G scenario. The values or assumptions in table 3.6-1 and notes are starting point for further discussion.

Proposal 15:  NTN scenario should be supplemental scenario of terrestrial 6G scenario. The values or assumptions in table 3.7-1 are starting point for further discussion.

References

- RP-250810, “New SID: Study on 6G Scenarios and requirements”, RAN#107, March 2025.

- RP-243325, “DRAFT LTI Response to LS on Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, RAN#106, December 2024.

- RP-250819, “Moderator's summary for IMT-2030 TPRs”, RAN#107, March 2025.

- Recommendation ITU-R M.2160 , “Framework and overall objectives of the future development of IMT for 2030 and beyond”, ITU-R Working Party (WP) 5D.

- RP-234018, “New SID: Study on channel modelling enhancements for 7-24GHz for NR” Nokia, Nokia Shanghai Bell.

- 3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”

- 3GPP TS 38.101-5, “User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance”

Text proposal for the TR

Proposed wording based on the conclusion of this document.

- Deployment scenarios

- 4.0	General

This subsection briefly introduces the six usage scenarios defined by ITU-R IMT for 2030 and beyond [4] is envisaged to expand and support diverse families of usage scenarios and applications that will continue beyond the current IMT. Furthermore, a broad variety of capabilities would be tightly coupled with these intended different usage scenarios and applications for IMT for 2030 and beyond. The families of usage scenarios for IMT for 2030 and beyond include:

-	IC (immersive communication)

-	MC (massive communication)

-	HRLLC (hyper reliable and low latency communication)

-	ISAC (integrated sensing and communication)

-	AIC (AI and communication)

-	UC (ubiquitous connectivity)

- 4.1	Deployment scenarios

- 4.1.1		    Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density, consistent user experience indoor, , positioning performance and sensing-related performance.

Some of its attributes are listed in Table 4.1.1-1.

Table 4.1.1-1: Attributes for Indoor hotspot scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | At least 7GHz. FFS for other frequency. |
| System bandwidth | Around 7GHz: e.g. 400MHz NOTE1,2 |
| Layout | Single layer: - Indoor floor (Open office) |
| ISD | 20m (Equivalent to 12TRxPs per 120m x 50m) |
| TRP antenna elements | Around 7GHz: Up to 768 Tx and Rx antenna elements |
| UE antenna elements | Around 7GHz: Up to [16] Tx and Rx antenna elements |
| User distribution and UE speed | 100% Indoor, 3km/h, [10] users per TRxP, |
| Sensing target distribution and Sensing target speed | 100% Indoor, 3km/h [1] sensing target per TRxP. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study..

NOTE4:	[10] users per TRxP is the baseline, other number of users per TRxP (e.g., 30) is not precluded.

NOTE5:	Sensing target number per TRxP depends on the use cases and sensing target type, [1] sensing target per TRxP are starting point, other number of sensing targets per TRxP (e.g., 5 or others) is not precluded.

- 4.1.2		    Dense urban

The dense urban cellular deployment scenario focuses on macro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage, energy saving, positioning performance and sensing-related performance. This scenario will be interference-limited, using macro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 4.1.2-1.

Table 4.1.2-1: Attributes for Dense Urban scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | At least 7GHz. FFS for other frequency. |
| System bandwidth | Around 7GHz: e.g. 400MHz NOTE1,2 |
| Layout | Single layer |
| ISD | 200m NOTE3, |
| TRP antenna elements | Around 7GHz: Up to 3072 Tx and Rx antenna elements |
| UE antenna elements | Around 7GHz: Up to [16] Tx and Rx antenna elements |
| User distribution and UE speed | [10, 30] users per TRxP, 80% indoor (3km/h), 20% outdoor (30km/h) |
| Sensing target distribution and Sensing target speed | [10] sensing targets per TRxP UAV is up to 160km/h, Vehicle is up to 60km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	ISD 200m is the baseline, other number of ISD (e.g., 250 m) is not precluded.

NOTE4:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE5:	[10, 30] users per TRxP is the baseline, other number of users per TRxP (e.g., 20, 50) is not precluded..

NOTE6:	Sensing target number per TRxP depends on the use cases and sensing target type, [10] sensing targets per TRxP are starting point, other number of sensing targets per TRxP (e.g., 20 or others) is not precluded.

- 4.1.3		    Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles, energy saving, positioning performance, sensing-related performance. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 4.1.3-1.

Table 4.1.3-1: Attributes for Rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | At least 7GHz. FFS for other frequency. |
| System bandwidth | Around 7GHz: e.g. 400MHz NOTE1,2 |
| Layout | Single layer |
| ISD | 1732m NOTE3, |
| TRP antenna elements | Around 7GHz: Up to 3072 Tx and Rx antenna elements |
| UE antenna elements | Around 7GHz: Up to [16] Tx and Rx antenna elements |
| User distribution and UE speed | [10] users per TRxP, 50% outdoor vehicles (120km/h) and 50% indoor (3km/h), |
| Sensing target distribution and Sensing target speed | [5] sensing targets per TRxP UAV is up to 160km/h, Vehicle is up to 120km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	ISD 1732m is the baseline, other number of ISD (e.g., 6000 m) is not precluded.

NOTE4:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE5:	[10] users per TRxP is the baseline, other number of users per TRxP (e.g., 30) is not precluded.

NOTE6:	Sensing target number per TRxP depends on the use cases and sensing target type, [5] sensing targets per TRxP are starting point, other number of sensing targets per TRxP (e.g., 10 or others) is not precluded.

- 4.1.4		    Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage, energy saving, positioning performance, sensing-related performance in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 4.1.4-1.

Table 4.1.4-1: Attributes for Urban Marco scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | At least 7GHz. FFS for other frequency. |
| System bandwidth | Around 7GHz: e.g. 400MHz NOTE1,2 |
| Layout | Single layer |
| ISD | 500m |
| TRP antenna elements | Around 7GHz: Up to 3072 Tx and Rx antenna elements |
| UE antenna elements | Around 7GHz: Up to [16] Tx and Rx antenna elements |
| User distribution and UE speed | [10] users per TRxP, 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h |
| Sensing target distribution and Sensing target speed | [5] sensing targets per TRxP UAV is up to 160km/h, Vehicle is up to 120km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE4:	[10] users per TRxP is the baseline, other number of users per TRxP (e.g., 30) is not precluded.

NOTE5:	Sensing target number per TRxP depends on the use cases and sensing target type, [5] sensing targets per TRxP are the baseline, other number of sensing targets per TRxP (e.g., 10 or others) is not precluded.

- 4.1.5		    Sub-Urban macro

The sub urban deployment scenario focuses on large and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 4.1.5-1.

Table 4.1.5-1: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | At least 7GHz. FFS for other frequency. |
| System bandwidth | Around 7GHz: e.g. 400MHz NOTE1,2 |
| Layout | Single layer |
| ISD | 1299m, 1732m |
| TRP antenna elements | Around 7GHz: Up to 3072 Tx and Rx antenna elements |
| UE antenna elements | Around 7GHz: Up to [16] Tx and Rx antenna elements |
| User distribution and UE speed | [10] users per TRxP, 20% Outdoor in cars: 40km/h, 80% Indoor in houses: 3km/h |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the  system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE4:	10 users per TRxP is the baseline, other number of users per TRxP (e.g., 30) is not precluded.

- 4.1.6		    Urban Grid

The urban macro deployment scenario focuses on scenario of highly densely deployed vehicles placed in urban area. It could cover a scenario where freeways lead through an urban grid. The main KPI evaluated under this scenario are continuous and ubiquitous coverage, energy saving, positioning performance, a and sensing-related performance in high network load and high UE density scenarios.

Some of its attributes are listed in Table 4.1.6-1.

Table 4.1.6-1: Attributes for Urban Grid scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | At least 7GHz. FFS for other frequency. |
| System bandwidth | Around 7GHz: e.g. 400MHz NOTE1,2 |
| Layout | Single layer |
| ISD | 250m, 500m |
| TRP antenna elements | Around 7GHz: Up to 3072 Tx and Rx antenna elements |
| UE antenna elements | Around 7GHz: Up to [16] Tx and Rx antenna elements |
| User distribution and UE speed | - Pedestrian UE dropping using equally spaced along the sidewalk with a fixed inter-pedestrian X m dropped - Total number of pedestrian UEs is 500 - Pedestrian UE is in the middle of the sidewalk - The inter-pedestrian UE distance (m) (i.e., X) is calculated by ‘A/500’, where ‘A’ is the total length of sidewalk where the pedestrian UEs are dropped under the assumption of ‘N’ road grids (i.e., ‘{(250m – 17m) + (433m – 17m)} * 2 * N’). For example, if the pedestrian UEs are dropped in ‘14’ road grids, the inter-pedestrian UE distance (m) is ‘36.344’. - Companies should explain how many road grids (i.e., ‘N”) are assumed in the evaluation. - Pedestrian UE speed is 3 km/h |
| Sensing target distribution and Sensing target speed | Vehicle is up to 120km/h. how many sensing targets from vehicle distribution per TR 37.885 depend on the evaluation methodology. |
| Environment Objects when sensing enable | At least including 4 walls modelled per building of size 413m x 230m x 20m and/or other sizes, e.g. 40m x 20m x 30m. How many buildings and building types depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the  system bandwidth used for either DL or UL via switching in time-domain.

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types and need further study.

NOTE4:	The vehicle type is type 1/2 (passenger vehicle) and/or type 3 (truck/bus)

- 4.1.7		    Non-Terrestrial Network scenario

This deployment scenario is defined for non-Terrestrial network. Satellite acts as a fill-in especially on roadways and rural/disaster areas where the terrestrial service isn’t available.

Some of its attributes according to [8] are listed in Table 3.7-1.

Table 3.7-1: Examples for Satellite Deployment

| Attributes | Deployment-1 | Deployment-2 |
| --- | --- | --- |
| Carrier Frequency | Around 2 GHz for both DL and UL | Around 20 GHz for DL Around 30 GHz for UL |
| Duplexing | FDD | FDD/HD-FDD |
| Satellite architecture | Bent-pipe On-board Processing | Bent-pipe, On-Board Processing |
| Typical satellite system positioning in the 6G architecture | Access network | Access network Backhaul network |
| System Bandwidth (DL + UL) | Up to 2*30 MHz | Up to 2*400 MHz |
| Satellite Orbit | GEO, MEO, LEO | GEO, MEO, LEO |
| UE Distribution | 100% Outdoors | 100% Outdoors |
| UE Mobility | Fixed, Portable, Mobile | Fixed, Portable, Mobile |