3GPP TSG RAN Meeting #108												RP-251367

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source:	Huawei, HiSilicon

Title:	Consideration on 3GPP requirements and KPIs in addition to ITU-R IMT-2030 TPRs

Document for:	Discussion and decision

Introduction

A RAN-level study on “6G Scenarios and requirements” was approved in RAN#107[1]. Regarding the key performance indicators and relevant key assumptions in the scenario, the objectives of this SID mainly include:

- Investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

- Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

  - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting

  - Energy efficiency and energy saving: both for network and device

  - Enhanced spectral efficiency

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

  - Target scalable and forward compatible design for diverse device types

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

In this contribution, the considerations and views on additional 6G requirements and key performance indicators are discussed. The text proposal to the section of 6G requirements and key performance indicators is also provided.

Consideration on 6G requirements and KPIs in addition to ITU-R IMT-2030

This section mainly discusses additional methodologies to evaluate ITU KPI items, new KPIs and new requirements in addition to ITU-R IMT-2030.

The supplementary evaluation methodologies for defined KPIs

Additional consideration for Area traffic capacity

In ITU-R Report M.2410 [2], the area traffic capacity is a measure of how much traffic a network can carry per unit area. It depends on site density, bandwidth and spectrum efficiency. In the case of full buffer traffic and a single layer single band system, it can be expressed as:

Area capacity (bps/m2) = site density (site/m2) × bandwidth (Hz) × spectrum efficiency (bps/Hz/site)

However, full buffer case may not be sufficient for evaluation, which may not completely reflect the traffic in reality. This is why 3GPP defined non-full buffer traffic in TR38.913 [3] and made corresponding evaluation for 5G. The non-full buffer model represents the total traffic throughput served per geographic area (in Mbit/s/m2) and it is seen useful to consider this traffic for 6G. System simulation can be used for detailed evaluation.

Proposal 1: The evaluation of Area traffic capacity should not only consider full buffer traffic, but also non-full buffer traffic with system simulation methodology.

Additional consideration to Cell edge user experienced data rate for DL and UL

In existing deployment, likely users at cell center can have good experience, while users at cell edge may experience degraded performance. With more and more emerging applications, e.g. video, XR, mobile AI etc., the user experience is requested by the consumers to be more consistent regardless whether it is in cell center or cell edge. Therefore the cell edge user experienced data rate becomes one important metric for future 6G deployment.

For example, looking into the immersive communication use cases of TR22.870 [4], the experienced user data rate needs to support high-definition (higher type) and smooth (high frame rate) video transmission. The bitrate requirements from the application layer are very high and can be up to 30~100Mbps for the UL, and 500Mbps for the DL as shown in [7].Therefore the 6G RAN design should consider supporting cell edge data rate targeted up to 100Mbps for the UL and 500Mbps for the DL.

Proposal 2: The target value of cell edge user experienced data rate shall support up to 30~100Mbps for the UL and up to 500Mbps for the DL.

Similarly as the above, although in ITU discussion only full buffer traffic is considered, it is more realistic to consider non-full buffer traffic as defined in 3GPP. The reason is that both BS and UE have transmission power limitation, so it is difficult to increase power with increasing bandwidth linearly. If there is no linear relationship between user experienced data rate and bandwidth, the non-full buffer evaluation methodology and system simulation should be considered, instead of using calculation method. Some traffic also has stringent latency requirements, and this aspect is also good to be reflected into the system simulation.

Proposal 3: The evaluation of cell edge user experience data should not only consider full buffer traffic, but also non-full buffer traffic with system simulation methodology.

Additional consideration to Composite requirement

According TR 38.838[6], eXtended Reality(XR) is a term for different types of realities and refers to all real-and-virtual combined environments and human-machine interactions generated by computer technology and wearables. The Video-based AI inference task is one of the most important media applications under consideration in immersive communication. So, the quantitative KPI should be considered to reflect 6G system capacity.

According to our papers in [7], different immersive communication services require different composite requirements in terms of different combination of service data rates and latency for DL and UL, respectively. Some representative cases are shown in Table 2.1-4

Table 2.1.3-1: Example cases of composite requirement with KPIs

| Examples of composite requirement | data rate (Mbps) | Latency (ms) | Reliability | Number of satisfied users (/km2) |
| --- | --- | --- | --- | --- |
| Composite Requirement for Immersive Services (UL) |  |  |  |  |
| Case 1 | 20Mbps | 15ms | [99%] | TBD |
| Case 2 | 100Mbps | 15ms | [99%] | TBD |
| Composite Requirement for Immersive Services (DL) |  |  |  |  |
| Case 4 | 100Mbps | 10ms | [99%] | TBD |
| Case 5 | 500Mbps | 10ms | [99%] | TBD |

Composite requirement refers to total number of devices fulfilling a target immersive communication QoS per unit area (per km2) with a large percentage of UEs being satisfied, where the target QoS is to ensure the user experience by considering different QoS requirements, e.g. combined data rate, packet transmission delay and reliability requirement. By having such a definition, both the requirements for user experience and network capacity can be reflected at the same time, which can serve more advanced services in the future. The system level simulation need to be used to evaluate such a composite requirement.

Proposal 4: The evaluation of composite requirement for immersive communication should be supported by system simulation, with composite requirement of the number of satisfied users, by meeting the different QoS of data rate, latency and reliability per user.

For immersive communication, as shown in the above table, the data rate and latency may vary according to different use cases. With more emerging new immersive services, there might be more different values for data rate and latency as described in [7]. In this case it may be too stringent if the radio transmission has to always satisfy the minimized latency requirement. If RAN and CN can have a better delay budget negotiation, this may give flexibility for the network to schedule resources in a more efficient way.

As also introduced in in [7], a smooth user experience for interactive services relies on whether the end-to-end round-trip time (RTT) is met, e.g., 300ms. If the knowledge of the RTT outside 3GPP network is given, the delay budget of air interface and transport can be jointly considered to meet the overall E2E RTT. How to evaluate reliability may also need further discussion according to [7].

Proposal 5: 6G should consider dynamic latency budget coordination among different layers of the network to jointly meet the requirement of E2E RTT.

Energy saving for Network and UE

Energy saving is very important for both network and UE sides. The quantitative KPI should be considered to better reflect the gains from 6G. Network energy saving was studied from 3GPP R18, according to TR 38.864 [8], the BS power consumption model and BS power consumption method in TR 38.864 can be reused to evaluate 6G technologies for FR1 and FR2. UE power saving was studied from 3GPP R16, according to TR 38.840 [9] the UE power consumption model and UE power consumption method in TR 38.840 can be reused to evaluate 6G technologies for FR1 and FR2.

For FR3 bands, the values in these tables in TR 38.864 (BS) and TR 38.840 (UE) may not be able to be fully reused, which need to be studied and updated accordingly.

In addition, the current evaluation on network energy saving is based on the UE perceived throughput (UPT), which requires the UE to be served as soon as possible when the service triggers. This leads to the consequence that the network has to schedule and transmit anytime when the UE has a request, and consumes much power accordingly. In reality as long as the QoS requested by the UE is satisfied, the user experience can be achieved and this does not necessarily require the network to schedule or transmit immediately. The following figure shows the difference on energy consumption between satisfying UPT and satisfying QoS options, it can been seen transmission by satisfying QoS could leave more flexibility for the network on when to schedule UEs and transmit, and can avoid unnecessary power consumption from the network side.

Figure 2.1.4-1:  comparison between network transmission only for UPT and for QoS satisfaction

Thereby, when evaluating the energy saving, it is more reasonable to consider satisfying the QoS requested by the UE instead of considering throughput/UPT as the target. In other words, the network energy saving should be evaluated under the user’s QoS satisfaction.

For UE energy saving, as defined in TR38.840, the evaluation of energy saving uses separate assessment of energy consumption for each traffic type e.g., FTP, IM and VoIP. For a single user, the UE may handle different types of traffic with varying QoS requirements over a long period of time. Such evaluation of energy saving is seen useful to reflect the realistic situation and could also be taken this into account in 6G.

Overall the system level simulation is used as the evaluation methodology for both network and UE energy saving.

Proposal 6: Energy saving should be 6G quantitative KPI for both network and UE, and should be evaluated by system simulation methodology.

Proposal 6a: The power consumption model/assumptions need to be studied considering:

- BS and UE power consumption model for FR3

- Energy saving metric based on QoS satisfaction

Consideration on various device types

In previous RAN plenary discussion there is common understanding that the 6G RAN needs to support multiple devices types within a single framework. In this case, it is very important to understand the specific design targets for low-end UEs and high-end UEs.

For example, whether the low-end UEs support bandwidth smaller than 5MHz (like NB-IoT), or bandwidth like (e)RedCap (e.g. 5MHz, 20MHz) would impact on the initial access for the bandwidth selection. Whether coverage extension needs to be supported and how much extension is required also matters (e.g. 154dB, 164dB), as the coverage extension would require physical layer repetition. If the number of repetition becomes high, especially for those common signaling like SSB, SIB etc., this may also impact the performance for eMBB or more advanced devices.

On the other hand the marketing needs should also be taken into account. In different regions, the existing deployment situation and trends for IoT services, e.g. (e)RedCap, eMTC, NB-IoT, as well as the potential bands for future IoT services are important factors to justify the needs on how much low-end devices need to be supported in 6G Day One.

Proposal 7: Design targets for channel bandwidth, coverage and data rate should be clearly defined for low-end and high-end UEs.

Consideration on coverage

Coverage is one important KPI for operators to deploy the network with good balance between the performance and cost.

From users perspective, based on the discussion in Sec 2.1.2, it is not sufficient to consider coverage itself as a standalone KPI, the cell edge data rate should be considered together to provide consistent user experience everywhere.

From operators perspective, the coverage of new spectrum, e.g. ~7GHz, should achieve similar range as existing bands, e.g. C-band, which can enable co-site deployment of new spectrum and reduce the cost of building up new sites.

In addition, the diverse device types may have different coverage requirement according to different use cases as discussed in Sec 2.2.

Therefore the coverage should be considered as one additional requirement and evaluated with the considerations on cell edge data rate, deployment cost and diverse device types. Link budget methodology can be used for evaluations.

Proposal 8: Coverage should be considered as the quantitative KPI with link budget evaluation methodology, considering

- ~7GHz shall reach same coverage as existing C-band to enable co-site deployment

- Coverage requirement should also support certain cell edge data rate to ensure consistent user experience

- Coverage requirement should take diverse device types into account

Consideration to the KPI and requirement of sensing

In TS 22.137[11], the service requirements for Integrated Sensing and Communication were discussed in 3GPP R19. The requirements of most ISAC detection services include detection probability, localization accuracy (in terms of horizontal and vertical accuracy), and velocity accuracy (in terms of horizontal and vertical velocity accuracy). Besides, according to the ISAC channel model and scenario definition in R19, these capabilities should be considered as quantitative KPIs for 6G ISAC. This has been addressed in IMT-2030 as well.

Proposal 9: The ISAC KPI definitions and quantitative evaluation methodology of the items below shall be supported for 6G.

- Sensing detectability

- Sensing localization accuracy

- Sensing velocity accuracy

Sensing reconstruction accuracy

According to TR22.870 [4], the environmental object reconstruction as one of 6G ISAC use case is agreed. The 3GPP ISAC provides a non-invasive dual-functionality of communication and sensing data collection of environmental features for reconstruction. This sensing data collection is expected to unlock new services and support enhanced performance, deployment utilization, energy, and spectral efficiencies due to the nature of ISAC and its wide-area coverage and radio-signal availability.

Based on ISAC channel modeling in Chair notes RAN1#116b, the large sensing targets represented by multiple scattering points were agreed, e.g., buildings or trucks. For static environmental objects, such as outdoor building/city construction and indoor machinery/wall, 3GPP wireless sensing is effective to provide a wealth of low-cost environmental information for both wide-areas and certain areas. Additionally, for monitoring dynamic targets of interest, such as vehicles, this is essential for enhancing environmental perception, especially in scenarios where non-line-of-sight (NLOS) conditions or poor visibility may limit traditional sensing methods.

Figure 2.4.1-1:  Ground truth scattering points and sensed scatter points

Reconstruction accuracy refers to the closeness of the reconstructed scatter points set to the real scatter points set of sensing target, measured by e.g. Root Mean Square Error (RMSE) and can be defined as the following:

Reconstruction accuracy =1Ni(GTi-Si)2

where N is the number of multiple scattering points of the sensing target, and GTi and Si are the ground truth location and the sensed location of ithscattering point of the sensing target, respectively.

System-level and/or link-level simulation are to be performed for the evaluation of reconstruction accuracy.

Proposal 10: Reconstruction accuracy should be defined as a 6G quantitative KPI for sensing.

Environment-awareness to assist communication

In current system, we have not assumed prior knowledge of environmental information, so the system has to be designed based on the unknown channel condition. It will introduce unnecessary system overhead or lead to non-optimized scheduling algorithm, since the system only relies on reference signal to measure spatial channel information. Besides, it remains difficult to predict future channel conditions for the BS because BS cannot comprehensively reconstruct precise environmental information based solely on sporadic measurement reports. Therefore, this requires 6G RAN to obtain real-time wireless spatial channel information for overcoming this issue.

Figure 2.4.2-1	From original communication to sensing for communication

When the environment information is well known by 6G RAN, 6G RAN can use this information as prior knowledge to facilitate communication such as channel acquisition and prediction, MIMO operation, and etc. to achieve better performance with reduced overhead and power consumption.

To achieve the goal in universal area, 6G RAN could utilize the data from sensing, and then use the relevant information to provide better user experience for communication, such as higher data rate, lower latency, etc.

Proposal 11:  Sensing functions should be supported to improve communication performance by utilizing environment awareness information, e.g. to reduce overhead and power consumption.

Environment-awareness to provide positioning

Positioning is studied from Release 16 to Release 18, as below.

- TR 38.855, Study on NR Positioning Support.

- TR 38.856, Study on local NR positioning in NG-RAN.

- TR 38.857, Study on NR positioning enhancements.

As described in TR 38.857, the impact of NLOS/multipath on NR positioning has been investigated, the performance could be improved assuming the NLOS impact is mitigated by algorithms.

Figure 2.4.3-1	NLOS positioning challenges

However, in fact, the NLOS/multipath will lead to poor positioning accuracy and even positioning failure in heavy NLOS such as urban city canyon, indoor factory, etc. It is very difficult to optimize algorithm to improve the performance without any prior information regarding NLOS/multipath. As described in section 2.4.1, the network could utilize the local digital twin or environmental object to identify the NLOS/multipath information for positioning.

To enable positioning services in universal area under NLOS/multipath conditions, e.g. Dense Urban, 6G RAN could utilize sensing to do (sub) meter high positioning accuracy with a large percentage confidence interval everywhere.

Sensing can also be considered for more use cases. For example, sensing-based motion monitoring has been proposed since Rel-19 in TS 22.137 for services like respiration rate estimation in sleep monitoring, and sit-up rate estimation in sports monitoring. Such services as described in [7] can provide significant data for people’s health monitoring, which is becoming a new trend nowadays. The potential requirements for sensing functions on these use cases can also be further discussed.

Proposal 12:  Sensing functions should be supported to provide better positioning performance, e.g. (sub)meter level under NLOS conditions, by utilizing environment awareness information.

Requirements of AI

As in TR 22.870[4], the system performance optimization using AI in section 6.3 is one of the key AI use cases for 6G. This use case addresses optimization of communication between terminals and network through implementing and using the AI capabilities for the network functions (AI-enabled).In this case, 6G base stations can further enhance not only for communication but also include sensing, positioning, and other 6G system capabilities. For instance, the prior information could be utilized to do wireless channel predication and multipath identification for better channel estimation, enhanced beamforming, interference suppression and etc., which can help to enhance system efficiency and user experience.

Proposal 13:  6G RAN design should support AI capabilities to enhance communication performance and user experience.

To support AI use case of collaborative AI Agents and ISAC use case of collaborative robots using digital twinning, the 6G RAN would process communication, sensing, and AI capabilities simultaneously. As elaborated in [7], the new data management and procedure should be considered to collect and manage data for better utilization of various types of data.

Proposal 14: A E2E unified framework between RAN and Core network for data collection, management and monitoring should be supported, considering various types of data or traffic related to measurements, AI and sensing etc.

Summary

We have discussed the principle to address key performance indicators in 6G. We suggest to define the additional or supplementary evaluation methodologies for ITU-R TPRs, or introduce new KPIs in 6G Immersive communication.

Proposal 1: The evaluation of Area traffic capacity should not only consider full buffer traffic, but also non-full buffer traffic with system simulation methodology.

Proposal 2: The target value of cell edge user experienced data rate shall support up to 30~100Mbps for the UL and up to 500Mbps for the DL.

Proposal 3: The evaluation of cell edge user experience data should not only consider full buffer traffic, but also non-full buffer traffic with system simulation methodology.

Proposal 4: The evaluation of composite requirement for immersive communication should be supported by system simulation, with composite requirement of the number of satisfied users, by meeting the different QoS of data rate, latency and reliability per user.

Proposal 5: 6G should consider dynamic latency budget coordination among different layers of the network to jointly meet the requirement of E2E RTT.

Proposal 6: Energy saving should be 6G quantitative KPI for both network and UE, and should be evaluated by system simulation methodology.

Proposal 6a: The power consumption model/assumptions need to be studied considering:

- BS and UE power consumption model for FR3

- Energy saving metric based on QoS satisfaction

Proposal 7: Design targets for channel bandwidth, coverage and data rate should be clearly defined for low-end and high-end UEs.

Proposal 8: Coverage should be considered as the quantitative KPI with link budget evaluation methodology, considering

- ~7GHz shall reach same coverage as existing C-band to enable co-site deployment

- Coverage requirement should also support certain cell edge data rate to ensure consistent user experience

- Coverage requirement should take diverse device types into account

Besides, we suggest to introduce new KPIs in 6G ISAC.

Proposal 9: The ISAC KPI definitions and quantitative evaluation methodology of the items below shall be supported for 6G.

- Sensing detectability

- Sensing localization accuracy

- Sensing velocity accuracy

Proposal 10: Reconstruction accuracy should be defined as a 6G quantitative KPI for sensing.

Proposal 11:  Sensing functions should be supported to improve communication performance by utilizing environment awareness information, e.g. to reduce overhead and power consumption.

Proposal 12:  Sensing functions should be supported to provide better positioning performance, e.g. (sub)meter level under NLOS conditions, by utilizing environment awareness information..

Finally, we suggest to define the new requirement on 6G AI functions.

Proposal 13:  6G RAN design should support AI capabilities to enhance communication performance and user experience.

Proposal 14: A E2E unified framework between RAN and Core network for data collection, management and monitoring should be supported, considering various types of data or traffic related to measurements, AI and sensing etc.

References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, 3GPP RAN#107, March, 2025.

- ITU-R Report M.2410, Minimum requirements related to technical performance for IMT-2020 radio interface(s)

- TR38.913, Study on scenarios and requirements for next generation access technologies

- TR22.870, Study on 6G Use Cases and Service Requirements

- https://support.google.com/youtube/answer/1722171?hl=en-GB

- TR38.838, Study on XR (Extended Reality) evaluations for NR

- RP-25xxxx, Consideration on new and enhanced services for 6G, Huawei, 3GPP RAN#108, June, 2025.

- TR38.864, Study on network energy savings for NR

- TR38.840, Study on User Equipment (UE) power saving in NR

- S1-252898, Use case on joint energy saving for network and UE with various loads, Huawei, 3GPP RAN1#110, May, 2025

- TS22.137, Integrated Sensing and Communication

- TR38.830, Study on NR coverage enhancements

Text proposal for the TR

Proposed wording based on the conclusion of this document.

- 5.1 Requirements and key technical principles

This section describes the definitions of all KPIs and requirements.

The KPI targets defined in this section as well as in Report ITU-R M.[IMT-2030. TECH PERF REQ] shall be supported by next generation access technologies. The evaluation methodology defined in Report ITU-R M.[IMT-2030.EVAL] shall be used to evaluate the targets specified in Report ITU-R M.[IMT-2030. TECH PERF REQ].

- 5.1.x1	Area traffic capacity

Area traffic capacity means total traffic throughput served per geographic area (in Mbit/s/m2). This metric can be evaluated by two different traffic models: Full buffer model and Non full buffer model

-	By full buffer model: Total traffic throughput served per geographic area (in Mbit/s/m2). The computation of this metric is based on full buffer traffic.

-	By non-full buffer model: Total traffic throughput served per geographic area (in Mbit/s/m2). Both the user experienced data rate and the area traffic capacity need to be evaluated at the same time using the same traffic model.

NOTE1:	Non-full buffer simulations are preferred for the evaluation of this KPI.

In the case of non-full buffer traffic and a single layer single band system, system simulation methodology is used.

- 5.1.x2	Cell edge user experienced data rate

User experienced data rate can be evaluated for non-full buffer traffic.

For non-full buffer traffic, user experienced data rate is the 5%-percentile (5%) of the user throughput. User throughput (during active time) is defined as the size of a burst divided by the time between the arrival of the first packet of a burst and the reception of the last packet of the burst.

For non-full buffer evaluation, system simulation methodology is used.

- 5.1.x3	Composite requirement

The composite requirement is defined as the number of users, satisfying the QoS of data rate and latency at the same time.

For immersive communication, the requirements are as follows:

-	Case 1: UL data rate = [20 and/or 100]Mbps, Reliability = 1-10-2, and user plane latency = 15 msec

-	Case 2: DL data rate = [500]Mbps, Reliability = 1-10-2, and user plane latency = 10 msec

System simulation methodology is used.

To provide more flexibility for the network to efficiently manage resource allocations, a more dynamic latency budget coordination among different layers of the network can be considered to jointly meet the requirement of E2E RTT

Editor’s note:	details on defining data rate, latency and reliability is FFS.

- 5.1.x4	 Network Energy saving

The capability is to minimize the RAN energy consumption while providing a much better area traffic capacity. The quantitative KPIs is used.

Network energy saving shall be considered as a basic principle in the 6G design and be evaluated based on the assumption that the QoS is satisfied by users.

System simulation methodology is used.

Editor’s note: Details on how to define energy consumption per QoS request is FFS.

- 5.1.x5	UE Energy saving

UE energy efficiency means the capability of a UE to sustain much better mobile broadband data rate while minimizing the UE modem energy consumption. The quantitative KPIs is used.

UE energy saving shall be considered as a basic principle in the 6G design.

System simulation methodology is used.

Editor’s note: Details on how to define energy consumption per QoS request is FFS.

- 5.1.x6	 Sensing Requirements

Sensing reconstruction:

Sensing reconstruction accuracy is a capability to detect and track the characteristics of individual environmental object. The metric for assessing the reconstruction accuracy refers to the closeness of the reconstructed scatter points set to the real scatter points set of sensing target, measured by e.g. Root Mean Square Error (RMSE).

Reconstruction accuracy =1Ni(GTi-Si)2

where N is the number of multiple scattering points of the sensing target, and GTi and Si are the ground truth location and the sensed location of ithscattering point of the sensing target, respectively.

Other requirements for sensing

The sensing functions are not only providing object detection, and can also provide environment awareness etc. The data from sensing can be utilized to improve communication performance, e.g. to reduce overhead and power consumption by reducing measurements.

The sensing functions can also provide useful information to aware the environment, and identify the NLOS/multipath information better. This can be utilized to improve the positioning performance especially under NLOS conditions.

Editor’s note: The parts for Sensing detectability, Sensing localization accuracy, Sensing velocity accuracy are assumed to be added by IMT-2030 discussion.

- 5.1.x7	 AI Requirements

6G RAN should support AI capabilities to enhance communication performance and user experience. For instance, the prior information could be utilized to do wireless channel predication and multipath identification for better channel estimation, enhanced beamforming, interference suppression and etc., which can help to enhance system efficiency and user experience.

To better utilizing data collected for different purposes, e.g. from channel measurements, AI, sensing etc. it is good to have a unified framework for data collection and management. In this case multiple types of data can be better managed and used by customers.