3GPP TSG RAN Meeting #108	RP-251382

Prague, Czech, June 9-13, 2025

Agenda item:	15.1.3

Source:	CMCC

Title:	Consideration on Ambient IoT for Rel-20

Document for:	Discussion

# Introduction

The Rel-19 Ambient IoT WI has been progressed on schedule. RAN1 objective of Rel-19 Ambient IoT has been completed. And a follow up Rel-20 WI for Ambient IoT solutions is expected. This contribution addresses the scopes that need to be considered in the Rel-20 Ambient IoT WI/SI.

# Discussion

## Application scenario for Rel-20 Ambient-IoT

Figure 1. Wide-area Ambient IoT application scenarios

Wide-area IoT supervision

By collecting information on the quantity and status of objects in fixed local environments, and then integrating multiple discrete fixed local scenarios into a wide-area scene, it enables wide-area supervision applications at the city and even national levels.

Wide-area Tracking

The application involves locating and inventorying various highly mobile objects in continuous wide-area environments, enabling the tracking and tracing of managed objects within specific spatiotemporal ranges.

## High-level consideration of Rel-20 A-IoT

Background for R20 A-IoT
In RAN#106, part of the work plan of Rel-20 Ambient IoT has been discussed as follows [7],

- Includes D2T2

- Includes Device 2

- A limited number of additional use cases, deployments, connectivity topologies, devices, traffic types in TR38.848 as a starting point

- Note: work on D2T2, device 2 shall follow the Rel-19 study conclusions, and further down-selection of architecture options for topology 2 may be needed. For indoor use cases, deviations from Rel-19 study conclusion shall be well justified and agreed.

- Details to be discussed at a later RAN plenary meeting, including the need for study phase.

For both Rel-19 and Rel-20, co-ordination with SA2/3 is expected.

High level timeline for R20 A-IoT

Figure 2. Rel-19/20 Ambient IoT WI timeline

Ambient IoT is an important 5G-A technology covers both indoor and outdoor solutions. Rel-19 A-IoT is mainly focused on indoor scenarios. For outdoor scenarios, which gains interests in the future, this can also be included in Rel-20 WI. The step-by-step release of specifications for A-IoT effectively supports market utilization.

Proposal 0-1:

- Rel-20 Ambient SI/WI from 25Q3 to 26Q4 (18 month), includes both outdoor and indoor scenarios

For outdoor scenarios, if device 2b or device C is introduced, a study phase is needed to ensure the coverage and coexistence is feasible.

Proposal 0-2:

- A study phase is needed to ensure the coverage and coexistence is feasible if device 2b or device C is introduced in Rel-20 Ambient IoT.

Device 2b and C definition

During the RAN-P and WG SI, the following definitions of device 2b and C are concluded,

In TR38.848 [2], device C is captured as follows,

Ambient IoT devices are characterized in the study according to their energy storage capacity, and capability of generating RF signals for their transmissions. The study considers that a device has either: - No energy storage at all; or - Limited energy storage Relying on these storage capacities, the study considers the following set of Ambient IoT devices: - Device A: No energy storage, no independent signal generation/amplification, i.e. backscattering transmission. - Device B: Has energy storage, no independent signal generation, i.e. backscattering transmission. Use of stored energy can include amplification for reflected signals. - Device C: Has energy storage, has independent signal generation, i.e., active RF components for transmission. A limited energy storage can be different among implementations within Device B or implementations within Device C, and different between Device B and Device C. Such storage is expected to be order(s) of magnitude smaller than an NB-IoT device would typically include. Device A, B, and C are able to demodulate control, data, etc from the relevant entity in RAN according to connectivity topology.

In TR38.769 [8] section 3.1, device 2b is captured as follows,

Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device.

Proposal 0-3: Both device 2b and C are considerd to be study for Rel-20 Ambient IoT.

Rel-20 Ambient IoT and 6G LPWA

Rel-20 A-IoT device are not replacement of LPWA technologies. Differentiation in market positioning is necessary. It can be reflected in aspects such as coverage, power consumption, cost, and complexity.

## Overview of Rel-20 Ambient IoT WI scopes

| Features |  | Status | Enhancement in Rel-20 | Justification | Proposals |
| --- | --- | --- | --- | --- | --- |
| General aspects |  |  |  |  |  |
| Deployment scenarios and topology |  | Only Topology 1 and A1/A2/B supported in Rel-19. | - In additional to topology 1, support topology 2 - D4 in additional to D1/D2 | 2.4.1 | Proposal 1-1a: D4T1 and D4T2 are recommended for Rel-20 WI. Proposal 1-2: Considering topology 1 and topology 2 are targeting to different deployment scenarios both of which are essential, both topology 1 and topology 2 are recommended for Rel-20 ambient IoT. |
| Device type |  | Device 1 | Study of device C is needed | 2.4.2 | Proposal 1-5: For Rel-20 WI, Study of the Device C and /or Device 2b with IF/ZIF device architecture for R2D should be considered. Strive for common design for Device C and /or Device 2b as much as possible. |
| Use case |  | Rel-19 SI only covers indoor inventory and command use cases | In addition to support - Sensor - Positioning (study or down-selection needed) | 2.4.1 | Proposal 1-1b: In addition to support - Sensor - Positioning (study or down-selection needed) |
| Traffic type |  | Rel-19 has focused on DO-DTT and DT. DO-A has been concluded not to be supported in Rel-19. | In addition to support DO-A | 2.4.1 | Proposal 1-3: study should include supporting DO-A traffic for device 2b/C for Rel-20 ambient IoT. |
| Device (un)availability |  | Device 1 is supported in Rel-19 and Direction1 is considered for device 1. | Necessary solutions to extend device’s sustainable operation time for device 2, e.g., Direction 2 | 2.4.7 | Proposal 1-12: Study and specify of device (un)availability direction 2 for device 2 for Rel-20 due to energy harvesting is recommended. |
| Multi-cell deployment |  | For device C for outdoor deployments, multiple cell deployment should be considered. This is not considered so far. | Related signal/channels design for supporting multiple cells deployment. |  |  |
| RAN 1 related aspects |  |  |  |  |  |
| Device 2b/C coverage |  | Only device 2b for indoor coverage studied. | Study and evaluate the coverage of device 2b/C for outdoor deployment. | 2.4.9 | Proposal 1-15: Study and evaluate the coverage of device 2b/C for outdoor deployment. |
| Physical layer signals/channels and procedures for device 2b/C | Initial access procedure, e.g., synchronization | Not completed | For device C, study the followings - Syncronization and initial acess procedures, e.g., new signals for R2D synchronization | 2.4.3 | Proposal 1-6: At least for device C, study synchronization and initial access procedures, e.g., new signals for R2D synchronization. |
|  | Active transmission in D2R for device type 2b/C | Device architecture has been studied in the Rel-19 SI. How to work for active transmission with low power consumption and cost is not studied. | For device C, study the followings, - CFO calibrate for better UL Tx/Rx - Neccesasry signals/channels for supporting DOA | 2.4.4 | Proposal 1-7: For device 2b/C, related studies to enable better active transmission by considering clock inaccuracy, e.g., CFO, is recommended for Rel-20 ambient IoT. |
|  | Frame structure |  | Study of the frame structure | 2.4.5 |  |
|  | Modulations | Not completed | Study modulation schemes including, - OOK for R2D - Phase modulation for D2R | 2.4.5 | Proposal 1-10: Consider at least the following modulation schemes for Rel-20 device 2b/C including, - OOK for R2D - Phase modulation for D2R |
|  | Coding | Not completed | - Study FEC for both R2D and D2R - no need for line code | 2.4.5 | Proposal 1-8: Study of the channel coding schemes for both R2D and D2R link for Rel-20 device 2b/C. |
|  | Multiple Access | Not completed | Study TDMA and/or FDMA | 2.4.5 |  |
|  | HARQ/Scheduling | Not completed | Study the followings - Support of HARQ - Scheduling | 2.4.5 | Proposal 1-9: Study of the scheduling and HARQ for both R2D and D2R link for Rel-20 device 2b/C. |
|  | Power control | Not completed | Study the followings - D2R power control - Device power class, i.e., maximum Tx power | 2.4.5 | Proposal 1-20: Study the followings - D2R power control - Device power class, i.e., maximum Tx power |
| Positioning |  | Solution 1 is purely reader implementation, and no specification impact is expected. On the contrary, solution 2 is dependent on measurements at the reader side, and therefore further discussion and specification work on the measurement may be required. Due to the limited time for Rel-19 normative work, Solution 1 is recommended for Rel-19 A-IoT normative work. | Target for backscattered devices positioning enhancements and simple positioning solutions based on existing mechanism for active devices (i.e., device 2b/C) | 2.4.8 | Proposal 1-13: For Rel-20, study backscattered devices positioning enhancements - study requriements of the positioning by backscattering. - study potential positioning solutions and specify nessasary signals/procedures/interfaces to support. Proposal 1-14: For Rel-20, study simple positioning solutions based on existing mechanism for active devices (i.e., device 2b/C) |
| Topology 2 resource allocation |  | Option 1 and option 2 have been studied. | Minimize RAN1 impact for topology 2 resource allocation. | 2.4.1 | Proposal 1-4: For resource allocation and/or controlling of the intermediate UE in topology 2, minimized RAN1 specification impact is pursued, e.g., Higher-layer signaling is used. |
| RAN 2 related aspects |  |  |  |  |  |
| High layer aspects for supporting Device 2b/C in Topology 1 |  | In Rel-19 Ambient IoT, in order to common design for all device types, quite a lot of functionalities are NOT supported, e.g., RRC, PDCP, system information and MIB, HARQ, mobility and etc. | Study A-IoT radio interface high layer protocol stack, functionalities and procedures for supporting Device 2b/C in Topology 1, including, - Layer 2/3 Protocol stack and functionalities of sublayer(s) to support device 2b/C - system information, paging, DoA RACH procedure and data transmission procedure for device 2b/C - necessity and feasibility of mobility for device 2b/C |  | Proposal 1-16: Study A-IoT radio interface high layer protocol stack, functionalities and procedures for supporting Device 2b/C in Topology 1, including, - Layer 2/3 Protocol stack and functionalities of sublayer(s) to support device 2b/C - system information, paging, DoA RACH procedure and data transmission procedure for device 2b/C necessity and feasibility of mobility for device 2b/C |
| Topology 2 for device 2b/C/1 |  | RRC based solution, NAS based solution and UP based solution are studied in Release 19. For the radio resources allocation, Alternative a and Alternative 2 are studied in TR 38.769. | Study topology 2 for device 2b/C/1 - down-selection or support both of RRC/UP based solutions for topo 2 - Uu interface signaling and procedures to support UE-reader - necessity and feasibility of coordination between UE-readers - Note: A-IoT interface for topo 1 should be reused as much as possible2 |  | Proposal 1-17: Study topology 2 for device 2b/C/1 - down-selection or support both of RRC/UP based solutions for topo 2 - Uu interface signaling and procedures to support UE-reader - necessity and feasibility of coordination between UE-readers - Note: A-IoT interface for topo 1 should be reused as much as possible2 |
| RAN 3 related aspects |  |  |  |  |  |
| NG interface enhancement for supporting Device 2b/C in Topology 1 |  | RAN3 have studied the architecture and protocol stack for support of Ambient IoT embedded in the overall 5G system architecture for topology 1 and topology 2. | Study NG interface signaling and procedures for supporting Device 2b/C in Topology 1 | 2.4.6 | Proposal 1-11: Study RAN architecture enhancement to support device 2b/C/1 in topology 2, including, - Down-selection or support both of RRC/UP based solutions for topo 2. - Note: NAS based solution is not recommended by SA2 - Specify the NG interface procedures and signalling for topo 2. Note: A-IoT interface for topo 1 should be reused as much as possible |
| Topology 2 for device 2b/C/1 |  | For topology 2, NAS based solution has been ruled out by SA2, so no need to further discuss in RAN WGs. | Study RAN architecture enhancement to support device 2b/C/1 in topology 2, including, - Down-selection or support both of RRC/UP based solutions for topo 2. - Note: NAS based solution is not recommended by SA2 - Specify the NG interface procedures and signalling for topo 2. - Note: A-IoT interface for topo 1 should be reused as much as possible |  |  |
| RAN 4 related aspects |  |  |  |  |  |
| Co-existence |  | Outdoor coexistence are not studied | Study outdoor ambient IoT and NR/LTE co-existence is required if it targets for outdoor |  | Proposal 1-18: Study outdoor ambient IoT and NR/LTE co-existence is required if it targets for outdoor |

## Discussion of Rel-20 Ambient IoT WI scopes

### Deployment scenarios and topology

It is discussed in RAN#106 that, Rel-20 Ambient IoT includes D2T2, Device 2. A limited number of additional use cases, deployments, connectivity topologies, devices, traffic types in TR38.848 as a starting point.

For the new use cases and deployments, according to the application scenarios described in section 2.1, the outdoor scenario is of interests. It is also captured in TR38.848 as follows. D4 includes T1 and T2 which is in the consideration.

4.2.2.4 Deployment scenario 4: Device outdoors, basestation outdoors With Ambient IoT device outdoors and basestation outdoors, this deployment scenario is characterized according to Table 4.2.2.4-1. Table 4.2.2.4-1: Characteristics of deployment scenario 4 Applicable representative use cases; Characteristics; Description (NOTE 1). Outdoor inventory Outdoor sensor Outdoor positioning Outdoor command; Environment (of device); Outdoor. Basestation characteristic (if any); Macro- or Micro- cell BS. Connectivity topology; Topology (1), (2), (3). Spectrum; Licensed FDD, licensed TDD, or unlicensed.. Coexistence with existing 3GPP technologies; Co-site or new site. Traffic assumption; DT and DO. Device characteristic; Device C may support Topology (1), (2), (3), Device A may support Topology (2),Device B may support Topology (2), (3). NOTE 1: Descriptions may not be applicable for some Devices (A, B).

Proposal 1-1a: D4T1 and D4T2 are recommended for Rel-20 WI.

Proposal 1-1b: In addition to support

- Sensor

- Positioning (study or down-selection needed)

RAN1/2/3/4 have studied both topology 1 and topology 2 during SI. RAN2 has concluded that both are feasible. RAN 1 also studied schemes for topology resource allocation. Considering topology 1 and topology 2 are targeting to different deployment scenarios both of which are essential, we propose both topology 1 and topology 2 are recommended to normative work.

Proposal 1-2: Considering topology 1 and topology 2 are targeting to different deployment scenarios both of which are essential, both topology 1 and topology 2 are recommended for Rel-20 ambient IoT.

For active device, e.g., device 2b/C, device-originated autonomous (DO-A) traffic is feasible, which reduces periodic monitoring that increase the device power consumption. Random access signal/channel can be designed to enable such mechanism.

Proposal 1-3: study should include supporting DO-A traffic for device 2b/C for Rel-20 ambient IoT.

RAN1 have studied two options for resource allocation and/or controlling of the intermediate UE in topology 2 for R2D and D2R transmissions via the Uu interface between the BS and intermediate UE:

- Option 1: Higher-layer signalling is used.

- Option 2: L1 and higher-layer signalling are used.

Option 1 has less interaction between physical and higher layer, which lead to less specification impact. For topology 2 resource allocation and/or controlling, RAN1 specification impact should be minimized, and Option 1 is preferred.

Proposal 1-4: For resource allocation and/or controlling of the intermediate UE in topology 2, minimized RAN1 specification impact is pursued, e.g., Higher-layer signaling is used.

### Device types

For device 2b, if IF/ZIF is supported in Rel-20 WI, there should be additional specification impact for device to determine the initial access carrier frequency.

For Rel-20 target for outdoor, Device C IF/ZIF should be considered for better coverage.

The differences between device C and deivce 2b incluldes,

- Device C is targeting for outdoor scenario, while device 2b is targeting for indoor scenario

- Device C can consider higher D2R Tx power thatn device 2b.

It is expected to have common design for device 2b and C as much as possible. Hence minimum specification impact is expected, at the same it can avoid split of the device type in the market.

Proposal 1-5: For Rel-20 WI, Study of the Device C and /or Device 2b with IF/ZIF device architecture for R2D should be considered. Strive for common design for Device C and /or Device 2b as much as possible.

### New signals

For device 2b/C with ZIF/IF receiver in R2D link, the device need initial information, e.g., the center carrier frequency and other nessasary system informations, to faciliate its initial access procedure. Similar SSB-like signals can be used for the device to search for the center frequency. Other mechanisms such as prior/predefined knowledges known by the device can be investigated. However, it should be flexible for operators to operator on different frequencies.

Study of mechanisms such as new signals for initial access procedures can be considered for Rel-20 device 2b/C.

Proposal 1-6: At least for device C, study synchronization and initial access procedures, e.g., new signals for R2D synchronization.

### Active transmission in D2R

The active devices are with limitation of clock accuracy, e.g., 10-100ppm CFO. Hence, the D2R reception will be challenging. For example, more guard band may be needed, and coherent detection with less accurate clock is challenge due to less accurate estimation of the channel.

It should be noted that the CFO calibrate for better UL Tx/Rx can be benificial. Related studies to enable better active trasnmission in D2R is recommended.

Proposal 1-7: For device 2b/C, related studies to enable better active transmission by considering clock inaccuracy, e.g., CFO, is recommended for Rel-20 ambient IoT.

### Related physical procedures

Coding schemes

For Rel-19 ambient-IoT, no FEC is included for R2D link considering the device has 1uW power consumption and limited complex constrains. However, for device 2 with higher power consumption and cost/complexity, FEC is needed to increase the device coverage by much better link robustness. Related coding schemes with sustainable complexity/power consumption can be considered. For D2R, FEC is also important and should be considered.

Moreover, interleaving are not considered in the Rel-19 Ambient IoT physical layer design since the device does not have enough buffer to operator interleaving. However, Rel-20 device 2b/C can be considered with interleaving in order to resist to channel fading.

Proposal 1-8: Study of the channel coding schemes for both R2D and D2R link for Rel-20 device 2b/C.

HARQ / scheduling

Allowing scehduling based transmission is more flexible for system design, which allows TDMed with different devices' transmission/reception.

Particularly, HARQ for R2D and D2R transmission is benificial for reliability, that can be considered. However, complexity should also be taken into account for the device, e.g., limited buffer size, HARQ process number and etc.

Proposal 1-9: Study of the scheduling and HARQ for both R2D and D2R link for Rel-20 device 2b/C.

Modulation schemes

OOK-based receiver for R2D can particularly reduce the device power consumption and complexity, which is considered to be most significant consideration for device 2b/C design. For uplink, different modulation schemes considering limited device accuracy/complexity can be investigated. Phased modulation is simplist modulation schemes, such as BPSK/QPSK.

Proposal 1-10: Consider at least the following modulation schemes for Rel-20 device 2b/C including,

- OOK for R2D

- Phase modulation for D2R

### RAN architecture

A-IoT architecture

RAN2 and RAN3 have studied the architecture and protocol stack for support of Ambient IoT embedded in the overall 5G system architecture for topology 1 and topology 2.

For topology 2, NAS based solution has been ruled out by SA2, so no need to further discuss in RAN WGs. We propose RAN WGs focus on RRC based solution and UP based solution.

Benefit to support RRC based solution for topology 2:

- Topology 1 is based on control plane only. After operator deploy topology 1, it is easier for operator to extend control plane to further support topology 2.

- With RRC based topology 2, gNB can be responsible to select the UE reader. gNB can better select the UE reader than core network, because gNB has the camping beam related information for CONNECTED mode UE readers, with which information, gNB can select the UE readers to perform inventory with less inter-UE interference than core network selection.

- Suitable for 2B market. Better to support server triggered inventory. UE reader is deployed to compensate the coverage hole of topology 1.

Benefit to support user plane based solution for topology 2:

- The specification impact to RAN WGs is minimized.

- Suitable for 2C market. Better to support UE triggered inventory and command.

From the analysis above, we see the benefit for both RRC based and UP based solution and propose to recommend both into normative work.

Proposal 1-11:

Study RAN architecture enhancement to support device 2b/C/1 in topology 2, including,

- Down-selection or support both of RRC/UP based solutions for topo 2.

  - Note: NAS based solution is not recommended by SA2

- Specify the NG interface procedures and signalling for topo 2.

Note: A-IoT interface for topo 1 should be reused as much as possible

### Device (un)availability

In Rel-19, RAN1 have studied two directions for potential impact of device unavailability due to energy harvesting:

- Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable.

- Direction 2: Reader can provide information to a device based on which the device may become available/unavailable.

Based on the study, Direction 1 is sufficient for Device 1 and therefore Direction 1 is considered for Rel-19 device 1. For Device 2, Direction 2 provides some gain over Direction 1. For Rel-20, since device 2 is considered, Direction 2 can be also considered for better understanding.

Proposal 1-12: Study and specify of device (un)availability direction 2 for device 2 for Rel-20 due to energy harvesting is recommended.

### Positioning

RAN1 have studied two solutions for proximity determination at reader side in Rel-19 SI and concluded both as feasible:

- Solution 1: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader.

- Solution 2: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side.

Solution 1 is purely reader implementation, and no specification impact is expected. On the contrary, solution 2 is dependent on measurements at the reader side, and therefore further discussion and specification work on the measurement may be required.

Rel-20 can further consider the positioning techniques by considering the Solution 2 as well.

Proposal 1-13: For Rel-20, study backscattered devices positioning enhancements

- study requriements of the positioning by backscattering.

- study potential positioning solutions and specify nessasary signals/procedures/interfaces to support.

For active devices, e.g., device 2b/C, considering the basic designs for device 2b/C is needed to be considered first before any advacend positioning techniques, we consider some coarse positioning solution, e.g., cell-level tracking, to be studied first.

Proposal 1-14: For Rel-20, study simple positioning solutions based on existing mechanism for active devices (i.e., device 2b/C)

### Coverage

For the coverage of device 2b/C, the following can be considered,

- Alt 1: limited coverage

  - Target to have better coverage compared to Rel-19. May not be able to continuous coverage for the outdoor.

- Alt 2: Full coverage of the outdoor cell

  - Target for e.g., MCL 144dB

  - For Alt2, related UL transmission power should also consider limited power constraint.

Proposal 1-15: Study and evaluate the coverage of device 2b/C for outdoor deployment.

# Conclusion

This contribution discusses on design for Rel-20 Ambient IoT, and provides the following proposals:

Proposal 0-1:

- Rel-20 Ambient SI/WI from 25Q3 to 26Q4 (18 month), includes both outdoor and indoor scenarios

Proposal 0-2:

- A study phase is needed to ensure the coverage and coexistence is feasible if device 2b or device C is introduced in Rel-20 Ambient IoT.

Proposal 0-3: Both device 2b and C are considerd to be study for Rel-20 Ambient IoT.

Proposal 1:

| Features |  | Proposals |
| --- | --- | --- |
| General aspects |  |  |
| Deployment scenarios and topology |  | Proposal 1-1a: D4T1 and D4T2 are recommended for Rel-20 WI. Proposal 1-2: Considering topology 1 and topology 2 are targeting to different deployment scenarios both of which are essential, both topology 1 and topology 2 are recommended for Rel-20 ambient IoT. |
| Device type |  | Proposal 1-5: For Rel-20 WI, Study of the Device C and /or Device 2b with IF/ZIF device architecture for R2D should be considered. Strive for common design for Device C and /or Device 2b as much as possible. |
| Use case |  | Proposal 1-1b: In addition to support - Sensor - Positioning (study or down-selection needed) |
| Traffic type |  | Proposal 1-3: study should include supporting DO-A traffic for device 2b/C for Rel-20 ambient IoT. |
| Device (un)availability |  | Proposal 1-12: Study and specify of device (un)availability direction 2 for device 2 for Rel-20 due to energy harvesting is recommended. |
| RAN 1 related aspects |  |  |
| Device 2b/C coverage |  | Proposal 1-15: Study and evaluate the coverage of device 2b/C for outdoor deployment. |
| Physical layer signals/channels and procedures for device 2b/C | Initial access procedure, e.g., synchronization | Proposal 1-6: At least for device C, study synchronization and initial access procedures, e.g., new signals for R2D synchronization. |
|  | Active transmission in D2R for device type 2b/C | Proposal 1-7: For device 2b/C, related studies to enable better active transmission by considering clock inaccuracy, e.g., CFO, is recommended for Rel-20 ambient IoT. |
|  | Modulations | Proposal 1-10: Consider at least the following modulation schemes for Rel-20 device 2b/C including, - OOK for R2D - Phase modulation for D2R |
|  | Coding | Proposal 1-8: Study of the channel coding schemes for both R2D and D2R link for Rel-20 device 2b/C. |
|  | HARQ/Scheduling | Proposal 1-9: Study of the scheduling and HARQ for both R2D and D2R link for Rel-20 device 2b/C. |
|  | Power control | Proposal 1-20: Study the followings - D2R power control - Device power class, i.e., maximum Tx power |
| Positioning |  | Proposal 1-13: For Rel-20, study backscattered devices positioning enhancements - study requriements of the positioning by backscattering. - study potential positioning solutions and specify nessasary signals/procedures/interfaces to support. Proposal 1-14: For Rel-20, study simple positioning solutions based on existing mechanism for active devices (i.e., device 2b/C) |
| Topology 2 resource allocation |  | Proposal 1-4: For resource allocation and/or controlling of the intermediate UE in topology 2, minimized RAN1 specification impact is pursued, e.g., Higher-layer signaling is used. |
| RAN 2 related aspects |  |  |
| High layer aspects for supporting Device 2b/C in Topology 1 |  | Proposal 1-16: Study A-IoT radio interface high layer protocol stack, functionalities and procedures for supporting Device 2b/C in Topology 1, including, - Layer 2/3 Protocol stack and functionalities of sublayer(s) to support device 2b/C - system information, paging, DoA RACH procedure and data transmission procedure for device 2b/C necessity and feasibility of mobility for device 2b/C |
| Topology 2 for device 2b/C/1 |  | Proposal 1-17: Study topology 2 for device 2b/C/1 - down-selection or support both of RRC/UP based solutions for topo 2 - Uu interface signaling and procedures to support UE-reader - necessity and feasibility of coordination between UE-readers - Note: A-IoT interface for topo 1 should be reused as much as possible2 |
| RAN 3 related aspects |  |  |
| NG interface enhancement for supporting Device 2b/C in Topology 1 |  | Proposal 1-11: Study RAN architecture enhancement to support device 2b/C/1 in topology 2, including, - Down-selection or support both of RRC/UP based solutions for topo 2. - Note: NAS based solution is not recommended by SA2 - Specify the NG interface procedures and signalling for topo 2. Note: A-IoT interface for topo 1 should be reused as much as possible |
| Topology 2 for device 2b/C/1 |  |  |
| RAN 4 related aspects |  |  |
| Co-existence |  | Proposal 1-18: Study outdoor ambient IoT and NR/LTE co-existence is required if it targets for outdoor |

# References

- TR 22.840 Study on Ambient power-enabled Internet of Things (Release 19) V1.2.0, 2023-06; 3GPP SA1, S1-231333

- TR 38.848 Study on Ambient IoT (Internet of Things) in RAN (Release 18) V0.2.0, 2023-06; 3GPP RAN

- RP-240156, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC

- RP-242967	Views on Rel-20 5G-Advanced	CMCC

- RP-242965	Status report for Study on solutions for Ambient IoT (Internet of Things) in NR	CMCC

- RP-250796	New Work Item: Solutions for Ambient IoT (Internet of Things) in NR	CMCC, Huawei, T-Mobile USA Inc.

- RP-243280	Way forward for Ambient IoT for REL-19 and REL-20	RAN1 Vice Chair (Huawei)

- TR38.769		Study on solutions for ambient IoT (Internet of Things) in RAN1 (Release 19) V1.1.0,