3GPP TSG RAN Meeting #108	                                 RP-251391

Prague, Czech Republic, June 9-13, 2025

Source: 	CMCC

Title:	Text proposal for TR 38.914 on KPIs

Agenda item:	16.1

Document for:	Approval

- Background

In response to the Liaison Statement (LS) from ITU-R WP5D (Document 5D/TEMP/167) on minimum requirements related to technical performance (TPR) for IMT-2030 radio interface(s) [1], 3GPP TSG RAN#106 plenary initiated a study on 6G scenarios/use cases and requirements [2] and focused on investigating a candidate set of items for minimum TPRs for IMT-2030 radio interfaces based on the Recommendation ITU-R M.2160 [3]. In RAN#107, an additional part on general 6G RAN study was added to the RAN 6G SI [4].

During the previous two RAN plenary meetings [5][6], the following set of items was endorsed for IMT-2030 TPRs from RAN’s perspective.

|  | IMT-2030 TPRs | Method | Note |
| --- | --- | --- | --- |
| 1 | Peak data rate | Quantitative | Agreed in RAN#106 [5] Legacy TPRs of IMT-2020 |
| 2 | Peak spectral efficiency | Quantitative |  |
| 3 | User experienced data rate | Quantitative |  |
| 4 | 5th percentile user spectral efficiency | Quantitative |  |
| 5 | Average spectral efficiency | Quantitative |  |
| 6 | Energy efficiency | TBD |  |
| 7 | Area traffic capacity | Quantitative |  |
| 8 | User plane latency | Quantitative |  |
| 9 | Control plane latency | Quantitative |  |
| 10 | Connection density | Quantitative |  |
| 11 | Reliability | Quantitative |  |
| 12 | Mobility | Quantitative |  |
| 13 | Mobility interruption time | Quantitative |  |
| 14 | Bandwidth | Quantitative |  |
| 15 | Positioning accuracy | Quantitative | Agreed in RAN#107 [6] New TPRs for IMT-2030 |
| 16 | Sensing-related capabilities | TBD |  |
| 17 | AI-related capabilities | Qualitative |  |
| 18 | Joint/composite requirement | TBD |  |

This contribution gives text proposal for TR 38.914 on key performance indicators (KPIs) for the RAN 6G SI “Study on 6G Scenarios and requirements”.

- Text Proposal for TR 38.914

Based on the discussion on the IMT-2030 TPR, we propose the following text proposal for TR 38.914 on KPIs.

## 5	Requirements and key technical principles

5.1	Key performance indicators

This section describes the definitions and requirements of all KPIs.

The KPI targets defined in this section as well as in Report ITU-R M.[IMT-2030. TECH PERF REQ] shall be supported by next generation access technologies. The evaluation methodology defined in Report ITU-R M.[IMT-2030.EVAL] shall be used to evaluate the targets specified in Report ITU-R M.[IMT-2030. TECH PERF REQ].

5.1.1	Peak data rate

Peak data rate is the maximum achievable data rate under ideal conditions (in bit/s), which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the peak data rate Rp is given by:

Rp = W × SEp

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands then the total peak data rate is

R=i=1Q  Wi × SEpi

where Wi and SEpi (i = 1, …, Q) are the component bandwidths and spectral efficiencies respectively.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak data rate are as follows:

–	Downlink peak data rate is 60-100 Gbit/s.

–	Uplink peak data rate is 30-50 Gbit/s.

5.1.2	Peak spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak spectral efficiencies are as follows:

–	Downlink peak spectral efficiency is 45-75 bit/s/Hz.

–	Uplink peak spectral efficiency is 22.5-37.5 bit/s/Hz.

These values were defined assuming an antenna configuration to enable TBD spatial layers (streams) in the downlink and TBD spatial layers (streams) in the uplink. However, this does not form part of the requirement and the conditions for evaluation are described in Report ITU-R M.[IMT-2030.EVAL].

5.1.3	[User experienced data rate]

Editor note: name of this KPI may need to be changed

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

For the evaluation of uplink user experienced data rate, use system level simulation with no bandwidth scaling taking UE maximum output power into account.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for the user experienced data rate are as follows in the Dense Urban – IC test environment:

–	Downlink user experienced data rate is 150-300 Mbit/s.

–	Uplink user experienced data rate is TBD Mbit/s.

5.1.4	5th percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

With Ri (Ti) denoting the number of correctly received bits of user i, Ti the active session time for user i and W the channel bandwidth, the (normalized) user throughput of user i, ri, is defined according to the following equation:

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for 5th percentile user spectral efficiency for various test environments are summarized in Table 5.1.4-1.

Table 5.1.4-1: 5th percentile user spectral efficiency

| Test Environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot – IC |  |  |
| Dense Urban – IC | 0.45-0.675 | 0.3-0.45 |
| Rural – IC |  |  |

5.1.5	Average spectral efficiency

Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Let Ri (T) denote the number of correctly received bits by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs. Furthermore, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency, SEavg is then defined according to the following equation:

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for average spectral efficiency for various test environments are summarized in Table 5.1.5-1.

Table 5.1.5-1: Average spectral efficiency

| Test Environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – IC |  |  |
| Dense Urban – IC | 15.6-23.4 | 10.8-16.2 |
| Rural – IC |  |  |

5.1.6	Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through the following equation:

Carea = ρ × W × SEavg

In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirement for area traffic capacity in downlink is 15-30 Mbit/s/m² in the Indoor Hotspot – IC test environment.

5.1.7	User plane latency

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirements for user plane latency are:

–	2 ms for IC

–	0.5 ms for HRLLC

5.1.8	Control plane latency

Control plane latency refers to the time to move from a battery efficient state (e.g., IDLE) to start of continuous data transfer (e.g., ACTIVE).

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirement for control plane latency is 10 ms.

5.1.9	Connection density

Connection density is the number of connections fulfilling a specific quality of service (QoS) per unit area (per km2) per day.

Connection density should be achieved for a given bandwidth and a given number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability, as specified in [TBD].

This requirement is defined for the purpose of evaluation in the Massive Communication usage scenario.

The minimum requirement for connection density is 108-109 connections per km2 per day.

5.1.10	Reliability

Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability.

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

This requirement is defined for the purpose of evaluation in the Hyper Reliable and Low Latency Communication usage scenario.

The minimum requirement for the reliability is TBD success probability of transmitting a layer 2 PDU (protocol data unit) of TBD bytes within TBD ms in channel quality of coverage edge for the TBD test environment, assuming TBD application data (e.g. TBD bytes application data + protocol overhead).

5.1.11	Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

The following classes of mobility are defined:

–	Stationary: 0 km/h

–	Pedestrian: 0 km/h to 10 km/h

–	Vehicular: 10 km/h to 120 km/h

–	High speed vehicular: 120 km/h to 500 km/h

–	Airplane: 500 km/h to 1200 km/h

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

Table 5.1.11-1 defines the mobility classes that shall be supported in the respective test environments.

Table 5.1.11-1: Mobility classes

|  | Test environments for IC |  |  |
| --- | --- | --- | --- |
|  | Indoor Hotspot – IC | Dense Urban – IC | Rural – IC |
| Mobility classes supported |  |  |  |

A mobility class is supported if the traffic channel link data rate on the uplink, normalized by bandwidth, is as shown in Table 5.1.11-2. This assumes the user is moving at the maximum speed in that mobility class in each of the test environments.

Table 5.1.11-2: Traffic channel link data rates normalized by bandwidth

| Test environment | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot – IC |  |  |
| Dense Urban – IC |  |  |
| Rural – IC |  |  |

These values were defined assuming an antenna configuration as described in Report ITU-R M.[IMT-2030.EVAL].

5.1.12	Mobility interruption time

Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the mobile station and the radio access network, as applicable to the candidate RIT/SRIT.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirement for mobility interruption time is 0 ms.

5.1.13	Bandwidth

Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability of the RIT/SRIT is defined for the purpose of IMT-2030 evaluation.

The requirement for bandwidth is at least 400 MHz.

5.1.14	Positioning

Positioning is the ability to estimate the position of active devices. The positioning accuracy is the [90%] point of the CDF of the device positioning error for both horizontal and vertical directions. The calculated positioning information would be indicated with the use of three-dimensional coordinates.

The horizontal/vertical positioning error is defined as the difference between the estimated horizontal/vertical position and the actual horizontal/vertical position of a device.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for the positioning accuracy for various test environments are summarized in Table 5.1.14-1.

Table 5.1.14-1: Positioning accuracy

| Test Environment | Bandwidth (MHz) | Horizontal Accuracy | Vertical Accuracy |
| --- | --- | --- | --- |
| Indoor Hotspot –IC | [20] |  |  |
|  | [100] |  |  |
| Dense Urban – IC | [20] |  |  |
|  | [100] |  |  |
| … |  |  |  |

5.1.15	Energy efficiency

Energy efficiency is an important metric of sustainability. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics.

This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario.

The requirement is the relative energy savings between the selected load case(s) and a reference case. For network energy efficiency, the reference case is when network is fully loaded. For UE energy efficiency, the reference case is when UE is fully-loaded/has full-buffer service.

Proponents should report the power model used for the evaluation (e.g. BS power model in 3GPP TR 38.864 and UE power model in 3GPP TR 38.840), the number of UE under each BS and the traffic model.

The relative target of certain metric (e.g. threshold value for the metric, or certain performance loss compared with the benchmark) can be set. Constant that certain percent (e.g. 90%) of UE fulfils the target should be achieved while evaluating the relative power saving for each load case. The certain metric can be selected according to the service type:

- For Vo6G (i.e. VoNR-like) service, the certain metric is latency

- For burst-buffer service, the certain metric is UPT

- For XR service, the certain metric is latency

This requirement is defined for the purpose of evaluation in the IC usage scenario.

The minimum requirements for energy efficiency for various test environments are summarized in Tables below.

Table 5.1.15-1: The minimum requirements for Network Energy efficiency

| [Test environment(s)] ** | Selected load case(s) | The reference case | Relative power saving (%) |
| --- | --- | --- | --- |
| Indoor Hotspot-IC | [empty load, low load, light load and/or medium load] * | Fully loaded IMT-2030 case * | [X1%, X2%, ...] |
| Dense Urban-IC | [empty load, low load, light load and/or medium load] * | Fully loaded IMT-2030 case * | [Y1%, Y2%, ...] |
| Rural-IC | [empty load, low load, light load and/or medium load] * | Fully loaded IMT-2030 case * | [Z1%, Z2%, ...] |

*empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100

** needed only in case of simulation

Table 5.1.15-2: The minimum requirements for Device Energy efficiency

| [Test environment(s)] ** | Selected load case(s) | The reference case | Relative power saving (%) |
| --- | --- | --- | --- |
| Indoor Hotspot-IC | [empty load, low load, light load and/or medium load] * | Device is fully-loaded/has full-buffer service | [A1%, A2%, …] |
| Dense Urban-IC | [empty load, low load, light load and/or medium load] * | Device is fully-loaded/has full-buffer service | [B1%, B2%, …] |
| Rural-IC | [empty load, low load, light load and/or medium load] * | Device is fully-loaded/has full-buffer service | [C1%, C2%, …] |

5.1.16	[Composite requirements]

Editor note: KPI related to data rate, latency, reliability, capacity. Name of this KPI may need to be changed

This composite requirement is defined as the number of users that fulfils the required packet success rate within a required maximum latency, i.e., packet delay budget (PDB), where the packet success rate is defined as the ratio of the number of packets that are successfully delivered to the destination receiver within a transmission delay of less than or equal to the PDB to the total number of packets.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for the composite requirement for various test environments are summarized in Table 5.1.16-1.

Table 5.1.16-1: Composite requirement

| Test environment | Data rate (Mbits/s) | Latency (ms) | Reliability/ Packet success rate | Number of satisfied users (/km2) |
| --- | --- | --- | --- | --- |
| Dense Urban – IC | DL=30Mbps UL=10Mbps | DL=10ms UL=30ms | 99% |  |
| … |  |  |  |  |

5.1.17	Sensing-related capabilities

Sensing-related capabilities refer to the ability to provide functionalities in the radio interface to support sensing tasks, including passive object presence/absence detection, object classification, movement characterization, object localization, mapping, and composite sensing tasks such as tracking, SLAM, etc. These capabilities could be measured in terms of the following performance metrics/requirements to evaluate the radio access technology’s ability to support sensing related capabilities:

- Detection probability with a false alarm probability: Detection probability, PD, is the probability of correctly detecting the presence of the object, and false alarm probability, PFA, is the associated probability that an object is detected given that the object is not present.

- Localization Accuracy: Localization Accuracy is defined as the three-dimensional distance between the measured and the actual location of the sensing target with [95%] confidence.

- Velocity Accuracy: Velocity Accuracy is defined as the three-dimensional distance between the measured and the actual velocity of the sensing target with [95%] confidence.

- Sensing resolution: Sensing resolution is defined as the minimum distance between two distinguishable objects, representing the ability to distinguish between two adjacent objects.

- Sensing capacity: Sensing capacity is defined as the maximum number of the targets per unit area when all objects fulfil QoS requirements with [95%] confidence, which is a system-level requirement to reflect multi-object sensing performance.

The metrics of QoS requirements for sensing capacity depend on sensing tasks and are defined as follows:

- QoS requirement of detection purpose is defined by the detection probability with a fixed false alarm probability of [TBD].

- QoS requirement of localization purpose is defined by the localization accuracy.

- QoS requirement of tracking purpose is defined as the maximum latency between two consecutive measurements with given localization accuracy.

This requirement is defined for the purpose of evaluation in the ISAC usage scenario.

5.1.18	AI-related capabilities

Applicable AI-related capabilities refer to the ability to provide certain functionalities throughput IMT-2030 to support AI enabled applications.

The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support mechanisms and/or signalling/related to AI functionalities over the radio interface (between UE and network). This should include support for one or more of the following:

- Manageable AI data collection

- Distributed data processing

- Distributed learning

- AI model execution

- AI model inference

- Or other AI capabilities of the RITs/SRITs (to be reported by the proponent)

- Efficient AI computing processing

- Low Total Cost of Ownership (TCO)

  - Decoupling computing module with legacy communication module

  - Openness of hardware infrastructure

  - Flexible scheduling of computing resources

- Conclusion

In this contribution, we propose to add the text proposal in above Section 2 to TR 38.914 Clause 5.

- References

- RP-243202, LS on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN#106, Madrid, Spain, December 9-12, 2024.

- RP-243327, New SID: Study on 6G scenarios/use cases and requirements, RAN#106, Madrid, Spain, December 9-12, 2024.

- ITU-R Recommendation M.2160-0 (11/2023), “Framework and overall objectives of the future development of IMT for 2030 and beyond”.

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, RAN#107, Incheon, Korea, March 12-14, 2025.

- RP-243325, Draft Reply to ITUR_WP5D_TEMP_167 = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN#106, Madrid, Spain, December 9-12, 2024.

- RP-250819, Moderator’s summary for IMT-2030 TPRs, RAN#107, Incheon, Korea, March 12-14, 2025.

Average spectral efficiency corresponds to “spectrum efficiency” in Recommendation ITU-R M.2160.