3GPP TSG RAN Meeting #108	                       RP-251397

Prague, Czech Republic, June 9-13, 2025

Source: 	CMCC

Title:	Views on Migration and Interworking of 6G RAN

Agenda item:	16.1

Document for:	Discussion

- Introduction

The study of 6G is initiated in 3GPP, and in the early stage of 6G construction, the co-existence of 5G network and 6G network cannot be avoid. In this contribution we will provide our view on the potential issues related to the co-existence and interworking between 6G and 5G.

- Discussion

Considering the co-existence of 6G network and 5G network, the interworking between the two networks should be discussed. In our view, at least the following issues should be considered:

- Inter-RAT mobility between 6G and 5G

- Multi-RAT spectrum sharing for 6G

Inter-RAT mobility between 6G and 5G

The inter-RAT mobility is to ensure UE’s service continuity when it moves between a 6G cell and a 5G cell, the direct way as defined in other generation is through inter-RAT handover, which involves the participant of core networks. And we think this should be inherited in 6G as baseline. In this way, the UE registers in one cell, for example 6G cell and keeps the link with 6G network, and when the inter-RAT handover to 5G ends, the 5G network maintains UE’s registration information and builds the link between the UE and 5G network, while the link between the UE and 6G network is released. For inter-RAT handover, the interruption of UE’s data transmission cannot be avoid.

Proposal 1: Inter-RAT handover between 6G and 5G is supported.

As mentioned above, inter-RAT handover is a relatively simple way to support the inter-RAT mobility, as we can drawn on the solutions of other generations. However, the corresponding interruption cannot be avoided. For 6G, we may aim for a solution for 0 interruption during inter-RAT mobility, or to minimize the interruption time. To achieve this, we think Dual steer/dual stack mechanism can be considered. For Dual steer/dual stack, the UE maintains two separate links and registration contexts with the 6G and 5G core networks respectively, and the resources of both networks can be utilized flexibly. When the UE moves between the 6G and 5G cells, quick and dynamic switch for the UE can be achieved, while the interruption can be avoided or reduced.

Proposal 2: Dual steer/Dual stacks in 6G and 5G can be considered.

Multi-RAT spectrum sharing for 6G

In 5G, dynamic spectrum sharing is supported, which can improve the resource efficiency and satisfy different service requirements. Besides, it also helps to the smooth migration from 4G to 5G, reduce operator’s construction costs and increase the flexibility of the network. Taking these advantages into consideration, multi-RAT spectrum sharing, like spectrum sharing for 6G and 5G, should be supported. The spectrum allocation or sharing between 6G RAN and 5G RAN, should be decided by the 6G RAN, considering the traffic demand, service requirements and operator policies. MRSS resource allocation granularity in the time/frequency domain, as well as the potential guards between 6G and 5G resources are to be determined by the study on 6G. What’s more, MRSS can be supported in co-located case of 6G RAN and 5G RAN, also in the none co-located case. And MRSS are applicated for paired and unpaired spectrum.

Proposal 4: MRSS should be supported for 6G.

- Conclusions

In this contribution we will provide our view on inter-RAT mobility and MRSS for 6G, which are related to the co-existence and interworking between 6G and 5G. Following are the proposals:
Proposal 1: Inter-RAT handover between 6G and 5G is supported.

Proposal 2: Dual steer/Dual stacks in 6G and 5G can be considered.

Proposal 4: MRSS should be supported for 6G.