3GPP TSG-RAN Meeting #108	RP-251405

# Prague, Czech Republic, 9 – 13 June 2025

Agenda item:		15.2.5

Source:	Xiaomi

Title:	Views on Rel-20 UAV/UAM scope

Document for:		Discussion and Decision

# Introduction

For RAN#107 meeting, RP-250812 [1] captures the following for Rel-20 UAV:

- As a starting point we consider:

  - Specify a UAV/UAM specific frequency/cell/beam prioritization mechanism for IDLE/INACTIVE mobility.  TBD next meeting whether we have a separate WI or TEI

  - FFS CHO for RAN#108

- If this is a WI, the work should be completed in 2 Quarters

In this contribution, we discuss Rel-20 UAV scope.

# Discussion

CHO

One of the open issues for Rel-20 UAV is whether to consider CHO in the scope. Currently, the following CHO triggering conditions are defined in TS 38.331 [2]:

CondEvent A3: Conditional reconfiguration candidate becomes amount of offset better than PCell/PSCell;

CondEvent A4: Conditional reconfiguration candidate becomes better than absolute threshold;

CondEvent A5: PCell/PSCell becomes worse than absolute threshold1 AND Conditional reconfiguration candidate becomes better than another absolute threshold2;

CondEvent D1: Distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a reference location referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2;

CondEvent T1: Time measured at UE becomes more than configured threshold t1-Threshold but is less than t1-Threshold + duration;

Potential CHO topics for Rel-20 include new CHO triggering events based on height. There are already UAV related measurement events defined in TS 38.331 [2]:

Event H1:	Aerial UE altitude becomes higher than a threshold;

Event H2:	Aerial UE altitude becomes lower than a threshold;

Event A3H1:	Neighbour becomes offset better than SpCell and the Aerial UE altitude becomes higher than a threshold;

Event A3H2:	Neighbour becomes offset better than SpCell and the Aerial UE altitude becomes lower than a threshold;

Event A4H1:	Neighbour becomes better than threshold1 and the Aerial UE altitude becomes higher than a threshold2;

Event A4H2:	Neighbour becomes better than threshold1 and the Aerial UE altitude becomes lower than a threshold2;

Event A5H1:	SpCell becomes worse than threshold1 and neighbour becomes better than threshold2 and the Aerial UE altitude becomes higher than a threshold3;

Event A5H2:	SpCell becomes worse than threshold1 and neighbour becomes better than threshold2 and the Aerial UE altitude becomes lower than a threshold3.

Therefore defining new CHO events based on height is a low-hanging fruit for Rel-20 UAV work.

Proposal 1: Rel-20 UAV includes the scope of CHO triggering events based on height.

TEI vs. WI

Another open issue for Rel-20 UAV is whether to have a separate WI or TEI. This mainly depends on whether there are RAN3 (e.g. Xn / F1 signalling) and RAN4 impacts (e.g. performance requirements). Regarding UAV/UAM specific frequency/cell/beam prioritization mechanism for IDLE / INACTIVE mobility, there are no RAN3 impacts since it is related to IDLE / INACTIVE mobility. It is expected that existing RAN4 requirements on IDLE / INACTIVE mobility (e.g. TS 38.133 [3] clause 4 and 5) are applicable without additional changes, hence no RAN4 work is expected.

As for CHO triggering events based on height, although RAN4 defined CHO performance requirements (e.g. TS 38.133 [3] clause 6.1.4), the requirements are related to handover delay and measurement time, which do not depend on triggering events, as shown below:

In addition, for UAV related measurement events defined in TS 38.331 [2] (e.g. event H1, event A3H1), RAN4 has not define additional requirements e.g. for L3 handover. Therefore no additional RAN4 work is needed for CHO triggering events based on height. As for RAN3 impacts, contributions [4][5] proposes to consider NG based CHO and UAV UE identification management, which have RAN3 impacts. Since existing CHO triggering events do not impact RAN3 specifications (e.g. TS 38.423 and 38.473), it is expected that CHO triggering events based on height does not require RAN3 work.

In summary, RAN3 and RAN4 work for Rel-20 UAV is not needed. Therefore Rel-20 UAV work can be a TEI. Given that there is only one RAN2 meeting before Rel-19 feature frozen (RAN2#131 meeting in India), including UAV work as TEI19 might not be appropriate. Consequently, Rel-20 UAV can be handled as TEI20.

Proposal 2: Rel-20 UAV work is handled as TEI20.

# Conclusion

In this contribution, we discuss Rel-20 UAV scope, and propose the following:

Proposal 1: Rel-20 UAV includes the scope of CHO triggering events based on height.

Proposal 2: Rel-20 UAV work is handled as TEI20.

# References

[1] RP-250812, RAN Vice Chairs, RAN1, RAN2, RAN3, RAN4 Chairs, “Summary for RAN Release 20 5G-Adv”

[2] 3GPP TS 38.331, "NR; Radio Resource Control (RRC); Protocol specification"

[3] 3GPP TS 38.133, "NR; Requirements for support of radio resource management"

[4] RP-250508, CATT, SK Telecom, "Consideration on UAV in 5G-A Rel-20"

[5] RP-250558, ZTE et al, "Views on NR-enabled UAV in 5G-A Rel-20"