3GPP TSG RAN Meeting #108								RP-251435

Prague , CZ, June 9-13, 2025

Status Report to TSG

Agenda item:			9.3.4.2

| WI / SI Name | # NR base station (BS) RF requirement evolution for FR1/FR2 and testing |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_BS_RF_req_evo |  |  |  |  |
| Unique ID | 1030078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-243270 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 09/2025 | Performance part: 03/2026 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 70% | Performance Part: 40% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Xue, fei |
|  | Company | ZTE Corporation |
|  | Email | Xue.fei25@zte.com.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4 #114bis:

Core part for CLTA:

Sub-topic 1-1: TX IMD co-location

Agreement

Intra-system TX IMD requirement is not within the scope of this WI. Therefore, no impact on Intra-system TX IMD requirement.

Sub-topic 1-2: Beam steering

Agreement:

Beam steering impact on the RF requirements can be investigated case-by-case for the pre-selected requirements (as agreed in R4-2502279):

- Transmitter OFF power requirement,

- Co-location spurious emission requirement/Protection of the BS receiver of own or different BS,

- Tx IMD requirement,

- Co-located OOBB requirement.

No conclusion on exclusion of any of the above requirements from the beam steering coupling analysis.

Sub-topic 1-3: Assumptions for the beam steering scenario analyses

Agreement: To improve simulation results alignment, the following assumptions are taken for the beam steering consideration (track changes showing delta compared to RAN4#114 agreements):

Data collection to prioritize the following scenarios:

- Beam steering range +/- 60 degrees,

- Reuse CLTA arrangements, e.g., same plane orientation; 10 cm offset enclosure to enclosure, etc.

Sub-topic 1-4: Capturing conclusions on Core aspects

Agreement: Previous WF (R4-2502279, RAN4#114) on the timeline for BS-to-BS isolation data collection (i.e., ”conclusion on the BS-to-BS isolation analysis to be reached no later than August 2025 meeting”) is amended as follows:

- Close BS-to-BS isolation data collection by RAN4#115 (May 2025) to allow drawing initial conclusions.

NOTE: Further BS-to-BS isolation data provision during RAN4#116 (August 2025) is not precluded.

- For RAN4#115 (May 2025) provide Draft CRs to TR 37.941 with the initial summary of the findings on the following areas (work-split proposed below):

  - Background and motivation for CLTA-alternatives discussion (Huawei),

  - BS-to-BS isolation analysis, including related assumptions (Ericsson),

  - Impact of BS-to-BS isolation analysis on the legacy co-location requirements (ZTE),

  - CLTA alternatives: horn antenna (Nokia).

Sub-topic 1-5: BS-to-BS isolation analysis

Agreement: Current baseline for BS-to-BS isolation analysis is the measurement and simulation data as captured in the previous WF (i.e., R4-2502279, RAN4#114).

Core part for Tx spurious emissions co-existence limits re-evaluation:

Sub-topic 2-1: Evaluation methodology

Agreement:

- Consider MCL methodology, including AAS and non-AAS implementations.

  - TR 25.942 is used as reference for the methodology derivation.

- System level evaluations are not precluded.

- Aim Rel-19 conclusion.

Sub-topic 2-2: Network layout

Agreement:

To determine coupling loss between the aggressor BS and victim UE/BS, the following is the starting point:

- For UL BS-BS coexistence, 100% grid shift as a starting point; evaluations of grid shift of 50% is not precluded.

Sub-topic 2-3: BS classes consideration

Agreement:

While the legacy Tx spur co-existence limits are defined without distinctions of the BS class, further investigate whether and how to differentiate the spurious emission requirements based on BS classes.

For system level evaluations:

  - For the protection of BS receiver, to prioritize the Macro BS scenario;

  - For the protection of UE receiver, to start with Macro BS scenario; other BS class analyses are not precluded.

Sub-topic 2-4: Frequency bands for co-existence work

Agreement:

- Aggressor carrier frequencies: prioritize 2 GHz, 3.5 GHz,

- Victim frequency bands: prioritize lower bound frequency of band n104, i.e., 6425 MHz.

Sub-topic 2-5: Antenna model assumptions

Agreement:

- BS:

  - Aggressor BS: AAS and non-AAS;

  - Victim BS: AAS only.

  - Assumptions for antenna gains for OOB are FFS.

- UE: no beamforming; omnidirectional antenna model with 0 dBi.

Sub-topic 2-6:  Protection margin

Agreement:

- For DL (UE Rx) protection: reuse the 3 dB desensitisation assumption (Reference from TR 25.942).

- For UL (BS Rx) protection: reuse the 0.8 dB desensitisation assumption (Reference from TR 25.942).

Sub-topic 2-7: System-level simulation assumptions

Agreement:

For AAS BS: follow TR 38.922 as baseline

- For both 2 GHz and 3.5 GHz, reuse 4400 - 4800 MHz frequency range antenna parameters,

- For lower bound frequency of band n104, i.e., 6425 MHz, reuse 7125 to 8400 MHz frequency range antenna parameters

For non-ASS: follow 37.840 as starting point.

For Propagation Model, use TR 38.901 as starting point.

Other simulation parameters

The following parameter value are considered as a starting point:

- ISD for Macro – 500m (Reference from TR 25.942);

- Macro BS antenna height – 25m

- BS to UE minimum distance – 35m

- UE outdoor – 100%

Perf part for EEIRP:

Issue 2-1: Reference case for the calculation of EEIRP error

Agreement:

- To capture some justification in the TR why the standard ellipse is representative for the beam steering range.

Issue 2-2a: standard deviation for EEIRP summation error

Agreement:

- Agree the MU as [0.5dB] for EEIRP mask.

Issue 2-2b: C1 Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter)

Agreement:

- Keep the C1 as 0.26dB;

- To capture some note for C1 in TR to clarify how MU value could be improved.

Issue 2-3: overall MU for EEIRP requirement

- Agreement:

- [1.7dB]

Issue 2-4: mechanical tilts related issue

- Agreement:

- To capture the EEIRP equation transformation in the Annex N with the mechanical tilt;

- To clarify the coordinate system for measurement and EEIRP calculation in the Annex N

Perf part for Test scope simplification and TRP simplification:

Issue 1-1-1: test scope reduction for Tx requirement

- Agreement:

  - Further discuss how to reduce the test simplification for OBUE limits.

    - Option 1: 1st frequency offset

    - Option 2: 3rd frequency offset to check OBUE limits

  - Further discuss how to reduce the test simplification for spurious limits.

    - Option 1: 3rd frequency offset

    - Option 2: For bands with operating band < 100 MHz to use 1st interferer offset and for bands with operating band >= 100MHz to use 3rd interferer offset.

Issue 1-1-2: Reduction of  test configuration for Tx requirement

- Agreement:

  - Proposal 1: 	For the reduction of test configurations for TX IM requirement, regardless of the test configuration selected (ATCRx or ANTCRx), the condition of maximum output power – Rated total output power (Prated,t,AC, or Prated,t,TABC) – should be met.

Issue 1-2-1: Out of band blocking

- Agreement on step size of OOBB requirement for further discussion:

    - Option 1: 1/3 of the minimum supported bandwidth

    - Option 2: Option 2: Use 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz

    - Other options are not precluded.

Issue 2-2: Procedure wise clarification for different TRP measurement

Agreement for test method/ sampling grid simplification:

  - To follow the previous RAN4 agreement in RAN4#114 meeting: to keep the current TRP testing method, TRP sampling grid and testing chamber in the specification

Agreement for applicability for different requirement:

  - For OTA base station output power, ACLR and operating band unwanted emissions, use the following option 1 as starting point.

    - Option 1:

      - Step 1: [Orthogonal cut grid] (two-cuts and optional three cuts) as described in annex I.5 or Beam-based directions as described in annex I.10 may be performed if chosen for the test, go to step 2 if this step is not chosen or the BS under test fails the test in this step.

      - Step 2: One of the TRP measurement procedure as described in annex I.2 to I.9 except annex I.5 shall be performed for the test.

  - For OTA base station spurious emission, use the option 1 as starting point

    - Option 1:

      - Step 1: Pre-scan as described in annex I.13 if chosen for the test, go to step 2 if this step is not chosen.

      - Step 2: [Orthogonal cut grid] (two-cuts and optional three cuts) as described in annex I.5 or Peak method as described in annex I.11 or Equal sector with peak average as described in annex I.12 may be performed if chosen for the test, go to step 3 if this step is not chosen or the BS under test fails the test in this step.

      - Step 3: One of the TRP measurement procedure as described in annex I.2 to I.9 except annex I.5 shall be performed for the test.

Issue 2-3:  the improvement of Pres-scan approach

Agreement:

  - Encourage companies to provide the draft CR focusing on how to improve the test procedures for Pre-scan approach next meeting and further discuss the improvement based on draft CR directly.

Issue 2-4:  TRP uncertainty

Agreement:

  - To follow the previous RAN4 agreement in RAN4#114 meeting.

RAN4 #115:

Core part for CLTA:

Sub-topic 1-1: BS-to-BS isolation analysis

Agreement: The following data is considered baseline for the BS-BS isolation conclusions derivation:

- No beam steering consideration; intra-band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) | ZTE (R4-2504358, M): -54 (SA-to-SA) | ZTE (R4-2504358, M): -52 (SA-to-SA) | ZTE (R4-2504358, M): -56 (SA-to-SA) |  |
| Isolation in horizontal domain (dB) | ZTE (R4-2504358, M): -31 (SA-to-SA) | ZTE (R4-2504358, M): -34 (SA-to-SA) Huawei (R4-2504594, M): -46 Nokia (R4-2507633, M): -35 (A-to-SA) | ZTE (R4-2504358, M): -46 (SA-to-SA) Huawei (R4-2507445, M): -50.6 (A-to-SA) | Nokia (R4-2507633, M): -55 (A-to-SA) |
| - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling - SA-to-SA: array to sub-array coupling |  |  |  |  |

- Beam steering consideration; intra-band; same BS type:

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) |  |  |  |  |
| Isolation in horizontal domain (dB) |  | Ericsson (R4-2506307, S): A-to-SA: -55±15 dB | ZTE (R4-2507511, M): -69 dB (A-to-SA) | Ericsson (R4-2506307, S): A-to-A: -77±15; A-to-SA: -70±15 |

- Beam steering consideration; inter-band:

|  | 3.5 GHz ÷ 7 GHz |
| --- | --- |
| Isolation in vertical domain (dB) |  |
| Isolation in horizontal domain (dB) | Ericsson (R4-2506307, S): A-to-A: -54±10 dB A-to-SA: -57±10 dB |

Based on date captured in bullets above, the following BS-BS isolation data summary is extracted:

Intra-band BS-BS isolation results

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) | -54 | -52 | -56 |  |
| Isolation in horizontal domain (dB) | -31 | -34 | -46 | -55 |

As per previous agreement in R4-2504701 (RAN4#114bis), further BS-to-BS isolation data provision during RAN4#116 (August 2025) is not precluded.

Sub-topic 1-2: Impacted RF requirements

Agreement:

- Based on the BS-to-BS isolation data, no RF (including core and conformance test) co-location requirements impact below 4.9 GHz was agreed.

- OTA TX IMD:

  - Impact on the OTA TX IMD requirement is FFS, considering the argument that this is a regulatory requirement.

  - Provide further inputs to address the testability issue.

  - Further investigate impact of higher than 43dBm BS output power.

Core part for Tx spurious emissions co-existence limits re-evaluation:

Sub-topic 1-1: Evaluation methodology

Agreement:

- Keep both MCL and system level evaluations.

- Derivation approach of the final conclusions (i.e., Tx spurious emissions co-existence limits, applicable frequency ranges, etc.) is FFS.

Sub-topic 1-2: Antenna / array gain considerations

Agreement: The out-of-band antenna gain information is to be provided together with the simulation results.

Sub-topic 1-3: Performance metrics

Agreement: Use 99th percentile of the coupling loss CDF as a starting point.

Sub-topic 1-4: Timeline

Agreement: Aim for conclusion of the Tx spurious emissions co-existence levels re-evaluation not later that the end of 2025 (due to timeline in ETSI MSG TFES).

Perf part for EEIRP:

Issue 2-1: standard deviation for EEIRP summation error

Agreement:

- 0.5dB

Issue 2-2: overall MU for EEIRP requirement

Agreement:

- 1.7dB

Issue 2-3: mechanical tilts related issue

- Agreement: the rotation equations in Annex N.2.1 are modified as follows

θH=90−arccoscosφsinθ+90sin−θMT+cosθ+90cos−θMT

φH=argcosφsinθ+90cos−θMT−cosθ+90sin−θMT+jsinφsinθ+90

The angle in the reference coordinate system defined in sub-clause 4.14 between the projection of the x/y plane and the radiation vector defined between -90° and 90°. 0° represents the direction perpendicular to the y/z plane. The angle is aligned with the down-tilt angle.

H	The angle with respect to the horizon defined between +90° and -90°, above the horizon is positive below the horizon is negative. Note the orientation is opposite to 

MT	The down-tilt angle representing the mechanical tilt defined between -90° and +90° (positive towards the ground).

The angle in the reference coordinate system defined in sub-clause 4.14 between the x-axis and the projection of the radiation vector onto the x/y plane defined between -180° and 180°.

φH	The horizontal angle in the global coordinate system, defined with respect to the horizon.

Perf part for Test scope simplification and TRP simplification:

Issue 1-1-1: test scope reduction for OBUE

- Agreement:

  - Further discuss how to reduce the test simplification for OBUE limits.

    - Option 1: 1xCBW offset

    - Option 2: 3xCBW offset

    - Option 3: 3xCBW offset if 3rd interfering signal is falling within the range of FDL_low to FDL_high of the operating bands and 1xCBW and 2xCBW if 3rd interfering signal is falling higher than FDL_high+ ΔfOBUE or lower than FDL_low-ΔfOBUE

Issue 1-1-2: test scope reduction for spurious emission

- Agreement:

    - Option 1: 3xCBW offset

Issue 1-2-1: Out of band blocking

- Agreement on step size of OOBB requirement for further discussion:

    - Option 1: 1/3 of the minimum supported bandwidth

    - Option 2:  Use 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz

    - Option 3: mixed EMC and option 1 approach, FFS on the details.

    - Option 4: To use 2/3 of the test signal configuration channel bandwidths

    - Option 5: Use 1MHz steps size below 100MHz and 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz.

    - Other options are not precluded

Issue 2-2: Reference method and preferred method

- Agreement:

    - Further discuss TRP estimation simplifications and the applicability improvements on TRP sampling grid

    - FFS on how to incorporate the OTA chamber related issues into the discussion on the reference test method.

2.4.2	Remaining Open issues

Core part:  Open issues for co-location reference antenna

Sub-topic 1-1: BS-to-BS isolation analysis

- Continue the BS-to-BS isolation analysis and the core requirement definition according to the consensus reached

Sub-topic 1-6: Horn antenna aspects

- Companies continue to investigate the horn antenna.

Perf part: Open issues for the reduction of testing scope

test scope reduction for Tx requirement

  - Further discuss the applicable interference freq offset for OBUE;

Out of band blocking

  - FFS on the step size for OOBB conformance testing

Open issues for the reduction of simplification BS TRP test methods for FR1 and FR2

  - Further discuss how to improve the test procedure and the applicability of testing method for different requirements in Annex I of TS 38.141-2.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

RAN4#114bis

- R4-2503167	Simplification of BS TRP test methods	Nokia

- R4-2503577	[37.941] Incorrect MU data for n104 band	ROHDE & SCHWARZ

- R4-2503588	[37.941] Incorrect MU data for n104 band	ROHDE & SCHWARZ

- R4-2503589	[38.141-2] Incorrect MU data for n104 band	ROHDE & SCHWARZ

- R4-2503593	[38.141-2] Incorrect MU data for n104 band	ROHDE & SCHWARZ

- R4-2503601	[Not inc C1-1] PWS specific MU data to be included for n104 band	ROHDE & SCHWARZ

- R4-2503602	[C1-1 focus] Spec An MU to be updated for n104 band	ROHDE & SCHWARZ

- R4-2503726	Rel-19 BS RF evolution overview	Ericsson

- R4-2504012	OTA test scope reduction	Ericsson

- R4-2504013	Simulation of BS-to-BS out-of-band isolation for edge-to-edge co-location isolation between 3.5 and 7 GHz	Ericsson

- R4-2504191	On OTA test enhancements	Nokia

- R4-2504192	On TX co-existence spurious emissions requirements	Nokia

- R4-2504267	On Co-existence parameters for BS Tx spurious emission co-existence requirement limits	Ericsson

- R4-2504295	Further discussion on Expected EIRP test	Huawei, HiSilicon

- R4-2504296	Draft CR to TS 38.908 EEIRP measurement uncertainty	Huawei, HiSilicon

- R4-2504297	Draft CR to TS 38.141-2 EEIRP measurement uncertainty and coordinate alignment	Huawei, HiSilicon

- R4-2504298	Transmitter co-existence spurious emission requirements	Huawei, HiSilicon

- R4-2504299	OTA test case reduction	Huawei, HiSilicon

- R4-2504343	Views on BS FR1 transmitter co-existence spurious emission requirements	Qualcomm Germany

- R4-2504358	Further discussion on OTA test enhancement	ZTE Corporation, Sanechips

- R4-2504359	Discussion on FR1 BS transmitter co-existence spurious emission requirements	ZTE Corporation, Sanechips

- R4-2504360	Discussion on conformance testing for U6GHz BS EEIRP mask	ZTE Corporation, Sanechips

- R4-2504361	Draft CR to TS 38.141-2: MU value and angular transformation for U6GHz EEIRP mask	ZTE Corporation, Sanechips

- R4-2504362	Draft CR to TR 38.908 EEIRP measurement uncertainty and angular transformation	ZTE Corporation, Sanechips

- R4-2504363	Big Draft CR to TS 38.908: MU and others	ZTE Corporation, Sanechips

- R4-2504364	Discussion on the OTA testing scope reduction	ZTE Corporation, Sanechips

- R4-2504365	Discussion on the applicability improvement of TRP measurement	ZTE Corporation, Sanechips

- R4-2504433	Draft CR to TS 38.106 for introduction of expected EIRP mask core requirement for NCR	Nokia, ZTE Corporation, Ericsson, Huawei, Samsung, Qualcomm, CATT

- R4-2504434	Draft CR to TS 38.115-2 for introduction of expected EIRP mask conformance test for NCR	Nokia, ZTE Corporation, Ericsson, Huawei, Samsung, Qualcomm, CATT

- R4-2504435	Draft CR to TS 38.141-2 with updates for EEIRP requirement conformance testing	Nokia

- R4-2504436	Draft CR to TR 38.908 with updates to EEIRP sampling Error simulation results	Nokia

- R4-2504437	Draft CR to TR 38.908 with updates to test procedure	Nokia

- R4-2504438	Further discussion BS OTA test scope reduction	Nokia

- R4-2504513	Draft CR to TR 38-908: Corrections and Improvements related to EEIRP	Ericsson

- R4-2504514	Draft CR to TS 38.141-2: Improvements related to Expected EIRP (EEIRP)	Ericsson

- R4-2504515	TS 38.141-2 Annex I: Further discussion on improvement of TRP measurement methods	Ericsson

- R4-2504594	Further discussion on CLTA related matters	Huawei, HiSilicon

- R4-2504596	Draft CR to TS 37.941: CLTA agreements	Huawei, HiSilicon

- R4-2504597	Information on European Commission feedback wrt. alternative test methods	Huawei, HiSilicon

- R4-2504627	Topic summary for [114bis][302] NR_BS_RF_Part1_E_EIRP	Moderator (ZTE)

- R4-2504628	Topic summary for [114bis][303] NR_BS_RF_Part2_CLTA_SE	Moderator (Huawei)

- R4-2504629	Topic summary for [114bis][304] NR_BS_RF_Part3_OTA_TRP	Moderator (ZTE)

- R4-2504701	Way Forward for [114bis][303] NR_BS_RF_Part2_CLTA_SE	Huawei

- R4-2504732	Draft CR to TS 38.908 EEIRP measurement uncertainty	Huawei, HiSilicon

- R4-2504733	Draft CR to TR 38.908 EEIRP measurement uncertainty and angular transformation	ZTE Corporation, Sanechips

- R4-2504734	Draft CR to TS 38.115-2 for introduction of expected EIRP mask conformance test for NCR	Nokia, ZTE Corporation, Ericsson, Huawei, Samsung, Qualcomm, CATT

- R4-2504735	Draft CR to TS 38.141-2 with updates for EEIRP requirement conformance testing	Nokia

- R4-2504736	Draft CR to TR 38.908 with updates to test procedure	Nokia

- R4-2504737	Draft CR to TR 38-908: Corrections and Improvements related to EEIRP	Ericsson

- R4-2504738	Draft CR to TS 38.141-2: Improvements related to Expected EIRP (EEIRP)	Ericsson

- R4-2504739	Way Forward for [114bis][302] NR_BS_RF_Part1_E_EIRP	ZTE

- R4-2504740	Way Forward for [114bis][304] NR_BS_RF_Part3_OTA_TRP	ZTE

- R4-2504764	Draft CR to TR 38-908: Corrections and Improvements related to EEIRP	Ericsson

- R4-2504765	Draft CR to TS 38.141-2: Improvements related to Expected EIRP (EEIRP)	Ericsson

- R4-2504766	Way Forward for Tx spurious emissions co-existence limits re-evaluation	Huawei

- R4-2504769	Way Forward for Tx spurious emissions co-existence limits re-evaluation	Huawei

RAN4#115

- R4-2506307	Overview of BS-to-BS coupling loss simulation results	Ericsson

- R4-2506308	CR to TR 37.941: Addition of Rel-19 co-location requirement improvements in subclause 6.4	Ericsson

- R4-2506309	CR to TS 38.104: Addition of missing row in applicability table for OTA spatial emission in sub-clause 4.6	Ericsson

- R4-2506535	Views on BS FR1 transmitter co-existence spurious emission requirements	Qualcomm Germany

- R4-2506933	Discussion on OTA test scope reduction	NEC

- R4-2506934	CR to TS 38.104: Requirement set applicability table	NEC

- R4-2506935	CR to TS 38.141-2: Correction on the test procedure	NEC

- R4-2506936	CR to TR 38.908: Correction on the test procedure	NEC

- R4-2507084	Simplification of BS TRP test methods	Nokia

- R4-2507154	CR to TS 38.141-2: improvements related to Expected EIRP for band n104	Ericsson

- R4-2507155	Discussion and text proposal on improvement of TRP measurement methods in Annex I in TS 38.104	Ericsson

- R4-2507244	CR to TS 38.106 for introduction of expected EIRP mask core requirement for NCR	Nokia, ZTE Corporation, Ericsson, Huawei, Samsung, Qualcomm, CATT

- R4-2507245	On BS OTA test scope reduction	Nokia

- R4-2507246	Draft CR to TR 38.908 with improvements and updates	Nokia

- R4-2507247	Draft CR to TS 38.141-2 with updates to OTA spatial emission requirement	Nokia

- R4-2507248	CR to TS 38.115-2 for introduction of expected EIRP mask conformance test for NCR	Nokia

- R4-2507445	Discussion on CLTA-related aspects	Huawei, HiSilicon

- R4-2507446	Tx spurious emissions co-existence limits re-evaluation	Huawei, HiSilicon

- R4-2507447	Draft CR to TR 37.941: Background and motivation for CLTA alternatives discussion	Huawei, HiSilicon

- R4-2507448	Further discussion on European Commission’s feedback wrt. alternative test methods	Huawei, HiSilicon

- R4-2507468	Big CR to TR 37.941: Rel-19 CLTA-related agreements	Huawei, HiSilicon

- R4-2507511	Further discussion on OTA test enhancement	ZTE Corporation, Sanechips

- R4-2507512	Draft CR to TR 37.941: Impact of BS-to-BS isolation analysis on the legacy co-location requirements	ZTE Corporation, Sanechips

- R4-2507513	Further discussion on Coexistence spurious emission requirements	ZTE Corporation, Sanechips

- R4-2507514	Discussion on conformance testing for U6GHz BS EEIRP mask	ZTE Corporation, Sanechips

- R4-2507515	CR to TS 38.908: MU and others	ZTE Corporation, Sanechips

- R4-2507516	Discussion on the OTA testing scope reduction	ZTE Corporation, Sanechips

- R4-2507517	Discussion on the applicability improvement of TRP measurement	ZTE Corporation, Sanechips

- R4-2507633	On OTA test enhancements	Nokia

- R4-2507634	DraftCR to TR 37.941 on CLTA alternatives - wideband horn antenna	Nokia

- R4-2507635	On TX co-existence spurious emissions requirements	Nokia

- R4-2507670	On Transmitter Spurious Emission Co-existence discussion	Ericsson

- R4-2507677	On EEIRP equations and step size equations	Huawei, HiSilicon

- R4-2507678	Draft CR to TS 38.908 corrctions	Huawei, HiSilicon

- R4-2507679	Draft CR to TS 38.141-2 EEIRP measurement uncertainty and coordinate alignment	Huawei, HiSilicon

- R4-2507680	OTA test case reduction	Huawei, HiSilicon

- R4-2507681	Draft CR to TS 38.141-2 OTA test case reduction for TX IM requirement	Huawei, HiSilicon

- R4-2507729	OTA test scope reduction	Ericsson

- R4-2507834	Topic summary for [115][302] NR_BS_RF_Part1_E_EIRP	Moderator (ZTE)

- R4-2507835	Topic summary for [115][303] NR_BS_RF_Part2_CLTA_SE	Moderator (Huawei)

- R4-2507836	Topic summary for [115][304] NR_BS_RF_Part3_OTA_TRP	Moderator (ZTE)

- R4-2507891	PWS MU to be updated for n104 band	ROHDE & SCHWARZ

- R4-2508633	Ad-hoc meeting minutes for [115][302] NR_BS_RF_Part1_E_EIRP	ZTE

- R4-2508634	Ad-hoc meeting minutes for [115][303] NR_BS_RF_Part2_CLTA_SE	Huawei

- R4-2508651	CR to TS 38.104: Requirement set applicability table	NEC, Ericsson

- R4-2508652	CR to TS 38.106 for introduction of expected EIRP mask core requirement for NCR	Nokia, ZTE Corporation, Ericsson, Huawei, Samsung, Qualcomm, CATT

- R4-2508653	CR to TR 38.908: MU and others	ZTE Corporation, Sanechips, Huawei, Rohde&Schwarz, Ericsson,NEC,Nokia

- R4-2508654	CR to TS 38.141-2: improvements related to Expected EIRP for band n104	Ericsson

- R4-2508655	CR to TS 38.115-2 for introduction of expected EIRP mask conformance test for NCR	Nokia

- R4-2508656	Way Forward for [115][302] NR_BS_RF_Part1_E_EIRP	ZTE

- R4-2508657	Way Forward for [115][303] NR_BS_RF_Part2_CLTA_SE	Huawei

- R4-2508658	Way Forward for [115][304] NR_BS_RF_Part3_OTA_TRP	ZTE

- R4-2508761	Analysis of 3GPP TRP TRS AC lab alignment of 2Tx	vivo

- R4-2508762	Way Forward for [115][303] NR_BS_RF_Part2_CLTA_SE: Tx spur co-ex	Huawei

- R4-2508783	CR to TS 38.106 for introduction of expected EIRP mask core requirement for NCR	Nokia, ZTE Corporation, Ericsson, Huawei, Samsung, Qualcomm, CATT

- R4-2508784	CR to TS 38.141-2: improvements related to Expected EIRP for band n104	Ericsson, Nokia, Huawei, ZTE, Keysight, NEC

- R4-2508785	Big CR to TR 37.941: Rel-19 CLTA-related agreements	Huawei, HiSilicon

- R4-2508910	PWS MU to be updated for n104 band	ROHDE & SCHWARZ