3GPP TSG RAN Meeting #108				RP-251497

Prague, Czech Republic, June 9-13, 2025

Agenda item:	16.1

Title:		Proposals for IMT-2030 TPR

Source:		SK Telecom

Document for:	Discussion

Introduction

In RAN#107 meeting, the IMT-2030 TPR was discussed with summary noted in [1]. The list of TPR produced at WP5D February is as follows:

| MTPR | Method |
| --- | --- |
| AI-related capabilities | (TBD) |
| Area traffic capacity | (Quantitative) |
| Average Spectral efficiency | (Quantitative) |
| Peak spectral efficiency | (Quantitative) |
| 5th percentile user Spectral efficiency | (Quantitative) |
| Bandwidth | (Quantitative) |
| Connection density | (Quantitative) |
| Control plane latency | (Quantitative) |
| User plane latency | (Quantitative) |
| Energy Efficiency | (Quantitative) |
| Mobility | (Quantitative) |
| Mobility interruption time | (Quantitative) |
| Peak data rate | (Quantitative) |
| User Experienced Data Rate | (Quantitative) |
| Sensing-related capabilities | (Quantitative) |
| Positioning | (Quantitative) |
| Reliability | (Quantitative) |
| [Joint/composite requirement] |  |
| [Coverage] |  |
| [Security] |  |
| [Resilience] |  |
| [Sustainability] |  |
| [Interoperability] |  |

In this contribution, our views on IMT-2030 TPR discussed in the previous CC#1 and CC#2 are presented, to be addressed in the RAN#108 meeting. For the TPR, two conference calls were held due to the scheduling constraints of WP5D, and SK Telecom also participated in these calls and presented the following proposal.

The overarching principle is that, based on lessons learned from 5G, we should pursue more realistic values for 6G, and discussions on TPR should not exclude new services that users can tangibly experience. In this context, 6G should not be represented or perceived merely as a numerical enhancement of 5G. Instead, performance criteria and evaluation methods for new services should be addressed in continuity with the previous ITU-R Framework document. We hope that 3GPP will prepare future liaison statements in alignment with this direction.

Discussion points on IMT-2030 (6G) TPR

‘values of legacy TPRs’ in section 1.5

Here is a summary table of the legacy TPR 1.5 values, as proposed by SK Telecom.

| TU IMT-2030 TPR | IMT-2020 Value |  | Values proposed in RAN 107 meeting | Moderator proposal | SKT's proposal |
| --- | --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) | DL: 20 UL: 10 |  | [1~10x] of IMT-2020 | TBD | Same as / or more than IMT-2020 |
| Peak spectral efficiency (bit/s/Hz) | DL: 30 UL: 15 |  | [1~3x] of IMT-2020 | [2x] of IMT-2020 | Same as / or more than IMT-2020 |
| User experienced data rate (Mbit/s) | DL: 100 UL: 50 |  | DL: [1.25~10x] of IMT-2020 UL: [1.25~4x] of IMT-2020 2 contributions propose to revisit the evaluation methodology before setting values: - Bandwidth scaling of simulation results for the UL User Experienced Data Rate performance should NOT be used. | DL: TBD UL: Revisit the evaluation methodology before setting values Further discuss applicable deployment scenarios | Same as / or more than IMT-2020 |
| 5th percentile user spectral efficiency (bit/s/Hz) | Indoor Hotspot-eMBB: - - DL: 0.3 - - UL: 0.21 Dense Urban - eMBB: - - DL: 0.225 - - UL: 0.15 Rural-eMBB: - - DL: 0.12 - - UL: 0.045 |  | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | [1.5-2x] IMT-2020 |
| Average spectral efficiency (bit/s/Hz) | Indoor Hotspot - eMBB: - - DL: 9; - - UL: 6.75 Dense Urban - eMBB: - - DL: 7.8; - - UL: 5.4 Rural - eMBB: - - DL: 3.3; - - UL: 1.6 |  | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | [1.5-2x] IMT-2020 |
| Area traffic capacity (Mbit/s/m²) | DL: 10 |  | [1~10x] of IMT-2020 | [10] | Same as / or more than IMT-2020 |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | eMBB: [1~4] HRLLC: [0.1~1] | TBD | Same as / or more than IMT-2020 |
|  | Control plane latency (ms) | 20 | [5~20] | TBD | Same as / or more than IMT-2020 |
| Connection density (devices/km²) | 10⁶ |  | [1~100x] of IMT-2020 | [10x] of IMT-2020 | Same as / or more than IMT-2020 |
| Reliability | 1 - 10⁻⁵ |  | [1~100x] of IMT-2020 | [1-10-5] | Same as / or more than IMT-2020 |
| Mobility (bit/s/Hz @ km/h) | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h |  | 1x of IMT-2020~ 1000/1200km/h | Keep the velocities defined for IMT-2020. Should new velocity be added? (up to 1000/1200km/h for airplane) Should 50% spectrum efficiency be increased for the velocity defined in IMT-2020? e.g. [1.5x] of IMT-2020 | keep the velocity defined for IMT-2020 No new velocity class |
| Mobility interruption time (ms) | 0 |  | 0 ms | [0 ms] | Same as / or more than IMT-2020 |
| Bandwidth (MHz) | 100 |  | 200MHz or 400MHz | 400MHz for system bandwidth (same definition as IMT-2020) | 400 MHz |

Proposals for ‘other’ TPRs in section 2.2.

We hope to retain coverage and interoperability as TPR items. We would like to further discuss and define the detailed requirements, evaluation criteria, and evaluation methods through additional discussions.

Conclusion

The proposals in Section 2 are proposed to be considered in RAN#108 meeting of IMT-2030 TPR discussion.

References

- RP-250819, Moderator's summary for IMT-2030 TPRs, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025.