3GPP TSG RAN Meeting #108	RP-251517
Prague, Czech Republic, June 9-13, 2025

Agenda item:	16.2

Source:	Telstra

Title:	Telstra views on 6G IoT

Document for:	Discussion and Decision

# 1	Introduction

The 3GPP 6G workplan and timelines [1] was noted at the 3GPP 6G workshop held in Incheon in March 2025. The timeline calls for the approval of RAN working group Study Item at RAN#108, June 2025. This submission offers insights and proposals for consideration around operator requirements for 6G IoT and follows from Telstra vision presented its vision for 6G IoT at the 3GPP 6G workshop held in Incheon in March 2025. [2]

Furthermore, a joint submission from a mix of operators [3] was presented that provided guiding principles for 6G:

- Scalable 6G RAT that supports diverse device types ranging from the lowest capability device (e.g., 3GPP LPWA) to higher capability devices in the first release. Ensure a globally aligned meaningful minimum set of capabilities and normatively specified testing and certification procedure for 6G RAT.

- Introduction of 6G native IoT technology, including LPWA technology, in the first release that facilitates long term commercial obligations that may outlive the 6G RAN.

# 2	Justification

NB-IoT and LTE-M were initially specified as 4G MTC solutions, with Telstra enabling support nationally in 2017. Since then, operators are increasingly being asked to make very long-term commitments (10+ years) to support massive IoT use cases (for example: water meters, connected cars) using these technologies.

3GPP recognised NB-IoT & LTE-M as the massive IoT solutions for 5G and included support for these categories in Release 16, with native support in 5G RAN and Core. However, it has become increasingly obvious that many ecosystem vendors do not see the need to uplift capabilities beyond Release 13, particularly when considering many simple use cases the categories are aimed at, for example location tracking.

This means these devices can only be supported on 4G, which inhibits the operator’s ability to migrate spectrum to 5G (or 6G) and means missing out on significant spectral efficiency gains.

Furthermore, 3GPP introduced higher capability IoT categories in Release 17 (Redcap) and Release 18 (eRedcap) to support higher throughput use cases, for example surveillance cameras. These new categories are limited to 5G standalone only and do not specifically support coverage recovery techniques, so cannot match NB-IoT or LTE-M coverage.

# 3	Objectives

Two 6G IoT categories (high & low) are needed from day one with the following capabilities:

- Maximum and minimum data rates for each category are to be determined by 3GPP study

- Maximum Coverage Loss (MCL) to meet or exceed NB-IoT for 6G IoT - low

- Optimise for very long battery life and low device cost for 6G IoT - low

- Support for higher UE Power Classes, eg PC1, 1.5, 2

- Native support in 6G Core

We do not see the need for backward compatible support of NB-IoT and Cat-M devices in the 6G system.

In addition, IoT technologies are theoretically capable of supporting thousands, or tens of thousands of IoT devices per cell. However, application developers frequently fail to apply sound design practices to avoid IoT devices causing signalling storms.

6G shall offer a network-controlled access mode in which the network allocates a precise time window in which each IoT device is allowed to access the network. In addition, an early indication technique shall be considered.

It shall be possible for the 6G RAN to isolate IoT device access to avoid unexpected IoT signalling surges from overwhelming other device types.

# 4	Conclusion

In this contribution we propose the objectives presented are considered for inclusion in TR 38.914 [4].

# References

- [1] 	6GWS-250240: 3GPP 6G workplan and timelines

- [2] 	6GWS-250091: Telstra’s Vision and Priorities for 6G RAN

- [3]	RP-250771:	6G Standardization – Principles, “T-Mobile USA, AT&T, BT, Ericsson, KPN, KT, MediaTek, Nokia, NTT DOCOMO, Qualcomm, Rakuten, Samsung, Softbank, Telstra”

- [4]	TR 38.914: Study on scenarios and requirements for next generation access technologies