3GPP TSG RAN Meeting #108						          RP-251524

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.2

Source:	CEWiT

Title:	Views on 6G RAN WG Study Items

Document for:	Discussion/Decision

# Introduction

In RAN#107, the drafting of the scope of the study on 6G scenarios and requirements was initiated and was captured in RP-250810 [1] with acronym: FS_6G_RAN_Scen_Req. The document defines the basic 6G design requirements and paves the way for defining the scope of 6G RAN WG level SIs to be taken up as a part of Rel-20. Accordingly, it is expected that RAN#108 will approve the final package on Rel-20 RAN WG level SI on 6G. The table below shows the primary objectives considered in RP-250810.

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types

- Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework Note: The detailed time plan for the high-level decisions above will be decided at RAN#108 at the approval of the 6G RAN WG SI and is to be aligned with the RAN SI progress.

In this contribution, we discuss our views on different aspects to be considered as a part of Rel-20 RAN WG SI on 6G.

# Guidelines for Rel-20 RAN WG SI on 6G

## Considerations from FS_6G_RAN_Scen_Req

Our main take based on all the companies view on defining the scenarios and requirements is that the 6G SI in the WG level should be very focused on defining a sustainable single drop release of 6G satisfying the IMT 2030 requirements.  Based on the content of the agreed SID RP-250810 [1], we list down the considerations 3GPP should follow , in defining the Rel-20 6G SI for all the WGs.

- Though it started with the intention of defining flexible configurations, 5G reached the direction where each feature has numerous solutions with only a minimum of them is implementable . So, striving to minimize the options considered and flexibility required and also avoid defining excessive UE capabilities making the features optional and unnecessarily extensive capability reporting

- Taking into account of the requirements defined by IMT2030, 6G should be designed in an energy efficiency manner both for network and device, still satisfying the spectral efficiency and coverage (DL/UL) requirements. This includes enhanced Phy features like MIMO, Waveform, encoding etc along with a lean design of the higher layers.

- Based on all the operators view during RAN#107, defining a proper migration plan should be considered with the top priority in defining the scope of WG SI for 6G as 6G is expected to be deployed re-using the existing 5G mid-band (~3.5GHz) and site grid using spectrum refarming, and the around 7 GHz band targeting comparable coverage to 5G mid-band, using enhanced beam-management features.

- Along with the migration plan, defining a proper architecture for 5G-6G integration from network and UE perspective is very important. Based on the inputs by companies during RAN#107, the options on the table include 5G/6G-Stand Alone, 5G-6G Dual Connectivity, and 5G-6G-Dual Steering with CN-aggregation/split.

- Regarding support of multiple device types, 6G from Day1 should consider a unified design without compromising the base design for corner-case usages. Mainly this impact is expected in Initial access design, minimum BW for the design of broadcast signals, waveform capability and requirement across different device types. Though, this is not achievable for all features from the initial release, 3GPP should target scalable and forward compatible design considering the support of diverse device types in later releases.

## Guidelines for Rel-20 6G WG SI

The following section defines detailed guidelines to be considered to define the SI scope of 6G to the WGs, based on various aspects like

- Migration plan from 5G to 6G

- Basic design of RAN

- Unified and future proof design

- Target device types and capabilities with minimum mandatory features

### Migration plan from 5G to 6G

One of the key driving requirements to consider in executing the 6G SI in 3GPP WGs is the commercially viable migration plan for the operators and network vendors to move from 5G to 6G. Based on the inputs received during RAN#107 and also the SID scope of FS_6G_RAN_Scen_Req , we propose possible 6G design aspects to be considered for smooth migration from 5G network deployment.

#### Multi-RAT Spectrum Sharing

MRSS was discussed by the majority of companies as a simple and efficient way of deploying 6G in the current 5G (3.5GHz) bands. The system architecture for MRSS in a co-located site of 5G and 6G using the same band can be depicted as shown below

Fig 1. MRSS Architecture with optional co-ordination of resources (static / dynamic)

Based on the deployment requirements a static configuration of 5G and 6G TDD configurations can be assumed, or an interface from the 5G RAN node to the 6G RAN node can be facilitated, reusing the existing TDD configurations of 5G. This interface can be assumed to be used in a semi-static or dynamic manner to share TDD configurations as per the network requirements. This guarantees a simple standalone 6G system co-existing with the 5G deployments preserving legacy 5G UEs and allowing 6G UEs into the network.

#### 5G-6G NW architecture based on UE stack options

Though MRSS provides a simple solution for the co-existence of 5G and 6G, this is achieved at the cost of

dividing the spectrum between 5G and 6G using TDM/FDM. This result in reduced data rates for both 5G and 6G UEs. Though 6G UEs may benefit from the new 6G bands around 7GHz using CA, the 6G UE may not be utilized for it’s full potential in the anchor cell under the 3.5GHz band. To counter this, 6G NW can be architected

Fig 2. NW configurations for single stack UE

considering the UE stack options.

If we consider 6G UE to be capable of having only a single active stack at the time, still supporting both 5G and 6G, the following options are considered for the NW configurations .Now as shown in Fig.2, the UE supports both 5G and 6G stacks with only one active at a time. Wherein the option on left, the UE is connected to either 5G or 6G with support for inter-RAT mobility and in the option on right, the UE is connected to both 5G and 6G at a time with a dynamic TDM switching between 5G and 6G connectivity.

Fig 3a. NW configurations for dual stack UE with single CORE

Similarly, if the UE supports dual connectivity, the UE can be connected to both 5G and 6G at the same time with the options of RAN level aggregation, CORE level aggregation with dual connectivity and dual steering based architectures, as depicted in Fig.3a. considering a single 5GC or 6GC or using separate 5GC and 6GC as in Fig.3b.

Fig 3b. NW configurations for dual stack UE with 5GC and 6GC

#### Future proof enhanced network architecture

Though migration from 5G to 6G is the first step, we strongly believe, the study should also focus on future proofing the architecture for advance scenarios, which will be required from day1 targeting the current 5G pain points. One of the major pain points in 5G identified by many operators and NW vendors was the usability of DC beyond EN-DC. Though targeted towards capacity improvement, NR-DC proved to be of little use except for HetNet deployments in certain countries. We believe studying better Intra-RAT connectivity options for capacity improvement and spectrum utilization of the 6G NW also plays important role in making the 6G network future proof. So, we propose the following guideline to be studied during Rel-20 considering both the migration from 5G-6G and also future proofing the network architecture.

Proposal 1: Consider the following bullets as guidelines for defining proper NW architecture, interfaces between the RAN node and the CORE NW of 6G by studying various options of network architecture, radio interface protocol architecture and procedures [in co-operation with SA2]

    - Study the feasibility of different options of different RAN node connectivity and MRSS for the co-existence of 5G and 6G in the same band, with the following options

      - Static/Dynamic MRSS reusing existing 5G TDD configurations

      - Inter/Intra-RAT DC, Inter/Intra-RAT dual steering from CORE

      - 5G and 6G RAN node connectivity to a single CORE NW (5GC/6GC)

    - Study and outline the RAN-Core Network interface options based on the UE stack and functional split

### Basic design of RAN

#### Physical Layer signal structure

The important requirement in defining the first release of 6G is to have a very simple design for the RAN node with features targeting realistic scenarios and feasible design. This starts with a simple frame structure design, unlike 5G which was targeted to support multiple numerology. Considering the realistic usage of multiple numerology feature, we propose to support a simple frame structure design supporting single numerology per component carrier at a time.

##### Initial Access Design

The next major design aspect to consider is the initial access of the 6G NW. Though 5G initial access is a major improvement over the LTE in terms of managing the deployment requirement of multiple virtual sectors, 6G initial access is required to accommodate more features to simplify the beam management procedures. Beam management in NR is a set of procedures at the BS used for identifying the best transmit and receive beam pair that can be used to serve a connected UE, tracking the quality of the beams and changing the serving beam of a UE appropriately as and when necessary. To accommodate this, the 3GPP 5G NR standards have defined a set of procedures viz., P1, P2 and P3. The main disadvantage of beam management in NR is that the time taken for obtaining a best Tx-Rx beam pair link is very large. As a result of this, the data transmission for the UE gets delayed, which will impact the overall system performance. Also, considering the SID direction of re-using existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band, beam management is one crucial feature in 6G to acquire the best servable beam and doing so as early as possible is also a crucial part of the initial access requirement.

In order to mitigate this problem, in 6G the initial access procedure should include a simple beam management or a low-resolution positioning phase such that the UE doesn’t experience any delay in beam acquisition and also facilitate other features like multi-user MIMO pairing, mobility, energy saving etc. The beam management phase in the initial access should be a lite version of the actual P2 procedure that will be performed in the later stage. Similarly, the requirements for the UE location accuracy need not be as stringent as the legacy positioning services realized in the 5G NR systems while assuring better precision than the serving mobile network tower location and the timing-advance (TA) information can offer.

Another major concern to be resolved in 6G is the energy expenditure of the network primarily due to broadcast and periodic signals. And initial access is one important area which requires proper designing re-using the on-demand design framework both from the network side and UE side. Features/Capabilities specific to initial access adaptations (E.g., lean carrier design, OD-signal transmissions, flexible adaptation of transmission pattern/periodicity per SSB, early msg 1 etc.), considering UE initiated procedures as baseline, are needed to be considered from day1 as a part of the basic design in 6G. Considering the extensive work in NR, Rel19 features on NES for Initial access can be leveraged.

During the 6G workshop, a general view was presented by many companies to have Day1 support to NTN and strive to integrate NTN and TN in 6G. In 5G NR, NTN was after thought which brough many limitations to the solution devised for NTN. This should be avoided. Therefore, NTN related aspects should be considered during designing the 6G feature including initial access, data and control channel, reference signal design, network split and interfaces, etc. Also different device type requirements like LPWA with limited BW capabilities (3MHz) should also be considered in the initial access design.

##### Control and shared channel design

For the design of control and shared channels, it is important to first identify the requirements of different device types and scenarios to be considered, as a baseline design constraint. For e.g. we have different device types like LPWA devices expecting a simpler waveform and NTN requiring a energy efficient waveform, while TN is happy to continue with OFDM. Therefore, the waveform for 6G should be flexible enough to accommodate different device and network requirements while still utilizing the benefits of OFDM. Also simple DCI design splitting the information targeting reduced blind detection and improving control plane latency should be considered.

Similarly, modulation and coding schemes and RS design should consider realistic scenarios and requirements based on them to lay the foundation of the 6G physical layer. Especially considering different channels requiring different reliability and coverage, the modulation and coding scheme along with the RS design should be targeted for these specific requirements. Also, for the RS design considering the extension of the ports, overhead and energy consumption becomes very crucial to be optimized from day 1. So, adaptation of RS, targeting different scenarios and requirements using on-demand signaling, AIML should be considered.

#### Radio interface protocol architecture and procedures

Next major consideration towards the 6G RAN node is the design of the protocol stack. 5G protocol was an adaptation from  age-old generations. Though it is operational, it is not optimized for the new requirements of different traffic types like XR, which has stringent latency requirements. Therefore, it is time to take up the long overdue work of redesigning the protocol stack considering the new QoS requirements of many traffic types.

By targeting a lean protocol stack, 6G can achieve a simpler implementation, making it lighter on the UE side, facilitating a smoother handling of packets by different applications. Also having the native design of PDCP and RLC handling the packet QoS flows makes the MAC slower and inefficient in making a lot of decisions. Hence in 6G we propose to have a MAC centric protocol stack with more freedom and involvement for the MAC in making a lot of decisions on handling the packet queues and also managing ACK/NACK for the packets.

Though 5G tried providing more power to the MAC in LTM mobility design, security became one major concern to be addressed in NR for LTM handovers. Therefore, in 6G the more and more MAC centric, the design becomes, security at MAC level needs to be ensured targeting at least sensitive network information like security keys.

Considering all these requirements we propose the following

Proposal 2: Consider the following bullets as study guidelines for defining proper design of the 6G RAN node

  - Physical layer signal structure for 6G [RAN1]

    - Simple frame structure design for different device types and bands, supporting at least a single numerology configuration.

    - Initial access design

      - Faster/stable beam management (e.g. incorporating BM reports, localization/positioning) for new 6G bands including supporting measurements as part of RAN measurement framework

      - On-demand signaling and adaptation (UE initiated, with NR as baseline)

      - Supporting different device type requirements (e.g. LPWA with 3MHz BW)

      - Supporting diverse deployment scenarios (TN & NTN)

    - Control and shared channels design

      - Waveform for different device types (e.g. LPWA) and different scenarios (e.g. NTN), and coverage of different channels (e.g. PDCCH Vs PDSCH Vs PUCCH Vs PUSCH)

      - Modulation and Channel coding scheme(s)

      - RS Design accommodating realistic scenarios, optimizing on overhead (On demand, AIML based, device requirement based)

      - Design targeting reduced blind decoding

  - Radio interface protocol architecture and procedures [RAN2]

    - Targeting latency aware lean architecture

    - MAC centric design, including management of QoS flow handling, ACK/NACK handling, handovers etc.

    - Unified UP and CP protocols for TN and NTN

    - Feasibility of MAC level security, at least for sensitive network information (e.g. Security key exchange)

### Unified and Future proof design

To ensure an appropriate set of functionalities in 6G, minimizing the adoption of multiple options for the same functionality, avoiding excessive configurations and excessive UE capabilities along with the reporting, simple and unified framework design is the key. And this can be achieved only if we strive to design a RRC which has simple configuration options along with a unified framework for multiple similar features. Some of the key features according to us to be considered in optimizing for a simpler RRC are

#### Spectrum aggregation under single cell

Even though the current carrier-aggregation framework supports independent configuration of PCell and Scells under a cell group, network deployments use very minimal variation of the configurations across carriers. This reflects the over-design of the NR RRC, where at least the intra-band carriers can be aggregated under single cell having a lot of common configurations along with simpler measurements. This was discussed by multiple companies in the 6GWS and considered to be one of the best approaches for 6G to handle carrier aggregation in RRC.

#### Unified BM-mobility and measurement framework

Though beam management and mobility are two distinct features, BM was extended from the intra-cell co-ordination to inter-cell co-ordination (ICBM and MTRP) and similarly mobility was extended from RRM to MAC closer to BM (LTM). Moving forward into 6G, having two independent frameworks for mobility and beam management is unnecessarily making the features complex and also making the configurations redundant. So in 6G, we should strive to design a unified framework supporting different scenarios of mobility and beam management from Inter-CU handover and Inter-DU handover to Intra-DU beam management, making measurements for both beam management and mobility under a single place in RRC.

#### Unified LCM framework for AIML

Life cycle management for AIML in NR is one major accomplishment, but one minor drawback of the design to be rectified in day1 of 6G is the diversification of the LCM procedures for different AIML features. Though it felt flexible and reasonable, moving forward to 6G with the target of AI-native 6G, having different LCM procedures for each feature becomes hectic and redundant. So 3GPP should strive to design a unified framework for the LCM, accommodating the requirements of the basic AIML features to be adopted from NR like BM, positioning, CSI prediction, mobility etc. and considering future releases of 6G.

Considering all these requirements we propose the following,

Proposal 3: Consider the following bullets as study guidelines for unified and Future proof design of the 6G network

  - Redesigning Radio Resource Configuration [RAN1, RAN2]

    - Supporting simpler NW configuration

      - Spectrum aggregation under single cell

      - Unified BM-mobility and measurement framework

      - Unified LCM framework for AIML

### Target device types and capabilities

One final requirement to be considered as a part of the 6G WG SI is the support of different device types. Unlike 5G, 6G may not prioritize diverse device types from day1 but should consider minimal devices with market requirements.

#### FWA customizations

FWA device is one major market for operators to provide broadband connections in many countries where fiber to the home is not optional. And for such devices, though NR provides optimizations like relaxed measurements based on stationarity conditions through RRC, beyond that, not many optimizations were considered. FWA devices experience very high data traffic as the FWA device serves multiple users at a time. Hence, to serve an FWA device, the BS has to constantly schedule the resources to satisfy the high traffic requirements of FWA. This will result in very high scheduling overhead and also RS overhead which needs to be optimized considering the device's stationarity.

Another key feature to consider for FWA customizations is to consider the sophisticated device capability of FWA, with no power limitation, advanced form factor and circuitry. This allows only FWA to handle certain functionalities like an increased number of RX antennas and advanced duplexing capabilities. Unlike the existing use cases, the use cases for FWA demand high performance in both DL and UL. e.g., holographic conferencing, which involves high throughput and low latency in UL, along with the DL throughput and latency requirements. Two major features to consider as a part of FWA customizations are increased Rx antennas and also SBFD capability on UE sides. Both these features at the UE side can ensure significant reduction in UL latency without compromising on DL latency from a UE perspective and increases both UL and DL throughput.

#### LPWA device support

LPWA devices are expected to be of two extremes with one device type is a high-end devices like RedCap/e-RedCap device and supporting minimum BW of 3 MHz, same coverage as NR, energy saving features, etc. Second device type is a low-end device like NB-IoT, supporting minimum BW of 1 PRB, same coverage as NR, energy saving features, etc.

RedCap devices were introduced in later releases of NR, with limited optimization of features/capabilities due to high impact on specification and legacy UEs. This imposed design constraints like following existing cell common procedures in NR (E.g., Initial access/RACH procedure) and minimum BW supported is same as in NR (i.e., 5 MHz). In 6G to support high-end LPWA devices, we should strive for a unified design of initial access, common channels etc. while to support the low-end LPWA devices, we propose to consider a separate design from eMBB to avoid performance degradation to other device categories and UEs.

Proposal 4: Consider the following bullets as study guidelines for different device types and capabilities as a part of the 6G RAN SI

  - Consider different device types and capabilities to optimize and enhance features

    - Support LPWA device types

      - Low-end, using IoT upgradation

      - High-end, using RedCap enhancements

    - Support FWA customizations considering stationarity and capability

      - Overhead optimizations, increased Rx antennas and SBFD capability

### Additional features for 6G

#### 6G positioning

Positioning being one of the IMT 2030 TPR is important for the 6G system to specify the positioning framework from Day1. 5G positioning framework should be adopted as based line. In NR and LTE positioning is considered a service and trigger for external applications predominately. In 6G positioning can be used for multiple purposes including network optimization, AIML data parameterization, SON, lawful interception (for NTN case) along with the traditional positioning services. Considering the modifications to positioning framework, the UE privacy and consent check framework should be enhanced to support new services protecting user privacy.

#### 6G ISAC

ISAC being an IMT 2030 use case, it is important to enable this feature in 6G. ISAC in 6G should be standalone and should not be derived from 5G-A feature if defined in NR in Rel 20. This is because, in NR ISAC scope is decided to be minimum and may not include an aspect to make feature robust and complete. Therefore, in 6G, ISAC should be studied fresh without bearing on 5G NR work. Further ISAC study should include feasibility of different scenarios, suitable sensing modes, sensing performance in different frequency bands (FR1, FR2, FR3), KPIs for ISAC, resource sharing, waveform design and protocol support. User planes should be explored more for transporting sensing data.

#### Single cell large radius deployment scenario

Figure 1: Test environment covering multiple small villages with a large standalone cell.

We envision terrestrial coverage to be an important component of Ubiquitous Connectivity, and it should evolve with every new generation. In IMT-2030, TN will be complemented by NTN wherever terrestrial coverage is missing. TN is expected to provide coverage to 95% of the population and with a higher user experienced data rate and cell edge data rate than NTN.

While NTN can be a big driver for serving under connected areas where there is not much habitation of people, we strongly believe that enhanced terrestrial coverage is still necessary to serve under connected areas which do have significant habitation of people (example: rural areas including villages and small towns such as those in the Indian subcontinent). NTN is not expected to handle the very large population density seen in rural environments of developing countries including India and is also not expected to provide the above-mentioned user experienced data rate and cell edge data rate.

One type of deployment that we envision includes sparse deployment of large cells without contiguous multi-cell coverage. We believe that this type of deployment is to offer broadband services and is therefore best suited for a new test environment, which we call Rural-UC. The evaluation parameters should be revisited (compared to Rural-eMBB in IMT-2020) to include different UE heights and different UE power classes.

Proposal 5: Consider the following bullets as new feature in 6G to compliance with IMT 2030 requirements

  - Consider the following feature for 6G study

    - 6G positioning

    - ISAC in 6G

  - Adopt single cell large radius as one of the mandatory deployment scenario for 6G study (Targeting ubiquitous connectivity)

# Conclusion

The following proposals need to be considered in drafting the SID of the new 6G SI during the RAN#108 meeting.

Proposal 1: Consider the following bullets as guidelines for defining proper NW architecture, interfaces between the RAN node and the CORE NW of 6G by studying various options of network architecture, radio interface protocol architecture and procedures [in co-operation with SA2]

  - Study the feasibility of different options of different RAN node connectivity and MRSS for the co-existence of 5G and 6G in the same band, with the following options

    - Static/Dynamic MRSS reusing existing 5G TDD configurations

    - Inter/Intra-RAT DC, Inter/Intra-RAT dual steering from CORE

    - 5G and 6G RAN node connectivity to a single CORE NW (5GC/6GC)

  - Study and outline the RAN-Core Network interface options based on the UE stack and functional split

Proposal 2: Consider the following bullets as study guidelines for defining proper design of the 6G RAN node

  - Physical layer signal structure for 6G [RAN1]

    - Simple frame structure design for different device types and bands, supporting at least a single numerology configuration.

    - Initial access design

      - Faster/stable beam management (e.g. incorporating BM reports, localization/positioning) for new 6G bands including supporting measurements as part of RAN measurement framework

      - On-demand signaling and adaptation (UE initiated, with NR as baseline)

      - Supporting different device type requirements (e.g. LPWA with 3MHz BW)

      - Supporting diverse deployment scenarios (TN & NTN)

    - Control and shared channels design

      - Waveform for different device types (e.g. LPWA) and different scenarios (e.g. NTN), and coverage of different channels (e.g. PDCCH Vs PDSCH Vs PUCCH Vs PUSCH)

      - Modulation and Channel coding scheme(s)

      - RS Design accommodating realistic scenarios, optimizing on overhead (On demand, AIML based, device requirement based)

      - Design targeting reduced blind decoding

  - Radio interface protocol architecture and procedures [RAN2]

    - Targeting latency aware lean architecture

    - MAC centric design, including management of QoS flow handling, ACK/NACK handling, handovers etc.

    - Unified UP and CP protocols for TN and NTN

    - Feasibility of MAC level security, at least for sensitive network information (e.g. Security key exchange)

Proposal 3: Consider the following bullets as study guidelines for unified and Future proof design of the 6G network

  - Redesigning Radio Resource Configuration [RAN1, RAN2]

    - Supporting simpler NW configuration

      - Spectrum aggregation under single cell

      - Unified BM-mobility and measurement framework

      - Unified LCM framework for AIML

Proposal 4: Consider the following bullets as study guidelines for different device types and capabilities as a part of the 6G RAN SI

  - Consider different device types and capabilities to optimize and enhance features

    - Support LPWA device types

      - Low-end, using IoT upgradation

      - High-end, using RedCap enhancements

    - Support FWA customizations considering stationarity and capability

Overhead optimizations, increased Rx antennas and SBFD capability

Proposal 5: Consider the following bullets as new feature in 6G to compliance with IMT 2030 requirements

  - Consider the following feature for 6G study

    - 6G positioning

    - ISAC in 6G

  - Adopt single cell large radius as one of the mandatory deployment scenario for 6G study (Targeting ubiquitous connectivity)

# Reference

[1] RP-250810, Revised SID: Study on 6G Scenarios and requirements, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025