3GPP TSG RAN Meeting #108						           RP-251557

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.4.4

Source:	CSCN, China Unicom, Xiaomi, ZTE Corporation, Sanechips, CATT, OPPO

Title:	Views on NTN related RAN4 led R20 topics

Document for:	Discussion/Decision

Introduction

In RAN#107 meeting, some NTN related RAN4 led topics in Rel-20 has been summarized in RP-250817 [1] as following.

Summary/conclusions

In the section, the high level candidate objectives for NTN requirements evolution and/or enhancement in Rel-20 are captured, where companies share the common interests in the bullet(s) highlighted by blue. The candidate objectives below are listed without priority.

The following list of high level candidate objectives can be used as the starting point for NTN requirements evolution and/or enhancement in Rel-20, considering the proper TU splitting between 5G and 6G.

- NTN HPUE

  - Study whether the high power than PC1 for IoT-NTN and/or NR-NTN, and what high power classes should be considered.

  - FFS whether the proposed high power class can be applied to handheld + FWA or only FWA.

  - Need differentiate NR-NTN and IoT-NTN in terms of power class to be discussed.

- Enhancements for Ku band NTN, including

  - Intra-SAN intra-band CA in Ku band

  - Fixed and other mobile types VSAT (e.g., with small antenna size or LEO only) RF [and RRM] requirements for single carrier

    - For small antenna size, HD-FDD would be needed.

  - Smaller channel bandwidth support (e.g. 3, 5 MHz) and other Rel.19 leftovers

- NTN CA including intra-band and inter-band UL/DL CA for NR-NTN

- NTN mobile VSAT

- Testing method for VSAT

- NTN power domain enhancements

- MSR for in-band IoT-NTN in NR

- Irregular CBW 7MHz for NR-NTN

- Mobility between NR-TN/LTE TN and NR-NTN in the RRC_CONNECTED state.

  - Need check whether RAN2 package includes it.

- Repeater for NTN

NOTE: there are overlapping objectives above and FFS on how to merge them.

NTN related RAN4 led R20 topics

NTN HPUE

For NTN operation, uplink coverage is one of the major issues, while the study on HPUE is an effective solution. Device types supported for each power class depend on feasibility study and corresponding requirements. In addition to non-handheld device with different power classes discussed in previous releases, we are interested in supporting PC1.5 and PC1 for handheld device as well. PC1.5 hand-held UE for NR-NTN is precluded in Rel-19. At least for PC1.5, we understand there are some potential implementation challenge on RF front-end design (e.g. Duplexer). However, on existing TN TDD bands, PC1.5 has already been supported for handheld devices with 2Tx (2 PC2 PA).  One possible way to support PC 1.5 for handheld device on NR NTN FDD bands is to reuse PC2 PA with HD-FDD mode by alleviating the implementation challenge of the SAW/BAW filter in low FR1 FDD bands.

Proposal 1: Support PC1.5 HPUE for NR NTN handheld devices in 5G-A Rel-20.

NTN HD-FDD mobile VSAT

Although full-duplex systems have the advantages of higher throughput and lower latency in general, half-duplex systems are fully capable of meeting the performance requirements in many realistic UE applications without high data rates, and achieving lower implementation complexity as the same time.

Considering the critical commercial interests of potential NTN enhancements in 5G-A Rel-20, half-duplex FDD mobile VSAT has a clear commercial demand which comes from the lower cost and smaller antenna. In this configuration, UE device can use one phased antenna array to fulfill the requirement of transmission and reception in half duplex mode. It can significantly reduce the size and weight of NTN device and simplify RF design with higher reliability, which is very necessary in the scenarios of vehicle mounted terminals, ship-borne terminals and portable terminals. Consequently, it is the key to making NTN VSAT devices more popular, and then achieves commercial success in the end.

Proposal 2: Support NTN HD-FDD mobile VSAT in 5G-A Rel-20.

In addition, compared with Ku band, Ka band can accommodate a wider channel bandwidth. Owe to the flat array antenna with even smaller size and lighter weight, mobile HD-FDD VSAT operating in Ka band may have better prospects and more potential applications.

In light of the similar RF and RRM requirements for mobile VSAT, it is a feasible and more efficient approach to consider Ka band and Ku band within a unified framework. If the outcomes of the study on Ku band can be reused to Ka band as much as possible, or vice versa, the overall workload can be minimized to the greatest extent possible.

Proposal 3: Propose to study the NTN HD-FDD mobile VSAT for both Ka band and Ku band.

Conclusion

Proposal 1: Support PC1.5 HPUE for NR NTN handheld devices in 5G-A Rel-20.

Proposal 2: Support NTN HD-FDD mobile VSAT in 5G-A Rel-20.

Proposal 3: Propose to study the NTN HD-FDD mobile VSAT for both Ka band and Ku band.

Reference

[1] RP-250817, Moderator's summary for RAN4 led REL-20 topics