3GPP TSG RAN Meeting #108						          RP-251559

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.2

Source:	CSCN, China Telecom, China Unicom, CATT

Title:	Views on 6G RAN WG SI for NTN

Document for:	Discussion/Decision

Introduction

In RAN#107, the Revised SID: Study on 6G Scenarios and Requirements was approved in RP-250810 [1]. It was agreed to aim at a harmonized 6G Radio design for TN and NTN, including their integration in 6G as follows.

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

  - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting

  - Energy efficiency and energy saving: both for network and device

  - Enhanced spectral efficiency

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

  - Target scalable and forward compatible design for diverse device types

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

In this RAN plenary meeting, the new RAN WG SI on 6G is expected to be discussed. In this contribution, we provide our views to cover the 6G study for NTN.

6G RAN WG SI for NTN

Before the discussion on 6G NTN, we need to first take a look at the development of 5G NTN.

In 5G, a R15 Technical Specification Group (TSG) RAN led study item (SI) "Study on NR to support non-terrestrial networks" (FS_NR_nonterr_nw) with TR 38.811 was working on the following [2]:

- Summarizing the use cases.

- Adapting the 3GPP channel model (developed in REL-14 TR 38.901 under the 5G REL-14 SI FS_NR_newRAT) for non-terrestrial networks.

- Providing a detailed description of the deployment scenarios for non-terrestrial networks and analyzing the necessary adaptations to support operation via Satellite or HAPS in NR.

In R16, a Working Group (WG) RAN3 led SI "Study on solutions for NR to support non-terrestrial networks (NTN)" (FS_NR_NTN_solutions) resulted in TR 38.821 [3]. The focus of this SI was on:

- Satellite access (with transparent GEO satellite and LEO-based non-terrestrial access network (moving beam on earth), i.e. UAS/HAPS based access was considered here as a special case of NTN with lower Doppler and variation rate.

- Usage scenarios: pedestrians or users on board a vehicle (high speed train, airplane).

In R17, timing, synchronization and HARQ enhancements, mobility management, switch-over, NG-RAN signalling, AMF (Re-)Selection by gNB, O&M Requirements, RF performances and RRM requirements were specified in WI “Solutions for NR to support non-terrestrial networks (NTN) (NR_NTN_solutions) ”  [4].

In R18, the WG RAN2 led WI "NR NTN (Non-Terrestrial Networks) enhancements" (NR_NTN_enh) had the goal to enhance NTN-related features introduced in R17, including uplink coverage enhancement, network-verified UE location, NTN-TN and NTN-NTN mobility and service continuity enhancements, and NR-NTN deployment in bands above 10 GHz [5].

In R19, the WG RAN2 led WI " Non-Terrestrial Networks (NTN) for NR Phase 3" continues to enhance NTN-related features in R18 [6]. The objectives of the work are as follows:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters, covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN.

- Enhance uplink capacity/cell throughput for FR1-NTN.

- Specify signaling of the intended service area of a broadcast service (e.g., MBS broadcast) via NR NTN.

- Support of regenerative payload.

- Support Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands.

Based on continuous study on NTN from R16 to R19, lots of enhancements have been introduced as the supplement and improvement to NTN. Considering the harmonized radio design for TN and NTN, 6G network necessitates an integrated space-air-ground architecture to overcome geographical limitations and achieve ubiquitous connectivity. This architecture combines satellite networks (for global coverage) and terrestrial base stations (for dense urban areas) to ensure seamless coverage in remote regions, oceans, and disaster zones. Hence, NTN functions in 6G could be categorized into two main parts:

- Part 1

Beam hopping: In R19 NR NTN, beam hopping and extended SSB periodicity were introduced to improve downlink coverage in NR NTN. 6G should natively support these features, enabling an extended periodicity of control channel signals such as SSB.

TDD/FDD: 6G RAT shall support both FDD and TDD to enable operation in frequency bands which may be paired or unpaired bands. The air interface design for 6G TN, including waveform and modulation, frame structure, and duplexing modes such as TDD/FDD, can be effectively adapted/applied to 6G NTN.

Access and Mobility Optimization: Lower latency and finer granularity for access and mobility should be supported, for instance, beam-specific broadcast for mobility and paging. Due to the rapid movement of the satellite gNB, a UE needs to obtain information such as the ephemeris, service termination time, SMTC configuration from system information to perform time and/or frequency pre-compensation, cell reselection and neighboring cell measurement, etc. In particular, if the UE doesn’t acquire the ephemeris of the service cell in a timely manner, pre-compensation cannot be carried out. It is recommended to study the mechanism for UEs to timely receive system information from NTN gNB.

TN/NTN mobility: In 6G, the mobility between TN and NTN should be supported. Given that NTN was introduced in 5G, it is essential to include NTN scenarios into network interworking. We have outlined four cases that should be considered in the following: 5G TN/6G TN inter-RAT mobility, 5G NTN/6G TN inter-RAT mobility, 5G TN/6G NTN inter-RAT mobility, and 5G NTN/6G NTN inter-RAT mobility.

Power saving: Energy saving is always an important topic for operators, not only for the device, but also for the network. 5G has introduced numerous power saving technologies for UE and the network respectively across multiple releases. 6G design should support native energy saving for both the device and the network from Day 1.

Diverse device types: (e)Redcap, mobile VSAT, HPUE for NR and IoT were discussed in 5G NTN. It is assumed that the 6G system should also have the capability to provide connectivity via NTN access with diverse UE types. To achieve a unified design in 6G, we suggest supporting different UE types, such as HD-FDD VSAT, HPUE with various power constraints, bandwidth capabilities, and usage scenarios from Day 1.

- Part 2

AI in NTN: Considering the computing resource characteristics, such as distributed computing power in NTN, native AI design should take into account the characteristics of computing resource in NTN. At the same time, AI use cases for NTN should also be studied in 6G.

TN and NTN cooperation: Resource sharing (e.g., spectrum) and interference coordination between TN and NTN should be considered to achieve better performance in 6G.

Multi-orbit coordination: 6G shall provide coverage area coordination and interference coordination for multi-orbit deployment.

Proposal: RAN WG study focusing on 6G NTN should cover the following aspects from day 1:

| Part 1 | Beam hopping, TDD/FDD, Access and mobility optimization, TN/NTN mobility, Power saving, diverse device types |
| --- | --- |
| Part 2 | AI in NTN, TN and NTN cooperation, Multi-orbit coordination |

Conclusion

In this contribution, we provide our views on 6G RAN WI SID for NTN with the following proposal:

Proposal: RAN WG study focusing on 6G NTN should cover the following aspects from day 1:

| Part 1 | Beam hopping, TDD/FDD, Access and mobility optimization, TN/NTN mobility, Power saving, diverse device types |
| --- | --- |
| Part 2 | AI in NTN, TN and NTN cooperation, Multi-orbit coordination |

Reference

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, 3GPP TSG RAN#107 Incheon, Korea, March 12-14, 2025

- RP-170717, Study on NR to support Non-Terrestrial Networks, 3GPP TSG RAN#75 Dubrovnik, Croatia, March 6 - 9, 2017

- RP-182880, Study on solutions for NR to support non-terrestrial networks (NTN), 3GPP TSG RAN#82 Sorrento, Italy, December 10-13, 2018

- RP-222556, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), 3GPP TSG RAN meeting #97-e e-meeting, September 12 - 16th, 2022

- RP-240779, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, 3GPP TSG RAN#103, Maastricht, The Netherlands, March 18-22, 2024

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, 3GPP TSG RAN#106, Madrid, Spain, December 9-12, 2024