3GPP TSG RAN WG1 #121	R1-2504981

# St Julian’s, Malta, May 19 – 23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0152 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of low-power wake-up signal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-06-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of low-power wake-up signal |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specification of WUS and LPSS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for a low-power wake-up signal |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2, 7.4.4, 7.4.4.1, 7.4.4.1.1, 7.4.1.2, 7.4.4.2, 7.4.5, 7.4.5.1, 7.4.5.1.1, 7.4.5.1.2, 7.4.5.1.3, 7.4.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 38.212, 38.213, 38.214, 38.215 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.1.2	Overview of physical signals

A downlink physical signal corresponds to a set of resource elements used by the physical layer but does not carry information originating from higher layers.

The following downlink physical signals are defined:

-	Demodulation reference signals, DM-RS

-	Phase-tracking reference signals, PT-RS

-	Positioning reference signal, PRS

-	Channel-state information reference signal, CSI-RS

-	Primary synchronization signal, PSS

-	Secondary synchronization signal, SSS

-	Wake-up signal, WUS

-	Low-power synchronization signal, LPSS

### 7.4.4	Wake-up signal

#### 7.4.4.1	Sequence generation

##### 7.4.4.1.1	Generation of rZC,m(n)

The sequence rZC,m(n) is defined by

rZC,m(n)=xq(n+ncs) mod NZC

xqi=e-jπqi(i+1)NZC

n=0,1,…,MZC-1

where

-	NZC is the largest prime number such that NZC<MZC

-	MZC=NscWUSMWUS

The root sequence number q∈1, …, NZC-1 is obtained as entry cmP∈0,1 of the root sequence numbers configured by the higher-layer parameter XXX and the cyclic shift ncs is given by

ncs=cm mod PNZCP
P=NseqNroot

where

-	Nseq is the number of sequences configured by the higher-layer parameter XXX

-	Nrootϵ1,2 is the number of root sequence numbers configured by the higher-layer parameter XXX

The sequence number cm=0 if L=1, otherwise is given by

cm=i=0δ-1f1(i+δm)2δ-1-i

δ=log2L

m=0,1,…,E1δ-1

where

-	L is given by the higher-layer parameter XXX

-	f1i and E1 are given by clause 7.4.2.2 of [4, 38.212]

##### 7.4.4.1.2	Generation of rWUS(n)

The block of complex-valued symbols rWUS(0),…,rWUS(MbitMZC-1) is defined by

rWUSlNscWUS+k=1NscWUSi=0NscWUS-1rWUSlNscWUS+ie-j2πikNscWUS

k=0,1,…,NscWUS-1

l=0,1,…,MbitMWUS-1

NscWUS=11NscRB

where

rWUSmMZC+n=b(m)rZC,m(n)

m=m2

m=0,1,…,Mbit-1

n=0,1,…,MZC-1

The quantity MWUS∈1, 2, 4 is given by the higher-layer parameter LP-WUS_Mvalue_IDLE/INACTIVE or LP-WUS_Mvalue_CONNECTED.

The bit sequence b0, …, b(Mbit-1) and the number of bits Mbit corresponds to g00,g01,…,g0G0-1 and G0, respectively, in clause 7.4.3 of [4, 38.212].

#### 7.4.4.2	Mapping to physical resources

The UE shall assume the block of complex-valued symbols rWUS(0),…,rWUS(MbitMZC-1) is scaled by a factor βWUS and mapped to resource elements k,lp,μ used for WUS transmission in increasing order of first k and then l.

### 7.4.5	Low-power synchronization signal

#### 7.4.5.1	Sequence generation

##### 7.4.5.1.1	Generation of rOOK(n)

The sequence rOOK0,…,rOOK(NOOK-1) is defined by Tables 7.4.5.1.1-1 to 7.4.5.1.1-3 with the quantity MLPSS given by the higher-layer parameter XXX.

Table 7.4.5.1.1-1: The sequence rOOK0⋯rOOK(NOOK-1) for MLPSS=1.

| Configuration | Length 6 | Length 8 |
| --- | --- | --- |
| 0 | [1 0 1 0 1 0] | [1 0 1 0 0 1 0 1] |
| 1 | [0 1 0 1 0 1] | [1 0 1 0 1 0 0 1] |
| 2 | [1 0 0 1 0 1] | [1 0 0 1 0 1 0 1] |
| 3 | [1 0 1 0 0 1] | [0 1 0 1 0 1 0 1] |

Table 7.4.5.1.1-2: The sequence rOOK0⋯rOOK(NOOK-1) for MLPSS=2.

| Configuration | Length 12 | Length 16 |
| --- | --- | --- |
| 0 | [1 0 0 1 1 0 0 1 1 0 0 1] | [1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1] |
| 1 | [0 1 1 0 1 0 0 1 1 0 0 1] | [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1] |
| 2 | [0 1 1 0 0 1 1 0 1 0 0 1] | [1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1] |
| 3 | [0 1 1 0 0 1 0 1 1 0 0 1] | [1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] |

Table 7.4.5.1.1-3: The sequence rOOK0⋯rOOK(NOOK-1) for MLPSS=4.

| Configuration | Length 16 | Length 32 |
| --- | --- | --- |
| 0 | [0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0] | [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] |
| 1 | [0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0] | [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1] |
| 2 | [1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1] | [0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] |
| 3 | [1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] | [0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1] |

##### 7.4.5.1.2	Generation of rZC(n)

If the quantity qϵ1,…,NZC-1 is configured by the higher-layer parameter XXX, the sequence rZC(n) is defined by

rZC(n)=xqn mod NZC

xqi=e-jπqi(i+1)NZC

n=0,1,…,MZC-1

where

-	NZC is the largest prime number such that NZC<MZC

-	MZC=NscWUSMLPSS

##### 7.4.5.1.3	Generation of rLPSS(n)

The block of complex-valued symbols rLPSS(0),…,rLPSS(NOOKMZC-1) is defined by

rLPSSlNscWUS+k=1NscWUSi=0NscWUS-1rLPSSlNscWUS+ie-j2πikNscWUS

k=0,1,…,NscWUS-1

l=0,1,…,NOOKMLPSS-1

NscWUS=11NscRB

where

rLPSSmMZC+n=rOOK(m)rZC(n)

m=0,1,…,NOOK-1

n=0,1,…,MZC-1

#### 7.4.5.2	Mapping to physical resources

The UE shall assume the block of complex-valued symbols rLPSS(0),…,rLPSS(NOOKMZC-1) is scaled by a factor βLPSS and mapped to resource elements k,lp,μ used for LPSS transmission in increasing order of first k and then , then l.