3GPP TSG RAN WG1 #120bis	        		   			R1-2504971

# St Julian’s, Malta, May 17th – 21st, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0708 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of low-power wake-up signal and receiver for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-06-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of LP-WUS/WUR for NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of LP-WUS/WUR for NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of LP-WUS/WUR for NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 10.4C (new), 10.4D (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.212, TS 38.321, TS 38.304, TS 38.331 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged parts are omitted ***

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in [1, TR 21.905].

## *** Unchanged parts are omitted ***

GSCN	Global Synchronization Channel Number

HARQ-ACK	Hybrid Automatic Repeat reQuest Acknowledgement

LPSS	Low power synchronization signal

MBS	Multicast Broadcast Services

MCG	Master Cell Group

## *** Unchanged parts are omitted ***

UCI	Uplink Control Information

UE	User Equipment

UL	Uplink

UL-SCH	Uplink Shared Channel

USS	UE-specific Search Space

UTO-UCI	Unused Transmission Occasion - UCI

WUS	Wake-Up Signal

## *** Unchanged parts are omitted ***

## 10.4C	PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE

A UE configured with DRX mode operation and operating in the RRC_IDLE or RRC_INACTIVE state can be provided for LPSS/WUS reception

-	a number of OOK symbols per OFDM symbol, a first RB, and an overlaid OFDM sequence per OOK symbol for LPSS reception, and an EPRE ratio relative to SS/PBCH blocks [4, TS 38.211],

-	a number of OOK symbols per OFDM symbol, the first RB, and one or more overlaid OFDM sequences per OOK symbol for WUS reception, and an EPRE ratio relative to SS/PBCH blocks [4, TS 38.211], and

A UE determines to receive LPSS/WUS based on procedures defined in [17, TS 38.304].

A UE assumes that an SCS configuration for LPSS/WUS receptions is same as an SCS of the initial DL BWP and an SCS configuration of an SS/PBCH block the UE used to obtain SIB1.

A UE receives an LPSS in consecutive symbols within a slot. The UE can be provided one or two first symbols for respective one or two LPSS reception occasions in the slot by lpss-StartSymbol. The UE determines slots for LPSS reception occasions based on a periodicity and a time offset, relative to a system frame with SFN 0, provided by lpss-periodicityoffset. Within a period of LPSS reception occasions, LPSS reception occasions are in a set of KL consecutive slots that have all symbols indicated as downlink by tdd-UL-DL-ConfigurationCommon, if provided, and start from the first slot provided by the time offset in the period, where K is the number of transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1 and L is the number of LPSS reception occasions in a slot.

LPSS reception occasions are indexed sequentially in time. An LPSS reception at the k-th LPSS reception occasion is quasi co-located with the k-th transmitted SS/PBCH block, with respect to quasi co-location 'typeC' or 'typeD' properties when applicable, where 1≤k≤K.

If a UE is provided wus-LPSS-beamSubset, the UE receives LPSS/WUS based on the quasi co-location properties of transmitted SS/PBCH blocks indicated by wus-LPSS-beamSubset [12, TS 38.331]; otherwise, the UE receives LPSS/WUS based on the quasi co-location properties for transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1.

A WUS occasion includes K⋅M WUS monitoring occasions that are indexed sequentially in time, where

-	K is the number of transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1, M is a number of WUS monitoring occasions associated with each of the K transmitted SS/PBCH blocks provided by MONumperLO, and

-	a WUS monitoring occasion with index (k-1)⋅M+m, where 1≤m≤M and 1≤k≤K, is quasi co-located with the k-th transmitted SS/PBCH block with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable

A UE can be provided, by WUS_available_slot_IDLE/INACTIVE, a bitmap that corresponds to a set of time units that repeats continuously and indicates a subset of time units from the set of time units that is available for the UE to monitor WUS [12, TS 38.331]. A time unit includes one slot or two slots. The UE can be additionally provided, by WUS_available_symbol_IDLE/INACTIVE, an indication of symbols in each time unit from the subset of time units that is available for the UE to monitor WUS. If the UE is not provided WUS_available_slot_IDLE/INACTIVE, the UE assumes that all time units are available for the UE to monitor WUS. If the UE is not provided WUS_available_symbol_IDLE/INACTIVE, the UE assumes that, for a time unit that is available for the UE to monitor WUS, all symbols in the time unit are available for the UE to monitor WUS. The UE assumes that a symbol is not available to monitor WUS when

-	the symbol is indicated as uplink, by tdd-UL-DL-configurationCommon

-	the symbol is indicated for an SS/PBCH block transmission, by ssb-PositionsInBurst in SIB1, and the SS/PBCH block transmission would overlap in frequency with the WUS transmission

-	the symbol is indicated for PDCCH transmissions, by pdcch-ConfigSIB1, and CORESET 0 for the PDCCH transmissions would overlap in frequency with the WUS transmission

A WUS monitoring occasion is over a first number of symbols, provided by WUS_NominalMO_duration_ IDLE/INACTIVE. If a number of available symbols for the UE to monitor WUS in a WUS monitoring occasion is smaller than a second number of symbols, provided by WUS_ActualMO_duration_ IDLE/INACTIVE, the UE does not monitor WUS in the WUS monitoring occasion. The UE monitors WUS in a WUS monitoring occasion over the earliest available WUS_ActualMO_duration_ IDLE/INACTIVE symbols in the WUS monitoring occasion. If a number of available symbols for the UE to monitor WUS in a WUS monitoring occasion includes a symbol for LPSS reception, the UE does not monitor WUS in the WUS monitoring occasion.

A UE assumes that WUS occasions occur with a periodicity equal to the I-DRX cycle in the RRC_IDLE/RRC_INACTIVE state [17, TS 38.304]. The UE determines WUS occasions associated with a paging occasion based on PO-to-LO association. A reference frame of a WUS occasion starts a number of frames prior to the first of a number of paging frames associated with the WUS occasion. Each number of frames is provided by LO-FrameOffsets. The first WUS monitoring occasion of a WUS occasion starts at an offset provided by offset_firstMO_withinLO relative to the start of the reference frame. If multiple values for the number of frames provided by LO-FrameOffsets are larger than or equal to the value of XYZ, the UE monitors WUS starting at a WUS occasion corresponding to the smallest of the multiple values. If all values for the number of frames provided by LO-FrameOffsets are smaller than the value of XYZ, the UE monitors PDCCH according to Type2-PDCCH CSS sets associated with the paging occasion and does not monitor WUS.

A paging occasion associated with a WUS occasion has index iPO=UE_ID mod N⋅NS+i_s mod NPOWO where NPOWO is a number of paging occasions associated with a WUS occasion, N, NS, iSG, and i_s are defined in [17, TS 38.304], and UE_ID is defined in clause 7.1 of [17, TS 38.304]. If a number of NSGPO subgroups per paging occasion, provided by subgroupNumber-PO-WUS, is NSGPO>1, the codepoint for the subgroup index iSG in a PO iPO is iPO*NSGPO+1+iSG, and the codepoint for all subgroups in the PO is (iPO+1)*NSGPO+1-1; otherwise, the codepoint for the PO iPO is iPO.

If, in a WUS monitoring occasion, a UE determines a codepoint associated with the UE [17, TS 38.304], the UE performs PDCCH monitoring according to Type2-PDCCH CSS sets for the paging occasion associated with the WUS monitoring occasion when a time from the end of the WUS reception to the start of the PDCCH monitoring occasion is not smaller than the value of XYZ; otherwise, the UE is not required to perform the PDCCH monitoring. The UE may also perform PDCCH monitoring for Type2A-PDCCH CSS sets for DCI format 2_7, if provided.

## 10.4D	PDCCH monitoring activation by WUS in RRC_CONNECTED

A UE configured with DRX mode operation and operating in the RRC_CONNECTED state can be provided for WUS reception on the primary cell of a cell group

-	a number of OOK symbols per OFDM symbol, a first RB, and overlaid OFDM sequences per OOK symbol for WUS reception [4, TS 38.211], and

-	a number of codepoints provided for the UE by the WUS [6, TS 38.212], by WUS-codepointCONNECTED

A UE assumes that a WUS is quasi co-located with an SS/PBCH block or a CSI-RS with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable.

If a UE is provided ABC, the UE receives WUS based on the quasi co-location information of the TCI states indicated by a most recent DCI format or MAC CE, after a respective application time; otherwise, the UE receives WUS based on the quasi co-location information of the TCI states for a CORESET with controlResourceSetId value that is same as the one indicated by WUS_TCI_states_CONNECTED.

A UE assumes that an SCS configuration for WUS receptions is same as an SCS configuration for the active DL BWP.

A UE does not monitor a WUS during Active Time [11, TS 38.321].

A UE does not monitor WUS during DTX inactive period for the primary cell.

A UE can be provided by WUS-MOCONNECTED-Option1-1 a periodicity, by periodicityMO-Option 1-1, and a time offset, by offsetMO-Option 1-1, relative to the start of a system frame with SFN 0, for the UE to determine WUS monitoring occasions. The UE starts to monitor WUS in a first WUS monitoring occasion that is not earlier than a first slot that is prior to a second slot where the drx-onDurationTimer would start by a time provided by timeOffsetCONNECTEDOption1-1, and monitors WUS for a number of monitoring occasions provided by numMO-Option 1-1. The UE reports a number of slots [18, TS 38.306] where the UE is not required to monitor WUS prior to the slot where the drx-onDurationTimer would start. The UE is not required to monitor WUS within the reported number of slots prior to the slot where the drx-onDurationTimer would start. If the UE determines to monitor PDCCH based on a detected WUS, the UE starts the drx-onDurationTimer [11, TS 38.321].

A UE can be provided by WUS-MOCONNECTED-Option1-2 a periodicity, by periodicityMO-Option 1-2, and a time offset, by offsetMO-Option 1-2, relative to the start of a system frame with SFN 0, for the UE to determine first WUS monitoring occasions from a number of WUS monitoring occasions per periodicity, provided by numMO-perPeriodicity-Option 1-2. The UE reports a number of slots [18, TS 38.306] and expects that a time gap, from a last WUS monitoring occasion from the number of WUS monitoring occasions per periodicity to the slot where the wus-PDCCHMonitoringTimer would start, is no smaller than the reported number of slots. If the UE determines to monitor PDCCH based on a detected WUS, the UE starts wus-PDCCHMonitoringTimer [11, TS 38.321] after a time, provided by timeOffsetCONNECTEDOption1-2, with respect to the start of the first WUS monitoring occasion from the number of WUS monitoring occasions per periodicity.

A UE can be provided, by WUS_available_slot_CONNECTED, a bitmap that corresponds to a set of time units that repeats continuously and indicates a subset of time units from the set of time units that is available for the UE to monitor WUS [12, TS 38.331]. A time unit includes one slot or two slots. The UE can be additionally provided, by WUS_available_symbol_CONNECTED, an indication of symbols in each time unit from the subset of time units that is available for the UE to monitor WUS. If the UE is not provided WUS_available_slot_CONNECTED, the UE assumes that all time units are available for the UE to monitor WUS. If the UE is not provided WUS_available_symbol_CONNECTED, the UE assumes that, for a time unit that is available for the UE to monitor WUS, all symbols in the time unit are available for the UE to monitor WUS. The UE assumes that a symbol is not available to monitor WUS when

-	the symbol is indicated as uplink, by tdd-UL-DL-configurationCommon or tdd-UL-DL-ConfigurationDedicated

-	the symbol is indicated for transmission of SS/PBCH blocks, by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon

A WUS monitoring occasion is over a first number of symbols, provided by WUS_NominalMO_duration_CONNECTED. If a number of available symbols for the UE to monitor WUS in a WUS monitoring occasion is smaller than a second number of symbols, provided by WUS_ActualMO_duration_CONNECTED, the UE does not monitor WUS in the WUS monitoring occasion. The UE monitors WUS in a WUS monitoring occasion over the earliest available WUS_ActualMO_duration_CONNECTED symbols in the WUS monitoring occasion.

If a UE detects a codepoint in a WUS reception, from the number of codepoints, on the primary cell of the cell group, the UE starts monitoring PDCCH on all applicable serving cells of the cell group.

## *** Unchanged parts are omitted ***