3GPP TSG-WG Meeting #121	R1- 2504977

# St. Julian's, Malta, May 19–23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR | 0071 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Rel-19 MIMO evolution for TS38.215 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | May 30, 2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new measurements, coherent joint transmission calibration delay/frequency/phase offset based on RAN1 agreements from RAN1 #116 to RAN1 #120bis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined new measurement coherent joint transmission calibration delay offset, coherent joint transmission calibration frequency offset, coherent joint transmission calibration phase offset under section 5.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Coherent joint transmission calibration measurement is not supported on Release 19 NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.214 CR 0677 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- unchanged text omitted ---

### 5.1.50	Coherent joint transmission calibration delay offset

| Definition | Coherent joint transmission calibration delay offset is defined as the relative receive time difference, Dn,offset, between reference CSI-RS resource set, nref, and selected CSI-RS resource set and out-of-interval indicator, dn. Out-of-interval indicator, dn, indicates corresponding relative receive time difference plus delay spread is not larger than the duration of the cyclic prefix for symbols other than 0 and 7∙2μ and set to ‘1’, otherwise set to ‘0’, Determination of reference CSI-RS resource set, nref, and determination of delay offset and out-of-interval indicator, labelled as ‘CJTC-Dd’, are describe in Clause 5.2.1.4.2 and 5.2.1.4.6 of [6, TS38.214], respectively. [For frequency range 1, the reference point for the coherent joint transmission calibration delay offset shall be the antenna connector of the UE.] |
| --- | --- |
| Applicable for | RRC_CONNECTED |

### 5.1.51	Coherent joint transmission calibration frequency offset

| Definition | Coherent joint transmission calibration frequency offset is defined as the relative frequency offset, FOn, between reference CSI-RS resource set nref, and selected CSI-RS resource set. Determination of reference CSI-RS resource set, nref, and determination of frequency offset, labelled as ‘CJTC-F’, are describe in Clause 5.2.1.4.2 and 5.2.1.4.7 of [6, TS38.214], respectively. [For frequency range 1, the reference point for the coherent joint transmission calibration frequency offset shall be the antenna connector of the UE.] |
| --- | --- |
| Applicable for | RRC_CONNECTED |

### 5.1.52	Coherent joint transmission calibration phase offset

| Definition | Coherent joint transmission calibration phase offset is defined as the relative phase offset, Φn or Φn,s , between reference CSI-RS resource, nref, and selected CSI-RS resource over the entire configured reporting band or over the indicated subband, s, respectively. Determination of reference CSI-RS resource, nref, and determination of phase offset, labelled as ‘CJTC-P’, are describe in Clause 5.2.1.4.2 and 5.2.1.4.8 of [6, TS38.214], respectively. For frequency range 1, coherent joint transmission calibration phase offset shall be measured on the receiver branch(es) associated with the configured port for an SRS resource in the latest SRS transmission occasion before the occasions of the corresponding CSI-RS resources. [For frequency range 1, the reference point for the coherent joint transmission calibration phase offset shall be the antenna connector of the UE.] |
| --- | --- |
| Applicable for | RRC_CONNECTED |

--- unchanged text omitted ---