3GPP TSG RAN Meeting #108 	          	      		            	 RP-251591

Prague, Czech Republic, June 9-13, 2025

Agenda Item:   	15.1.5

Source:   	LG Electronics

Title: 	Discussion on Non-Terrestrial Networks (NTN) for NR Phase 4

Document for:   	Discussion and decision

- Introduction

The following summary [1] were made at the RAN#107 meeting on the topics of further enhancements to NR-NTN in Rel-20.

Conclusion for GNSS resilient operation: - Study Item with checkpoint to determine whether we have normative work and the scope based on results of study item phase. - Study should consider Rel-16 as baseline for discussion. Study should focus on NR NTN and be backward compatible with legacy UEs. Aim to minimize physical layer procedures impact to 5G-Advanced and avoid physical layer channel/signal changes. - Objective to be narrowed down for RAN#108 - Max TU estimate: RAN2 (0.5), RAN1 (1), RAN4 [0.5] - No additional NR NTN enhancements with RAN1 impact can be considered at this point given limited 5GA TUs Additional topic to be included in objective: - Specify necessary enhancements to support E-UTRA TN to NR NTN handover in RRC connected mode - TU estimate RAN2 (0.25 TU), RAN4 (0.5) - Deadline to complete the work (first half of release)

In this contribution, we will provide our view on the details of further enhancements to NR-NTN in Rel-20.

- Discussion

In the last plenary meeting, the support of GNSS resilient operation to achieve and maintain UL time/frequency synchronization under the situation of GNSS jamming/spoofing etc. was discussed as a topic of further enhancement of NR-NTN in Rel-20. We think that the situations in which GNSS resilient operation is applied can be classified into the following three representative cases. Note that further enhancements to NR-NTN in Rel-20 are assumed to target only UEs with GNSS capabilities.

- Case 1: A UE with GNSS capability temporarily loses GNSS availability while in RRC_CONNECTED mode.

- Case 2: A UE with GNSS capability in RRC_IDLE mode that temporarily lost GNSS availability performs initial access.

- Case 3: (Regardless of UE state mode) a UE with GNSS capability loses GNSS availability for a significant period of time.

Considering that enhancements that require too much specification work in a late release of 5G NR-NTN are not desirable, it is preferred to prioritize Case 1 and Case 2 mentioned above, which is also consistent with the current NR-NTN framework where a UE performs pre-compensation based on its own location. For Case 2, it would be necessary for a plenary meeting to provide guidance or for RAN1 WG to conduct analysis on what level of relaxed positioning accuracy is acceptable in terms of performance of UL transmissions related to initial access (e.g. PRACH, Msg3/A PUSCH) with no PHY layer channel/signal changes. Case 3 is similar to the situation where a UE without GNSS capability performs GNSS-free operation, and it should be studied as one of the 6G NTN features that differentiate it from 5G NTN in the 6G Day-1 standardization.

Proposal 1: Specifying GNSS resilient operation as a further enhancement of NR-NTN in Rel-20 should be limited to targeting UEs with GNSS capabilities in the following situations (i.e., Cases 1 and 2).

- A UE with GNSS capability temporarily loses GNSS availability while in RRC_CONNECTED mode.

- A UE with GNSS capability in RRC_IDLE mode that temporarily lost GNSS availability performs initial access.

  - Note: It would be necessary for a plenary meeting to provide guidance or for RAN1 WG to conduct analysis on what level of relaxed positioning accuracy is acceptable with no PHY layer channel/signal changes.

The WI to support E-UTRA TN to NR NTN mobility in Rel-19 only covers the enhancement of inter-RAT RRC_IDLE mode mobility, but operators have argued for the need to extend this to the case of inter-RAT RRC_CONNECTED mode mobility in Rel-20 (due to the still high percentage of the world population connected to E-UTRA, etc.). We support considering E-UTRA TN to NR NTN mobility in RRC_CONNECTED mode as a further enhancement of NR-NTN in Rel-20. Note that after Rel-20 NR-NTN standardization, seamless mobility/handover of E-UTRA TN to NR NTN will be supported regardless of UE state mode.

Proposal 2: Specify support of E-UTRA TN to NR NTN mobility/handover in RRC_CONNECTED mode as a further enhancement to NR-NTN in Rel-20.

- Conclusion

In this contribution, we discussed the topics and their scopes that will be introduced as further enhancements to NR-NTN in Rel-20. The following proposals are given:

Proposal 1: Specifying GNSS resilient operation as a further enhancement of NR-NTN in Rel-20 should be limited to targeting UEs with GNSS capabilities in the following situations (i.e., Cases 1 and 2).

- A UE with GNSS capability temporarily loses GNSS availability while in RRC_CONNECTED mode.

- A UE with GNSS capability in RRC_IDLE mode that temporarily lost GNSS availability performs initial access.

  - Note: It would be necessary for a plenary meeting to provide guidance or for RAN1 WG to conduct analysis on what level of relaxed positioning accuracy is acceptable with no PHY layer channel/signal changes.

Proposal 2: Specify support of E-UTRA TN to NR NTN mobility/handover in RRC_CONNECTED mode as a further enhancement to NR-NTN in Rel-20.

- Reference

[1] RP-250803, “Moderator's summary for RAN2 led REL-20 topics,” RAN2 Chair (IntersDigital).