3GPP TSG RAN#108 Meeting 	RP-251604

Prag, Czech Republik, June 9th -13th, 2025

Title:	New band for NR-NTN in Rel-20 work plan

Source: 	SES, Thales

Type:	discussion

Document for:	discussion

Agenda Item:	15.3 - RAN4 led topics

# Introduction

This document proposes to undertake a RAN4 led study followed by a possible normative activity on the use of a new band for the NR-NTN radio interface as part of the Rel-20 work plan.

# Discussion

The following topic for RAN4 led NR-NTN activity is proposed as part of the Rel-20 work plan .

## Supplementary DL channel in C band

### Justification

As part of 5G system, it has been demonstrated the benefit of being able to extend the service beyond the coverage of the “terrestrial” network component. Hence adding a non-terrestrial network (NTN) component would allow to directly connect smartphones anywhere in the world including in the most remote and low densely populated areas to enable the ubiquitous coverage targeted by 6G networks.

As part of releases 17, 18 and 19, a number of frequency bands allocated to satellite service below 7.125 GHz (i.e. L and S bands) have been defined to support Direct to smartphones or IoT devices connectivity.

In the context of Rel-20, we propose to study the use of the “upper C band” (3800 – 4200 MHz) as supplementary DL channel for Direct to smartphones and IoT devices connectivity.

### Objectives

The following assumptions are taken a baseline for this work:

- GSO and NGSO (e.g. LEO, MEO) based satellite access to be considered

- Targeted UE types: PC3 UE.

The following covers the objectives for the deployment of NR-NTN in downlink in the “upper C band” (3800 – 4200 MHz) :

- Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios.

  - NOTE: in some countries, regulations may have allocated a specific band to both mobile and satellite services, hence an efficient NTN/TN coexistence shall be ensured in adjacent channels.

- Conduct coexistence study. Reuse S band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing.

- Check study results @ RAN#112 and decide to launch normative phase with

  - Specification of RF requirements for UE and SAN

  - Specification of RRM requirements including timing requirements

### Working groups

- RAN4

# Conclusion and Way forward

Proposal 1: RAN to study as part of the Rel-20 6G study item the use of new satellite service allocated bands for NTN and in particular the “upper C band” (3800 – 4200 MHz) for Direct to Devices application.

END