3GPP TSG RAN meeting #108							RP-251617

Prague, Czech Republic, June 9-13, 2025

Title:	Proposals for RAN-led 6G Study Items

Source: 	ST Engineering iDirect

Type:	discussion

Agenda Item:	16.2. New RAN WG SI on 6G

Abstract

This contribution presents four proposals to enhance the radio access technology for 6G. The first proposal is the possibility for an NTN-capable UE to perform time and frequency offset compensation without knowing its position. A second proposal is to enhance the security of the physical layer. A third proposal is to obtain a lower error floor to avoid retransmissions, e.g., via updated forward error-correcting schemes. The last proposal is the addition of a signaling waveform that allows the gNB to estimate the distortion caused by the non-linearity of the VSAT’s PA.

# Introduction

Figure 1 presents the usage scenarios and design principles that are envisioned in the ITU Recommendation M.2160 “Framework and overall objectives of the future development of IMT for 2030 and beyond” that is approved in November 2023 [1].

Figure 1. Usage scenarios and overarching aspects of IMT-2030 [1]

In Rel-17, normative requirements for non-terrestrial networks (NTN) were added in 3GPP specifications. The overall goals for 6G should include further security enhancements, reliability improvements, and a harmonization between terrestrial networks (TN) and NTN.

This document proposes four topics to be considered during the development of the 6G radio access technology (RAT). The proposals relate to the ubiquitous connectivity and hyper reliable communication usage scenarios, as well as the security and resilience design principles.

# GNSS independence

## Justification

In the current 5G specifications, an NTN-capable UE requires a GNSS receiver that is used to determine its position. The position of the satellite can be determined based on broadcasted ephemeris data. Based on the position of the UE and the satellite, the UE pre-compensates timing and frequency offsets (TS 38.300). As such, if the GNSS system is unavailable (e.g., due to jamming), it will not be possible to connect, or stay connected, to the 5G NTN system (TS 36.300).

In [2], the drawbacks of GNSS were discussed, including its narrowband signaling making it susceptibility to spoofing and jamming, as well as the longer start-up times and increased power consumption.

## Requirements

A study item with a potential normative phase is being prepared to enhance GNSS resilience in Rel-20 5G-Advanced [3]. The study should consider Rel-16 as baseline for the discussion and should focus on NR NTN with backward compatibility with legacy UEs. It is expected that this study item will be approved in RAN#108 (Prague).

In case the normative work is not finished in Rel-20 5G-A, this should be further addressed in Rel-21, to have an NTN system that does not depend on position information of the UE or satellite.

The proposed requirements for GNSS independent operation are

- The solution shall not use the position of the UE or satellite

- Both low earth orbit (LEO) and geostationary orbit (GEO) with a single satellite in view are considered

- All types of user equipment (UE) are considered

- NTN operating in frequency range (FR) 1, and in above 10 GHz frequency bands

- If the NTN-capable UE has a GNSS receiver, the existing position-based pre-compensation can be used, i.e., the system is backwards compatible

These objectives align with Thales’ view on 6G, i.e., an NTN-capable UE shall be able to operate with 6G RAT NTN without depending on GNSS for uplink timing/frequency synchronization, i.e., in initial access, idle and connected modes [4].

## Possible solution

As presented in [2], GNSS-independent frequency tracking in connected mode can be performed by introducing a new MAC control element (CE) to communicate frequency offset estimates from the gNodeB towards the UE, similar to the existing Timing Advance MAC CE. These two MAC CEs allow the implementation of more advanced closed loops for time and frequency tracking [5].

For GNSS-independent logon, the existing PRACH format, e.g., format C2, can already afford larger timing and frequency errors [6]. Modifications to the PRACH allow network logon without usage of the satellite or UE position, e.g., using unslotted PRACH with a payload data that contains the transmit timestamp, or by using a low-power density logon waveform that is spread below the PUSCH traffic.

# Physical layer security

## Justification

In the 3GPP specifications up to 5G, security features focused on encryption and authentication, with the 5G system security mechanism being specified in TS 33.501. With the possibility that quantum computers would be able to break existing cryptographic schemes, the focus has been on quantum-resistant cryptographic primitives, which was the conclusion of TS 33.841 and is also advocated in multiple 6G vision documents, including [4][7-9], and reflected in the chair’s summary of the 3GPP workshop on 6G [10].

As outlined in Section 2, a GNSS system can be easily jammed or spoofed because of its narrowband nature. A similar observation can be made for the 5G broadcast and control channels. The 5G broadcast and control channels can be easily jammed or eavesdropped. The downlink control information (DCI) with downlink and uplink scheduling information is broadcasted by the gNB. Even though the DCI frames do not contain any payload, and they cannot be mapped directly to a UE identity, they allow creating traffic maps of network activity for the different terminals in the network.

It is already announced that 5G NR NTN will be the baseline for IRIS2 [11], which targets to provide worldwide access to secure governmental satellite communication services, featuring the integration of military needs [12]. For these usage scenarios, improvements in the physical layer security are necessary, whereas these requirements are welcome for commercial scenarios and private networks.

## Requirements

It is proposed to add a requirement to add channels with increased physical layer security, e.g.,

- Make the 6G broadcast and control channels more resilient against jamming and eavesdropping

- Enable an option to hide the network activity, and if possible, the network traffic, e.g., for military or governmental use cases

## Proposed solution

A possible solution to meet the proposed requirements is to add secure channels in addition to the existing broadcast and downlink control channels.

- A Secure Synchronization Signaling Block (SSSB) that does not use reserved resources in the OFDM grid, but spreads a long synchronization sequence (LSS) and Physical Broadcast Channel below the other channels

- A Secure Physical Downlink Control Channel (SPDCCH) that extends the control information blocks, i.e., DCI and UCI, up to the full bandwidth

Because of the wideband nature and low-power density (down to 20 dB below the shared channel), these secure channels are not easily decodable by an eavesdropper. They also increase the efficiency.

# Improved error floor

## Justification

In 5G, adaptive modulation and coding (AMC) is used. Modulation and coding scheme (MCS) selection is performed at the gNB and can be based on channel measurement reports from the UE, or it can be based on whether the HARQ processes trigger retransmissions. The latter is used in the Open Air Interface implementation [13].

For voice applications, a block error rate (BLER) of 10-2 is sufficient, whereas, the current BLER requirement for ultra reliable communication is 10-5. However, due to the long propagation delay in satellite systems, they typically operate in quasi error-free mode with even lower BLER down to 10-7.

As the MCS selection in satellite communication systems is based on the signal-to-noise ratio (SNR), there is a need to obtain a low BLER in low SNR scenarios.

## Objectives

The error floor should be improved for satellite systems to operate in quasi error-free mode, not using HARQ. This is possible with FEC changes.

We propose that the 6G RAT shall provide a more efficient FEC to operate at a target BLER as low as 10-6 or lower with very low SNR preventing the use of re-transmission at physical layer. This also aligns with Thales’ vision for 6G [4]. The possibility for new FEC coding schemes was also advocated by Panasonic in [14].

## Proposal

Study the introduction of new LDPC codes to obtain low BLER in low-SNR scenarios.

# Distortion detection for VSATs

## Justification

Existing 5G specifications provide maximum power density limits, as well as limitations to the maximum output power for any transmission bandwidth within the channel bandwidth [15]. The UE can signal its available power, i.e., the difference between the estimated TX power and nominal maximum TX power, via Power Headroom Reporting (PHR) [16]. The gNB can use PHR for scheduling and UE power control.

Compared to 5G TN systems, where the power amplifier (PA) is integrated in the 5G UE, satellite terminals such as VSATs typically have an external outdoor unit that is connected via cabling to the modem. This challenges accurate TX power estimations, and hence PHR, at the UE. Nevertheless, TX power is critical in NTN systems to close the link budget for GEO satellites, and the non-linearity distortion introduced by a PA that is driven in saturation mode causes spectral regrowth [17].

To cope with the non-linearity distortion of the terminal’s PA, current satellite systems add sufficient backoff [18] or perform daily out-of-band calibrations [19] to meet regulatory requirements.

Distortion detection by the gNB allows using the PA at maximum power without spectral regrowth. For instance, if the gNB can measure the distortion caused by the UE’s PA up to a certain threshold under the noise floor, uplink power control can be enhanced.

## Proposal

Addition of a signaling waveform that allows estimating the distortion of the terminal at the gNB to improve uplink power control.

# References

[1] Recommendation ITU-R M.2160-0, “Framework and overall objectives of the future development of IMT for 2030 and beyond”, November, 2023

[2] RP-250574 ST Engineering iDirect, “Proposal on improved resilience to GNSS interference”, RAN#107, Incheon, Korea, March 12-14, 2025

[3] RP-250812 RAN chairs and RAN1/2/3/4 vice chairs, “Summary for RAN Release 20 5G-Adv”, RAN#107, Incheon, Korea, March 12-14, 2025

[4] 6GWS-250040 Thales, “Vision and priorities for 6G RAT to support NTN”, 6G Workshop, Incheon, Korea, March 10-11, 2025

[5] P. Delbeke and D. Duyck, “5G-NR GNSS independent time and frequency synchronization in NTN scenarios”, 28th Ka and Broadband Communications Conference, October 2023

[6] RP-250406 Huawei, HiSilicon, “Views on Rel-20 NR NTN scope”, RAN#107, Incheon, Korea, March 12-14, 2025

[7] 6GWS-250235 Philips, “Philips view on 6G (RAN)”, 6G Workshop, Incheon, Korea, March 10-11, 2025

[8] 6GWS-250003 Nokia, “6G Architecting Tomorrow”,  6G Workshop, Incheon, Korea, March 10-11, 2025

[9] 6GWS-250008 ZTE, “6G Motivation and Day1 Functions – RAN Aspects”)”, 6G Workshop, Incheon, Korea, March 10-11, 2025

[10] 6GWS-250243 TSG chairs, “Chair’s summary of the 3GPP workshop on 6G”, 6G Workshop, Incheon, Korea, March 10-11, 2025

[11] RP-250765 ESA, “IRIS2: The first fully-fledged 5G NR NTN multi-orbit satellite communications system”, RAN#107, Incheon, Korea, March 12-14, 2025

[12] European Union Agency for the Space Programme (EUSPA), “Factsheet IRIS2: Infrastructure for resilience, interconnectivity and security by satellite”, March, 2023

[13] OpenAirInterface Software Alliance, “OpenAirInterface – 5G Software Alliance for Democratising Wireless Innovation”, 2024

[14] 6GWS-250034 Panasonic, “Panasonic view on 6G RAN”, 6G Workshop, Incheon, Korea, March 10-11, 2025

[15] 3GPP TS 38.101-5, “User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”, Rel-19, March, 2025

[16] 3GPP TS 38.321, “Medium Access Control (MAC) protocol specification”, Rel-18, March, 2025

[17] A.M. Fominykh et al., “Comparative analysis of different types of modulation in satellite communications systems with class AB amplifiers operating in the saturation mode,” Antennas (Radiotekhnika), 2024

[18] O. Koudelka, “Link budget calculations”, ITU Symposium and Workshop on small satellite regulation and communication systems, Prague, Czech Republic, March 2-4, 2015

[19] B. Kerns, “On the Matter of Interference: An AvL Insight”, SatMagazine, March, 2020

END