3GPP TSG-RAN Meeting #108                                              	RP-251641

Prague, Czech Republic, June 9-13, 2025

Source:	CATT, Thales, SK Telecom, ETRI, Inmarsat, Viasat, Fraunhofer IIS, Fraunhofer HHI, BMW, Continental Automotive, Bosch, Toyota ITC, ESA, CSCN

Title:	Discussion on E-UTRA TN to NR NTN handover enhancements in Rel-20 5GA

Agenda Item:	15.2.5

Document for:	Discussion and Decision

Introduction

In this contribution, we further discuss E-UTRA TN to NR NTN handover for Rel-20 5GA based on the conclusions endorsed in RAN#107 as follows [1]:

Additional topic to be included in objective. - Specify necessary enhancements to support E-UTRA TN to NR NTN handover in RRC connected mode - TU estimate RAN2 (0.25 TU), RAN4 (0.5) - Deadline to complete the work (first half of release)

Also, the detailed justifications and objectives for the potential Rel-20 WI for this topic are also provided.

Discussion and proposals

Based on the endorsed conclusion, the scope of a potential WI for E-UTRA TN to NR NTN handover should include the RAN2 part and the RAN4 part which are discussed respectively.

  - Work item management

The enhancement on E-UTRA TN to NR NTN handover impacts the LTE specifications. Therefore, similar to Rel-19, a LTE work item, which is separate from the potential NR NTN work item, should be approved in Rel-20 5GA for this E-UTRA TN to NR NTN handover enhancement. This is proposed as follows:

Proposal 1: Approve an LTE work item for E-UTRA TN to NR NTN handover enhancement (separate from potential NR NTN work item) in Rel-20.

Potential RAN2 Scope

Potential RAN2 scope corresponds to "Specify necessary enhancements to support E-UTRA TN to NR NTN handover in RRC connected mode". In the current TS 36.331 [1], measurement configurations for inter-RAT mobility to NR are specified separately from those for E-UTRAN (i.e. measObjectNR/ReportConfigInterRAT). However, current E-UTRA specifications may not be able to support the “E-UTRA TN to NR NTN” handover, due to the lack of (at least) following necessary Satellite assistance information in inter-RAT measurement configuration, making the UE unable to perform measurements towards NR NTN from E-UTRA TN:

- DL polarization information

This information is needed to avoid 3dB loss for measurements due to antenna polarization mismatch. It corresponds to NR NTN parameter ntn-PolarizationDL in TS 38.331 [3], and agreed to be introduced for IDLE mode mobility

- Satellite Ephemeris information

This information is needed for the UE to do frequency compensation when performing DL measurement (which is the case for both IoT NTN and NR NTN). Note that this is Irrelevant to location-based CHO specified for NR NTN in TS 38.331 [3].

Therefore, the potential RAN2 objective should be enabling E-UTRAN to provide satellite assistance information (e.g. polarization information, Ephemeris data, etc.) into NR-specific measurement configurations

Proposal 2: Agree the following RAN2 objective for the E-UTRA TN to NR NTN handover enhancement work item:

- E-UTRAN is enabled to provide satellite assistance information (e.g. polarization information, Ephemeris data, etc.) into NR-specific measurement configurations.

Note that the RAN2 efforts needed by above proposed objective are simply the addition of the NR NTN satellite information into the NR specific measurement configuration, and can be simple enough to fit the endorsed TU budget, i.e. 0.25 TU, half a release.

Potential RAN4 Scope

From RAN4 perspective, the RRM requirements for mobility are only defined for the following scenarios in current TS 38.133:

- NTN to NTN:

NR NTN to NR NTN mobility for IDLE / INACTIVE/CONNECTED mode

- NTN to TN:

NR NTN to NR TN mobility only for IDLE / INACTIVE mode

NR NTN to E-UTRA TN mobility only for IDLE / INACTIVE mode

- TN to NTN：

NR TN to NR NTN mobility only for IDLE / INACTIVE mode

According to the above background, it is noted that only the scenario of "E-UTRA TN to NR NTN" is missing, including requirements for IDLE / INACTIVE/CONNECTED mode. Actually, the requirements for CONNECTED mode in all existing "TN to NTN" and "NTN to TN" scenarios already introduced are also missing.

In addition, from the perspective of operator demand and deployment, considering that the same operator will also have a demand for Space-Terrestrial Integrated Networks and it will deploy both TN and NTN networks at the same time, it is necessary to define the missing mobility requirements. Further considering the time that Rel-20 will allocate for 5G-A as per the endorsed conclusion in [1], RAN4 will prioritize to define the RRM requirements for "E-UTRA TN to NR NTN" mobility in CONNECTED mode.

Observation 1: In the mobility scenario of "E-UTRA TN to NR NTN", the CONNECTED mode RRM requirements are missing in the current specifications.

Based on the above discussion, we propose to support E-UTRA TN to NR NTN handover in Rel-20 5G-A with following proposals.

Proposal 3: As a potential RAN4 objective, RAN4 need to define RRM requirements for mobility scenario of "E-UTRA TN to NR NTN" in CONNECTED mode.

Proposal 4: Agree the following RAN4 objective for the E-UTRA TN to NR NTN handover enhancement work item:

- Specify necessary RRM requirements for mobility scenario of "E-UTRA TN to NR NTN" in CONNECTED mode.

Proposed "Justification" and "bullet lists of objectives"

For the E-UTRA TN to NR NTN handover enhancement WID, the TP to "Justification" part and "bullet lists of objectives" (which are based on above proposals and also include scenarios/assumptions within the scope) are provided in the Annex.  It is proposed that RAN plenary endorses the TP in the Annex.

Proposal 5: RAN plenary endorses the "Justification" and "bullet lists of objectives" in the Annex for the E-UTRA TN to NR NTN handover enhancement WID.

Conclusion

This contribution discusses the potential Rel-20 work item for E-UTRA TN to NR NTN handover enhancement. Observations and proposals are listed as follows:

On work Item management

Proposal 1: Approve an LTE work item for E-UTRA TN to NR NTN handover enhancement (separate from potential NR NTN work item) in Rel-20.

On RAN2 scope

Proposal 2: Agree the following RAN2 objective for the E-UTRA TN to NR NTN handover enhancement work item:

- E-UTRAN is enabled to provide satellite assistance information (e.g. polarization information, Ephemeris data, etc.) into NR-specific measurement configurations.

On RAN4 scope

Observation 1: In the mobility scenario of "E-UTRA TN to NR NTN", the CONNECTED mode RRM requirements are missing in the current specifications.

Proposal 3: As a potential RAN4 objective, RAN4 need to define RRM requirements for mobility scenario of "E-UTRA TN to NR NTN" in CONNECTED mode.

Proposal 4: Agree the following RAN4 objective for the E-UTRA TN to NR NTN handover enhancement work item:

- Specify necessary RRM requirements for mobility scenario of "E-UTRA TN to NR NTN" in CONNECTED mode.

On Justification and bullet lists of objectives of WID

Proposal 5: RAN plenary endorses the "Justification" and "bullet lists of objectives" in the Annex for the E-UTRA TN to NR NTN handover enhancement WID.

Reference

- RP-250812	Summary for RAN Release 20 5G-Adv

- 3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

- Annex: TP for E-UTRA TN to NR NTN handover enhancement WID

# 3 Justification In Release 19, a work item was carried out to define solutions enabling idle mode mobility between LTE and NR-NTN. However, handover between E-UTRA TN and NR NTN in connected mode has not been well supported. This results in potential impacts on service continuity when UE with on-going services is switched from E-UTRA to NR NTN. As part of Release 20, a new work item is valuable to define further enhancements for NG-RAN based Non-Terrestrial Networks in order to support E-UTRA TN to NR NTN handover in RRC connected mode. # 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The work item aims at specifying enhancements for handover from E-UTRA TN to NR NTN with the following assumptions: - GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit). - NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO). - Earth fixed tracking area. Quasi-Earth fixed & Earth moving cells for NGSO - UEs with GNSS (Global Navigation Satellite Systems) capability - NTN-FR1 band is considered in priority The objectives of the work item are the following: - Specify connected mode mobility based on handover from E-UTRA TN to NR NTN, where E-UTRA TN provides necessary satellite information for NR NTN neighbour cell measurements (e.g. polarization information, ephemeris data, etc.) in NR specific measurement configurations. [RAN2] - Note: For this objective, the solutions shall be reusing the existing signalling for inter-RAT measurements to NR in TS 36.331 and the related satellite assistance information specified in TS 38.331 for NR NTN. - Note: Support of Multiple SMTCs is not within the scope. - Specify necessary RRM requirements for mobility scenario of "E-UTRA TN to NR NTN" in CONNECTED mode. [RAN4] ### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. - ### 4.3 RAN time budget request (not applicable to RAN5 WIs/SIs) NOTE: For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings. The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI. One time unit (TU) corresponds to ~ 2 hours in the meeting. If no TU is needed, then leave the field empty otherwise enter a number >0 in the field. For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID. If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table. See the time budget request in the attached Excel table.