3GPP TSG RAN Meeting #108																   RP-251654

Prague, Czechia, June 9th-13th, 2025

Agenda Item:		15.1.2

Source:				InterDigital, Inc.

Title:						Scope for MIMO for Rel-20 5G-Advanced

Document for:		Discussion and Decision

# Background

For Rel-20, MIMO standardization activities are divided into two separate tracks, namely, NR MIMO 5G-A and MIMO 6G. Therefore, due to limited TUs, the objectives for the former need to be carefully scoped. As such, in our assessment of topics for Rel-20 NR MIMO 5G-A, the following aspects have been considered,

- Addressing market-urgent issues of 5G with limited specifications impacts,

- A balanced focus between UL and DL enhancements,

- Having a manageable workload (i.e., study/evaluate/specify) given the limited allocated TU(s).

In this contribution, based on the discussion and guidance from the last RANP meeting, we provide our views regarding the general scope of NR MIMO in Rel-20 5G-Advanced.

# Discussion

Table 1 shows a short list of potential topics for Rel-20 NR MIMO 5G-A after the first round of discussion in the last RAN meeting [1]. The topics are structured in two main categories of enhancements for UL capacity/coverage and DL CSI acquisition.

Table 1 - Summary of potential scope for Rel-20 NR MIMO 5G-A

| Objective | Topic |
| --- | --- |
| (1) UL capacity and coverage | (a) UL frequency-selective precoder cycling (open-loop), mainly 2Tx, using legacy NR UL codebooks |
|  | (b) Additional precoders for 2Tx full-coherent UL codebook with 8PSK/16PSK alphabet |
|  | (c) SRS capacity/coverage: cross-slot repetition (U and S slots), hopping for partial sounding |
|  | (d) Asymmetric DL sTRP/UL mTRP: PDCCH order PRACH beam indication, same Tx beam constraint across SRS occasions (per given SRS resource(s)) for beam sweeping, targeting FR2 |
| (2) DL CSI acquisition | (a) Early SRS/CSI/CSI-RS triggering for UEs transitioning from IDLE/INACTIVE to CONNECTED mode via PRACH MSG2/4, as well as activation (Scell, out of dormancy) in CONNECTED mode |
|  | (b) CSI-RS overhead reduction in frequency-domain (to 1/4 and 1/8 RE/RB/port) and possibly port-domain (targeting large # ports e.g. 128) |
|  | (c) Sub-band scalar IMR measurement report (targeting reciprocity-based operation) with larger dynamic range than the current 4-bit CQI |

While the discussion resulted in further narrowing down the list of proposed topics, given the very limited time, additional down-selection is needed. In the following, we share our views on the remaining items.

(1) UL capacity/coverage

1-a) UL frequency-selective precoder cycling (open-loop): In general, frequency-selective precoding is an effective transmission strategy to achieve higher transmission robustness. An open-loop frequency-selective precoding based on precoder cycling mechanism can be supported with a very small standardization effort. In an open-loop mechanism, a set of precoders are cycled per subband to enhance the performance by introducing additional diversity in the system. As demonstrated in Figure 1 in the Appendix, the performance of uplink transmission using subband-based precoder cycling is superior to the case of wideband (labeled as 1 SB) precoder cycling.

1-b) Additional precoders for 2Tx full-coherent UL codebook: In general, there may be some benefits in enhancing the resolution of the uplink codebook. However, we believe that potential gain that can be achieved by adding a few more 8PSK/16PSK-based precoders is very limited. We prefer to study uplink codebook resolution enhancements with more depth as part of 6G studies.

1-c) SRS capacity/coverage: The proposed specification support for cross-slot repetition (U and S slots) and hopping for partial sounding seem to be relatively small enhancements. In our view, the expected benefits, especially for the hopping-related enhancement, seem marginal. However, given its limited scope, we would be open to further discussion.

1-d) Asymmetric DL sTRP/UL mTRP: PDCCH order PRACH beam indication and SRS beam sweeping: Since, there is no DL reference signal in an Asym-TRP scenario, especial solutions should be considered to support a proper beam selection for UE Tx and gNB Rx beams. We believe this is an essential enhancement that addresses some of the issues left from Rel-19 Asym-TRP discussion. In our view, an SRS beam sweeping with constant beam across SRS resources, and with different beams across SRS resource sets, enable determination of the best beam pair for UL Tx and gNB Rx beams. Once the pair is determined, gNB can indicate the best UL beam by a PDCCH order for a PRACH transmission.

To manage the workload, only a subset of the proposed topics for uplink should be considered for normative work in MIMO in Rel-20 5G-A. In our view, objective 1-d represents the most essential enhancement as it completes the functionality of Rel-19 Asymmetric DL sTRP/UL mTRP.

Observation 1: Objective 1-d represents the most essential enhancement as it completes the functionality of Asymmetric DL sTRP/UL mTRP in Rel-19.

Proposal 1: For UL MIMO in Rel-20 5G-A, support Objective 1-d that addresses PDCCH order PRACH beam indication and SRS beam sweeping for an asymmetric DL sTRP/UL mTRP deployment.

- DL CSI acquisition

2-a) Early SRS/CSI-RS/TRS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode: An early DL CSI acquisition is an effective way to enhance system operational efficiency. As such, instead of waiting for completion of transitioning to the connected mode and then receive an RRC configuration for CSI measurement and report, by support of this enhancement, a UE can enable a gNB to acquire DL CSI in an accelerated manner. We believe there are some benefits and expect a reasonable workload for support of this enhancement.

2-b) CSI-RS overhead reduction in frequency-domain and possibly port-domain: Considering recent Rel-19 CSI enhancements for support of up to 128 CSI ports, we believe this is an important objective that could further improve the spectrum efficiency of NR 5G system. Since we are in the last phase of 5G standardization, it is not reasonable to revisit CSI-RS design, however for now, frequency-domain or port-domain down-sampling could be considered for use in NR 5G systems. We consider this objective a highly important enhancement with clear benefit for the operation of NR 5G. Further, the required specification work is not extensive.

2-c) Sub-band IMR measurement report (targeting reciprocity-based operation): We understand the motivation of the enhancement; however, we believe that it requires more investigation and a mere increase in dynamic range may not be sufficient. Since we are in the very last stage of NR 5G standardization, our preference is to visit this enhancement as part of 6G studies.

Observation 2: To support objective 2-c, more investigation and evaluation is needed, hence our preference is to visit this enhancement as part of 6G studies.

Proposal 2: For DL MIMO in Rel-20 5G-A, support objectives 2-a and 2-b.

In conclusion, given the limited allocated TUs for MIMO 5G-A in Rel-20, only market-urgent issues of 5G with limited specifications impact with manageable corresponding workloads should be prioritized.

Observations 3: Given the limited allocated TUs for MIMO 5G-A in Rel-20, only market-urgent issues of 5G with limited specifications impact should be considered.

Proposal 3: For Rel-20 MIMO, support the following objectives,

- Objective 1d: Asymmetric DL sTRP/UL mTRP: PDCCH order PRACH beam indication, same Tx beam constraint across SRS occasions (per given SRS resource(s)) for beam sweeping, targeting FR2

- Objective 2a: Early SRS/CSI/CSI-RS triggering for UEs transitioning from IDLE/INACTIVE to CONNECTED mode via PRACH MSG2/4, as well as activation (Scell, out of dormancy) in CONNECTED mode

- Objective 2b: CSI-RS overhead reduction in frequency-domain (to 1/4 and 1/8 RE/RB/port) and possibly port-domain (targeting large # ports e.g. 128)

# Conclusion

In this contribution, we discussed our views with respect to the scope of Rel-20 NR MIMO 5G-A. Based on the presented discussion, following observations and proposals are made,

Observation 1: Objective 1-d represents the most essential enhancement as it completes the functionality of Asymmetric DL sTRP/UL mTRP in Rel-19.

Proposal 1: For UL MIMO in Rel-20 5G-A, support Objective 1-d that addresses PDCCH order PRACH beam indication and SRS beam sweeping for an asymmetric DL sTRP/UL mTRP deployment.

Observation 2: To support objective 2-c, more investigation and evaluation is needed, hence our preference is to visit this enhancement as part of 6G studies.

Proposal 2: For DL MIMO in Rel-20 5G-A, support objectives 2-a and 2-b.

Observations 3: Given the limited allocated TUs for MIMO 5G-A in Rel-20, only market-urgent issues of 5G with limited specifications impact should be considered.

Proposal 3: For Rel-20 MIMO, support the following objectives,

- Objective 1d: Asymmetric DL sTRP/UL mTRP: PDCCH order PRACH beam indication, same Tx beam constraint across SRS occasions (per given SRS resource(s)) for beam sweeping, targeting FR2

- Objective 2a: Early SRS/CSI/CSI-RS triggering for UEs transitioning from IDLE/INACTIVE to CONNECTED mode via PRACH MSG2/4, as well as activation (Scell, out of dormancy) in CONNECTED mode

- Objective 2b: CSI-RS overhead reduction in frequency-domain (to 1/4 and 1/8 RE/RB/port) and possibly port-domain (targeting large # ports e.g. 128)

# References

RP-250802, RAN1 Chair (Samsung), “Moderator’s summary for RAN1-led Rel-20 topics”, RANP #107, Incheon, Korea, March 2025

# Appendix

Figure 1 – Uplink open-loop frequency-selective precoding (precoder cycling)

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| OFDM Type | CP-OFDM |
| Bandwidth | 3 MHz |
| PRG Size | 16 RBs/ 8 RBs/ 4RBs/ 2RBs |
| Channel | TDL-A |
| Delay Spread | 300 ns |
| Subcarrier Spacing | 15 kHz |
| Number of Layers | 1 |
| Modulation | QPSK |
| Code Rate | 0.5 |
| UE antenna configuration | (2, 1, 2, 1, 1, 2, 2), (dH, dV) = (0.5, 0.5) λ |
| gNB antenna configuration | (2, 1, 2, 1, 1, 2, 2), (dH, dV) = (0.5, 0.5) λ |