3GPP TSG RAN Meeting #108							RP-251677

Prague, Czech Republic, June 9-13, 2025

## Status Report to TSG

Agenda item:			9.3.1.5

| WI / SI Name | Enhancements of network energy savings for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | Netw_Energy_NR_enh |  |  |  |  |
| Unique ID | 1020095 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-250343 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 90% | Performance Part: 10% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Ajit Nimbalker |
|  | Company | Ericsson |
|  | Email | Ajit.Nimbalker@ericsson.com |
| Rapporteur (RAN2) | Name | Peng Cheng |
|  | Company | Apple |
|  | Email | pcheng24@apple.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#120bis, the following agreements were made.

Agreement

For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, adopt Option 3 to determine SCS for Nslotsubframe,μ for determining T

- Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

For the case when the center frequency locations of always-on SSB and on-demand SSB are different, at least the following is supported for QCL between AO-SSB and OD-SSB

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

Agreement

For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst),

- Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values}

  - N2= [8]

  - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE.

  - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

Agreement

Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B.

- Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’.

  - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB.

    - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification.

  - (Working assumption) T=T_min

  - RAN4 to confirm the above two working assumptions

  - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases

  - The case where center frequency of AO-SSB and OD-SSB are different

  - Case 1

- Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst)

FFS: Additional restrictions

Agreement

LS to RAN4 on time instance B for on-demand SSB SCell operation is agreed. Final LS in R1-2503108.

Agreement

For a cell supporting on-demand SSB SCell operation,

- Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) can be absent for Case #2

  - If absent, the center frequency of on-demand SSB is the same as that of always-on SSB.

- od-ssb-PositionsInBurst, if provided, has the same bitmap structure as ssb-PositionsInBurst provided in ServingCellConfigCommon, that is od-ssb-PositionsInBurst is choice of 4-bit shortBitmap, 8-bit mediumBitmap, or 64-bit longBitmap.

- od-ssb-PositionsInBurst can be absent for Case #2

  - If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE.

- Scenario #3B and Case #1

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- Scenario #3B and Case #2

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Agreement

For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell)

- UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

- A CSI report configuration can be associated with

  - both of AO-SSB and OD-SSB

    - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met

    - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst

    - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”.

  - OD-SSB only

    - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies

Conclusion

There is no RAN1 consensus to support SSB with on-demand SIB1 not on sync raster.

Agreement

The following agreement is updated as follows:

The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to at least the SSB associated with the transmitted PRACH for UL-WUS if this is indicated via UL WUS configuration by a 1-bit indication

- If the 1-bit indication is included in UL WUS config and indicate as TRUE, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS.

- If the 1-bit indication is not included in UL WUS config or the 1-bit indication is FALSE, the UE assumes the same as legacy, i.e., in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

- Further study possible down selection from the following options to indicate additional information related to SSBs associated with PDCCH monitoring occasions

  - Alt1: The information is provided in UL-WUS config

  - Alt2: The information is provided in RAR in response to UL-WUS transmission

  - Alt3: No additional information is provided

Agreement

Indication of K_SSB value in the UL-WUS configuration should be 5 bits for FR1 and 4 bits for FR2.

- Note: there is no change to current PBCH structure.

- UE does not expect K_SSB to be larger than 23 for FR1 and larger than 11 for FR2

Agreement

The value of the time offset between the reference time point and the starting time for the time window of OD-SIB1 can be either 0 or positive value.

Agreement

After transmitting a UL-WUS, UE is not required to monitor Type0-PDCCH occasion (for OD-SIB1) before UE successfully received its RAR in response to the UL WUS.

Agreement

If a UE has SIB1 request configuration of a cell and before transmitting UL WUS,

- If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=12 for FR2, select the following:

  - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

Agreement

In the UL WUS configuration, how to support multiple NES-CellId(s) is up to RAN2 discussion.

Agreement

From RAN1 perspective, for agreed UL WUS parameters, regarding their mandatory or optional presence and applicability to TDD and/or FDD, adopt the followings:

- PhysCellId and ARFCN-ValueNR are mandatory

- frequencyBandList and absoluteFrequencyPointA are present in IE FrequencyInfoUL for FDD (as in the legacy specification)

- K_SSB is mandatory

- searchSpaceZero and controlResourceSetZero are mandatory

- ra-PreambleStartIndex, od-sib1-duration, offsetToTimeWindow are mandatory

Agreement

The parameter ra-SearchSpace in the UL-WUS configuration is of type SearchSpace.

Agreement

Replace the current parameter name offsetToTimeWindow with od-sib1-windowStartOffset (to make the naming more comprehensive).

Agreement

Include CarrierBandwidth and locationAndBandwidth in the UL-WUS configuration.

- Previous RAN1 #120 conclusion is reverted

Agreement

The following previous RAN1 #118-bis agreement is updated as:

- Msg1-FrequencyStart is with respect to the first RB of the UE carrier InitialUplinkBWP determined by locationAndBandwidth offsetToCarrier and absoluteFrequencyPointA (same as legacy)

Agreement

RAN1 to discuss the value range for od-sib1-duration. For example,

- Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point.

- Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity).

- Option 3: The duration of the time window can be a multiple of the on-demand SIB1 repetition periodicity (if supported)

- Other options are not precluded.

Agreement

RAN1 clarifies frequencyBandList in UL WUS configuration refers to the IE within FrequencyInfoUL-SIB.

Agreement

For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI,

- Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources

  - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon

Agreement

For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI

- Use one bit from the Bits 5-8 within the Short Message field (from upper layers)

- Send LS to RAN2 to confirm the use of this bit.

Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs

Agreement

For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI

- the validity duration is configured via higher layer signalling.

Agreement

For DCI-based adaptation for additional PRACH resources, PRACH mask that identifies the subset of the additional PRACH resources is applicable at unit of

- PRACH association period

- This PRACH mask applies to every [configured] K SSB RO association pattern period(s)

Agreement

For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell,

- The DCI is scrambled a new RNTI,

  - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both

Agreement

For DCI 2_9-based SSB burst periodicity adaptation for an SCell

- the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter

- the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell

Agreement

For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell.

- Xmax=2

Agreement

Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported

- CB-PreamblesPerSSB

Agreement

LS on DCI-based PRACH adaptation endorsed with the ACTION part modified compared to draft LS in R1-2503085 as follows:

- ACTION: RAN1 respectfully asks RAN2 to confirm whether the use of above bit is feasible.

Final LS in R1-2503086.

Agreement

For adaptation of PRACH in time-domain, for a connected mode UE,

- One of the reserved bits of PDCCH order (DCI 1_0 with C-RNTI) is used for the new DCI field that indicates the availability of additional PRACH resources.

Working Assumption

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213].

- FFS: how to determine the first [possible] SSB burst

Agreement

For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is given by:

- Option 1-2:  Semi-static signalling of a PRACH mask index and a value of K (number of association pattern periods)

  - For K: one from up to four candidate values {2,4,8, [1 or 16]}

| Mask index | Indication of association periods (AP) for subset of additional PRACH resources within every K association pattern periods (APP) |
| --- | --- |
| 0 | 1st half of the APs in K APPs |
| 1 | 1st quarter of the APs in K APPs |
| 2 | 1st eighth of the APs in K APPs |
| 3 | 1st sixteenth of the APs in K APPs |

Agreement

- For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is the start of first frame of the current SI modification period where UE receives the DCI

- The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is

  - (Option 4) Multiple of SI modification period ({[1],2,[3],4,8,[],[],..})

In RAN1#121, the following agreements were made.

Agreement

For a cell supporting on-demand SSB SCell operation,

- The following cases are NOT supported in Rel-19.

  - AO-SSB is CD-SSB and not on a synchronization raster.

  - OD-SSB is CD-SSB and on sync-raster.

  - When AO-SSB is not CD-SSB, the center frequency of AO-SSB is different from the center frequency of OD-SSB.

- When the center frequency locations of always-on SSB and on-demand SSB are different, OD-SSB is not CD-SSB.

Agreement

For a cell supporting on-demand SSB SCell operation, the following is supported for QCL between different OD-SSB configurations

- SS/PBCH blocks with the same SSB indexes according to OD-SSB configuration and indication are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index according to OD-SSB configuration and indication with the same QCL parameters according to existing specifications.

- FFS: Case where there is different SSB indexes before and after OD-SSB transmission adaptation

Conclusion

For a cell supporting on-demand SSB SCell operation,

- LTM based on OD-SSB is NOT supported in Rel-19.

- L1 measurement based on OD-SSB for a serving cell in Scenario #2 is NOT supported in Rel-19.

  - Note: As per previous RAN1 agreement, Scenario #2 is when SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321).

- L1 measurement based on OD-SSB for a non-serving/neighbor cell is NOT supported in Rel-19.

Agreement

For a cell supporting on-demand SSB SCell operation,

- If a PDCCH candidate and a currently being transmitted OD-SSB are overlapped in at least one RE, the UE is not required to monitor the PDCCH candidate.

Agreement

For a UE supporting on-demand SSB SCell operation,

- When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, Multicast MCCH-RNTI or PDSCHs with SPS.

  - At least the UE shall assume that the PRBs containing currently being transmitted OD-SSB resources are not available for PDSCH in the OFDM symbols where OD-SSB is currently being transmitted on the cell.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #1 (i.e., No always-on SSB on the cell),

- For RO validation before SSB-to-RO mapping,

  - Alt 3: RO validation rule is determined based on all configured OD-SSB configuration(s) in RRC.

- For SSB-to-RO mapping,

  - SSB-to-RO mapping rule is determined based on all the SSB indices configured by all configured value(s) of od-ssb-PositionsInBurst in RRC.

Conclusion

For a cell supporting on-demand SSB SCell operation,

- Whether or not to assume quasi co-location between different SSB indices is up to UE implementation

Agreement

For a cell supporting on-demand SSB SCell operation,

- For OD-SSB, the SS/PBCH block in TS 38.213 Clause 11.1 can be replaced by the SS/PBCH block that is transmitted according to an on-demand SSB transmission operation in the cell.

Note: Whether/How to capture the above is up to the editor.

Agreement

For a cell supporting on-demand SSB SCell operation, it is supported to use OD-SSB as

- PL-RS,

- Reference signal for QCL/TCI configuration, or

- Candidate beam detection for beam failure recovery.

Note: Whether/How to capture the above is up to the editor.

Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations are supported.

- For OD-SSB transmission activation (OD-Tact) and OD-SSB transmission adaptation (OD-TA),

  - Case A1: RRC-based OD-Tact without N (i.e., od-ssb-nrofBurst) configured + MAC CE-based OD-TA;

    - Subject to UE capability

  - Case B1: MAC CE-based OD-Tact without N configured + MAC CE-based OD-TA;

  - Case B2: MAC CE-based OD-Tact with N configured + MAC CE-based OD-TA.

- For OD-SSB transmission deactivation (OD-TD),

  - Case X1: RRC-based OD-Tact without N configured + MAC CE-based OD-TD;

    - Subject to UE capability

  - Case Y1: MAC CE-based OD-Tact or OD-TA without N configured + MAC CE-based OD-TD;

  - Case Y2: MAC CE-based OD-Tact or OD-TA with N configured + implicit OD-TD;

  - Case Y3: MAC CE-based OD-Tact or OD-TA with N configured + MAC CE-based OD-TD.

- Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst.

- Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication.

Agreement

For a cell supporting on-demand SSB SCell operation, for configuring od-ssb-nrofBurst of which the value range is {N2 integer values},

- N2= 8

  - Note: This is updated from the previous RAN1 agreement.

- The following values for od-ssb-nrofBurst are taken as the starting point and to be confirmed in RAN1#122

  - For FR1, the value range of od-ssb-nrofBurst is {5, 10, 15, 20, 25, 30, 40, 50}.

  - For FR2, the value range of od-ssb-nrofBurst is {25, 30, 40, 50, 75, 100, 150, 200}.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

- For RO validation before SSB-to-RO mapping,

  - Alt 3: RO validation rule is determined based on all configured AO-SSB and OD-SSB configuration(s) in RRC.

- For SSB-to-RO mapping,

  - SSB-to-RO mapping rule is determined based on all the SSB indices configured by all configured value(s) of od-ssb-PositionsInBurst and SSB indices configured for AO-SSB in RRC.

- NW to ensure that the SSB to RO mapping is the same between different UEs under the same serving cell.

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - For Case #1 (i.e., No always-on SSB on the cell),

    - A CSI report configuration is associated with OD-SSB only,

      - UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

    - For the case where AO-SSB and OD-SSB have the same center frequency,

      - A CSI report configuration is associated with both of AO-SSB and OD-SSB,

      - UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used is up to UE implementation as long as RAN4 requirements are met.

    - For the case where AO-SSB and OD-SSB have the different center frequencies,

      - Option 2: A CSI report configuration is associated with both of AO-SSB and OD-SSB.

        - UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used is up to UE implementation as long as RAN4 requirements are met.

  - SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet.

- Note: The CSI report in the above agreement applies only to what is supported up to Rel-18.

Agreement

For a cell supporting on-demand SSB SCell operation and for L1 measurement based on on-demand SSB,

- For a CSI report configuration configured with reportConfigType set to periodic,

  - Do not support periodic CSI reporting based on OD-SSB, for Case #1 and Case #2

Conclusion

On whether/how to indicate additional information related to SSBs associated with PDCCH monitoring occasions, no additional information is provided

Conclusion

For the purpose of finalizing Rel-19 NES, RAN1 assumes the following UE behavior for OD-SIB1.

- The SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms.

Agreement

On the value range for od-sib1-duration, adopt the following:

- {20, 40, 80, 160, 320} ms

Agreement

For UE to obtain the UL point A for TDD system, configure offsetToPointA in the UL WUS configuration for TDD system.

- No RAN1 specification impact is expected

Conclusion

There is no consensus on the support of OD-SIB1 for NR-U in Rel-19.

Agreement

RAN1 to adopt TP below.

--------------- TP of TS 38.213 in Clause 23 -----------------------

If the UE identifies a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the UE can be indicated by higher layers to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero. If the UE is provided XYZ, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block. The UE starts monitoring monitors PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration.

--------------- end of TP -------------------------------------------------------

Reason of change: Considering the RAR processing time and OD-SIB1 window starting time may fall into the middle of SSB burst and/or SIB1 transmission time corresponding to one SSB burst, when to start the PDCCH monitoring in the OD-SIB1 window is up to UE implementation.

Agreement

The parameters ‘absoluteFrequencyPointA’, ‘offsetToCarrier’ and ‘locationAndBandwidth’ are mandatorily present in the UL-WUS configuration for both FDD and TDD system.

Agreement

The frequencyBandList is mandatorily present in WUS configuration for TDD system, which refers to the IE within FrequencyInfoDL-SIB.

Agreement

The reference point to determine the starting slot of on-demand SIB1 window is the start of the slot containing the starting symbol of the RAR window.

Agreement

RAN1 adopts TP below for clause 23 of TS 38.213.

23 UE procedure to request SIB1 reception -----------------------omitted text----------------------- [If the SS/PBCH block on the second cell indicates kSSB>23 for FR1 or kSSB>11 for FR2,] the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI prior to a reception of a PDSCH carrying a RAPID associated with a corresponding PRACH transmission scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI.

Agreement

For DCI 2_9-based SSB burst periodicity adaptation for an SCell for the case when cell DTX/DRX is not configured, reuse existing search space configuration parameter for DCI 2_9-based monitoring and existing DCI 2_9 size configuration parameter and update in specification that these are also applicable to SSB burst periodicity adaptation (when configured)

Agreement

Value d (in the WA from RAN1#120bis) is the number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1 of TS 38.213.

Agreement

Update the agreement from RAN1#120bis as shown below (i.e. updates in red).

Agreement (from RAN1#120bis)

For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI,

- Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources

  - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon

Note: Whether the additional PRACH configuration can be from RRC other than SIB1 is up to RAN2.

Agreement

The fourth candidate value for K (for PRACH subset mask) is 16. K=1 is default value (if parameter is not configured).

Agreement

Supported values for validity duration configured by higher layers are

- {2,4,8,16} x SI modification period.

Agreement

Value range for PRACH-Config Index parameter for additional RACH configuration is same as legacy, i.e. INTEGER (0...255).

- Note: Final decision on the value range is up to RAN2

Conclusion

Using DCI 1_0 with P-RNTI to explicitly deactivate the additional PRACH resources is NOT supported.

Conclusion

There is no consensus to support adaptation of RACH in time domain for 2-step RA in Rel-19.

Agreement

‘PRACH resource indicator’ field is present in DCI 1_0 with C-RNTI for PDCCH order when the configuration of the additional RACH resources is provided in SIB1(i.e. addl-RACH-Config-Adaptation).

- Above applies for PCell

Agreement

When the SSB burst periodicity is switched from periodicity value P1 to periodicity value P2 based on DCI format 2_9 indication,

- Alt 1: SFN offset (relative to SFN0) and half-frame index are configured per additional SSB periodicity value.

  - the first SSB burst according to the periodicity value P2 is determined as the first SSB burst according to the SSB burst periodicity value P2 and associated SFN offset and half-frame index that is no earlier than slot m+d.

SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity.

Agreement

Both CBRA and CFRA based on additional PRACH resources is supported for PDCCH order via DCI 1_0 with C-RNTI.

- For CFRA,

  - The indicated SSB index and PRACH mask index are applied to both legacy PRACH resources and additional PRACH resources.

    - Note: The PRACH mask applies to either additional resources and/or legacy resources, depending on which one satisfies the conditions for the mask to be applicable.

Agreement

Adopt the below TP for Clause 8.1 of TS 38.213, as per Editor CR available in R1-2503167.

*** Unchanged text omitted ***

For valid PRACH occasions associated with addl-RACH-Config-Adaptation [in RACH-ConfigCommon], the UE can be additionally provided a PRACH mask index, by prach-SubsetMask-Index-Adaptation that, if provided, indicates one or more association periods per K_mask association pattern periods according to Table 8.1-0, where K_mask is provided by KforAPPForPRACHsubsetMask.

Table 8.1-0: Mapping of mask index to association periods per Kmask association pattern periods

| Mask Index | Association periods (APs) perKmask association pattern periods (APPs) |
| --- | --- |
| 0 | First half of APs in Kmask APPs |
| 1 | First quarter of APs in Kmask APPs |
| 2 | First eighth of APs in Kmask APPs |
| 3 | First sixteenth of APs in Kmask APPs |

Valid PRACH occasions associated with addl-RACH-Config-Adaptation, and additionally associated with in association periods indicated by prach-SubsetMask-Index-Adaptation, if provided, are activated indicated as available for PRACH transmission based on an indication in a DCI format 1_0 with CRC scrambled by a P-RNTI [or a C-RNTI] [5, TS 38.212]. For activation indication by DCI format 1_0 with CRC scrambled by the P-RNTI, the PRACH occasions are available for a duration provided by validity-DurationForAddlRACHAdaptation, starting from the first frame of the SI modification period [12, TS 38.331] that includes a PDCCH monitoring occasion where the UE receives a PDCCH providing the DCI format 1_0 with CRC scrambled by the P-RNTI.

*** Unchanged text omitted ***

Agreement

Additional PRACH availability indication can be carried by a DCI 1_0 with P-RNTI with Short Messages Indicator set to 00, 01,10,11.

- Note: Above is already reflected in the endorsed editor CR 38.212

#### 2.1.2	Remaining Open issues

None

## 2.2	RAN2

#### 2.2.1	Agreements

In RAN2#129bis, the following agreements were made.

Agreements on stage-3 details

Define features, not devices. That is, definition of OD-SIB1, OD-SSB, paging adaptation etc in 38.300 and in other specifications refer to 38.300:

- Avoid NES, Cell A, UL-WUS

- “UE supporting OD-SIB1” where/if needed, and to replace with the UE capability name

- UL-WUS is replaced with SIB1 request in e.g. field descriptions

- ul-WUS-Config renamed as od-sib1-Config

Current RRC only has this for idle inactive in 5.2.2.5, add what is needed for connected mode. SIBxx is not defined as essential SI and that EN is deleted.

Keep FFS “FFS how to capture the UE first should acquire a valid SIB1 (e.g. via SIB1 request) for camping on an OD-SIB1 NES cell.” And “FFS how does UE check is SIB1 is already provided.” for now but change “whether to capture”.

Keep the FFS for normal uplink for now.

“acquire the requested SIB1 message as defined in FFS, immediately;” changed to “acquire the requested SIB1 message as defined in FFS, immediately;”

Add in 331 a branch where UE receives notification from MAC that number to max RACH attempts were reached and UE barrs the cell according to 304.

Following agreement on “barring behavior clean-up”, capture the UE barring behavior of OD-SIB1 in both TS 38.331 and TS 38.304. Check offline whether we can rely on existing legacy UE barring specification in 38.304 (e.g. the case of RACH failure).

(modified) On the unbarring behavior of OD-SIB1 UE, capture it as normative text in TS 38.304. Way-forward Proposal (on offline check of Proposal 1): For UE barring the OD-SIB1 cell in case of RACH failure, capture in TS 38.304, e.g.

- If the cell is to be treated as if the cell status is "barred" due to the OD-SIB1 request number reaching preambleTransMax;

2> UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

Capture the following understanding in meeting notes: RA on an SpCell can include a cell on which the UE is requesting OD-SIB1. No specification changes needed.

Reuse legacy SSB selection procedure in MAC spec for RA initiated for SIB1 request, i.e. the UE selects any SSB if no SSB is above the configured RSRP threshold. This can be revisited if RAN1 agrees otherwise.

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

Agreements on OD-SIB1

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

Agreements on common channel adaptation

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

- Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

- For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

- Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

In RAN2#130, the following agreements were made.

Agreements on stage-3 details

- Proposal (include ARFCN inside the list with the PCI to have a NES-CellId list associated to a OD-SIB1 config) is not proceeded.

- Keep RRC CR as it is for the short message and UE behaviour.

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter for Rel-19 UEs that support adaptive paging.

- The maximum PO per PF, i.e., maxPO-perPF-r19 is 8.

- The maximum offset value is extended to 32 radio frames.

- The value range for the new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to accommodate SCS=480kHz and N=T/32.

- A new pei-ConfigBWP IE is not introduced for Rel-19 UEs that support adaptive paging.

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter for Rel-19 UEs that support adaptive paging.

- The maximum number of PEI occasion per paging frame is extended to 8.

- The maximum offset value is extended to 32 radio frames.

- Rel-19 excluded cell list can be configured as an empty list. No new UE behavior is introduced (i.e. the UE supporting OD-SIB1 ignores legacy excluded cell list only if NES excluded cell list is present).

- Same as the other 2 agreed cases of OD-SIB1 acquisition failure, capture the failure case of OD-SIB1 window expiry in TS 38.304. RAN2 confirm no other cases need to be considered and specified in TS 38.304.

- The capability for paging adaption to be included in UE-RadioPagingInfo is a separate capability from other NES features and no need to define a common capability for all NES features.

- Use a “container” using OCTET STRING with content generated by UE for paging adaption capability.

- The paging adaption capability is per UE.

- Introduce new capability for R19 PEI in NES and follow the existing PEI capability definition to have this new capability per band.

- Rely on PHY layer specs to capture availability conditions related to usage of additional PRACH resources. It can be revisited if needed after checking PHY running specifications.

- A note in TS 38.321 is added to clarify that UE selects only amongst preambles configured for the next valid RO(s) for the selected SSB, when CB-PreamblesPerSSB is configured for additional ROs. Wording of the note can be refined during the running CR review discussion.

- The following two proposals were discussed and not pursued:

- (P9 R2-2503712) If UE has on-going RACH procedure upon reception of DCI with RACH adaptation indication, the UE stops the on-going RACH procedure, and re-initiates the RACH procedure with updated RACH resources.

- (P8 R2-2503945) RAN2 to discuss whether the RA procedure can be terminated earlier if the selected RA resource is deactivated.

Agreements on OD-SSB

- (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

- The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional field is added to indicate the applicable OD-SSB configuration index. TP in R2-2504280 will be baseline for the running CR. Detailed wordings can be further discussed.

- When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated:

• OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped.

- For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the OD-SSB Activation/Deactivation MAC CE.

- In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation:

• The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation.

• The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

Spec impact can be further discussed in running CR preparation.

- When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

- RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

- RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.

- When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

- RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

Agreements on OD-SIB1

- Not to include access related information of NES into WUS configuration.

- Normative text for 2nd check, e.g. “if SIB1 is not broadcast”, is removed. RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before triggering OD-SIB1 request procedure of that cell.

- TP1 in R2-2503636 is agreed with the removal of “until receiving SIBxx from this cell”.

- Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

- If there is no common searchspace configuration on current active BWP, the network will ensure the RRC_CONNECTED UE can acquire the updated UL WUS configuration, i.e., the dedicatedSystemInformationDelivery IE in RRCReconfiguration is also applied to new SIB containing UL WUS configuration.

- Include the following parameters from the RAN1 parameter list in SIBxx: frequencyBandList, Prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, zeroCorrelationZoneConfig, preambleReceivedTargetPower, powerRampingStep, ra-ResponseWindow, searchSpaceZero, controlResourceSetZero, ra-SearchSpace, n-TimingAdvanceOffset, ssb-PeriodicityServingCell, od-sib1-WindowDuration, od-sib1-windowStartOffset.

Agreements on common channel adaptation

- Regarding RA type for PRACH adaptation, support only 4-step RA in Rel-19 unless RAN1 decides supporting 2-step RACH this meeting.

- addl-RACH-Config-Adaptation is placed under RACH-ConfigCommon

- Introduce addl-RACH-Config-Adaptation under BWP-UplinkCommon  RACH-ConfigCommon in SIB1. addl-RACH-Config-Adaptation can also be configured under BWP-UplinkCommon  AdditionalRACHConfig  RACH-ConfigCommon in SIB1.

- Agree that the following information is included in additional RO configuration (see the agreed table in the discussion of R2-2504579)

- If adaptation for additional PRACH resources is supported in active UL BWP, network can configure that paging search space in active DL BWP if network want to activate additional RO to the corresponding UEs.

- RRC connected UE monitors the DCI-based adaptation indication at least once every once every SI modification period.

- RAN2 will not do anything to handle the UEs that missed the short message.

- No need to extend the RA-RNTI space for additional PRACH occasions, i.e. no RAN2 impact for RA-RNTI calculation. RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs. RAN2 understand there is no MAC impact.

- RAN2 de-prioritizes the discussion of NES (including all NES scopes) and SBFD co-existence.

- BI is commonly applied to both legacy RO and additional RO.

- For SSB adaptation, R2 concludes the support of L3 measurement configuration based on a single MO. No RAN2 spec impact is foreseen.

- RACH adaptation is not applied to the following cases (for non-serving cell case): 1) PDCCH order for additional RACH, 2) L3 HO command, 3) LTM CSC MAC-CE, 4) PDCCH order for LTM early UL sync.

- RACH adaptation is not applied to BFR.

- RAN2 confirms the newly introduced 1-bit indication in DCI 1_0 for C-RNTI, i.e., PDCCH order, applies to both CBRA and CFRA.

- RAN2 will select one of the following options. (CE here means MSG1 repetition).

Option-1b: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon.

Option-1c: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, but rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon. (i.e., if [feature combination including CE / CE-only feature] is included in featureCombinationPreamblesList-r17 as one of the associated feature(-combination):s, the additional RACH is not supposed to be included in this RACH-ConfigCommon for this featureCombinationPreamblesList-r17).

#### 2.2.2	Remaining Open issues

- Stage-3 specification details on on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA.

- Stage-3 specification details on on-demand SIB1 for UEs in idle/inactive mode.

- Stage-3 specification details on adaptation of common signal/channel transmissions.

## 2.3	RAN3

#### 2.3.1	Agreements

In RAN3#127bis, the following agreements were made.

Step4 is not needed.

In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that:

- One Choice “Start with UL WUS Condiguration”

- One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

Introduce a new indication for NES paging adaptation in UE Paging Capability over F1.

| R3-251566 | (BL CR to 38.473) Introduction of Network Energy Saving Enhancement (Huawei, Ericsson, CMCC, Samsung, ZTE, Nokia, Deutsche Telekom, Lenovo, NEC, Jio) | CR1531r3, TS 38.473 v18.5.0, Rel-19, Cat. B Endorsed as BL CR |
| --- | --- | --- |
| R3-251567 | Introduction of Network Energy Saving Enhancement (Ericsson, Huawei, CMCC, Samsung, Qualcomm, Nokia, ZTE, Lenovo, Deutsche Telekom, NEC) | CR1436r4, TS 38.423 v18.5.0, Rel-19, Cat. B - Change to one Author Rev in R3-252307 Endorsed unseen as BL CR |
| R3-251848 | Xn impact of On-demand SIB1 for UEs in idle/inactive mode (Ericsson, Deutsche Telekom, China Unicom, Jio Platforms) | Other Rev in R3-252385 - Add CMCC, ZTE, CATT as co-source Rev in R3-252467 Agreed unseen |
| R3-251641 | (TP to BLCR for TS 38.300) Introduction of OD-SIB1 (CATT, Ericsson, Huawei, Samsung, China Telecomm, Rakuten, ZTE, CMCC, Lenovo) | Other Rev in R3-252450 - it informs the gNB which has requested the broadcast over the Xn interface. Rev in R3-252468 Agreed unseen |
| R3-251725 | Introduction of Network Energy Saving Enhancement (Samsung, Huawei, NEC, CATT, ZTE, Ericsson) | CR0161r, TS 38.470 v18.4.0, Rel-19, Cat. B Rev in R3-252451 - The F1 setup and gNB-DU Configuration Update functions allow to provide the UL WUS configuration information for on-demand SIB1 at the gNB-DU. - Align the title with other BL CRs Rev in R3-252469 Endorsed unseen as BL CR |
| R3-251752 | (TP to BLCR for TS 38.473 and TS 38.423) Discussion on on-demand SIB1 for UEs in idle or inactive mode (Huawei) | Other Rev in R3-252386 - Add CMCC, ZTE as co-soource - Add [FFS] on the “Provision” and “Stop provision” IEs Rev in R3-252466 Agreed unseen |
| R3-251894 | (TP to BL CR for 38.473) Discussion on UE capability for paging adaptation (ZTE Corporation, CATT, Lenovo) | Other Rev in R3-252361 Agreed |

In RAN3#128, the following agreements were made.

One “Provision Request message includes one “OD-SIB1 config R19” referring to the TS 38.331 definition, it is a RRC Container in octet string (presence M)  +  one NES Cell ID (presence M ) +  one Cell-A ID (presence O )

Cell A gNB-CU encoding the SIBxx.

The NES gNB-CU sends the indication to NES gNB-DU. The NES gNB-DU MAY go to OD-SIB 1 operation up to gNB-DU decision.

| R3-253101 | (BL CR to 38.300) Introduction of Network Energy Saving Enhancement (CATT, Ericsson, Huawei, Samsung, China Telecomm, Rakuten, ZTE, CMCC, Lenovo, Deutsche Telekom, Nokia) | draftCR Endorsed as BL CR |
| --- | --- | --- |
| R3-253102 | Introduction of Network Energy Saving Enhancement (Ericsson, Huawei, CMCC, Samsung, Qualcomm, Nokia, ZTE, Lenovo, Deutsche Telekom, NEC) | CR1436r7, TS 38.423 v18.5.0, Rel-19, Cat. B Endorsed as Bl CR |
| R3-253103 | (BL CR to 38.470) Introduction of Network Energy Saving Enhancement (Samsung, Huawei, NEC, CATT, ZTE, Ericsson) | CR0161r3, TS 38.470 v18.4.0, Rel-19, Cat. B Endorsed as BL CR |
| R3-253104 | (BL CR to 38.473) Introduction of Network Energy Saving Enhancement (Huawei, Ericsson, CMCC, Samsung, ZTE, Nokia, Deutsche Telekom, Lenovo, NEC, Jio) | CR1531r5, TS 38.473 v18.5.0, Rel-19, Cat. B Endorsed as BL CR |
| R3-253539 | Xn impact of On-demand SIB1 for UEs in idle/inactive mode (Ericsson, Deutsche Telekom, China Unicom, Jio Platforms) | Other Rev in R3-253856 - Delete the blank line in 9.1.3.t - Keep the newly added IE in 9.1.3.z and 9.1.3.t as FFS - Add SS, CATT, ZTE as co-source Rev in R3-253957 Agreed unseen |
| R3-253243 | Introduction of Network Energy Saving Enhancement (ZTE Corporation, Samsung, Lenovo) | CR0049r, TS 38.420 v18.1.0, Rel-19, Cat. B Rev in R3-253917 Endorsed as BL CR |
| R3-253259 | (TP to BLCR for TS 38.473 and TS 38.423) Discussion on on-demand SIB1 for UEs in idle or inactive mode (Huawei) | Other Rev in R3-253857 Agreed |

#### 2.3.2	Remaining Open issues

- Details of specifying procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA.

- Details of specifying support for on-demand SIB1 for UEs in idle/inactive mode.

- Details of specifying adaptation of common signal/channel transmissions.

## 2.4	RAN4

#### 2.4.1	Agreements

Core Part

In RAN4#114bis, the following agreements were made.

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW

Sub-issue 1: Lower bound for measurement in FMW

Agreement

Introduce UE capability on whether a lower bound is needed, i.e.,

  - The lower bound is not needed (default value), or,

  - A lower bound is needed and based on OD-SSB periodicity of [10] ms

    - Further confirm whether [10] is feasible, and only one value

Sub-issue 2: CSSF

Agreement

- When NW configures single OD-SSB carrier with multiple legacy SSB carriers, CSSF =1 for OD-SSB layer

  - Extend the FMW window time by one OD-SSB periodicity

  - Longer measurement delay of other frequency layers with AO-SSBs are expected

Sub-issue 5: Searcher assumption

Agreement

- RAN4 shall not change searcher assumption in this work item.

Issue 1-2-1: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

Agreement

    - The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation indication within the time window as follow.

      - max(5*measCycleSCell,  5*DRX cycles) for FR1, or

      - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2.

    - Otherwise, the same approach in Case 1 can be reused.

    - Note: The agreement is applied when OD-SSB and AO-SSB are in the same frequency and with same spatial relation.

    - Note 2: T2 discussion is a separate issue.

Issue 1-2-2: Requirement for RAN1 Alt Time-C1

Agreement:

- RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario.

  - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification.

  - FFS for other procedures.

Issue 1-2-3: Requirement for RAN1 Alt Time-C2

Agreement

- Option 1: Define the requirement based on OD-SSB periodicity.

- Option 2: Not to define requirement.

Issue 1-2-4: Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies)

Agreement

RAN4 to first discuss the RAN4 impact based on RAN2 LS R2-2503015.

- Option 1: RAN4 to define requirement for different frequencies

- Option 2: RAN4 not to define requirements for different frequencies

Issue 1-4-1: Additional Processing time for OD-SSB reception – scenario

Issue 1-4-2: Additional Processing time for OD-SSB reception - value

Agreement

UE applies the additional processing timing only in deactivated SCell measurement.

- Note: The additional UE processing timing is not applied to SCell activation procedure.

The additional processing timing is T_processing, the value of T_processing:

- Introduce different values with UE capability: [2 or 3]ms and 5ms.

Issue 1-4-4: L1 measurement after SCell activation when DRX is configured

Agreement

For Scenario 2 and Scenario 2a, when DRX is configured, following the legacy requirements based on DRX with the consideration of OD-SSB is not deactivated.

Issue 1-4-7: OD-SSB based Neighbor cell measurement in FR2

Agreement

Neighbor cell measurements for FR2 should follow legacy requirements and no change in Rx beam sweeping is required.

Issue 1-4-9: Interruption

Agreement

- New interruption is allowed for scenario 2 due to OD-SSB activation/deactivation.

- New interruption is also allowed for scenario 2 at the end of the FMW if UE does not receive SCell activation command.

- The interruption can be applied in both Case 1 and Case 2.

- The detail condition, duration and position of the interruption is FFS.

- The interruption is allowed for scenario 2a due to OD-SSB activation/deactivation.

- No new interruption requirement is defined.

- The interruption requirement follows the same principle of legacy SCell activation.

Issue 2-1-2: Fast SCell Activation

Agreement

RAN4 to only update the known cell condition considering OD-SSB for Rel-17 fast SCell activation.

- The requirement is based on A-TRS of Rel-17 fast SCell activation requirement.

- The details can be discussed in the CR directly.

- It doesn’t impact the WI’s completion.

Issue 2-1-3: Deactivated PUCCH SCell activation

Agreement

- RAN4 to define OD-SSB based deactivated PUCCH SCell activation requirement.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

Issue 2-1-4: Rel-18 SCell Activation with the L3 reporting during activation

Agreement

- FFS RAN4 to define OD-SSB based Rel-18 SCell activation with the L3 reporting requirement.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

Issue 1-1-1: Scenario clarification based on RAN1 progress

Agreement:

- Further clarification on the scope of RRM impact for SSB adaption:

  - Existing SSB burst periodicity (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

  - SSB adaptation in SCell for NCD-SSB.

  - Time domain positions of SSBs within a burst is same before and after SSB adaptation

- Note: Above could be further updated based on further RAN1 progress if any.

Issue 1-2-1: L1 requirements at transition

Agreement

  - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline.

  - No impacts on section 9.5.3 of L1 reporting requirements due to SSB adaptation, unless identified by other WGs.

Issue 1-3-1: L3 requirements for SSB adaptation

Agreement:

- RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation.

    - For neighbour cell L3 measurement, legacy requirements apply.

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements

Agreement:

- Normal SCell activation

- Direct SCell activation

- PUCCH SCell activation

- FFS Rel-18 SCell activation with L3 reporting

- FFS Multiple SCells activation

Issue 2-1-2: RRM requirements impacts for OD-SIB1 – timing

Agreement

- Clarify in transmit timing requirements to include UL WUS transmission (PRACH) for OD-SIB1:

  - reference cell is the NES cell

  - NTA for UL-WUS is defined as 0

  - NTA offset is the [n-TimingAdvanceOffset] from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133

Note: The IE can be updated based on other WG’s progress.

In RAN4#115, the following agreements were made.

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW

Sub-issue 1: Lower bound for measurement in FMW

Agreement:

Introduce UE capability on whether a lower bound is needed, i.e.,

- No signalling is needed if UE doesn’t need the lower bound (default value), or,

- The lower bound for measurement/cell identification period in FMW is derived based on 10ms periodicity in case 1 and case 2

Note: RAN4 requests RAN2 to implement the UE’s capability based on the above agreement.

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure

Sub-issue 1: The start point of T2

Agreement:

- The time point of OD-SSB deactivation.

Sub-issue 2: The minimum interval(T2) between two FMWs

Agreement:

- For FR1, 5*max (measCycleSCell, DRX cycles)

- For FR2, 4s for power class 1/5 and 3s for power class 2/3/4

Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2

Agreement:

- The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2.

Issue 1-2-1: Requirement for RAN1 Alt Time-C1

Agreement

Update RAN4 agreement to apply EMP also to L1 measurement in the active SCell after SCell activation.

Issue 1-2-2: Requirement for RAN1 Alt Time-C2

Agreement

- Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity.

- Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if

  - The periodicity of AO-SSB and OD-SSB are same

- It doesn’t impact the WI’s completion.

Issue 1-2-3: Requirement for scenario 3 (OD-SSB and AO-SSB are within different frequencies)

Sub issue 1: RAN2 LS for scenario 3

Agreement:

- RAN4 to confirm RAN2’s WS to introduce a new servingCellMO.

Sub issue 2: Requirement for scenario 3

Agreement:

RAN4 will further discuss whether and how to define the requirements for scenario 3, and meanwhile the discussion will not impact the WI completion.

Issue 1-2-5: Scheduling restriction for L1 measurement after SCell activation

Agreement

- In Case 2, no scheduling restriction and measurement restriction is expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement.

- The scheduling restriction shall be applied to both AO-SSB and OD-SSB if the requirement is defined based on both of them.

Issue 1-3-1: The known cell condition in Case 2

Agreement

- The current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with the following combination.

  - When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB.

- FFS whether apply to Alt Time-C2, if the requirement is defined.

- FFS whether apply to scenario 3, if the requirement is defined.

- FFS whether additional AGC time should be included in the known SCell activation delay requirement for scenario 3, if defined.

Issue 1-3-2: The known cell condition – period of known condition

Agreement

- Reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4  in FR2.

Issue 1-4-1: OD-SSB based multiple SCells activation

Agreement

- RAN4 to define the requirements in the case of multiple OD-SSB SCells.

  - It doesn’t impact the WI’s completion.

  - The scope can be limited as follows:

    - In deactivated SCell stage, the time periods from OD-SSB activation to deactivation among multiple OD-SSBs’ frequency layers are non-overlapping.

    - [For multiple Scells activation, multiple Scells are purely OD-SSB’s Scells.

        - OB-SSB is assumed to be activated on all the multiple Scells to be activated before Scell activation command.]

Issue 1-4-3: Additional Processing time for OD-SSB activation

Agreement

- RAN4 should introduce UE capability with values of 2 ms and 5 ms.

Issue 1-4-4: Additional Processing time for OD-SSB parameter update

Agreement

- Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

  - The additional processing time is 5ms or 2ms based on UE capability reporting.

  - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated].

- Option 2: When NW updates the OD-SSB parameters for an active Scell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE.

  - The additional processing time is 5ms

  - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated].

  - Note 2: No capability is needed

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline

Agreement

- RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS.

- Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation.

  - Option 1: No UE additional processing time for OD-SSB deactivation

  - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

Issue 1-4-7: L1 measurement in scenario 3B

Agreement

  - When MAC-CE indicates OD-SSB updating in scenario 3B,

    - UE performs the L1-RSRP measurement based on updated OD-SSB periodicity from Tfirst_SSB.

    - When DRX is configured, follow legacy principle to define requirements

  - Tfirst_SSB is the uncertainty time from the timepoint when MAC-CE for updating the transmission parameter is received till the first SSB that UE is able to process based on timeline defined in RAN1 and the addition processing time defined in RAN4.

Issue 1-4-8: Neighbor cell measurement in the carrier of OD-SSB serving cell before OD-SSB activation in Case 1

Agreement

- UE to perform neighbour cell measurement based on legacy requirement.

Issue 1-1-1: Processing time for SSB adaption

Agreement

RAN4 to define the additional delay requirement in TfirstSSB for L1 transition period measurement for SSB adaptation.

- FR1: 2ms FR2: 3ms

Issue 1-2-2: L1 requirements after transition  - DRX

Agreement:

- For L1 requirements impacts due to SSB adaptation, when DRX is configured, follow legacy principle to define requirements, which means the SSB adaption only impacts TSSB in the measurement period

Issue 1-2-3: L1 requirements - whether to release the measurement results from resources before transition

Agreement:

- No spec. impact is identified for releasing the measurement results.

- It’s up to UE implementation.

Issue 1-3-1: L3 requirements for SSB adaptation – transition for serving cell

Agreement

RAN4 to define L3 requirements at transition for SSB adaptation same as legacy L3 transition period requirement.

- The cell identification and measurement period requirements apply based on the longer period before or after the transition.

Issue 1-3-2: L3 requirements for SSB adaptation – after transition for serving cell

Agreement

- Subsequent to transition, serving cell identification and measurement period requirements are based on the additional SMTC configuration according to SSB adaptation mapped with SSB periodicity.

Issue 1-4-1: SCell activation requirement for SSB adaptation – SSB adaptation command related

Agreement:

For the conditions for SCell activation with SSB adaptation, SSB adaptation command comes no later than SCell activation command.

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption

Agreement:

- For core requirements: Clarify that the paging interruption could be longer when the target Cell is NES Cell

- Define test case with longer TSI-NR with reasonable value in performance part.

Performance Part

Issue 1-1: Whether to define demodulation requirements for PDSCH/PDCCH/PBCH for Rel-19 NES?

Agreement:

-	Do not introduce new PDSCH/PDSCH/PBCH requirements for Rel-19 NES.

Issue 1-2: Whether to introduce new CSI report requirements for Rel-19 NES?

Agreement:

-	Do not introduce new CSI report requirements for Rel-19 NES.

Issue 2-1: Whether to introduce new PRACH demodulation requirement for Rel-19 NES?

Agreement:

-	Do not introduce new BS demodulation requirements for Rel-19 NES.

#### 2.4.2	Remaining Open issues

- Details of specifying procedures and core requirement(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA.

- Details of specifying core requirement(s) of adaptation of common signal/channel transmissions.

- Details of specifying core requirement(s) for on-demand SIB1 for UEs in idle/inactive mode.

- Performance requirements and test cases for the network energy saving techniques.

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

#### 4.1	List of RAN WG1 contributions

#### 4.1.1	RAN1#120bis

R1-2501692	LS on paging enhancement in R19 NES	RAN2, xiaomi

R1-2501693	Reply LS on SSB adaptation	RAN2, Apple

R1-2501713	Discussion of on-demand SSB Scell operation	FUTUREWEI

R1-2501714	Discussion of on-demand SIB1 for idle/inactive mode UEs	FUTUREWEI

R1-2501715	Discussion of the adaptation of common signal/channel transmissions	FUTUREWEI

R1-2501736	On-demand SSB Scell operation for NES	TCL

R1-2501764	Discussion on on-demond SSB for NES	ZTE Corporation, Sanechips

R1-2501765	Discussion on on-demand SIB1 for NES	ZTE Corporation, Sanechips

R1-2501766	Discussion on common signal channel for NES	ZTE Corporation, Sanechips

R1-2501770	Discussion on NES features	ZTE Corporation, Sanechips

R1-2501772	On-demand SSB SCell Operation	Nokia, Nokia Shanghai Bell

R1-2501773	On-demand SIB1 for Idle/Inactive mode UEs	Nokia, Nokia Shanghai Bell

R1-2501774	Adaptation of common signal/channel transmissions	Nokia, Nokia Shanghai Bell

R1-2501810	Discussions on on-demand SSB Scell operation	vivo

R1-2501811	Discussions on on-demand SIB1 for idle/inactive mode UEs	vivo

R1-2501812	Discussions on adaptation of common signal/channel transmissions	vivo

R1-2501832	Discussions on UE features for enhancements of network energy savings for NR	vivo

R1-2501872	Discussion on on-demand SSB SCell operation	Spreadtrum, UNISOC

R1-2501873	Discussion on on-demand SIB1 for idle/inactive mode UEs	Spreadtrum, UNISOC

R1-2501874	Discussion on adaptation of common signal/channel transmissions	Spreadtrum, UNISOC

R1-2501932	Discussion of Rel-19 NES UE Features	FUTUREWEI

R1-2501952	On-demand SSB SCell Operation	Google

R1-2501953	On-demand SIB1 for Idle/Inactive Mode UE	Google

R1-2501954	Adaptation of Common Signals	Google

R1-2501982	Discussion on UE features for Rel-19 NES	CATT

R1-2501996	Discussion on on-demand SSB SCell operation	CATT

R1-2501997	Discussion on on-demand SIB1	CATT

R1-2501998	Discussion on adaptation of common signal/channel transmissions	CATT

R1-2502010	OD-SSB for SCELL	Tejas Network Limited

R1-2502026	Discussion on on-demand SSB operation for SCell	China Telecom

R1-2502027	Discussion on on-demand SIB1 for idle/inactive mode UEs	China Telecom

R1-2502028	Discussion on common signal/channel adaptation	China Telecom

R1-2502035	Discussion on UE features for Rel-19 Network Energy Saving	China Telecom

R1-2502044	Discussion on on-demand SSB SCell operation	Ofinno

R1-2502045	Discussion on on-demand SIB1 for idle/inactive mode UEs	Ofinno

R1-2502050	UE features for enhancements of network energy savings for NR	Ofinno

R1-2502092	Discussion on on-demand SSB SCell operation	KT Corp.

R1-2502093	Discussion on adaptation of common signal/channel transmissions	KT Corp.

R1-2502127	Discussion on on-demand SSB SCell operation	Fujitsu

R1-2502128	Discussion on on-demand SIB1 transmission for network energy savings	Fujitsu

R1-2502129	Discussion on adaptation of common signal channel transmissions	Fujitsu

R1-2502136	Views on Network Energy Saving Enhancement UE features	Nokia

R1-2502164	Discussion on on-demand SSB SCell operation	CMCC

R1-2502165	Discussion on on-demand SIB1 for UEs in idle/inactive mode	CMCC

R1-2502166	Discussion on adaptation of common signal/channel transmissions	CMCC

R1-2502181	Discussion on UE features for enhancements of network energy savings for NR	CMCC

R1-2502192	Discussion on adaptation of common signal/channel transmission	Panasonic

R1-2502198	Discussion on on-demand SSB for SCell operation	NEC

R1-2502199	Discussion on on-demand SIB1 for UEs in idle/inactive mode	NEC

R1-2502200	Discussion on adaptation of common signal/channel transmissions	NEC

R1-2502229	On-demand SSB SCell operation for eNES	Huawei, HiSilicon

R1-2502230	Discussion on on-demand SIB1 for eNES	Huawei, HiSilicon

R1-2502231	On common channel/signal adaptation for eNES	Huawei, HiSilicon

R1-2502232	UE features for Rel-19 NES	Huawei, HiSilicon

R1-2502254	Adaptation of common signals and channels	Lenovo

R1-2502261	Discussion on the enhancement to support on demand SSB SCell operation	OPPO

R1-2502262	Discussion on the enhancement to support on demand SIB1 for idle/inactive mode UE	OPPO

R1-2502263	Discussion on adaptation of common signal/channel transmission	OPPO

R1-2502264	Discussion on UE features for enhancements of network energy savings for NR	OPPO

R1-2502321	On-demand SIB1 for idle/inactive mode UEs	Sony

R1-2502334	Discussion on on-demand SSB SCell operation	InterDigital, Inc.

R1-2502335	Discussion on on-demand SIB1 for idle/inactive mode UEs	InterDigital, Inc.

R1-2502336	Discussion on adaptation of common signal/channel transmissions	InterDigital, Inc.

R1-2502372	On-demand SSB SCell operation	Samsung

R1-2502373	On-demand SIB1 for idle/inactive mode UEs	Samsung

R1-2502374	Adaptation of common signal/channel transmissions	Samsung

R1-2502394	UE features for enhancements of network energy savings for NR	Samsung

R1-2502403	Discussion on remaining details of on-demand SSB operation on SCell	Sharp

R1-2502445	Discussion on on-demand SSB SCell operation	Xiaomi

R1-2502446	Discussion on on-demand SIB1 for idle/inactive mode UEs	Xiaomi

R1-2502447	Discussion on adaptation of common signal and channel transmissions	Xiaomi

R1-2502462	UE features for NES enhancements	Xiaomi

R1-2502469	OD-SIB1 for Idle/Inactive UEs	Tejas Network Limited

R1-2502470	Adaptation of common signal/channels	Tejas Network Limited

R1-2502478	On-demand SSB SCell operation	LG Electronics

R1-2502479	On-demand SIB1 for idle/inactive mode UEs	LG Electronics

R1-2502480	Adaptation of common signal/channel transmissions	LG Electronics

R1-2502486	Discussion on UE features for enhancements of NES	LG Electronics

R1-2502513	Discussion on On-demand SSB SCell operation	ETRI

R1-2502514	On-demand SIB1 for idle/inactive mode UEs	ETRI

R1-2502515	Adaptation of common signal/channel transmissions	ETRI

R1-2502542	Discussion on On-Demand SSB SCell operation	Transsion Holdings

R1-2502543	Discussion on on-demand SIB1 transmission for idle/inactive mode UEs	Transsion Holdings

R1-2502544	Discussion on adaptive transmission of common signal or common channel	Transsion Holdings

R1-2502569	On-demand SSB SCell operation	Lenovo

R1-2502570	On-demand SIB1 for idle/inactive mode UEs	Lenovo

R1-2502578	Discussion on on-demand SSB SCell operation	Panasonic

R1-2502579	Discussion on on-demand SIB1 for idle/inactive mode UEs	Panasonic

R1-2502612	On-demand SSB SCell Operation	Apple

R1-2502613	On On-demand SIB1 for IDLE/INACTIVE mode UEs	Apple

R1-2502614	On adaptation of common signal/channel for NES enhancements	Apple

R1-2502642	Views on UE features for Rel-19 NES	Apple

R1-2502658	On-demand SIB1 for NES	Fraunhofer IIS, Fraunhofer HHI

R1-2502659	Adaptation of Common Signals and Channels for NES	Fraunhofer IIS, Fraunhofer HHI

R1-2502678	Remaining details for OD-SIB1 request	ASUSTeK

R1-2502679	DCI based signaling for on-demand SSB	ASUSTeK

R1-2502681	Discussion on on-demand SIB1 transmission for idle UEs	Sharp

R1-2502682	Discussion on adaptation of common signal/channel transmissions	Sharp

R1-2502693	Discussion on adaptation of common signal channel transmissions	HONOR

R1-2502708	On-demand SSB SCell operation	MediaTek Inc.

R1-2502709	On-demand SIB1 for idle or inactive mode UEs	MediaTek Inc.

R1-2502710	Adaptation of common signal/channel transmissions	MediaTek Inc.

R1-2502740	Summary of UE features for enhancements of network energy savings for NR	Moderator (AT&T)

R1-2502750	Discussion on on-demand SIB1 for idle/inactive mode UEs	DENSO CORPORATION

R1-2502770	Discussion on on-demand SSB SCell operation	NTT DOCOMO, INC.

R1-2502771	Discussion on on-demand SIB1 for idle/inactive mode UEs	NTT DOCOMO, INC.

R1-2502772	Discussion on adaptation of common signal/channel transmissions	NTT DOCOMO, INC.

R1-2502788	Discussion on UE features for eNES	NTT DOCOMO, INC.

R1-2502796	Discussion on on-demand SSB SCell operation	ITRI

R1-2502797	Discussion on common signal adaptation	KDDI Corporation

R1-2502801	Update of RAN1 RRC parameter list for R19 NES WI	Rapporteur(Ericsson)

R1-2502802	On-demand SSB SCell operation	Ericsson

R1-2502803	On-demand SIB1 for UEs in idle/inactive mode for NES	Ericsson

R1-2502804	Adaptation of common signal/channel transmissions for NES	Ericsson

R1-2502808	UE features for R19 NES	Ericsson

R1-2502843	On-demand SSB operation for Scell	Qualcomm Incorporated

R1-2502844	On-demand SIB1 procedure	Qualcomm Incorporated

R1-2502845	Adaptation of common channel transmissions	Qualcomm Incorporated

R1-2502867	UE features for Rel-19 NES	Qualcomm Incorporated

R1-2502917	Discussion on on-demand SSB Scell operation	CEWiT

R1-2502918	Discussion on  on-demand SIB1	CEWiT

R1-2502919	Discussion on adaptation of common signal and channel transmissions	CEWiT

R1-2502921	Discussion on on-demand SIB1 in idle/inactive mode	CAICT

R1-2502961	Session Notes of AI 9.15.4	Ad-Hoc Chair (NTT DOCOMO, INC.)

R1-2503004	FL summary 1 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2503005	FL summary 2 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2503006	FL summary 3 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2503007	FL summary 4 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2503008	FL summary 5 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2503045	Summary #1 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2503046	Summary #2 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2503047	Summary #3 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2503048	Summary #4 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2503052	Summary#1 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2503053	Summary#2 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2503054	Summary#3 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2503085	Draft LS on DCI-based PRACH adaptation	Moderator (Ericsson)

R1-2503086	LS on DCI-based PRACH adaptation	RAN1, Ericsson

R1-2503096	DRAFT LS to RAN4 on time instance B for on-demand SSB SCell operation	LG Electronics

R1-2503108	LS to RAN4 on time instance B for on-demand SSB SCell operation	RAN1, LG Electronics

R1-2503119	Summary #5 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2503124	Summary#4 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2503125	Final Summary of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2503126	Updated list of RAN1 agreements for Rel-19 NES WI	Rapporteur (Ericsson)

R1-2503133	Final Summary of UE features for enhancements of network energy savings for NR	Moderator (AT&T)

R1-2503145	Summary of the discussion on RAN1 RRC parameter list for R19 NES	Rapporteur (Ericsson)

R1-2503153	DRAFT LS on Rel-19 higher layers parameters list Post RAN1#120bis	Moderator (Ericsson)

R1-2503154	LS on Rel-19 higher layers parameters list Post RAN1#120bis	RAN1, Ericsson

R1-2503155	Consolidated Rel-19 higher layers parameters list Post RAN1#120bis	Moderator (Ericsson)

R1-2503156	Collection of Rel-19 higher layers parameters list Post RAN1#120bis	Moderator (Ericsson)

R1-2503167	Introduction of enhancements of network energy savings for NR	Samsung

R1-2503168	Summary of email discussions [120bis-R19-38.213-Netw_Energy_NR_enh]	Moderator (Samsung)

#### 4.1.2	RAN1#121

R1-2503203	Reply LS on paging enhancement in R19 NES	CT1, Huawei

R1-2503209	LS on on-demand SSB	RAN2, Apple

R1-2503233	Discussion of on-demand SSB Scell operation	FUTUREWEI

R1-2503234	Discussion of on-demand SIB1 for idle/inactive mode UEs	FUTUREWEI

R1-2503235	Discussion of the adaptation of common signal/channel transmissions	FUTUREWEI

R1-2503236	Discussion of Rel-19 NES UE Features	FUTUREWEI

R1-2503267	On-demand SSB SCell operation for eNES	Huawei, HiSilicon

R1-2503268	Discussion on on-demand SIB1 for eNES	Huawei, HiSilicon

R1-2503269	On common channel/signal adaptation for eNES	Huawei, HiSilicon

R1-2503270	UE features for Rel-19 NES	Huawei, HiSilicon

R1-2503315	Discussion on on-demond SSB for NES	ZTE Corporation, Sanechips

R1-2503316	Discussion on on-demand SIB1 for NES	ZTE Corporation, Sanechips

R1-2503317	Discussion on common signal channel for NES	ZTE Corporation, Sanechips

R1-2503322	Discussion on NES features	ZTE Corporation, Sanechips

R1-2503365	Remaining issues on-demand SSB Scell operation	vivo

R1-2503366	Remaining issues on-demand SIB1 for idle/inactive mode UEs	vivo

R1-2503367	Remaining issues on adaptation of common signal/channel transmissions	vivo

R1-2503388	Discussion on UE features for enhancements of network energy savings for NR	vivo

R1-2503402	Network Energy Saving Enhancement UE features	Nokia

R1-2503412	On-demand SSB SCell Operation	Nokia, Nokia Shanghai Bell

R1-2503413	On-demand SIB1 for Idle/Inactive mode UEs	Nokia, Nokia Shanghai Bell

R1-2503414	Adaptation of common signal/channel transmissions	Nokia, Nokia Shanghai Bell

R1-2503416	Discussion on on-demand SSB for SCell operation	NEC

R1-2503417	Discussion on on-demand SIB1 for UEs in idle/inactive mode	NEC

R1-2503418	Discussion on adaptation of common signal/channel transmissions	NEC

R1-2503463	Introduction of Rel-19 enhancements of network energy savings for NR	Huawei

R1-2503464	Summary of email discussion [Post-120bis-Rel19-38.212-Netw_Energy_NR_enh-Core]	Moderator (Huawei)

R1-2503477	Summary of the discussion concerning the Netw_Energy_NR_enh for Release 19	Moderator (Nokia)

R1-2503519	Discussion on on-demand SSB SCell operation	Spreadtrum, UNISOC

R1-2503520	Discussion on on-demand SIB1 for idle/inactive mode UEs	Spreadtrum, UNISOC

R1-2503521	Discussion on adaptation of common signal/channel transmissions	Spreadtrum, UNISOC

R1-2503539	On-demand SSB Scell operation for NES	TCL

R1-2503570	On-demand SSB SCell operation	Samsung

R1-2503571	On-demand SIB1 for idle/inactive mode UEs	Samsung

R1-2503572	Adaptation of common signal/channel transmissions	Samsung

R1-2503594	UE features for enhancements of network energy savings for NR	Samsung

R1-2503622	LS on the creation of the Getting Ready for Energy-Efficient Networking  Working Group in the IETF	IETF GREEN

R1-2503716	Adaptation of common signals or channels	Tejas Network Limited

R1-2503717	OD-SIB1 for idle/inactive UEs	Tejas Network Limited

R1-2503718	OD-SSB for SCELL operation	Tejas Network Limited

R1-2503737	Discussion on on-demand SSB SCell operation	Ofinno

R1-2503738	Discussion on on-demand SIB1 for idle/inactive mode UEs	Ofinno

R1-2503784	Discussion on UE features for Rel-19 NES	CATT

R1-2503797	Discussion on on-demand SSB SCell operation	CATT

R1-2503798	Discussion on on-demand SIB1	CATT

R1-2503799	Discussion on adaptation of common signal/channel transmissions	CATT

R1-2503835	Discussion on on-demand SSB SCell operation	CMCC

R1-2503836	Discussion on on-demand SIB1 for UEs in idle/inactive mode	CMCC

R1-2503837	Discussion on adaptation of common signal/channel transmissions	CMCC

R1-2503853	Discussion on UE features for enhancements of network energy savings for NR	CMCC

R1-2503867	Discussion on adaptation of common signal/channel transmission	Panasonic

R1-2503886	Discussion on on-demand SSB SCell operation	Xiaomi

R1-2503887	Discussion on on-demand SIB1 for idle/inactive mode UEs	Xiaomi

R1-2503888	Discussion on adaptation of common signal and channel transmissions	Xiaomi

R1-2503903	UE features for NES enhancements	Xiaomi

R1-2503946	Draft CR on CSI sub configurations in TS 38.214	NEC

R1-2503972	Discussion on on-demand SSB SCell operation	InterDigital, Inc.

R1-2503973	Discussion on on-demand SIB1 for idle/inactive mode UEs	InterDigital, Inc.

R1-2503974	Discussion on adaptation of common signal/channel transmissions	InterDigital, Inc.

R1-2503999	Discussion on on-demand SIB1 transmission for network energy savings	Fujitsu

R1-2504000	Discussion on adaptation of common signal / channel transmissions	Fujitsu

R1-2504014	Update of RAN1 RRC parameter list for R19 NES WI	Rapporteur(Ericsson)

R1-2504015	On-demand SSB SCell operation	Ericsson

R1-2504016	On-demand SIB1 for UEs in idle/inactive mode for NES	Ericsson

R1-2504017	Adaptation of common signal/channel transmissions for NES	Ericsson

R1-2504021	UE features for R19 NES	Ericsson

R1-2504030	On-demand SSB SCell Operation	Google

R1-2504031	On-demand SIB1 for Idle/Inactive Mode UE	Google

R1-2504032	Adaptation of Common Signals	Google

R1-2504037	Discussion on on-demand SSB SCell operation	KT Corp.

R1-2504038	Discussion on adaptation of common signal/channel transmissions	KT Corp.

R1-2504050	Discussion on on-demand SSB operation for SCell	China Telecom

R1-2504051	Discussion on on-demand SIB1 for idle/inactive mode UEs	China Telecom

R1-2504052	Discussion on common signal/channel adaptation	China Telecom

R1-2504057	Discussion on UE features for Rel-19 Network Energy Saving	China Telecom

R1-2504065	On-demand SIB1 for idle/inactive mode UEs	Sony

R1-2504075	Discussion on remaining details of on-demand SSB operation on SCell	Sharp

R1-2504092	Discussion on on-demand SSB SCell operation	Fujitsu

R1-2504101	Discussion on adaptation of common signal channel transmissions	HONOR

R1-2504107	On-demand SSB SCell operation	Lenovo

R1-2504108	On-demand SIB1 for idle/inactive mode UEs	Lenovo

R1-2504141	Discussion on On-demand SSB SCell operation	ETRI

R1-2504142	On-demand SIB1 for idle/inactive mode UEs	ETRI

R1-2504143	Adaptation of common signal/channel transmissions	ETRI

R1-2504176	Discussion on On-Demand SSB SCell operation	Transsion Holdings

R1-2504177	Discussion on on-demand SIB1 transmission for idle/inactive mode UEs	Transsion Holdings

R1-2504178	Discussion on adaptive transmission of common signal or common channel	Transsion Holdings

R1-2504192	Discussion on the enhancement to support on demand SSB SCell operation	OPPO

R1-2504193	Discussion on the enhancement to support on demand SIB1 for idle/inactive mode UE	OPPO

R1-2504194	Discussion on adaptation of common signal/channel transmission	OPPO

R1-2504195	Discussion on UE features for enhancements of network energy savings for NR	OPPO

R1-2504235	Discussion on on-demand SSB SCell operation	Panasonic

R1-2504236	Discussion on on-demand SIB1 for idle/inactive mode UEs	Panasonic

R1-2504247	On-demand SSB SCell operation	LG Electronics

R1-2504248	On-demand SIB1 for idle/inactive mode UEs	LG Electronics

R1-2504249	Adaptation of common signal/channel transmissions	LG Electronics

R1-2504256	Discussion on UE features for enhancements of NES	LG Electronics

R1-2504262	On-demand SSB SCell operation	MediaTek Inc.

R1-2504263	On-demand SIB1 for idle or inactive mode UEs	MediaTek Inc.

R1-2504264	Adaptation of common signal/channel transmissions	MediaTek Inc.

R1-2504325	On-demand SSB SCell Operation	Apple

R1-2504326	On On-demand SIB1 for IDLE/INACTIVE mode UEs	Apple

R1-2504327	On adaptation of common signal/channel for NES enhancements	Apple

R1-2504355	Views on UE features for Rel-19 NES	Apple

R1-2504371	Summary of UE features for enhancements of network energy savings for NR	Moderator (AT&T)

R1-2504398	On-demand SSB operation for Scell	Qualcomm Incorporated

R1-2504399	On-demand SIB1 procedure	Qualcomm Incorporated

R1-2504400	Adaptation of common channel transmissions	Qualcomm Incorporated

R1-2504422	UE features for Rel-19 NES	Qualcomm Incorporated

R1-2504457	Discussion on on-demand SIB1 for idle/inactive mode UEs	DENSO CORPORATION

R1-2504467	Discussion on on-demand SIB1 transmission for idle UEs	Sharp

R1-2504468	Discussion on adaptation of common signal/channel transmissions	Sharp

R1-2504505	Discussion on on-demand SSB SCell operation	NTT DOCOMO, INC.

R1-2504506	Discussion on on-demand SIB1 for idle/inactive mode UEs	NTT DOCOMO, INC.

R1-2504507	Discussion on adaptation of common signal/channel transmissions	NTT DOCOMO, INC.

R1-2504526	Discussion on UE features for eNES	NTT DOCOMO, INC.

R1-2504538	Discussion on on-demand SSB SCell operation	ITRI

R1-2504547	Adaptation of common signals and channels	Lenovo

R1-2504554	Discussion on on-demand SSB SCell operation	KDDI Corporation

R1-2504556	Discussion on on-demand SIB1 in idle/inactive mode	CAICT

R1-2504558	DCI based signaling for on-demand SSB	ASUSTeK

R1-2504602	Discussion on on-demand SSB Scell operation	CEWiT

R1-2504603	Discussion on  on-demand SIB1	CEWiT

R1-2504604	Discussion on adaptation of common signal and channel transmissions	CEWiT

R1-2504673	Updated RAN1 UE features list for Rel-19 NR after RAN1 #121	Moderators (AT&T, NTT DOCOMO, INC.)

R1-2504674	Draft LS on updated Rel-19 RAN1 UE features list for NR after RAN1#121	Moderators (AT&T, NTT DOCOMO, INC.)

R1-2504675	LS on updated Rel-19 RAN1 UE features list for NR after RAN1#121	RAN1, AT&T, NTT DOCOMO, INC.

R1-2504679	FL summary 1 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2504680	FL summary 2 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2504681	FL summary 3 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2504682	FL summary 4 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2504683	FL summary 5 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2504744	Summary #1 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2504745	Summary #2 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2504746	Summary #3 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2504747	Summary #4 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2504757	Summary#1 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2504758	Summary#2 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2504890	Summary#3 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2504927	Session Notes of AI 9.15.4	Ad-Hoc Chair (AT&T)

R1-2504937	Final Summary of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2504938	List of RAN1 agreements for Rel-19 NES WI	Rapporteur (Ericsson)

#### 4.2	List of RAN WG2 contributions

#### 4.2.1	RAN2#129bis

R2-2501722	LS on time location of on-demand SSB for Scell (R1-2501633; contact: LGE)

R2-2501740	LS on neighboring cell measurement for Rel-18 SSB-less (R4-2502694; contact: ZTE)

R2-2501956	Running 38.306 CR for NES UE capability	ZTE Corporation, Sanechips, draftCR, 38.306.

R2-2501984	Introduction of Network Energy Savings Enhancements running CR, Huawei, HiSilicon, draftCR,	38.300.

R2-2502129	Summary report of [POST129][104][NES] 38.304 running CR and stage 3 open issues, Apple (Rapporteur)

R2-2502150	Summary of Comments to 38.331 CR for NES, Ericsson.

R2-2502173	Running RRC CR for enhancements for network energy efficiency, Ericsson, draftCR, 38.331.

R2-2502323	Running 38.304 CR for network energy saving, Apple (Rapporteur), draftCR, 38.304.

R2-2502584	Summary of Comments to 38.321 CR for NES, InterDigital.

R2-2502587	Running 38.321 CR for network energy saving, InterDigital, draftCR, 38.321.

R2-2501805	Discussion on on-demand SSB	Xiaomi.

R2-2501833	Discussion on on-demand SSB	HONOR

R2-2501867	On-demand SSB SCell Operation	Samsung

R2-2501886	Discussion on On-Demand SSB	OPPO

R2-2501901	Consideration on on-demand SSB SCell operation	CATT

R2-2501957	Remaining issues of on demand SSB Scell operation	ZTE Corporation, Sanechips

R2-2502130	Remaining issues on on-demand SSB for Scell	Apple

R2-2502136	Discussion on OD-SSB	Rakuten Mobile, Inc

R2-2502148	Discussion on on-demand SSB for NES	Ericsson

R2-2502219	NES SCell handling for OD-SSB	ETRI

R2-2502225	Remaining issues on on-demand SSB SCell operation 	Fujitsu

R2-2502320	Further discussion on On-demand SSB for SCell	NEC

R2-2502384	Remaining issues of on-demand SSB SCell operation	vivo

R2-2502487	On-demand SSB Scell operation discussion	Sony

R2-2502540	Discussion on On-demand SSB SCell Operation	Qualcomm Incorporated

R2-2502582	On demand SSB transmission for SCell	InterDigital

R2-2502724	Discussion on On-demand SSB for Scell	CMCC

R2-2502742	On-demand SSB SCell operation	Panasonic

R2-2502780	Discussion on on-demand SSB SCell operation

R2-2502840	Discussion on on-demand SSB SCell operation for NES

R2-2502847	Discussion on on-demand SSB transmission	LG Electronics France

R2-2502864	Discussion on On-demand SSB	KDDI Corporation (TTC)

R2-2502907	Discussion on on-demand SSB SCell operation

R2-2502915	On demand SSB handling	Nokia, Nokia Shanghai Bell

R2-2501795	Discussion on on-demand SIB1	Xiaomi

R2-2501834	Discussion on on-demand SIB1	HONOR

R2-2501864	Remaining issues for On-demand SIB1 request and UE behaviour	NEC

R2-2501866	On-demand SIB1	 Samsung

R2-2501902	Consideration on on-demand SIB1	CATT

R2-2501958	Remaining issues of on demand SIB1	ZTE Corporation, Sanechips

R2-2501982	Discussion on remaining issues of on-demand SIB1 operation for NES	Huawei, HiSilicon

R2-2502104	Remaining issues on the support for OD-SIB1	Google

R2-2502131	Remaining issues on on-demand SIB1	Apple

R2-2502134	Discussion on on-demand SIB1 for NES	Rakuten Mobile, Inc

R2-2502149	Discussion on on-demand SIB1 for NES	Ericsson

R2-2502226	Remaining issues on on-demand SIB1 procedure	Fujitsu

R2-2502229	Remaining issues on OD-SIB1	LG Electronics Inc.

R2-2502261	Discussion on Ondemand-SIB1	KDDI Corporation (TTC)

R2-2502385	Discussion on OD-SIB1 for RRC IDLE and INACTIVE UE	vivo

R2-2502400	Discussion on on-demand SIB1 for handling RLF	ITRI

R2-2502488	Remaining details for on-demand SIB1	Sony

R2-2502541	Discussion on On-demand SIB1	Qualcomm Incorporated

R2-2502577	SIB1 acquisition for Rel-19 NES UE in RRC_CONNECTED state	Fainity Innovation

R2-2502580	On-demand SIB1 request and reception	InterDigital

R2-2502596	Remaining OD SIB1 issues	Lenovo

R2-2502725	Discussion on On-demand SIB1	CMCC

R2-2502761	Consideration on on-demand SIB1	OPPO

R2-2502781	Discussion on on-demand SIB1	NTT DOCOMO, INC.

R2-2502876	Discussion on remaining issues of on-demand SIB1	SHARP Corporation

R2-2502900	Discussion on On-demand SIB1 for NES	Fraunhofer IIS, Fraunhofer HHI

R2-2502916	On demand SIB1 handling	  Nokia, Nokia Shanghai Bell

R2-2501796	Discussion on common signal adaptation	Xiaomi

R2-2501817	Discussion on adaptation of common signal channel transmission	OPPO

R2-2501865	PRACH, paging and SSB adaptation for NES	NEC

R2-2501868	Adaptation of common signal channel transmissions	Samsung

R2-2501903	Adaptation of Common signal channel transmissions	CATT

R2-2501959	Remaining issues of common signal/channel transmissions	ZTE Corporation, Sanechips

R2-2502031	Adaptation of common signal or channel	Fujitsu

R2-2502132	Remaining issues on common signal transmission adaptation	Apple

R2-2502372	Paging and PRACH adaptation	Lenovo

R2-2502386	Discussion on adaptation of common signal transmissions	vivo

R2-2502489	RAN2 impacts on SSB and RACH adaptations	Sony

R2-2502542	Discussion on Adaptation of Common Signal/Channel Transmissions	Qualcomm Incorporated

R2-2502581	Time domain adaptation of common signalling and channels	InterDigital

R2-2502726	Discussion on Common signalling adaptation	CMCC

R2-2502766	Adaptation of common signal/channel transmissions for NES	Ericsson

R2-2502782	Discussion on adaptation of common signal and channel	NTT DOCOMO, INC.

R2-2502841	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon

R2-2502917	Adaptation of common signals	Nokia, Nokia Shanghai Bell

#### 4.2.2	RAN2#130

R2-2503305	Reply LS on paging enhancement in R19 NES (C1-252548; contact: Huawei)	CT1

R2-2503312	LS on DCI-based PRACH adaptation (R1-2503086; contact: Ericsson)	RAN1

R2-2503319	Reply LS on energy saving indication from CN to RAN (R3-252404; contact: Huawei)	RAN3

R2-2503504	Introduction of Network Energy Savings Enhancements running CR, Huawei, HiSilicon, draftCR, 38.300.

R2-2503607	Running RRC CR for enhancements for network energy efficiency, Ericsson, draftCR, 38.331.

R2-2503608	Report of [POST129b][111][NES] (Ericsson)	Ericsson

R2-2503708	Running 38.304 CR for network energy saving, Apple (Rapporteur), draftCR, 38.304.

R2-2503709	Summary report of [POST129b][113][NES] 38.304 CR (Apple), Apple (Rapporteur)

R2-2503752	Summary of [POST129b][121] Initial discussion on UE capabilities in NES, ZTE Corporation, Sanechips.

R2-2503753	Draft 38.331 CR for NES UE capability, ZTE Corporation, Sanechips, draftCR, 38.331.

R2-2503754	Draft 38.306 CR for NES UE capability, ZTE Corporation, Sanechips, draftCR, 38.306.

R2-2504281	Summary of Comments to 38.321 CR for NES, InterDigital.

R2-2504282	Running 38.321 CR for network energy saving, InterDigital, draftCR, 38.321.

R2-2503376	Discussion on On-Demand SSB	OPPO

R2-2503392	On-demand SSB SCell Operation	Samsung

R2-2503407	Remaining issues on OD-SSB SCell operation	LG Electronics France

R2-2503414	Consideration on on-demand SSB SCell operation	CATT

R2-2503573	Discussion on on-demand SSB	Xiaomi

R2-2503609	Discussion on on-demand SSB for NES	Ericsson

R2-2503710	Remaining issues on on-demand SSB for Scell	Apple

R2-2503755	Remaining issues of on-demand SSB SCell operation	ZTE Corporation, Sanechips

R2-2503805	Remaining issues on on-demand SSB SCell operation	 Fujitsu

R2-2503824	Remaining issues of OD-SSB	Rakuten Mobile, Inc

R2-2503827	Discussion on on-demand SSB SCell operation	Sharp

R2-2503893	On demand SSB handling	Nokia, Nokia Shanghai Bell

R2-2503916	Issues on the procedure of on-demand SSB SCell operation	Lenovo

R2-2503944	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon

R2-2503988	Remaining issues on On-demand SSB for SCell	NEC

R2-2504036	Remaining issues of on-demand SSB SCell operation	vivo

R2-2504061	Remaining details on OD-SSB MAC CE	Sony

R2-2504242	Discussion on On-demand SSB SCell Operation	Qualcomm Incorporated

R2-2504280	On demand SSB transmission for SCell	InterDigital

R2-2504386	Discussion on On-demand SSB for Scell	CMCC

R2-2504417	On-demand SSB SCell operation	Panasonic

R2-2504418	Discussion on on-demand SSB SCell operation	NTT DOCOMO, INC.

R2-2504540	Discussion on On-demand SSB	KDDI Corporation

R2-2503351	Discussion on on-demand SIB1	Xiaomi

R2-2503391	On-demand SIB1	Samsung

R2-2503415	Consideration on on-demand SIB1	CATT

R2-2503467	Remaining issues for On-demand SIB1 request and UE behaviour	NEC

R2-2503485	Providing Cell Reselection Assistance information	Lenovo

R2-2503502	Discussion on remaining issues of on-demand SIB1 operation for NES	Huawei, HiSilicon

R2-2503636	Discussion on on-demand SIB1 for NES	Ericsson

R2-2503711	Remaining issues on on-demand SIB1	Apple

R2-2503756	Remaining issues of on-demand SIB1	ZTE Corporation, Sanechips

R2-2503797	Open issues in the running CRs for OD-SIB1	Google

R2-2503806	Remaining issues on on-demand SIB1 procedure	Fujitsu

R2-2503823	Remaining issues of OD-SIB1	Rakuten Mobile, Inc

R2-2503839	Remaining issues on OD-SIB1 operation	LG Electronics Inc.

R2-2503894	On demand SIB1 handling	Nokia, Nokia Shanghai Bell

R2-2503955	Discussion on SIBxx validity check during T311	ITRI, Acer Incorporated

R2-2503999	Consideration on on-demand SIB1	OPPO

R2-2504037	Remaining issues on OD-SIB1	vivo

R2-2504054	Remaining details for on-demand SIB1	Sony

R2-2504243	Discussion on On-demand SIB1	Qualcomm Incorporated

R2-2504387	Discussion on On-demand SIB1	CMCC	discussion

R2-2504419	Discussion on on-demand SIB1	NTT DOCOMO, INC.

R2-2504470	Discussion on on-demand SIB1	HONOR	discussion

R2-2504493	Broadcasting status for OD-SIB1 request	ASUSTeK

R2-2504533	Discussion on On-demand SIB1	KDDI Corporation

R2-2504538	Discussion on Ondemand-SIB1	KDDI Corporation (TTC)

R2-2504604	Reducing On-demand SIB1 overhead	Fraunhofer IIS, Fraunhofer HHI

R2-2504606	Remaining issues on OD-SIB1 Request	SHARP Corporation

R2-2503374	Summary of [POST129bis][122][NES] CFRA Applicability (OPPO)	OPPO

R2-2503393	Adaptation of common signal channel transmissions	Samsung

R2-2503416	Adaptation of Common signal channel transmissions	CATT

R2-2503454	Discussion on adaptation of common signal channel transmission	OPPO

R2-2503468	Discussion on Adaptation of common signal channel transmissions	NEC

R2-2503553	Adaptation of common signal or channel	Fujitsu

R2-2503574	Discussion on PRACH adaptation	Xiaomi

R2-2503712	Remaining issues on common signal transmission adaptation	Apple

R2-2503757	Remaining issues of common signal/channel adaption	ZTE Corporation, Sanechips

R2-2503828	Discussion on PRACH adaptation	Sharp

R2-2503895	Adaptation of common signals	Nokia, Nokia Shanghai Bell

R2-2503945	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon

R2-2503968	Issue for additional PRACH resources with multiple initial BWPs	Spreadtrum, UNISOC

R2-2504038	Discussion on adaptation of common signal transmissions	vivo

R2-2504055	RA-RNTI issue and RACH adaptations	Sony

R2-2504244	Discussion on Adaptation of Common Signal/Channel Transmissions	Qualcomm Incorporated

R2-2504279	Time domain adaptation of common signalling and channels	InterDigital

R2-2504388	Discussion on Common signalling adaptation	CMCC

R2-2504420	Discussion on adaptation of common signal and channel	NTT DOCOMO, INC.

R2-2504427	Adaptation of common signal/channel transmissions for NES	Ericsson

R2-2504429	[DRAFT] Reply LS on DCI-based PRACH adaptation	Ericsson

R2-2504471	Discussion on adaptation of common signal/channel transmissions	HONOR

R2-2504494	Discussion on PRACH adaptation	ASUSTeK

R2-2504579	Remaining issues on common signal and channel transmissions adaptation	LG Electronics Inc.

#### 4.3	List of RAN WG3 contributions

#### 4.3.1	RAN3#127bis

R3-251511	LS on Rel-19 higher layers parameters list Post RAN1#120	RAN1(Ericsson)

R3-251517	LS on paging enhancement in R19 NES	RAN2(Xiaomi)

R3-251566	(BL CR to 38.473) Introduction of Network Energy Saving Enhancement	Huawei, Ericsson, CMCC, Samsung, ZTE, Nokia, Deutsche Telekom, Lenovo, NEC, Jio

R3-251567	Introduction of Network Energy Saving Enhancement	Ericsson, Huawei, CMCC, Samsung, Qualcomm, Nokia, ZTE, Lenovo, Deutsche Telekom, NEC

R3-251623	Aspects of on-demand SSB for NES enhancements	Qualcomm Inc.

R3-251624	Aspects of on-demand SIB1 for NES enhancements	Qualcomm Inc.

R3-251641	(TP to BLCR for TS 38.300) Introduction of OD-SIB1	CATT, Ericsson, Huawei, Samsung, China Telecomm, Rakuten, ZTE, CMCC, Lenovo

R3-251642	(TP for BLCR to TS 38.423, 38.473) On support of OD-SSB	CATT

R3-251643	(TP to BLCR for F1AP) On support of OD-SIB1 for idle UE	CATT, Fujitsu

R3-251650	Open points on OD-SSB. Adaptation of common signals/channels	Nokia

R3-251651	(TP for BLCR to TS 38.423, TS 38.300) Stage 2 and stage 3 Proposals for On-Demand SIB1	Nokia

R3-251683	Support on-demand SSB operation	NEC

R3-251684	Support on-demand SIB1	NEC

R3-251723	Discussion on on-demand SSB SCell operation in low-carbon green network	Samsung

R3-251724	Discussion on on-demand SIB1 in low-carbon green network	Samsung

R3-251725	Introduction of Network Energy Saving Enhancement	Samsung, Huawei, NEC, CATT, ZTE, Ericsson

R3-251751	(TP to BLCR for TS 38.473 and TS 38.470) Discussion on on-demand SSB SCell operation	Huawei

R3-251752	(TP to BLCR for TS 38.473 and TS 38.423) Discussion on on-demand SIB1 for UEs in idle or inactive mode	Huawei

R3-251753	(TP to BLCR for TS 38.473) Discussion on common signal adaptation	Huawei, CATT, CMCC

R3-251802	Discussion on energy saving indication from CN to RAN	Huawei, CATT, NEC

R3-251803	[draft] Reply LS on energy saving indication from CN to RAN	Huawei, CATT, NEC

R3-251848	Xn impact of On-demand SIB1 for UEs in idle/inactive mode	Ericsson, Deutsche Telekom, China Unicom, Jio Platforms

R3-251849	F1 impact of On-demand SIB1 for UEs in idle/inactive mode	Ericsson, Deutsche Telekom, China Unicom, Jio Platforms

R3-251860	(TP to 38.473) On support on-demand SSB SCell operation	China Telecom

R3-251887	Discussion on energy saving indication from CN to RAN	ZTE Corporation

R3-251888	[DRAFT] Reply LS on energy saving indication from CN to RAN	ZTE Corporation

R3-251892	(TP to BL CR for 38.473) Further discussion on on-demand SSB Scell operation	ZTE Corporation

R3-251893	(TP to BL CR for 38.423) Further discussion on on-demand SIB1	ZTE Corporation

R3-251894	(TP to BL CR for 38.473) Discussion on UE capability for paging adaptation	ZTE Corporation, CATT, Lenovo

R3-251895	[Draft] Reply LS on paging enhancement in R19 NES	ZTE Corporation, CATT, Lenovo

R3-251951	Discussion on On-demand SSB for SCell	Lenovo

R3-251952	(TP to BL CR 38.423 and 38.473) Discussion on On-demand SIB1 for Idle/Inactive UE	Lenovo

R3-252019	WI Work plan for R19 Network Energy Savings	Ericsson

R3-252041	Discussion on On-Demand SSB SCell Operation	CMCC

R3-252042	(TPs to BL CRs) Support On-Demand SIB1 for UEs	CMCC

R3-252044	On-demand SSB SCell operation	Ericsson

R3-252047	Adaptation of common signal/channel transmissions	Ericsson

R3-252117	XnAP Enhancements for UL WUS Configuration in On-Demand SIB1	Jio Platforms Ltd (JPL)

R3-252208	Support for OD-SIB1 mechanism at Cell A and NES Cell	Rakuten Mobile, Inc

R3-252210	Signalling to support OD-SIB1	Rakuten Mobile, Inc

R3-252307	Introduction of Network Energy Saving Enhancement	Ericsson, Huawei, CMCC, Samsung, Qualcomm, Nokia, ZTE, Lenovo, Deutsche Telekom, NEC

R3-252310	CB:#ES3	ZTE

R3-252322	[draft] Reply LS on energy saving indication from CN to RAN	Huawei, CATT, NEC

R3-252357	CB: # ES1_OndemandSSB	Huawei

R3-252361	(TP to BL CR for 38.473) Discussion on UE capability for paging adaptation	ZTE Corporation, CATT, Lenovo

R3-252385	(TP to BLCR for TS 38.423) On-demand SIB1 for Ues in idle/inactive mode	Ericsson, Deutsche Telekom, China Unicom, Jio Platforms

R3-252386	(TP to BLCR for TS 38.473) On-demand SIB1 for Ues in idle or inactive mode	Huawei

R3-252387	CB: # ES2 (Summary of offline discussion)	Ericsson

R3-252450	(TP to BLCR for TS 38.300) Introduction of OD-SIB1	CATT, Ericsson, Huawei, Samsung, China Telecomm, Rakuten, ZTE, CMCC, Lenovo, Deutsche Telekom

R3-252451	Introduction of Network Energy Saving Enhancement	Samsung, Huawei, NEC, CATT, ZTE, Ericsson

R3-252466	(TP to BLCR for TS 38.473) On-demand SIB1 for Ues in idle or inactive mode	Huawei, Ericsson, CATT, Samsung, Deutsche Telekom, Qualcomm, NEC, CMCC, ZTE

R3-252467	(TP to BLCR for TS 38.423) On-demand SIB1 for Ues in idle/inactive mode	Ericsson, Deutsche Telekom, China Unicom, Jio Platforms, Samsung, Huawei, NEC,CMCC, ZTE, CATT

R3-252468	(TP to BLCR for TS 38.300) Introduction of OD-SIB1	CATT, Ericsson, Huawei, Samsung, China Telecomm, Rakuten, ZTE, CMCC, Lenovo, Deutsche Telekom

R3-252469	(BL CR to 38.470) Introduction of Network Energy Saving Enhancement	Samsung, Huawei, NEC, CATT, ZTE, Ericsson

R3-252497	(BL CR to 38.300) Introduction of Network Energy Saving Enhancement	CATT, Ericsson, Huawei, Samsung, China Telecomm, Rakuten, ZTE, CMCC, Lenovo, Deutsche Telekom, Nokia

R3-252498	Introduction of Network Energy Saving Enhancement	Ericsson, Huawei, CMCC, Samsung, Qualcomm, Nokia, ZTE, Lenovo, Deutsche Telekom, NEC

R3-252499	(BL CR to 38.473) Introduction of Network Energy Saving Enhancement	Huawei, Ericsson, CMCC, Samsung, ZTE, Nokia, Deutsche Telekom, Lenovo, NEC, Jio

#### 4.3.2	RAN3#128

R3-253005	Reply LS on paging enhancement in R19 NES	CT1(Huawei)

R3-253034	LS on Rel-19 higher layers parameters list Post RAN1#120bis	RAN1(Ericsson)

R3-253101	(BL CR to 38.300) Introduction of Network Energy Saving Enhancement	CATT, Ericsson, Huawei, Samsung, China Telecomm, Rakuten, ZTE, CMCC, Lenovo, Deutsche Telekom, Nokia

R3-253102	Introduction of Network Energy Saving Enhancement	Ericsson, Huawei, CMCC, Samsung, Qualcomm, Nokia, ZTE, Lenovo, Deutsche Telekom, NEC

R3-253103	(BL CR to 38.470) Introduction of Network Energy Saving Enhancement	Samsung, Huawei, NEC, CATT, ZTE, Ericsson

R3-253104	(BL CR to 38.473) Introduction of Network Energy Saving Enhancement	Huawei, Ericsson, CMCC, Samsung, ZTE, Nokia, Deutsche Telekom, Lenovo, NEC, Jio

R3-253171	BL CR to 38.401 on Introduction of Network Energy Saving Enhancement	Qualcomm Inc.

R3-253172	(TP to BL CR 38.401) Stage-2 procedures for OD SIB1	Qualcomm Inc.

R3-253209	(TP to BLCR TS38.473) Support on-demand SSB	NEC

R3-253210	Support on-demand SIB1	NEC

R3-253241	(TP to BL CR for 38.473) Further discussion on on-demand SSB Scell operation	ZTE Corporation

R3-253242	(TP to BL CR for 38.423) Further discussion on on-demand SIB1	ZTE Corporation

R3-253243	Introduction of Network Energy Saving Enhancement	ZTE Corporation, Samsung, Lenovo

R3-253258	(TP to BLCR for TS 38.473 and TS 38.470) Discussion on on-demand SSB SCell operation	Huawei

R3-253259	(TP to BLCR for TS 38.473 and TS 38.423) Discussion on on-demand SIB1 for UEs in idle or inactive mode	Huawei

R3-253306	(TP for BLCRs to 38.423, 38.473) On support of OD-SSB	CATT

R3-253307	(TP for BLCR to F1AP) On support of OD-SIB1 for idle UE	CATT

R3-253328	Discussion on On-demand SSB for SCell	Lenovo

R3-253329	(TP to BL CR 38.423) Discussion on On-demand SIB1 for Idle/Inactive UE	Lenovo

R3-253362	(TP to 38.473) On support on-demand SSB SCell operation	China Telecom

R3-253363	On support on-demand SIB1	China Telecom

R3-253407	Further discussion on common signal adaptation	Huawei, NEC, CATT, CMCC

R3-253532	Open points on OD-SSB. Adaptation of common signals/channels	Nokia

R3-253533	(TP for BLCR to TS 38.423) Message definitions for inter-node signalling for On-Demand SIB1	Nokia

R3-253539	Xn impact of On-demand SIB1 for UEs in idle/inactive mode	Ericsson, Deutsche Telekom, China Unicom, Jio Platforms

R3-253548	F1 impact of On-demand SIB1 for UEs in idle/inactive mode	Ericsson, Deutsche Telekom, China Unicom, Jio Platforms, China Telecom

R3-253549	On-demand SSB SCell operation	Ericsson

R3-253550	Adaptation of common signal/channel transmissions	Ericsson

R3-253551	WI Work plan for R19 Network Energy Savings	Ericsson

R3-253639	Discussion on on-demand SSB SCell operation in low-carbon green network	Samsung

R3-253640	Discussion on on-demand SIB1 in low-carbon green network	Samsung

R3-253671	Discussion on On-Demand SSB SCell Operation	CMCC

R3-253672	(TPs to BL CRs) Support On-Demand SIB1 for UEs	CMCC

R3-253721	End-to-End Xn and F1 Signalling to support OD-SIB1	Rakuten Mobile, Inc

R3-253780	Network Energy Saving Summary of offline discussion	Ericsson

R3-253811	CB:#R19NES	Ericsson

R3-253856	(TP to BL CR for TS 38.423) Xn impact of On-demand SIB1 for Ues in idle/inactive mode	Ericsson, Deutsche Telekom, China Unicom, Jio Platforms, Rakuten, NEC, Nokia, Lenovo, Huawei, Qualcomm

R3-253857	(TP to BLCR for TS 38.473) Discussion on on-demand SIB1 for UEs in idle or inactive mode	Huawei, Ericsson, NEC, Qualcomm, CATT, Nokia, Deutsche Telekom, Rakuten, ZTE, Samsung, Lenovo

R3-253917	Introduction of Network Energy Saving Enhancement	ZTE Corporation, Samsung, Lenovo, Huawei, Qualcomm, Ericsson, CATT, Nokia, Rakuten, NEC

R3-253957	(TP to BL CR for TS 38.423) Xn impact of On-demand SIB1 for Ues in idle/inactive mode	Ericsson, Deutsche Telekom, China Unicom, Jio Platforms, Rakuten, NEC, Nokia, Lenovo, Huawei, Qualcomm, Samsung, CATT, ZTE

R3-253974	(BL CR to 38.423) Introduction of Network Energy Saving Enhancement	Ericsson, Huawei, CMCC, Samsung, Qualcomm, Nokia, ZTE, Lenovo, Deutsche Telekom, NEC

R3-253975	(BL CR to 38.473) Introduction of Network Energy Saving Enhancement	Huawei, Ericsson, CMCC, Samsung, ZTE, Nokia, Deutsche Telekom, Lenovo, NEC, Jio

#### 4.4	List of RAN WG4 contributions

#### 4.4.1	RAN4#114bis

R4-2503124	Views on on-demand SSB SCell operation	Qualcomm Technologies Ireland

R4-2503125	About RRM requirements for adaptation of common signal/channel transmission	Qualcomm Technologies Ireland

R4-2503126	On RRM requirements for on-demand SIB1	Qualcomm Technologies Ireland

R4-2503130	Discussion on RRM requirements for OD-SSB for network energy saving	MediaTek inc.

R4-2503131	Discussion on RRM requirements for adaptation of common signal/channel transmission	MediaTek inc.

R4-2503293	Discussion on RRM requirements for on-demand SSB SCell operation in R19 NES	vivo

R4-2503294	On RRM requirements for common signal or channel adaptation in R19 NES	vivo

R4-2503295	Discussion on RRM requirement impacts from on-demand SIB1 in R19 NES	vivo

R4-2503327	Discussion on OD-SSB requirements	Xiaomi

R4-2503328	Discussion on RRM requirements for adaptation of common signal/channel in NES	Xiaomi

R4-2503343	On RRM requirements of on-demand SSB SCell operation	OPPO

R4-2503344	On RRM requirements of SSB adaptation	OPPO

R4-2503345	On RRM requirement for on-demand SIB1	OPPO

R4-2503380	Discussion on RRM requirement of OD-SSB for NES enhancement	CMCC

R4-2503381	Discussion on RRM requirement of signal adapatation for NES enhancement	CMCC

R4-2503382	Discussion on RRM requirement of OD-SIB1 for NES enhancement	CMCC

R4-2503429	Discussion on RRM requirements for on-demand SSB SCell operation	CATT

R4-2503430	Discussion on RRM requirements for adaptation of common signalchannel transmission	CATT

R4-2503431	Discussion on RRM requirements for On-demand SIB1	CATT

R4-2503499	RRM requirements for OD-SSB SCell operation	Nokia, Nokia Shanghai Bell

R4-2503569	Discussion on on-demand SSB SCell operation	Apple

R4-2503570	Discussion on adaptation of common signal/channel transmission	Apple

R4-2503571	Discussion on on-demand SIB1	Apple

R4-2503591	Discussion on RRM requirements of on-demand SSB Scell operation for Rel-19 NES	LG Electronics

R4-2503594	Discussion on RRM requirements of SSB adaptation for Rel-19 NES	LG Electronics

R4-2503596	Discussion on RRM requirements of on-demand SIB1 for Rel-19 NES	LG Electronics

R4-2503625	Topic summary for [114bis][215] Netw_Energy_NR_enh_Part1	Moderator (Ericsson)

R4-2503626	Topic summary for [114bis][216] Netw_Energy_NR_enh_Part2	Moderator (Huawei)

R4-2503866	Discussion on RRM impact for Rel-19 NES OD-SSB	Huawei, HiSilicon

R4-2503867	Discussion on RRM impact for Rel-19 NES on adaptation	Huawei, HiSilicon

R4-2503868	Discussion on RRM impact for Rel-19 OD-SIB1	Huawei, HiSilicon

R4-2503917	Discussion on On-demand SSB SCell operation for enhancements of network energy savings for NR	China Telecom

R4-2503918	Discussion on adaptation of common signal/channel transmission for enhancements of network energy savings for NR	China Telecom

R4-2504019	Discussion on RRM requirements for On-demand SSB SCell operations	Intel Corporation

R4-2504225	Discussion on OD-SSB general part in Rel-19 NES		Ericsson

R4-2504226	Discussion on OD-SSB in Rel-19 NES				Ericsson

R4-2504227	Discussion on RS and channel adaptation in Rel-19 NES	Ericsson

R4-2504228	Discussion on OD-SIB1 in Rel-19 NES	Ericsson

R4-2504246	Discussion on On-demand SSB SCell operation of R19 NES	ZTE Corporation, Sanechips

R4-2504247	Discussion on adaptation of common signal and channel of R19 NES	ZTE Corporation, Sanechips

R4-2504289	Discussion on RRM requirements for NES OD-SSB	Samsung

R4-2504290	General discussion on the NES common signal adaptation	Samsung

R4-2504506	RRM requirements for SSB adaptation	Nokia, Nokia Shanghai Bell

R4-2504507	RRM requirements for On-demand-SIB1	Nokia, Nokia Shanghai Bell

R4-2504906	WF on RRM requirements for Netw_Energy_NR_enh_Part1	Ericsson

R4-2504907	WF on RRM requirements for Netw_Energy_NR_enh_Part2	Huawei

R4-2504920	Ad-hoc minutes for Netw_Energy_NR_enh			Ericsson

#### 4.4.2	RAN4#115

R4-2505353	On core requirements for on-demand SSB SCell operation	Qualcomm Technologies Ireland

R4-2505354	On core requirements for adaptation of common signal/channel transmission	Qualcomm Technologies Ireland

R4-2505355	On core requirements for on-demand SIB1	Qualcomm Technologies Ireland

R4-2505356	On RRM performance requirements for Rel-19 NES	Qualcomm Technologies Ireland

R4-2505357	UE and BS demodulation performance requirements	Qualcomm Technologies Ireland

R4-2505570	Topic summary for [115][221] Netw_Energy_NR_enh_Part1	Moderator (Ericsson)

R4-2505571	Topic summary for [115][222] Netw_Energy_NR_enh_Part2	Moderator (Huawei)

R4-2505662	On remaining issues for on-demand SSB SCell operation in R19 NES	vivo

R4-2505663	On remaining issues for adaptation on common signal in R19 NES	vivo

R4-2505664	On remaining issues for on-demand SIB1 in R19 NES	vivo

R4-2505672	Draft CR for Scheduling for L1-SINR measurements for Reporting for NES	vivo

R4-2505702	On RRM requirements of on-demand SSB SCell operation	OPPO

R4-2505703	On RRM requirements of SSB adaptation	OPPO

R4-2505704	On RRM requirement for on-demand SIB1	OPPO

R4-2505705	draftCR on requirements for interruption in paging reception	OPPO

R4-2505706	on RRM performance requirements of R19 NES	OPPO

R4-2505777	Discussion on OD-SSB requirements	Xiaomi

R4-2505778	Discussion on RRM requirements for adaptation of common signal/channel in NES	Xiaomi

R4-2505779	"draftCR on Rel19 NES (8.2.2.2.2	Interruptions at SCell activation/deactivation)"	Xiaomi

R4-2505780	"draftCR on Rel19 NES (8.2.2.2.3	interruption during measurements on deactivated SCC)"	Xiaomi

R4-2505870	Discussion on on-demand SSB SCell operation	Apple

R4-2505871	DraftCR of interruptions for OD-SSB transmission	Apple

R4-2505872	Discussion on adaptation of common signal/channel transmission	Apple

R4-2505873	DraftCR of NR intra-frequency measurement due to SSB adaptation	Apple

R4-2505874	Discussion on on-demand SIB1	Apple

R4-2505975	View on BS demodulation requirement for NR NES	Samsung

R4-2506098	Discussion on RRM requirements for on-demand SSB SCell operation	CATT

R4-2506099	Draft CR to TS 38.133 on carrier-specific scaling factor for OD-SSB	CATT

R4-2506100	Discussion on RRM requirements for On-demand SIB1	CATT

R4-2506101	Discussion on RRM performance requirements for Rel-19 NES for NR	CATT

R4-2506205	RRM requirements for OD-SSB SCell operation	Nokia, Nokia Shanghai Bell

R4-2506206	draftCR on OD-SSB based SCell activation delay with L3 reporting	Nokia, Nokia Shanghai Bell

R4-2506207	RRM performance requirements for network energy saving	Nokia, Nokia Shanghai Bell

R4-2506294	Discussion on RRM requirements of on-demand SSB Scell operation for Rel-19 NES	LG Electronics Inc.

R4-2506295	Discussion on RRM requirements of SSB adaptation for Rel-19 NES	LG Electronics Inc.

R4-2506296	Discussion on RRM requirements of on-demand SIB1 for Rel-19 NES	LG Electronics Inc.

R4-2506298	Draft CR for NES for SCell Deactivation Delay Requirement for Activated SCell	LG Electronics Inc.

R4-2506299	Discussion on RRM performance requirements for Rel-19 NES	LG Electronics Inc.

R4-2506340	Discussion on On-demand SSB SCell operation for enhancements of NES for NR	China Telecom

R4-2506341	Draft CR on OD-SSB based L1-RSRP measurements for Reporting	China Telecom

R4-2506342	Discussion on adaptation of common signal channel transmission for enhancements of NES for NR	China Telecom

R4-2506343	Discussion on On-demand SIB1 for enhancements of NES for NR	China Telecom

R4-2506344	Discussion on RRM performance requirements of NES for NR	China Telecom

R4-2506421	draftCR on Introduce OD-SSB based direct SCell Activation requirement at SCell addition	CMCC

R4-2506422	Discussion on RRM requirement of OD-SSB for NES enhancement	CMCC

R4-2506423	Discussion on RRM requirement of signal adaptation for NES enhancement	CMCC

R4-2506424	draftCR on Introduce timing requirements for UL-WUS for OD-SIB1	CMCC

R4-2506425	Discussion on RRM requirement of OD-SIB1 for NES enhancement	CMCC

R4-2506426	Discussion on test cases for R19 NES	CMCC

R4-2506482	Discussion on RRM requirements for OD-SSB for network energy saving	MediaTek inc.

R4-2506483	DraftCR on introduction and measurement restriction for L1-SINR measurement	MediaTek inc.

R4-2506484	Discussion on RRM requirements for OD-SIB1	MediaTek inc.

R4-2506683	Discussion on UE demodulation and CSI performance requirements for Rel-19 NES	Samsung

R4-2506705	Discussion on RRM impact for Rel-19 NES OD-SSB	Huawei, HiSilicon

R4-2506706	DraftCR on OD-SSB based SCell Activation Delay Requirement for Deactivated PUCCH SCell	Huawei, HiSilicon

R4-2506707	Discussion on RRM impact for Rel-19 NES on adaptation	Huawei, HiSilicon

R4-2506708	DraftCR on SSB adaptation for L1-RSRP	Huawei, HiSilicon

R4-2506709	Discussion on RRM impact for OD-SIB1	Huawei, HiSilicon

R4-2506710	Discussion on performance requiremnets for Rel-19 NES	Huawei, HiSilicon

R4-2506824	Discussion on RRM requirements for On-demand SSB SCell operations	Intel Corporation

R4-2506825	DraftCR for scheduling availability during UE intra-frequency measurements based on OD-SSB	Intel Corporation

R4-2506904	Discussion on OD-SSB in Rel-19 NES	Ericsson

R4-2506905	Discussion on SSB adaptation in Rel-19 NES	Ericsson

R4-2506906	Discussion on OD-SIB1 in Rel-19 NES	Ericsson

R4-2506907	Discussion on UE feature list for Rel-19 NES	Ericsson

R4-2506908	Discussion on RRM performance requirements for Rel-19 NES	Ericsson

R4-2506909	Work plan for R19 NES enhancement performance part	Ericsson

R4-2506910	draftCR on OD-SSB based NR intra-frequency measurements	Ericsson

R4-2506911	draftCR on SSB adapation for SCell actiation	Ericsson

R4-2507043	Discussion on demodulation performance requirements for R19 NES	Huawei, HiSilicon

R4-2507108	Demodulation requirements for NR network energy saving enhancements	Ericsson

R4-2507251	Discussion on NES common signal adaptation	Samsung

R4-2507252	Discussion on RRM requirements for NES OD-SSB	Samsung

R4-2507253	Discussion on performance requirements for NES	Samsung

R4-2507254	Draft CR on OD-SSB based SCell Activation Delay Requirement for Deactivated SCell	Samsung

R4-2507286	Discussion on On-demand SSB SCell operation of R19 NES	ZTE Corporation, Sanechips

R4-2507287	Discussion on adaptation of common signal and channel of R19 NES	ZTE Corporation, Sanechips

R4-2507294	Discussion on Perf part of R19 NES	ZTE Corporation, Sanechips

R4-2507298	Draft CR on scheduling restriction of L1-RSRP measurement of R19 NES	ZTE Corporation, Sanechips

R4-2507299	Draft CR on SSB adaptation impact on the CBD of R19 NES	ZTE Corporation, Sanechips

R4-2507352	RRM requirements for SSB adaptation	Nokia, Nokia Shanghai Bell

R4-2507353	Draft CR38.133 L1-SINR measurements for SSB periodicity adaptation	Nokia, Nokia Shanghai Bell

R4-2507354	RRM requirements for On-demand-SIB1	Nokia, Nokia Shanghai Bell

R4-2507625	On Enhancements of Network Energy Savings for NR	Nokia

R4-2507863	Topic summary for [115][331] NR_Network_energy_demod	Moderator (Ericsson)

R4-2508276	Ad-hoc minutes for Netw_Energy_NR_enh	Ericsson

R4-2508277	WF on RRM requirements for Netw_Energy_NR_enh_Part1	Ericsson

R4-2508278	WF on RRM requirements for Netw_Energy_NR_enh_Part2	Huawei

R4-2508330	Work plan for R19 NES enhancement performance part	Ericsson

R4-2508331	Reply LS on time instance B for on-demand SSB SCell operation	Ericsson

R4-2508332	Reply LS on new servingCellMO of OD-SSB	Apple

R4-2508376	Draft CR38.133 L1-SINR measurements for SSB periodicity adaptation	Nokia, Nokia Shanghai Bell

R4-2508421	DraftCR of NR intra-frequency measurement due to SSB adaptation	Apple

R4-2508422	DraftCR on SSB adaptation for L1-RSRP	Huawei, HiSilicon

R4-2508423	draftCR on SSB adapation for SCell actiation	Ericsson

R4-2508424	Draft CR on SSB adaptation impact on the CBD of R19 NES	ZTE Corporation, Sanechips

R4-2508425	draftCR on requirements for interruption in paging reception	OPPO

R4-2508426	draftCR on Introduce timing requirements for UL-WUS for OD-SIB1	CMCC

R4-2508440	Reply LS on new servingCellMO of OD-SSB on SCell	Apple

R4-2508441	WF on RRM requirements for Netw_Energy_NR_enh_Part2	Huawei

R4-2508628	Way Forward for [115][331] NR_Network_energy_demod	Ericsson