3GPP TSG RAN Meeting #108														RP-251680

Prague, Czech Republic, June 9-13, 2025

Agenda Item:			15.1.4

Source:					Ofinno

Title:							Proposal for NR coverage enhancements Phase 3

Document for:		Decision

# Introduction

Further enhancements on coverage enhancements have been discussed in the last RAN plenary [2], and there was good support for a few objectives. In this contribution we provide our views on the proposed objectives of the Rel-20 WI on the NR coverage enhancements Phase 3.

# PRACH repetition with different UE TX beams

## Motivation

Multiple PRACH transmissions with different beams have been studied since Rel-17, where companies showed a performance gain between 2 dB-4.7 dB for different urban and rural scenarios [1]. Companies have also shown an additional 1-3 dB gain over multiple PRACH transmission with the same beam that is supported in Rel-18.

Multiple PRACH transmission with different TX beams gives the opportunity to refine beams at the random-access procedure stage and helps in beam refinement for Msg3 transmissions.

## Multiple PRACH transmissions

In supporting multiple PRACH transmission with different beams, there are two main approaches as discussed during Rel-18. In the first approach, the UE determines a single SSB index and selects multiple ROs corresponding to the single SSB index (e.g., a RO set in Rel-18). The UE uses different TX beams across the multiple ROs. A base station can indicate a best beam among the different TX beams based on RO index/resource among the multiple ROs. In the second approach, the UE determines multiple SSB indexes corresponding to multiple TX beams, and the UE selects multiple ROs corresponding to the multiple SSB indexes. To align between the UE and the base station, the second approach would require a predetermined pattern of selecting SSB indexes or need to inform the multiple SSB indexes to the base station. Moreover, this approach does not allow the UE to use finer beams that are QCL-ed with the best SSB index for multiple PRACH transmissions. Based on potential drawbacks of the second approach, we propose to limit the scope to the first approach (i.e., a single SSB is used for multiple PRACH transmissions with different beams).

Given the limited time unit available for the work item, we also like to clarify the scope that legacy (e.g., Rel-18) SSB-RO mapping is reused.

Proposal 1: For multiple PRACH transmissions with different beams, the following assumptions are captured in a WID:

- Multiple PRACH transmissions with different beams are associated with an SSB.

- SSB-RO mapping and RO set selection for PRACH repetition (Rel-18) is reused for PRACH TX with different beams.

## Msg3 beam refinement

The network indicates, during the random-access response window (i.e., Msg2), the best beam among the different beams used for the multiple PRACH transmissions. The UE can determine spatial domain filter parameters for Msg3 transmission (including any retransmission) based on the best beam indicated by the network.

For other potential objectives discussed at the workshop [2], the following captures our views:

- Msg 5 PUSCH repetition: Based on operator’s observation on bottleneck on Msg5, we are supportive of Msg 5 repetition with minimum specification impact. Currently DCI format 1_0 does not have remaining/spare bits. Therefore, we may use some bits from MCS field to indicate the repetition similar to UL grant indication in RAR.

- PUSCH coverage for high UL data rate: we are neutral, though we feel this can be postponed to 6G study.

- Open reconfigurable intelligent surface (RIS): we feel this can be postponed to 6G study.

# Summary and conclusion

Based on the above, we suggest the following proposals for the Rel-20 WI on NR coverage enhancement Phase 3:

Proposal 1: Specify multiple PRACH transmissions with different UE TX beams [RAN1, RAN2]:

- Multiple PRACH transmissions with different beams are associated with an SSB.

- Indication of UE TX beam by the network in Msg2 for Msg3 PUSCH transmission.

- NOTE: SSB-RO mapping and RO set selection for preamble repetition (Rel-18) is reused for PRACH TX with different beams.

# References

- 3GPP TR 38.830 V17.0.0, “Study on NR coverage enhancements (Release 17)” 2020-12-17

- RP-250802, “Moderator's summary for RAN1 led REL-20 topics” RAN1 Chair (Samsung), 3GPP TSG RAN #107, Incheon, Korea, March 12th – 14th, 2025.