3GPP TSG RAN Meeting #108					        RP-251701

Prague, Czech Republic, June 9-13, 2025

Source:	AT&T

Title:	AT&T Views on minimum TPRs for IMT-2030

Agenda Item:	16.1

Document for: 	Discussion/Decision

# 1. Introduction

At the TSG RAN #107 meeting in Incheon, Korea a revised SID was approved to study 6G scenarios and requirements [1]. One of the objectives is for TSG RAN to investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. In this document, we provide our views on minimum TPRs for IMT-2030.

# 2. Discussion

Based on the moderator input in [2], we provide our input and proposed values as below.

| ITU IMT-2030 TPR |  | AT&T Proposed Values | Moderator Proposal [2] |
| --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | Alt. 1 or Alt. 2 with 1x of IMT-2020 | Alternative 1: Not to define a requirement, e.g. Leave the value for peak data rate for the technology proponent to provide Alternative 2: Define a requirement with reasonable value |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 1.5x of IMT-2020 UL: 1.5x of IMT-2020 | DL: [1.5-2.5x] of IMT-2020 UL: [1.5-2.5x] of IMT-2020 |
| User experienced data rate (Mbit/s) |  | Discuss and agree evaluation methodology before setting values | For UL: - RAN#107 agreement: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL - Proposal 1: Use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Discuss and agree evaluation methodology before setting values | Companies are encouraged to provide antenna assumption for different frequency ranges together with the requirements, e.g. 4GHz, 7GHz, 30GHz Applicable deployment scenarios should also be considered. |
| Average spectral efficiency (bit/s/Hz) |  | [1.5~2x] of IMT-2020 | [3x] of IMT-2020 |
| Area traffic capacity (Mbit/s/m²) |  | Discuss and agree evaluation methodology before setting values | [5x] of IMT-2020 |
| latency | User plane latency (ms) | 1x of IMT-2020 | IC: [2] HRLLC: [1] |
|  | Control plane latency (ms) | 1x of IMT-2020 | [10] |
| Connection density (devices/km²) |  | 10x of IMT-2020 Current discussion in ITU is to break this up into IoT Connection Density and XR Connection Density/Connection capacity. Clarify if this is for eMBB, XR, mMTC … | [10x] of IMT-2020 |
| Reliability |  | 1x of IMT-2020 | [1-10-5] |
| Mobility (bit/s/Hz @ km/h) |  | 1x of IMT-2020 | Keep the velocities defined for IMT-2020. Should new velocity be added? (up to 1000/1200km/h for airplane) Should 50% spectrum efficiency be increased for the velocity defined in IMT-2020? e.g. [1.5x] of IMT-2020 |
| Mobility interruption time (ms) |  | 1x of IMT-2020 | [0 ms] |

Additionally, the following two open issues need to be resolved.

Proposal 2-5: For coverage, further discuss the following three options: - Option 1: define coverage as a quantitative TPR - Option 2: do not define coverage as a TPR, use link budget evaluation approach (same as IMT-2020) - Option 3: define extra connectivity as a qualitative TPR, which was revised from a part of coverage

Proposal 2-6: For resilience, further discuss the following two options: - Option 1: define resilience as a qualitative TPR - Option 2: do not define resilience as a TPR, these can be covered by submission template for IMT-2030.

For Proposal 2-5, our preference is Option 2. Similarly, for Proposal 2-6, we prefer Option 2.

Proposal 1: Agree the values as proposed in the table below.

Proposal 2: Do not define coverage as a TPR, use link budget evaluation approach (same as IMT-2020)

Proposal 3: Do not define resilience as a TPR, these can be covered by submission template for IMT-2030

| ITU IMT-2030 TPR |  | Proposed Values |
| --- | --- | --- |
| Peak data rate (Gbit/s) |  | Do not define a requirement or 1x of IMT-2020 |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 1.5x of IMT-2020 UL: 1.5x of IMT-2020 |
| Average spectral efficiency (bit/s/Hz) |  | [1.5~2x] of IMT-2020 |
| latency | User plane latency (ms) | 1x of IMT-2020 |
|  | Control plane latency (ms) | 1x of IMT-2020 |
| Connection density (devices/km²) |  | 10x of IMT-2020 for mMTC Current discussion in ITU is to break this up into IoT Connection Density and XR Connection Density/Connection capacity. Clarify if this is for eMBB, XR, mMTC … |
| Reliability |  | 1x of IMT-2020 |
| Mobility (bit/s/Hz @ km/h) |  | 1x of IMT-2020 |
| Mobility interruption time (ms) |  | 1x of IMT-2020 |

Recommendation ITU-R M.2160 includes the framework and overall objective of the future development of IMT for 2030 and beyond. The recommendation includes the envisioned usage scenarios of IMT-2030, and the capabilities associated with the envisioned usage scenarios, as well as envisaged frequency bands for the deployment.

While discussing the capabilities of the usage scenarios for IMT-2030, it is paramount to discuss the guidelines for evaluating these capabilities and the corresponding test environments and evaluation configurations. In ITU M.2412 [4], “guidelines for evaluation of radio interface technologies for IMT-2020,” the evaluation methodology and the testing environments and evaluation configurations were described. Five test scenarios were chosen, including three outdoor scenarios, - dense urban for eMBB, rural-eMBB, urban Macro-mMTC and URLLC. The deployment parameters of the 3GPP channel models in TR38.901 [5] are used for evaluating the identified scenarios.

The 3GPP channel models are widely used in the industry at large to evaluate various technology components. Their validity and ability to model realistic deployment scenarios is of utmost importance to corroborate the performance of the algorithms used to improve the communication links.

While 3GPP channel models’ parameters are used to model equivalent deployment scenarios in various cities for different operators, often the urban nature of different cities differs across countries and across continents. There is, for example, a big difference in the terrain, urban planning and consequently deployment scenarios between San Francisco and Dallas or New York city, as well as a big difference between San Francisco and Prague or Paris. In the United States, a typical inter-site distance in suburban environments is on average around 1700m. The UMa channel model for urban Macro environments assumes a 500m ISD.

A typical city in the United States, outside of the downtown area, which can be modeled as a sub-urban or sometimes urban environment, is characterized by big residential areas of one or two-story buildings, high vegetation, sometimes rising above 25m, and commercial buildings that are 4 floors in height. These types of deployment scenarios are not well captured by the urban macro UMa scenario, used in the ITU-2020 evaluations, nor the rural macro RMa scenario, used to evaluate eMBB in ITU-2020; yet they constitute more than 70% of the deployment configurations in North America.

Observation 1: Urban Macro and Dense Urban Macro used for evaluating capabilities in ITU-2020 do not include typical urban scenarios in North America

In Rel. 19, an ongoing study item on channel modelling enhancements for 7-24GHz for NR [6], agreed to introduce a new channel model for suburban deployments called SMa – suburban macro scenario. The suburban macro scenario captures all the deployment characteristics for a suburban macro scenario, including the inter-site distance, the base station height and the nature of the clutter in these environments.  It is thus very important to include this model in the evaluations of capabilities for IMT-2030, for all the considered frequencies, and all relevant usage scenarios.

Proposal 4: Suburban Macro deployments, based on the SMa model introduced and developed in Rel. 19, is included in the test scenarios for evaluating the capabilities of the usage scenarios in IMT-2030.

Proposal 5: Suburban Macro deployments, based on the SMa model introduced and developed in Rel. 19, is included in the test scenarios for evaluating the capabilities of the usage scenarios in IMT-2030 for all envisioned frequency bands, especially low bands and mid bands (up to 24GHz).

# 4. Conclusion

In this document, we provided our views on minimum TPRs for IMT-2030. The following is observed and proposed:

Observation 1: Urban Macro and Dense Urban Macro used for evaluating capabilities in ITU-2020 do not include typical urban scenarios in North America

Proposal 1: Agree the values as proposed in the table below.

Proposal 2: Do not define coverage as a TPR, use link budget evaluation approach (same as IMT-2020)

Proposal 3: Do not define resilience as a TPR, these can be covered by submission template for IMT-2030

| ITU IMT-2030 TPR |  | Proposed Values |
| --- | --- | --- |
| Peak data rate (Gbit/s) |  | Do not define a requirement or 1x of IMT-2020 |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 1.5x of IMT-2020 UL: 1.5x of IMT-2020 |
| Average spectral efficiency (bit/s/Hz) |  | [1.5~2x] of IMT-2020 |
| latency | User plane latency (ms) | 1x of IMT-2020 |
|  | Control plane latency (ms) | 1x of IMT-2020 |
| Connection density (devices/km²) |  | 10x of IMT-2020 for mMTC Current discussion in ITU is to break this up into IoT Connection Density and XR Connection Density/Connection capacity. Clarify if this is for eMBB, XR, mMTC … |
| Reliability |  | 1x of IMT-2020 |
| Mobility (bit/s/Hz @ km/h) |  | 1x of IMT-2020 |
| Mobility interruption time (ms) |  | 1x of IMT-2020 |

Proposal 4: Suburban Macro deployments, based on the SMa model introduced and developed in Rel. 19, is included in the test scenarios for evaluating the capabilities of the usage scenarios in IMT-2030.

Proposal 5: Suburban Macro deployments, based on the SMa model introduced and developed in Rel. 19, is included in the test scenarios for evaluating the capabilities of the usage scenarios in IMT-2030 for all envisioned frequency bands, especially low bands and mid bands (up to 24GHz).

# References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC/Verizon/NTT DOCOMO, INC./Deutsche Telekom

- RT-250024, Moderator's summary for IMT-2030 TPRs in CC#1, Moderator (CMCC)