3GPP TSG RAN WG Meeting #108	                                                                            RP-251718

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	7

Source:	ZTE Corporation, Sanechips

Title:	R17 and R18 PWS for NTN feature removal alignment between RAN and SA

Document for:		Discussion and Decision

1. Introduction

In SA1#108 meeting, SA1 has agreed to introduce a new requirement for the support for PWS over NG-RAN from Rel-17 [1]. However, after evaluating the requirement of SA1, CT1 decided to introduce the support for PWS over satellite NG-RAN in Rel-19 and sent the LS to SA1[1].

1. Overall Description: SA1 has agreed to introduce a new requirement for the support for PWS over satellite NG-RAN (CR#0081 to TS 22.268 in S1-244777) from Rel-17. CT1 has discussed the stage 2 aspects of this requirement and determined that this requirement would have UE impacts that would create backward compatibility issues (e.g. country-specific duplicate detection timers for PLMNs using non-country specific PLMN ID with MCC 9xx over satellite access). Consequently, CT1 has decided to introduce the support for PWS over satellite NG-RAN in Rel-19. This work will be done under the new CT-wide Rel-19 work item on the support for PWS over satellite E-UTRAN and satellite NG-RAN. CT1 kindly asks SA1 to align their specifications accordingly. 2. Actions: To SA1 group. ACTION: CT1 kindly asks SA1 to align the stage 1 specifications to reflect the support for PWS over satellite NG-RAN from Rel-19.

Then, in SA1#110 meeting, SA1 has agreed to align the stage 1 specifications to reflect the support for PWS over satellite NG-RAN from Rel-19 and replied the LS to CT1 and RAN3[2].

# 1 Overall description SA1 thanks CT1 for the LS in C1-250715. SA1 has agreed the following CRs in order align the stage 1 specifications to reflect the support for PWS over satellite NG-RAN from Rel-19. # 2 Actions To CT1 and RAN3 ACTION: Please take the above information into account in the future work

Meanwhile, in RAN3#128 meeting, RAN3 has also discussed this issue and agreed to remove the support of PWS for NTN in R17/R18. And the reply LS from RAN3 is given as below[3].

1. Overall Description: RAN3 thanks SA1 for the LS on stage 1 requirements for PWS support over satellite NG-RAN. RAN3 agrees that PWS for NTN is not supported in Rel-17/18 and has endorsed the attached CRs for NR NTN and IoT NTN. 2. Actions: To SA1, CT1, RAN2 groups. ACTION: RAN3 asks SA1, CT1, RAN2 groups to take the above into account and align their specifications as needed.

In this contribution, we would to draw the attention on this cross SA and RAN issue and propose to remove the support of PWS for NTN in RAN and SA WGs in R17/R18 to make sure such feature removal among all impacted WGs are aligned and the WI summary should also reflect such change.

- Discussion

From the perspective of RAN, to align with the decision of SA1, RAN3 has agreed to remove the support of PWS on the usage of Mapped Cell ID in R17/R18, as given in the package of endorsed CRs to TS 38.300/36.300[3].

In addition, in RAN2#130 meeting, it was proposed to introduce a clarification that PWS is not supported for NR NTN in R17/R18 TS 38.304 that limited service within an acceptable cell definition refers only to emergency calls[4][5]. While there is no consensus on this clarification in RAN2, this correction is postponed to RAN2#131 meeting. Accordingly, not only for RAN3, other RAN WGs, such as RAN1, RAN2 and RAN4 should also review their specifications to ensure the removal of PWS for NTN in R17/R18.

Proposal 1: RAN WGs shall review the corresponding impacted specifications to remove the support of PWS for NTN in R17/R18.

Moreover, it was introduced in R17 in below WIs:

| 860046 | Solutions for NR to support non-terrestrial networks (NTN) | NR_NTN_solutions |  | RP-221169 | Nicolas CHUBERRE, Thales |
| --- | --- | --- | --- | --- | --- |

| 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks | LTE_NBIOT_eMTC_NTN | R1 | RP-211601 | MediaTek |
| --- | --- | --- | --- | --- | --- |

For R17 WI summary, in section 5.1.2 of TR 21.917[6], the Mapped Cell ID is used for Public Warning Services in NR NTN, therefore, the corresponding description should be removed accordingly based on the above analysis.

NG-RAN signalling (RAN3) The Cell Identity, indicated by the gNB to the Core Network as part of the User Location Information corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported. It is used for Paging Optimization in NG interface, Area of Interest and Public Warning Services. The Cell Identity included within the target identification of the handover messages allows identifying the correct target radio cell as well as for RAN paging.

Meanwhile, there is no similar description in IoT NTN in section 5.2 of TR 21.917. Therefore, TR 21.917 only needs to be updated to remove the corresponding feature for NR NTN. Since TR 21.917 is under control of SA, the CR to TR 21.917 to SA plenary is needed..

Proposal 2: A CR to TR 21.917 to SA plenary to remove the support of PWS for NR NTN is needed.

Furthermore, such kind of feature removal from previous releases may have impact not only on WI summary, but also on multiple RAN/SA WGs. It is better  to ask  WI rapporteur to  review it and check whether anything is missing in other WGs besides CT1, SA1, RAN3, RAN2.

Proposal 3: The WI rapporteur is tasked to review this removal feature in R17 and R18 and check whether anything is missing in other WGs besides CT1, SA1, RAN3, RAN2. The corresponding TEI corrections for R17 and R18 should be treated in the involved RAN/SA WGs.

3. Conclusion

Proposal 1: RAN WGs shall review the corresponding impacted specifications to remove the support of PWS for NTN in R17/R18.

Proposal 2: A CR to TR 21.917 to SA plenary to remove the support of PWS for NR NTN is needed.

Proposal 3: The WI rapporteur is tasked to review this removal feature in R17 and R18 and check whether anything is missing in other WGs besides CT1, SA1, RAN3, RAN2. The corresponding TEI corrections for R17 and R18 should be treated in the involved RAN/SA WGs.

4. Reference

[1] C1-250715, LS on stage 1 requirements for the support for PWS over satellite NGRAN in Rel-17, CT1

[2] S1-252393, Reply LS to LS on stage 1 requirements for the support for PWS over satellite NGRAN in Rel-17, SA1

[3] R3-253867, Reply LS on stage 1 requirements to support PWS over satellite NG-RAN in Rel-17, RAN3

[4] R3-2504661, Remove of PWS support in NR NTN, Ericsson

[5] R3-2504660, Remove of PWS support in NR NTN, Ericsson

[6] TR 21.917 v17.0.1