3GPP TSG RAN Meeting #1	08                                                RP-251721

Prague, Czech Republic, June 9-13, 2025

Title: 	Views on Ambient IoT enhancement in 5G-A Rel-20

Source: 	ZTE Corporation, Sanechips

Agenda item:	15.1.3

Document for:	Discussion and decision

Introduction

In RAN#106 meeting, the way forward [1] for Ambient IoT in Rel-19 WI and Rel-20 work plan were approved. In RAN#107 meeting, Rel-20 scope were further discussed and the conclusion can be found in [2].

General Scope for Rel-19 WI in RP-243280 The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following A-IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. Work plan for Rel-20 in RP-243280 - Includes D2T2 - Includes Device 2 - A limited number of additional use cases, deployments, connectivity topologies, devices, traffic types in TR38.848 as a starting point - Note: work on D2T2, device 2 shall follow the Rel-19 study conclusions, and further down-selection of architecture options for topology 2 may be needed. For indoor use cases, deviations from Rel-19 study conclusion shall be well justified and agreed. - Details to be discussed at a later RAN plenary meeting, including the need for study phase. Moderator observations and recommendations in RP-250802 - Device 1 in D2T2 - Device 2b in D1T1 and D2T2 - Study on A-IoT for outdoor scenarios - Further discuss other aspects of Rel-20 A-IoT (e.g. DO-A traffic, unavailability direction 2) in RAN#108 - [3~3.5] TUs in RAN1 and [2] TUs in RAN2 for Release 20 5G-Adv A-IoT

In this contribution, the potential enhancements for Rel-20 A-IoT are discussed.

Discussion

2.1 Outdoor scenario

Use case

- According to RAN studies, eight representative use cases (rUCs) have been formulated for A-IoT. The mapping between SA1 use cases and traffic scenarios to these RAN rUCs is captured in Table 4.1.1-1 of TR38.848 [3]. Since Rel-19 A-IoT focuses on indoor scenarios, a significant number of representative use cases listed in Table 1 haven’t yet been addressed. Extending A-IoT from indoor to outdoor scenarios is crucial for improving operational efficiency and reducing maintenance costs in applications such as automated supply chain management, logistics, and smart grids, etc. Moreover, the combination of A-IoT with extremely low cost/complexity with wide-area coverage will provide a compelling advantage over other technologies with limited coverage and narrower use cases.

- Table 1 Outdoor use cases for A-IoT

| Representative use case (rUC) | Applicable scenarios |
| --- | --- |
| rUC5: Outdoor inventory | Airport terminal / shipping port, automated supply chain distribution, etc. |
| rUC6: Outdoor sensor | Smart grids, forest fire monitoring, dairy farming, etc. |
| rUC7: Outdoor positioning | Finding remote lost item, location service, etc. |
| rUC8: Outdoor command | Device activation and deactivation, elderly health care, etc. |

- While indoor inventory/command were studied and specified in Rel-19, it is straightforward to extend them to outdoor scenario with necessary enhancements.

- A-IoT sensors can support to report real-time monitoring, such as temperature, humidity, which requires devices to autonomously initiate D2R transmission (i.e.,DO-A traffic) with larger message sizes. However, due to the limited TUs, DO-A traffic was deprioritized in Rel-19. To support these use cases effectively, DO-A needs to be considered in Rel-20.

- Additionally, the design target of positioning has been provided in TR38.848, but the locating introduced in Rel-19 is limited to reader ID granularity. For topology 1 with wide area coverage in outdoor, positioning can be left to BS reader implementation. For topology 2 with limited coverage, locating with reader ID granularity may be sufficient. Therefore, positioning enhancements can be deprioritized in Rel-20.

- In summary, inventory, sensors and command should be considered for A-IoT outdoor scenarios in Rel-20.

- Support outdoor inventory/command/sensor for Rel-20 A-IoT.

Requirements

- In Rel-19 A-IoT RAN WG SI, three device types, device 1, 2a and 2b, were analyzed. The peak power consumption is assumed to be several micro watts for device 1, and up to hundreds of micro watts for device 2. The basic blocks of device architectures based on RF-ED receivers for devices 1, 2a, and 2b have been defined, as well as those for receiver architectures based on IF-ED or ZIF for device 2b. Furthermore, Device C, studied in the Rel-18 RAN SI, is characterized by higher power consumption, ranging from 1 mW to 10 mW.

- To extend A-IoT use cases to outdoor environments, coexistence with NR and achieving comparable coverage are essential. Devices with varying peak power consumption, complexity, and capability are designed to meet diverse application requirements. Therefore, it is important to identify which device types are best suited for outdoor scenarios.

Co-existence with NR and providing similar coverage as NR are critical to extend the A-IoT use cases to outdoor.

Co-existence

- Based on the evaluation of indoor scenarios in Rel-19, co-existence between A-IoT and NR is feasible when NR UEs are not deployed indoors alongside A-IoT devices. But if NR UEs are present indoors along with A-IoT devices, the interference caused by their co-existence will result in obvious NR capacity reduction and/or A-IoT SINR performance degradation. Therefore, A-IoT and NR systems should be operated with indoor/outdoor separation or to adopt other techniques to mitigate interference. For outdoor scenarios, the interference between macro cells and devices becomes more serious, leading to significant performance degradation in R2D link or NR. Solutions to enable better co-existence between A-IoT and NR are needed. For example, narrower RF filters can help A-IoT devices filter out unwanted frequency-domain signals, device without CW can resolve the CW interference issues, thereby improving overall performance.

- Solutions to enable better co-existence between A-IoT and NR need to be considered for outdoor scenarios.

- In Rel-19 WI, only device 1 was supported for A-IoT. The maximum coverage of device 1 is up to tens of meters, which doesn’t meet outdoor coverage requirements. Thus, device 1 cannot be supported for outdoor deployment.

- For device 2a, high CW transmission power is required to improve the coverage. However, in outdoor scenarios, this will cause significant interference to the NR system. Additionally, since device 2a may have a large receiving bandwidth of 20MHz, NR signals from neighboring frequency could also interfere with R2D reception of device 2a. In other words, supporting device 2a will face larger coexistence challenges with NR in outdoor scenarios. Therefore, device 2a should be deprioritized in Rel-20.

- For device 2b, since CW is not needed and an RF BPF can be implemented, the coexistence impact on NR is minimal. Moreover, considering that device 2b can support larger coverage distance, it should be introduced in Rel-20. In addition, a device type (device C) with higher power consumption could also be considered.

- In Rel-19, clock synchronization is achieved through sample counting via R2D TAS, resulting in a clock accuracy of 10%. For active device 2b/C equipped with an RF BPF, synchronization of both R2D and D2R should be further investigated.

- Device 2b/C is introduced to support outdoor scenarios in Rel-20 A-IoT.

- The synchronization of R2D and D2R should be further studied for Device 2b/C.

Coverage

In Rel-19 SI, it was agreed that the maximum coverage target is 50m. To improve coverage performance and better utilize the higher capabilities of device 2b/C for outdoor cases, some potential enhancements are needed.

In Rel-19, given the limited capabilities of device 1, only binary modulation, i.e., OOK for the R2D link and OOK/BPSK for the D2R link, is utilized. Furthermore, FEC coding is not applied in the R2D link. In Rel-20,  enhancements like FEC for R2D transmission can be considered to improve the performance and coverage.

- Techniques to improve coverage should be considered in Rel-20 A-IoT.

Connectivity topology

- In TR38.848, the following deployment scenarios and connectivity topologies are defined for A-IoT networks.

- Deployment scenario

- Deployment scenario 1: Device indoors, basestation indoors

- Deployment scenario 2: Device indoors, basestation outdoors

- Deployment scenario 3: Device indoors, UE-based reader

- Deployment scenario 4: Device outdoors, basestation outdoors

- Deployment scenario 5: Device outdoors, UE-based reader

- Conectivity topologies

- Topology 1: BS ↔ Ambient IoT device

- Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

- Topology 3: BS ↔ assisting node ↔ Ambient IoT device ↔ BS

- Topology 4: UE ↔ Ambient IoT device

- In Rel-19 SI, Deployment scenario 1 with Topology 1 (D1T1) and Deployment scenario 2 with Topology 2 (D2T2) were studied and evaluated for A-IoT. Specifically, D1T1 and D2T2 are further categorized into several scenarios in Figure 1 considering different CW deployments:

| D1T1-A1 | D1T1-A2 | D1T1-B | D1T1-C |
| --- | --- | --- | --- |
| D2T2-A1 | D2T2-A2 | D2T2-B | D2T2-C |

Figure 1  Topologies for D1T1 and D2T2

- Among the scenarios listed in Figure 1, only D1T1-B is supported in Rel-19 WI. As discussed above, outdoors scenarios are essential for supporting more use cases and traffic. Therefore, some deployment scenarios and topologies for outdoor services should be considered in Rel-20 A-IoT. Specifically, D4T1 should be supported, as it is designed for large coverage.

- D4T1 for device 2b/C outdoors can be considered for Rel-20 A-IoT.

2.2 Topology 2

According to the work plan for Rel-20 A-IoT, Topology 2 (TP#2) will be supported at least for D2T2. Given that the maximum transmission power and coverage of TP#2 are comparable to those of an RFID reader, it is more practical and cost-effective to deploy Device 1 in the D2T2-A2 configuration. In contrast, configurations such as D2T2-A1/B require additional transmission nodes and coordination mechanisms, leading to increased deployment complexity and cost. Therefore, the D2T2-A2 topology is recommended for Device 1.

D2T2-A1/B require additional transmission nodes and coordination mechanisms, leading to increased deployment complexity and cost.

- D2T2-A2 topology is recommended for Device 1.

For TP#2, RRC based resource allocation is sufficient. Thus, we have proposed to specify the following aspects for TP#2 [4]:

- RRC signaling to provision radio resources at the UE reader for A-IoT operation

- Dedicated signaling in RRC_CONNECTED state for configuring A-IoT radio resources

- Procedures to enable the UE to release the A-IoT resources upon cell change or moving to out of coverage

- UE assistance information from the reader to gNB for configuration of A-IoT radio resources

- As a second priority, support operation of A-IoT whilst the UE reader is in one of RRC_INACTIVE state or RRC_IDLE state (RAN2 to down select between INACTIVE and IDLE states)

- For TP#2, specify RRC signaling to provision radio resources at the UE reader for A-IoT operation.

- Dedicated signaling in RRC_CONNECTED state for configuring A-IoT radio resources

- Procedures to enable the UE to release the A-IoT resources upon cell change or moving to out of coverage

- UE assistance information from the reader to gNB for configuration of A-IoT radio resources

- As a second priority, support operation of A-IoT whilst the UE reader is in one of RRC_INACTIVE state or RRC_IDLE state (RAN2 to down select between INACTIVE and IDLE states)

2.3 Device 2b indoor

According to the conclusions from RAN#107, the indoor variant of Device 2b (i.e., D1T1 and D2T2) is also recommended for consideration within the scope of Rel-20 A-IoT. However, it is important to note that specifying separate active device types for indoor and outdoor scenarios could lead to duplicated design efforts and increased fragmentation within the IoT market.

Instead, a more efficient approach would be to define a device type targeting outdoor use cases. For indoor deployment of active device, the same design can be reused with appropriate trimming or simplification as needed.

Furthermore, based on the spectrum usage described in TR 38.769, both R2D and D2R links in the D2T2-C (i.e., Device 2b) utilize FDD uplink spectrum. In contrast, the spectrum usage of Device 2b in the D1T1-C configuration aligns with that of a normal FDD UE. Given this spectrum usage consideration, it is recommended that Device 2b in Topology 2 be deprioritized.

Specifying separate active device types for indoor and outdoor scenarios could lead to duplicated design efforts and increased fragmentation within the IoT market

- For indoor deployment of active device, the outdoor design can be reused with appropriate trimming or simplification as needed.

- Device 2b in topology 2 is de-prioritized due to the spectrum usage issue.

2.4 Other enhancements for Rel-20

Random access enhancement

For contention-based random access, the slotted-ALOHA algorithm was considered in Rel-19 to reduce collision. Under this approach, the device randomly selects one access occasion for the A-IoT Msg1 within the range of access occasions assigned by the reader. This method may still lead to conflicts, requiring re-access or multiple rounds of inventory. To address this issue, several enhanced anti-collision algorithms can be considered for Rel-20, e.g., combined algorithm of Q-selection and BTree, combined algorithm of Q-selection and ALOHA [6].

Alternatively, we understand deterministic access occasion selection can be considered to reduce access collision in multiple devices paging or group paging cases, e.g., access occasion selection based on device identity.

Furthermore, in Rel-19, RAN2 has studied and evaluated 2-step RA procedure. It is seen that 2-step RA provides significant gains at low and medium collision rates (see R2-2500778). 2-step RA procedure will also improve the latency of the overall RA procedure. Unified RA which includes both RN16 and the device ID in MSG1 was also considered as a compromise between the device complexity and the overall gains. However, given that new device types will be introduced in Rel-20 and low complexity devices can use the Rel-19 baseline, it is possible to directly consider 2-step RA as a possible candidate for A-IoT RACH in Rel-20.

- Solutions to increase network capacity and improve access efficiency should be considered for outdoor scenario, e.g., deterministic access occasion selection.

- 2-step RA procedure should be considered for Rel-20 A-IoT.

In Rel-19, devices are not expected to receive parallel service request for overlapping reader scenario. Whilst it is not realistic to completely avoid overlapping coverage from readers, the restriction to avoid parallel service requests is left to network implementation in Rel-19. In Rel-20, especially with topology 2, ensuring the above restriction is not realistic. Furthermore, even for a single reader case, enabling parallel service requests is useful and is supported in other competing technologies such as RFID. Hence RAN2 agreed that this issue is properly handled in Rel-20:

Agreements on parallel service request 1 Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in stage 2 specification. 2 The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in stage 3 specification. 3 Send LS to RAN3 to notify them of agreements 1 and 2 4 Parallel service request for overlapping reader scenario can be addressed in Rel-20

Based on the above, we propose the following objective:

- Enhanced mechanisms to enable parallel service request procedures both for single reader and multi-reader scenarios should be specified in Rel-20.

- Device (un)availability

Due to limited energy storage and energy harvesting capabilities, the device availability time may be shorter than its unavailability time.

According to the current slotted-ALOHA procedure, the device is required to randomly select a counter from the access occasions provided by reader and continuously monitor each R2D trigger to decrease the counter until it equals to 0. This process significantly consumes device energy, especially for device type 2 with relatively high power consumption. In scenarios where involves large number of devices, the prolonged R2D detection time may cause many devices to deplete their charge before completing the access procedure. Therefore, device energy conservation becomes particularly important.

The following directions are studied to better utilize the limited device availability time due to energy harvesting, and solutions are captured in TR38.769.

- Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable.

- Direction 2: Reader can provide information to a device based on which the device may become available /unavailable.

However, only direction 1 is considered in Rel-19 WI. Given the challenges described above for device 2, device (un)availability issue can be further studied for device 2b/C in Rel-20.

- Device (un)availability can be further studied for device 2b/C in Rel-20.

Timeline and work scope for WGs

Based on the discussion above, the key aspects for Rel-20 A-IoT can be summarized as below:

- Outdoor scenarios with device 2b/C at least in Topology 1,

  - Note: device 2b indoor can reuse the outdoor designs.

- DO-A traffic,

- Topology 2 in higher layer,

- Device availability

- and relevant enhancements.

Considering the urgent market needs for outdoor uses cases, a 6-month study item followed by a 12-month work item is suggested. The following scope for each WG is suggested.

- Consider the following scope for each WG in Rel-20.

Outdoor scenario(SI+WI): led by RAN1

                - RAN1

- Study and specify device 2b/C in Topology 1 for outdoor inventory/command/sensor, including the evaluation of coverage, synchronization, device (un)availability and necessary enhancements

                - RAN2/3

- Study and specify procedures of device 2b/C in Topology 1 to support outdoor inventory/command/sensor, including enhancements to support DO-A traffic

                - RAN4

- Study and specify requirements of device 2b/C and applicable BS in Topology 1 for outdoor scenario, including the evaluation of co-existence with NR

Topology 2(WI): led by RAN2

- RAN2

- Protocol enhancements to support Topology 2

- Enhancements to the random access procedure including support of 2-step RACH

- Enhancements to avoid duplicate responses to the same service request

                - RAN3

- Protocol stack and signalling support for Topology 2

                - RAN4

- RF/RRM/de-modulation requirements of readers

Conclusion

Based on the above discussion about Rel-20 A-IoT scope, we have the following observations and proposals:

- Co-existence with NR and providing similar coverage as NR are critical to extend the A-IoT use cases to outdoor.

D2T2-A1/B require additional transmission nodes and coordination mechanisms, leading to increased deployment complexity and cost.

Specifying separate active device types for indoor and outdoor scenarios could lead to duplicated design efforts and increased fragmentation within the IoT market

- Support outdoor inventory/command/sensor for Rel-20 A-IoT.

- Solutions to enable better co-existence between A-IoT and NR need to be considered for outdoor scenarios.

- Device 2b/C is introduced to support outdoor scenarios in Rel-20 A-IoT.

- The synchronization of R2D and D2R should be further studied for Device 2b/C.

- Techniques to improve coverage should be considered in Rel-20 A-IoT.

- D4T1 for device 2b/C outdoors can be considered for Rel-20 A-IoT.

- D2T2-A2 topology is recommended for Device 1.

- For TP#2, specify RRC signaling to provision radio resources at the UE reader for A-IoT operation.

- Dedicated signaling in RRC_CONNECTED state for configuring A-IoT radio resources

- Procedures to enable the UE to release the A-IoT resources upon cell change or moving to out of coverage

- UE assistance information from the reader to gNB for configuration of A-IoT radio resources

- As a second priority, support operation of A-IoT whilst the UE reader is in one of RRC_INACTIVE state or RRC_IDLE state (RAN2 to down select between INACTIVE and IDLE states)

- For indoor deployment of active device, the outdoor design can be reused with appropriate trimming or simplification as needed.

- Device 2b in topology 2 is de-prioritized due to the spectrum usage issue.

- Solutions to increase network capacity and improve access efficiency should be considered for outdoor scenario, e.g., deterministic access occasion selection.

- 2-step RA procedure should be considered for Rel-20 A-IoT.

- Enhanced mechanisms to enable parallel service request procedures both for single reader and multi-reader scenarios should be specified in Rel-20.

- Device (un)availability can be further studied for device 2b/C in Rel-20.

- Consider the following scope for each WG in Rel-20.

Outdoor scenario(SI+WI): led by RAN1

                - RAN1

- Study and specify device 2b/C in Topology 1 for outdoor inventory/command/sensor, including the evaluation of coverage, synchronization, device (un)availability and necessary enhancements

                - RAN2/3

- Study and specify procedures of device 2b/C in Topology 1 to support outdoor inventory/command/sensor, including enhancements to support DO-A traffic

                - RAN4

- Study and specify requirements of device 2b/C and applicable BS in Topology 1 for outdoor scenario, including the evaluation of co-existence with NR

Topology 2(WI): led by RAN2

                - RAN2

- Protocol enhancements to support Topology 2

- Enhancements to the random access procedure including support of 2-step RACH

- Enhancements to avoid duplicate responses to the same service request

                - RAN3

- Protocol stack and signalling support for Topology 2

                - RAN4

- RF/RRM/de-modulation requirements of UE readers

References

- RP-243280. Way forward for Ambient IoT for REL-19 and REL-20, Huawei, RAN#106

- RP-250802. Moderator's summary for RAN1 led REL-20 topics, RAN1 Chair (Samsung), RAN#107