3GPP TSG RAN Meeting #108                                                 RP-251723

Prague, Czech Republic, June 9-13, 2025

Title: 	Views on NR-NTN in 5G-A Rel-20

Source: 	ZTE Corporation, Sanechips

Agenda item:	15.1.5

Document for:	Discussion and decision

Introduction

The following consensus has been reached at RAN#107[1] on NR-NTN in Rel-20:

Conclusion for GNSS resilient operation - Study Item with checkpoint to determine whether we have normative work and the scope based on results of study item phase. - Study should consider Rel-16 as baseline for discussion. Study should focus on NR NTN and be backward compatible with legacy UEs. Aim to minimize physical layer procedures impact to 5GA and avoid physical layer channel/signal changes. - Objective to be narrowed down for RAN#108 - No additional NR NTN enhancements with RAN1 impact can be considered at this point given limited 5GA TUs Additional topic to be included in objective. - Specify necessary enhancements to support E-UTRA TN to NR NTN handover in RRC connected mode - TU estimate RAN2 (0.25 TU), RAN4 (0.5) - Deadline to complete the work (first half of release) TU allocation: Title; First-order TU Estimate (# TUs). NR-NTN Enhancement; RAN1: [1] RAN2: [0.5].

In general, this conclusion provides valuable guidances for the scope discussion, especially for reasonable workload management. In this contribution, the detailed analysis on the relevant topics are provided with recommended objectives.

Discussion on the scope of NR-NTN

  - Analysis on the “GNSS resilient operation”

As summarized in [2], in RAN#107 meeting, the “GNSS resilisent operation” is considered as the candidate scope for 5G-A Rel-20 NR-NTN given the support from majority. However, there are still different interpretations on this aspect along with different scopes. More specifically,

- Interpretation-1: Positioning-free

According to this interpretation, the “GNSS resilient operation” means that the “UE Location information” is assumed to be unavailable at UE side. However, it will totally revert the following fundamental assumption in 5G for both NR- and IoT-NTN:

- UEs with GNSS (Global Navigation Satellite Systems) capabilities

Moreover, from technical perspective, this direction will lead to the “re-design” of 5G NTN system on many key aspects, e.g.,:

  - UL synchronization: As the basic feature for NTN, the pre-compensation mechanism (i.e., UE will calculate and compensate the TA and Doppler for service link) is adopted since Rel-17 with the many merits, e.g., reusage of the legacy preamble, minimized changes on the changes of procedure, reduced gNB and satellite complexity for implementation, etc.

  - Mobility: For the mobility enhancement, e.g., NTN-NTN, or NTN-TN, based on the comparison bewteen the inaccuracy (i.e., RSRP margin) and very limited difference among RSRP measurements from different cells (e.g., satellites or beams in one satellite), and consideration on the long latency, the “location-/Time information” based mechanism are supported to harvest the benefits of such deterministic system.

Additionally, given the above impact, this direction is also not aligned with the conclusion and allocated TUs above. Then, any study based on Interpretation-1 should be avoided in 5G-A stage and can be considered for new RAT design, e.g., 6G, if justified.

- Interpretation-2: GNSS-free but NTN-based positioning

According to this interpretation, the “GNSS resilient operation” means that the “UE Location information” is still assumed to be avaiable at UE side, but from different mechanism, i.e., no GNSS but NTN system based positioning.

However, for this direction,

  - Firstly, “study” the NTN based positioning metholdogy is required, which shares totally different assumptions as the TN celluar based approaches, with high workload for evaluation, e.g., similar as location verfication in Rel-18 NR-NTN to satisfy the needs of regulation.

  - Secondly, the applicable scenario for this direction is unclear. For example, if the intention is to replace the overall GNSS-based location information, it means that in the initial access, the positioning should be performed before the Msg-1 transmission to ensure the pre-compensation mechanims can still work. In this way, more changes are required for the initial access along with other procedures (e.g., positioning procedure, UE capabilities, etc).

Additionally, except for the impacts on RAN1/2, RAN3 and RAN4 TU are also needed to complete the study for both procedure and feasibility/performance for each “Positioning” approach.

- Interpretation-3: GNSS accuracy relaxation

According to this interpretation, the “GNSS resilient operation” means that the accuracy of “UE Location information” may not satisfy the existing RAN4 requirement, which will impact the overall accuracy for UL synchronization. However, for this case, different implementations for the GNSS at UE side and use case will lead to the mis-aligned assumption on the accuracy, especially comparing to the typical value as defined in RAN4. Without clear understanding on the scenario and target, it will lead to diverged discussion in WGs.

So, for this direction, the study should start from RAN4 firstly and reasonable assumption. Otherwise, similar changes as Interpretation-1 may still need.

- Interpretation-4: Optimized GNSS usage

According to this interpretation, the “GNSS resilient operation” is to minimize the usage of the GNSS module, and some additional mechanims are needed to ensure the performance of system e,g., UL synchronization and mobility. This is originally considered for IoT-NTN in Rel-18[3] “for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed.”

However, for NR system, the later one is not hold since Rel-17 and UE can obtain the new position during the UL transmission. Regarding the additional consideration for power saving due to the GNSS module, although it highly depends on the UE implementation, given the stong interests along the progress made in Rel-18 for IoT-NTN, it can be further studied in NR-NTN based on the allocated TU.

Observation 1:	For the “GNSS resilisent operation”, different interpretations exist with various objectives for study.

Proposla 1: Based on the conclusion made in RAN#107, the study on “GNSS resilisent operation” should only focus on the following objective:

- Study the potential enhancement to improve GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. [RAN1, RAN2]

  - The solution defined in Rel-18 IoT is considered as the starting point.

Proposla 2: Other aspects/interpretation on the “GNSS resilisent operation” with fundamental changes (i.e., not aligned with the conclusion made in RAN#107) are recommended to be studied in new RAT (e.g., 6G).

  - Other Aspect for NTN

For other aspects, e.g.,

- Specify necessary enhancements to support E-UTRA TN to NR NTN handover in RRC connected mode

since it can be easily supported based on the solution defined in Rel-19 for idle mode, taking it as an separate LTE WI is reasonable given the high interest level.

Proposla 3: Additional LTE WI with the following objective is supported:

- Specify necessary enhancements to support E-UTRA TN to NR NTN handover in RRC connected mode.

Conclusion

Based on the discussion about NR-NTN in 5G-A Rel-20, the following observations and proposals are made:

Observation 1:	For the “GNSS resilisent operation”, different interpretations exist with various objectives for study.

Proposla 1: Based on the conclusion made in RAN#107, the study on “GNSS resilisent operation” should only focus on the following objective:

- Study the potential enhancement to improve GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. [RAN1, RAN2]

  - The solution defined in Rel-18 IoT is considered as the starting point.

Proposla 2: Other aspects/interpretation on the “GNSS resilisent operation” with fundamental changes (i.e., not aligned with the conclusion made in RAN#107) are recommended to be studied in new RAT (e.g., 6G).

Proposla 3: Additional LTE WI with the following objective is supported:

- Specify necessary enhancements to support E-UTRA TN to NR NTN handover in RRC connected mode.

References

- RP-250812 Summary for RAN Release 20 5G-Adv

- RP-250803	Moderator's summary for RAN2 led REL-20 topics	RAN2 Chair (InterDigital)

- RP-241351 Revised WID on IoT NTN enhancements