3GPP TSG RAN meeting #108	                                                RP-251726

Prague, Czech Republic, June 9-13, 2025

Title: 	Views on IOT-NTN in 5G-A Rel-20

Source: 	ZTE Corporation, Sanechips

Agenda item:	15.2.2

Document for:	Discussion and decision

Introduction

The following consensus has been reached at RAN#107 on IOT-NTN in Rel-20 [1]:

- Only the following objective will be considered as baseline for IoT NTN WI:

- Voice over NB-IoT-NTN via GEO

  - Study phase for 1 or 2 Quarters to down select between CP or UP. Checkpoint by RAN#1xy (TBD)

  - Work on detailed objectives for RAN#108

We are fine with this way forward and would like to share our understanding on the voice over NB-IOT-NTN via GEO and the detailed objectives for Rel-20.

Discussion

  - CP or UP solution for Voice

For NB-IoT, the technical specifications support transfer of user data in two different modes:

- Over CP, using NAS signalling between the UE and the MME (Control Plane CIoT EPS optimisation or DoNAS).

- Over UP (User Plane CIoT EPS optimisation).

In order to support voice over IOT-NTN, we have analyse the CP and UP solution from RAN side with a comparison table provided below.

Table 1: Comparison of CP and UP solution to support voice over IOT-NTN

|  | CIoT CP Solution | CIoT UP Solution |
| --- | --- | --- |
| QoS | Single set of E-RAB Level QoS Parameters | Mapping the SIP and data to different E-RAB Level QoS Parameters(List) |
| RRC | No RRC Reconfiguration No HO | Support RRC Reconfiguration No HO |
| Encryption | No AS encryption (NAS only) | AS-level |
| Integrity | No AS integrity (NAS only) | AS-level |
| PDCP | --No PDCP entity. --The traffics is capsulated in NAS PDU via SRB1 or SRB1bis. --No IP/RTP header compression (header compression in NAS only). | --Maximum 2 DRBs, one for SIP and the other for Voice --Support IP/RTP header compression. |
| RLC | AM only | --Both AM and UM. --Recommended: SIP via AM and Voice via UM |
| MAC | No flexibility to enable HARQ Feedback disable | DL SIP: HARQ Feedback enabled DL Voice: HARQ Feedback disabled |

As mentioned in the above table, for CP solution, there is only one single set of E-RAB level QoS parameters thus the eNB cannot differentiate SIP signaling and Voice and support differentiated handling. While for UP solution, 2 DRBs can be supported and it is expected to have one for SIP signaling and the other for Voice, making it easy to handle them differently. And if we aim to support differentiated handling of Voice and SIP signaling, spec impact in both RAN and CN is foreseen and the solution would be pretty similar as what we have defined for UP solution.

Observation 1:		For CP solution, there is no differentiated handling for Voice and SIP as there is only one single set of QoS parameters.

Observation 2:		If aiming to support differentiated handling of Voice and SIP, Voice and SIP signaling should be visible to eNB, which would have significant spec impact in both CN and RAN side and would be pretty similar as UP solution.

With the above analysis, we have the following proposal with a natural solution to support:

- Proposal 1:	UP solution is recommended for voice over IOT-NTN with limited efforts to support differentiated handling of Voice and SIP signaling.

  - Other RAN impacts

In addition, we understand the following functions will be impacted to support Voice over IOT-NTN:

ROHC

#### –	PDCP-Config-NB

The IE PDCP-Config-NB is used to set the configurable PDCP parameters for data radio bearers.

PDCP-Config-NB information element

-- ASN1START

PDCP-Config-NB-r13 ::=		SEQUENCE {

discardTimer-r13			ENUMERATED {

ms5120, ms10240, ms20480, ms40960,

ms81920, infinity, spare2, spare1

}	OPTIONAL,			-- Cond Setup

headerCompression-r13		CHOICE {

notUsed						NULL,

rohc						SEQUENCE {

maxCID-r13					INTEGER (1..16383)				DEFAULT 15,

profiles-r13				SEQUENCE {

profile0x0002				BOOLEAN,

profile0x0003				BOOLEAN,

profile0x0004				BOOLEAN,

profile0x0006				BOOLEAN,

profile0x0102				BOOLEAN,

profile0x0103				BOOLEAN,

profile0x0104				BOOLEAN

},

...

}

},

...,

[[	cipheringDisabled-r16		ENUMERATED {true}		OPTIONAL    -- Cond ConnectedTo5GC

]]

}

-- ASN1STOP

As shown above, ROHC has already been supported for UP solution in NB-IOT but we observe that some profiles, e.g. 0x0001 and 0x0101that would be useful for Voice need to be included for Voice over IOT-NTN.

Table 5.5.1.1: Supported ROHC protocols and profiles

| Profile Identifier | Usage: | Reference |
| --- | --- | --- |
| 0x0000 | No compression | RFC 5795 |
| 0x0001 | RTP/UDP/IP | RFC 3095, RFC 4815 |
| 0x0002 | UDP/IP | RFC 3095, RFC 4815 |
| 0x0003 | ESP/IP | RFC 3095, RFC 4815 |
| 0x0004 | IP | RFC 3843, RFC 4815 |
| 0x0006 | TCP/IP | RFC 6846 |
| 0x0101 | RTP/UDP/IP | RFC 5225 |
| 0x0102 | UDP/IP | RFC 5225 |
| 0x0103 | ESP/IP | RFC 5225 |
| 0x0104 | IP | RFC 5225 |

- Proposal 2:	Support ROHC for Voice over NTN and include useful ROHC profiles for data radio bearers.

Establishment cause for RRC connection establishment

As mentioned in [2], the establishment cause for voice and emergency call is not supported for NB-IOT and need to be addressed if we decide to support voice and emergency call.

| Establishment Cause for NB-IoT (Not including voice nor emergency) | Establishment Cause for LTE (Including voice and emergency) |
| --- | --- |
| -- ASN1START EstablishmentCause-NB-r13 ::= ENUMERATED { mt-Access, mo-Signalling, mo-Data, mo-ExceptionData, delayTolerantAccess-v1330, mt-EDT-v1610, spare2, spare1} -- ASN1STOP | EstablishmentCause ::= ENUMERATED { emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1} |

- Proposal 3:	Introduce establishment cause for Voice call in EstablishmentCause-NB-r13.

- With the above analysis, we have the following proposal for the detailed objectives.

- Proposal 4:	Objectives for Voice over IOT-NTN in Rel-20:

- Specify UP solution to support Voice over IOT-NTN via GEO

  - Include useful ROHC profiles for data radio bearers

  - Introduce establishment cause for Voice call in EstablishmentCause-NB-r13 IE.

Conclusion

Based on the discussion about IOT-NTN in 5G-A Rel-20, we have the following observations and proposals.

Observation 1:		For CP solution, there is no differentiated handling for Voice and SIP as there is only one single set of QoS parameters.

Observation 2:		If aiming to support differentiated handling of Voice and SIP, Voice and SIP signaling should be visible to eNB, which would have significant spec impact in both CN and RAN side and would be pretty similar as UP solution.

- Proposal 1:	UP solution is recommended for voice over IOT-NTN with limited efforts to support differentiated handling of Voice and SIP signaling.

- Proposal 2:	Support ROHC for Voice over NTN and include useful ROHC profiles for data radio bearers.

- Proposal 3:	Introduce establishment cause for Voice call in EstablishmentCause-NB-r13 IE.

- Proposal 4:	Objectives for Voice over IOT-NTN in Rel-20:

- Specify UP solution to support Voice over IOT-NTN via GEO

  - Include useful ROHC profiles for data radio bearers

  - Introduce establishment cause for Voice call in EstablishmentCause-NB-r13 IE

References

[1] RP-250812 Summary for RAN Release 20 5G-Adv

[2] RP-250391	Scope for Rel-20 IoT NTN	Ericsson